| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0694-v2 |
| --- | --- |

| Title: | CE4-related: Combination of affine mode clean up and line buffer reduction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Huanbang Chen Haitao Yang Jianle Chen Han Huang Wei-Jung Chien | Tel: Email: | +8615017554651 chenhuanbang@huawei.com haitao.yang@huawei.com jianle.chen@huawei.com hanhuang@qti.qualcomm.com wchien@qti.qualcomm.com |
| Source: | Huawei Technologies. Co., Ltd, Qualcomm |  |  |

_____________________________

- Abstract

In this contribution, it’s proposed to use the same sub-block motion vector field for the motion compensation, merge/skip/AMVP list derivation, deblocking, and storage of the temporal motion vector predictors (TMVPs). The control point motion vectors (CPMVs) are used for the affine motion data inheritance (i.e. for merge mode and affine AMVP mode) if the candidate block is in the current CTU. However, the bottom-left and bottom-right sub-block motion vectors are used for the affine motion data inheritance if the candidate block is in the above CTU. Therefore, no additional line buffer is required to store the CPMVs. It’s also proposed to use subblock MV at bottom-left and bottom-right location for affine data inheritance from above CTU without division by setting the coordinates of bottom-left and bottom-right corner are set to (xNb, yNb+neiH) and (xNb+neiW, yNb+neiH) for inheritance.

The BD-rate differences are 0.0x% in RA, -0.0x% in LB on top of VTM-2.0.1 plus already adopted affine proposals.

Introduction

In this contribution, the following are proposed:

- Affine mode clean-up. The CPMVs are stored in a separate buffer. The CPMVs are only used in the affine motion data inheritance. The sub-block MVs are used for motion compensation, merge/skip/AMVP list derivation, deblocking, and storage of the temporal motion vector predictors. And the sub-block based motion compensation for ATMVP and affine mode are aligned.

- Affine motion data inheritance from above CTU. If the candidate PU for affine motion data inheritance is in the above CTU line, the sub-block MVs in the line buffer instead of the CPMVs are used for the affine MVP derivation. In this way, the CPMVs are only stored in local buffer.

- Use sub-block MV at bottom-left and bottom-right location for affine data inheritance from above CTU. Since the sub-block MV represent the motion at the center of the sub-block, the distance of two corner sub-block MVs at the bottom of a candidate PU is neiW-4, wherein neiW is the width of the candidate PU. To avoid division by (neiW-4), which may not be power of 2, the rough distance neiW is used for inheritance. The coordinates of bottom-left and bottom-right corner are set to (xNb, yNb+neiH) and (xNb+neiW, yNb+neiH) for inheritance.

Simulation results

The contribution has been implemented on top of VTM-2.0.1 plus already adopted affine proposals. The test condition descripted in JVET-K1024 [1] is used for verifying the contribution. The summary results are provided in Table 1. On average, the BD-rate differences are 0.0x% in RA, and 0.0x% in LB, respectively.

Table 1 Coding performance over VTM-2.0.1 plus already adopted affine proposals

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | 0.01% | 0.02% | -0.01% | 100% | 103% |
| Class C | 0.00% | -0.07% | -0.17% | 100% | 102% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 0.02% | -0.14% | 0.10% | 100% | 103% |
| Class F (optional) | 0.00% | -0.03% | -0.06% | 100% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | -0.03% | -0.26% | 0.02% | 101% | 104% |
| Class E | -0.17% | -0.25% | -0.23% | 101% | 101% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.06% | 0.15% | -0.31% | 100% | 102% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

References

- H. Yang, S. Liu, K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding”, JVET-K1024, the 11th JVET meeting, July 2018, Ljubljana, SI.

- M. Zhou, B. Heng, “CE4-related: A clean up for affine mode”, JVET-L0047, the 12th JVET meeting, Macao, CN, 3–12 Oct. 2018.

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).