| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L1024-v3 |
| --- | --- |

| Title: | Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding |  |  |
| --- | --- | --- | --- |
| Status: | Output document |  |  |
| Purpose: | Core Experiment description |  |  |
| Author(s) or Contact(s): | Haitao Yang Shan Liu Kai Zhang | Email: | haitao.yang@huawei.com shanl@tencent.com zhangkai.video@bytedance.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

The goal of Core Experiment 4 (CE4) is to investigate various inter prediction and motion vector coding techniques on top of VTM 3. It comprises 5 categories,

- Merge mode simplification

- Merge mode enhancement

- Parallel processing for merge mode

- Motion vector coding

- Motion compensation constraints for complexity reduction

# CE4.1: Merge mode simplification

## Test specification

| Test # | Description | Source | Tester | Crosschecker |
| --- | --- | --- | --- | --- |
| 4.1.1.a | Limit the number of history candidates to check with threshold 5 | JVET-L0309 | J. Zhao (LGE) | L. Zhang (ByteDance) |
| 4.1.1.b | Limit the number of history candidates to check with threshold 6 |  | J. Zhao (LGE) | L. Zhang (ByteDance) |
| 4.1.1.c | 4.1.1.a with condition on the number of existing candidates |  | J. Zhao (LGE) | L. Zhang (ByteDance) |
| 4.1.1.d |  | 4.1.1.b with condition on the number of existing candidates |  |  |  | J. Zhao (LGE) |  | L. Zhang (ByteDance) |  |
| 4.1.2.a |  | HMVP with simplified pruning algorithm (number of HMVP to prune with threshold 3) |  | JVET-L0401 |  | Y. Han (Qualcomm) |  | C.-C. Chen (MediaTek) |  |
| 4.1.2.b |  | HMVP with simplified pruning algorithm (number of HMVP to prune with threshold 2) |  |  |  | Y. Han (Qualcomm) |  | C.-C. Chen (MediaTek) |  |
| 4.1.3 |  | History-based MVP without using the last lookup table entry |  | JVET-L0425 |  | T. Solovyev (Huawei) |  |  |  |
| 4.1.4.a | Composite average candidates | JVET-L0144 | J. Lee (LGE) |  |
| 4.1.4.b | 4.1.4.a harmonized with other merge candidates |  | J. Lee (LGE) |  |
| 4.1.5.a | Motion vector rounding before any MV pruning | JVET-L0214 | A. Robert (Technicolor) | T. Solovyev (Huawei) |
| 4.1.5.b | In merge apply some pruning on pairwise averaged candidates |  | A. Robert (Technicolor) | T. Solovyev (Huawei) |
| 4.1.5.c | 4.1.5.a + 4.1.5.b |  | A. Robert (Technicolor) | T. Solovyev (Huawei) |
| 4.1.6 | Hash-based pruning for merge list construction | JVET-L0470 | T. Solovyev (Huawei) |  |

## Evaluation criteria

The difference in the number of candidate comparison/pruning operations compared with the anchor (VTM 3) should be provided and crosschecked for all tests. Complexity change in other aspects should also be reported.

| Difference in the number of candidate comparison (+/- xx) | Max number of pruning stages | Others |
| --- | --- | --- |

Results when merge list size is set being equal to 6 is mandatory, if applicable to a proposed method. Testing other merge list size should be listed explicitly in the test specification.

## Technical description

### JVET-L0309 CE4-related: Simplification to History Based Motion Vector Prediction (LGE)

This test investigates methods to reduce the worst case number of pruning checks for History Based Motion Vector prediction (HMVP). To reduce the worst case number of pruning checks, the following methods are proposed:

- Limit the number of existing merge candidates to check,

- Limit the number of history candidates to check

### JVET-L0144 Composite merge candidate

In JVET-L0144, composite merge candidates are proposed for the merge list construction. The composite merge candidates are generated by averaging the candidates in current merge list according to spatial/temporal positions and reference indices. The following tests will be evaluated and studied in CE4:

- Composite average candidates

- Harmonization with other merge candidates

### JVET-L0214 Motion Vector Pruning (Technicolor)

This proposal includes two aspects. First, before any pruning operation takes place the involved motion vectors are computed in their final representation. This means the MVs are set to the appropriate precision level, rounded to the right motion vector resolution or clipped, before being compared to candidates already present in the concerned list. Second aspect consists in adding some pruning operations between candidates to improve the trade-off between coding efficiency and complexity.

### JVET-L0401 Modification on History-based Mode Vector Prediction

In JVET-L0401, two modifications on HMVP mode were proposed. One of the modification is removing the redundancy check from HMVP table update. The other modification is subsampling the HMVP table when adding HMVP candidates in merge list and AMVP list.

Test 1: HMVP with simplified pruning algorithm (number of HMVP to prune with threshold 3)

Test 2: HMVP with simplified pruning algorithm (number of HMVP to prune with threshold 2)

### JVET-L0425 History-based MVP without using the last lookup table entry

Method denies using last HMVP LUT entry/entries to reduce number of redundant comparisons during merge/AMVP candidate list construction.

### JVET-L0470 Hash-based pruning for merge list construction

Method for motion information comparison. Method relies on comparison hash values calculated based on motion vectors and reference indices. Hash values can be stored during AMVP/merge candidate list construction.

# CE4.2: Merge mode enhancement

## Test specification

| Test # | Description | Source | Tester | Crosschecker |
| --- | --- | --- | --- | --- |
| 4.2.1.a | Non-scaling STMVP (one candidate) | JVET-L0354 | T. Zhou (Sharp) | Y.-W. Chen (Kwai Inc.) |
| 4.2.1.b |  | Non-scaling STMVP (two candidates) |  |  |  | T. Zhou (Sharp) |  | Y.-W. Chen (Kwai Inc.) |  |
| 4.2.2.a |  | STMVP using a single adjacent candidate, a single adjacent left candidate and a TMVP candidate |  | JVET-L0207 |  | F. Le Léannec (Technicolor) |  | Y.Han (Qualcomm) |  |
| 4.2.2.b |  | STMVP using a single adjacent candidate, a single adjacent left candidate and a TMVP candidate with merge list size = 8 |  |  |  | F. Le Léannec (Technicolor) |  | Y.Han (Qualcomm) |  |
| 4.2.2.c |  | Multiple STMVP candidates |  |  |  | F. Le Léannec (Technicolor) |  | Y.Han (Qualcomm) |  |
| 4.2.2.d |  | Multiple STMVP candidates with merge list size = 8 |  |  |  | F. Le Léannec (Technicolor) |  | Y.Han (Qualcomm) |  |
| 4.2.3.a |  | STMVP simplification (no scaling, remove the non-adjacent candidates part of JVET-L0399) |  | JVET-L0399 |  | Y.-H. Cha (Qualcomm) |  | F. Le Léannec (Technicolor) |  |
| 4.2.3.b | Multiple STMVP candidates |  | Y.-H. Cha (Qualcomm) | F. Le Léannec (Technicolor) |
| 4.2.4.a | History based STMVP (one candidate) without scaling | JVET-L0302 | X. Xu (Tencent) | T. Zhou (Sharp) |
| 4.2.4.b | History based STMVP (two candidates) without scaling |  | X. Xu (Tencent) |  |
| 4.2.5.a | Merge offset extension based on 3 candidates 8 x N (Position fixed offset candidates) | JVET-L0176 | X. Chen (Hisilicon) | L. Xu (Hikvision) |
| 4.2.5.b | Merge offset extension based on 2 candidates 8 x N (Position fixed offset candidates) |  | X. Chen (Hisilicon) | L. Xu (Hikvision) |
| 4.2.5.c | Merge offset extension that combined with MV coding Merge List size = 6 with 8 x N (Position fixed offset candidates) |  | X. Chen (Hisilicon) |  |
| 4.2.6.a | Spatial merge candidate reordering in combination with merge offset extension scheme (JVET-L0176) | JVET-L0187 | L. Xu (HikVision) |  |
| 4.2.6.b | 4.2.6.a in combination with 4.2.5.a |  | L. Xu (HikVision) |  |
| 4.2.6.c | 4.2.6.a in combination with 4.2.5.b |  | L. Xu (HikVision) |  |
| 4.2.6.d | 4.2.6.a in combination with 4.2.5.c |  | L. Xu (HikVision) |  |

## Evaluation criteria

Generally, RD optimization during merge list construction should not be changed. Any change on RD optimization should be reported.

The complexity analysis of list construction for all tests should be provided and crosschecked.

| Merge list size | Max number of potential candidates | Max number of candidate comparison | Max number of pruning stages | Max number of MV scaling | Others |
| --- | --- | --- | --- | --- | --- |

Results when merge list size is set being equal to 6 is mandatory, if applicable to a proposed method. Testing other merge list size should be listed explicitly in the test specification.

## Technical description

### JVET-L0399 Improvement on Merge/Skip mode

In JVET-L0399, multiple non-sub-PU STMVP candidates were proposed. The STMVP candidates is generated by averaging two spatial candidates and TMVP candidate. For example, the STMVP candidate is generated by averaging the first available candidate of candidate 4 and candidate 1, the first available candidate of candidate 3 and candidate 2, and the TMVP. In this CE, the following tests will be further studied. In order to simplify STMVP, on scaling is used.

Test 1: one STMVP candidates

Test 2: multiple STMVP candidates

### JVET-L0354 Spatial-temporal merge mode (Sharp)

The proposed method derives two spatial neighbouring motion information and one temporal neighbouring motion information and generates a spatial-temporal candidate.

For the spatial neighbouring positions, the default spatial candidate positions are checked as well as up to two additional positions. i.e. (nPbW * 2, -1) and (-1, nPbH * 2).

For the temporal neighbouring position, the same position as the collocated position is used.

The motion vector is derived with an average of the two or three motion vectors without full scaling. The average of spatial motion vectors without temporal motion vector is also utilized for two candidate test.

Note: If the number of available spatial motion vectors is less than two, the STMVP mode is considered as not available.

Test1; One STMVP candidate

Test2: Two STMVP candidates

### JVET-L0302 History based spatial-temporal merge mode (Tencent)

The proposed method uses two sets of motion information from history based MV buffer and one temporal neighboring motion information to generate a spatial-temporal candidate.

The last two entries in the history buffer are used. For the temporal neighbouring position, the same position as the collocated position is used.

The motion vector is derived with an average of the two or three motion vectors without scaling. The average of two history-based motion vectors is also considered as an additional merge candidate.

Note: If the number of available spatial motion vectors is less than two, the STMVP mode is considered as not available.

Test1; One candidate: average of two history based MV + one TMVP

Test2: Two candidates: average of two history based MV + one TMVP; average of two history based MV

### JVET-L0207 Simplified Spatial Temporal Motion Vector Prediction (Technicolor)

In this proposal, the spatial temporal motion vector prediction is generated by checking one single MVP position on the left of current block, one single position above current block, and computing the average between the spatial candidate(s) and the temporal (STMVP) motion vector predictor. If less than 2 spatial/temporal candidates are available, then the STMVP candidate is considered as not available. The computation of the TMVP candidate is modified compared to HEVC, by clipping the bottom-right TMVP position inside the current CTU row if necessary. For limited complexity increase, STMVP will be tested without added MV scaling or candidate pruning. In addition, multiple STMVP candidates may be added to the merge candidate list.

### JVET-L0176 Merge offset extension

In Merge offset extension, more candidates are checked based on existed candidate. New candidates has mv offset to the mv of existed candidate. New extended mv offset candidates are constructed only based on the first candidate of merge candidate list.

### JVET-L0187 Merge Candidate List Construction

This contribution proposes a merge candidate list construction method. This method is composed by two parts.

Firstly, spatial candidates will be reordered to provide a better candidate in the front position.

| Width>= height | B1A1A0B0 |
| --- | --- |
| Width<height | A1B1B0A0 |

Secondly, some new candidates produced by adding mv offset to the first candidate will be added to the list before default zero candidates. The offset method can be simply clarified as the below Table. At most, four candidates are added. The total candidate number keep unchanged.

# CE4.3: Parallel processing for merge mode

## Test specification

| Test # | Description | Source | Tester | Crosschecker |
| --- | --- | --- | --- | --- |
| 4.3.1.a | Share merge list for translational merge and sub-CU merge with fixed threshold = 32 | JVET-L0091 | C.-C. Chen (MediaTek) | Y. Han (Qualcomm) |
| 4.3.1.b | Share merge list for translational merge and sub-CU merge with fixed threshold = 64 |  | C.-C. Chen (MediaTek) | Y. Han (Qualcomm) |
| 4.3.1.c | Share merge list for translational merge and sub-CU merge with fixed threshold = 128 |  | C.-C. Chen (MediaTek) | Y. Han (Qualcomm) |
| 4.3.1.d | Share merge list for translational merge and sub-CU merge with fixed threshold = 256 |  | C.-C. Chen (MediaTek) | Y. Han (Qualcomm) |
| 4.3.1.e | Share merge list for translational merge and sub-CU merge with fixed threshold = 512 |  | C.-C. Chen (MediaTek) | Y. Han (Qualcomm) |
| 4.3.2.a | Fixed square merge estimation region for parallel estimation with MER = 8x8 | JVET-L0216 | S. Esenlik (Huawei) | J. Ma (HHI) |
| 4.3.2.b | Fixed square merge estimation region for parallel estimation with MER = 16x16 |  | S. Esenlik (Huawei) | J. Ma (HHI) |
| 4.3.2.c | Fixed square merge estimation region for parallel estimation with MER = 32x32 |  | S. Esenlik (Huawei) | J. Ma (HHI) |
| 4.3.2.d | Fixed square merge estimation region for parallel estimation with MER = 64x64 |  | S. Esenlik (Huawei) | J. Ma (HHI) |
| 4.3.2.e | Fixed square MER is applied if all CUs are covered by the MER with MER = 8x8 |  | S. Esenlik (Huawei) | J. Ma (HHI) |
| 4.3.2.f | Fixed square MER is applied if all CUs are covered by the MER with MER = 16x16 |  | S. Esenlik (Huawei) | J. Ma (HHI) |
| 4.3.2.g | Fixed square MER is applied if all CUs are covered by the MER with MER = 32x32 |  | S. Esenlik (Huawei) | J. Ma (HHI) |
| 4.3.2.h | Fixed square MER is applied if all CUs are covered by the MER with MER = 64x64 |  | S. Esenlik (Huawei) | J. Ma (HHI) |
| 4.3.2.i | Rectangular MER is tested with MER = 32 |  | S. Esenlik (Huawei) | J. Ma (HHI) |
| 4.3.2.j | Rectangular MER is tested with MER = 64 |  | S. Esenlik (Huawei) | J. Ma (HHI) |
| 4.3.2.k | Rectangular MER is tested with MER = 128 |  | S. Esenlik (Huawei) | J. Ma (HHI) |
| 4.3.2.l | Rectangular MER is tested with MER = 256 |  | S. Esenlik (Huawei) | J. Ma (HHI) |

## Technical description

### JVET-L0091 Shared merge list

In JVET-L0091, a shared merge list method is described to share the same merge list for all leaf coding units (CUs) of one ancestor node in the CU split tree for enabling parallel processing and shared computation of merge mode for small CUs. The ancestor node is named merge sharing node. The shared merge list is generated at the merge sharing node pretending the merge sharing node is a leaf CU.

Within this CE, the performance of the method described in JVET-L0091 is to be studied for different requirements, such as shared translational merge only or shared translational merge and sub-CU merge. The different shared region size threshold will be evaluated.

### JVET-L0216 Parallel Merge Estimation for VVC

A method for parallel estimation of merge mode of neighboring blocks is proposed, which takes into account the binary tree and ternary tree partitions of VVC. The proposed method allows estimation of the merge mode and affine merge mode of two coding blocks if any part of the two blocks are in the same merge estimation region. The proposed method takes into account the historical merge candidates as well as sub-pu based merge modes. Optionally it is applied to the affine merge mode.

Test 1 – fixed square MER grid is tested.

Test 2 – fixed square MER is applied if all CUs are covered by the MER.

Test 3 – rectangular MER is tested.

# CE4.4: Motion vector coding

## Test specification

| Test # | Description | Source | Tester | Crosschecker |
| --- | --- | --- | --- | --- |
| 4.4.1.a | Unified MVD coding with reduced context coded bins | JVET-L0300 | S. Paluri (LGE) | L.P. Van (Qualcomm) |
| 4.4.1.b | MVD coding with reduced context coded bins |  | S. Paluri (LGE) | L.P. Van (Qualcomm) |
| 4.4.2 | Bilinear motion vector prediction with AMVP candidates | JVET-L0301 | B. Choi (Sharp) |  |
|  | Bilinear motion vector prediction with Merge candidates | JVET-L0298 | B. Choi (Sharp) |  |
| 4.4.3 | Symmetrical MVD mode | JVET-L0370 | H. Chen (Huawei) | G. Li (Tencent) |
| 4.4.4.a | Diagonal direction candidate for MMVD | JVET-L0355 | T. Hashimoto (Sharp) | J. Li (Panasonic) |
| 4.4.4.b | Multiple distance lists for MMVD |  | T. Hashimoto (Sharp) | J. Li (Panasonic) |
| 4.4.4.c | Combination of directional candidates and multiple distance list |  | T. Hashimoto (Sharp) | J. Li (Panasonic) |
| 4.4.4.d | Combination of directional candidates and multiple distance list with full-pel position for large range |  | T. Hashimoto (Sharp) | J. Li (Panasonic) A. Karabutov (Huawei) |
| 4.4.4.e | Combination of 4.4.4.a and 4.4.5.c |  | T. Hashimoto (Sharp) | L. Xu (Hikvision) |
| 4.4.5.a | MMVD with adaptive distance table based on occurrences | JVET-L0408 | J. Li (Panasonic) | T. Hashimoto (Sharp) |
| 4.4.5.b | MMVD with adaptive distance table based on picture resolution |  | J. Li (Panasonic) | T. Hashimoto (Sharp) |
| 4.4.5.c | 4.4.5.a + 4.4.5.b |  | J. Li (Panasonic) | T. Hashimoto (Sharp) A. Karabutov (Huawei) |

## Technical description

### JVET-L0300 CE4-related: Generic Vector Coding of Motion Vector Difference (LGE)

Current VTM MVD coding uses 4 context coded bins per MVD coding so that maximum 24 context coded bins per CU/PU is required (ex: affine 6 parameter model is biprediction). In this test, several unified MVD coding methods are tested to reduce the number of context coded bins considering the following aspects.

– Unified MVD coding between MVDx and MVDy

– MVD coding based on reduced context coded bins

### JVET-L0301: Bilinear motion vector prediction

In the bilinear motion vector prediction (BIMVP) mode, mvp_flag, coded MVD for list0 and mvp_flag for list1 are signaled. This means that the MVP for list1 is not derived from the MVP for list0. Instead, it is explicitly indicated by signaling. For example, when the MV for list0 is equal to (V0x + offset_x, V0y + offset_y), its paired MV for list1 is set equal to (V1x - offset_x, V1y - offset_y).

The BIMVP is improved with the following features:

- bimvp_flag is signalled before inter_pred_idc as a short-cut flag. If bimvp_flag is equal to 1, the current CU is coded with BIMVP mode.

- In the BIMVP mode, inter_pre_idc and ref_idx are inferred to be equal to PRED_BI and 0 respectively. The values of  mvp_flag and motion vector difference (mvd) for list1 are derived from those values for list0.

3.  Alternatively, merge candidates are used as MV predictors instead of AMVP candidates.

### JVET-L0370: Symmetrical mode for bi-prediction

In JVET-L0370, firstly, in slice level, variables BiDirPredFlag, RefIdxSymL0 and RefIdxSymL1 are derived by searching the reference pictures in List 0 and List 1.

In CU level, a symmetrical mode flag indicating whether symmetrical mode is used or not is explicitly signaled if the prediction direction for the CU is bi-prediction and BiDirPredFlag is equal to 1.

When the flag is true, only mvp_l0_flag, mvp_l1_flag and MVD0 are explicitly signaled. The reference indices are set equal to RefIdxSymL0, RefIdxSymL1 for list 0 and list 1, respectively. MVD1 is just set equal to –MVD0. The final motion vectors are shown in below formula.

mvx0, mvy0=(mvpx0+mvdx0,mvpy0+mvdy0) mvx1, mvy1=(mvpx1-mvdx0,mvpy1-mvdy0)

Figure 1 Illustration for symmetrical mode

### JVET-L0355 Non-CE4: Enhanced ultimate motion vector expression (Sharp)

In the proposal, MMVD is extended in the following points.

- Directional candidates

- Multiple distance lists

- Full-pel (integer) position for large distance (from JVET-L0408)

Directional candidates are used in addition to horizontal/vertical candidates in VTM-3. The directional candidates are shown as below.

| Direction IDX | 000 | 001 | 010 | 011 | 100 | 101 | 110 | 111 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x-axis | +1 | –1 | 0 | 0 | +1/2 | -1/2 | -1/2 | +1/2 |
| y-axis | 0 | 0 | +1 | –1 | +1/2 | -1/2 | +1/2 | -1/2 |

Multiple set of the distance lists is used. The example of the list is shown as follows:

First distance list

| Distance IDX | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| Pixel distance | 1/4-pel | 1/2-pel | 3/4-pel | 5/4-pel |

Second distance list

| Distance IDX | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| Pixel distance | 1-pel | 2-pel | 4-pel | 8-pel |

The values of directional candidate and the distance lists can be modified to harmonize VTM3.0 encoding. Those values can be different on each subtest since the best performing distance list should be different depending on usage of directional candidate.

In an additional test, the motion vectors are converted into an integer position if the distance is large for complexity reduction.

### JVET-L0408 CE4-related: Improvement on ultimate motion vector expression (Panasonic)

The following modifications are introduced to MMVD:

(1) Use an additional table consisting of diagonal direction candidates as shown in Table 1. One of two tables is selected according to the direction of base motion vector as shown in Figure 1.

(2) Use adaptive distance table based on picture resolution, i.e., if picture resolution is not larger than 2K, i.e., 1920×1080, Table 2 is used as base distance table; Otherwise, Table 3 is used as base distance table.

(3) Use adaptive distance table based on occurrence-based distance table reordering. Distance index is reordered according to the occurrence of usage of each distance index in previous coded pictures ranking from high to low.

(4) MMVD candidate values are modified so that CU of MMVD mode has full-pel instead of sub-pel motion vector if the MMVD distance is greater than a threshold.

Table 1: Direction table

|  | Direction IDX | 00 | 01 | 10 | 11 |
| --- | --- | --- | --- | --- | --- |
| Hor./Ver. table | x-axis | + | – | N/A | N/A |
|  | y-axis | N/A | N/A | – | + |
|  | Direction IDX | 00 | 01 | 10 | 11 |
| Diagonal table | x-axis | + | – | + | – |
|  | y-axis | + | – | – | + |

Table 2: MMVD distance table candidate

| Distance IDX | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pixel distance | 1/4-pel | 1/2-pel | 1-pel | 2-pel | 4-pel | 8-pel | 16-pel | 32-pel |

Table 3: MMVD distance table

| Distance IDX | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pixel distance | 1-pel | 2-pel | 4-pel | 8-pel | 16-pel | 32-pel | 64-pel | 128-pel |

Figure 1. Direction table selection

# CE4.5: Motion compensation constraints for complexity reduction

## Test specification

| Test # | Description | Source | Tester | Crosschecker |
| --- | --- | --- | --- | --- |
| 4.5.1 | Disabling bi-prediction for 4x8 and 8x4 CUs | JVET-L0122 | R.-L. Liao (Panasonic) | S. Sethuraman (Ittiam) |
| 4.5.2 | Disabling bi-prediction or inter prediction for CU which memory bandwidth exceeds that of 8x8 bi-prediction |  | R.-L. Liao (Panasonic) | S. Sethuraman (Ittiam) |

## Technical description

### JVET-L0122 AHG5: Reduction of worst case memory bandwidth (Panasonic)

For merge and inter mode, bi-prediction is disabled for small CU to guarantee that the worst-case memory bandwidth is not exceeded that of 8x8 bi-prediction. For the CU coded in merge mode, the motion vector from L0 direction of a bi-prediction motion vector candidate is used to predict the CU, which is the same situation in HEVC. For the CU coded in inter mode, bi-prediction is prohibited. Besides, the first bin of the inter direction which indicates uni-prediction or bi-prediction is not signaled.

# Test condition and evaluation criteria

Each testing item is required to be compared against VTM 3, evaluated using BD rates, encoding time, decoding time according to Common Test Conditions (CTC) in JVET-L1010. Simulation with RA and LDB configurations are mandatory.

Sub-CE specific test condition and evaluation criteria may be defined for a sub-CE if considered informational for tool evaluation. Please refer to the section of test condition and evaluation criteria in each sub-CE for information.

# SW integration guideline

All tests are required to be implemented on top of VTM 3.

# Timeline

| T1 | Nov. 2, 2018 | Finalize CE description |
| --- | --- | --- |
| T2 | 2 weeks after VTM 3 release | Deliver software packages and begin cross-checking |
| T3 | Dec. 19, 2018 | Software frozen (only bugfix allowed since T2), testers provide full results, and final crosschecking begins |

# Participant list

| Company/ Organization | Participant Name | Email Address |
| --- | --- | --- |
| Technicolor | Antoine Robert Philippe Bordes Tangi Poirier Fabrice Le Léannec Franck Galpin Ya Chen | antoine.robert@technicolor.com philippe.bordes@technicolor.com tangi.poirier@technicolor.com fabrice.leleannec@technicolor.com franck.galpin@technicolor.com Ya.CHEN@technicolor.com |
| HHI | Martin Winken Robert Skupin Jens Brandenburg Anastasia Henkel | martin.winken@hhi.fraunhofer.de robert.skupin@hhi.fraunhofer.de jens.brandenburg@hhi.fraunhofer.de anastasia.henkel@hhi-extern.fraunhofer.de |
| Huawei | Huanbang Chen Haitao Yang Jianle Chen Sychev Maxim Solovyev Timofey Alexey Filippov Vasily Rufitskiy Semih Esenlik Alexander Karabutov | chenhuanbang@huawei.com haitao.yang@huawei.com jianle.chen@huawei.com Sychev.Maxim@huawei.com solovyev.timofey@huawei.com alexey.filippov@huawei.com vasily.rufitskiy@huawei.com semih.esenlik@huawei.com karabutov.alexander@huawei.com |
| LGE | Seung Hwan Kim Jaehyun Lim Hyeongmun Jang Jie Zhao Seethal Paluri | seunghwan3.kim@lge.com Jaehyun.lim@lge.com hm.jang@lge.com jie.zhao@lge.com seethal.paluri@lge.com |
| Tencent | Xiaozhong Xu Jing Ye Guichun Li Shan Liu Meng Xu Xiang Li | xiaozhongxu@tencent.com jingye@tencent.com guichunli@tencent.com shanl@tencent.com mengxxu@tencent.com xlxiangli@tencent.com |
| USTC | Yue Li Li Li | lytt@mail.ustc.edu.cn lili90th@gmail.com |
| HiSilicon | Na Zhang Xu Chen Jianhua Zhang | zhangna25@huawei.com anderson.chen@hisilicon.com zhengjianhua@hisilicon.com |
| BBC | Saverio Blasi | saverio.blasi@bbc.co.uk |
| ETRI | Jungwon Kang Sung-Chang Lim Hahyun Lee | jungwon@etri.re.kr sclim@etri.re.kr hanilee@etri.re.kr |
| Panasonic | Kiyofumi Abe Jingya Li Ru-Ling Liao | abe.kiyo@jp.panasonic.com jingya.li@sg.panasonic.com ruling.liao@sg.panasonic.com |
| Fujitsu | Jianqing Zhu Kimihiko Kazui | zhujianqing@cn.fujitsu.com kazui.kimihiko@jp.fujitsu.com |
| Sony | Kenji Kondo | kenji.kondo@sony.com |
| Qualcomm | Wei-Jung Chien Vadim Seregin Chun-Chi Chen Yan Zhang Yu Han Han Huang Chao-Hsiung Hung Luong Pham Van Yung-Hsuan Chao | wchien@qti.qualcomm.com vseregin@qti.qualcomm.com chunchic@qti.qualcomm.com yzh@qti.qualcomm.com yuhan@qti.qualcomm.com hanhuang@qti.qualcomm.com chaohsiu@qti.qualcomm.com lphamvan@qti.qualcomm.com yunghsua@qti.qualcomm.com |
| Alibaba | Yu-Chen Sun | yuchen.s@alibaba-inc.com |
| Hikvision | Fangdong Chen | chenfangdong@hikvision.com |
| Sharp | Weijia Zhu Byeongdoo Choi Tomonori Hashimoto Takeshi Chujoh Tomohiro Ikai Kiran Misra Eiichi Sasaki Tianyang Zhou | zhuw@sharplabs.com choib@sharplabs.com tomonori.hashimoto@sharp.co.jp chujoh.takeshi@sharp.co.jp ikai.tomohiro@sharp.co.jp misrak@sharplabs.com eiichi.sasaki@sharp.co.jp zhou.tianyang@sharp.co.jp |
| Ericsson | Ruoyang Yu Julien Le Tanou | ruoyang.yu@ericsson.com julien.le.tanou@ericsson.com |
| Samsung Electronics | Min Woo Park Seungsoo Jeong Anish Tamse Kiho Choi Jie Chen Minsoo Park Yinji Piao | m.w.park@samsung.com ss00.jeong@samsung.com tamse.anish@samsung.com kiho14.choi@samsung.com jie1222.chen@samsung.com mss.park@samsung.com yj1003.piao@samsung.com |
| Nokia | Jani Lainema Alireza Aminlou Ramin Ghaznavi-Youvalari | jani.lainema@nokia.com alireza.aminlou@nokia.com ramin.ghaznavi-youvalari@nokia.com |
| InterDigital | Yuwen He Xiaoyu Xiu Daniel Luo Yan Ye | Yuwen.He@interdigital.com Xiaoyu.Xiu@interdigital.com Daniel.Luo@InterDigital.com Yan.Ye@interdigital.com |
| Arris | Krit Panusopone Yue Yu David Baylon Seungwook Hong Limin Wang | Krit.panusopone@arris.com Yue.yu@arris.com David.baylone@arris.com Seungwook.hong@arris.com Limin.wang@arris.com |
| MediaTek | Yu-Wen Huang Chih-Wei Hsu Ching-Yeh Chen Tzu-Der Chuang Shih-Ta Hsiang Yuling Hsiao Jenny Lai Yu-Chi Su Zhi-Yi Lin Chun-Chia Chen Yucheng Lin | yuwen.huang@mediatek.com cw.hsu@mediatek.com chingyeh.chen@mediatek.com Peter.Chuang@mediatek.com shih-ta.hsiang@mediatek.com Yuling.Hsiao@mediatek.com Jenny.Lai@mediatek.com Yu-Chi.Su@mediatek.com Zhi-Yi.Lin@mediatek.com chunchia.chen@mediatek.com Yucheng.Lin@mediatek.com |
| NHK | Shunsuke Iwamura | iwamura.s-gc@nhk.or.jp |
| Netflix | Andrey Norkin | anorkin@netflix.com |
| Orange | Gordon Clare | gordon.clare@orange.com |
| Broadcom | Minhua Zhou Pensong Chen Wade Wan | minhua.zhou@broadcom.com peisong.chen@broadcom.com wade.wan@broadcom.com |
| Canon | Jonathan Taquet | Jonathan.TAQUET@crf.canon.fr |
| PKU | Zhao Wang | zhaowang@pku.edu.cn |
| Chips&Media | Wook Je Jeong | nick.jeong@chipsnmedia.com |
| Aachen University | Johannes Sauer | sauer@ient.rwth-aachen.de |
| HIT | Yang Wang Xiaopeng Fan | wangyang.cs@hit.edu.cn fxp@hit.edu.cn |
| WILUS | Greg Ko | greg.ko@wilusgroup.com |
| ITRI | Ching-Chieh Lin | JackLin@itri.com |
| Bytedance | Kai Zhang Li Zhang | zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Kwai | Yi-Wen Chen Xianglin Wang | yiwenchen@kwai.com xianglinwang@kwai.com |
| HNU | Haechul Choi Heeji Han Daehyeok Gwon jaeryun Choe | choihc@hanbat.ac.kr sarhakor@naver.com skyeye0530@naver.com tjswo9395@naver.com |
| Dolby | Peng Yin | pyin@dolby.com |
| Intel | Grace Yu | grace.yu@intel.com |