| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L1027-v1 |
| --- | --- |

| Title: | Description of Core Experiment 7 (CE 7): Quantization and coefficient coding |  |  |
| --- | --- | --- | --- |
| Status: | Output document approved by JVET |  |  |
| Purpose: | CE Description |  |  |
| Author(s) or Contact(s): | Heiko Schwarz Muhammed Coban Cheung Auyeung | Email: | heiko.schwarz@hhi.fraunhofer.de mcoban@qti.qualcomm.com cheung.auyeung@huawei.com |
| Source: | CE Coordinators |  |  |

_____________________________

# Introduction

The purpose of this core experiment is to explore the coding efficiency and complexity impact of proposed algorithms for quantization and transform coefficient coding. This core experiment evaluates the impact of:

- Context modeling/selection/coding order for syntax elements related to transform coefficients

The objectives of this CE include:

- Investigation of the impact of context selection and signaling schemes for transform coefficient levels on the coding efficiency and complexity.

# Participants List

| Affiliation | Person, Email address |
| --- | --- |
| Qualcomm | Muhammed Coban <mcoban@qti.qualcomm.com> Dmytro Rusanovskyy <dmytror@qti.qualcomm.com> Jie Dong <jiedong@qti.qualcomm.com> Ted Hsieh <thsieh@qti.qualcomm.com> Marta Karczewicz <martak@qti.qualcomm.com> |
| Fraunhofer HHI | Heiko Schwarz <heiko.schwarz@hhi.fraunhofer.de> Tung Nguyen <tung.nguyen@hhi.fraunhofer.de> Christian Helmrich <christian.helmrich@hhi.fraunhofer.de> |
| Samsung | Yinji Piao <yj1003.piao@samsung.com> Kiho Choi <kiho14.choi@samsung.com> |
| Huawei | Alexey Filippov <alexey.filippov@huawei.com> Jianle Chen <Jianle.chen@huawei.com> Cheung Auyeung <cheung.auyeung@huawei.com> Yang Haitao <haitao.yang@huawei.com> Yin Zhao <yin.zhao@huawei.com> Sergey Ikonin <Sergey.Ikonin@huawei.com> Biao Wang <biao.wang@huawei.com> Vasily Rufitskiy <vasily.rufitskiy@huawei.com> Alexander Karabutov <karabutov.alexander@huawei.com> Sergey Ikonin <Sergey.Ikonin@huawei.com> |
| Orange | Pierrick Philippe <pierrick.philippe@orange.com> Felix Henry <felix.henry@orange.com> Gordon Clare <gordon.clare@orange.com> |
| MediaTek | Mohammed Sarwer <Mohammed.Sarwer@mediatek.com> Olena Chubach <Olena.Chubach@mediatek.com> Yu-Wen Huang <yuwen.huang@mediatek.com> Chih-Wei Hsu <cw.hsu@mediatek.com> Ching-Yeh Chen <chingyeh.chen@mediatek.com> T.-D. Chuang <peter.chuang@mediatek.com> Shih-Ta Hsiang <shih-ta.hsiang@mediatek.com> |
| Sharp | Eiichi Sasaki <eiichi.sasaki@sharp.co.jp> Takeshi Chujoh <chujoh.takeshi@sharp.co.jp> Tomohiro Ikai <ikai.tomohiro@sharp.co.jp> Kiran Misra <misrak@sharplabs.com> Andrew Segall <asegall@sharplabs.com> Frank Bossen <frank.bossen@me.com> |
| Apple | Alexis Tourapis <atourapis@apple.com> Jae Hoon Kim <i.jaehoon.kim@icloud.com> |
| KDDI | Kei Kawamura <ki-kawamura@kddi.com> Kyohei Unno <ky-unno@kddi-research.jp> Yoshitaka Kidani <yo-kidani@kddi.com> |
| Kwai | Yi-Wen Chen <yiwenchen@kwai.com> Xianglin Wang <xianglinwang@kwai.com> |
| Bytedance | Li Zhang <lizhang.idm@bytedance.com> Hongbin Liu <liuhongbin.01@bytedance.com> |
| Nokia | Jani Lainema <jani.lainema@nokia.com> |
| Broadcom | Minhua Zhou <minhua.zhou@broadcom.com> Wade Wan <wade.wan@broadcom.com> Peisong Chen <peisong.chen@broadcom.com> |
| Ericsson | Julien Le Tanou <julien.le.tanou@ericsson.com> |
| Sony | Takeshi Tsukuba <Takeshi.Tsukuba@sony.com> Karl Sharman <karl.sharman@eu.sony.com> |
| InterDigital | Xiaoyu Xiu <xiaoyu.xiu@interdigital.com> Yuwen He <Yuwen.He@InterDigital.com> Rahul Vanam <Rahul.vanam@interdigital.com> |
| Canon | Chris Rosewarne <chris.rosewarne@cisra.canon.com.au> Andrew Dorrell <andrew.dorrell@cisra.canon.com.au> |
| BBC | Andre Seixas Dias <Andre.SeixasDias@bbc.co.uk> Saverio Blasi <saverio.blasi@bbc.co.uk> |
| Technicolor | Fabien Racape <fabien.racape@technicolor.com> Ya Chen <Ya.chen@technicolor.com> Edouard Francois <Edouard.francois@technicolor.com> Philippe de Lagrange <Philippe.delagrange@technicolor.com> Leleannec Fabrice <Fabrice.Leleannec@technicolor.com> |
| HiSilicon | Jianhua Zheng <zhengjianhua@hisilicon.com> Quanhe Yu <yuquanhe@hisilicon.com> |
| ETRI | Jungwon Kang <jungwon@etri.re.kr> Sung-Chang Lim <sclim@etri.re.kr> |
| Panasonic | Kiyofumi Abe <abe.kiyo@jp.panasonic.com> Han Boon Teo <hanboon.teo@sg.panasonic.com> Chong Soon Lim <chongsoon.lim@sg.panasonic.com> |
| LG | Jung-Ah Choi <jungah.choi@lge.com> Jin Heo <jin78.heo@lge.com> Sunmi Yoo <sunmi.yoo@lge.com> Jangwon Choi <jangwon84.choi@lge.com> Ling Li <l.li@lge.com> Seung Hwan Kim <seunghwan3.kim@lge.com> Mehdi Salehifar <mehdi.salehifar@lge.com> Moonmo Koo <moonmo.koo@lge.com> Jaehyun Lim <jaehyun.lim@lge.com> |
| Dolby | Sean McCarthy <smcca@dolby.com> Peng Yin <pyin@dolby.com> |
| Comcast | Dan Grois <grois@ieee.org> |
| Intel | Sabine Roessel <sabine.roessel@intel.com> |
| Tsinghua University | Benben Niu <nbb16@mails.tsinghua.edu.cn> |
| Tencent | Min Gao <maxwellgao@tencent.com> Xiang Li <xlxiangli@tencent.com> |
| WILUS | Dongcheol Kim <dongcheol.kim@wilusgroup.com> |
| Foxconn | Yao-Jen Chang <yaojen.chang.foxconn@gmail.com> |

# Tools under Test

The following tools will be considered in this CE. Details are described in sec. 6.

|  | Proponent | Document | Tool description |
| --- | --- | --- | --- |
| 1 | MediaTek | JVET-L0145 | Different constraints on number of context-coded bins for luma and chroma (and different subblock sizes) |
| 2 | MediaTek | JVET-L0145 | Constraints on number of context-coded bins depend on last significant scan position |
| 3 | MediaTek | JVET-L0145 | Include greater-than-two flag into first pass over subblocks |

# Test Description

## Software, reference configurations, and test conditions

All tools have to be integrated into the VTM-2 software (outcome of the Ljubljana meeting).  Results have to be reported for the following two configurations:

- VTM-2 software and common test conditions (standard configuration files);

- VTM-2 software in low-complexity encoder configuration (common test conditions, but dependent quantization and RDOQ disabled and sign data hiding enabled).

All tools are tested against the corresponding anchor (same set of tools, except the tool to be tested).

## Coding Performance Measurements

Measure impact on bitrate/PSNR using provided data (4 points, BD-rate). Proponents are encouraged to provide supplementary results at lower or higher QPs.

## Complexity Considerations

To provide an indicative assessment of complexity, the following numbers should be provided:

- Encoding time (compared to the reference configurations);

- Decoding time (compared to the reference configuration).

In addition, proponents are encouraged to provide the following (but not limited to) information for complexity and entropy throughput analysis:

- Number of context models

- Number of context-coded and bypass bins in average case and the worst case

- Table size of scanning paths

- Analysis of the frequency of context switches

- Analysis of the dependency between two consecutive bins

- Analysis of entropy parsing dependency (whether bits to bins conversion depends on complex reconstruction and/or pixel processing process)

- Analysis of entropy de-binarization dependency (whether bins to symbols conversion depends on complex reconstruction and/or pixel processing process)

## Test plan

The core experiment is split into the following tests:

Transform coefficient coding

|  | Tester | Tool | Cross checker |
| --- | --- | --- | --- |
| 7.1 | Tzu-Der Chuang (MediaTek) | [1] Constraints for number of context-coded bins depend on whether luma or chroma is coded (and on the subblock size) |  |
| 7.2 | Tzu-Der Chuang (MediaTek) | [1] Constraints for number of context-coded bins depend on whether luma or chroma is coded (and on the subblock size) + [2] Constraints for number of context-coded bins depend on last significant scan position | Jung-Ah Choi (LGE) |
| 7.3 | Tzu-Der Chuang (MediaTek) | [1] Constraints for number of context-coded bins depend on whether luma or chroma is coded (and on the subblock size) + [2] Constraints for number of context-coded bins depend on last significant scan position + [3] Include rem_abs_gt2_flag into first coding pass | H. Schwarz (HHI) |
| 7.4 | Tzu-Der Chuang (MediaTek) | [3] Include rem_abs_gt2_flag into first coding pass | H. Schwarz (HHI) |

# Timeline

General timeline for core experiments (as agreed by JVET).

| T1 | 3 weeks after meeting | - Final CE description uploaded |
| --- | --- | --- |
| T2 | 2 weeks after VTM-3 software release | - Proponents completed initial integration of tools (in individual branch) - Cross checkers can start initial study of tools / software (Note: Proponents can continue to modify the software until T3) |
| T3 | 3 weeks before next meeting | - Proponents froze changes to software branches - Proponents tag software and announce on JVET reflector - The names of the cross checkers and the list of specific tests for each CE tool are announced on JVET reflector - Proponents provide full test results - Cross checkers start final cross checks |

Test results and cross check results are reported to CE coordinators (for inclusion into CE report).

# Detailed Description of Test

## Reduce number of context-coded bins in coefficient coding [JVET-L0145]

In JVET-L0145 [1], three methods to constrain the usage of context-coded bins are proposed. The first constraint is dependent on color component and coefficient sub-block size. The second method is to release the constraints values according to the last significant sub-block position to improve the coding efficiency. The third modification is to move the greater than 2 flag into the first pass to further improve the decoding throughput. The detail is described as follow.

[1] Different constraints according to color components and sub-block sizes

The number of context-coded bins (including greater than 2 flag) constraints are set to 30 on 4x4 luma subblocks, 16 on 4x4 chroma subblocks, and 4 on 2x2 chroma subblocks, respectively. The number of context-coded bin constraints on greater than 2 are set to 4 on 4x4 luma subblocks, 2 on 4x4 chroma subblocks, and 1 on 2x2 chroma sub-blocks, respectively.

[2] Last significant subblock position dependent constraint

According to the last significant subblock position, the number of to-be-coded sub-blocks in a TB can be derived. The constraint value is determined according to the color component and the coefficient sub-block size. With the number of to-be-coded sub-blocks (NumToBeCodedSb) and the number of total coefficient sub-blocks in a TB (NumTotalSb), the constraint value (ConstraintValue) is modified as follows.

if (NumToBeCodedSb * 4 <= NumTotalSb)

ConstraintValue = ConstraintValue * 4;

else if (NumToBeCodedSb * 2 <= NumTotalSb)

ConstraintValue = ConstraintValue * 2;

else if (NumToBeCodedSb * 3 <= NumTotalSb * 2)

ConstraintValue = (ConstraintValue * 3) >> 1;

else if (NumToBeCodedSb * 5 <= NumTotalSb * 4)

ConstraintValue = (ConstraintValue * 5) >> 2;

[3] Include greater than 2 flag into the first coding pass

The greater than 2 flag is proposed to be moved to the first coding pass after the parity bit. The parsed greater than 2 flag is used to calculate the locSumAbsPass1in the context modeling of sig_coeff_flag, par_level_flag, rem_abs_gt1_flag, and rem_abs_gt2_flag. The reserved number of context-coded bins for greater than 2 flag in the second coding pass is also merged into the number of context-coded bins for the first coding pass. The test that moving the greater than 2 flag before parity bit will also be evaluated.

# References

- T.-D. Chuang, S.-T. Hsiang, Z.-Y. Lin, C.-Y. Chen, Y.-W. Huang, S.-M. Lei, “CE7-related: Constraints on context-coded bins for coefficient coding,” Joint Video Experts Team, doc. JVET-L0145, Oct. 2018.