| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M_Notes_d6 |
| --- | --- |

| Title: | Meeting Report of the 13th meeting of the Joint Video Experts Team (JVET), Marrakech, MA, 9–18 January 2019 |  |  |
| --- | --- | --- | --- |
| Status: | Report document from the chairs of JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Gary Sullivan Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 USA Jens-Rainer Ohm Institute of Communication Engineering RWTH Aachen Melatener Straße 23 D-52074 Aachen | Tel: Email: Tel: Email: | +1 425 703 5308 garysull@microsoft.com +49 241 80 27671 ohm@ient.rwth-aachen.de |
| Source: | Chairs of JVET |  |  |

_____________________________

# Summary

The Joint Video Experts Team (JVET) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its thirteenth meeting during 9–18 January 2019 at the Palais des Congrès Mogador (Tourist Zone Agdal, 40000, Marrakech, Morocco, Tel: + 212 530 530 530). The JVET meeting was held under the chairmanship of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany). For rapid access to particular topics in this report, a subject categorization is found (with hyperlinks) in section 2.13 of this document. It is further noted that the unabbreviated name of JVET was formerly known as “Joint Video Exploration Team”, but the parent bodies had modified it when entering the phase of formal development of a new standard by the previous meeting. The name Versatile Video Coding (VVC) was chosen as the informal nickname for the new standard.

The JVET meeting began at approximately 0910 hours on Wednesday 9 January 2019. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately XXXX hours on Friday 18 October 2019. Approximately XXX people attended the JVET meeting, and approximately XXX input documents and 17 AHG reports were discussed. The meeting took place in a collocated fashion with a meeting of WG11 – one of the two parent bodies of the JVET. The subject matter of the JVET meeting activities consisted of developing video coding technology with a compression capability that significantly exceeds that of the current HEVC standard, or otherwise gives better support regarding the requirements of future application domains of video coding. As a primary goal, the JVET meeting reviewed the work that was performed in the interim period since the twelfth JVET meeting in producing a third draft of the VVC standard and the third version of the associated VVC test model (VTM). Further important goals were reviewing the results of 13 Core Experiments (CE), reviewing other technical input on novel aspects of video coding technology, and producing the next versions of the VVC draft text and VTM, and plan next steps for further investigation of candidate technology towards the formal standard development.

The JVET produced XX output documents from the meeting (update):

- JVET-L1001 Versatile Video Coding specification text (Draft 3)

- JVET-L1002 Algorithm description for Versatile Video Coding and Test Model 3 (VTM 3)

- JVET-L1004 Algorithm descriptions of projection format conversion and video quality metrics in 360Lib (Version 8)

- JVET-L1005 and JVET-L1006 Methodology and reporting template for coding tool testing and for neural network tool testing

- JVET-L1010, JVET-L1011, and JVET-L1012 JVET common test conditions and software reference configurations for SDR, HDR/WCG, and 360° video

- JVET-L1021 through JVET-L1033, Description of Core Experiments 1 through 13

For the organization and planning of its future work, the JVET established XX “ad hoc groups” (AHGs) to progress the work on particular subject areas. At this meeting, XX Core Experiments (CE) were defined. The next four JVET meetings were planned for 19–27 March 2019 under ITU-T SG16 auspices in Geneva, CH, during 3–12 July 2019 under WG 11 auspices in Gothenburg, SE, during 1–9 October 2019 under ITU-T SG16 auspices in Geneva, CH, and during 8–17 January 2020 under WG 11 auspices in Brussels, BE.

The document distribution site http://phenix.it-sudparis.eu/jvet/ was used for distribution of all documents.

The reflector to be used for discussions by the JVET and all its AHGs is the JVET reflector:
jvet@lists.rwth-aachen.de hosted at RWTH Aachen University. For subscription to this list, see
https://mailman.rwth-aachen.de/mailman/listinfo/jvet.

# Administrative topics

## Organization

The ITU-T/ISO/IEC Joint Video Experts Team (JVET) is a group of video coding experts from the ITU-T Study Group 16 Visual Coding Experts Group (VCEG) and the ISO/IEC JTC 1/SC 29/WG 11 Moving Picture Experts Group (MPEG). The parent bodies of the JVET are ITU-T WP3/16 and ISO/IEC JTC 1/SC 29/WG 11.

The Joint Video Experts Team (JVET) of ITU-T WP3/16 and ISO/IEC JTC 1/SC 29/WG 11 held its thirteenth meeting during 9–18 January 2019 at the Palais des Congrès Mogador (Tourist Zone Agdal, 40000, Marrakech, Morocco, Tel: + 212 530 530 530). The JVET meeting was held under the chairmanship of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany).

It is further noted that the unabbreviated name of JVET was formerly known as “Joint Video Exploration Team”, but the parent bodies had modified it when entering the phase of formal development of a new standard by the previous meeting. The name Versatile Video Coding (VVC) was chosen as the informal nickname for the new standard.

## Meeting logistics

Information regarding logistics arrangements for the meeting had been provided via the email reflector jvet@lists.rwth-aachen.de and at http://wftp3.itu.int/av-arch/jvet-site/2019_01_M_Marrakech/.

## Primary goals

As a primary goal, the JVET meeting reviewed the work that was performed in the interim period since the twelfth JVET meeting in producing a third draft of the VVC standard and the third version of the associated VVC test model (VTM). Further important goals were reviewing the results of 13 Core Experiments (CE), reviewing other technical input on novel aspects of video coding technology, and producing the next versions of draft text and VTM, and plan next steps for further investigation of candidate technology towards the formal standard development.

## Documents and document handling considerations

### General

The documents of the JVET meeting are listed in Annex A of this report. The documents can be found at http://phenix.it-sudparis.eu/jvet/.

Registration timestamps, initial upload timestamps, and final upload timestamps are listed in Annex A of this report.

The document registration and upload times and dates listed in Annex A and in headings for documents in this report are in Paris/Geneva time. Dates mentioned for purposes of describing events at the meeting (other than as contribution registration and upload times) follow the local time at the meeting facility.

Highlighting of recorded decisions in this report is practised as follows:

- Decisions made by the group that might affect the normative content of a future standard are identified in this report by prefixing the description of the decision with the string “Decision:”.

- Decisions that affect the VTM or BMS software but have no normative effect are marked by the string “Decision (SW):”.

- Decisions that fix a “bug” in the VTM description (an error, oversight, or messiness) or in the software are marked by the string “Decision (BF):”.

This meeting report is based primarily on notes taken by the JVET chairs. The preliminary notes were also circulated publicly by ftp and http during the meeting on a daily basis. It should be understood by the reader that 1) some notes may appear in abbreviated form, 2) summaries of the content of contributions are often based on abstracts provided by contributing proponents without an intent to imply endorsement of the views expressed therein, and 3) the depth of discussion of the content of the various contributions in this report is not uniform. Generally, the report is written to include as much information about the contributions and discussions as is feasible (in the interest of aiding study), although this approach may not result in the most polished output report.

### Late and incomplete document considerations

The formal deadline for registering and uploading non-administrative contributions had been announced as Wednesday, 2 January 2019. Any documents uploaded after 1159 hours Paris/Geneva time on Thursday 3 January were considered “officially late”, giving a grace period of 12 hours to accommodate those living in different time zones of the world. The deadline does not apply to AHG reports, CE summaries, and other such reports which can only be produced after the availability of other input documents.

All contribution documents with registration numbers JVET-M0535 and higher were registered after the “officially late” deadline (and therefore were also uploaded late). However, some documents in the “M0535+” range might include break-out activity reports that were generated during the meeting, and are therefore better considered as report documents rather than as late contributions. Also, many cross-check reports were uploaded late.

In many cases, contributions were also revised after the initial version was uploaded. The contribution document archive website retains publicly accessible prior versions in such cases. The timing of late document availability for contributions is generally noted in the section discussing each contribution in this report.

One suggestion to assist with the issue of late submissions was to require the submitters of late contributions and late revisions to describe the characteristics of the late or revised (or missing) material at the beginning of discussion of the contribution. This was agreed to be a helpful approach to be followed at the meeting.

The following technical design proposal contributions were registered and/or uploaded late:

- JVET-M0XXX (a proposal on …), uploaded 01-XX.

- ….

It may be observed that some of the above-listed contributions were submissions made in response to issues that arose in discussions during the meeting or from the study of other contributions, and thus could not have been submitted by the ordinary deadline. For example, some of them were proposing combinations or simplifications of other proposals.

The following other document not proposing normative technical content, but with some need for consideration, were registered and/or uploaded late:

- JVET-M0XXX (a document on …), uploaded 01-XX.

- ….

The following cross-verification reports were registered before the deadline and uploaded late: JVET-M0XXX [uploaded XX-XX], … . Cross-verification reports that were both registered late and uploaded late (those with numbers higher than JVET-L0XXX) are not specifically identified here, in the interest of brevity. Initial upload times for each document are recorded in Annex A of this report.

The following contribution registrations were later cancelled, withdrawn, never provided, were cross-checks of a withdrawn contribution, or were registered in error: JVET-M0XXX, ….

“Placeholder” contribution documents that were basically empty of content, or lacking any results proving benefit of the proposed technology, and obviously uploaded with an intent to provide a more complete submission as a revision, had been agreed to be considered unacceptable and to be rejected in the document management system until a more complete version was available (which would then typically be counted as a late contribution). At the current meeting, this situation applied to the initial uploads of documents JVET-M0245, JVET-M0325 (later withdrawn), JVET-M0351, JVET-M0396, JVET-M0424, JVET-M0526, and JVET-M0660.

Contributions that had significant problems with uploaded versions included the following:

- M0042 (improperly formatted filename)

- M0067 (proposal described as a report in header)

- M0080 (incorrect document number in a filename)

- M0084 (incomplete patent rights declaration)

- M0147 (no author or contact information in header)

- M0202 (incomplete patent rights declaration)

- M0204 (unreadable file uploaded)

- M0209 (unreadable file uploaded)

- M0244 (incorrect company identified in patent rights declaration)

- M0245 (proposal missing all clearly necessary experiment results)

- M0276 (incomplete patent rights declaration)

- M0325 (proposal missing all clearly necessary experiment results) – Later withdrawn

- M0336 (unreadable file uploaded in -v2)

- M0339 (unreadable file uploaded in -v2)

- M0351 (proposal missing all clearly necessary experiment results)

- M0352 (document number missing in header and most test results missing)

- M0374 (wrong meeting number in header)

- M0376 (wrong meeting start date in header)

- M0396 (proposal missing all clearly necessary experiment results)

- M0400 (wrong meeting start date and incorrect country abbreviation in header)

- M0416 (wrong meeting identified in header)

- M0424 (proposal missing all clearly necessary experiment results)

- M0481 (unreadable file uploaded)

- M0526 (proposal missing all clearly necessary experiment results)

- M0588 (copy of wrong contribution uploaded)

- M0594 (zero-byte zip file uploaded in -v2, another unreadable file uploaded later)

- M0621 (uploaded file with confusing change marks relative to an irrelevant document)

- M0692 (uploaded file with confusing change marks relative to an irrelevant document)

- M0657 (title referred to the wrong document)

- M0660 (cross-check document with only a tiny fraction of the necessary experiment results)

- M0802, M0803, M0804, M0805, M0806, M0819, M0839 (document number missing in header)

As a general policy, missing documents were not to be presented, and late documents (and substantial revisions) could only be presented when there was a consensus to consider them and there was sufficient time available for their review. Again, an exception is applied for AHG reports, CE summaries, and other such reports which can only be produced after the availability of other input documents. There were no objections raised by the group regarding presentation of late contributions, although there was some expression of annoyance and remarks on the difficulty of dealing with late contributions and late revisions.

It was remarked that documents that are substantially revised after the initial upload can also be a problem, as this becomes confusing, interferes with study, and puts an extra burden on synchronization of the discussion. This can especially be a problem in cases where the initial upload is clearly incomplete, and in cases where it is difficult to figure out what parts were changed in a revision. For document contributions, revision marking is very helpful to indicate what has been changed. Also, the “comments” field on the web site can be used to indicate what is different in a revision although participants tend to seldom notice what is recorded there.

A few contributions may have had some problems relating to IPR declarations in the initial uploaded versions (missing declarations, declarations saying they were from the wrong companies, etc.). These issues were corrected by later uploaded versions in a reasonably timely fashion in all cases (to the extent of the awareness of the responsible coordinators).

Some other errors were noticed in other initial document uploads (wrong document numbers or meeting dates or meeting locations in headers, etc.) which were generally sorted out in a reasonably timely fashion. The document web site contains an archive of each upload.

### Outputs of the preceding meeting

All output documents of the previous meeting, particularly the meeting report JVET-L1000, the Versatile Video Coding specification text (Draft 3) JVET-L1001, the Algorithm description for Versatile Video Coding and Test Model 3 (VTM 3) JVET-L1002, the Algorithm descriptions of projection format conversion and video quality metrics in 360Lib Version 8 JVET-L1004, the Methodology and reporting template for tool testing JVET-L1005, the JVET common test conditions and software reference configurations for SDR, HDR/WCG, and 360° video (JVET-L1010, JVET-L1011, and JVET-L1012), and the Description of Core Experiments 1 through 13 (JVET-L1021 through JVET-L1033), had been completed and were approved. The software implementation of VTM (versions 3.0 and 3.1), and the 360Lib software implementation (version 8.0) were also approved.

The group had initially been asked to review the meeting report of the previous meeting for finalization. The meeting report was later approved … .

The available output documents of the previous meeting and the software had been made available in a reasonably timely fashion.

## Attendance

The list of participants in the JVET meeting can be found in Annex B of this report.

The meeting was open to those qualified to participate either in ITU-T WP3/16 or ISO/IEC JTC 1/‌SC 29/‌WG 11 (including experts who had been personally invited as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the responsible coordinators.

## Agenda

The agenda for the meeting was as follows:

- Opening remarks and review of meeting logistics and communication practices

- IPR policy reminder and declarations

- Contribution document allocation

- Review of results of the previous meeting

- Reports of ad hoc group (AHG) activities

- Reports of core experiments planned at the previous meeting

- Consideration of contributions and communications on project guidance

- Consideration of additional video coding technology contributions

- Consideration of information contributions

- Coordination activities

- Approval of output documents and associated editing periods

- Future planning: Determination of next steps, discussion of working methods, communication practices, establishment of coordinated experiments, establishment of AHGs, meeting planning, other planning issues

- Other business as appropriate for consideration

## IPR policy reminder

Participants were reminded of the IPR policy established by the parent organizations of the JVET and were referred to the parent body websites for further information. The IPR policy was summarized for the participants.

The ITU-T/ITU-R/ISO/IEC common patent policy shall apply. Participants were particularly reminded that contributions proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.

This obligation is supplemental to, and does not replace, any existing obligations of parties to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC.

Participants were also reminded of the need to formally report patent rights to the top-level parent bodies (using the common reporting form found on the database listed below) and to make verbal and/or document IPR reports within the JVET necessary in the event that they are aware of unreported patents that are essential to implementation of a standard or of a draft standard under development.

Some relevant links for organizational and IPR policy information are provided below:

- http://www.itu.int/ITU-T/ipr/index.html (common patent policy for ITU-T, ITU-R, ISO, and IEC, and guidelines and forms for formal reporting to the parent bodies)

- http://ftp3.itu.int/av-arch/jvet-site (JVET contribution templates)

- http://www.itu.int/ITU-T/dbase/patent/index.html (ITU-T IPR database)

- http://www.itscj.ipsj.or.jp/sc29/29w7proc.htm (JTC 1/‌SC 29 Procedures)

It is noted that the ITU TSB director’s AHG on IPR had issued a clarification of the IPR reporting process for ITU-T standards, as follows, per SG 16 TD 327 (GEN/16):

“TSB has reported to the TSB Director’s IPR Ad Hoc Group that they are receiving Patent Statement and Licensing Declaration forms regarding technology submitted in Contributions that may not yet be incorporated in a draft new or revised Recommendation. The IPR Ad Hoc Group observes that, while disclosure of patent information is strongly encouraged as early as possible, the premature submission of Patent Statement and Licensing Declaration forms is not an appropriate tool for such purpose.

In cases where a contributor wishes to disclose patents related to technology in Contributions, this can be done in the Contributions themselves, or informed verbally or otherwise in written form to the technical group (e.g. a Rapporteur’s group), disclosure which should then be duly noted in the meeting report for future reference and record keeping.

It should be noted that the TSB may not be able to meaningfully classify Patent Statement and Licensing Declaration forms for technology in Contributions, since sometimes there are no means to identify the exact work item to which the disclosure applies, or there is no way to ascertain whether the proposal in a Contribution would be adopted into a draft Recommendation.

Therefore, patent holders should submit the Patent Statement and Licensing Declaration form at the time the patent holder believes that the patent is essential to the implementation of a draft or approved Recommendation.”

The responsible coordinators invited participants to make any necessary verbal reports of previously-unreported IPR in technology that might be considered as prospective candidate for inclusion in future standards, and opened the floor for such reports: No such verbal reports were made.

## Software copyright disclaimer header reminder

It was noted that the VTM software implementation package uses the same software copyright license header as the HEVC reference software, where the latter had been agreed at the 5th meeting of the JCT-VC and approved by both parent bodies at their collocated meetings at that time. This license header language is based on the BSD license with a preceding sentence declaring that other contributor or third party rights, including patent rights, are not granted by the license, as recorded in N10791 of the 89th meeting of ISO/IEC JTC 1/‌SC 29/‌WG 11. Both ITU and ISO/IEC will be identified in the <OWNER> and <ORGANIZATION> tags in the header. This software is used in the process of designing the VTM software, and for evaluating proposals for technology to be potentially included in the design. This software or parts thereof might be published by ITU-T and ISO/IEC as an example implementation of a future video coding standard and for use as the basis of products to promote adoption of such technology.

Different copyright statements shall not be committed to the committee software repository (in the absence of subsequent review and approval of any such actions). As noted previously, it must be further understood that any initially-adopted such copyright header statement language could further change in response to new information and guidance on the subject in the future.

These considerations apply to the 360Lib video conversion software and and HDRtools as well.

## Communication practices

The documents for the meeting can be found at http://phenix.it-sudparis.eu/jvet/.

It was reminded to send a notice to the chairs in cases of changes to document titles, authors etc.

JVET email lists are managed through the site https://mailman.rwth-aachen.de/mailman/options/jvet, and to send email to the reflector, the email address is jvet@lists.rwth-aachen.de. Only members of the reflector can send email to the list. However, membership of the reflector is not limited to qualified JVET participants.

It was emphasized that reflector subscriptions and email sent to the reflector must use real names when subscribing and sending messages and subscribers must respond to inquiries regarding the nature of their interest in the work. The current number of subscribers was 928.

For distribution of test sequences, a password-protected ftp site had been set up at RWTH Aachen University, with a mirror site at FhG-HHI. Accredited members of JVET may contact the responsible JVET coordinators to obtain the password information (but the site is not open for use by others).

## Terminology

Some terminology used in this report is explained below:

- ACT: Adaptive colour transform.

- AI: All-intra.

- AIF: Adaptive interpolation filtering.

- ALF: Adaptive loop filter.

- AMP: Asymmetric motion partitioning – a motion prediction partitioning for which the sub-regions of a region are not equal in size (in HEVC, being N/2x2N and 3N/2x2N or 2NxN/2 and 2Nx3N/2 with 2N equal to 16 or 32 for the luma component).

- AMVP: Adaptive motion vector prediction.

- AMT or MTS: Adaptive multi-core transform, or multiple transform set.

- AMVR: (Locally) adaptive motion vector resolution.

- APS: Active parameter sets.

- ARC: Adaptive resolution conversion (synonymous with DRC, and a form of RPR).

- ARSS: Adaptive reference sample smoothing.

- ATMVP or “subblock-based temporal merging candidates” : Alternative temporal motion vector prediction.

- AU: Access unit.

- AUD: Access unit delimiter.

- AVC: Advanced video coding – the video coding standard formally published as ITU-T Recommendation H.264 and ISO/IEC 14496-10.

- BA: Block adaptive.

- BC: See CPR or IBC.

- BD: Bjøntegaard-delta – a method for measuring percentage bit rate savings at equal PSNR or decibels of PSNR benefit at equal bit rate (e.g., as described in document VCEG-M33 of April 2001).

- BDOF: Bi-directional optical flow (formerly known as BIO)

- BL: Base layer.

- BMS: Bench-mark set, a compilation of coding tools on top of VTM, which provide somewhat better compression performance, but are not deemed mature for standardzation.

- BoG: Break-out group.

- BR: Bit rate.

- BV: Block vector (used for intra BC prediction).

- CABAC: Context-adaptive binary arithmetic coding.

- CBF: Coded block flag(s).

- CC: May refer to context-coded, common (test) conditions, or cross-component.

- CCLM: Cross-component linear model.

- CCP: Cross-component prediction.

- CE: Core Experiment – a coordinated experiment conducted toward assessment of coding technology.

- CG: Coefficient group.

- CGS: Colour gamut scalability (historically, coarse-grained scalability).

- CIIP: Combined Inter/Intra prediction

- CL-RAS: Cross-layer random-access skip.

- CPMV: Control-point motion vector

- CPMVP: Control-point motion vector prediction (used in affine motion model).

- CPR: Current-picture referencing, also known as IBC – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction.

- CTC: Common test conditions.

- CVS: Coded video sequence.

- DCT: Discrete cosine transform (sometimes used loosely to refer to other transforms with conceptually similar characteristics).

- DCTIF: DCT-derived interpolation filter.

- DF: Deblocking filter.

- DMVR: Decoder-side motion vector refinement.

- DRC: Dynamic resolution conversion (synonymous with ARC, and a form of RPR).

- DT: Decoding time.

- ECS: Entropy coding synchronization (typically synonymous with WPP).

- EMT: Explicit multiple-core transform.

- EOTF: Electro-optical transfer function – a function that converts a representation value to a quantity of output light (e.g., light emitted by a display.

- EPB: Emulation prevention byte (as in the emulation_prevention_byte syntax element).

- ECV: Extended Colour Volume (up to WCG).

- EL: Enhancement layer.

- ET: Encoding time.

- FRUC: Frame rate up conversion (pattern matched motion vector derivation).

- HDR: High dynamic range.

- HEVC: High Efficiency Video Coding – the video coding standard developed and extended by the JCT-VC, formalized by ITU-T as Rec. ITU-T H.265 and by ISO/IEC as ISO/IEC 23008-2.

- HLS: High-level syntax.

- HM: HEVC Test Model – a video coding design containing selected coding tools that constitutes our draft standard design – now also used especially in reference to the (non-normative) encoder algorithms (see WD and TM).

- HMVP: History based motion vector prediction.

- HyGT: Hyper-cube Givens transform (a type of NSST).

- IBC (also Intra BC): Intra block copy, also known as CPR – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction.

- IBDI: Internal bit-depth increase – a technique by which lower bit-depth (8 bits per sample) source video is encoded using higher bit-depth signal processing, ordinarily including higher bit-depth reference picture storage (ordinarily 12 bits per sample).

- IBF: Intra boundary filtering.

- ILP: Inter-layer prediction (in scalable coding).

- IPCM: Intra pulse-code modulation (similar in spirit to IPCM in AVC and HEVC).

- JEM: Joint exploration model – the software codebase for future video coding exploration.

- JM: Joint model – the primary software codebase that has been developed for the AVC standard.

- JSVM: Joint scalable video model – another software codebase that has been developed for the AVC standard, which includes support for scalable video coding extensions.

- KLT: Karhunen-Loève transform.

- LB or LDB: Low-delay B – the variant of the LD conditions that uses B pictures.

- LD: Low delay – one of two sets of coding conditions designed to enable interactive real-time communication, with less emphasis on ease of random access (contrast with RA). Typically refers to LB, although also applies to LP.

- LIC: Local illumination compensation.

- LM: Linear model.

- LP or LDP: Low-delay P – the variant of the LD conditions that uses P frames.

- LUT: Look-up table.

- LTRP: Long-term reference pictures.

- MC: Motion compensation.

- MCP: Motion compensated prediction.

- MDNSST: Mode dependent non-separable secondary transform.

- MMLM: Multi-model (cross component) linear mode.

- MPEG: Moving picture experts group (WG 11, the parent body working group in ISO/IEC JTC 1/‌SC 29, one of the two parent bodies of the JVET).

- MPM: Most probable mode (in intra prediction).

- MV: Motion vector.

- MVD: Motion vector difference.

- NAL: Network abstraction layer (as in AVC and HEVC).

- NSQT: Non-square quadtree.

- NSST: Non-separable secondary transform.

- NUH: NAL unit header.

- NUT: NAL unit type (as in AVC and HEVC).

- OBMC: Overlapped block motion compensation (e.g., as in H.263 Annex F).

- OETF: Opto-electronic transfer function – a function that converts to input light (e.g., light input to a camera) to a representation value.

- OOTF: Optical-to-optical transfer function – a function that converts input light (e.g. l,ight input to a camera) to output light (e.g., light emitted by a display).

- PDPC: Position dependent (intra) prediction combination.

- PMMVD: Pattern-matched motion vector derivation.

- POC: Picture order count.

- PoR: Plan of record.

- PPS: Picture parameter set (as in AVC and HEVC).

- QM: Quantization matrix (as in AVC and HEVC).

- QP: Quantization parameter (as in AVC and HEVC, sometimes confused with quantization step size).

- QT: Quadtree.

- BT: Binary tree.

- TT: Ternary tree.

- RA: Random access – a set of coding conditions designed to enable relatively-frequent random access points in the coded video data, with less emphasis on minimization of delay (contrast with LD).

- RADL: Random-access decodable leading.

- RASL: Random-access skipped leading.

- R-D: Rate-distortion.

- RDO: Rate-distortion optimization.

- RDOQ: Rate-distortion optimized quantization.

- ROT: Rotation operation for low-frequency transform coefficients.

- RPLM: Reference picture list modification.

- RPR: Reference picture resampling (e.g., as in H.263 Annex P), a special case of which is also known as ARC or DRC.

- RPS: Reference picture set.

- RQT: Residual quadtree.

- RRU: Reduced-resolution update (e.g. as in H.263 Annex Q).

- RVM: Rate variation measure.

- SAO: Sample-adaptive offset.

- SbTMVP: Subblock based temporal motion vector prediction

- SD: Slice data; alternatively, standard-definition.

- SDT: Signal dependent transform.

- SEI: Supplemental enhancement information (as in AVC and HEVC).

- SH: Slice header.

- SHM: Scalable HM.

- SHVC: Scalable high efficiency video coding.

- SIMD: Single instruction, multiple data.

- SPS: Sequence parameter set (as in AVC and HEVC).

- STMVP: Spatial-temporal motion vector prediction.

- TBA/TBD/TBP: To be announced/determined/presented.

- TGM: Text and graphics with motion – a category of content that primarily contains rendered text and graphics with motion, mixed with a relatively small amount of camera-captured content.

- UCBDS: Unrestricted center-biased diamond search.

- UWP: Unequal weight prediction.

- VCEG: Visual coding experts group (ITU-T Q.6/16, the relevant rapporteur group in ITU-T WP3/16, which is one of the two parent bodies of the JVET).

- VPS: Video parameter set – a parameter set that describes the overall characteristics of a coded video sequence – conceptually sitting above the SPS in the syntax hierarchy.

- VTM: VVC Test Model.

- VVC: Versatile Video Coding, the standardization project developed by JVET.

- WCG: Wide colour gamut.

- WG: Working group, a group of technical experts (usually used to refer to WG 11, a.k.a. MPEG).

- WPP: Wavefront parallel processing (usually synonymous with ECS).

- Block and unit names in HEVC:

  - CTB: Coding tree block (luma or chroma) – unless the format is monochrome, there are three CTBs per CTU.

  - CTU: Coding tree unit (containing both luma and chroma, synonymous with LCU), with a size of 16x16, 32x32, or 64x64 for the luma component.

  - CB: Coding block (luma or chroma), a luma or chroma block in a CU.

  - CU: Coding unit (containing both luma and chroma), the level at which the prediction mode, such as intra versus inter, is determined in HEVC, with a size of 2Nx2N for 2N equal to 8, 16, 32, or 64 for luma.

  - PB: Prediction block (luma or chroma), a luma or chroma block of a PU, the level at which the prediction information is conveyed or the level at which the prediction process is performed in HEVC.

  - PU: Prediction unit (containing both luma and chroma), the level of the prediction control syntax within a CU, with eight shape possibilities in HEVC:

    - 2Nx2N: Having the full width and height of the CU.

    - 2NxN (or Nx2N): Having two areas that each have the full width and half the height of the CU (or having two areas that each have half the width and the full height of the CU).

    - NxN: Having four areas that each have half the width and half the height of the CU, with N equal to 4, 8, 16, or 32 for intra-predicted luma and N equal to 8, 16, or 32 for inter-predicted luma – a case only used when 2N×2N is the minimum CU size.

    - N/2x2N paired with 3N/2x2N or 2NxN/2 paired with 2Nx3N/2: Having two areas that are different in size – cases referred to as AMP, with 2N equal to 16 or 32 for the luma component.

  - TB: Transform block (luma or chroma), a luma or chroma block of a TU, with a size of 4x4, 8x8, 16x16, or 32x32.

  - TU: Transform unit (containing both luma and chroma), the level of the residual transform (or transform skip or palette coding) segmentation within a CU (which, when using inter prediction in HEVC, may sometimes span across multiple PU regions).

- Block and unit names in JEM (Note: Need to put VVC terminology here):

  - CTB: Coding tree block (luma or chroma) – there are three CTBs per CTU in P/B slice, and one CTB per luma CTU and two CTBs per chroma CTU in I slice.

  - CTU: Coding tree unit (synonymous with LCU, containing both luma and chroma in P/B slice, containing only luma or chroma in I slice), with a size of 16x16, 32x32, 64x64, or 128x128 for the luma component.

  - CB: Coding block, a luma or chroma block in a CU.

  - CU: Coding unit (containing both luma and chroma in P/B slice, containing only luma or chroma in I slice), a leaf node of a QTBT. It’s the level at which the prediction process and residual transform are performed in JEM. A CU can be square or rectangle shape.

  - PB: Prediction block, a luma or chroma block of a PU.

  - PU: Prediction unit, has the same size to a CU.

  - TB: Transform block, a luma or chroma block of a TU.

  - TU: Transform unit, has the same size to a CU.

## Opening remarks

Remarks during the opening session of the meeting 0900 Wednesday 9 January (chaired by GJS and JRO) were as follows.

- The meeting logistics, agenda, working practices, policies, and document allocation were reviewed.

- The results of the previous meeting were reviewed.

  - On placeholders – there were a number of cases where there was some description of a concept but no test results.

    - …

- The primary goals of the meeting were to review the results of CEs, identify promising technology directions, and adopt proposed technology into the VVC draft text and VTM.

- Due to the high number of input contributions, parallelization and breakout work were expected to be needed.

- Principles of standards development were discussed.

## Scheduling of discussions

Scheduling: Generally meeting time was scheduled during 0900–2100+ hours, with coffee and lunch breaks as convenient. Ongoing scheduling refinements were announced on the group email reflector as needed. Some particular scheduling notes are shown below, although not necessarily 100% accurate or complete:

- Wed. 9 January, 1st day

  - 0900–1100, 1130-1400 Opening plenary (chaired by GJS & JRO)

  - 1530–1700 Track A CE1: Partitioning [GJS]

  - 1750–2030 Track A CE3.1: Intra prediction and mode coding [GJS]

  - 1530–2100 Track B CE2: Subblock motion compensation [JRO]

- Thu. 10 January, 2nd day

  - 0900–1330, 1500–1630 Track A CE3.2 & CE3.3 Intra prediction & mode coding [GJS]

  - 1700–2030 Track A CE5: Arithmetic coding engine [Y. Ye]

  - 2030–2200 Track A CE6: Transforms and transform signalling [GJS]

  - 0900–1100 Track B CE2: Subblock motion compensation [JRO]

  - 1120–1340, 1500-2000 Track B CE4: Inter prediction and motion vector coding [JRO]

  - 2030–2230 Track B CE9: Decoder-side motion vector derivation [JRO]

  - 0900–2100 BoG on tiles & wavefronts [Y.-K. Wang & M. M. Hannuksela in Chellah]

  - 2030–2300 BoG on CE2 [Y. He in Lixus]

- Fri. 11 January, 3rd day

  - 0900–1100, 1130–1400, 1530–2000 Track A CE6: Transforms and transform signalling [GJS]

  - 0900–1100 Track B CE11: Deblocking [JRO]

  - 1120–1330 and 1500–1615 Track B CE8: Screen content coding tools [JRO]

  - 1620–2010 Track B CE8 related screen content coding [JRO]

  - 0900–2100 BoG on high-level syntax [J. Boyce in Chellah]

  - 0900–1300, 1500–1900 BoG on CE3 and CE3-related intra prediction and mode coding [G. Van der Auwera in Ourica]

  - 1540–2020 BoG on CE4 related inter prediction and motion vector coding [K. Zhang]

  - 2045–2330 BoG on CE2 subblock motion compensation [Y. He in Lixus]

- Sat. 12 January, 4th day

  - 0900–1330 JCT-VC [outside of JVET in Chella]

  - 0900–1045, 1830–2300 BoGs sharing room in Olympia, then Colliseum

    - BoG on CE4 related inter prediction and motion vector coding [K. Zhang]

    - BoG on CE2 subblock motion compensation [Y. He]

  - 0900–1300, 1500–1800 BoG on CE3 and CE3-related intra prediction and mode coding [G. Van der Auwera in Colliseum in morning, Salon marocain in afternoon]

  - 1130–1345, 1515–1815 Track B on CE10: Combined and multi-hypothesis prediction [JRO]

  - 1300– BoG on CE9 related decoder-side motion vector derivation [S. Esenlik]

  - 1500–1600 Track A CE7: Quantization and coefficient coding [GJS]

  - 1600–1645, 1700-1800 Track A CE12 – Mapping functions [GJS]

  - 1600–1740 BoG on tiles & wavefronts [Y.-K. Wang & M. M. Hannuksela in Chellah]

  - 1800–1830 Track A CE12 related – Mapping functions [GJS]

  - 1815–2130 Track B CE8 related screen content coding [JRO]

  - 1800–2100 BoG on high-level syntax [J. Boyce in Chellah]

- Sun. 13 January, 5th day

  - 0900–1045, 1115–1400 Plenary

  - 1530–1645, 1700–1800 Track A high-level syntax BoG reporting

  - 1530–1800 Track B Loop filtering [JRO]

  - XXXX Track B Tools based on NN technology

  - 1530-XXXX BoG on CE13 and CE13 related 360° video coding [J. Boyce in Chella]

  - 1530-XXXX BoG on CE6 related transforms and transform signalling [X. Zhao in Luxor]

  - 1530-XXXX BoGs sharing room in Salon marocain

    - BoG on CE4 related inter prediction and motion vector coding [K. Zhang]

    - BoG on CE2 subblock motion compensation [Y. He]

  - 1800 BoG on CE10 related [C.-W. Hsu and M. Winken in Olympia]

- Mon. 14 January, 6th day

  - 0900–1400 MPEG parent-body plenary

  - 1300–XXXX Track A BoG reporting

    - 1300-1430 BoG on HLS

    - 1430-1630, 1920-2000 BoG on tiles and wavefronts

  - 2015–2240 Track A Use case studies and general prediction [GJS] (some TBP)

  - 1400?, 1930– BoG on CE10 related [C.-W. Hsu and M. Winken in Olympia, then Ourika]

  - 1600–1900 Track B

    - 1600-1715 6.14 Loop filtering tools

    - 1715–1815 6.11 CE11 related (deblocking)

    - 1815–1900 6.18 Tools based on NN technology

  - 1700–1830 VCEG plenary [outside of JVET in Chellah]

  - 1900 BoG on CE7 related Quantization and coefficient coding (13) [Y. Ye in Sabah]

  - 1845–XXXX BoG on CE13 and CE13 related 360° video coding [J. Boyce in Lodge 2, then Chella]

  - 1900–XXXX BoGs sharing room in Olympia

    - BoG on CE4 related inter prediction and motion vector coding [K. Zhang]

    - BoG on CE2 subblock motion compensation [Y. He]

  - 2230–XXXX BoG on CE6 related transforms and transform signalling [X. Zhao in Luxor]

- Tue. 15 January, 7th day

  - 0900 VCEG-MPEG joint meeting [incl. JVET systems coord, in Lixus]

  - 1000 Track B BoG reporting in Olympia [JRO]

  - 1300 Track A BoG reporting on intra prediction and mode coding (CE3 related)

  - XXXX Track A Use case studies and general prediction [GJS]

  - XXXX Track A Quantization control (8)

  - XXXX Track A CE5-related entropy coding (5 – only 1 needing presentation)

  - XXXX Track A Entropy coding (1)

  - XXXX Track A CE1 related Partitioning (4)

  - XXXX [to be reviewed Tues] Track A BoG on 360° video (CE13 & related)

  - XXXX BoG on tools based on NN technology (7) (section 6.18) (Y. Li)

- Wed. 16 January, 8th day

  - XXXX BoG on CE7 related Quantization and coefficient coding (13) [Y. Ye in Sabah]

## Contribution topic overview

The approximate subject categories and quantity of contributions per category for the meeting were summarized as follows (note that document count does not include crosschecks, and may not be up to date):

- AHG reports (17) (section 3) (Plenary)

- Project development (5) (section 4) (Plenary)

- Core Experiments (xx) (section 5) with subtopics

  - CE1: Partitioning (3) (section 5.1) (Track A)

  - CE2: Subblock motion compensation (24) (section 5.2) (Track B)

  - CE3: Intra prediction and mode coding (16) (section 5.3) (Track A)

  - CE4: Inter prediction and motion vector coding (18) (section 5.4) (Track B)

  - CE5: Arithmetic coding engine (10) (section 5.5) (Track A)

  - CE6: Transforms and transform signalling (21) (section 5.6) (Track A)

  - CE7: Quantization and coefficient coding (2) (section 5.7) (Track A)

  - CE8: Screen content coding tools (15) (section 5.8) (Track B)

  - CE9: Decoder side motion vector derivation (8) (section 5.9) (Track B)

  - CE10: Combined and multi-hypothesis prediction (15) (section 5.10) (Track B)

  - CE11: Deblocking (9) (section 5.11) (Track B)

  - CE12: Mapping functions (2) (section 5.12) (Track A)

  - CE13: Coding tools for 360° video (11) (section 5.13) (Track A)

- Non-CE technology proposals (xx) (section 6) with subtopics

  - CE1 related – Partitioning (3) (section 6.1) (Track A)

  - CE2 related – Subblock motion compensation (31) (section 6.2) (Track B & BoG Y. He)

  - CE3 related – Intra prediction and mode coding (50) (section 6.3) (Track A & BoG G. Van der Auwera)

  - CE4 related – Inter prediction and motion vector coding (49) (section 6.4) (Track B & BoG K. Zhang)

  - CE5 related – Arithmetic coding engine (4) (section 6.5) (Track A)

  - CE6 related – Transforms and transform signalling (25) (section 6.6) (Track A & BoG X. Zhao)

  - CE7 related – Quantization and coefficient coding (13) (section 6.7) (Track A & BoG Y. Ye)

  - CE8 related – Screen content coding tools (29) (section 6.8) (Track B)

  - CE9 related – Decoder side motion vector derivation (12) (section 6.9) (Track B & BoG S. Esenlik)

  - CE10 related – Combined and multi-hypothesis prediction (42) (section 6.10) (Track B & BoG C.-W. Hsu / M. Winken)

  - CE11 related – Deblocking (3) (section 6.11) (Track B)

  - CE12 related – Mapping functions (2) (section 6.12) (Track A)

  - CE13 related – Coding tools for 360° content (7) (section 6.13) (Track A & BoG J. Boyce)

  - Loop filtering tools (12) (section 6.14) (Track B)

  - General prediction aspects (5) (section 6.15) (Track A)

  - Quantization control (8) (section 6.16) (Track A & BoG?)

  - Entropy coding (2) (section 6.17) (Track A)

  - Tools based on NN technology (7) (section 6.18) (Track B & BoG Y. Li)

  - HL syntax (51) (section 6.19) (Track A & BoG Y.-K. Wang & M. M. Hannuksela on tiles & wavefronts & BoG J. Boyce)

- Complexity analysis and reduction (3) (section 7) (Track A)

- Encoder optimization (3) (section 8) (Track A)

- Metrics and evaluation criteria (0) (section 9) (Track none)

- Withdrawn (13) (section 10) (Track none)

- Joint meetings, plenary discussions, BoG reports, Summary of actions (section 11)

- Project planning (section 12)

- Establishment of AHGs (section 13)

- Output documents (section 14)

- Future meeting plans and concluding remarks (section 15)

Track A (238) was generally chaired by GJS, and Track B (272) by JRO.

As a general remark, it was established in Track B that “further study” means that technology should be studied in next CE on the subject area, whereas if such a remark is missing it implicitly means it should not be studied in CE. If further study in an AHG is expected, that would be explicitly expressed.

# AHG reports (17)

These reports were discussed Wednesday 9 January XXXX-XXXX (chaired by GJS and JRO).

JVET-M0001 JVET AHG report: Project management (AHG1) [J.-R. Ohm, G. J. Sullivan]

This document reports on the work of the JVET ad hoc group on Project Management, including an overall status report on the VVC standardization project and the progress made during the interim period since the preceding meeting.

At the 12th meeting of the ITU-T/ISO/IEC Joint Video Experts Team (JVET), an ad hoc group on Project Management was established with the following mandates:

- Coordinate overall JVET interim efforts.

- Supervise CE and AHG studies.

- Report on project status to JVET reflector.

- Provide a report to next meeting on project coordination status.

The reflector used for discussions by the JVET and all of its AHGs is the JVET reflector:
jvet@lists.rwth-aachen.de. For subscription to this list, see
http://mailman.rwth-aachen.de/mailman/listinfo/jvet.

In the interim period since the 12th JVET meeting, work towards finalizing the following (21) documents had been performed:

- JVET-L1001 Versatile Video Coding specification text (Draft 3)

- JVET-L1002 Algorithm description for Versatile Video Coding and Test Model 3 (VTM 3)

- JVET-L1004 Algorithm descriptions of projection format conversion and video quality metrics in 360Lib (Version 8)

- JVET-L1005 and JVET-L1006 Methodology and reporting template for coding tool testing and for neural network tool testing

- JVET-L1010, JVET-L1011, and JVET-L1012 JVET common test conditions and software reference configurations for SDR, HDR/WCG, and 360° video

- JVET-L1021 through JVET-L1033, Description of Core Experiments 1 through 13

The work of the JVET overall had proceeded well in the interim period, and many input documents had been submitted for consideration at the current meeting. Intense discussion had been carried out on the group email reflector, and all output documents from the preceding meeting had been produced.

As noted below, all output documents from the preceding meeting had been made available at the "Phenix" site (http://phenix.it-sudparis.eu/jvet/) or the ITU-based JCT-VC site (http://wftp3.itu.int/av-arch/jvet-site/2018_10_M_Macao/), although some of these documents may need further refinement:

- The meeting report (JVET-L1000) [Posted 2019-01-08]

- Versatile Video Coding (Draft 3) (JVET-L1001) [Posted 2018-10-31, last update 2019-01-08]

- Algorithm description for Versatile Video Coding and Test Model 3 (VTM 3) (JVET-L1002) [Posted 2018-12-03, last update 2018-12-24]

- Algorithm descriptions of projection format conversion and video quality metrics in 360Lib Version 8 (JVET-L1004) [Posted 2018-11-10]

- Methodology and reporting template for coding tool testing (JVET-L1005) [Posted 2018-10-27]

- Methodology and reporting template for neural network coding tool testing (JVET-L1006) [Posted 2018-10-25]

- JVET common test conditions and software reference configurations (JVET-L1010) [Posted 2018-11-16]

- JVET common test conditions and evaluation procedures for HDR/WCG video (JVET-L1011) [Posted 2018-11-20]

- JVET common test conditions and evaluation procedures for 360° video (JVET-L1012) [Posted 2018-11-19]

Description of CE 1..13 (JVET-L1021..35) [all first posted 2018-10-11 or 2018-10-12, further updated during the CE definition period of 3 weeks after the meeting, i.e., until 2018-11-02]. The following CE description documents had substantially later updates (more than 4 weeks after the meeting):

- JVET-L1022 [last updated 2019-01-08] (for which some experiment plans had changed and the change had been announced on the JVET reflector)

- JVET-L1023 [last updated 2018-12-27]

- JVET-L1024 [last updated 2019-01-08] (for which some experiment plans had changed but the change had not been announced on the JVET reflector)

- JVET-L1025 [last updated 2019-01-04]

- JVET-L1026 [last updated 2019-01-04]

- JVET-L1027 [last updated 2018-11-13]

- JVET-L1033 [last updated 2018-11-27]

The seventeen ad hoc groups had made progress, and reports from those activities had been submitted.

Software integration of the VTM was finalized approximately according to the plan.

Various problem reports relating to asserted bugs in the software, draft specification text, and reference encoder description had been submitted to an informal "bug tracking" system. That system is not intended as a replacement of our ordinary contribution submission process. However, the bug tracking system was considered to have been helpful to the software coordinators and text editors. The bug tracker reports had been automatically forwarded to the group email reflector, where the issues were discussed – and this is reported to have been helpful.

The software distribution had been migrated to GitLab before the previous meeting, and the method for obtaining access to the software had been changed after the last meeting – using shared accounts for read access by members (with one account for MPEG members using its usual credentials and another for VCEG members with access managed through the TIES system). The bug tracking system for software aspects was not integrated yet with GitLab for the time being.

Roughly 700 input contributions to the current meeting (not counting the AHG reports) had been registered for consideration at the meeting. Though the topics of Core Experiments and related documents for the development of low-level coding tools reflect the bulk of these documents, around 50 documents were submitted on aspects of high-level syntax, including tile partitioning.

A preliminary basis for the document subject allocation and meeting notes for the 13th meeting had been made publicly available on the ITU-hosted ftp site.

JVET-M0002 JVET AHG report: Draft text and test model algorithm description editing (AHG2) [B. Bross, J. Chen, J. Boyce, S. Kim, S. Liu, Y. Ye]

[incorporate from -v3]

Note three topics of resolving feature interactions and

three open tickets.

- No action needed on ticket #128.

- Software fix needed for #132.

- DCT2 downsampling of transform matrix should be separate horizontally and vertically (not assumed square) #135.

JVET-M0003 JVET AHG report: Test model software development (AHG3) [F. Bossen, X. Li, K. Sühring]

[Add content from report]

Ticket #105 on mode combinations allowed.

v3.1 to be released during meeting.

To add software account access information in a revision.

Remarks on CTC:

- CPR is not enabled (perhaps should enable for SCC test sequences)

- MTS is not enabled for inter (perhaps should enable for low-res test sequences)

JVET-M0004 JVET AHG report: Test material and visual assessment (AHG4) [T. Suzuki, V. Baroncini, R. Chernyak, P. Hanhart, A. Norkin, J. Ye]

The test sequences used for CfP/CTC are available on ftp://jvet@ftp.ient.rwth-aachen.de in directory “/jvet-cfp” (accredited members of JVET may contact the JVET chairs for login information).

Due to copyright restrictions, the JVET database of test sequences is only available to accredited members of JVET (i.e. members of ISO/IEC MPEG and ITU-T VCEG).

There was discussion that the current directory structure of test sequence ftp site is not good for the current activities. The ftp directory was created during the preparation of CfE/CfP and the same directory structure is still used. One possibility is to re-design the directory as follows, for example,

- ctc/ : Contains the active test set of the common testing conditions

- ahg/ : Contains subdirectories with sequences under consideration. The subfolder might be structured by meeting period (e.g. named by the doc-number of the corresponding meeting report?)

- ce/ : Contains subdirectories for data exchange for specific CE (already implemented, see ce/JVET-{K,L}1031_Deblocking

- upload : stays as before

During the last meeting, there was a comment that all sequences, all classes should be at the same place. But there is still meaningful to separate SDR, HDR, 360. Further detail should be discussed in the meeting.

The AHG recommended:

- To discuss further ftp directory structure

- To continue to collect new test sequences available for JVET with licensing statement

It was noted that some test sequences had been submitted to the previous meeting, and that there was especially some interest in improving the SCC test set. Side activity was encouraged to study that.

JVET-M0005 JVET AHG report: Memory bandwidth consumption of coding tools (AHG5) [R. Hashimoto, T. Ikai, X. Li, D. Luo, H. Yang, M. Zhou]

The document summarizes activities of AHG on memory bandwidth consumption of coding tools between the 12th and the 13th JVET meetings.

There is no related email discussion during this meeting cycle.

A patch on memory bandwidth measurement tools on VTM-3.0 has been provided by the AHG

https://jvet.hhi.fraunhofer.de/trac/vvc/ticket/121

Contributions to this meeting are as follows.

- CE2.4 (affine motion / block restriction) and related contributions

  - JVET-M0049 “CE2-related: A restriction on memory bandwidth consumption of affine mode”, M. Zhou (Broadcom)

  - JVET-M0150 “CE2.4.3: Affine restriction for worst-case memory bandwidth reduction”, L. Pham Van, W.-J. Chien, H. Huang, V. Seregin, M. Karczewicz (Qualcomm)

  - JVET-M0226 “CE2: Reducing worst-case memory bandwidth of affine mode (test 2.4.1)”, Y.-W. Chen, X. Wang (Kwai Inc.)

  - JVET-M0309 “CE2: Memory bandwidth reduction for affine mode (test 2.4.2)”, J. Li, R.-L. Liao, C. S. Lim (Panasonic)

  - JVET-M0311 “CE2-related: Memory bandwidth reduction for affine mode with less dependency”, J. Li, R.-L. Liao, C. S. Lim (Panasonic)

  - JVET-M0400, “CE2-related: Worst-case memory bandwidth reduction for VVC”, W.-J. Chien, L. Pham Van, H. Huang, V. Seregin, M. Karczewicz (Qualcomm)

  - JVET-M0472 “CE2: Affine sub-block size restrictions (Test 2.4.4)”, H. Chen, T. Solovyev, H. Yang, J. Chen (Huawei)

  - JVET-M0488 “CE2: Sub-block MV clip in affine prediction (test 2.4.5)”, M Gao, X Li, M Xu, S Liu(Tencent)

  - JVET-M0702 “CE2-related: Adaptive sub-block MV clip for affine blocks”, X. Li, M. Gao, S. Liu (Tencent)

- CE4.5 (block size restriction) and related contributions

  - JVET-M0313 “CE4: Motion compensation constraints for complexity reduction (test 4.5.1 and test 4.5.2)”, R.-L. Liao, J. Li, C. S. Lim (Panasonic)

  - JVET-M0348 “CE4-related: Further reducing VVC memory bandwidth worst case by combining 4x4/4x8/8x4 bi-prediction with AMVR”, X.W. Meng (Peking University), X. Zheng (DJI), S.S. Wang, S.W. Ma

- Others

  - JVET-M0357 “CE10-related: Reduction of the worst-case memory bandwidth and operation number of OBMC”, Y. Kidani, K. Kawamura, K. Unno, S. Naito (KDDI)

The AHG recommended to review the related contributions.

JVET-M0006 JVET AHG Report: 360 video conversion software development (AHG6) [Y. He, K. Choi]

The document summarizes activities on 360-degree video content conversion software development between the 12th (3 – 12 Oct. 2018) and the 13th (9 – 18 Jan. 2019) JVET meetings.

Brief summary for the activities:

The 360Lib-8.0 software package included following changes:

- Projection format conversion:

  - Chroma sample location type support (JVET-L0238).

- Configurations:

  - Added chroma sample location type for the output in the encoding parameter settings.

Software:

- Fixed the compilation error reported by ticket #118 to support GCC 8.2.1.

360Lib-8.0 related release:

- 360Lib-8.0rc1 with support of VTM-3.0rc1 was released on Nov. 16, 2018;

- 360Lib-8.0 with support of VTM-3.0 was released on Nov. 22, 2018;

The 360Lib software is developed using a Subversion repository located at:

https://jvet.hhi.fraunhofer.de/svn/svn_360Lib/

The released version of 360Lib-8.0 can be found at:

https://jvet.hhi.fraunhofer.de/svn/svn_360Lib/tags/360Lib-8.0/

360Lib-8.0 testing results can be found at:

ftp.ient.rwth-aachen.de/testresults/360Lib-8.0

360Lib bug tracker

https://hevc.hhi.fraunhofer.de/trac/jem/newticket?component=360Lib

The first table below is for the projection formats comparison using VTM-3.0 according to 360o video CTC (JVET-L1012). It compares padded hybrid equi-angular cubemap (PHEC) coding and padded equi-rectangular projection (PERP) coding using VTM-3.0.

Table 1. VTM-3.0 PHEC vs PERP (VTM-3.0 PERP as anchor)

|  | PHEC over PERP (VTM-3.0) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | End-to-end WS-PSNR |  |  | End-to-end S-PSNR-NN |  |  |
|  | Y | U | V | Y | U | V |
| Class S1 | -11.14% | -8.48% | -8.95% | -11.06% | -8.40% | -8.90% |
| Class S2 | -4.99% | -5.50% | -5.75% | -4.97% | -5.40% | -5.67% |
| Overall | -8.68% | -7.29% | -7.67% | -8.62% | -7.20% | -7.61% |

The second table below is for PERP comparison between these two codecs. Table 3 is to compare VTM-3.0 with PHEC coding and HM-16.16 with CMP coding.

Table 2. VTM-3.0 PERP vs HM-16.16 PERP (HM-16.16 PERP as anchor)

|  | VTM-3.0 PERP - Over HM-16.16 PERP |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | End-to-end WS-PSNR |  |  | End-to-end S-PSNR-NN |  |  |
|  | Y | U | V | Y | U | V |
| Class S1 | -21.25% | -40.15% | -40.88% | -21.24% | -40.15% | -40.82% |
| Class S2 | -29.62% | -44.01% | -44.94% | -29.61% | -44.03% | -44.97% |
| Overall | -24.60% | -41.69% | -42.50% | -24.59% | -41.70% | -42.48% |

The following table is to compare VTM-3.0 with PHEC coding and HM-16.16 with CMP coding.

Table 3. VTM-3.0 PHEC vs HM-16.16 CMP (HM-16.16 CMP as anchor)

|  | VTM-3.0 PHEC - Over HM-16.16 CMP |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | End-to-end WS-PSNR |  |  | End-to-end S-PSNR-NN |  |  |
|  | Y | U | V | Y | U | V |
| Class S1 | -25.82% | -41.35% | -41.88% | -25.71% | -41.32% | -41.84% |
| Class S2 | -32.70% | -47.14% | -48.10% | -32.69% | -47.13% | -48.11% |
| Overall | -28.57% | -43.66% | -44.37% | -28.50% | -43.65% | -44.35% |

The AHG recommended to continue software development of the 360Lib software package, to generate CTC VTM anchors according to the 360° video CTC, and to finalize the reporting template for the common test conditions.

JVET-M0007 JVET AHG report: Coding of HDR/WCG material (AHG7) [A. Segall, E. François, D. Rusanovskyy]

This document summarizes the activity of AHG7: Coding of HDR/WCG Material between the 12th meeting in Macao, CN (3–12 October 2018) and the 13th meeting in Marrakech, MA (9–18 January 2019).

The AHG used the main JVET reflector, jvet@lists.rwth-aachen.de, with an [AHG7] indication on message headers. The primary activity of the AhG was related to the mandates of arranging for a demonstration event for viewing JVET-L0205 and JVET-L0245 and maintaining test content. This work is described in the following subsection.

Demonstration Event

In the previous AHG study (as well as the previous JVET meeting), it was determined that it was unlikely that HDR viewing equipment would be present at the Marrakech meeting due to meeting logistics. In response to that, some HDR proponents proposed to have a demonstration event during the recent AHG period. The goal of the demonstration event was to facilitate the viewing of JVET-L-0205 and JVET-L0245.

During the AHG study period, several companies and locations were contacted to schedule such an event. However, all contacts reported scheduling issues due to year-end HDR movie title releases as well as industry preparation for the Consumer Electronics Show (CES).  Because of these scheduling issues, no event was held.

It was observed that the situation continues to enforce the importance of HDR content in current, consumer entertainment and consumer electronic industries.

Maintaining Test Content

During the recent meetings, it has been determined that there are some inconsistencies in how the copyright statements are managed within the HDR test sequences.  For example, some sequences contain the copyright statement as a text file, while other sequences include the copyright sequences in the last frame of the sequence.  During the AHG period, there was an effort to clean up and unify the copyright handling, with emphasis on the HLG content.  While this did not affect the content of any sequence, it did result in changing the md5sums for the sequences (in cases that had copyright frames).  These changes were also reflected in the CTCs.

There were 6 contributions identified as related to HDR video coding:

- JVET-M0032 CE12: Summary report on mapping functions [E. Francois, P. Yin]

- JVET-M0109 CE12-related: block-based in-loop reshaping [E. Francois, C. Chevance, F. Hiron (Technicolor)]

- JVET-M0427 CE12: Mapping functions (test CE12-1 and CE12-2) [T. Lu, F. Pu, P. Yin, W. Husak, S. McCarthy, T. Chen (Dolby)]

- JVET-M0580 Crosscheck of JVET-M0109 (CE12-related: block-based in-loop luma reshaping) [T. Lu, P. Yin (Dolby)]

- JVET-M0640 CE12-related: in-loop reshaping with approximate inverse mapping function [E. Francois (Technicolor)]

- JVET-M0703 Crosscheck of JVET-M0640 (CE12-related: in-loop luma reshaping with approximate inverse mapping function) [T. Lu, P. Yin]

The AHG recommended to review the related input contributions and discuss whether a demonstration and/or viewing event should be performed prior to (or at the) 14th JVET meeting.

JVET-M0008 JVET AHG report: 360° video coding tools and test conditions (AHG8) [J. Boyce, K. Choi, P. Hanhart, J.-L. Lin]

This document summarizes the activity of AHG8: 360º video coding tools and test conditions between the between the 12th meeting in Macao, CN (3–12 October 2018) and the 13th meeting in Marrakesh, MA (9–18 January 2019).

There was no AHG email activity on the main jvet reflector, jvet@lists.rwth-aachen.de, with an [AHG8] indication on message headers.

There were five non-CE related input documents identified (three contributions and one cross-check) related to 360º video coding, which are listed below. In addition, CE13 on projection formats is related to 360º video coding, and has ten input documents, which will be described in the CE report in JVET-M0033. There are four additional CE13-related input documents (three contributions and one cross-check) listed below.

360 video contributions not related to CE13

JVET-M0225

AHG8: On wrap around motion compensation [B. Choi, W. Feng, S. Liu (Tencent)

JVET-M0368

AHG8: 360Lib support for chroma sample location in PHEC blending process [C.-H. Shih, Y.-H. Lee, J.-L. Lin, Y.-C. Chang, C.-C. Ju (MediaTek)

JVET-M0452

AHG8: Hemisphere cubemap projection format [J. Boyce, M. Dmytrychenko (Intel)

Crosschecks of 360 video contributions not related to CE13

JVET-M0644

Crosscheck of JVET-M0368 (AHG8: 360Lib support for chroma sample location in PHEC blending process) [P. Hanhart (InterDigital)

CE13-related contributions

JVET-M0322

CE13-related: In-loop filters disabled across face discontinuities on PHEC with 2-pixel padding [Yule Sun, Xuchang Huangfu, Lu Yu (Zhejiang Univ.)

JVET-M0323

CE13-related: Adaptive QP to improve subjective quality for PHEC [Yule Sun, Xuchang Huangfu, Lu Yu (Zhejiang Univ.)

JVET-M0534

CE13-related: HEC with Pre-rotation + Adaptive Frame Packing(Test 4.2.a+4.1) [C. Pujara, A. Konda, A. Singh, R. Gadde, W. Choi, K. Choi, K.P. Choi(Samsung)

JVET-M0547

360° coding tools using uncoded areas [J. Sauer, M. Bläser

Crosschecks of 360 video contributions not related to CE13

JVET-M0645

Crosscheck of JVET-M0534 (CE13-related: HEC with Pre-rotation + Adaptive Frame Packing(Test 4.2.a+4.1)) [P. Hanhart (InterDigital)

Recommendations

The AHG recommends the following:

Review input contributions

Conduct informal subjective viewing of contributions

Review common test conditions for 360° video, including objective metrics and viewports

Review 360° video test material, and consider adding or replacing test sequences for common test conditions

JVET-M0009 JVET AHG report: Neural Networks in Video Coding (AHG9) [S. Liu, B. Choi, K. Kawamura, Y. Li, L. Wang, P. Wu, H. Yang]

[add notes]

JVET-M0010 JVET AHG report: Encoding algorithm optimizations (AHG10) [A.M. Tourapis, A. Duenas, C. Helmrich, S. Ikonin, A. Norkin, R. Sjöberg]

[add abstract]

JVET-M0011 JVET AHG report: Screen Content Coding (AHG11) [S. Liu, J. Boyce, A. Filippov, Y.-C. Sun, J. Xu, M. Zhou]

This document summarizes the activity of AHG11: Screen Content Coding between the 12th meeting in Macao, CN (3 – 12 Oct 2018) and the 13th meeting in Marrakech, MA (9–18 Jan. 2019).

The AHG used the main JVET reflector, jvet@lists.rwth-aachen.de, with [AHG11] in message headers. There were about a dozen emails exchanged on the JVET reflector with some discussions about testing sequences. There were also some email discussions about the interaction between CPR and inter coding tools adopted in Macao. Through the discussions, some mismatches between software VTM3 and spec VVC Draft 3 were identified. Some possible solutions are suggested in JVET-M0409. More in-depth technical discussions were carried in CE8 mailing list.

In total there are 26 CPR related technical contributions, 8 Palette related technical contributions and 7 other SCC related technical contributions identified for this meeting.

- CPR related:

  - JVET-M0151, CE8-related: Virtual search area for current picture referencing (CPR), L. Pham Van, T. Hsieh, W.-J. Chien, V. Seregin, H. Wang, M. Karczewicz (Qualcomm)

  - JVET-M0174, CE8-related: Removal of subblock-based chroma MC in CPR, C.-Y. Lai, T.-D. Chuang, Y.-L. Hsiao, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)

  - JVET-M0175, CE8-related: Clarification on interaction between CPR and other inter coding tools, C.-Y. Lai, T.-D. Chuang, Y.-L. Hsiao, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)

  - JVET-M0254, Non-CE8: Subblock Operation Removal for Chroma CPR, J. Xu, K. Zhang, L. Zhang, H. Liu, Y. Wang, P. Zhao, D. Hong (Bytedance)

  - JVET-M0325, CE8-related: CPR with previous CTU availability at frame and tile edge, C. Rosewarne, A. Dorrell (Canon) – Later withdrawn

  - JVET-M0326, CE8-related: Remove the redundancy of CPR-related syntax coding, S. Ye, F. Chen, L. Wang (Hikvision)

  - JVET-M0327, CE8-related: A new CPR syntax scheme, S. Ye, F. Chen, L. Wang (Hikvision)

  - JVET-M0332, CE8: Block vector prediction for CPR (test 8.1.1a and test 8.1.1b), J. Nam, J. Lim, S. Kim (LGE)

  - JVET-M0333, Non-CE8: Coding on block vector difference, J. Nam, J. Lim, S. Kim (LGE)

  - JVET-M0334, Non-CE8: Removal of redundant syntax between CPR and other inter coding tools, J. Nam, J. Lim, S. Kim (LGE)

  - JVET-M0335, Non-CE8: modification on SbTMVP process regarding with CPR, H.Jang, J.Nam, S.Kim, J.Lim(LGE)

  - JVET-M0336, Non-CE11: Considering boundary strength on CPR coded block boundary, H.Jang, J.Nam, S.Kim, J.Lim(LGE)

  - JVET-M0341, Non-CE8: MMVD harmonization with CPR, H.Jang, J.Nam, S.Kim, J.Lim(LGE)

  - JVET-M0393, Non-CE8: chroma block vector initialization for CPR in dual tree, T. Poirier, F. Le Léannec, F. Galpin (Technicolor)

  - JVET-M0402, Non-CE8: Proposed Cleanup for Current Picture Referencing, B. Heng, M. Zhou, W. Wan (Broadcom)

  - JVET-M0407, CE8: CPR reference memory reuse without increasing memory requirement (CE8.1.2a and CE8.1.2d), X. Xu, X. Li, S. Liu (Tencent), E. Chai (Ubilinx)

  - JVET-M0408, CE8: CPR reference memory reuse with reduced memory requirement (CE8.1.2b and CE8.1.2c), X. Xu, X. Li, S. Liu (Tencent), E. Chai (Ubilinx)

  - JVET-M0409, Non-CE8: Mismatch between text specification and reference software on ATMVP candidate derivation when CPR is enabled, X. Xu, X. Li, S. Liu (Tencent), W.-J. Chien, M. Karczewicz (Qualcomm)

  - JVET-M0410, Non-CE8: CPR flag signaling at slice level, X. Xu, X. Li, S. Liu (Tencent)

  - JVET-M0411, Non-CE8: Inter mode related flag signaling when current picture is the only reference picture, X. Xu, X. Li, S. Liu (Tencent)

  - JVET-M0418, CE8-related: Context modeling on pred_mode_flag when current picture is the only reference picture (CPR), Yu-Chen Sun, Jian Lou (Alibaba)

  - JVET-M0474, CE8.1.3: Extended CPR reference with 1 buffer line, L. Pham Van, V. Seregin, W.-J. Chien, T. Hsieh, M. Karczewicz (Qualcomm)

  - JVET-M0483, CE8-related: CPR mode signaling and interaction with inter coding tools, W.-J. Chien, V. Seregin, M. Karczewicz (Qualcomm)

  - JVET-M0541, Non-CE8: Combination of MMVD and CPR mode, Y. Li, Z. Chen (Wuhan Univ.), X. Xu, S. Liu (Tencent)

  - JVET-M0542, Non-CE8: Combination of Multi Hypothesis Intra and CPR mode, Y. Li, Z. Chen (Wuhan Univ.), X. Xu, S. Liu (Tencent)

  - JVET-M0544, Non-CE8: CPR with chroma 4x4 sub-block size when dual-tree is on, X. Xu, X. Li, S. Liu (Tencent)

- Palette related:

  - JVET-M0050, Palette Mode in HEVC (test 8.2.1), Yu-Chen Sun, Jian Lou (Alibaba), Yung-Hsuan Chao, Hongtao Wang, Vadim Seregin, Marta Karczewicz (Qualcomm)

  - JVET-M0051, CE8: Palette Mode and Intra Mode Combination (test 8.2.2), Yu-Chen Sun, Jian Lou (Alibaba)

  - JVET-M0052, CE8: Separate Palette Coding for Luma and Chroma (test 8.2.5), Yu-Chen Sun, Jian Lou (Alibaba), Yung-Hsuan Chao, Hongtao Wang, Vadim Seregin, Marta Karczewicz (Qualcomm), Roman Chernyak, Sergey Ikonin, Jianle Chen (Huawei)

  - JVET-M0417, CE8-related: Combination test of CE8.2.2 and CE8.2.5, Yu-Chen Sun, Jian Lou (Alibaba)

  - JVET-M0419, CE8-related: Context modeling on palette mode flag, Yu-Chen Sun, Jian Lou (Alibaba)

  - JVET-M0455, CE8: Palette index map scan order constraints (Test 8.2.3), J. Ye, X. Xu, M. Xu, X. Li, S. Liu (Tencent)

  - JVET-M0456, CE8: palette mode when dual-tree is enabled (Test 8.2.4), J. Ye, X. Xu, X. Li, S. Liu (Tencent)

  - JVET-M0457, CE8: Palette predictor list enhancement (Test 8.2.6), J. Ye, X. Xu, M. Xu, X. Li, S. Liu (Tencent)

- Other:

  - JVET-M0056, CE8: BDPCM with LOCO-I and independently decodable areas (test 8.3.1a), F. Henry, A. Mohsen (Orange), P. Philippe, G. Clare (bcom)

  - JVET-M0057, CE8: BDPCM with horizontal/vertical predictor and independently decodable areas (test 8.3.1b), F. Henry, M. Abdoli (Orange), P. Philippe, G. Clare (bcom)

  - JVET-M0058, CE8: BDPCM with modified binarization (test 8.3.2), F. Henry, M. Abdoli (Orange), G. Clare, P. Philippe (bcom)

  - JVET-M0253, Non-CE8: Hash-based Motion Search, J. Xu, J. Li, K. Zhang, L. Zhang (Bytedance), R. Xiong (Peking University)

  - JVET-M0365, Non-CE3: modified PDPC for horizontal and vertical modes, A. Filippov, V. Rufitskiy, J. Chen (Huawei)

  - JVET-M0449, CE8-related: BDPCM entropy coding with reduced number of context coded bins, M. Xu, X. Li, X. Xu, M. Gao, S. Liu (Tencent)

  - JVET-M0464, Non-CE8: Unified Transform Type Signalling and Residual Coding for Transform Skip, B. Bross, T. Nguyen, P. Keydel, H. Schwarz, D. Marpe, T. Wiegand (HHI)

The AHG recommended to review all related contributions, to continue investigating SCC coding tool performance, complexity and interactions between themselves and with other coding tools, and to continue evaluating new and modified test materials and consider including some of them in class F

See the notes for AHG3 relating to a desire to improve the SCC test sequences.

JVET-M0012 JVET AHG report: High-level parallelism and coded picture regions (AHG12) [T. Ikai, M. M. Hannuksela, R. Sjöberg, R. Skupin, W. Wan, Y.-K. Wang, S. Wenger]

The document summarizes activities of AHG on High-level parallelism and coded picture regions between the 12th and the 13th JVET meetings.

Related contributions to this meeting were categorized as follows:

- Flexible size tiles

  - JVET-M0066 AHG12: Flexible Tile Partitioning, Y. Yasugi, T. Ikai (Sharp)

  - JVET-M0423 Cross-check of JVET-M0066: AHG12: Flexible Tile Partitioning, A. Wieckowski (HHI)

  - JVET-M0376 AHG12: On signalling of flexible tiles, M. Damghanian, R.Sjöberg, M. Pettersson (Ericsson)

  - JVET-M0459 AHG12: On tiles with partial CTUs, R. Skupin, K. Suehring, Y. Sanchez, T. Schierl (HHI)

  - JVET-M0527 AHG12: Comments on Tiles and Flexible Tile Partitioning, W. Wan, M. Zhou, T. Hellman, B. Heng, P. Chen (Broadcom)

  - JVET-M0530 is also related to the flexible tile size.

- Tile partitioning and signaling (extraction also related)

  - JVET-M0121 AHG12: On Rectangular Tile Group, Y. He, A. Hamza (InterDigital)

  - JVET-M0123 AHG12: On hierarchical tile design, Y. He, A. Hamza (InterDigital)

  - JVET-M0129 AHG12: On flexible tiling, Y.-K. Wang, Hendry, M. Sychev (Huawei)

  - JVET-M0130 AHG12: On tile grouping, Y.-K. Wang, Hendry, J. Chen, M. Sychev (Huawei)

  - JVET-M0134 AHG12: On explicit signalling of tile IDs, Hendry, Y.-K. Wang, J. Chen, M. Sychev (Huawei)

  - JVET-M0137 AHG12: On tile configuration signaling, M. Sychev, Hendry, Y.-K. Wang (Huawei)

  - JVET-M0160 AHG17: Flexible tile grouping for VVC, L. Chen, T.-D. Chuang, Y.-W. Huang, S.-M. Lei (MediaTek)

  - JVET-M0261 AHG12: On grouping of tiles, M. M. Hannuksela, A. Aminlou (Nokia)

  - JVET-M0373 AHG12: Merge friendly tile group address signaling, R.Sjöberg, M. Damghanian, M. Pettersson (Ericsson)

  - JVET-M0374 AHG12: Flexible tiles to support MCTS use cases, R.Sjöberg, M. Damghanian, M. Pettersson (Ericsson)

  - JVET-M0375 AHG12: On uniform tile spacing, M. Damghanian, R.Sjöberg, M. Pettersson (Ericsson)

  - JVET-M0388 AHG12/AHG17: On merging of MCTSs for viewport-dependent streaming. M. M. Hannuksela (Nokia)

  - JVET-M0416 AHG12: On Tile Information Signalling, S. Deshpande (Sharp)

  - JVET-M0430 AHG12: On Tiles and Tile Groups for VVC, R. Skupin, K. Suehring, Y. Sanchez, T. Schierl (HHI)

  - JVET-M0530 AHG12: On signalling of tiles, M. Coban, M. Karczewicz (Qualcomm)

- Motion constraint tile (inter) and high level signalling

  - // MCTS

    - JVET-M0136 AHG12: Treating tile and tile group boundaries as picture boundaries, J. Chen, Y.-K. Wang, Hendry, M. Sychev (Huawei)

    - JVET-M0445 AHG12: On motion constrained tiles for VVC, R. Skupin, V. George, K. Suehring, Y. Sanchez, T. Schierl (HHI)

  - // Nal unit level signalling

    - JVET-M0155 AHG12: On tile group identification for VVC, B. Choi, S. Wenger, S. Liu (Tencent)

    - JVET-M0536 AHG12: On picture-level tiles and sequence-level tiles for VVC, E. Thomas, A. Gabriel (TNO)

- Wrap around tile (intra)

  - JVET-M0209 AHG12: On tile group configuration, W. Choi, K. Choi, K. Choi (Samsung)

- Specific tool changes for tile

  - JVET-M0300 CE4-related: HMVP and parallel processing with tiles and tile groups, A. M. Kotra, J. Chen, B. Wang, S. Esenlik, H. Gao (Huawei)

  - JVET-M0325 CE8-related: CPR with previous CTU availability at frame and tile edge, C. Rosewarne, A. Dorrell (Canon) – Later withdrawn

- WPP related

  - JVET-M0070 AHG12: Wavefront processing in a tile group, T. Ikai, S. Deshpande, T. Chujoh, E. Sasaki, T. Aono (Sharp)

  - JVET-M0071 AHG12: Improved parallel processing capability with WPP, Y. Fujimoto, M. Ikeda, T. Suzuki (Sony)

  - JVET-M0593 Crosscheck of JVET-M0071 (AHG12: Improved parallel processing capability with WPP), Y. Yasugi, T. Ikai (Sharp)

The AHG recommended to review the related contributions.

JVET-M0013 JVET AHG report: Tool reporting procedure (AHG13) [W.-J. Chien, J. Boyce, R. Chernyak, R. Hashimoto, Y.-W. Huang, S. Liu, D. Luo]

This document summarizes the activity of AHG13: “Tool reporting procedure” between the 12th Meeting in Macao, CN (3–12 Oct. 2018) and the 13th meeting in Marrakech, MA (9–18 Jan. 2019). Tool on/off experimental results vs. VTM anchor are provided for the tools specified in JVET-L1005.

The initial version of JVET-L1005 “Methodology and reporting template for tool testing” was provided on Oct 27th.  The document contained a reporting template.

All tests described in JVET-L1005 were conducted, except 67IPM and PDPC. VTM tool tests were conducted on VTM-3.0 software with VTM configuration by switching off specific tool either in configuration files or macros. Tool tests of 67IPM and PDPC were not conducted because there was no associated configuration setting nor associated macros in VTM-3.0 in order to disable the coding tools.

The tested tools, testers, and cross-checkers are listed in the tables below.

[add notes]

For two topics, testing was not performed because this part of the design could not be disabled in the software:

- 67 intra prediction mode +6 MPM intra mode coding + Wide angle intra prediction

- Position dependent prediction combination

It was remarked that decisions need to be made regarding which features can be disabled, and it is difficult to measure the benefit for a feature if it cannot be turned off. See also the AHG15 report on this topic.

[Integrate glossary terms]

JVET-M0014 JVET AHG report: Progressive intra refresh (AHG14) [J.-M. Thiesse, A. Duenas, K. Kazui, A. Tourapis]

This document summarizes activities of AhG on progressive intra refresh between the 12th and 13th JVET meetings.

An AHG14 kick-off email was sent on 17 December 2018.

A Gitlab repository had been setup for the AHG participants (https://vcgit.hhi.fraunhofer.de/jvet-l-ahg-14/VVCSoftware_VTM). The software based on VTM 3.0 with changes proposed in contributions JVET-M0197 and JVET-M0387 was shared in respective branches for review by participants.

Relevant contributions to this meeting were as follows:

- JVET-M0197 “AHG14: Software for ultra low-latency encoding“, K. Kazui (Fujitsu).

- JVET-M0387 “AHG14: Updates on Intra Refresh Proposal”, J.-M. Thiesse, D. Gommelet, D. Nicholson (Vitec).

- JVET-M0529 “AHG14: Normative Recovery Point Indication”, M. Pettersson, R. Sjöberg, M. Damghanian (Ericsson).

The AhG recommended:

- To review all related contributions.

- To combine all proposed software modifications in an AHG software (Gitlab branch).

- To agree on common test conditions for progressive intra refresh.

- To integrate the encoder-only modifications on next VTM.

JVET-M0015 JVET AHG report: Bitstream decoding properties signalling (AHG15) [J. Boyce, J. Chen, S. Deshpande, M. Karczewicz, A. Tourapis, Y.-K. Wang, S. Wenger]

[deferred]

JVET-M0016 JVET AHG report: Implementation studies (AHG16) [M. Zhou, J. An, E. Chai, K. Choi, S. Sethuraman, T. Hsieh, X. Xiu]

This document summarizes the activity of AHG16: implementation studies, between the 12th JVET meeting in Macao, CN (3-12 October 2018) and the 13th JVET meeting in Marrakesh, MA (9–18 January 2019).

There were not many email exchanges on the main JVET email reflector (jvet@lists.rwth-aachen.de) with an [AHG16] indication on message headers. A summary of the AHG activities is provided as follows:

Feedback provided during the finalization of CE descriptions:

- Multiple hypothesis inter prediction (MHIP): It was suggested to test coding efficiency impact after the line buffer for motion vectors of the additional hypotheses is removed and the total number of distinct reference frames is constrained to 2 for the hypotheses. This is captured in the CE10 description.

- Decoder-side motion vector refinement (DMVR): It was suggested to take the following elements as the baseline DMVR algorithm, i.e.

  - Block sizes for DMVR W*H=64 && H>=8 && W*H<=1024

  - Reference block size (w+7)*(h+7) (for luma)

  - Integer DMVR

  - MV mirroring between list0 and list1 to allow bilateral matching

  - 25 points SAD-based integer-pel search (i.e. (+-) 2 refinement search range)

  - “Parametric error surface equation” based sub-pel refinement

  - Refined MVs used for MC only

  - Luma/chroma MC w/ reference block padding (if needed)

Additional tests were recommended to quantify the benefits of the following DMVR elements in the context of VTM3.0:

  - Mean removed SAD (MRSAD)

  - Bilateral interpolation

  - Use of the refined MVs from the top neighboring CTUs for merging/AMVP/TMVPs/de blocking

  - and splitting a large block to multiple of 16x16 sub-blocks for the DMVR

  - The suggestion above is integrated into in the CE9 description.

- Bi-directional optical flow (BDOF aka. BIO): It was commented that it is desirable to avoid the processing irregularity in the extended area during the prediction block generation for the BIO. In the current design the bilinear filters are used in the extended area. If the 8-tap MC filters are consistently used for the prediction block generation including the extended area, the reference block padding should be used to avoid memory bandwidth increase. Such a test is already planned in CE9.

- Overlapped motion compensation (OBMC): It was suggested to test OBMC when current block is uni-prediction and the neighboring blocks only use uni-prediction to generate OBMC region. When testing sub-block OBMC combined with affine mode, it is suggested to only do blending for top one line and left one column without doing blending for right column and bottom line. Such tests are planned in CE10.

- Current picture referencing (CPR, aka. IBC): It was commented that sharing the line buffer in the de-blocking filter for IBC will likely lead to the local memory increase (to avoid memory access conflicts between the de-blocking and the IBC). This is generally agreed and reflected in the final CE8 description.

- Planar motion vector prediction (PMVP): It was commented that using 8x8 sub-block size (or 8x8 for bi-pred and 8x4/4x8 for uni-pred) is most relevant for the CE tests. It is also desirable to avoid further stressing the merging list derivation process and signal the PMVP mode separately (as opposed to signaling it as an additional merging candidate. This comment might be valid for the regression based motion vector field (RMVF) too). Such tests are planned in CE2.

Other general comments

- Flexible tiling has a profound impact on block-level decoder implementation and should be carefully studied. Potential implications include the increased VDPU processing rate, the increased memory bandwidth if tiling is not VDPU aligned vertically, the increased buffering for frame compression, for in-loop filters and for TMVP storage, and the increased VDPU memory addressing overhead.

- (Affine) merging/AMVP list derivation is a block by block sequential operation within a CTU. Throughput study is needed to make sure that the current VTM3.0 design meets the throughput requirement (e.g. cycle budget of ~50 cycles per 8x8 block for 4K@60 at ~400 MHz clock rate).

- Separate luma/chroma partitioning tree introduces a long latency for chroma intra prediction when the CCLM is on. In the worst case, the chroma intra prediction cannot be kicked off until the whole 64x64 luma block is reconstructed (e.g. when a 64x64 luma block is split horizontally while the chroma blocks are split vertically, and vice versa). Suggest quantifying the gain of separate tree on the top of CCLM to see whether the CCLM and the separate tree can be made mutually exclusive.

- 2x2 Chroma intra-prediction itself is a serious concern from the throughput and processing overhead point of view, given the fact that the CCLM made the luma/chroma intra-prediction sequential, the luma intra prediction has been made more complicated (e.g. 4-tap interpolation filters, PDPC), and the combined intra/inter prediction was already adopted into the intra prediction/reconstruction critical path. It is helpful to test whether the CCLM can be disabled for 4x4 PUs.

- New tools such as diffusion filters, local illumination compensation (LIC), Hadamard transform domain/bilateral filtering of reconstructed blocks, and combined intra/inter prediction are all in the critical path of the intra prediction/reconstruction loop. Given the CCLM, PDPC and combined intra/inter prediction that are already adopted, the additional tools that could be absorbed into this loop is likely very limited from the throughput point of view. Therefore, the benefits of those tools should be carefully studied against each other.

- Line buffer removal from the Adaptive Loop Filter (ALF) is a significant cost reduction which should be pursued.

- The current design of the Bi-directional optical flow (BDOF aka. BIO) prohibits the 64x64 based decoder pipeline due to the early termination which requires a SAD computation up to 128x128 block size.

- 32x32/32x16/16x32 DST7/DCT8 based transform units are the bottleneck for the implementation of primary transforms.

- Transform coefficients “Zero out” of large TUs (e.g. 64x64/32x64/64x32) should be enforced at the syntax-level to avoid unnecessary CABAC context storage on the decoder side.

- Combination of Current Picture Referencing (CPR, aka. IBC) with other tools requires careful thinking. CPR combinations with triangle prediction, bi-prediction, weighted prediction, GBI prediction, BDOF and OBMC will require replication of those motion compensation logics into the intra prediction/reconstruction block. Moreover, using other tools (e.g. MMVD) to signal a CPR vector or using the CPR vectors as neighboring spatial and/or temporal candidates may further complicate the logic of the (affine) merging/AMVP list derivations due to additional CPR-related checks.

Broadcom conducted a memory bandwidth study on multiple hypothesis inter prediction by running both the CE software provided by HHI and the VTM3.0 with a commercial motion compensation cache model integrated. The summary results of the random access configuration are provided in the Table 1 and Table 2 (for informational purposes).

The following contributions were identified for the AHG.

- JVET-M0527, “AHG12: Comments on Tiles and Flexible Tile Partitioning”, W. Wan, M. Zhou, T. Hellman, B. Heng, P. Chen (Broadcom)

  - Elaborated the potential implications of the flexible tiling on decoder designs.

- Contributions aiming for addressing the 64x64 decoder pipeline issue of the BDOF (aka. BIO)

  - JVET-M0284, “CE9-related: BDOF Modifications to Enable 64x64 VPDU”, H. Chen, X. Ma, S. Esenlik, H. Yang, J. Chen (Huawei)

  - JVET-M0073, “Non-CE9: On early termination for BIO”, K. Kondo, M. Ikeda, T. Suzuki (Sony)

  - JVET-M0249”, Non-CE9: Modifications on Bi-Directional Optical Flow”, H. Liu, L. Zhang, K. Zhang, J. Xu, Y. Wang, P. Zhao, D. Hong (Bytedance)

- Contributions relating to ALF line buffer reduction

  - JVET-M0164, “Adaptive loop filter with virtual boundary processing”, C.-Y. Chen, T.-D. Chuang, Z.-Y. Lin, C.-Y. Lai, Y.-W. Huang, S.-M. Lei (MediaTek)

  - JVET-M0301, “Non-CE: Loop filter line buffer reduction”, A.M. Kotra, S. Esenlik, B. Wang, H. Gao, J. Chen (Huawei)

- Contributions aiming to prohibit the transform coefficients from being transmitted in the “zero-out” regions of 64x64/64x32/32x64 transform units at syntax level

  - JVET-M0250, “Non-CE7: Simplified CSBF coding for large block-size transforms”, J. Choi, J. Heo, S. Yoo, J. Choi, L. Li, J. Lim, S. Kim (LGE)

  - JVET-M0257, “CE7-related: coefficient scanning and last position coding for TUs of greater than 32 width or height”, M. Coban, M. Karczewicz (Qualcomm)

- Contributions relating to CPR vector usage and CPR combination with other tools

  - JVET-M0402, “Non-CE8: Comments on Current Picture Referencing”, B. Heng, M.Zhou, W.Wan (Broadcom)

  - JVET-M0175, “CE8-related: Clarification on interaction between CPR and other inter coding tools”, C.-Y. Lai, T.-D. Chuang, Y.-L. Hsiao, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)

  - JVET-M0483, “CE8-related: CPR mode signaling and interaction with inter coding tools”, W.-J. Chien, V. Seregin, M. Karczewicz (Qualcomm)

- JVET-M0046, “CE6-related: A study of primary transforms”,  M. Zhou, Y. Hu (Broadcom)

  - Identified the 32-point DST7/DCT8 as the bottleneck for the implementation of primary transforms and recommended a “zero-out” solution.

- JVET-M0245 , “AHG16-related: Chroma block coding and size restriction”, C. Rosewarne, A. Dorrell (Canon)

  - JVET-M0065 Non-CE: Intra chroma partitioning and prediction restriction [Add authors]

  - Removal of 2x2/2x4/4x2 block sizes from the chroma intra prediction (for the separate tree case only)

- JVET-M0248, “AHG16: Motion compensation with padded samples for small coding units”, H. Liu, J. Chon, H.-C. Chuang, L. Zhang, K. Zhang, J. Xu (Bytedance)

  - Memory bandwidth reduction for motion compensation of small size PUs.

  - There are many other contributions in this category.

- JVET-M0265, “AHG16: Clean-up on MV Rounding”, K. Zhang, L. Zhang, H. Liu, J. Xu, Y. Wang, P. Zhao, D. Hong(Bytedance)

  - Improves design consistency of the MV rounding.

The AHG recommended reviewing the input contributions.

JVET-M0017 JVET AHG report: High-level syntax (AHG17) [R. Sjöberg, S. Deshpande, M. M. Hannuksela, R. Skupin, Y.-K. Wang, S. Wenger]

This document summarizes the activities of the AHG on High-level syntax (HLS) between the 12th JVET meeting in Macao, CN (3-12 Oct. 2018) and the 13th meeting in Marrakesh, MA (9-18 Jan. 2019).

No e-mail related to AHG17 was sent to the JVET reflector during the AHG period except a kick-off message.

It was reported that the amount of input contributions related to the mandates of this AHG has increased from 8 in Macau to 22 for this meeting. For the wider category of high-level syntax contributions including AHG12, AHG14, AHG15 and AHG17, the number of input contributions is reported to have increased from 28 in Macau to 50 for this meeting. Note that the count differs from the JVET-M_Notes_d0.docx categorization in which 49 documents are allocated to section 6.19 “High-level syntax”.

It is reported that of the 12 JCT-VC meetings where the first version of HEVC was developed, the first 6 meetings had fewer high-level syntax contributions than this JVET meeting and the last 6 had a higher number with a peak of 96 of high-level syntax contributions at the 11th meeting in Stockholm 2012.

The number of documents in the high-level syntax category has increased from 8 to 50 in two meeting cycles.

22 of the input documents registered by January 7 were identified as related to the mandates of this AHG.

- JVET-M0101 AHG17: On VVC HLS [R. Skupin, K. Suehring, Y. Sanchez (HHI),M. M. Hannuksela, K. Kammachi-Sreedhar (Nokia), Y.-K. Wang,Hendry (Huawei), S. Wenger, B. Choi (Tencent),S. Deshpande (Sharp)

- JVET-M0120 AHG17: Proposed NAL Unit Header Design Principles [S. Wenger,B. Choi,S. Liu (Tencent)

- JVET-M0128 AHG17: On reference picture management for VVC [Y.-K. Wang,Hendry (Huawei),S. Deshpande (Sharp),M. M. Hannusela (Nokia),G. Ryu,W. Choi (Samsung),X. Wang,Y.-W. Chen (Kawi),L. Zhang (Bytedance),P. Wu,M. Li (ZTE),S.-H. Kim (LG),J. Boyce (Intel),A. M. Tourapis,D. Singer (Apple),F. Edouard,P. Andrivon (Technicolor),Y.-W. Huang,C.-W. Hsu,C.-Y. Chen,T.-D. Chuang,L. Chen (MediaTek),K. Kawamura (KDDI),Y.-C. Sun,J. Lou (Alibaba)

- JVET-M0131 AHG17: On NAL unit types for IRAP pictures and leading pictures [Y.-K. Wang, Hendry (Huawei)

- JVET-M0132 AHG17: On header parameter set (HPS) [Y.-K. Wang, Hendry, J. Chen (Huawei)

- JVET-M0133 AHG17: On parsing dependency between parameter sets [Y.-K. Wang (Huawei), J. Boyce (Intel)

- JVET-M0152 AHG17: On random access point for VVC [B. Choi, S. Wenger, S. Liu (Tencent)

- JVET-M0153 AHG17: On leading picture for VVC [B. Choi, S. Wenger, S. Liu (Tencent)

- JVET-M0154 AHG17: On decoded picture buffer management for VVC [B. Choi, S. Wenger, S. Liu (Tencent)

- JVET-M0156 AHG17: On component type indication for VVC	B. Choi, S. Wenger, S. Liu (Tencent)

- JVET-M0157 AHG17: On picture order count for VVC	B. Choi, S. Wenger, S. Liu (Tencent)

- JVET-M0160 AHG17: Flexible tile grouping for VVC	L. Chen, T.-D. Chuang, Y.-W. Huang,S.-M. Lei (MediaTek)

- JVET-M0161 AHG17: Signalling random access properties in the NAL unit header	L. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)

- JVET-M0260 AHG17: Carriage of tile group header parameters in higher level structures	M. M. Hannuksela (Nokia)

- JVET-M0377 AHG17: Picture header NAL unit type	R. SjÃ¶berg, M. Damghanian, M. Pettersson (Ericsson)

- JVET-M0378 AHG17: RPS for VVC	R. SjÃ¶berg, M. Damghanian, M. Pettersson (Ericsson)

- JVET-M0386 AHG17: On slice_type (tile_group_type)	K. Suehring, Y. Sanchez, R. Skupin (HHI)

- JVET-M0388 AHG12/AHG17: On merging of MCTSs for viewport-dependent streaming	M. M. Hannuksela (Nokia)

- JVET-M0415 AHG17: Comments on High-Level Syntax of VVC	S. Deshpande (Sharp)

- JVET-M0520 AHG17: On NAL unit header design for VVC	S. Wenger, B. Choi, S. Liu

- JVET-M0537 AHG17: On signaling of tile group set with MCTS properties (NAL unit header and new parameter set) 	E. Thomas, A. Gabriel(TNO)

- JVET-M0579 On Frame Rate Support and Extraction in VVC	A. Segall,S. Deshpande (Sharp Labs of America),M. Hannuksela (Noka)

It was noted that at some point in the HEVC development, we started having AHG pre-meetings for HLS work.

The AHG recommended reviewing the related input contributions and to continue to study VVC high-level syntax aspects.

# Project development (5)

Contributions in this category were discussed XXday XX July XXXX–XXXX (chaired by XXX).

## Text and general standard development (0)

## Software development (1)

JVET-M0055 AHG3: VTM transcoding capabilities for bitrate matching and debugging [T. Hinz, A. Wieckowski (HHI)]

## Common test conditions (1)

JVET-M0090 On the use of chroma QP offsets in the VVC common test conditions [C. Helmrich, H. Schwarz, D. Marpe, T. Wiegand (HHI)]

## Coding studies on specific use cases (3)

Contributions on this topic were discussed in Track A Monday 14 January 2015-2240 (GJS).

JVET-M0135 On adaptive resolution change (ARC) for VVC [Hendry, Y.-K Wang, J. Chen (Huawei), T. Davies, A. Fuldseth (Cisco), Y.-C Sun, T.-S Chang, J. Lou (Alibaba)]

This contribution discusses use cases or application scenarios that would benefit from an adaptive resolution change (ARC) capability where the spatial resolution may change at a non-IRAP picture. Then a preliminary design of ARC as is presented and suggested to be a place-holder just to trigger the discussion. It is proposed that the support of ARC for VVC is discussed, and if there is sufficient interest, either start a CE or include aspects of ARC support as one or more mandates of an AHG.

The need to switch may be motivated by wanting to reduce bit rate – being required to send an I frame is obviously a problem for that purpose.

- Rate adaption in video telephony and conferencing

- Active speaker change in multi-party video conferencing

- Fast start in streaming

- Adaptive stream switching in streaming

The basic tools constraints for supporting ARC are as follows:

- The spatial resolution may differ from the nominal resolution by a factor 0.5, applied to both dimensions. The spatial resolution may increase or decrease, yielding scaling ratios of 0.5 and 2.0.

- The aspect ratios and chroma formats of the video format are not changed.

- The cropping areas are scaled in proportion to spatial resolutions.

- The reference pictures are simply re-scaled as needed and inter prediction is applied as usual.

It is proposed to use simple, zero-phase separable down- and up-scaling filters. Note that these filters are for prediction only; a decoder may use more sophisticated scaling for output purposes.

The following 1:2 down-scaling filter is used, which has zero phase and 5 taps:

( −1, 9, 16, 9, −1 ) / 32

The down-sampling points are at even sample positions and are co-sited. The same filter is used for luma and chroma.

For 2:1 up-sampling, additional samples at odd grid positions are generated using the half-pel motion compensation interpolation filter coefficients in the latest VVC WD.

The combined up- and down-sampling will not change phase or the position of chroma sampling points.

This can be considered a coding efficiency feature (esp. for subjective benefit).

A suggested alternative was spatial pre-filtering in the encoder while sending a higher resolution. This could be tested as an alternative. The technique known as “reduced resolution update” was also noted as an alternative.

It was commented that this would also be a natural element of scalable coding. With this an reference picture selection, this is basically all that is needed to support spatial scalability.

Further study in an AHG was encouraged. See also M0259.

JVET-M0259 Use cases and proposed design choices for adaptive resolution changing (ARC) [M. M. Hannuksela, A. Aminlou (Nokia)]

The following is high-level design choices are proposed for VVC version 1:

- It is proposed to include a reference picture resampling process in VVC version 1 for the following use cases:

- Usage of efficient prediction structures (e.g. so-called open groups of pictures) in adaptive streaming without compromising from the fast and seamless representation switching capability between representations of different properties, such as different spatial resolutions.

- Adapting low-delay conversational video content to network conditions and application-originated resolution changes without significant delay or delay variation.

- The VVC design is proposed to allow merging of a sub-picture originating from a random-access picture and another sub-picture originating from a non-random-access picture into the same coded picture conforming to VVC. This is asserted to enable efficient handling of viewing orientation changes in mixed-quality and mixed-resolution viewport-adaptive 360° streaming. This design choice is discussed also in JVET-M0388.

- It is proposed to include sub-picture-wise resampling process in VVC version 1. This is asserted to enable efficient prediction structure for more efficient handling of viewing orientation changes in mixed-resolution viewport-adaptive 360° streaming.

Section 5.13 ("Support for Adaptive Streaming") of MPEG N 17074 includes the following requirement for VVC:

The standard shall support fast representation switching in the case of adaptive streaming services that offer multiple representations of the same content, each having different properties (e.g. spatial resolution or sample bit depth). The standard shall enable the use of efficient prediction structures (e.g. so-called open groups of pictures) without compromising from the fast and seamless representation switching capability between representations of different properties, such as different spatial resolutions.

The contribution suggested that using this with a CRA picture can provide a gradual quality change.

See also the prior JCT-VC contribution JCTVC-F158 (T. Davies/Cisco) July 2011.

Region-wise mixed resolution encoding, including the use of MCTSs, was described. Using the region-wise packing SEI message with MCTSs can provide mixed-resolution view-port dependent streaming for 360° video content and is described in an informative annex of OMAF.

It was commented that DASH has something called stream structure ID that supports stream switching without resolution change.

Further study in an AHG was encouraged. See also M0135.

JVET-M0360 Video coding based on cross RAP referencing (CRR) [H. Yu, X. Gao, Q. Yuan, X. Lin, L. Yu (Zhejiang Univ.), Y. Fan, Y. Zhao, H. Yang, Y.-K. Wang, J. Chen (Huawei)] [late]

This contribution proposes an approach to allow inter prediction referencing across random access points (RAPs), referred to as cross RAP referencing (CRR). Simulation results reportedly show BD coding gains in the range from 11.21% to 25.91% based on VTM3.0 in VVC CTC and from 19.6% to 37.38% based on HM16.15 in HEVC CTC. A brief description of signalling, decoding process, and random access operations is provided. It is proposed to start a CE on this.

The idea of a “library picture” provided by external (unicast) means and referenced across what are otherwise random access points was described.

This scheme does not have any “drift”.

It was noted that the DRAP of an HEVC SEI message (see JCTVC-S0095) has a close relationship to this.

Impact on syntax? It was remarked that, with system support for managing the library, this seems possible with the current syntax (e.g., using long-term reference pictures – possibly more than one of them).

System support would be required no matter what, as the system needs to deliver the library pictures.

Possibly some property indicator could be used to assist – e.g., an SEI message that just indicates that no following pictures in decoding and/or output order will refer to any previous pictures in decoding that are not in the RPS of the current picture.

Further study in an AHG was encouraged to study the properties of such an approach and determine whether this or similar functionality would need to have some impact on the syntax or decoding process. Such an impact does not seem strictly necessary for basically achieving this usage, given appropriate systems support, but may be determined helpful in some way in further study. It was suggested for systems experts to also study this and determine whether file-format / DASH / other system support should be specified for this sort of usage.

JVET-M0855 Crosscheck of JVET-M0360 (Video coding based on cross RAP referencing (CRR)) [H. Liu (Bytedance)] [late] [miss]

JVET-M0514 Removal of CIP from Multi-hypothesis Intra Prediction [C.-C. Chen, W.-J. Chien, M. Karczewicz (Qualcomm)]

JVET-M0667 Crosscheck of JVET-M0514 (Removal of CIP from Multi-hypothesis Intra Prediction) [J. Li, C. S. Lim (Panasonic)] [late]

JVET-M0197 AHG14: Software for ultra low-latency encoding [K.Kazui (Fujitsu)]

TBP.

JVET-M0387 AHG14: Updates on Intra Refresh Proposal [J.-M. Thiesse, D. Gommelet, D. Nicholson (VITEC)] [late] [miss]

This contribution reports an update of the software modifications for the study of progressive intra refresh (used in AHG 14). This code has been made available in the AHG Gitlab repository (see the AHG14 report). Both encoder-only modifications and a normative solution have been reported to VTM 3.0.1 with handling of new tools adopted at the Macao meeting. The corresponding performance is reportedly in the same range as the results presented at the Macao meeting, with a loss of 21.29% in average for the encoder-only modifications against a low-delay B reference with a comparable intra period of 32 for classes B, C and E. The normative proposal reportedly brings an improvement of approximately 5.26% (test results for classes C and E; class B data not available) against an encoder-only modification solution by adding identify the position of the intra refresh area and then exploit this position for decoding process.

Test conditions were proposed for further AHG activity.

It was also proposed to integrate the proposed encoder-only modifications for intra-refresh support on VTM 4.

In the proposed normative approach, there is a disabling of the loop filter at the boundary of the refresh area and avoidance of MVs that reference potentially corrupted samples. Tiles can be used with disabling of the loop filter at tile boundaries.

The proposal used whole-picture encoding. It was asked whether that is appropriate for what is intended as ultra-low-delay operation. This implies a whole picture latency.

The refresh was not using whole-CTU refresh; it was using CU-level intra. It was saving the sending of a couple of CU-level flags based on a signalled width for the refresh region. There was discussion of whether that aspect is desirable and provides any meaningful bit rate savings by itself.

It was suggested to consider thicker refresh regions, perhaps not spanning the whole picture vertically. This would present a much less severe restriction on motion vectors. Using an adaptive-size region was also suggested.

Part of the measured gain seems to be based on the use of a fine-granularity refresh area.

The application usage was discussed. Suggested applications were multicast real-time communication, video gaming, surveillance, drone video, remote driving, remote display.

It did not yet seem fully clear whether the proposed method is appropriate and the testing conditions are reasonable.

JVET-M0871 AHG14: Performance Evaluation by CTU based Intra Refresh [K. Kawamura, S. Naito (KDDI)] [late]

TBP.

JVET-M0466 Adaptive Streaming Test Conditions for VTM [M. Afonso, A. Norkin, A. Aaron, J. Sole, K. Swanson (Netflix), Y. Ye, W. Jiang (Alibaba), J. Kim, K. Kolarov, D. Singer, A. Tourapis (Apple)]

TBP.

## Test material (0)

# Core Experiments

## CE1: Partitioning (3)

JVET-M0021 CE1: Summary report on partitioning [J. Ma, F. Le Léannec, M. W. Park]

This contribution was discussed on Wednesday 9 January 1530–1700 (GJS).

This document evaluates CE1: Partitioning JVET-L1021. There were two tests that were conducted by proponents and cross-checked by at least one cross-checker. There were no mismatches reported to the coordinators.

The current VTM Software has split restrictions implemented to allow 64x64 pipelining. More precisely, the concept of (square shaped) virtual pipelining data units (VPDUs) is used to allow for 64x64 pipelining inside the picture.

The software for both tests can be found here

https://vcgit.hhi.fraunhofer.de/JVET-L-CE1/VVCSoftware_VTM.git

SubCE1.1.1

(JVET-M0446, Tencent)

Proponents study non square VPDUs to enhance RD-performance and unify with boundary partitioning of the current VTM software.

Conditions for CU_ TRIH_SPLIT and CU_ TRIV_SPLIT are changed so that, e.g. 128x32 and 32x128 block sizes are possible. Sub-partitions of such blocks are also allowed. However, 128x64 and 64x128 are not allowed to be split using a ternary split whereas 128x128 can be split using a ternary split. Results do not effect AI.

| # | Tester | Comments | Cross-checkers |
| --- | --- | --- | --- |
| 1.1.1 | M. Xu (Tencent) | The cross-checkers (CC) generally confirm the simulation results and the matching of the implementation with the CE description. Mismatches for encoder timings were reported as follows: LDB 110% (CC) vs 105% (proponent). Further comment/suggestions were made by CC: - Formulate the constraints in a more general way, e.g. instead of not allowing a ternary split for 128x64 and 64x128, the proponents can constraint the size by 2·MAX_TU_SIZE_FOR_PROFILE x MAX_TU_SIZE_FOR_PROFILE - It should be considered how many extra partitions would be supported if using rectangular VPDUs mixed with squared VPDUs instead of only squared VPDUs. - More details on the latency impact of using different VPDU shapes are desired. For example, in Figure 1 two cases are compared: First: all 64x64 VPDU structures vs Second: First four 32x128 VPDUs and then four 64x64 VPDUs. Now VPDU4 in case 1 of the figure below would have less pipeline dependency and latency issues if data from VPDU1 is used compared to case 2 of the figure below were data from VPDU3 is always used. - Interest for interactions with other tools such as CPR/WPP were expressed. | C. W. Hsu (MediaTek) |

Example for different VPDU structures. Numbers denote the processing order of VPDUs

While the cross-checker indicated that case 2 had more cases where the immediately previous decoded region was needed for prediction (e.g., intra prediction) than case 1. The proponent indicated that the worst case was the same in both cases. And if what matters is average rather than worst case, case 2 would be more rarely encountered.

It was suggested that the key test sequences for this issue are the high resolution ones, since there is generally little gain for using very large blocks with low-resolution test sequences.

The VTM uses 64x64 VPDUs (for luma). The proposal tries to get coding efficiency by allowing rectangular shapes 32x128/128x32.

In the current draft spec, these rectangular shapes are available for the edge of the picture.

|  | All Intra Main 10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 97% | 98% | -0.48% | -0.48% | -0.42% | 117% | 99% |  |  |  |  |  |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 100% | -0.48% | -0.37% | -0.31% | 110% | 99% |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 100% | -0.17% | -0.17% | -0.24% | 110% | 100% | -0.15% | -0.65% | -0.09% | 105% | 96% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | -0.03% | 104% | 102% | -0.01% | 0.26% | 0.19% | 102% | 98% |
| Class E | 0.00% | 0.00% | 0.00% | 102% | 101% |  |  |  |  |  | -0.54% | -1.06% | -1.41% | 109% | 96% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% | -0.25% | -0.23% | -0.23% | 110% | 100% | -0.20% | -0.45% | -0.33% | 105% | 97% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 101% | 0.02% | -0.12% | -0.09% | 102% | 102% | -0.01% | 0.31% | 0.22% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% | -0.11% | -0.11% | -0.04% | 108% | 102% | -0.17% | -0.15% | -0.25% | 102% | 99% |

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-3.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.14% | 0.00% | 0.07% | 106% | 96% |
| Class C | 0.03% | 0.14% | -0.34% | 101% | 100% |
| Class E | -0.49% | -0.87% | -0.91% | 109% | 101% |
| Overall | -0.17% | -0.17% | -0.31% | 105% | 99% |
| Class D | 0.01% | -0.40% | -0.36% | 101% | 101% |
| Class F | -0.07% | -0.40% | -0.72% | 102% | 101% |

The proponent highlighted the benefit of 0.5% for Class A in RA configuration.

Further simulation results are available in JVET-M0446 including:

- Simulations without some VTM encoder speedups (0.48% becomes 0.54% benefit for Class A in RA configuration)

- Simulations for limiting the VTM to not use 32x128/128x32 partitions at boundaries (0.0X% penalty for Class A in RA configuration)

It was commented that the coding efficiency benefit of the proposal comes with a substantial encoder search penalty (~13%) and a similar increase in encoder search complexity without changing the syntax/decoding process would also provide some coding efficiency benefit.

Revisit after getting that 0.0X% number. It was suggested that removing the special treatment of the edges without adding support for alternative VPDU shapes seemed to be the likely outcome.

SubCE1.2.1

(JVET-M0236, Canon)

Proponents extend current TU tiling such that TUs do not cross the boundaries of a 64x64 grid. It is also proposed that by using this technology the ternary split at the top level becomes available and therefore an improved RD-performance is possible.

Conditions for TU_MAX_TR_SPLIT are changed such that each resulting TU is contained in a single 64x64 region on a 64x64 grid. This is obtained by using smaller TUs than what is currently being used in the VTM software. For instance, by allowing the ternary split at the top level a 64x128 center block arises which is then tiled into four 64x32 TUs to match the constraint. Results do not effect AI.

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  |  |  |  | Low delay B Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 |  |  |  |  |  |  |  | Over VTM-3.0 |  |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y |  | U |  | V |  | EncT | DecT | Y |  | U |  | V |  | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 102% | 106% | -0.52% |  | -0.71% |  | -0.50% |  | 132% | 98% |  |  |  |  |  |  |  |  |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 103% | -0.66% |  | -0.59% |  | -0.45% |  | 123% | 100% |  |  |  |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 102% | -0.28% |  | -0.26% |  | -0.24% |  | 122% | 101% | -0.20% |  | -0.47% |  | 0.03% |  | 118% | 97% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 103% | -0.03% |  | -0.05% |  | 0.00% |  | 113% | 104% | 0.00% |  | 0.24% |  | -0.12% |  | 106% | 107% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 99% |  |  |  |  |  |  |  |  | -0.70% |  | -0.61% |  | -0.40% |  | 123% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 103% | -0.34% |  | -0.36% |  | -0.27% |  | 122% | 101% | -0.26% |  | -0.27% |  | -0.13% |  | 115% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 96% | 105% | -0.05% |  | -0.08% |  | -0.03% |  | 113% | 105% | 0.00% |  | 0.29% |  | 0.21% |  | 103% | 96% |
| Class F (optional) | 0.00% | 0.00% | 0.00% | 100% | 102% | -0.17% |  | -0.22% |  | -0.16% |  | 120% | 101% | -0.35% |  | -0.21% |  | -0.36% |  | 110% | 104% |

Remarks:

- The gain is substantial (0.59% for Class A in RA configuration)

- There is a substantial encoder complexity increase (~27%)

- This has a latency issue for decoding, since the TU coefficient data needs to be buffered up for VPDU pipeline processing. A related contribution M0237 proposes to shift this buffering burden to the encoder.

No action was taken on this.

Related proposals M0195 and M0285 allows PUs to straddle the VPDU boundaries but requires there to be no residual in those cases (0.38% benefit for Class A in RA configuration with encoder complexity increase of 19% if allowing all modes with zero residual, or 0.2% benefit for Class A in RA configuration with encoder complexity increase of 5% if allowing only skip mode in these cases). No action was taken on these as well.

JVET-M0236 CE1: Transform tiling for pipelined processing of CTUs (Test 1.2.1) [C. Rosewarne, A. Dorrell (Canon)]

JVET-M0446 CE1: Rectangular virtual pipeline data unit (test 1.1.1) and supplementary results [M. Xu, X. Li, S. Liu (Tencent)]

JVET-M0856 Cross-check of JVET-M0446: CE1: Rectangular virtual pipeline data unit (test 1.1.1) and supplementary results [A. Wieckowski (HHI)] [late]

## CE2: Subblock motion compensation (24)

Contributions in this category were discussed Wednesday 9 Jan. 1540–2100 and Thursday 10 Jan. 0900-1100 ( Track B chaired by JRO)

JVET-M0022 CE2: Summary report on sub-block based motion prediction [Y. He, C.-Y. Chen, C.-C. Chen]

The goal of Core Experiment 2 (CE2) is to investigate sub-block based motion prediction techniques on top of VTM-3.0. It comprises 5 categories,

- CE 2.1: Affine motion compensation

- CE 2.2: Affine merge mode

- CE 2.3: Sub-block based merge mode

- CE 2.4: Complexity reduction

- CE 2.5: ATMVP and related topics

For each test, a comparative study along with related tests is conducted, results and complexity analysis are provided. Crosschecking reports of all tests are integrated in this document as well.

CE2.1: Affine motion compensation

| Test# | Description | Document# |
| --- | --- | --- |
| 2.1.1 | Adaptive precision for affine MVD coding: a{1-pel,1/4-pel,1/4-pel}, b{1/4-pel,1/4-pel,1/4-pel}, c{1/8-pel,1/8-pel,1/8-pel} | JVET-M0420 |
| 2.1.2 | Adaptive MVD precision for affine inter mode coding: {1-pel,1-pel,1-pel}, {1/4-pel,1/4-pel, 1/4-pel}, {1/16-pel,1/16-pel,1/16-pel} | JVET-M0246 |

The numbers are referring to {top-left,top-right,bottom-left}

Current affine uses 1/4 pel precision for signalling, but 1/16 pel precision in sub-blocks; affine merge uses 1/16 pel (but does not need to signalling)

Both tests use signalling and are only modifying affine MV coding, merge is unchanged. Test 2.1.1 uses 1 flag to signal 1/4 (case b) or not, and 1 flag to signal (if not) if it is a or c. Test 2.1.2 re-uses the adaptive MV precision signalling (but with 2 additional contexts) for affine, and also uses switching 1/16,1/4, 1 pel (instead 1/4, 1, 4 pel) precision.

The method comes with increase in encoder runtime, whereas decoder complexity increase is marginal (some additional logic and 2 contexts)

Method of 2.1.2 has slightly higher gain, and is assessed to be conceptually more straightforward, consistent with AMVR (it is however reported that by just reusing AMVR with same precision steps and same context coding, loss would occur).

The motion vector rounding is also consistent with the current design.

Decision: Adopt JVET-M0246 (Test 2.1.2), extending AMVR to affine. Use the AMVR high level flag for disabling both “normal” AMVR and “affine” AMVR.

Revisit: Specification text to be provided and reviewed.

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 2.1.1 | -0.19% | -0.08% | -0.12% | 105% | 101% | -0.13% | -0.16% | 0.04% | 106% | 100% |
| 2.1.2 | -0.23% | -0.21% | -0.27% | 104% | 99% | -0.20% | -0.08% | -0.30% | 112% | 100% |

CE2.2: Affine merge

| Test# | Description | Document# |
| --- | --- | --- |
| 2.2.1.a | Bypass Affine flag for Skip CU | JVET-M0380 |
| 2.2.1.b | CABAC context change for Affine Flag | JVET-M0380 |
| 2.2.2.a | 2 contexts: one for Affine and one for regular Merge index | JVET-M0381 |
| 2.2.2.b | 1 context: one for both Affine Merge index and regular Merge index | JVET-M0381 |
| 2.2.3.a | Applying affine HMVP candidates to affine merge | JVET-M0125 |
| 2.2.3.b | Applying affine HMVP candidates for affine motion vector prediction | JVET-M0125 |
| 2.2.3.c | Combined test of 2.2.3a and 2.2.3b | JVET-M0125 |
| 2.2.3.d | Replace existing inherited affine candidates with affine HMVP candidates | JVET-M0125 |
| 2.2.4.a | Affine merge with offset and block level signalling | JVET-M0431 |
| 2.2.4.b | Based on 2.2.4a, but offset value range depends on picture height | JVET-M0431 |
| 2.2.4.c | Based on 2.2.4b, but POC distance based offset mirroring for Bi-prediction | JVET-M0431 |
| 2.2.4.d | Based on 2.2.4b, but POC distance based offset scaling for Bi-prediction | JVET-M0431 |
| 2.2.5 | Affine MV offset merge candidates | JVET-M0476 |
| 2.2.6.a | Simplification for 6-param and 4-param constructed candidates by setting P = 3 in 7 availability cases | JVET-M0477 |
| 2.2.6.b | Simplification for 6-param and 4-param constructed candidates by setting P = 3 in 2 availability cases | JVET-M0477 |
| 2.2.7.a | “Low complexity” affine merge with temporal candidates (however, increases complexity compared to VTM, see notes below) | JVET-M0256 |
| 2.2.7.b | “Medium complexity” affine merge with temporal candidates (however, increases complexity compared to VTM, see notes below) | JVET-M0256 |
| 2.2.7.c | Withdraw |  |
| 2.2.7.d | Withdraw |  |
| 2.2.7.e | Withdraw |  |
| 2.2.8.a | In affine AMVP, apply some pruning on candidates. Rounding, clipping applied before pruning | JVET-M0282 |
| 2.2.8.b | In affine merge, apply some pruning on inherited candidates | JVET-M0282 |
| 2.2.8.c | 2.2.8.a + 2.2.8.b | JVET-M0282 |
| 2.2.8.d | 2.2.8.c + 4.1.5.c | JVET-M0282 |

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 2.2.1.a | 0.09% | 0.21% | 0.20% | 102% | 98% | 0.03% | 0.04% | 0.06% | 103% | 92% |
| 2.2.1.b | -0.01% | 0.02% | -0.01% | 100% | 100% | 0.00% | 0.02% | 0.06% | 99% | 98% |
| 2.2.2.a | 0.00% | 0.00% | -0.01% | 100% | 98% | 0.03% | -0.05% | 0.10% | 99% | 94% |
| 2.2.2.b | 0.07% | 0.04% | 0.06% | 100% | 104% | 0.05% | 0.03% | 0.15% | 100% | 96% |
| 2.2.3.a | -0.06% | -0.04% | -0.02% | 100% | 100% | -0.06% | 0.04% | 0.09% | 101% | 100% |
| 2.2.3.b | -0.01% | 0.01% | 0.02% | 100% | 100% | 0.00% | 0.00% | 0.09% | 102% | 101% |
| 2.2.3.c | -0.07% | -0.04% | -0.04% | 100% | 100% | -0.03% | -0.05% | -0.18% | 101% | 102% |
| 2.2.3.d | 0.07% | 0.10% | 0.06% | 100% | 100% | 0.09% | 0.09% | 0.02% | 101% | 101% |
| 2.2.4.a | -0.16% | -0.13% | -0.14% | 104% | 100% | 0.01% | -0.05% | -0.07% | 107% | 100% |
| 2.2.4.b | -0.17% | -0.12% | -0.13% | 104% | 100% | -0.02% | 0.09% | -0.04% | 106% | 100% |
| 2.2.4.c | -0.20% | -0.08% | -0.09% | 104% | 100% | 0.04% | -0.01% | 0.19% | 105% | 100% |
| 2.2.4.d | -0.20% | -0.09% | -0.09% | 104% | 99% | 0.02% | 0.07% | 0.07% | 106% | 99% |
| 2.2.5 | -0.06% | -0.02% | -0.05% | 104% | 101% | -0.03% | 0.06% | -0.03% | 105% | 99% |
| 2.2.6.a | 0.00% | 0.01% | -0.01% | 100% | 100% | 0.07% | -0.01% | 0.18% | 99% | 99% |
| 2.2.6.b | 0.02% | 0.04% | 0.00% | 100% | 100% | 0.08% | 0.14% | 0.04% | 99% | 98% |
| 2.2.7.a | -0.08% | -0.02% | -0.13% | 105% | 102% | -0.03% | -0.12% | -0.09% | 116% | 115% |
| 2.2.7.b | -0.10% | -0.10% | -0.14% | 106% | 102% | -0.07% | -0.21% | -0.08% | 117% | 116% |
| 2.2.8.a | 0.00% | 0.01% | 0.00% | 100% | 100% | -0.01% | 0.04% | -0.07% | 100% | 100% |
| 2.2.8.b | -0.01% | -0.01% | 0.00% | 101% | 100% | -0.03% | 0.00% | -0.05% | 100% | 100% |
| 2.2.8.c | -0.02% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.05% | 0.10% | 100% | 100% |
| 2.2.8.d | -0.04% | -0.01% | -0.01% | 100% | 100% | -0.03% | -0.12% | -0.17% | 100% | 100% |

Complexity analysis

|  | AMVP/Merge list size | Max number of potential candidates | Max number of MV comp. | Max number of ref idx comp. | Max number of MV scaling | Max number of conditional cases to enable constructed candidates | Others |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2.2.1.a | same | Same | Same | same | same | same |  |
| 2.2.1.b | same | Same | Same | same | same | same |  |
| 2.2.2.a | same | Same | Same | same | same | same |  |
| 2.2.2.b | same | Same | Same | same | same | same |  |
| 2.2.3.a | same | Merge +5 | Same | same | same |  | *Affine HMVP 170 bytes |
| 2.2.3.b | same | AMVP + 2 | Same | +2 | same |  | *Do ref index selection for affine HMVP based on the selected ref. in AMVP *Affine HMVP 68 bytes |
| 2.2.3.c | same | Merge +5 / AMVP +2 | Same | +2 | same |  | *Affine HMVP 170 bytes |
| 2.2.3.d | same | same | Same | -1 | same |  | Reduce CPMV buffer from 1296 bytes to 170 bytes. Note: 1296 bytes requires optimized affine CPMV buffer update. Otherwise, storage of a full CTU's CPMV needs 6192 bytes. Position comp: merge + 5/AMVP +4 |
| 2.2.4.a | same | Same | Same | same | same | same |  |
| 2.2.4.b | same | Same | Same | same | same | same |  |
| 2.2.4.c | same | Same | same | same | same | same |  |
| 2.2.4.d | same | same | same | same | same | same |  |
| 2.2.5 | same | same | same | same | same | same |  |
| 2.2.6.a | same | -3 | -8 | -22 | same | same |  |
| 2.2.6.b | same | -3 | -8 | -22 | same | 2(-5) |  |
| 2.2.7.a | same | same | + 38 | -12 | +2 |  |  |
| 2.2.7.b | same | +3 | +98 | +26 | +3 |  |  |
| 2.2.8.a | same | same | +3 | same | same |  |  |
| 2.2.8.b | same | same | +6 | +2 | same |  |  |
| 2.2.8.c | same | same | +3 for AMVP +6 for Merge | +0 for AMVP +2 for Merge | same |  |  |
| 2.2.8.d |  |  |  |  |  |  |  |

2.2.1a has loss, but does not really solve a problem in terms of complexity

2.2.1b uses different neighbor positions (A1/B1) for the context of affine flag coding (which currently uses A2/B3), claiming that these are positions different from those used in merge. However, other elements (e.g. AMVR, triangular, quadtree/MTT split flag) use also A2/B3. Therefore, making this change only for affine flag does not seem to be a unification/simplification.

The current affine merge has 4 context coded bins. Both tests 2.2.2a/b reduce this to only one (as in normal merge mode), where 2.2.2a uses different contexts for affine and normal merge, whereas 2.2.2b uses the same. 2.2.2a has no loss in RA, very small loss in LB, whereas 2.2b also shows 0.07% loss in RA.

Decision: Adopt JVET-M0381 Test 2.2.2a (reducing number of context coded bins in affine merge). Text is available with the contribution.

2.2.3.x establishes a history-based motion vector prediction for affine, however the HMVP candidates are different (specifically collected from affine coded blocks) than in normal HMVP.

It is commented that no coordinates are used for a/b/c (which theoretically might lead to wrong model depending on distance)

a/b/c have additional complexity in list derivation and storage of history camdidates, which is not justified by the small (highest 0.07% for 2.2.3c)

2.2.3d is interesting as it reduces the need of local storage for spatial CPMV (control point motion vectors) and replaces CPMV by history-based candidates. It however also disables the inheritance at CTU boundaries, which is asserted to be the main reason for the loss of 0.07% (there are CE related contributions JVET-M0432, JVET-M0110, JVET-M0168, JVET-M0262, JVET-M0270) that tackle this issue in various ways. The approach should be further studied to solve the complexity reduction with less penalty in terms of compression performance.

Test 2.2.4.x signals offset for affine merge mode (applied to all CPMV). The method b based on picture height only provides small additional benefit. The method d has some additional complexity for scaling, without showing benefit over c (which just uses mirroring). Gain is on average between 0.16% (for a) and 0.20% for c/d, generally higher for high resolution sequences.

Decision: Adopt JVET-M0431, test CE3-2.2.4c, but on top of 2.2.4a (using distance offsets 1/2, 1, 2, 4, and 8-pel as per table 2.1 of M0431). Add a high-level enabling flag.

Revisit: Specification text to be provided and reviewed.

See also the discussion of this topic in the notes of the plenary of Sunday 13 January in section 11.1.

2.2.5 is a similar approach with same number of offset candidates (total 20), but supports larger offset values, and different combinations. Gives less compression benefit.

2.2.6.x reduce the number of constructed affine candidates from 6 to 3. However, loss of 0.07% is observed in LDB (where constructed candidates may be used more often). Furthermore, the study of AHG16 shows that affine merge is not the most critical path in VVC. Therefore, proposals which reduce complexity in this area should rather come with very low penalty in compression.

2.2.7.a removes “mixed” spatial-temporal constructed candidates, replacing them by pure temporal candidates. The total number of candidates is unchanged, but the list construction (pruning) becomes more complicated. Gain is 0.08%/0.03% for RA/LB

2.2.7b keeps the original candidates, and adds three more pure temporal candidates. The list construction (pruning) has significantly higher complexity than current VVC (almost 3x number of MV comparisons). Gain is 0.1%/0.07% for RA/LDB

Some of the reported gain may also be due to encoder changes. No good tradeoff of performance compared to additional complexity. Also reports increased encoder runtime.

2.2.8.x adds some pruning processes to affine MV prediction and merge modes. Benefit in terms of compression not obvious. (Note that the technology had more promising compression benefit before VTM3).

Sub-CE 2.3: Sub-block based merge mode

| Test# | Description | Document# |
| --- | --- | --- |
| 2.3.1.a.1 | Planar motion vector prediction (PMVP) as a single mode with the sub-block size setting: 8x4 for BI, 4x4 for UNI | JVET-M0104 |
| 2.3.1.a.2 | PMVP as a single mode with the sub-block size setting: 4x8 for BI, 4x4 for UNI |  |
| 2.3.1.a.3 | PMVP as a single mode with the sub-block size setting: Adaptive 8x4/4x8 for BI, 4x4 for UNI |  |
| 2.3.1.a.4 | PMVP as a single mode with the sub-block size setting: 8x8 for BI, 4x4 for UNI |  |
| 2.3.1.a.5 | PMVP as a single mode with the sub-block size setting: Adaptive 8x4/4x8 for BI and UNI |  |
| 2.3.1.a.6 | PMVP as a single mode with the sub-block size setting: 8x8 for BI and UNI |  |
| 2.3.1.b | Withdraw |  |
| 2.3.1.c | PMVP as a candidate in sub-block (affine) merge list |  |
| 2.3.1.d | Withdraw |  |
| 2.3.2.a | The threshold is set as 1 pixel for the sub-blocks inside the 4x16, 16x4 and 8x8 blocks | JVET-M0485 |
| 2.3.2.b | The threshold is set to 0 pixel for 4x16 and 16x4 blocks along vertical and horizontal directions, respectively; and the threshold is set as 1 pixel for the sub-blocks inside 8x8 blocks |  |
| 2.3.3.a | Regression-based Motion Vector Field (RMVF) as a separate merge mode as described in JVET-L0171 | JVET-M0302 |
| 2.3.3.b | 2.3.3.a + the RMVF-coded blocks are used for affine merge candidate inheritance |  |
| 2.3.3.c | 2.3.3.a + complexity reduction (i.e., lower number of neighboring motion information for RMVF parameter derivation) |  |
| 2.3.3.d | 2.3.3.b + 2.3.3.c |  |

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 2.3.1.a.1 | -0.11% | -0.10% | -0.17% | 103% | 103% | -0.08% | 0.01% | 0.09% | 104% | 103% |
| 2.3.1.a.2 | -0.10% | -0.18% | -0.18% | 103% | 103% | -0.05% | 0.03% | 0.01% | 104% | 103% |
| 2.3.1.a.3 | -0.11% | -0.13% | -0.22% | 103% | 103% | -0.04% | -0.11% | -0.07% | 104% | 104% |
| 2.3.1.a.4 | -0.11% | -0.07% | -0.15% | 103% | 101% | -0.05% | -0.07% | -0.19% | 104% | 102% |
| 2.3.1.a.5 | -0.10% | -0.15% | -0.18% | 103% | 102% | -0.06% | 0.01% | 0.02% | 104% | 103% |
| 2.3.1.a.6 | -0.11% | -0.10% | -0.17% | 103% | 103% | -0.08% | 0.01% | 0.09% | 104% | 103% |
| 2.3.1.c | 0.08% | 0.12% | 0.06% |  |  | 0.07% | -0.01% | 0.22% |  |  |
| 2.3.2.a (over VTM-3.0) | -0.12% | -0.18% | -0.21% | 104% | 107% | -0.08% | -0.05% | 0.05% | 106% | 112% |
| Over 2.3.1 |  |  |  |  |  |  |  |  |  |  |
| 2.3.2.b | -0.12% | -0.15% | -0.23% | 104% | 107% | -0.08% | 0.22% | 0.08% | 106% | 112% |
| Over 2.3.1 |  |  |  |  |  |  |  |  |  |  |
| 2.3.3.a | -0.18% | -0.17% | -0.26% | 105% | 102% | -0.02% | -0.10% | 0.09% | 106% | 102% |
| 2.3.3.b | -0.20% | -0.25% | -0.24% | 104% | 101% | -0.02% | 0.12% | 0.21% | 103% | 102% |
| 2.3.3.c | -0.19% | -0.13% | -0.19% | 105% | 102% | 0.01% | 0.05% | -0.03% | 106% | 101% |
| 2.3.3.d | -0.19% | -0.22% | -0.23% | 105% | 100% | 0.00% | 0.07% | -0.16% | 106% | 101% |

Test 2.3.1.a adds PMVP as a new mode to determine subblock vectors. 2.3.1.a3 imposes the same restrictions as current (non-affine) subblock MC. Low gain (around 0.1%), whereas encoder/decoder run time increases. The CE report does not have a detailed complexity analysis, but it can be asserted to be not insignificant, as MV scaling is necessary for each subblock.

Test 2.3.1.c puts PMVP as additional candidate into the affine merge list (but uses 8x8 subblocks) – therefore, loss is observed, probably as affine usually has 4x4 subblocks, and/or merge list size is reduced.

PMVP had 0.6% gain before, which is now down to 0.1%. This does not justify adding another subblock mode with additional processing.

Test 2.3.3 is about Regression-based Motion Vector Field (RMVF), where a set of maximum of 112 candidates is fed into the regression in case of a 128x128 block , or 7 candidates in case of an 8x8 block (each of which has to be scaled before). The derivation performs regression versus a 6-parameter affine model. 2.3.3a determines the subblock candidates (8x8 SB) directly, 2.3.3b puts them as a candidate for affine merge, and then uses it with 4x4 subblocks.

Tests c and d correspond to a and b, but reduce the number of input candidates to roughly half. Gain is around 0.2% for b, and 0.19% for c/d. This gain is much lower than reported originally (which was around 0.7%). Also the dependency of the regression on block size and number of candidates is of some concern (integer 16 bit, division-free). No precise analysis of complexity was made, but it is likely that the complexity of the regression (worst case of number of multiplications, availability checks, scaling operations, number of cycles) as used in the current CE would not justify adding another subblock mode, or an additional derivation of the affine model construction. Further study in CE to further reduce the number of candidates, and perform analysis of complexity impact.

Sub-CE2.4: Complexity reduction

| Test# | Description | Document# |
| --- | --- | --- |
| 2.4.1.a | 4x4 block is used as the sub-block size for a uni-directional affine coded CU while 8x4/4x8 block is used as the sub-block size for a bi-directional affine coded CU according to the width/height of the CU. | JVET-M0226 |
| 2.4.1.b | 4x4 block is used as the sub-block size for a uni-directional affine coded CU while 8x4 block is used as the sub-block size for a bi-directional affine coded CU. |  |
| 2.4.1.c | 4x4 block is used as the sub-block size for a uni-directional affine coded CU while 4x8 block is used as the sub-block size for a bi-directional affine coded CU. |  |
| 2.4.2 | Restricted motion vector field for affine mode. | JVET-M0309 |
| 2.4.3.a | Affine restrictions for the worst-case bandwidth reduction: The worst-case bandwidth is bi-prediction of an INTER_8x4/INTER_4x8 block. (*changes only luma) | JVET-M0150 |
| 2.4.3.b | The worst-case bandwidth is uni-prediction of an INTER_4x4 block. (*changes only luma) |  |
| 2.4.3.c | The worst-case bandwidth is bi-prediction of an INTER_9x9 block. (*changes only luma) |  |
| 2.4.4.a | Affine sub-block size restrictions: 8x4 for BI, 4x4 for UNI. (* changes applied to both luma and chroma) | JVET-M0472 |
| 2.4.4.b | 4x8 for BI, 4x4 for UNI. (* changes applied to both luma and chroma) |  |
| 2.4.4.c | Adaptive 8x4/4x8 for BI, 4x4 for UNI. (* changes applied to both luma and chroma) |  |
| 2.4.4.d | 8x8 for BI, 4x4 for UNI. (* changes applied to both luma and chroma) |  |
| 2.4.5.a | clip sub-block MVs in 8x8 block for UNI and BI prediction. | JVET-M0488 |
| 2.4.5.b | Withdraw. |  |
| 2.4.6 | Modified affine inheritance from above CTU. | JVET-M0054 |
| 2.4.7 | Size constrain for inherited affine motion prediction. | JVET-M0053 |
| 2.4.8 | Affine model inheritance with reduced memory requirement. | JVET-M0262 |
| 2.4.9 | Withdraw. |  |

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 2.4.1.a | 0.11% | 0.10% | 0.09% | 100% | 99% | 0.10% | 0.12% | 0.11% | 101% | 97% |
| 2.4.1.b | 0.10% | 0.09% | 0.08% | 101% | 99% | 0.11% | 0.00% | 0.20% | 99% | 95% |
| 2.4.1.c | 0.12% | 0.07% | 0.09% | 99% | 96% | 0.13% | 0.16% | 0.16% | 99% | 95% |
| 2.4.2 | 0.01% | 0.00% | 0.05% | 101% | 100% | 0.01% | -0.05% | 0.07% | 101% | 100% |
| 2.4.3.a | -0.01% | 0.02% | 0.01% | 99% | 100% | 0.02% | -0.04% | -0.05% | 101% | 101% |
| 2.4.3.b | 0.27% | 0.14% | 0.18% | 97% | 98% | 0.27% | 0.22% | 0.08% | 99% | 100% |
| 2.4.3.c | 0.03% | 0.06% | 0.01% | 99% | 99% | -0.01% | -0.08% | -0.07% | 101% | 100% |
| 2.4.4.a | 0.10% | 0.09% | 0.08% | 99% | 99% | 0.11% | 0.00% | 0.20% | 98% | 98% |
| 2.4.4.b | 0.12% | 0.07% | 0.09% | 99% | 99% | 0.13% | 0.16% | 0.16% | 99% | 99% |
| 2.4.4.c | 0.11% | 0.10% | 0.09% | 99% | 99% | 0.10% | 0.12% | 0.11% | 99% | 99% |
| 2.4.4.d | 0.22% | 0.18% | 0.16% | 98% | 98% | 0.20% | 0.24% | 0.31% | 97% | 99% |
| 2.4.5.a | 0.01% | 0.00% | -0.03% | 100% | 100% | -0.02% | -0.10% | 0.06% | 102% | 103% |
| 2.4.6 | -0.01% | 0.01% | -0.02% | 100% | 100% | 0.02% | -0.03% | 0.02% | 100% | 99% |
| 2.4.7 | 0.02% | 0.02% | 0.03% | 100% | 100% | 0.05% | -0.11% | 0.05% | 100% | 99% |
| 2.4.8 | 0.00% | -0.02% | 0.02% | 100% | 99% | 0.03% | 0.15% | -0.04% | 100% | 103% |

Complexity analysis:

| Test# | Bandwidth for uni/bi-prediction | Affine MVs storage | Others (additional operations for MV clipping or derivation, and so on.) |
| --- | --- | --- | --- |
| Translational model | 6.25 / 8.56 |  |  |
| Affine model | 8.33 / 16.66 |  |  |
| 2.4.1.a | 8.33 / 11.84 | Same | 2 comparisons per CU (1 for width/height + 1 for uni/bi) |
| 2.4.1.b | 8.33 / 11.84 | Same | 1 comparison per CU (1 for uni/bi) |
| 2.4.1.c | 8.33 / 11.84 | Same | 1 comparison per CU (1 for uni/bi) |
| 2.4.2 | 8.33 / 8.56 | Same | MV clipping for subblk MVs |
| 2.4.3.a | 8.33 / 11.84 | Same | 3 comparisons per CU (1 for width/height + 1 for uni/bi + 1 for restriction) |
| 2.4.3.b | 8.33 /8.56 | Same | 1 comparison per CU (1 for uni/bi) |
| 2.4.3.c | 8.33 / 9.53 | Same | 13 comparisons per CU (1 for uni/bi + 12 for restriction) |
| 2.4.4.a | 8.33 / 11.84 | Same | 1 comparison per CU (1 for uni/bi) |
| 2.4.4.b | 8.33/11.84 | Same | 1 comparison per CU (1 for uni/bi) |
| 2.4.4.c | 8.33 /11.84 | Same | 2 comparisons per CU (1 for width/height + 1 for uni/bi) |
| 2.4.4.d | 8.33 / 8.56 | Same | 1 comparison per CU (1 for uni/bi) |
| 2.4.5.a | 4.77 / 9.53 | Same | MV clipping for subblk MVs + Find min every 4 subblk MVs (note: 9x9 luma) 14 comparisons for each 8x8 block |
| 2.4.6 | Same | Same | 4 POC comparisons |
| 2.4.7 | Same | 25% | Same |
| 2.4.8 | Same | 10 bit less per CU ??% |  |

The following aspects are studied in this sub-CE:

1. Reduce worst case memory bandwidth for affine. This can be done either by disallowing 4x4 subblocks (Tests 2.4.1, 2.4.3b, 2.4.4). All these come with some penalty in compression performance (typically 0.1% when 4x8/8x4 SB are used for bi pred, 0.2+% when restriction 8x8 is used). It is noted that these approaches also reduce the number of computations for motion comp (more lines have to be interpolated at boundaries of 4x4 blocks before the vertical interpolation can be performed)

It is also mentioned that possible impact on visual quality might occur when using larger subblocks in afine MC, in particular for sequences with stronger affine motion. This has not been investigated yet

Other approaches (Tests 2.4.2, 2.4.3a/c, 2.4.5) keep 4x4 subblocks and impose some restrictions (e.g. by clipping, grouping, adaptive selection of subblock size based on CPMV), such that adjacent subblocks’ vectors are not too much different, and joint memory access could be made. These come with practically no loss in compression, but may require some additional logic. Keeping 4x4 subblocks and not losing compression performance appears the more desirable concept, but more study is necessary to understand the impact of different approaches how it can be achieved. The following aspects are important:

- Number of operations at decoder (for determining SB MVs and MC)

- More detailed study of worst case memory bandwidth in relation to memory access models, cache size, etc.

- Possibility of formulating by encoder/bitstream restrictions

- Possibility of applying such concepts not only for affine, but also for other small-size CU cases

- Possible impact on subjective quality

2. Reduction of local buffers for storing affine related CPMV inheritance (2.4.7, 2.4.8). 2.2.3.d is targeting the same issue. It appears from CE results (and possibly from CE related contributions) that this can be achieved by almost no loss in compression. However, also here more detailed analysis is necessary in terms of additional logic, and the effective saving in number of bits for local buffers.

Further analysis of the different approaches in BoG (Y. He), also review the CE related proposals, and suggest approaches to be further studied in upcoming CE. It is unquestionable that something is necessary in VVC to restrict the worst case memory bandwidth, but the optimum solution of achieving this may not be identified yet.

3. Sub-Test 2.4.6 enables 6-parameter model inheritance across CTU boundary. However in VTM3 this does not seem to have any benefit.

Sub-CE 2.5: ATMVP related

| Test# | Description | Document# |
| --- | --- | --- |
| 2.5.1 | Simplification of ATMVP, add additional CU size restriction (>64) | JVET-M0165 |
| 2.5.2.a | Withdraw | JVET-M0227 |
| 2.5.2.b | Withdraw |  |
| 2.5.2.c | Insert the new ATMVP into the affine merge candidate list in additional to the original ATMVP for non-low-delay picture with pruning |  |
| 2.5.2.d | Withdraw |  |
| 2.5.2.e | Withdraw |  |
| 2.5.2.f | Withdraw |  |
| 2.5.2.g | Withdraw |  |
| 2.5.2.h | For non-low-delay picture, insert the new ATMVP; for low-delay picture, insert VTM-3.0 ATMVP |  |
| 2.5.3.a | Withdraw |  |
| 2.5.3.b | Withdraw |  |
| 2.5.4 | Non-sub-block ATMVP | JVET-M0086 |

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 2.5.1 | 0.00% | 0.00% | 0.05% | 100% | 100% | 0.00% | -0.04% | 0.08% | 100% | 101% |
| 2.5.2.c | -0.04% | -0.07% | -0.06% | 100% | 101% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| 2.5.2.h | 0.26% | 0.14% | 0.15% | 101% | 101% | 0.00% | 0.00% | 0.00% | 100% | 99% |
| 2.5.4 | 0.19% | 0.28% | 0.26% | 100% | 99% | 0.52% | 0.52% | 0.66% | 101% | 97% |

2.5.2.x is adding something without showing benefit any more on top of VTM3.

For 2.5.4, it is not obvious that the restriction is necessary (e.g., there is no memory bandwidth problem) – it generates loss. Further, there may be some interdependency with subblock boundary deblocking that was added in VTM3, so the subjective issue of ATMVP which might have occurred with VTM2 is no longer relevant.

2.5.1 is a simplification for subblock merge (including affine) by using it only for subblocks where no side is smaller than 8 (which is an existing condition), and the area shall be larger than 64 samples (which disallows 8x8 blocks. There is no loss in performance. However, there is no real problem in implementing 8x8 blocks, so this restriction seems unnecessary.

(include summaries of contribution from JVET-M0022 subsequently)

JVET-M0053 CE2.4.7 Size constrain for inherited affine motion prediction [H. Huang, W.-J. Chien, M. Karczewicz (Qualcomm)]

JVET-M0054 CE2.4.6 Modified affine inheritance from above CTU [H. Huang, W.-J. Chien, M. Karczewicz (Qualcomm)]

JVET-M0104 CE2.3.1: Planar Motion Vector Prediction [N. Zhang, X. Chen, J. Zheng, Y. Lin (HiSilicon)]

JVET-M0125 CE2: History Based Affine Motion Candidate (Test 2.2.3) [J. Zhao, S. Kim (LGE), G. Li, X. Xu, X. Li, S. Liu (Tencent)]

JVET-M0150 CE2.4.3: Affine restriction for worst-case memory bandwidth reduction [L. Pham Van, W.-J. Chien, H. Huang, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-M0664 Crosscheck of JVET-M0150 [S. Jeong, K. Choi (Samsung)] [late]

JVET-M0165 CE2.5.1: Simplification of SbTMVP [C.-C. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-M0226 CE2.4.1: Reducing worst-case memory bandwidth of affine mode [Y.-W. Chen, X. Wang (Kwai Inc.)]

JVET-M0227 CE2.5.2: A second ATMVP candidate [Y.-W. Chen, X. Wang (Kwai Inc.)]

JVET-M0246 CE2: Adaptive Motion Vector Resolution for Affine Inter Mode (Test 2.1.2) [H. Liu, K. Zhang, L. Zhang, J. Xu, Y. Wang, P. Zhao, D. Hong (Bytedance)]

JVET-M0256 CE2: Affine temporal constructed candidates (test 2.2.7) [F. Galpin, A. Robert, F. Le Léannec, T. Poirier (Technicolor)]

JVET-M0699 Crosscheck of JVET-M0256 (CE2: Affine temporal constructed candidates (test 2.2.7)) [H. Huang (Qualcomm)] [late]

JVET-M0262 CE2: Affine model inheritance from single-line motion vectors (Test 2.4.8) [K. Zhang, L. Zhang, H. Liu, J. Xu, Y. Wang, P. Zhao, D. Hong (Bytedance)]

JVET-M0282 CE2: Affine motion predictor pruning (test 2.2.8) [A. Robert, F. Le Léannec, T. Poirier, F. Galpin (Technicolor)]

JVET-M0302 CE2: Merge Mode with Regression-based Motion Vector Field (Test 2.3.3) [R. Ghaznavi-Youvalari, A. Aminlou, J. Lainema (Nokia)]

JVET-M0309 CE2: Memory bandwidth reduction for affine mode (test 2.4.2) [J. Li, R.-L. Liao, C. S. Lim (Panasonic)]

JVET-M0380 CE2: Affine Merge flag coding (Test 2.2.1) [G. Laroche, C. Gisquet, P. Onno, J. Taquet (Canon)]

JVET-M0381 CE2: On Subblock Merge index coding (Test CE2.2.2) [G. Laroche, C. Gisquet, P. Onno, J. Taquet (Canon)]

JVET-M0420 CE2: Adaptive precision for affine MVD coding (Test 2.1.1) [J. Luo, Y. He, X. Xiu (InterDigital)]

JVET-M0431 CE2: Affine merge with prediction offset (Test CE2.2.4) [G. Li, X. Xu, X. Li, S. Liu (Tencent)]

JVET-M0472 CE2: Affine sub-block size restrictions (Test 2.4.4) [H. Chen, T. Solovyev, H. Yang, J. Chen (Huawei)]

JVET-M0476 CE2: Control point MV offset for Affine merge mode (Test 2.2.5) [Y.-C. Yang, Y.-J. Chang (Foxconn)]

JVET-M0477 CE2: Simplification of Affine constructed merge candidates (Test 2.2.6) and supplementary results [Y.-C. Yang, Y.-J. Chang (Foxconn)]

JVET-M0798 Crosscheck of supplementary results of JVET-M0477 (CE2: Simplification of Affine constructed merge candidates (Test 2.2.6) and supplementary results) [H. Liu (Bytedance)] [late]

JVET-M0485 CE2: Sub-block MV clip in planar motion vector prediction (test 2.3.2) [M. Gao, X. Li, M. Xu, S. Liu (Tencent)]

JVET-M0488 CE2: Sub-block MV clip in affine prediction (test 2.4.5) [M. Gao, X. Li, M. Xu, S. Liu (Tencent)]

## CE3: Intra prediction and mode coding (16)

JVET-M0023 CE3: Summary report on intra prediction and mode coding [G. Van der Auwera, J. Heo, A. Filippov]

This contribution was discussed Wednesday 9 January 1750–2030 (GJS)

This is the summary report of the third Core Experiment (CE3). The goal of CE3 is to study intra prediction tools, including mode coding, for potential inclusion into the VVC standard.

The following is the list of defined sub-tests in CE3:

- CE3.1: Intra prediction modes (5 tests)

- CE3.2: Cross-component prediction (11 tests)

- CE3.3: Intra mode coding (10 tests)

This document summarizes the objective results (BD-rates, runtimes), cross-check reports, and related input contributions.

The source codes of the tests and full test results are uploaded by proponents into the following CE3 GitLab repository:

https://vcgit.hhi.fraunhofer.de/JVET-L-CE3/VVCSoftware_VTM.git

The following changes were made to the CE3 test description document after the T2 deadline (December 6, 2018) had passed. Besides changes to contact persons and assignment of cross-checkers, the following changes were requested on the JVET reflector and discussed for clarification, if needed:

- CE3.2:

  - Added test CE3.2.2.1 (MMLM; combination of tests CE3.2.1 and CE3.2.2)

  - Added test CE3.2.6.2 (CCLM; testing 3 columns of neighbouring samples on left side of block, 1 row above block)

- CE3.3:

  - Reduction and clarification of test CE3.3.1 from 7 subtests to 4 (decoder-side intra mode derivation)

  - Additional result for test 3.5 (multiple DM for chroma)

CE3.1 on ‘Intra prediction modes’

| Test # | Description | Doc. # |
| --- | --- | --- |
| 1.1.1 | Intra sub-partitions coding mode (conceptually similar to prior “short-distance intra prediction”) with a different trade-off between gain and encoding run-time (at least 16 samples per partition; 2 or 4 partitions) | JVET-M0102 |
| 1.1.2 | Test 1.1.1 with a restriction: the resulting partitions must have a width of at least 4 samples (but height can be 1, 2, or 4) |  |
| 1.2.1 | Affine linear weighted intra prediction modes with encoder speedup | JVET-M0043 |
| 1.2.2 | Affine linear weighted intra prediction modes with a fixed number of weights needed per NxN block |  |
| 1.3 | Harmonization simplified linear interpolation intra prediction (LIP) with PDPC | JVET-M0252 |

|  | All Intra Main 10 - Over VTM-3.0 |  |  |  |  | Random Access Main 10 - Over VTM-3.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 1.1.1 | -0.59% | -0.44% | -0.47% | 112% | 103% | -0.29% | -0.31% | -0.15% | 102% | 103% |
| 1.1.2 | -0.46% | -0.34% | -0.34% | 112% | 104% | -0.24% | -0.28% | -0.18% | 102% | 102% |
| 1.2.1 | -1.36% | -1.02% | -1.01% | 153% | 105% | -0.85% | -0.92% | -0.98% | 112% | 99% |
| 1.2.2 | -0.95% | -0.42% | -0.46% | 153% | 101% | -0.57% | -0.73% | -0.83% | 110% | 98% |
| 1.3 | -0.08% | -0.14% | -0.11% | 106% | 100% | -0.04% | 0.00% | -0.01% | 101% | 100% |

Regarding 1.1.x, it was commented that this does not really increase decoder complexity. Another participant commented that this has a difficult pipeline dependency. However, other difficult pipline dependencies are already in the VTM, and this is not increasing the difficulty of the worst case.

CE3.1: Related contributions

| Doc. # | Related test # | Title |
| --- | --- | --- |
| JVET-M0426 (HHI) | 1.1.2 | CE3-related: Improvement on the Intra Sub-Partitions Coding Mode |

CE3.1: Additional test results

|  |  | All Intra Main10 - Over VTM-3.0 |  |  |  |  | Random Access Main10 - Over VTM-3.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Description | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 1.1.2.1 | Test 1.1.2 with 1-D transform and entropy coding for 4xN (N>4) blocks that cannot be vertically divided (JVET-M0102) | -0.52% | -0.40% | -0.42% | 112% | 104% | -0.25% | -0.25% | -0.16% | 102% | 102% |

It was noted that 1.1.x shows more gain on Class F (SCC content) than on other content, and Class F is not included in the average. A non-proponent participant focused on implementation issues indicated that they had analyzed it and found it acceptable for implementation.

Decision: Adopt CE3-1.1.1 proposal, pending the provision of text and its review.

This was further discussed in the plenary on Sunday 13 January, and it was agreed to revisit this topic since it has an aspect of reversal of decoding order which may cause it to be difficult to implement. See the notes of the plenary discussion in section 11.1.

Regarding 1.2.1, this has (in the decoder perspective) a selection of a matrix of stored fixed values among a set of such matrices, followed by a matrix multiply applied to boundary sample values to generate the prediction signal in the frequency domain and then an inverse transform is applied to generate a spatial domain prediction, followed by an ordinary residual difference.

It was noted that this has a bit more gain on RA than is usual for intra coding efficiency proposals (0.85/1.36=0.625 versus the usual ~0.5).

This has some decoder runtime increase. For 1.2.1 there is an increase in computational operations.

It was commented that the need to include an inverse transform in the 1.2.1 variant is an additional functional block unlike anything typically done for intra.

The amount of stored coefficient data is another issue, especially for the 1.2.1 variant (~300 kbytes). The 1.2.2 variant omits the inverse transform and has a (simple 2-tap one-dimensional average) downsampling that reduces the size of the matrices (to about ~18 kbytes – a total of around 14,000 numbers of 10 bits each), with a corresponding (bilinear) upsampling in the decoder. The proponent pointed to CPR as an instance where added storage of a greater amount is needed (although, for screen content, that has quite high gain).

The encoding complexity is another significant concern; both proposed variants increase that by ~50%. Additional encoder-only variants are reported in M0043 with different trade-offs.

The proponent said the training set did not include the CTC test set.

Between 1/3 and 1/2 of the intra blocks were reportedly using this mode (which is a lot).

Further study was encouraged (not necessarily in a CE). Side activity was encouraged during the meeting to potentially come up with a plan for further study.

For 1.3, the proposal is to add an additional mode; the measured gain was quite small and the encoder runtime increased by about 6%, so no action was taken on this.

CE3.2 on ‘Cross-component prediction’

| Test # | Description | Doc. # |
| --- | --- | --- |
| 2.1 | CCLM + MDLM + MMLM | JVET-M0097 |
| 2.2 | CCLM + MDLM + MMLM + Above-MMLM + Left-MMLM | JVET-M0475 |
| 2.2.1 | Combination of 2.1 and 2.2 (*) | JVET-M0098 |
| 2.3 | CCLM + MDLM + Adaptive multiple cross-component linear model | JVET-M0504 |
| 2.4 | Modified CCLM downsampling filter for “type-2” content (chroma sampling) | JVET-M0142 |
| 2.5.1 | CCLM + MDLM, using MaxMin method, and β derived by using Cmean and Lmean | JVET-M0401 |
| 2.5.2 | CCLM + MDLM, using classification-based mean value method |  |
| 2.5.3 | CCLM + MDLM, using MaxMin method; using the first top row and the second left column |  |
| 2.5.4 | CCLM + MDLM, using classification-based mean value method; using the first top row and the second left column |  |
| 2.6.1 | CCLM accessing less neighbouring luma samples, interaction with other adopted coding tools: one row above and one column left luma samples are accessed | JVET-M0263 |
| 2.6.2 | 2.6.1 with three columns of left luma samples as in the anchor |  |

The VTM has 3 CCLM modes (CCLM, CCLM-above, and CCLM-left). The selection between these modes is signalled, but the model parameters are not. The tests in the CE are to improve coding efficiency by adding more models.

|  | Test 2.1 | Test 2.2 | Combination test 2.2.1 |
| --- | --- | --- | --- |
| Signalling | Adds MMLM modes in LM symbol list | Uses flag and index to indicate MMLM mode | Same as 2.2 |
| Number of lines used for model derivation | Same as CCLM/MDLM (i.e., Y: 2 lines above, 3 lines left, C: 1 line above and left) | (Y: 4 lines above, 5 lines left, C: 2 lines above and left) | Same as CCLM/MDLM (i.e., Y: 2 lines above, 3 lines left, C: 1 line above and left) |

- 2.1 uses 3 columns to the left and 2 lines above (except at the CTU boundary)

- 2.2 uses 5 columns to the left and 4 lines above (except at the CTU boundary)

- Between 2.1, 2.2, and 2.2.1, it was suggested to focus on the combination test 2.2.1, for using fewer lines and cleaner signalling

|  | All Intra Main10 - Over VTM-3.0 |  |  |  |  | Random Access Main10 - Over VTM-3.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 2.1 | -0.25% | -1.71% | -2.19% | 100% | 100% | -0.12% | -1.36% | -1.70% | 99% | 99% |
| 2.2 | -0.29% | -1.94% | -2.51% | 100% | 100% | -0.15% | -1.70% | -1.99% | 101% | 100% |
| 2.2.1 | -0.27% | -1.71% | -2.26% | 99% | 99% | -0.13% | -1.30% | -1.73% | 99% | 98% |
| 2.3 | -0.10% | -0.86% | -0.88% | 102% | 100% | -0.03% | -0.62% | -0.66% | 100% | 100% |
| 2.4 | See below (*) |  |  |  |  |  |  |  |  |  |
| 2.5.1 | -0.09% | -0.44% | -0.47% | 100% | 99% | -0.08% | -0.31% | -0.39% | 100% | 99% |
| 2.5.2 | -0.18% | -0.88% | -0.90% | 100% | 99% | -0.11% | -0.74% | -0.84% | 100% | 99% |
| 2.5.3 | 0.05% | 0.45% | 0.48% | 100% | 100% | 0.02% | 0.51% | 0.51% | 100% | 99% |
| 2.5.4 | -0.14% | -0.51% | -0.51% | 100% | 101% | -0.09% | -0.41% | -0.46% | 100% | 99% |
| 2.6.1 | 0.09% | 0.71% | 0.79% | 99% | 97% | 0.06% | 0.88% | 0.95% | 100% | 99% |
| 2.6.2 | 0.03% | 0.16% | 0.21% | 100% | 97% | 0.00% | 0.35% | 0.35% | 100% | 99% |

(*) Summary of test 2.4:

|  |  | All Intra – Over VTM3.0 WCG_EXT=1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Description | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | PsnrY | PsnrU | PsnrV | EncT | DecT |
| 2.4.a | LM determination: 3-tap filter; prediction: 3-tap filter | -2.5% | -0.2% | -0.2% | -4.2% | -8.6% | -0.2% | -3.4% | -6.7% | 101% | 101% |
| 2.4.b | LM determination: 3-tap filter; prediction: 5-tap filter | -2.2% | -0.2% | -0.2% | -4.7% | -8.6% | -0.2% | -4.0% | -6.8% | 101% | 102% |
| 2.4.c | LM determination: 5-tap filter; prediction: 5-tap filter | -2.6% | -0.3% | -0.2% | -5.5% | -9.6% | -0.2% | -4.6% | -7.6% | 99% | 101% |

|  |  | Random Access – Over VTM3.0 WCG_EXT=1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Description | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV | PsnrY | PsnrU | PsnrV | EncT | DecT |
| 2.4.a | LM determination: 3-tap filter; prediction: 3-tap filter | -1.9% | -0.2% | -0.1% | -4.2% | -5.8% | -0.1% | -3.3% | -4.5% | 101% | 99% |
| 2.4.b | LM determination: 3-tap filter; prediction: 5-tap filter | -1.7% | -0.1% | -0.1% | -4.7% | -6.0% | -0.1% | -3.7% | -4.6% | 101% | 100% |
| 2.4.c | LM determination: 5-tap filter; prediction: 5-tap filter | -2.1% | -0.2% | -0.2% | -5.1% | -6.6% | -0.2% | -4.1% | -5.1% | 102% | 101% |

Some comments:

- Chroma gain tends to roughly translate to luma gain at about a 1:10 ratio

- Chroma gain in the CTC is often dominated by one particular test sequence: CampFire

- The MDLM scheme adopted at the previous meeting provided about 2.7% chroma gain

- The proposed MMLM schemes double the number of models derived and used.

Notes of the previous meeting: “MMLM (and its add-ons MNLM, MFLM) need to determine two models. Whereas the number of samples that is used to compute the models is the same in total, it cannot be foreseen how many samples fall into which class. Therefore, it is more difficult for pipeline processing than CCLM. The classification step, though it is a simple averaging criterion, also may impose some additional pipelining issues. Gain of MMLM standalone is 0.3% luma, approx. 2.5% for chroma [for AI; RA: 0.1% luma, approx. 2.0% for chroma – for 5.2.3.4 scheme]. It is recommended to further study whether the complexity concerns are less valid in combination with the LM computation of JVET-L0191, and whether the gain would still be preserved. It should also be investigated if MMLM and CCLM can use same building blocks.”

In response to the above notes:

- The gain is somewhat less now: AI: -0.27%, -1.71%, -2.26%, RA: -0.13%, -1.30%, -1.73%

- The complexity concern (e.g., latency added by determining the threshold and applying the classification step; although the cross-component prediction may not be in a critical-latency path) remains valid.

- It was remarked that even with the current scheme, the complexity is a problem, esp. for small blocks (2x2 chroma).

Due to complexity concerns, no action was taken on 2.1/2.2/2.2.1. This will be further discussed in the CE-related BoG to clarify the concerns and recommend what to further study.

2.3 has an adaptive number of models, sometimes more than two per component, and showed less gain, so no action was taken on that.

Regarding 2.4.x, the current scheme is optimized for type 0 chroma (the usual chroma per Rec. 709); 2.4 compensates for co-sited chroma (type 2 per BT.2100). The 2.4 test was done only on the PQ sequences. A decoder might need to support two chroma types, with encoder high-level selection of which to apply.

Testing was not done to determine the potential penalty of running the type 2 optimized scheme on type 0 content. Test results for this were requested. Revisit for that.

A participant remarked that he had done some testing on self-generated content that used type 1 (centered both horizontally and vertically) and found not so much (~1-2%) benefit for customizing for that chroma siting. Here the experiments show a much more substantial gain for optimization to type 2.

It was commented that proper chroma alignment may be more visually important than what shows up in objective metrics.

The kernel of the 2.4.c 5-tap filter (except at CTU boundaries, where only one line above is used) is:

0 1 0

1 4 1

0 1 0

Decision: Adopt 2.4.c with a high-level flag to switch between two chroma format type optimizations (pending test results for applying the type 2 scheme to type 0 content).

For 2.5.x, there was a focus on the first two variants. These are adding some additional complexity. The gain seemed insuffient to justify changing the current scheme.

2.6.1 reduces the number of neighbour lines used to compute the model parameters to 1 (where the current VTM scheme uses 3 columns to the left and 2 rows above when within the current CTU). 2.6.2 uses 3 columns to the left but only one row above. These would make the processing consistent whether within a CTU or at the boundary.

But it was commented that adoption of the 2.6.1 or 2.6.2 scheme would not be reducing the line buffering needed in a decoder, since it does not apply for the other chroma type. It was also commented that the question of 1 or 2 lines had been discussed at the previous meeting and the 2 line scheme was considered not a problem. So no action was taken on the 2.6.x proposals.

CE3.3 on ‘Intra mode coding’

| Test # | Description | Doc. # |
| --- | --- | --- |
| 3.1.1 | Decoder-side intra mode derivation | JVET-M0094 |
| 3.1.2 | Decoder-side intra mode selection with predictor coding (3 candidates) |  |
| 3.1.3 | Decoder-side intra mode derivation with extended gradient filtering for directly neighboring pixels |  |
| 3.1.4 | Decoder-side intra mode derivation restricted for blocks with a number of samples <= 128 |  |
| 3.2 | 6 MPMs and 32 remaining modes (5-bit FLC) | JVET-M0495 |
| 3.3.1 | Simplified MDMS with 5 chroma intra modes (2-point DM, 5 non-LM) | JVET-M0218 |
| 3.3.2 | Simplified MDMS with the reduced chroma intra modes (2-point DM, 3 non-LM) |  |
| 3.4.1 | Simplified MDMS by reducing the number of operations (2-point DM, 5 non-LM) | JVET-M0503 |
| 3.4.2 | Simplified MDMS by reducing the number of candidates (2-point DM, 3 non-LM) |  |
| 3.5 | Simplified MDMS with one DM (1-point DM, 5 non-LM) | JVET-M0203 |

| Test # | All Intra Main10 - Over VTM-3.0 |  |  |  |  | Random Access Main10 - Over VTM-3.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 3.1.1 | -0.11% | -0.09% | -0.08% | 122% | 103% | -0.07% | -0.06% | -0.07% | 104% | 101% |
| 3.1.2 | -0.01% | -0.05% | -0.02% | 122% | 102% | 0.01% | 0.06% | 0.04% | 103% | 100% |
| 3.1.3 | -0.12% | -0.04% | -0.05% | 124% | 104% | -0.05% | -0.01% | 0.03% | 103% | 101% |
| 3.1.4 | -0.09% | -0.09% | -0.06% | 113% | 103% | -0.05% | 0.10% | 0.01% | 100% | 100% |
| 3.2 | -0.03% | -0.04% | -0.05% | 97% | 100% | 0.02% | 0.14% | 0.13% | 99% | 99% |
| 3.3.1 | -0.03% | -0.88% | -0.86% | 100% | 100% | -0.01% | -0.46% | -0.50% | 100% | 100% |
| 3.3.2 | -0.02% | -0.29% | -0.20% | 97% | 100% | 0.00% | -0.12% | -0.05% | 99% | 100% |
| 3.4.1 | -0.02% | -0.96% | -1.03% | 101% | 99% | 0.04% | -0.98% | -1.04% | 100% | 100% |
| 3.4.2 | 0.03% | -0.34% | -0.32% | 97% | 100% | 0.10% | -0.86% | -0.92% | 99% | 100% |
| 3.5 | -0.01% | -0.61% | -0.64% | 100% | 100% | 0.06% | -0.68% | -0.76% | 100% | 101% |

CE3.3: Additional test results

| Test # | Description | All Intra Main10 - Over VTM-3.0 |  |  |  |  | Random Access Main10 - Over VTM-3.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 3.5.1 | Simplified MDMS with one DM by using aligned chroma coding of VVC-3.0 (1-point DM, 4 non-LM) | -0.02% | -0.58% | -0.61% | 100% | 100% | -0.02% | -0.37% | -0.39% | 100% | 100% |

For 3.1.x, the proposal is to add another mode in which the intra prediction mode is inferred by decoder processing rather than signalled. This adds encoder (and some decoder) complexity, but the test results do not show much benefit from this, so no action was taken on this.

For 3.2, the proposal is to restrict the number of selectable intra prediction modes to 32 of the 67. Some gain is observed, but only a small amount, and it was commented that the restriction of what modes an encoder would be allowed to select would restrict encoder freedom on how to make its mode decisions (not allowing an encoder to choose a mode until after it is able to determine which 32 of the 67 are allowed for selection), so no action was taken on this.

3.3.1, 3.3.2, 3.4.1, 3.4.2, and 3.5 are regarding chroma mode coding, which is dependent on luma mode. 3.3.1 and 3.4.1 would increase decoder complexity. One key aspect is how many luma points are considered for deriving the chroma mode candidates (1 or 2). Another key aspect is how many chroma mode candidates there are (3 or 5).

Schemes 3.3.2 and 3.4.2 reduce the number of candidates from 5 to 3. As tested, this reduced encoding time since the encoder checked fewer modes, although the decoder complexity is higher than for the current scheme (because it uses a 2-point check for direct mode selection). It was commented that the DC, planar, horizontal and vertical modes are especially important for some encoder implementations. If these are not always selectable, it would force a dependency between luma and chroma for encoding decisions. These two schemes did not provide much coding gain, although in the way they were tested, they reduced encoding time. The lack of significant coding gain, together with that dependency, did not appear to justify action on those.

Results for an additional scheme called 3.5.1 (proposed in JVET-M0203) were included in the CE report. This was a late addition that was not in the CE plan, so it was considered a non-CE proposal.

3.3.1 and 3.4.1 check two luma locations, whereas the VTM checks only 1 (the central position of the luma block). The VTM sends a flag on whether to use that mode; if not, it sends a CCLM mode flag; if not, it sends 2 bypass-coded bins to select between four modes. If the luma mode was not DC, planar, horizontal or vertical, then those are the four modes; otherwise the luma mode is replaced with the vertical diagonal mode to determine the four modes.

3.3.1 and 3.4.1 check two luma locations and perform some comparison flowchart operations to determine what is the primary selectable mode and what are the other four modes. The DC and planar modes are always among the 5 selectable modes. It was noted that this forces a dependency between the luma and chroma mode decisions unless the encoder only used DC and planar modes for chroma.

The possibility of supporting both the current scheme and the alternative was discussed. The gain seemed insufficient to want to need two different ways to be supported in the decoder.

3.5 checks one luma location (same as VTM); if the luma mode is DC or planar and the block shape is vertical, then instead of the vertical diagonal mode being considered special, the horizontal diagonal mode is considered special; if the luma mode is angular, the other selectable modes are determined by a flowchart (but the horizontal and vertical modes are not always available). This has basically the same forced cross-component dependency as 3.3.1 and 3.4.1.

Since the gain is relatively small and the 3.3.1, 3.3.2, 3.4.1, 3.4.2, and 3.5 proposals introduce an undesirable cross-component dependency for encoders, no action was taken on on these.

JVET-M0043 CE3: Affine linear weighted intra prediction (test 1.2.1, test 1.2.2) [J. Pfaff, B. Stallenberger, M. Schäfer, P. Merkle, P. Helle, R. Rischke, H. Schwarz, D. Marpe, T. Wiegand (HHI)]

JVET-M0094 CE3: Decoder-side Intra Mode Derivation (tests 3.1.1, 3.1.2, 3.1.3 and 3.1.4) [E. Mora, A. Nasrallah, M. Abdoli, M. Raulet (ATEME)]

JVET-M0097 CE3: On MMLM (Test 2.1) [A. K. Ramasubramonian, G. Van der Auwera, T. Hsieh, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-M0098 CE3: Joint test on MMLM (Test 2.2.1) [A. K. Ramasubramonian, G. Van der Auwera, T. Hsieh, V. Seregin, M. Karczewicz (Qualcomm), H.-J. Jhu, Y.-J. Chang (Foxconn)]

JVET-M0102 CE3: Intra Sub-Partitions Coding Mode (Tests 1.1.1 and 1.1.2) [S. De-Luxán-Hernández, V. George, J. Ma, T. Nguyen, H. Schwarz, D. Marpe, T. Wiegand (HHI)]

JVET-M0142 CE3: Modified CCLM downsampling filter for “type 2” content (Test 2.4) [P. Hanhart, Y. He (InterDigital)]

JVET-M0203 CE3: DM-based chroma intra prediction mode (Test 3.5) [N. Choi, M. W. Park, K. Choi (Samsung)]

JVET-M0218 CE3: Simplified MDMS (test 3.3.1 and test 3.3.2) [J. Choi, J. Heo, S. Yoo, L. Li, J. Choi, J. Lim, S. Kim (LGE)]

JVET-M0252 CE3-1.3: Harmonization of Linear interpolation intra prediction (LIP) with Multiple reference line prediction (MRL) [J. Heo, J. Choi, J. Choi, S. Yoo, L. Li, J. Lim (LGE)]

JVET-M0263 CE3: CCLM prediction with single-line neighbouring luma samples (Test 2.6.1 and Test 2.6.2) [K. Zhang, L. Zhang, H. Liu, J. Xu, Y. Wang, P. Zhao, D. Hong (Bytedance)]

JVET-M0401 CE3: Classification-based mean value for CCLM coefficients derivation (tests 2.5.1-2.5.4) [X. Ma, A. Filippov, V. Rufitskiy, H. Yang, J. Chen (Huawei)]

JVET-M0475 CE3: Multiple neighbour LM (Test 3.2.2) [H.-J. Jhu, Y.-J. Chang (Foxconn)]

JVET-M0495 CE3: Intra mode coding (Test 3.2) [L. Zhao, X. Zhao, X. Li, S. Liu (Tencent)]

JVET-M0503 CE3: Chroma intra prediction simplification (Test 3.4.1 and 3.4.2) [C.-H. Yau, C.-C. Lin, C.-L. Lin (ITRI)]

JVET-M0504 CE3: adaptive multiple cross-component linear model(Test 3.2.3) [S.-P. Wang, C.-H. Yau, C.-C. Lin, C.-L. Lin (ITRI)]

## CE4: Inter prediction and motion vector coding (18)

Contributions in this category were discussed Thursday 10 Jan. 1120–1340 and 1500-2000 (Track B chaired by JRO).

JVET-M0024 CE4: Summary report on inter prediction and motion vector coding [H. Yang, S. Liu, K. Zhang]

This contribution provides a summary report of Core Experiment 4 on inter prediction and motion vector coding. CE4 comprises 5 categories,

- Merge mode simplification

- Merge mode enhancement

- Parallel processing for merge mode

- Motion vector coding

- Motion compensation constraints for complexity reduction

All techniques are implemented on top of and test against VTM 3.0. Simulation results and crosschecking reports of each test specified in this document are provided.

CE4.1: Merge mode simplification

| Test# | Source | Description |
| --- | --- | --- |
| 4.1.1.a | JVET-M0124 | Reduce HMVP candidates from 5 to 5 - NST (NST: number of spatial/temporal candidates) |
| 4.1.1.b | JVET-M0124 | Reduce HMVP candidates from 5 to 6 - NST |
| 4.1.1.c | JVET-M0124 | Apply 4.1.1.a when NST >=3 |
| 4.1.1.d | JVET-M0124 | Apply 4.1.1.b when NST >=3 |
| 4.1.2.a | JVET-M0126 | 1) HMVP buffer size is increased from 6 to 10, with no pruning to update the buffer. 2) One out of every 3 is picked to add in the merge list. 4 HMVP candidates at most 3) Pairwise candidates are generated not using HMVP candidates. 4) First 3 HMVP candidates are pruned to the left and above spatial candidates |
| 4.1.2.b | JVET-M0126 | 4.1.2.a 1) 2) 3) + 4) First 2 HMVP candidates are pruned to the left and above spatial candidates |
| 4.1.5.a | JVET-M0281 | Motion vector pruning: rounding before any MV pruning |
| 4.1.5.b | JVET-M0281 | In merge apply some pruning on combined HEVC candidates, and/or pairwise averaged candidates |
| 4.1.5.c | JVET-M0281 | 4.1.5.a + 4.1.5.b |

Regarding JVET-M0126, only aspects 1) and 2) were in the original proposal (JVET-L0401), whereas aspects 3) and 4) were included in the first release of the software. It is suggested that the proponents also provide separate results which show the benefit of combination 1/2, as well as 3-only and 4-only separately.

Revisit after availability of such results.

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 4.1.1.a | 0.00% | 0.07% | 0.02% | 100% | 100% | 0.07% | 0.14% | 0.22% | 101% | 99% |
| 4.1.1.b | 0.00% | 0.02% | 0.06% | 100% | 101% | 0.02% | 0.01% | 0.00% | 101% | 100% |
| 4.1.1.c | -0.04% | 0.00% | -0.01% | 100% | 100% | 0.05% | 0.06% | 0.00% | 101% | 99% |
| 4.1.1.d | -0.02% | 0.00% | 0.01% | 100% | 99% | 0.03% | -0.04% | -0.02% | 100% | 100% |
| 4.1.2.a | 0.03% | 0.06% | 0.03% | 100% | 100% | 0.08% | 0.07% | 0.13% | 100% | 99% |
| 4.1.2.b | 0.03% | 0.02% | 0.04% | 100% | 100% | 0.08% | -0.03% | 0.08% | 99% | 98% |
| 4.1.5.a | 0.00% | -0.01% | 0.00% | 100% | 100% | -0.01% | -0.04% | -0.01% | 100% | 100% |
| 4.1.5.b | -0.02% | 0.00% | 0.00% | 100% | 100% | 0.02% | 0.09% | -0.09% | 99% | 100% |
| 4.1.5.c | -0.02% | -0.03% | -0.01% | 100% | 100% | -0.02% | 0.06% | 0.07% | 99% | 100% |

| Test# | Difference in the number of candidate comparison (+/- xx) | Max number of pruning stages | Others | RA | LB |
| --- | --- | --- | --- | --- | --- |
| 4.1.1.a | -4 | unchanged |  | 0.00% | 0.07% |
| 4.1.1.b | -2 | unchanged |  | 0.00% | 0.02% |
| 4.1.1.c | -3 | unchanged |  | -0.04% | 0.05% |
| 4.1.1.d | -2 | unchanged |  | -0.02% | 0.03% |
| 4.1.2.a | -8 | -2 | HMVP table = 10 | 0.03% | 0.08% |
| 4.1.2.b | -10 | -3 | HMVP table=10, runtime saving of HMVP functions is 59%(RA), 62%(LB) | 0.03% | 0.08% |
| 4.1.5.a | +0 | +0 |  | 0.00% | -0.01% |
| 4.1.5.b | +6 | +1 | Discard pairwise calculation | -0.02% | 0.02% |
| 4.1.5.c | +6 | +1 | Discard pairwise calculation | -0.02% | -0.02% |

The table above should be updated to get better understanding of the properties. 4.1.1 and 4.1.2 are modifying HMVP. In particular, the number of maximum pruning operations and comparison operations necessary during table construction and during merge should be listed separately. Number of cycles should also be calculated rather than number of pruning stages. It is reported that 4.1.2 is not doing any pruning during table construction (using FIFO list, but with extended length from 5 to 10) and also reduces the number of pruning operations in merge list construction. 4.1.1. is keeping the HMVP list construction unchanged, but reduces the pruning operations in merge list construction.

Revisit after more analysis is available (to be done by CE proponents).

4.1.5a is performing rounding of candidates before the AMVP list construction. Currently, it is done before AMVP list construction for AMVP 1 pel and 4 pel accuracy, and from 1/16 pel to 1/4 pel after merge list construction for AMVP 1/4 pel accuracy. This makes the design more consistent. It is noted that alternatively consistency could be achieved by doing rounding after merge list construction for 1 pel and 4 pel accuracy, however this was reported to have been tried and gave loss, as candidates which were identical after rounding would have been different before rounding. Some support and no objection was raised on this approach. Decision: Adopt JVET-M0281 (subtest 4.1.5a)

4.1.5b adds another pruning stage in merge list construction for 0.02% coding gain. (before VTM3, there was higher gain on such an approach)

CE4.2: Merge mode enhancement

| Test# | Source | Description |
| --- | --- | --- |
| 4.2.1.a | JVET-M0059 | +One non-scaling STMVP: Averaging the first two spatial candidates and TMVP |
| 4.2.2.a | JVET-M0106 | +One non-scaling STMVP: STMVP using a single adjacent above candidate, a single adjacent left candidate and a TMVP candidate. The single adjacent candidate is the first valid one checked at three potential positions. |
| 4.2.2.b | JVET-M0106 | 4.2.2.a with merge list size = 8 |
| 4.2.2.c | JVET-M0106 | 4.2.2.a + One more STMVP candidate (two spatial average) + Remove pair-wise candidate |
| 4.2.2.d | JVET-M0106 | 4.2.2.c with merge list size = 8 |
| 4.2.3.a | JVET-M0221 | +One non-scaling STMVP: Averaging the left spatial, above spatial and TMVP |
| 4.2.4.a | JVET-M0404 | +One non-scaling STMVP: Averaging the first two HMVP candidates and TMVP |
| 4.2.5.a | JVET-M0291 | MMVD modifications: 1) increase base candidates from 2 to 3; 2) Decrease distance index from 8 to 4. |
| 4.2.5.b | JVET-M0291 | MMVD modifications: 1) increase directions from 4 to 8; 2) Decrease distance index from 8 to 4. |

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 4.2.1.a | -0.09% | -0.04% | -0.06% | 100% | 100% | 0.00% | 0.00% | 0.00% | 100% | 102% |
| 4.2.2.a | -0.11% | -0.03% | 0.00% | 101% | 99% | 0.00% | -0.12% | 0.02% | 107% | 110% |
| 4.2.2.b | -0.17% | -0.10% | -0.09% | 101% | 100% | -0.06% | -0.14% | -0.25% | 107% | 110% |
| 4.2.2.c | -0.15% | -0.10% | -0.13% | 100% | 100% | 0.00% | -0.18% | 0.01% | 107% | 110% |
| 4.2.2.d | -0.23% | -0.15% | -0.13% | 101% | 100% | -0.06% | -0.16% | -0.11% | 107% | 111% |
| 4.2.3.a | 0.00% | 0.00% | -0.01% | 101% | 101% | 0.11% | 0.18% | 0.29% | 100% | 101% |
| 4.2.4.a | -0.03% | 0.05% | 0.00% | 108% | 106% | 0.16% | 0.28% | 0.13% | 110% | 109% |
| 4.2.5.a | 0.07% | 0.17% | 0.14% | 100% | 95% | 0.03% | 0.13% | 0.24% | 100% | 97% |
| 4.2.5.b | 0.02% | 0.17% | 0.14% | 100% | 97% | -0.07% | 0.10% | 0.10% | 100% | 94% |

Current VTM3: Merge list size 6, Max number of potential candidates 18, Max number of candidate comparisons 15, Max number of pruning stages 8-9(?)

| Test# | Merge list size | Max number of potential candidates | Max number of candidate comparison | Max number of pruning stages | Max number of MV scaling | Others | RA | LB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4.2.1.a | 6 | +1 | 0 | 0 | 0 |  | -0.09% | 0.00% |
| 4.2.2.a | 6 | +1 | +4 | +1 | +0 |  | -0.11% | 0.00% |
| 4.2.2.b | 8 | +1 | +4 | +1 | +0 |  | -0.17% | -0.06% |
| 4.2.2.c | 6 | +2 | +9 | +2 | +0 |  | -0.15% | 0.00% |
| 4.2.2.d | 8 | +2 | +9 | +2 | +0 |  | -0.23% | -0.06% |
| 4.2.3.a | 6 |  |  |  |  |  | 0.00% | 0.11% |
| 4.2.4.a | 6 |  |  |  |  |  | -0.03% | 0.16% |
| 4.2.5.a | 6 |  |  |  |  |  | 0.07% | 0.03% |
| 4.2.5.b | 6 |  |  |  |  |  | 0.02% | -0.07% |

4.2.1 through 4.2.4 relate to STMVP, 4.2.5 relates to MMVD. All proposals target providing compression gain by doing some additional processing. 4.2.3 and 4.2.4 have not provided complexity data so far (by 2nd day of meeting), but also do not seem to provide advantage in compression.

From the results above, better results are obviously possible with more complexity.

It was agreed that the merge list size should not be increased. Therefore, only 4.2.1a, 4.2.2a/c are reasonable proposals. Among these, 4.2.1a has the best tradeoff performance/complexity. It is however mentioned that the method was disabled for LB test, because it would have provided small loss (due to missing scaling). It would be an undesirable design to change the list of candidates depending on the frame dependencies.

In 4.2.1a, the additional STMVP candidate is put into the list at the position before the B2 (and therefore also before pairwise and HMVP candidates). Furthermore, there are some additional checks in the algorithm that are not reflected in the table above.

After all, none of the proposals provides sufficient benefit to take action.

4.2.5 should better be considered in context of CE4.4 (MV coding)

CE4.3: Parallel processing for merge mode

| Test# | Source | Description |
| --- | --- | --- |
| 4.3.1.a | JVET-M0170 | Share merge list for translational merge and sub-CU merge with fixed threshold = 32 |
| 4.3.1.b | JVET-M0170 | Share merge list for translational merge and sub-CU merge with fixed threshold = 64 |
| 4.3.1.c | JVET-M0170 | Share merge list for translational merge and sub-CU merge with fixed threshold = 128 |
| 4.3.1.d | JVET-M0170 | Share merge list for translational merge and sub-CU merge with fixed threshold = 256 |
| 4.3.1.e | JVET-M0170 | Share merge list for translational merge and sub-CU merge with fixed threshold = 512 |
| 4.3.2.a | JVET-M0289 | S1: A neighbor coding block is marked as unavailable if its bottom right coordinate falls in the extended merge estimation region of the current block. Fixed square MER = 8x8 |
| 4.3.2.b | JVET-M0289 | S1. Fixed square MER = 16x16 |
| 4.3.2.c | JVET-M0289 | S1. Fixed square MER = 32x32 |
| 4.3.2.d | JVET-M0289 | S1. Fixed square MER = 64x64 |
| 4.3.2.e | JVET-M0289 | S2: A neighbor coding block is marked as unavailable if its bottom right coordinate falls in the estimation region of the current block and the QT depth of the current block is equal to or greater than the value of CTU size divided by MER size. Fixed square MER = 8x8 |
| 4.3.2.f | JVET-M0289 | S2. Fixed square MER = 16x16 |
| 4.3.2.g | JVET-M0289 | S2. Fixed square MER = 32x32 |
| 4.3.2.h | JVET-M0289 | S2. Fixed square MER = 64x64 |
| 4.3.2.i | JVET-M0289 | S3: A neighbor coding block is marked as unavailable if its bottom right coordinate falls in the rectangular merge estimation region of the current block. Rectangular MER = 32 |
| 4.3.2.j | JVET-M0289 | S3: Rectangular MER = 64 |
| 4.3.2.k | JVET-M0289 | S3: Rectangular MER = 128 |
| 4.3.2.l | JVET-M0289 | S3: Rectangular MER = 256 |

|  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Thrs/MER | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 4.3.1.a | 32 | 0.00% | 0.00% | 0.00% | 101% | 102% | 0.02% | -0.05% | -0.04% | 101% | 102% |
| 4.3.1.b | 64 | 0.04% | 0.02% | 0.05% | 101% | 104% | 0.08% | -0.04% | 0.15% | 101% | 102% |
| 4.3.1.c | 128 | 0.12% | 0.18% | 0.16% | 101% | 102% | 0.14% | 0.22% | 0.28% | 102% | 102% |
| 4.3.1.d | 256 | 0.44% | 0.52% | 0.66% | 102% | 103% | 0.63% | 0.68% | 0.69% | 103% | 102% |
| 4.3.1.e | 512 | 0.68% | 0.85% | 0.92% | 102% | 102% | 0.93% | 0.86% | 1.09% | 103% | 101% |
| 4.3.2.a | 8x8 (S1) | 0.16% | 0.27% | 0.25% | 100% | 101% | 0.14% | 0.13% | 0.36% | 100% | 101% |
| 4.3.2.b | 16x16 (S1) | 0.55% | 0.67% | 0.78% | 101% | 101% | 0.56% | 0.66% | 0.70% | 101% | 102% |
| 4.3.2.c | 32x32 (S1) | 1.41% | 1.64% | 1.69% | 104% | 100% | 1.41% | 1.38% | 1.54% | 103% | 100% |
| 4.3.2.d | 64x64 (S1) | 2.62% | 2.72% | 2.97% | 106% | 100% | 2.51% | 2.56% | 2.61% | 104% | 100% |
| 4.3.2.e | 8x8 (S2) | 0.02% | 0.01% | 0.03% | 99% | 100% | 0.01% | 0.08% | 0.04% | 100% | 101% |
| 4.3.2.f | 16x16 (S2) | 0.16% | 0.24% | 0.26% | 100% | 99% | 0.16% | 0.11% | 0.18% | 101% | 101% |
| 4.3.2.g | 32x32 (S2) | 1.09% | 1.27% | 1.44% | 102% | 99% | 1.13% | 1.20% | 1.20% | 102% | 100% |
| 4.3.2.h | 64x64 (S2) | 2.59% | 2.66% | 2.92% | 105% | 98% | 2.44% | 2.45% | 2.51% | 103% | 96% |
| 4.3.2.i | 32 (S3) | 0.00% | 0.07% | 0.04% | 100% | 101% | 0.00% | 0.04% | 0.04% | 101% | 104% |
| 4.3.2.j | 64 (S3) | 0.03% | 0.06% | 0.06% | 101% | 102% | 0.01% | 0.04% | -0.17% | 101% | 105% |
| 4.3.2.k | 128 (S3) | 0.15% | 0.20% | 0.27% | 101% | 102% | 0.15% | 0.22% | 0.06% | 102% | 104% |
| 4.3.2.l | 256 (S3) | 0.35% | 0.47% | 0.55% | 102% | 101% | 0.37% | 0.31% | 0.50% | 102% | 104% |

Similar concepts (shared merge of JVET-M0170, parallel merge of JVET-M0289) have been used in HEVC, where parallel is regarded mostly beneficial for parallel processing at the encoder, whereas shared merge could also have some benefit for the decoder, as it is not necessary to generate merge lists separately for very small blocks (however only if the sharing would be made mandatory, which is not the case in HEVC). However, due to the fact that VVC has more irregular (non-square) block structures, consistent definition of regions which use shared merge / parallel merge becomes more difficult. For example, if the regions are square, it may happen that a rectangular block is only partially included. 4.3.1 and 4.3.2 S3 try to solve that by introducing non-square regions below some parent node of the tree.

Both approaches require normative changes. Making the decoder more complex for the benefit of a parallel encoder may be not as desirable as it was in HEVC, in particular as it is more irregular due to non-square blocks. Reducing decoder complexity by sharing merge lists between very small blocks (e.g. two adjacent 4x4 blocks use same merge seems relative simple to define and would be beneficial if it is normative. Results from test 4.3.1a above are from a version called “type1” in JVET-M0170, whereas “type2” (that is also described in JVET-M0170) with threshold 32 would be the desirable solution. The type2 is understood in a way that if after a split any block is smaller than the threshold, all blocks of that split share the same merge list (e.g. 16x4 split ternary or 8x8 with quad split). This needs to be mandatory. Reportedly, this comes with no loss for luma, and some negligible loss (0.03/0.06%) for chroma. This is being cross-checked in JVET-M0584 (“supplementary test D1”) which is not finished yet.

Revisit after the cross-check is finished, cross-checkers are asked to investigate the code and associated specification. The specification text should be simplified such that the threshold is not signalled, but always used as 32. X.Li is also asked to inspect the code and text.

Further study on the aspects that simplify encoder, in particular also by investigating the potential of non-normative solutions. It is mentioned that by the time when parallel merge was introduced in HEVC, a corresponding non-normative solution would have lost about 2% in compression performance.

CE4.4: Motion vector coding

| Test# | Source | Description |
| --- | --- | --- |
| 4.4.1.a | JVET-M0403 | MVD is not signaled as x/y components but as layer and index: Two layer-groups. |
| 4.4.1.b | JVET-M0403 | MVD is not signaled as x/y components but as layer and index: Four layer-groups. |
| 4.4.3 | JVET-M0481 | Symmetrical MVD mode: BiDirPredFlag, RefIdxSymL0 and RefIdxSymL1 are derived at slice level; Only mvp_l0_flag, mvp_l1_flag and MVD0 are explicitly signaled; MVD1=-MVD0. |
| 4.4.4.a | JVET-M0060 | Diagonal direction candidate for MMVD: Increase MMVD directions from 4 to 8. |
| 4.4.4.b | JVET-M0060 | Multiple distance lists for MMVD: Two distance lists, each of which has 4 entries. |
| 4.4.4.c | JVET-M0060 | 4.4.4.a + 4.4.4.b |
| 4.4.4.d | JVET-M0060 | 4.4.4.c+ full-pel position for large range: MV is rounded to integer if the second list is selected and distance index > 1 |
| 4.4.4.e | JVET-M0061 | Combination of 4.4.4.a and 4.4.5.c |
| 4.4.5.a | JVET-M0312 | MMVD with adaptive distance table based on occurrences: The distance table is updated at slice level according to a gain-comparison algorithm. |
| 4.4.5.b | JVET-M0312 | MMVD with adaptive distance table based on picture resolution: A special distance table is used on 4K-sequences. |
| 4.4.5.c | JVET-M0312 | 4.4.5.a + 4.4.5.b |

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 4.4.1.a | 0.01% | 0.02% | 0.02% | 96% | 96% | 0.03% | -0.09% | -0.11% | 97% | 92% |
| 4.4.1.b | 0.00% | 0.04% | 0.02% | 96% | 96% | -0.01% | 0.01% | -0.02% | 97% | 92% |
| 4.4.3 | -0.16% | -0.12% | -0.16% | 105% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| 4.4.4.a | -0.13% | -0.07% | -0.08% | 106% | 100% | -0.05% | 0.03% | 0.00% | 104% | 100% |
| 4.4.4.b | -0.05% | 0.03% | 0.01% | 102% | 100% | 0.09% | 0.09% | 0.11% | 101% | 100% |
| 4.4.4.c | -0.28% | -0.16% | -0.19% | 106% | 100% | 0.01% | 0.14% | 0.25% | 104% | 100% |
| 4.4.4.d | -0.28% | -0.17% | -0.20% | 106% | 100% | 0.00% | 0.28% | 0.07% | 103% | 100% |
| 4.4.4.e | -0.38% | -0.39% | -0.48% | 101% | 100% | -0.15% | -0.11% | -0.01% | 102% | 100% |
| 4.4.5.a | -0.22% | -0.33% | -0.35% | 103% | 98% | -0.13% | -0.10% | -0.04% | 105% | 100% |
| 4.4.5.b | -0.21% | -0.34% | -0.41% | 103% | 98% | -0.08% | -0.11% | -0.10% | 105% | 100% |
| 4.4.5.c | -0.24% | -0.43% | -0.46% | 103% | 98% | -0.13% | -0.10% | -0.04% | 105% | 100% |
| 4.4.5 * | -0.11% | -0.14% | -0.18% | 103% | 98% | -0.08% | -0.11% | -0.10% | 105% | 106% |

4.4.5* is an encoder optimization that is done in VTM3. All other 4.4.5 results use this encoder optimization, but compare against VTM3 without it. 4.4.4.e is a combination of 4.4.5.c and 4.4.4.a, which additionally has some other encoder optimization.

4.4.1: The claimed benefit is that the number of context-coded bins in MVD coding is reduced from 4 to 1. This is achieved by joint coding of x and y differences, where the “layer” is the sum of x and y, and the index is an addressing in a layer. Index is coded as truncated binary (number of indices depends on the layer). In total, the number of bits for expressing the layer and the index is not changed relative to the independent coding of MVD.

Only the MVD coding part was changed, motion estimation, RDO etc. were identical.

4.4.1a is the simpler solution, difference in compression performance is marginal.

Decision: Adopt JVET-M0403 (test 4.4.1a, 2 layer groups, where the first group is just the 0,0 MVD).

Specification text is available.

Revisit: In the context of later discussion related to CE8, it was detected that this might have coding efficiency problems with CPR. Furthermore, it would be more difficult to check in the layer/index representation if the CPR range constraints are valid. Needs further discussion.

4.4.3: The compression gain of symmetric MVD coding is 0.16% (was approx. 0.6% before), could be reduced by the fact that VTM3 incudes MMVD, which also uses symmetric coding, and also other elements of improved MV coding. Encoding time increases by 5%, very simple for decoder.

Some concern is expressed that not all of the current gain may be retained when MMVD would be further improved.

There is another contribution (JVET-M0444) which reports that by further encoder optimization and disallowing combination with BDOF further gain is possible. Revisit in this context.

Proposals related to MMVD:

4.4.4.a: Add diagonal direction candidates. This saves 0.13% for RA, 0.05% for LB, encoder time increases by about 5%.

4.4.4.b: Instead one list with 8 MVD, use two lists with 4 each, where the first list is narrow range, the second list wider range of differences. Whereas the maximum MVD in current design is 32, here it is suggested as 8. This has 0.05% gain for RA, 0.09% loss for LB. One more context coded bin. Encoding time is increased by 2%

4.4.4.c: Combination of a/b, giving 0.28% in RA. The new distance table seems to be more efficient for diagonal direction. Furthermore, 4.4.4.a has more encoding checks, which may be another reason.

It is noted that the VTM encoder has also a “non fast” MMVD check which also provides additional compression gain by doing more checks.

4.4.4.d: Integer rounding does not give benefit.

Not clear how much the elements of the proposal (introducing diagonal directions, and modifying the MVD coding) contribute to the compression gain, and how much is due to additional encoder optimization. Generally for LB, no benefit at all.

4.4.5.a is recording the occurrences of the different MVD for each picture, and reordering of the distances for the current picture is performed based on the data that were collected in the picture of list 0, refidx 0. Some mechanism providing a “score” for a table is used as well.

It is pointed out that explicit signalling would be at almost no rate increase and without additional decoding complexity, and also give up the dependency between slices. Generally, from the note on 4.4.5* above, the benefit of reordering is relatively small (maybe in the range of 0.1%).

4.4.5.b makes the distance table dependent on picture resolution. The coding gain is observed mainly on 4K sequences (where a different table is used).

It is also pointed out that this could better be explicitly signalled rather than inferring it from the picture size. The signalling overhead would be low.

4.4.5.c is combination of a and b, where the gain seems not to be additive.

4.4.4.e is a combination of 4.4.4 and 4.4.5, where some more fast algorithm for 4.4.4.c is included.

The results of 4.4.4b and 4.4.5 indicate that some improvement could be possible by adapting/modifying the distance table, but it is not clear how much gain is due to encoder optimization as well. Further study is necessary, and there are also CE related proposals which may provide more information.

In upcoming experiments on MMVD, it should be observed that it is possible to judge the benefit of new normative aspects versus encoder optimization.

Proponents offer to provide additional results on just combining 4.4.4.a and 4.4.5.b. This should repofrt results versus an anchor that is VTM3+4.4.5*.

Revisit after those would be available, and then also after review of non-CE proposals and an initial expectation of how many different new aspects would be investigated on MMVD in the next round of CE, decide whether such a combination could be a candidate for VTM4, or be further studied in the next round of CE as well.

Revisit: Clarify where the encoder optimization of 4.4.5* is documented, if it was cross-checked, and if this would be a candidate for a SW adoption.

4.2.5 (JVET-M0291) is also MMVD related. The following aspect are included:

- increase base candidates from 2 to 3, and decrease size of distance table from 8 to 4 (4.2.5.a)

- introduce diagonal directions, and decrease size of distance table from 8 to 4 (4.2.5.b)

A misalignment of software and text is also reported, where test 4.2.5.b follows the VVC3 spec which disables combination of MMVD with triangle mode, and with CIIP. This may give some additional gain.

4.2.5.a gives small loss (0.07%, 0.03%) in RA and LB

4.2.5.b gives loss in RA (0.02%), small gain (-0.07%) in LB. These compare against the anchor without the misalignment fix, which may mean that the results are worse both for RA and LB.

There is report about decoding time decrease, but it is unclear how around 5% could be explainable by just reducing the distance table from 8 to 4.

CE4.5: Motion compensation constraints for complexity reduction

| Test# | Source | Description |
| --- | --- | --- |
| 4.5.1 | JVET-M0313 | Disabling bi-prediction for 4x8 and 8x4 CUs |
| 4.5.2 | JVET-M0313 | Disabling bi-prediction or inter prediction for CU which memory bandwidth exceeds that of 8x8 bi-prediction: bi-prediction is disabled for 4×8, 8×4, 4×16 and 16×4 CUs, and inter prediction is disabled for 4×4 CUs. |

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 4.5.1 | 0.09% | 0.19% | 0.25% | 99% | 96% | 0.13% | 0.08% | 0.31% | 99% | 105% |
| 4.5.2 | 0.32% | 0.71% | 0.81% | 99% | 97% | 0.49% | 0.88% | 0.80% | 98% | 99% |

The following table is provided:

| CU size (W×H) | Number of luma samples to be fetched per luma sample (W + 7) × (H + 7) / (W × H) |
| --- | --- |
| 8×8 Bi-prediction | 7.03 |
| 4×8 Bi-prediction | 10.31 |
| 4×16 Bi-prediction | 7.91 |
| 4×32 Bi-prediction | 6.70 |
| 4×64 Bi-prediction | 6.10 |
| 8×4 Bi-prediction | 10.31 |
| 16×4 Bi-prediction | 7.91 |
| 32×4 Bi-prediction | 6.70 |
| 64×4 Bi-prediction | 6.10 |
| 4×4 Uni-prediction | 7.56 |

It is noted that the table above does not consider memory access patterns, is only pixel level access.

Further study on these aspects is needed. There are also non-CE proposals which use other methods (e.g. shorter interpolation filters, integer pel, padding, …) which could be considered for reducing memory access.

Establish BoG (K. Zhang) to review the CE4 related proposals, and suggest aspects to be studied in CE. Consider either significant complexity reduction without losing compression performance, or proposals with significant improvement of compression without increasing complexity. If there are proposals that are closely related to proposals that were investigated in CE (e.g. some beneficial encoder optimization, or minor syntax change with benefit, or further complexity reduction), these should be reported to track B for possible consideration for adoption.

(include summaries of contribution from JVET-M0024 subsequently)

JVET-M0059 CE4: Non-scaling STMVP (Test 4.2.1) [T. Zhou, T. Ikai (Sharp)]

JVET-M0060 CE4: Enhanced Merge with MVD (Test 4.4.4) [T. Hashimoto, E. Sasaki, T. Ikai (Sharp)]

JVET-M0061 CE4: Combination of CE4.4.4.a and CE4.4.5.c (Test 4.4.4.e) [T. Hashimoto, E. Sasaki, T. Ikai (Sharp), J. Li, R.-L. Liao, C. S. Lim (Panasonic)]

JVET-M0687 Crosscheck of JVET-M0061 (CE4.4.4e: Combination of CE4.4.4.a and CE4.4.5.c) [L. Xu, F. Chen, L. Wang (Hikvision)] [late]

JVET-M0086 CE4: Non-sub-block ATMVP (test 2.5.4) [K. Abe, T. Toma (Panasonic)]

JVET-M0106 CE4: STMVP without scaling (tests 4.2.2) [F. Le Léannec, T. Poirier, F. Galpin (Technicolor)]

JVET-M0124 CE4: Methods of Reducing Number of Pruning Checks of History Based Motion Vector Prediction (Test 4.1.1) [J. Zhao, S. Kim (LGE)]

JVET-M0126 CE4: Modification on History-based Motion Vector Prediction [Y. Han, W.-J. Chien, C.-C. Chen, H. Huang, C.-H. Hung, Y. Zhang, M. Karczewicz (Qualcomm)]

JVET-M0826 Crosscheck of JVET-M0126 supplemental data (CE4: Modification on history-based motion vector prediction) [Y.-C. Lin (MediaTek)] [late] [miss]

JVET-M0170 CE4.3.1: Shared merging candidate list [C.-C. Chen, Y.-C. Lin, M.-S. Chiang, C.-W. Hsu, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-M0584 Crosscheck of JVET-M0170 (CE4: Modification on History-based Motion Vector Prediction) Supplementary Tests [Y. Han, H. Wang, C.-C. Chen, W.-J. Chien (Qualcomm)] [late]

Note: Title of crosscheck is wrong – should be CE4.3.1: Shared merging candidate list

JVET-M0221 CE4: STMVP simplification (test 4.2.3a) [Y.-H. Chao, Y. Han, W.-J. Chien, M. Karczewicz (Qualcomm)]

JVET-M0281 CE4: Inter motion predictor pruning (test 4.1.5) [A. Robert, F. Le Léannec, T. Poirier, F. Galpin (Technicolor)]

JVET-M0289 CE4: Parallel Merge Estimation for VVC (Test 4.3.2) [H. Gao, S. Esenlik, B. Wang, A.M. Kotra, J. Chen (Huawei)]

JVET-M0291 CE4: Extension on MMVD (Test 4.2.5) [X. Chen, J. Zheng (HiSilicon)]

JVET-M0686 Crosscheck of JVET-M0291 (CE4.2.5: Extension on MMVD) [L. Xu, F. Chen, L. Wang (Hikvision)] [late]

JVET-M0312 CE4: MMVD improvement (test 4.4.5) [J. Li, R.-L. Liao, C. S. Lim (Panasonic)]

JVET-M0867 Crosscheck of CE4.4.5* [E. Sasaki, T. Ikai (Sharp)] [late]

JVET-M0313 CE4: Motion compensation constraints for complexity reduction (test 4.5.1 and test 4.5.2) [R.-L. Liao, J. Li, C. S. Lim (Panasonic)]

JVET-M0403 CE4: Generic Vector Coding of Motion Vector Difference (Tests 4.4.1.a and 4.4.1.b) [S. Paluri, M. Salehifar, S. Kim (LGE)]

JVET-M0698 Crosscheck of JVET-M0403: CE4: Generic Vector Coding of Motion Vector Difference (Tests 4.4.1.a and 4.4.1.b) [L. Pham Van, W.-J. Chien, M. Karczewicz (Qualcomm)] [late]

JVET-M0404 CE4: History based spatial-temporal MV prediction (Test 4.2.4) [X. Xu, X. Li, S. Liu (Tencent)]

JVET-M0481 CE4: Symmetrical MVD mode (Test 4.4.3) [H. Chen, T. Solovyev, H. Yang, J. Chen (Huawei)]

## CE5: Arithmetic coding engine (10)

Contributions in this category were discussed XXday X Jan. XXXX–XXXX (chaired by XXX).

JVET-M0025 CE5: Summary report on the Arithmetic Coding Engine [H. Kirchhoffer, A. Said]

This contribution was discussed Thursday 10 January 1700–2030 (chaired by Y. Ye)

This is the summary report of Core Experiment 5 (CE5) on the Arithmetic Coding Engine. Subtest 1 (experiments CE5.1.1 – 5.1.13) addresses the effect on the compression efficiency of training parameters of the probability estimation stage of the proposed arithmetic coding engines. Subtest 2 addresses the achievable throughput of the different coding engines of subtest 1 in an optimized software implementation. Subtest 3 provides a summary of the analysis of hardware implementation aspects.

The experiments were implemented in separate branches in the following git repository:

https://vcgit.hhi.fraunhofer.de/JVET-L-CE5/VVCSoftware_VTM.git

For subtest 2, a testbed is available in branch CE5.2, which contains throughput-optimized software implementations of the coding engines of subtest 1.

The following table lists all tests that were performed in this CE and gives a summary of the tools.

Note on the naming convention of CE tests: ‘CE5.x.y.z’ means ‘CE5.x.y’ as specified in the CE5 description with configuration ‘z’ according to the following tables.

| CE5.1 – Probability estimation with retrained parameters |  |  |  |
| --- | --- | --- | --- |
| CE # (Proponent) | Related Docs. | Summary of the tool | Crosscheckers |
| 5.1.1 (HHI) | JVET-M0725 | 2 linear states (10+14 bits), fixed | MediaTek |
| 5.1.2 (HHI) | JVET-M0725 | 2 linear states (10+14 bits), variable | MediaTek |
| 5.1.3 (Qualcomm) | JVET-M0412 | 2 linear states (10+14 bits), variable: Configuration 1: Coding interval subdivision from JVET-L0335 is used Configuration 2: Coding interval subdivision from JVET-L0117 is used | HHI |
| 5.1.4 (Qualcomm) | JVET-M0413 | 1 linear state (14 bits), variable: Configuration 1: Coding interval subdivision from JVET-L0335 is used Configuration 2: Coding interval subdivision from JVET-L0117 is used | Sharp Labs |
| 5.1.5 (HHI) | JVET-M0727 | 2 log. states (8+12 bits), fixed | Sharp Labs |
| 5.1.6 (HHI) | JVET-M0727 | 2 log. states, variable Configuration 1: 8+12 bits Configuration 2: 10+14 bits | Sharp Labs |
| 5.1.7 (HHI) | JVET-M0727 | 1 log. state (12 bits), variable | Sharp Labs |
| 5.1.8 (Samsung) | JVET-M0199 | 2 linear states (15+15 bits), fixed Only the short window size model is updated for the first 31 bins of a context model. | MediaTek |
| 5.1.9 (MediaTek) | JVET-M0172 | 2 linear states (10+14 bits), fixed Multiplier-based coding interval subdivision. Configuration 1: 6-bit by 5-bit multiplier. Configuration 2: 6-bit by 4-bit multiplier. Both configurations can implemented as 32x8x8 bit LUT. | Samsung |
| 5.1.10 (Sharp Labs) | JVET-M0453 | 2 linear states, fixed Uses 2b−1 instead of 2b in probability update | HHI |
| 5.1.11 (Sharp Labs) | JVET-M0453 | 2 linear states, variable Uses 2b−1 instead of 2b in probability update | HHI |
| 5.1.12 (Sharp Labs) | JVET-M0453 | 1 linear state, variable Uses 2b−1 instead of 2b in probability update | MediaTek |
| 5.1.13 (Sharp Labs) | JVET-M0453 | 2 linear states, variable Uses 2b−1 instead of 2b in probability update Uses 5x4 multiplier for coding interval subdivision | HHI |

|  | AI |  |  |  | RA |  |  |  | LB |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CE # | Y | U | V | EncT/ DecT (%) | Y | U | V | EncT/ DecT (%) | Y | U | V | EncT/ DecT (%) |
| 5.1.1 | -0,81% | -1,15% | -1,20% | 105/101 | -0,71% | -1,06% | -0,97% | 102/100 | -0,60% | -0,22% | -0,18% | 102/100 |
| 5.1.2 | -0,97% | -1,18% | -1,27% | 107/101 | -0,97% | -0,99% | -0,93% | 103/99 | -0,89% | -0,08% | -0,25% | 102/99 |
| 5.1.3.1 | -0,95% | -1,35% | -1,44% | 107/103 | -0,90% | -1,06% | -0,97% | 105/106 | -0,77% | -0,01% | -0,23% | 106/103 |
| 5.1.3.2 | -0,99% | -1,42% | -1,53% | 107/105 | -0,95% | -1,07% | -1,04% | 106/106 | -0,82% | -0,04% | -0,15% | 107/104 |
| 5.1.4.1 | -0,80% | -1,12% | -1,11% | 108/107 | -0,64% | -0,11% | -0,01% | 107/110 | -0,56% | 1,01% | 1,51% | 107/104 |
| 5.1.4.2 | -0,89% | -1,21% | -1,18% | 108/107 | -0,71% | -0,26% | -0,05% | 108/109 | -0,61% | 1,02% | 1,76% | 107/107 |
| 5.1.5 | -0,76% | -1,47% | -1,41% | 101/98 | -0,63% | -0,79% | -0,70% | 97/93 | -0,68% | -0,29% | 0,08% | 94/91 |
| 5.1.6.1 | -0,83% | -1,51% | -1,53% | 103/98 | -0,76% | -0,77% | -0,75% | 98/93 | -0,76% | -0,44% | -0,15% | 95/92 |
| 5.1.6.2 | -0,91% | -1,44% | -1,44% | 104/97 | -0,81% | -0,92% | -0,85% | 98/93 | -0,72% | -0,42% | -0,05% | 95/92 |
| 5.1.7 | -0,66% | -1,12% | -1,10% | 100/98 | -0,53% | -0,96% | -0,95% | 97/93 | -0,52% | -0,66% | -0,52% | 94/92 |
| 5.1.8 | -0,86% | -1,36% | -1,37% | 106/101 | -0,68% | -0,86% | -0,71% | 102/101 | -0,53% | -0,60% | 0,51% | 99/95 |
| 5.1.9.1 | -0,79% | -1,05% | -1,18% | 105/101 | -0,57% | -1,05% | -1,00% | 102/100 | -0,50% | -0,82% | -0,66% | 103/101 |
| 5.1.9.2 | -0,76% | -1,02% | -1,15% | 105/102 | -0,55% | -1,03% | -0,98% | 102/100 | -0,49% | -0,81% | -0,65% | 102/101 |
| 5.1.10 | -0,79% | -1,10% | -1,16% | 108/104 | -0,67% | -1,06% | -1,02% | 104/102 | -0,54% | -0,38% | -0,78% | 103/101 |
| 5.1.11 | -0,94% | -1,14% | -1,23% | 108/103 | -0,93% | -0,95% | -1,05% | 103/101 | -0,80% | -0,27% | -0,38% | 102/101 |
| 5.1.12 | -0,66% | -0,74% | -0,50% | 108/103 | -0,55% | 0,18% | 0,94% | 102/100 | -0,54% | 0,81% | 2,33% | 101/99 |
| 5.1.13 | -0,93% | -1,13% | -1,21% | 109/103 | -0,91% | -0,93% | -1,03% | 104/101 | -0,78% | -0,24% | -0,35% | 101/98 |
| 2 states variable, 2 states fixed, 1 state variable |  |  |  |  |  |  |  |  |  |  |  |  |

It was reported that some of the encoding/decoding time in the table may not be reliable. It was commented that these tests have similar encoding/decoding time, and variation is in the noise range.

The CABAC engine and the initialization states are tested together.

VTM 3 uses HEVC CABAC engine with the following characteristics:

- 1 log. state, 7 bits, fixed window size

- Coding interval subdivision of 64x4x8 bits LUT

Throughput-optimized software implementations of the configurations in CE5.1 are tested in terms of the achievable throughput in the decoder. Two types of bin sequences are used for the evaluation:

- VTM3: Bin sequences extracted from VTM-3.0 CTC bitstreams

- RND:  Randomly generated bin sequences

Three contributions report results for CE5.2:

- JVET-M0453 (Sharp)

- JVET-M0463 (Qualcomm)

- JVET-M0762 (HHI)

It was commented that JVET-M0453 and JVET-M0463 reported consistent trends in terms of throughput numbers for the CE tests. JVET-M0762 was a late contribution that was updated shortly before the CE review started. It was agreed to review JVET-M0762 for further discussion of this CE. See notes under JVET-M0762.

JVET-M0172 CE5.1.9: CABAC engine with simplified range sub-interval derivation [T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-M0199 CE5: Counter-based probability estimation (CE5.1.8) [K. Choi, Y. Piao (Samsung)]

JVET-M0412 CE5: Per-context CABAC initialization with double windows (Test 5.1.3) [A. Said, J. Dong, H. Egilmez, Y.-H. Chao, M. Karczewicz, V. Seregin (Qualcomm)]

JVET-M0413 CE5: Per-context CABAC initialization with single window (Test 5.1.4) [A. Said, J. Dong, H. Egilmez, Y.-H. Chao, M. Karczewicz, V. Seregin (Qualcomm)]

JVET-M0453 CE5 on arithmetic coding: experiments 5.1.1, 5.1.2, 5.1.3, 5.1.4, 5.1.5, 5.1.6, 5.1.7, 5.1.8, 5.1.10, 5.1.11, 5.1.12, 5.1.13, 5.2, and more [F. Bossen (Sharp)]

This report provides results for the following CE5 experiments: 5.1.1, 5.1.2, 5.1.3, 5.1.4, 5.1.5, 5.1.6, 5.1.7, 5.1.8, 5.1.10, 5.1.11, 5.1.12, 5.1.13 and 5.2. Additional variants of these experiments are considered where constants, such as initial state values and shift amounts, are modified.

Version 2 of the document provides additional results for experiment 5.2 on throughput, as well as BD-rate results for a configuration that yields no encoder run time increase.

The following table reports the decoder throughput from an actual bin stream (first 10 million context-coded bins from ParkRunning, RA configuration, QP22). Three compilers have been tested: clang 10.0 (Apple LLVM version 10.0.0), gcc 7.4 (Homebrew GCC 7.4.0) and gcc 8.2 (Homebrew GCC 8.2.0).  The test bed was configured with the macro NOBRANCH_MPS either on (left half of table) or off (right half of table). Results were obtained on an Intel® Xeon® W-2140B CPU @ 3.20GHz.

In almost all cases the combination of clang and NOBRANCH_MPS enabled yielded the highest throughput. This configuration is thus considered for comparing the various engines.

The rightmost column in the table below reports the decoder throughput for a different bin stream (first 10 million context-coded bins from BQTerrace, RA configuration, QP22, clang, NOBRANCH_MPS enabled).

| Engine | clang | gcc 7 | gcc 8 | clang | gcc 7 | gcc 8 | clang stream #2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AVC/HEVC CABAC | 132.03 | 134.92 | 128.64 | 127.27 | 121.13 | 124.98 | 137.88 |
| CE5.1.1 | 120.17 | 117.48 | 118.58 | 113.54 | 108.52 | 109.26 | 121.51 |
| CE5.1.2 | 115.00 | 110.93 | 113.28 | 104.01 | 108.47 | 102.20 | 114.67 |
| CE5.1.3 | 106.63 | 104.98 | 104.60 | 95.89 | 96.84 | 96.37 | 102.79 |
| CE5.1.4 | 109.09 | 106.68 | 107.28 | 104.77 | 104.45 | 104.77 | 110.55 |
| CE5.1.4 mult | 134.85 | 133.09 | 131.12 | 120.09 | 116.32 | 117.72 | 135.13 |
| CE5.1.5 | 113.46 | 106.75 | 107.56 | 111.67 | 102.44 | 104.75 | 113.02 |
| CE5.1.6 (8+12 bit) | 110.75 | 105.07 | 103.62 | 103.64 | 98.89 | 100.07 | 108.50 |
| CE5.1.7 | 119.00 | 110.98 | 112.45 | 118.51 | 110.15 | 107.84 | 120.23 |
| CE5.1.8 | 118.80 | 114.27 | 107.99 | 107.71 | 103.96 | 100.47 | 119.39 |
| CE5.1.9 / CE 5.1.10 | 125.05 | 122.68 | 120.02 | 120.47 | 113.33 | 115.96 | 127.29 |
| CE5.1.11 | 121.79 | 117.33 | 117.14 | 107.25 | 113.08 | 111.22 | 123.35 |
| CE5.1.12 | 128.95 | 123.80 | 127.78 | 118.60 | 119.66 | 120.22 | 129.01 |
| CE5.1.13 / CE5.1.3 mult | 127.28 | 121.00 | 122.57 | 109.90 | 109.25 | 110.67 | 127.32 |

Experiments were run a second time, where each test is done 7 times and the highest throughput value is recorded. While the numbers are slightly higher, the same trends persist.

| Engine | clang | gcc 7 | gcc 8 | clang | gcc 7 | gcc 8 | clang stream #2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AVC/HEVC CABAC | 137.27 | 137.83 | 131.01 | 131.69 | 124.68 | 127.93 | 141.37 |
| CE5.1.1 | 124.17 | 121.64 | 120.26 | 117.41 | 112.53 | 110.42 | 124.48 |
| CE5.1.2 | 116.69 | 115.36 | 116.85 | 106.22 | 110.39 | 109.24 | 115.93 |
| CE5.1.3 | 107.96 | 106.13 | 105.99 | 96.69 | 98.59 | 97.68 | 108.09 |
| CE5.1.4 | 111.83 | 109.14 | 109.93 | 106.80 | 106.21 | 106.51 | 112.09 |
| CE5.1.4 mult | 136.30 | 133.96 | 132.36 | 120.85 | 116.32 | 121.01 | 137.27 |
| CE5.1.5 | 115.84 | 109.11 | 108.83 | 114.71 | 105.29 | 105.21 | 115.73 |
| CE5.1.6 (8+12 bit) | 112.69 | 106.88 | 103.99 | 105.00 | 102.27 | 102.21 | 112.68 |
| CE5.1.7 | 120.75 | 112.94 | 113.24 | 119.12 | 106.58 | 114.97 | 120.89 |
| CE5.1.8 | 122.01 | 116.04 | 108.10 | 110.09 | 107.50 | 102.54 | 125.28 |
| CE5.1.9 / CE 5.1.10 | 128.67 | 124.20 | 121.22 | 122.22 | 119.20 | 117.55 | 129.44 |
| CE5.1.11 | 124.55 | 118.73 | 119.37 | 114.53 | 114.48 | 112.39 | 125.28 |
| CE5.1.12 | 130.26 | 128.17 | 130.77 | 121.13 | 121.84 | 120.45 | 131.45 |
| CE5.1.13 / CE5.1.3 mult | 128.51 | 122.63 | 126.85 | 112.10 | 111.23 | 112.96 | 128.87 |

It was agreed that the second table of throughput numbers had much lower probability of having outlier values in it.

The table below summarizes the performance of various arithmetic coding engines, classifying them in three groups:

- Two states, fixed window sizes

- Two states, variable (context-adaptive) window sizes

- Single state, variable (context-adaptive) window sizes

BD rate numbers are reported as combined YUV, where BD rate numbers for Y, U and V are weighted as 90%/5%/5% to obtain a single number. This is done to account for the fact that results for individual color components can sometimes diverge.

Also provided for each engine are encoder/decoder run times relative to VTM3, as well as SW throughput results from experiment 5.2.

Within each group and each test condition the best BD rate and throughput results are highlighted in green. The worst results are highlighted in red.

Experiment numbers with a star indicate a variant, where some constants are modified as described.

| Engine | AI | RA | LB | LP | AI | RA | LB | Thruput |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 states, fixed window size |  |  |  |  |  |  |  |  |
| 5.1.1 | -0.85% | -0.74% | -0.56% | -0.56% | 106/103 | 103/102 | 101/100 | 120.17 |
| 5.1.5 | -0.83% | -0.64% | -0.62% | -0.60% | 109/107 | 105/103 | 103/103 | 113.46 |
| 5.1.8 | -0.91% | -0.69% | -0.48% | -0.50% | 106/102 | 103/101 | 100/98 | 118.80 |
| 5.1.10 | -0.83% | -0.70% | -0.55% | -0.61% | 108/104 | 104/102 | 103/101 | 125.05 |
| 5.1.10* + new init from 5.1.2 | -0.84% | -0.73% | -0.51% | -0.53% | 109/107 | 105/104 | 105/104 | 125.05 |
| 5.1.10* 5x6 mult | -0.85% | -0.73% | -0.58% | -0.64% | 109/107 | 105/104 | 104/104 | 127.86 |
| 2 states, variable window size |  |  |  |  |  |  |  |  |
| 5.1.2 | -1.00% | -0.97% | -0.82% | -0.76% | 109/105 | 104/103 | 101/101 | 115.00 |
| 5.1.3 clz + 8x8 table | -1.03% | -0.96% | -0.74% | -0.83% | 107/108 | 104/104 | 104/104 | 106.63 |
| 5.1.3 4x5 mult | -0.99% | -0.91% | -0.71% | -0.83% | 109/105 | 104/102 | 103/102 | 127.28 |
| 5.1.3* 5x6 mult | -1.07% | -0.99% | -0.79% | -0.91% | 108/104 | 104/103 | 104/103 | 127.28 |
| 5.1.6 8+12 bit state | -0.90% | -0.76% | -0.72% | -0.69% | 111/105 | 105/103 | 105/102 | 110.75 |
| 5.1.6 10+14 bit state | -0.97% | -0.82% | -0.67% | -0.73% | 112/105 | 105/102 | 105/102 | 110.75 |
| 5.1.11 | -0.97% | -0.93% | -0.75% | -0.76% | 108/103 | 103/101 | 102/101 | 121.79 |
| 5.1.11* + new init from 5.1.2 | -1.00% | -0.98% | -0.77% | -0.75% | 110/105 | 104/102 | 104/102 | 121.79 |
| 5.1.13 | -0.95% | -0.91% | -0.73% | -0.74% | 109/103 | 104/101 | 101/98 | 127.28 |
| 5.1.13* +new init from 5.1.2 | -0.98% | -0.96% | -0.75% | -0.73% | 110/105 | 104/103 | 106/103 | 127.28 |
| 5.1.13* 5x6 mult | -1.03% | -0.98% | -0.75% | -0.75% | 109/105 | 104/102 | 104/104 | 127.28 |
| 1 state, variable window size |  |  |  |  |  |  |  |  |
| 5.1.4 clz + 8x8 table | -0.92% | -0.65% | -0.41% | -0.44% | 108/107 | 103/104 | 104/103 | 109.09 |
| 5.1.4 4x5 mult | -0.83% | -0.58% | -0.37% | -0.41% | 108/105 | 104/102 | 102/102 | 132.95 |
| 5.1.4* 5x6 mult | -0.93% | -0.67% | -0.46% | -0.49% | 107/105 | 104/102 | 104/103 | 132.95 |
| 5.1.7 | -0.70% | -0.57% | -0.53% | -0.52% | 107/107 | 104/103 | 103/104 | 119.00 |
| 5.1.12 | -0.65% | -0.44% | -0.33% | -0.25% | 108/103 | 102/100 | 101/99 | 128.95 |

The table above was copied from JVET-M0453 and then updated in group discussion to form the modified table below. The each category in the table was reviewed and particular tests that provided the highest coding efficiency in each category with good throughput numbers were highlighted for further focused discussion. The properties, similarities and differences between the tested methods were discussed.

The following table was obtained by updating the table above as follows: 1) adding 5.1.9, 2) replacing throughput numbers with the second throughput number table, and 3) keeping only rows that pertain to the CE tests.

| Engine | AI | RA | LB | LP | AI | RA | LB | Thruput |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VTM3 w/ HEVC engine | 0 | 0 | 0 | 0 | 100/100 | 100/100 | 100/100 | 137.83 |
| 2 states, fixed window size |  |  |  |  |  |  |  |  |
| 5.1.1 | -0.85% | -0.74% | -0.56% | -0.56% | 106/103 | 103/102 | 101/100 | 124.17 |
| 5.1.5 | -0.83% | -0.64% | -0.62% | -0.60% | 109/107 | 105/103 | 103/103 | 115.84 |
| 5.1.8 | -0.91% | -0.69% | -0.48% | -0.50% | 106/102 | 103/101 | 100/98 | 122.01 |
| 5.1.9 | -0.79% | -0.57% | -0.50% | N/A |  |  |  | 128.67 |
| 5.1.10 | -0.83% | -0.70% | -0.55% | -0.61% | 108/104 | 104/102 | 103/101 | 128.67 |
| 5.1.10* + new init from 5.1.2 | -0.84% | -0.73% | -0.51% | -0.53% | 109/107 | 105/104 | 105/104 | 128.67 |
| 2 states, variable window size |  |  |  |  |  |  |  |  |
| 5.1.2 | -1.00% | -0.97% | -0.82% | -0.76% | 109/105 | 104/103 | 101/101 | 116.69 |
| 5.1.3 clz + 8x8 table (config. 2) | -1.03% | -0.96% | -0.74% | -0.83% | 107/108 | 104/104 | 104/104 | 107.96 |
| 5.1.3 4x5 mult (config. 1) | -0.99% | -0.91% | -0.71% | -0.83% | 109/105 | 104/102 | 103/102 | 128.51 |
| 5.1.6 8+12 bit state | -0.90% | -0.76% | -0.72% | -0.69% | 111/105 | 105/103 | 105/102 | 112.69 |
| 5.1.6 10+14 bit state | -0.97% | -0.82% | -0.67% | -0.73% | 112/105 | 105/102 | 105/102 | 112.69* |
| 5.1.11 | -0.97% | -0.93% | -0.75% | -0.76% | 108/103 | 103/101 | 102/101 | 124.55 |
| 5.1.11* + new init from 5.1.2 | -1.00% | -0.98% | -0.77% | -0.75% | 110/105 | 104/102 | 104/102 | 124.55 |
| 5.1.13 | -0.95% | -0.91% | -0.73% | -0.74% | 109/103 | 104/101 | 101/98 | 128.51 |
| 5.1.13* +new init from 5.1.2 | -0.98% | -0.96% | -0.75% | -0.73% | 110/105 | 104/103 | 106/103 | 128.51 |
| 1 state, variable window size |  |  |  |  |  |  |  |  |
| 5.1.4 clz + 8x8 table (config. 2) | -0.92% | -0.65% | -0.41% | -0.44% | 108/107 | 103/104 | 104/103 | 111.83 |
| 5.1.4 4x5 mult (config. 1) | -0.83% | -0.58% | -0.37% | -0.41% | 108/105 | 104/102 | 102/102 | 136.30 |
| 5.1.7 | -0.70% | -0.57% | -0.53% | -0.52% | 107/107 | 104/103 | 103/104 | 120.75 |
| 5.1.12 | -0.65% | -0.44% | -0.33% | -0.25% | 108/103 | 102/100 | 101/99 | 130.77 |

It was suggested to look at the hardware aspect of these different engines, which was provided as part of JVET-M0025 subtest 3.

It was agreed that no hardware problem had been identified for any of the CE tests, and that such a hardware problem could be fixed if/when it was identified.

When looking at different categories, it was remarked that the “1 state, variable window” category has the highest throughput (closest to HEVC engine), and the “2 state, variable window” category has the highest coding performance. Regarding the “2 state, fixed window” category, it was remarked that this category does not need custom window size parameters for different context models, but needs re-training of initialization parameters.

Regarding custom window size parameters, it was remarked that that does not seem to increase complexity to such a degree that it justifies going for a simpler solution (i.e. fixed window size).

It was remarked that some of the coding efficiency gain from the tests that use custom window size parameters may have come from training of custom window size parameters based on the test set.

It was remarked that the initialization parameters were also trained on the test set, which also may have provided some of the coding efficiency gain.

In terms of coding efficiency, the options “5.1.3 4x5 mult (config. 1)”, “5.1.13” and “5.1.13* +new init from 5.1.2” are the most attractive options. Between “5.1.3” and “5.1.13,” the latter has a slight advantage for hardware implementations due to needing to support fewer shift values. And the difference between “5.1.13” and “5.1.13* +new init from 5.1.2” is purely due to training of initialization parameters, with the new initialization parameters providing better coding efficiency.

Decision: adopt “5.1.13* +new init from 5.1.2”

JVET-M0463 CE5: Report of throughput analysis of CE5 contributions (CE5.2) [J. Dong, A. Said, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-M0725 CE5: Results of tests CE5.1.1 and CE5.1.2 [J. Stegemann, H. Kirchhoffer, H. Schwarz, D. Marpe, T. Wiegand (HHI)] [late]

JVET-M0727 CE5: Results of tests CE5.1.5, CE5.1.6, and CE5.1.7 [H. Kirchhoffer, C. Bartnik, P. Haase, S. Matlage, J. Stegemann, D. Marpe, H. Schwarz, T. Wiegand (HHI)] [late]

JVET-M0759 CE5: Report of subtest 3 on complexity and throughput aspects for hardware [B. Stabernack (HHI), T. Hsieh (Qualcomm)] [late]

JVET-M0762 CE5: Report of software throughput analysis for CE5.2 by HHI [H. Kirchhoffer, C. Bartnik, T. Hinz, J. Stegemann, P. Haase, S. Matlage, B. Stabernack, H. Schwarz, D. Marpe, T. Wiegand (HHI)] [late]

This contribution presents results for CE5 subtest 2. The throughput measurement testbed as implemented in CE5.2 was used to produce throughput numbers for the test configurations of subtest 1.

The throughput numbers in version 1 of this document contained a bug. They are replaced with corrected values in version 2.

The preprocessor macros of the testbed software of the CE5.2 branch are set according to the following table from JVET-M0453-r1, section 3.1.

The throughput is measured for 11 synthetical bin sequences and 12 VTM-3.0 bitstreams.

Three combinations of two macros (NOBRANCH_MPS and NOBRANCH) were tested as follows.

2.1 NOBRANCH off, NOBRANCH_MPS off

2.2	NOBRANCH off, NOBRANCH_MPS on

2.3	NOBRANCH on, NOBRANCH_MPS on

It was commented that these graphs contained outliers. It was also commented that the graphs were uploaded very late (~20 minutes before the CE5 discussion started). It was then agreed to review JVET-M0453 instead, which provided throughput numbers for each CE test, for the further discussion of this CE. See notes under JVET-M0453.

## CE6: Transforms and transform signalling (21)

Contributions in this category were discussed Thursday 10 January 2030–2230, Friday 11 January 0900–1100, 1130–1400, and 1530–2000 (chaired by GJS).

JVET-M0026 CE6: Summary Report on Transforms and Transform Signaling [A. Said, X. Zhao]

This contribution summarizes the activities of Core Experiment (CE) on Transforms and Transform Signaling. The goal of this CE is to study transform design and signaling for the VVC standard. The CE studies were divided into three categories, including:

(1) CE6.1: Transform core design (19 tests, 7 proposals)

(2) CE6.2: Fast transform (3 tests, 3 proposals)

(3) CE6.3: Transform selection and signalling (10 tests, 5 proposals)

(4) CE6.4: Sub-block transform (8 tests, 4 proposals)

(5) CE6.5: Secondary transform (1 tests, 1 proposal)

In this CE all experiments were done using based on the VTM 3.0 software with a bug-fix for low QP configuration (without which the encoder could crash, a fix having no impact on CTC results since the CTC didn’t use extremely low QP values). This document summarizes the test results, brief experiment definition, cross-check reports and complexity measurements.

For all tests, two configurations were tested: CTC and CTC+Inter MTS. For this meeting report, only the CTC tables were included. The M0026 document contains the other tables as well.

For CE6.1 and CE6.2, additional low QP tests were done for QP values 1, 5, 9, 13, using only 100 frames of each test sequence (since the encoding is very slow at these QP values).

CE6-1 Transform core design

| CE6.1: Transform core design |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test # | Docs. | Description | Tester | Cross-checker |
| 6-1.1a | JVET-M0496 | JVET-L0287: Replacing 4-pt DST-7 / DCT-8 by 4-pt DST-4 / DCT-4 used in MTS | X. Zhao (Tencent) | A. Karabutov (Huawei) |
| 6-1.1b |  | JVET-L0287: Replacing 4-pt and 8-pt DST-7 / DCT-8 by 4-pt and 8-pt DST-4 / DCT-4 used in MTS, respectively. | X. Zhao (Tencent) | A. Karabutov (Huawei) |
| 6-1.1c |  | JVET-L0287: Compound Orthonormal Transform | X. Zhao (Tencent) | A. Karabutov (Huawei) |
| 6-1.1d |  | 6-1.1c + 6.2.3a | X. Zhao (Tencent) | A. Karabutov (Huawei) |
| 6-1.1e | JVET-M0084 | JVET-L0262: Replacing all DST-7 / DCT-8 by DST-4 / DCT-4 used in MTS | K. Abe (Panasonic) Sony Technicolor | A. Karabutov (Huawei) |
| 6-1.2a | JVET-M0200 | JVET-L0060: Unified matrix for transform | K. Choi (Samsung) | X. Zhao (Tencent) |
| 6-1.3a | JVET-M0244 | JVET-L0353: MTS using DST-4 and transposed DCT-2 | Y. Lin (HiSilicon) | H. Egilmez (Qualcomm) |
| 6-1.3b |  | MTS using DCT-2 like transforms | Y. Lin (HiSilicon) | H. Egilmez (Qualcomm) |
| 6-1.4a | JVET-M0538 | JVET-L0386, JVET-L0682: TAF for 32-pt MTS | A. Said (Qualcomm) | A. Karabutov (Huawei) X. Zhao (Tencent) |
| 6-1.4b |  | JVET-L0386, JVET-L0682: TAF for 32-pt and 64-pt MTS | A. Said (Qualcomm) | A. Karabutov (Huawei) |
| 6-1.4c |  | JVET-L0386, JVET-L0682: TAF for 16-pt and 32-pt MTS | A. Said (Qualcomm) | A. Karabutov (Huawei) X. Zhao (Tencent) |
| 6-1.4d |  | JVET-L0386, JVET-L0682: TAF for 16-pt, 32-pt and 64-pt MTS | A. Said (Qualcomm) | A. Karabutov (Huawei) |
| 6-1.5a | JVET-M0080 | JVET-L0135: TAF simplification for 32-pt MTS | P. Philippe (Orange) | LGE |
| 6-1.5b |  | JVET-L0135: TAF simplification for 32-pt and 64-pt MTS | P. Philippe (Orange) | LGE |
| 6-1.5c |  | JVET-L0135: TAF simplification for 16-pt and 32-pt MTS | P. Philippe (Orange) | LGE |
| 6-1.5d |  | JVET-L0135: TAF simplification for 16-pt, 32-pt and 64-pt MTS | P. Philippe (Orange) | LGE |
| 6-1.6a | JVET-M0521 | JVET-L0395:4-pt DST-4 and DCT-4 replacing DST-7 and DCT-8 used in MTS | H. Egilmez (Qualcomm) | Y. Lin (HiSilicon) |
| 6-1.6b |  | JVET-L0395: 4-pt and 8-pt DST-4 and DCT-4 replacing DST-7 and DCT-8 used in MTS | H. Egilmez (Qualcomm) | Y. Lin (HiSilicon) |
| 6-1.6c |  | JVET-L0395: 4-pt, 8-pt and 16-pt DST-4 and DCT-4 replacing DST-7 and DCT-8 used in MTS | H. Egilmez (Qualcomm) | Y. Lin (HiSilicon) |

For CTC:

|  |  | All Intra |  |  |  |  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Doc. # | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE6-1.1a | JVET-M0496 | -0.17% | -0.14% | -0.11% | 101% | 99% | -0.06% | 0.05% | 0.04% | 101% | 99% | 0.02% | 0.30% | 0.13% | 100% | 98% |
| CE6-1.1b | JVET-M0496 | -0.10% | -0.17% | -0.18% | 102% | 99% | -0.01% | 0.14% | 0.05% | 100% | 100% | 0.05% | 0.00% | 0.01% | 100% | 97% |
| CE6-1.1c | JVET-M0496 | -0.03% | -0.09% | -0.09% | 101% | 100% | 0.07% | 0.12% | 0.11% | 101% | 100% | 0.15% | 0.20% | -0.42% | 99% | 98% |
| CE6-1.1d | JVET-M0496 | -0.04% | -0.11% | -0.13% | 96% | 91% | 0.06% | 0.08% | 0.04% | 99% | 99% | 0.10% | 0.27% | -0.05% | 100% | 100% |
| CE6-1.1e | JVET-M0084 | 0.23% | 0.03% | 0.07% | 103% | 98% | 0.16% | 0.30% | 0.26% | 101% | 101% | 0.04% | 0.22% | -0.17% | 101% | 100% |
| CE6-1.2a | JVET-M0200 | -0.06% | -0.14% | -0.12% | 101% | 99% | 0.07% | 0.14% | 0.11% | 99% | 99% | 0.17% | 0.32% | 0.18% | 97% | 95% |
| CE6-1.3a | JVET-M0244 | 0.08% | 0.11% | 0.10% | 98% | 97% | 0.07% | 0.25% | 0.19% | 100% | 99% | -0.02% | 0.32% | 0.33% | 100% | 101% |
| CE6-1.3b | JVET-M0244 | 0.22% | 0.34% | 0.32% | 97% | 93% | 0.17% | 0.39% | 0.36% | 99% | 99% | 0.03% | 0.34% | 0.10% | 100% | 101% |
| CE6-1.4a | JVET-M0538 | 0.00% | 0.07% | 0.05% | 98% | 94% | -0.01% | 0.08% | 0.13% | 100% | 99% | -0.02% | 0.09% | 0.11% | 99% | 99% |
| CE6-1.4b | JVET-M0538 | -0.07% | 0.01% | 0.00% | 98% | 94% | -0.12% | -0.12% | -0.22% | 100% | 98% | 0.01% | 0.28% | 0.01% | 99% | 97% |
| CE6-1.4c | JVET-M0538 | 0.12% | 0.17% | 0.15% | 95% | 91% | 0.04% | 0.09% | 0.18% | 99% | 99% | 0.04% | 0.20% | 0.19% | 99% | 100% |
| CE6-1.4d | JVET-M0538 | 0.05% | 0.09% | 0.08% | 95% | 90% | -0.06% | -0.11% | -0.14% | 99% | 98% | 0.01% | -0.03% | 0.01% | 98% | 96% |
| CE6-1.5a | JVET-M0080 | 0.07% | 0.06% | 0.06% | 96% | 88% | 0.06% | 0.11% | 0.11% | 99% | 99% | 0.03% | 0.18% | 0.16% | 100% | 100% |
| CE6-1.5b | JVET-M0080 | 0.02% | -0.03% | -0.02% | 98% | 88% | -0.03% | -0.14% | -0.12% | 101% | 99% | -0.01% | 0.04% | -0.15% | 101% | 100% |
| CE6-1.5c | JVET-M0080 | 0.15% | 0.09% | 0.09% | 94% | 85% | 0.09% | 0.19% | 0.25% | 99% | 98% | 0.06% | 0.16% | 0.14% | 100% | 100% |
| CE6-1.5d | JVET-M0080 | 0.09% | 0.01% | 0.02% | 96% | 85% | 0.01% | -0.07% | -0.07% | 101% | 98% | 0.07% | 0.04% | 0.05% | 101% | 100% |
| CE6-1.6a | JVET-M0521 | -0.17% | -0.14% | -0.11% | 99% | 103% | -0.06% | 0.05% | 0.04% | 100% | 101% | 0.02% | 0.30% | 0.13% | 100% | 99% |
| CE6-1.6b | JVET-M0521 | -0.12% | -0.18% | -0.16% | 99% | 101% | -0.02% | 0.16% | 0.02% | 100% | 100% | 0.02% | 0.11% | -0.07% | 100% | 100% |
| CE6-1.6c | JVET-M0521 | 0.08% | -0.06% | -0.04% | 101% | 101% | 0.09% | 0.14% | 0.18% | 100% | 100% | 0.07% | 0.00% | 0.18% | 100% | 100% |

For low QP

The following table summarizes the results for CE6-1 using low QP configuration and VTM-3.0 as anchor.

|  |  | All Intra |  |  |  |  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Doc. # | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE6-1.1a | JVET-M0496 | 0.00% | -0.06% | -0.06% | 99% | 100% | 0.00% | -0.02% | -0.01% | 100% | 100% | -0.01% | 0.00% | -0.01% | 101% | 101% |
| CE6-1.1b | JVET-M0496 | 0.24% | 0.00% | 0.01% | 101% | 101% | 0.06% | -0.03% | -0.01% | 100% | 101% | 0.02% | -0.01% | 0.00% | 100% | 101% |
| CE6-1.1c | JVET-M0496 | 0.26% | 0.03% | 0.04% | 102% | 101% | 0.12% | 0.04% | 0.06% | 100% | 101% | 0.05% | 0.06% | 0.07% | 99% | 101% |
| CE6-1.1d | JVET-M0496 | 0.28% | 0.04% | 0.04% | 99% | 100% | 0.13% | 0.05% | 0.06% | 100% | 100% | 0.05% | 0.08% | 0.06% | 100% | 102% |
| CE6-1.1e | JVET-M0084 | 0.41% | 0.20% | 0.20% | 101% | 101% | 0.11% | 0.05% | 0.08% | 100% | 100% | 0.04% | 0.03% | 0.03% | 100% | 100% |
| CE6-1.2a | JVET-M0200 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CE6-1.3a | JVET-M0244 | 0.20% | 0.17% | 0.18% | 99% | 100% |  |  |  |  |  |  |  |  |  |  |
| CE6-1.3b | JVET-M0244 | 0.21% | 0.18% | 0.18% | 99% | 99% |  |  |  |  |  |  |  |  |  |  |
| CE6-1.4a | JVET-M0538 | 0.03% | 0.04% | 0.05% | 87% | 101% | 0.01% | 0.04% | 0.04% | 95% | 104% | 0.00% | 0.01% | 0.01% | 91% | 102% |
| CE6-1.4b | JVET-M0538 | 0.03% | 0.04% | 0.05% | 87% | 102% | 0.01% | 0.04% | 0.05% | 95% | 105% | 0.00% | 0.01% | 0.01% | 91% | 101% |
| CE6-1.4c | JVET-M0538 | 0.13% | 0.18% | 0.18% | 85% | 100% | 0.05% | 0.09% | 0.10% | 94% | 103% | 0.02% | 0.05% | 0.04% | 90% | 101% |
| CE6-1.4d | JVET-M0538 | 0.13% | 0.18% | 0.18% | 85% | 101% | 0.05% | 0.09% | 0.09% | 95% | 104% | 0.02% | 0.04% | 0.03% | 90% | 101% |
| CE6-1.5a | JVET-M0080 | 0.05% | 0.07% | 0.07% | 98% | 100% | 0.02% | 0.08% | 0.07% | 99% | 100% |  |  |  |  |  |
| CE6-1.5b | JVET-M0080 | 0.05% | 0.07% | 0.07% | 99% | 100% | 0.02% | 0.07% | 0.06% | 100% | 100% |  |  |  |  |  |
| CE6-1.5c | JVET-M0080 | 0.23% | 0.28% | 0.28% | 96% | 100% | 0.10% | 0.16% | 0.19% | 99% | 100% |  |  |  |  |  |
| CE6-1.5d | JVET-M0080 | 0.23% | 0.28% | 0.28% | 97% | 100% | 0.10% | 0.16% | 0.19% | 99% | 100% |  |  |  |  |  |
| CE6-1.6a | JVET-M0521 | 0.00% | -0.06% | -0.06% | 94% | 97% | 0.00% | -0.02% | -0.01% | 99% | 105% | -0.01% | 0.00% | -0.01% | 96% | 102% |
| CE6-1.6b | JVET-M0521 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CE6-1.6c | JVET-M0521 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

- Coding efficiency

  - Replacing 4-pt DST-7/DCT-8 by DST-4/DCT4

    - CE6-1.1a/b, CE6-1.6a/b/c

  - Enabling 64-length MTS

    - CE6-1.4 b/d

    - CE6-1.5 b/d

- Simplification

  - Memory reduction

    - CE6-1.1 c/e

    - CE6-1.2 a

    - CE6-1.3 a/b

    - CE6-1.4 a/c, CE6-1.5 a/c

  - Complexity (operation) reduction

    - CE6-1.1 d

    - CE6-1.3 b

    - CE6-1.4 a/c, CE6-1.5a/c

Variant 1.6c tried also replacing the 8-pt and 16-pt in addition to the 4-pt and showed worse performance than the anchor. Variants 1.1b and 1.6b tried also replacing the 8-pt in addition to the 4-pt and did not perform as well as only replacing the 4-pt. It was suggested to focus on the “a” variants of 1.1 and 1.6.

Variants 1.1a and 1.6a are actually the same as each other, only replacing the 4-pt DST-7/DCT-8 by DST-4/DCT4; there is a little gain, but it is small (0.06% in RA, 0.17% in AI, mostly for lower resolutions). It was commented that introducing a design inconsistency was undesirable. It was remarked that the DST-4 and DCT4 are subsets of the 8-point DCT2, which could save some memory (16 bytes) if implemented to take advantage of that. In terms of the amount of computation, there is no difference – it’s just a matter of what numbers are used in a matrix multiply.

A complexity reduction proposal, 1.1e, proposes replacing all sizes (8, 16, and 32 as well as 4). It has a loss of 0.23% for AI, 0.16% for RA (more loss on class A: 0.6% for AI and 0.2-0.3% for RA). Its complexity benefit is reducing storage for the smaller block sizes since the matrix elements for the DST4 and DCT4 of smaller block sizes become subsets of those of larger block sizes of a DCT2. The implementation is a matrix multiply, so it does not affect cycle counts. The loss for that was considered excessive.

In Track A, it was initially planned to adopt 1.1a/1.6a, pending review of other things in CE6. It was later agreed in the plenary on Sunday 13 January not to take this action (see the notes in section 11.1).

[Copy in the short descriptions]

For considering 64-point transform, two basic approaches, 6-1.4 and 6-1.5, are proposed. For the decoder, to approximate an inverse DST7/DCT8:

- 6-1.4 uses an 8x8 “adjustment” transformation followed by a clipping and an inverse DCT2

- 6-1.5 use a forward DCT2 followed by a sparse matrix

For each of these, four sub-variants are tested:

- The scheme is applied for 32 length (and MTS is not used for 64-length)

- The scheme is applied for 32 and 64 lengths

- The scheme is applied for 16 length and 32 length (and MTS is not used for 64-length)

- The scheme is applied for 16, 32, and 64 lengths.

The number computations for the worst was reported to be cut approximately in half by 6-1.4 for the cases where the scheme is applied. In the VTM, the worst case is reportedly the length-32 MTS transform.

A participant commented that these increase the number of sequential stages in a way that is undesirable for latency (esp. in hardware). Another participant said it should be possible to “hide” the extra latency for 6-1.4 in the decoder but maybe not for 6-1.5. On the other hand, encoding latency may more of an issue for 6-1.4 (in hardware). It was asked to allow some time for study to consider that issue.

A participant commented that applying this technique to 16 length is not necessary because 32-length DCT2 also needs to be supported and a subset of the 32 point logic that can be used for the 16 point transform (at least for hardware – perhaps there could be an average benefit for software). Based on this understanding, it was suggested to focus on the “a” and “b” variants.

For the CTC, the “a” variant had roughly no effect on coding efficiency, but a little bit of loss was measured in the low QP case (although the loss is very small ~0.05%). In the discussion it was commented that there is some problem in the software that can cause prohibited “zero-out” coefficient positions to be coded (although very rarely – just once for an entire test sequence). A contributor commented that the loss was not observed if RDOQ is disabled.

A participant said that on some well-known sequences (SteamLocomotive and Nebuta) losses up to 1% were observed at ordinary QP values when the 16 point transform was done using this scheme.

For the “b” variants in CTC testing, 6-1.4b showed a little bit of gain (CTC: 0.1% for AI , 0.1% for RA 0.0% for LB) from enabling the longer transform, but 6-1.5b basically did not (on average). A proponent suggested to focus on class A, which is where a longer transform would seem to be more helpful (0.2% for AI , 0.2% for RA).

After discussion, it was discovered that “6-1.4” was some new proposal that was not really part of the CE. That specific scheme had not been documented in the CE plan. It was expressed that the latency issue would need significant time for detailed study.

For schemes targeting memory reduction, up to about 2.7 kbytes of ROM were reported to potentially be saved by each of the following schemes.

- CE6-1.1 c/e (previously discussed)

- CE6-1.2 a

- CE6-1.3 a/b

- CE6-1.4 a/c (non-CE)

- CE6-1.5 a/c

Except for CE6-1.5, these change the DCT2 so it is not quite a DCT2 anymore.

A loss for low QP in the range of 0.2-0.4% was reported for most of these – not for 6-1.5.

It was commented that implementations might not really have as much ROM savings is appears to be possible, and that the ability to reuse HEVC logic is a nice property of the current VTM.

CE6-1.5 had a cascading of stages (discussed above) that seemed to have a latency issue.

No action was taken after considering these issues.

Next category is fast transforms for DST7/DST8 (DCT is not proposed to be modified).

| CE6.2: Fast transform |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test # | Docs | Description | Tester | Cross-checker |
| 6-2.1a | JVET-M0288 | Fast DST-7/DCT-8 based on DFT Simplification of transform kernel coefficients, intermediate variables, operations, etc. Implementation-friendly architecture based on regular matrix multiply | M. Koo (LGE) | P. Philippe (Orange, bcom) |
| 6-2.2a | JVET-M0372 | JVET-L0421: Fast DST-7/DCT-8 based on DFT and matrix multiplication | K. Naser (Technicolor) | H. Gao (Huawei) |
| 6-2.3a | JVET-M0497 | JVET-L0286: Fast DST-7/DCT-8 with dual implementation support | X. Zhao (Tencent) | K. Choi (Samsung) |
| 6-1.4a | JVET-M0538 | See above description |  |  |
| 6-1.4c |  | See above description |  |  |
| 6-1.5a | JVET-M0080 | See above description |  |  |
| 6-1.5c |  | See above description |  |  |

The following table summarizes the results for CE6-2 using CTC configuration and VTM-3.0 as anchor.

|  |  | All Intra |  |  |  |  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Doc. # | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE6-2.1a | JVET-M0288 | 0.00% | 0.02% | 0.00% | 96% | 90% | -0.01% | 0.01% | 0.07% | 99% | 98% | 0.02% | 0.26% | -0.06% | 100% | 99% |
| CE6-2.2a | JVET-M0372 | 0.08% | 0.05% | 0.04% | 118% | 112% | 0.01% | 0.07% | 0.12% | 116% | 105% | 0.04% | 0.22% | -0.17% | 101% | 100% |
| CE6-2.3a | JVET-M0497 | 0.01% | -0.01% | 0.01% | 96% | 91% | -0.01% | -0.03% | -0.02% | 100% | 100% | 0.00% | 0.22% | -0.22% | 99% | 101% |

The following table summarizes the results for CE6-2 using low QP configuration and VTM-3.0 as anchor.

|  |  | All Intra |  |  |  |  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Doc. # | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE6-2.1a | JVET-M0288 | 0.08% | 0.08% | 0.09% | 98% | 99% |  |  |  |  |  |  |  |  |  |  |
| CE6-2.2a | JVET-M0372 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CE6-2.3a | JVET-M0497 | 0.02% | 0.00% | 0.00% | 98% | 99% | 0.01% | 0.00% | 0.01% | 99% | 99% | 0.00% | -0.01% | 0.00% | 100% | 100% |

The following table summarizes the results for CE6-2 using CTC w/ Inter MTS configuration and VTM-3.0 as anchor.

|  |  | All Intra |  |  |  |  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Doc. # | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE6-2.1a | JVET-M0288 | 0.00% | 0.02% | 0.00% | 96% | 90% | -0.01% | -0.03% | 0.03% | 97% | 98% | 0.01% | 0.11% | -0.19% | 97% | 96% |
| CE6-2.2a | JVET-M0372 | 0.08% | 0.05% | 0.04% | 118% | 112% | 0.03% | 0.00% | 0.12% |  |  | 0.07% | 0.13% | -0.18% |  |  |
| CE6-2.3a | JVET-M0497 | 0.01% | -0.01% | 0.01% | 96% | 91% | 0.00% | -0.04% | -0.04% | 98% | 100% | 0.00% | 0.18% | -0.37% | 97% | 97% |

Some test results were missing.

The reported runtimes were said to not reflect true differences in complexity; rather, they just reflect implementation-specific code optimizations.

6-2.1a and 6-2.2a were said to be very similar.

6-2.3a had identical results with a matrix multiply. It also had no significant penalty in coding efficiency.

A participant remarked that if a scheme requires cascading multiple stages to get the correct result, this would be undesirable, since the fastest implementation in hardware may typically use parallel-processing matrix multiply.

The 6-2.3a scheme would not provide a benefit for some implementations, but would not be worse to implement in any implementations, and would be desirable for some implementations.

From a spec perspective, 6-2.3a is just changing the value of some numbers in the spec by +/-1.

Decision (complexity reduction): Adopt CE6-2.3a.

In the plenary discussion of Sunday 13 January, it was noted that in the AI configuration, this provides a reported 9% speed-up for the decoder, 4% for the encoder (as tested) – see section 11.1.

Then MTS signalling and selection

| CE6.3: Transform signaling |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test # | Docs | Description | Tester | Cross-checker |
| 6-3.1a | JVET-M0303 | Shape adaptive transform selection (up to 32-point DST-7) | J. Lainema (Nokia) | H. Egilmez (Qualcomm) |
| 6-3.1b |  | Shape adaptive transform selection (up to 16-point DST-7) | J. Lainema (Nokia) | H. Egilmez (Qualcomm) |
| 6-3.2a | JVET-M0522 | Applying MTS to smaller side in case of Nx64 and 64xN) for N < 64, where max. MTS size is set to 32 by default | H. Egilmez (Qualcomm) | J. Lainema (Nokia) |
| 6-3.2b |  | Applying MTS for MxN in a way that DCT2 is used for M,N > 16, and MTS is applied for M,N <= 16. | H. Egilmez (Qualcomm) | X. Zhao (Tencent) |
| 6-3.2c |  | MTS with max. MTS size is set to 16 | H. Egilmez (Qualcomm) | J. Lainema (Nokia) |
| 6-3.3a | JVET-M0319 | For 64xN and Nx64 luma transform blocks (N < 64), MTS can be applied to the shorter side. | J. Jung (Wilus) | A. Karabutov (Huawei) |
| 6-3.3b |  | For 64xN and Nx64 luma transform blocks (N<64), MTS can be applied to the shorter side, with the increased max_BT and max_TT sizes for I-slices to 64. | J. Jung (Wilus) | A. Karabutov (Huawei) |
| 6-3.7a | JVET-M0079 | MTS size restriction to 16 | P. Philippe (Orange) | X. Zhao (Tencent) |
| 6-3.8a | JVET-M0498 | Signal horizontal transform if width is less than or equal to 16-point and height is less than or equal to 32, and signal vertical transform if height is less than or equal to 16-point and and width is less than or equal to 32 | J. Jung (Wilus) X. Zhao (Tencent) | P. Philippe (Orange) |
| 6-3.8b |  | Signal horizontal transform if width is less than or equal to 16-point and height is less than or equal to 64, and signal vertical transform if height is less than or equal to 16-point and and width is less than or equal to 64 | J. Jung (Wilus) X. Zhao (Tencent) | P. Philippe (Orange) |

The following table summarizes the results for CE6-3 using CTC configuration and VTM-3.0 as anchor.

|  |  | All Intra |  |  |  |  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Doc. # | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE6-3.1a | JVET-M0303 | -0.08% | 0.02% | 0.00% | 99% | 102% | -0.03% | 0.01% | 0.07% | 101% | 103% | 0.02% | 0.37% | 0.24% | 100% | 99% |
| CE6-3.1b | JVET-M0303 | -0.06% | -0.02% | -0.08% | 100% | 99% | -0.03% | 0.05% | 0.02% | 100% | 101% | 0.02% | 0.18% | -0.01% | 99% | 99% |
| CE6-3.2a | JVET-M0522 | 0.00% | 0.00% | 0.00% | 99% | 99% | -0.04% | -0.09% | -0.16% | 99% | 98% | 0.02% | -0.02% | 0.10% | 101% | 99% |
| CE6-3.2b | JVET-M0522 | 0.33% | 0.40% | 0.38% | 92% | 93% | 0.18% | 0.39% | 0.48% | 97% | 99% | 0.07% | 0.27% | 0.19% | 99% | 99% |
| CE6-3.2c | JVET-M0522 | 0.55% | 0.62% | 0.63% | 86% | 92% | 0.32% | 0.62% | 0.68% | 95% | 99% | 0.12% | 0.49% | 0.16% | 98% | 99% |
| CE6-3.3a | JVET-M0319 | 0.00% | 0.00% | 0.00% | 100% | 100% | -0.04% | -0.09% | -0.16% | 101% | 100% | 0.02% | -0.02% | 0.10% | 100% | 100% |
| CE6-3.3b | JVET-M0319 | -0.65% | -1.51% | -1.56% | 154% | 100% | -0.18% | -0.66% | -0.71% | 103% | 100% | 0.00% | -0.48% | -0.57% | 101% | 100% |
| CE6-3.7a | JVET-M0079 | 0.55% | 0.62% | 0.63% | 86% | 87% | 0.32% | 0.62% | 0.68% | 95% | 98% | 0.12% | 0.49% | 0.16% | 98% | 99% |
| CE6-3.8a | JVET-M0498 | 0.33% | 0.40% | 0.38% | 91% | 92% | 0.18% | 0.39% | 0.48% | 98% | 99% | 0.07% | 0.27% | 0.19% | 99% | 100% |
| CE6-3.8b | JVET-M0498 | 0.33% | 0.40% | 0.38% | 91% | 93% | 0.17% | 0.29% | 0.43% | 99% | 99% | 0.06% | 0.29% | 0.22% | 99% | 100% |

The following table summarizes the results for CE6-3.1 using CTC w/ MTS = 0 configuration for both test and the VTM-3.0 anchor.

|  |  | All Intra |  |  |  |  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Doc. # | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE6-3.1a | JVET-M0303 | -1.66% | -2.31% | -2.28% | 99% | 119% | -0.75% | -0.87% | -0.96% | 100% | 108% | -0.09% | -0.18% | -0.33% | 99% | 102% |
| CE6-3.1b | JVET-M0303 | -1.61% | -2.11% | -2.12% | 101% | 111% | -0.71% | -0.81% | -0.84% | 100% | 102% | -0.14% | -0.02% | -0.33% | 100% | 100% |

6-3.1 is motivated by improving coding efficiency (esp. for the case with MTS off). The others in this category are to reduce the maximum MTS transform size (eliminating the 32-point transform).

6-3.1 provides substantial coding gain for when MTS is disabled.

When MTS is disabled or when it is enabled but the low-level MTS flag is 0:

- The CE6-3.1b variant always uses DCT2 when the length of the transform is 32 (or 64).

- The CE6-31.a variant uses 2 transforms of length 32 (but only DCT for length 64).

When the low-level MTS flag is 1, the current 4 combinations are selectable (including for length 32)

Decision: Adopt CE6-3.1b, but with an extra high-level flag to use DCT2 always.

See also the notes of the further discussion of this topic in the plenary of Sunday 13 January in section 11.1.

CE6-3.2b/c, CE6-3.7a, CE6-3.8a/b limit the maximum non-DCT2 transform size to 16. All of these have a loss of more than 0.5% for AI in Class A, which seems unacceptable, so no action was taken on these.

CE6-3.2a, CE6-3.3a/b, use transform type signalling only on one side when one side is length 32 or smaller but the other side 64 long. If both sides are small, signalling is used in both dimensions. The gain for these is approximately negligible (no gain for AI since that case cannot occur, 0.04% in RA when compared to the current MTS, 0.03% benefit in LB when compared to the current MTS) and the encoder runtime is increased by the consideration of these additional cases. So no action was taken on these.

(CE6-3.8a/b also has some usage of signalling for only one direction, but is motivated by trying to eliminate the long non-DCT2 transforms, as discussed above – it has some compression loss.)

CE6-3.3b has gain from two changes: 1) increasing the maximum BT/TT size for I pictures from 32x32 to 64x64, and 2) the signalling scheme from CE6-3.3a. One of these is just an encoder configuration setting. The encoding runtime increased by 54% for AI, with a coding efficiency benefit of 0.65%. The benefit reported for the second modification when using the first modification as an anchor was reportedly 0.08% for AI. Since this affects only a case we chose not to use in CTC and provides only a small benefit, no action was taken on that.

| CE6.4: Sub-block transform |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test # | Docs | Description | Tester | Cross-checker |
| 6-4.1a | JVET-M0140 | Sub-block Transform (SBT) for inter blocks - 1-d split (symmetric or 1/4) - if symmetric, signal which half; others use the 1/4 - transform type of residual TU inferred | Y. Zhao (Huawei) | X. Zhao (Tencent) |
| 6-4.1b |  | Sub-block Transform (SBT) for inter blocks - transform type of residual TU always DCT-2 | Y. Zhao (Huawei) | C.-M. Tsai (MediaTek) |
| 6-4.1c |  | Sub-block Transform (SBT) for inter blocks - transform type of residual TU signaled (two transform candidates each TU) | Y. Zhao (Huawei) | K. Choi (Samsung) |
| 6-4.1d |  | Sub-block Transform (SBT) for inter blocks - transform type of residual TU signaled (four transform candidates each TU, like inter MTS but with more complex context modeling) | Y. Zhao (Huawei) | M. Ikeda (Sony) |
| 6-4.1e |  | Sub-block Transform (SBT) for inter blocks - splits of 6-4.1a are allowed, plus - with Quad-tree split (max TU depth still 1) - transform type of residual TU inferred | Y. Zhao (Huawei) | X. Zhao (Tencent) |
| 6-4.2a | JVET-M0523 | RQT-like concept of transform sub-block splitting - One 4-way QT split | Qualcomm | Y. Zhao (Huawei) |
| 6-4.3a | JVET-M0499 | RQT-like concept of transform sub-block splitting - Square blocks have one 4-way QT split - Rectangular blocks have one long-dimension binary split | X. Zhao (Tencent) | Y. Zhao (Huawei) |
| 6-4.4a | JVET-M0141 | RQT-like concept of transform sub-block splitting - 4 choices – binary or ternary split (horizontally or vertically) | Y. Zhao (Huawei) | H. Egilmez (Qualcomm) |

Two approaches to sub-block transform:qq

- Only one sub-block has non-zero coefficients:

  - CE6-4.1a/b/c/d/e

- Each sub-block can have coefficients, with RQT-like scheme

  - CE6-4.2a

  - CE6-4.3a

  - CE6-4.4a

These are all targeting higher coding gain and most are only applicable with inter prediction. Less coding gain is observed when inter-MTS is enabled.

It was suggested to focus on the inter-MTS reference case.

Among the 4.1a/b/c/d/e variations, we would rule out b as not performing so well, and rule out c and d as requiring signalling that doesn’t provide much benefit and adds encoder search complexity and increases decoder support combinations. Variation e is a superset of variation a, adding a 4-way split that provides a small additional gain (0.06% in RA when MTS is enabled).

It was remarked that the length 32 non-DCT2 transform is a problem, and the 4.1a/e schemes are using that transform. It was requested to run a variation where the 32-point DST is not used for 32x64 and 64x32, which are cases not supported in the current VTM and are difficult cases. It was expected that this restriction would have a very small impact. The proponent said they could run a test of that. Revisit for test results on that.

The 6-4.4a scheme has 4 choices to make within the sub-block transform mode. The 6-4.2a and 6-4.3a schemes have no choices needed by the encoder and no syntax for such choices.

4.1a/e, 6-4.3a and 6-4.4a are splitting both luma and chroma together, while 6-4.2a is applying the scheme only to luma.

6-4.3a is applying the scheme to intra CUs and the others are not.

Some data was missing for 6-4.3a.

It was commented that 4.1a had been studied quite a bit. It was in a CfP response and has been tested in two CE meeting cycles.

Decision (coding efficiency): Adopt CE6-4.1a (pending the test results of avoiding 32-point DST). The same high-level flag as used for CE6-3.1b is to be used to determine whether DCT2 is used always or not and also applies to CE3-1.1.1.

It was also agreed to not have another CE study of sub-block transform schemes for the next meeting cycle.

The following table summarizes the results for CE6-4 using CTC configuration and VTM-3.0 as anchor.

|  |  | All Intra |  |  |  |  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Doc. # | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE6-4.1a | JVET-M0140 |  |  |  |  |  | -0.47% | -0.16% | 0.00% | 108% | 101% | -0.83% | -0.98% | -0.06% | 113% | 102% |
| CE6-4.1b | JVET-M0140 |  |  |  |  |  | -0.25% | -0.16% | -0.07% | 107% | 101% | -0.42% | -0.89% | -0.04% | 111% | 101% |
| CE6-4.1c | JVET-M0140 |  |  |  |  |  | -0.54% | -0.20% | 0.08% | 110% | 101% | -0.94% | -0.75% | 0.23% | 116% | 102% |
| CE6-4.1d | JVET-M0140 |  |  |  |  |  | -0.52% | -0.10% | 0.14% | 114% | 101% | -0.90% | -0.70% | 0.25% | 121% | 102% |
| CE6-4.1e | JVET-M0140 |  |  |  |  |  | -0.55% | -0.26% | 0.00% | 109% | 101% | -0.93% | -0.91% | 0.07% | 115% | 102% |
| CE6-4.2a | JVET-M0523 |  |  |  |  |  | -0.36% | 0.09% | 0.35% | 122% | 102% | -0.59% | -0.65% | 0.47% | 129% | 101% |
| CE6-4.2a + InterMTS | JVET-M0523 |  |  |  |  |  | -0.62% | 0.61% | 0.80% | 146% | 105% | -0.94% | 0.68% | 1.41% | 161% | 107% |
| CE6-4.3a | JVET-M0499 | -0.29% | -0.16% | -0.17% | 192.17% | 107% | -0.42% | -0.30% | -0.10% | 143% | 103% | -0.45% | -0.64% | -0.01% | 142% | 104% |
| CE6-4.4a | JVET-M0141 |  |  |  |  |  | -0.59% | -0.35% | 0.10% | 120% | 102% | -1.01% | -1.77% | -0.19% | 127% | 103% |

The following table summarizes the results for CE6-4 using CTC w/ Inter MTS configuration and VTM-3.0 as anchor.

|  |  | All Intra |  |  |  |  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Doc. # | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE6-4.1a | JVET-M0140 |  |  |  |  |  | -0.28% | -0.21% | -0.06% | 98% | 101% | -0.43% | -0.72% | -0.26% | 96% | 100% |
| CE6-4.1b | JVET-M0140 |  |  |  |  |  | -0.12% | -0.15% | -0.06% | 97% | 101% | -0.20% | -0.67% | -0.21% | 95% | 101% |
| CE6-4.1c | JVET-M0140 |  |  |  |  |  | -0.33% | -0.25% | 0.03% | 100% | 100% | -0.53% | -0.88% | -0.28% | 98% | 101% |
| CE6-4.1d | JVET-M0140 |  |  |  |  |  | -0.29% | -0.14% | 0.03% | 103% | 100% | -0.47% | -0.64% | -0.06% | 102% | 100% |
| CE6-4.1e | JVET-M0140 |  |  |  |  |  | -0.34% | -0.23% | -0.06% | 99% | 101% | -0.55% | -0.73% | -0.21% | 97% | 101% |
| CE6-4.2a | JVET-M0523 |  |  |  |  |  | -0.25% | 0.06% | 0.24% | 120% | 103% | -0.40% | -0.23% | 0.31% | 124% | 101% |
| CE6-4.3a | JVET-M0499 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CE6-4.4a | JVET-M0141 |  |  |  |  |  | -0.30% | -0.29% | 0.01% | 107% | 101% | -0.52% | -1.32% | -0.54% | 105% | 102% |

| CE6.5: Secondary transform |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test # | Docs | Description | Tester | Cross-checker |
| 6-5.1 | JVET-M0292 | Reduced Secondary Transform (RST) - Searching best configurations of RST parameters, such as the number of transform kernels, transform set, and matrix dimension. - RST index signalling optimization | M. Koo (LGE) | P. Philippe (Orange, bcom) |

In JVET-M0292, test results of CE 6-5.1 on reduced secondary transform (RST) are reported. According to the request of the previous 12th JVET meeting, all tests were performed without normative change of MTS signaling, which are described as the following:

- Test 1:  (A),

- Test 2:  (A) + (B),

- Test 3:  (A) + (B) + (C),

- Test 4:  (A) + (B) + (D), where

| Feature | Description |
| --- | --- |
| (A) | 4 transform sets (instead of 35). 2 transforms per set |
| (B) | Secondary transform uses at most maximum 8 multiplications/sample |
| (C) | Secondary transform is disabled for 4x4 TU |
| (D) | 16x48 matrices are employed instead of 16x64 ones |

The transform set is determined from the intra prediction mode, then a syntax flag is sent to select which kernel in that set is to be applied. The “secondary” (inverse) transform is applied first in the decoding process and then the ordinary (inverse) transform is applied.

The following table summarizes the results for CE6-1 using CTC configuration and VTM-3.0 as anchor.

|  |  | All Intra |  |  |  |  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Doc. # | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE6-5.1 (Test 1) | JVET-M0292 | -1.59% | -2.75% | -3.22% | 128% | 98% | -0.88% | -1.88% | -2.27% | 108% | 100% | -0.24% | -0.83% | -1.19% | 104% | 101% |
| CE6-5.1 (Test 2) | JVET-M0292 | -1.40% | -2.60% | -3.09% | 128% | 97% | -0.76% | -1.83% | -2.25% | 108% | 100% | -0.21% | -0.78% | -0.85% | 104% | 98% |
| CE6-5.1 (Test 3) | JVET-M0292 | -1.25% | -2.48% | -2.88% | 125% | 97% | -0.69% | -1.59% | -2.03% | 107% | 100% | -0.18% | -0.68% | -1.11% | 103% | 100% |
| CE6-5.1 (Test 4) | JVET-M0292 | -1.34% | -2.50% | -2.95% | 129% | 97% | -0.71% | -1.60% | -2.02% | 107% | 100% | -0.18% | -0.35% | -1.00% | 104% | 99% |

The following table summarizes the results for CE6-1 using CTC w/ Inter MTS configuration and VTM-3.0 as anchor.

|  |  | All Intra |  |  |  |  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Doc. # | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE6-5.1 (Test 1) | JVET-M0292 |  |  |  |  |  | -0.91% | -1.94% | -2.34% | 106% | 100% | -0.29% | -0.84% | -1.37% | 103% | 100% |
| CE6-5.1 (Test 2) | JVET-M0292 |  |  |  |  |  | -0.80% | -1.89% | -2.27% | 106% | 99% | -0.24% | -0.66% | -1.21% | 103% | 98% |
| CE6-5.1 (Test 3) | JVET-M0292 |  |  |  |  |  | -0.74% | -1.61% | -2.03% | 105% | 99% | -0.22% | -0.64% | -1.21% | 102% | 100% |
| CE6-5.1 (Test 4) | JVET-M0292 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

A cross-checker said the gains are about the same regardless of the test sequence class. Chroma gain as well as luma gain was observed.

The scheme is only applied to intra.

Encoder complexity is increased due to checking both selectable transforms.

It was commented that the encoder runtime tradeoff looks reasonable.

This has some table storage requirement (about 10 kbytes) and a multi-stage latency (but not in the critical path of latency).

The A and B aspects had been tested before.

It was commented that the CE plan did not describe a specific scheme to be tested – a substantial amount of specific design aspects and details were not in the description (the values to be used in the matrices and other aspects).

At the previous meeting it had been suggested to

- Reduce the table size (5 kbytes and 10 kbytes had previously been proposed, where the 5 kbyte scheme had only one candidate secondary transform in each “set” rather than two to choose from); the current proposal has 10 kbytes or with variant D has 7.6 kbytes

- Reduce the number samples to process with the secondary transform – e.g., applying the secondary transform to only a minimally small subset of the coefficients of the primary transform. In the tested scheme the maximum input vector size is 16 coefficients.

It was remarked that since other changes to the transform have been made, there could be interactions.

It was also remarked that the AI encoding runtime impact is high ~28% and other encoding runtime increases have been adopted, so this could result in an excessive speed impact.

Revisit after offline study to determine next steps.

JVET-M0079 CE6: MTS size restriction to 16 (test 3.7) [P. Philippe (bcom Orange)]

JVET-M0080 CE6: MTS simplification with TAF (tests 1.5a-d) [P. Philippe (bcom Orange)]

JVET-M0084 CE6: JVET-L0262: Replacing all DST-7 / DCT-8 by DST-4 / DCT-4 used in MTS (test 6.1.1e) [K. Abe, T. Toma (Panasonic), M. Ikeda, T. Tsukuba (Sony), K. Naser, F. Le Léannec, E. François (Technicolor)]

JVET-M0140 CE6: Sub-block transform for inter blocks (Test 6.4.1) [Y. Zhao, H. Gao, H. Yang, J. Chen (Huawei)]

JVET-M0141 CE6: RQT-like sub-block transform for inter blocks (Test 6.4.4) [Y. Zhao, H. Gao, H. Yang, J. Chen (Huawei)]

JVET-M0200 CE6: Unified matrix for transform (Test 6-1.2a) [K. Choi, M. Park, M. W. Park, W. Choi (Samsung)]

JVET-M0244 CE6: MTS using DST-4 and transposed DCT-2 (test 6-1.3) [Y. Lin, J. Zheng, Q. Yu, N. Zhang (HiSilicon), C. Zhu (UESTC)]

JVET-M0288 CE6-2.1: Fast DST-7/DCT-8 based on DFT [M. Koo, M. Salehifar, J. Lim, S. Kim (LGE)]

JVET-M0292 CE6-5.1: Reduced Secondary Transform (RST) [M. Koo, M. Salehifar, J. Lim, S. Kim (LGE)]

JVET-M0303 CE6: Shape adaptive transform selection (Test 3.1) [J. Lainema (Nokia)]

JVET-M0319 CE6: MTS for non-square CUs (test 6.3.3) [J. Jung, D. Kim, G. Ko, J. Son, J. Kwak (WILUS)]

JVET-M0372 CE6-2.2: Fast DST-7/DCT-8 based on DFT and matrix multiplication [K. Naser, E. François, F. Le Léannec (Technicolor)]

JVET-M0496 CE6: Compound Orthonormal Transform (Test 6.1.1 a/b/c/d) [X. Zhao, X. Li, S. Liu (Tencent)]

JVET-M0497 CE6: Fast DST-7/DCT-8 with dual implementation support (Test 6.2.3) [X. Zhao, X. Li, Y. Luo, S. Liu (Tencent)]

JVET-M0498 CE6: MTS up to 16-length (Test 6.3.8) [J. Jung, D. Kim, G. Ko, J.-H. Son, J. S. Kwak (Wilus), X. Zhao, X. Li, S. Liu (Tencent)]

JVET-M0499 CE6: RQT-like transform sub-block splitting (Test 6.4.3) [X. Zhao, X. Li, S. Liu (Tencent)]

JVET-M0521 CE6: Replacement of 4-point DST7/DCT8 with DST4/DCT4 in MTS (Test 6.1.6) [H. Egilmez, V. Seregin, A. Said, T. Hsieh, M. Karczewicz (Qualcomm)]

JVET-M0522 CE6: MTS support for large rectangular blocks (Test 6.3.2) [H. Egilmez, V. Seregin, A. Said, M. Karczewicz (Qualcomm)]

JVET-M0523 CE6: RQT-like transform partitioning for inter blocks (Test 6.4.2) [H. Egilmez, V. Seregin, A. Said, M. Karczewicz (Qualcomm)]

## CE7: Quantization and coefficient coding (2)

Contributions in this category were discussed Saturday 12 Jan. 1500–XXXX (chaired by GJS).

JVET-M0027 CE7: Summary report on quantization and coefficient coding [H. Schwarz, M. Coban, C. Auyeng]

The CE report summarizes the test results and crosschecks reports for CE7 on quantization and coefficient coding.  The CE includes 4 tests on transform coefficient coding.

The purpose of this core experiment is to explore the coding efficiency and complexity impact of proposed algorithms for quantization and transform coefficient coding. This core experiment evaluates the impact of:

Context modeling/selection/coding order for syntax elements related to transform coefficients

The objectives of this CE include:

Investigation of the impact of context selection and signaling schemes for transform coefficient levels on the coding efficiency and complexity.

Transform coefficient coding

|  | Tester | Tool | Cross checker |
| --- | --- | --- | --- |
| 7.1 | Tzu-Der Chuang (MediaTek) | [1] Constraints on the number of context-coded bins depend on whether luma or chroma is coded (and, for chroma, on the subblock size) | Yu-Chen Sun (Alibaba) |
| 7.2 | Tzu-Der Chuang (MediaTek) | [1] Constraints on the number of context-coded bins depend on whether luma or chroma is coded (and, for chroma, on the subblock size) + [2] Constraints on the number of context-coded bins depend on last significant scan position | Jung-Ah Choi (LGE) M. Coban (Qualcomm) |
| 7.3 | Tzu-Der Chuang (MediaTek) | [1] Constraints on the number of context-coded bins depend on whether luma or chroma is coded (and, for chroma, on the subblock size) + [2] Constraints on the number of context-coded bins depend on last significant scan position + [3] Include rem_abs_gt2_flag into first coding pass | H. Schwarz (HHI) M. Coban (Qualcomm) |
| 7.4 | Tzu-Der Chuang (MediaTek) | [3] Include rem_abs_gt2_flag into first coding pass | H. Schwarz (HHI) |

In JVET-L0145, three methods to constrain the usage of context-coded bins are proposed. The first constraint is dependent on color component and coefficient sub-block size. The second method is to release the constraints values according to the last significant sub-block position to improve the coding efficiency. The third modification is to move the greater than 2 flag into the first pass to further improve the decoding throughput. The details are described as follow.

[1] Different constraints according to colour components and sub-block sizes

The number of context-coded bins (including greater than 2 flag) constraints are set to 30 on 4x4 luma subblocks, 16 on 4x4 chroma subblocks, and 4 on 2x2 chroma subblocks, respectively. The number of context-coded bin constraints on greater than 2 are set to 4 on 4x4 luma subblocks, 2 on 4x4 chroma subblocks, and 1 on 2x2 chroma sub-blocks, respectively.

[2] Last significant subblock position dependent constraint

According to the last significant subblock position, the number of to-be-coded sub-blocks in a TB can be derived. The constraint value is determined according to the color component and the coefficient sub-block size. With the number of to-be-coded sub-blocks (NumToBeCodedSb) and the number of total coefficient sub-blocks in a TB (NumTotalSb), the constraint value (ConstraintValue) is modified as follows.

if (NumToBeCodedSb * 2 <= NumTotalSb)

ConstraintValue = ConstraintValue * 2;

else if (NumToBeCodedSb * 5 <= NumTotalSb * 4)

ConstraintValue = (ConstraintValue * 5) >> 2;

[3] Include greater than 2 flag into the first coding pass

The greater than 2 flag is proposed to be moved to the first coding pass after the parity bit. The parsed greater than 2 flag is used to calculate the locSumAbsPass1in the context modeling of sig_coeff_flag, par_level_flag, rem_abs_gt1_flag, and rem_abs_gt2_flag. The reserved number of context-coded bins for greater than 2 flag in the second coding pass is also merged into the number of context-coded bins for the first coding pass. The test that moving the greater than 2 flag before parity bit will also be evaluated.

Specification of tests:

Test 7.1: Aspect [1]		To reduce worst-case context coded bins (2 per coeff to ~1.6)

Test 7.2: Aspects [1] + [2]		Improving coding efficiency relative to 7.1

Test 7.3: Aspects [1] + [2] + [3]	Reduces the scan passes relative to 7.2

Test 7.4: Aspect [3]		Reduces the scan passes without including other changes

Average test results for CE 7. The table shows results for a high-complexity encoder configurations (same as CTC) as well as a low-complexity encoder configuration (dependent quantization and RDOQ disabled). All results are relative to VTM-3.0 (using the same configuration as the tested approach).

| CE 7 |  | High complexity (CTC) |  |  |  |  | Low complexity (DQ off, RDOQ off) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | encT | decT | Y | U | V | encT | decT |
| AI | 7.1 | 0.08% | 0.64% | 0.44% | 102% | 101% | 0.12% | 0.42% | 0.47% | 100% | 101% |
|  | 7.2 | 0.06% | 0.19% | 0.10% | 104% | 102% | 0.06% | 0.12% | 0.14% | 100% | 102% |
|  | 7.3 | 0.00% | 0.09% | 0.05% | 103% | 101% | 0.00% | 0.03% | 0.11% | 99% | 101% |
|  | 7.4 | -0.08% | -0.09% | -0.10% | 100% | 101% | -0.05% | -0.08% | -0.08% | 99% | 101% |
| RA | 7.1 | 0.07% | 0.87% | 0.69% | 101% | 100% | 0.09% | 0.45% | 0.58% | 100% | 102% |
|  | 7.2 | 0.03% | 0.35% | 0.30% | 101% | 101% | 0.03% | 0.15% | 0.22% | 100% | 102% |
|  | 7.3 | 0.00% | 0.30% | 0.23% | 101% | 102% | 0.02% | 0.14% | 0.17% | 100% | 102% |
|  | 7.4 | -0.06% | 0.03% | -0.06% | 100% | 100% | -0.02% | -0.12% | -0.02% | 100% | 102% |
| LB | 7.1 | 0.09% | 1.58% | 0.89% | 101% | 102% | -0.02% | 1.03% | 0.66% | 100% | 100% |
|  | 7.2 | 0.00% | 0.57% | 0.14% | 102% | 101% | -0.01% | 0.40% | 0.13% | 100% | 99% |
|  | 7.3 | 0.02% | 0.58% | -0.12% | 101% | 100% | -0.01% | 0.50% | 0.09% | 100% | 99% |
|  | 7.4 | -0.04% | 0.00% | 0.12% | 100% | 100% | 0.02% | -0.15% | -0.09% | 100% | 99% |

For test 7.1, there is clear loss and especially in Class A.

It was asked whether this had been tested with low QPs. It had not.

The current scheme imposes this limit by counting the number of context coded bins in a coefficient group and switching to bypass when a limit is exceeded.

It was commented that limiting the context coded bins at this very low level – the 4x4 level, may not be desirable – that imposing such a limit at a higher level may be better. However, it was commented by others that HEVC has a similar counting and switching requirement.

Aspect number [3] (test CE7.4), which reduces scan passes, was suggested to be “cleaner” and should increase throughput.

Decision (complexity reduction): Adopt CE7.4.

There was interest in further work for reducing the worst-case number of context-coded bins, but the currently tested methods may not be the right approach for that.

A suggestion for the further study was to establish a limit at some higher level rather than on each coefficient group. It was recommended to study the matter of the worst-case number of context-coded bins in an AHG.

JVET-M0173 CE7 (Tests 7.1, 7.2, 7.3, and 7.4): Constraints on context-coded bins for coefficient coding [T.-D. Chuang, S.-T. Hsiang, Z.-Y. Lin, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

## CE8: Screen content coding tools (15)

Contributions in this category were discussed Friday 11 Jan. 1120–1330 and 1500-1615 (Track B chaired by JRO).

JVET-M0028 CE8: Summary Report on Screen Content Coding [X. Xu, Y.-C. Chao, Y.-C. Sun, J. Xu]

8.1: CPR related

8.2: Palette related

8.3: Block-based DPCM

| Test | Tester | Document | Tool description | Cross checker |
| --- | --- | --- | --- | --- |
| 8.1.1a | J. Nam (LGE) | JVET-M0332 | Block vector prediction for merge mode and AMVP mode using default positions. | X. Xu (Tencent) |
| 8.1.1b | J. Nam (LGE) | JVET-M0332 | Block vector prediction for merge mode and AMVP mode. Consider alternative candidates with various positions | A. Karabutov (Huawei) |
| 8.1.2a | X. Xu (Tencent) | JVET-M0407 | Reference sample memory reuse, update reference sample memory on 64x64 basis, keep the requirement for total memory (4 64x64 blocks) unchanged | A. Karabutov (Huawei) |
| 8.1.2b | X. Xu (Tencent) | JVET-M0408 | Reference sample memory reuse, update reference sample memory on 64x64 basis, reduce the requirement for total memory to 3 64x64 blocks | J. Nam (LGE) |
| 8.1.2c | X. Xu (Tencent) | JVET-M0408 | Reference sample memory reuse, update reference sample memory on 64x64 basis, reduce the requirement for total memory to 2 64x64 blocks | J. Nam (LGE) |
| 8.1.2d | X. Xu (Tencent) | JVET-M0407 | Reference sample memory reuse, update reference sample memory on CU basis, keep the requirement for total memory (4 64x64 blocks) unchanged | A. Karabutov (Huawei) |
| 8.1.3 | L. P. Van (Qualcomm) | JVET-M0474 | CPR using extended search range with line buffers from top of CTU and the left columns of the current CTU |  |
| 8.2.1 | Y.-H. Chao (Qualcomm) Y.-C. Sun (Alibaba) | JVET-M0050 | Palette mode as in HEVC SCC, CE base software | R. Chernyak (Huawei) |
| 8.2.2 | Y.-C. Sun (Alibaba) | JVET-M0051 | Palette mode and intra mode combination | Y.-W. Chen (Kwai) |
| 8.2.3 | J. Ye (Tencent) | JVET-M0455 | Palette index map scan order constraints | R. Chernyak (Huawei) |
| 8.2.4 | J. Ye (Tencent) | JVET-M0456 | Apply palette mode on separate chroma CU only when corresponding luma samples are all coded in palette mode | Y.-C. Sun (Alibaba) |
| 8.2.5a | R. Chernyak (Huawei) Y.-H. Chao (Qualcomm) Y.-C. Sun (Alibaba) | JVET-M0052 | Separate palette coding for luma and chroma Sub test 1: palette coding is applied separately on luma and chroma components in both intra and inter slices when dual tree is disabled in configuration (SPS) | J. Ye (Tencent) |
| 8.2.5b | R. Chernyak (Huawei) Y.-H. Chao (Qualcomm) Y.-C. Sun (Alibaba) | JVET-M0052 | Separate palette coding for luma and chroma Sub test 2: palette coding is applied separately on luma and chroma components in both intra and inter slices when dual tree is enabled in configuration (SPS) | J. Ye (Tencent) |
| 8.2.6 | J. Ye (Tencent) | JVET-M0457 | Palette predictor list enhancement by using palette coded spatial neighbors | Y.-C. Sun (Alibaba) |
| 8.3.1a | F. Henry (Orange) | JVET-M0056 | BDPCM with throughput reduction, investigating: - Block size limitation of BDPCM and impact on throughput - Independently decodable regions inside BDPCM block (in a manner similar to RDPCM throughput reduction proposed in JCT-VC M0439 [16]) and impact on throughput | B. Bross (HHI) |
| 8.3.1b | F. Henry (Orange) | JVET-M0057 | Same as 8.3.1a with vertical and horizontal predictors (in a way similar to RDPCM), allowing the reconstruction to proceed line by line / column by column and thus increase throughput | B. Bross (HHI) |
| 8.3.2 | F. Henry (Orange) | JVET-M0058 | BDPCM bin/context reduction - Use different approaches to encode the BDPCM residual in order to reduce the number of bins and associated contexts | B. Bross (HHI) |

Results compared to CTC (without CPR), all CE results have CPR enabled

|  |  | AI Over VTM-3.0 |  |  |  |  | RA Over VTM-3.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CTC overall | VTM+CPR | -0.27% | -0.40% | -0.33% | 137% | 100% | 0.09% | -0.01% | 0.05% | 100% | 100% |
|  | CE8.1.1a | -0.34% | -0.43% | -0.41% | 144% | 104% | 0.05% | -0.06% | 0.00% | 101% | 101% |
|  | CE8.1.1b | -0.34% | -0.41% | -0.37% | 144% | 104% | 0.05% | 0.00% | 0.05% | 101% | 102% |
|  | CE8.1.2a | -0.35% | -0.46% | -0.42% | 142% | 102% | 0.05% | -0.04% | -0.03% | 100% | 99% |
|  | CE8.1.2b | -0.31% | -0.41% | -0.35% | 143% | 98% | 0.07% | -0.04% | 0.04% | 99% | 98% |
|  | CE8.1.2c | -0.22% | -0.30% | -0.30% | 139% | 100% | 0.11% | -0.07% | 0.08% | 99% | 99% |
|  | CE8.1.2d | -0.39% | -0.48% | -0.48% | 143% | 99% | 0.05% | 0.01% | 0.02% | 99% | 99% |
|  | CE8.1.3 | -0.29% | -0.37% | -0.34% | 137% | 103% | 0.08% | 0.01% | 0.02% | 100% | 100% |
|  | CE8.2.1 | -0.19% | -0.26% | -0.26% | 146% | 105% | 0.20% | 0.07% | 0.20% | 108% | 104% |
|  | CE8.2.1* | -0.36% | -0.65% | -0.81% | 142% | 100% | 0.16% | -0.30% | -0.22% | 107% | 102% |
|  | CE8.2.2 | -0.19% | -0.24% | -0.26% | 147% | 105% | 0.22% | 0.07% | 0.18% | 106% | 103% |
|  | CE8.2.3 | -0.19% | -0.26% | -0.25% | 142% | 104% | 0.20% | 0.05% | 0.15% | 100% | 99% |
|  | CE8.2.4 | -0.21% | -0.30% | -0.26% | 141% | 105% | 0.20% | 0.10% | 0.22% | 101% | 99% |
|  | CE8.2.5a | -0.36% | -0.63% | -0.81% | 142% | 100% | 0.15% | -0.32% | -0.24% | 106% | 101% |
|  | CE8.2.5b | -0.19% | -0.26% | -0.26% | 146% | 105% | 0.20% | 0.05% | 0.13% | 105% | 103% |
|  | CE8.2.6 | -0.19% | -0.27% | -0.26% | 143% | 104% | 0.20% | 0.08% | 0.18% | 101% | 100% |
|  | CE8.3.1a | -0.24% | -0.42% | -0.36% | 134% | 104% | 0.11% | 0.03% | 0.09% | 101% | 102% |
|  | CE8.3.1b | -0.34% | -0.37% | -0.32% | 137% | 106% | 0.07% | -0.01% | 0.09% | 102% | 102% |
|  | CE8.3.2 | -0.34% | -0.37% | -0.33% | 137% | 106% | 0.07% | -0.01% | 0.08% | 102% | 102% |
| Class F | VTM+CPR | -12.09% | -12.04% | -12.10% | 158% | 99% | -9.89% | -9.92% | -9.95% | 107% | 99% |
|  | CE8.1.1a | -12.42% | -12.24% | -12.27% | 161% | 104% | -10.10% | -9.99% | -10.12% | 108% | 102% |
|  | CE8.1.1b | -12.41% | -12.37% | -12.34% | 162% | 106% | -10.07% | -9.94% | -10.16% | 108% | 102% |
|  | CE8.1.2a | -14.47% | -14.32% | -14.26% | 175% | 103% | -11.62% | -11.63% | -11.49% | 109% | 98% |
|  | CE8.1.2b | -13.66% | -13.52% | -13.49% | 172% | 97% | -11.04% | -11.05% | -11.07% | 109% | 97% |
|  | CE8.1.2c | -12.11% | -12.04% | -11.94% | 164% | 97% | -9.88% | -9.89% | -10.04% | 108% | 98% |
|  | CE8.1.2d | -15.34% | -15.20% | -15.15% | 178% | 96% | -12.34% | -12.28% | -12.22% | 109% | 97% |
|  | CE8.1.3 | -12.21% | -12.14% | -12.19% | 159% | 104% | -9.96% | -10.10% | -9.99% | 106% | 101% |
|  | CE8.2.1 | -15.51% | -14.05% | -14.13% | 174% | 97% | -11.90% | -11.77% | -11.99% | 119% | 103% |
|  | CE8.2.1* | -13.16% | -13.63% | -13.62% | 159% | 96% | -10.48% | -11.17% | -11.20% | 118% | 102% |
|  | CE8.2.2 | -15.53% | -14.03% | -14.11% | 174% | 97% | -11.94% | -11.72% | -11.94% | 119% | 103% |
|  | CE8.2.3 | -15.50% | -14.03% | -14.09% | 173% | 100% | -11.90% | -11.72% | -11.92% | 110% | 98% |
|  | CE8.2.4 | -15.62% | -13.29% | -13.31% | 169% | 99% | -12.03% | -11.00% | -11.12% | 110% | 98% |
|  | CE8.2.5a | -14.59% | -14.28% | -14.61% | 167% | 93% | -11.43% | -11.08% | -11.70% | 118% | 99% |
|  | CE8.2.5b | -15.51% | -14.05% | -14.13% | 174% | 97% | -11.97% | -11.64% | -11.89% | 118% | 104% |
|  | CE8.2.6 | -15.49% | -13.98% | -14.09% | 175% | 100% | -11.87% | -11.53% | -11.90% | 110% | 99% |
|  | CE8.3.1a | -13.09% | -13.59% | -13.60% | 136% | 103% | -10.41% | -10.92% | -10.98% | 109% | 101% |
|  | CE8.3.1b | -15.75% | -14.12% | -14.14% | 139% | 106% | -12.48% | -11.27% | -11.42% | 111% | 102% |
|  | CE8.3.2 | -15.68% | -14.05% | -14.06% | 139% | 106% | -12.43% | -11.30% | -11.34% | 110% | 102% |
| SCC 1080p | VTM+CPR | -36.92% | -36.11% | -36.36% | 143% | 90% | -22.03% | -21.58% | -22.10% | 103% | 96% |
|  | CE8.1.1a | -37.81% | -36.76% | -37.03% | 139% | 102% | -22.77% | -22.27% | -22.78% | 104% | 100% |
|  | CE8.1.1b | -37.80% | -36.76% | -37.03% | 138% | 101% | -22.69% | -22.18% | -22.73% | 104% | 100% |
|  | CE8.1.2a | -42.96% | -41.68% | -41.99% | 147% | 90% | -25.89% | -25.17% | -25.82% | 103% | 95% |
|  | CE8.1.2b | -41.02% | -39.81% | -40.12% | 151% | 86% | -24.55% | -23.96% | -24.54% | 104% | 94% |
|  | CE8.1.2c | -36.76% | -35.55% | -35.83% | 150% | 89% | -21.43% | -20.80% | -21.35% | 105% | 94% |
|  | CE8.1.2d | -45.00% | -43.77% | -44.06% | 151% | 85% | -27.25% | -26.61% | -27.20% | 103% | 94% |
|  | CE8.1.3 | -37.14% | -36.32% | -36.57% | 144% | 97% | -22.17% | -21.75% | -22.27% | 105% | 100% |
|  | CE8.2.1 | -44.77% | -41.06% | -41.27% | 157% | 75% | -25.67% | -24.47% | -24.82% | 110% | 96% |
|  | CE8.2.1* | -39.08% | -38.45% | -38.66% | 149% | 80% | -22.67% | -22.57% | -22.80% | 107% | 93% |
|  | CE8.2.2 | -45.18% | -41.22% | -41.45% | 155% | 76% | -26.03% | -24.68% | -25.04% | 110% | 95% |
|  | CE8.2.3 | -44.75% | -41.03% | -41.26% | 156% | 77% | -25.64% | -24.50% | -24.80% | 102% | 90% |
|  | CE8.2.4 | -44.78% | -41.04% | -41.26% | 158% | 80% | -25.65% | -24.40% | -24.77% | 102% | 91% |
|  | CE8.2.5a | -41.75% | -40.55% | -40.56% | 157% | 70% | -24.88% | -24.46% | -24.44% | 106% | 91% |
|  | CE8.2.5b | -44.77% | -41.06% | -41.27% | 157% | 75% | -26.03% | -24.75% | -24.95% | 108% | 95% |
|  | CE8.2.6 | -44.78% | -41.04% | -41.26% | 158% | 80% | -25.65% | -24.40% | -24.77% | 102% | 91% |
|  | CE8.3.1a | -38.67% | -38.38% | -38.57% | 104% | 94% | -23.03% | -22.87% | -23.29% | 105% | 98% |
|  | CE8.3.1b | -42.26% | -39.36% | -39.70% | 107% | 101% | -25.50% | -23.29% | -23.84% | 106% | 99% |
|  | CE8.3.2 | -41.97% | -39.22% | -39.55% | 107% | 102% | -25.45% | -23.27% | -23.78% | 104% | 99% |

* PLT mode in HEVC SCC, when dual-tree SPS is off (the baseline version of CE 8.2.1 uses dual tree)

Results compared to CTC (without CPR), all CE results have CPR+PLT enabled

|  |  | AI Over VTM-3.0 |  |  |  |  | RA Over VTM-3.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CTC overall | VTM+CPR+PLT | -0.19% | -0.26% | -0.26% | 145% | 103% | 0.20% | 0.07% | 0.20% | 105% | 102% |
|  | CE8.1.1a | -0.25% | -0.32% | -0.36% | 151% | 104% | 0.17% | 0.06% | 0.05% | 106% | 102% |
|  | CE8.1.1b | -0.24% | -0.30% | -0.30% | 151% | 105% | 0.18% | 0.09% | 0.04% | 105% | 101% |
|  | CE8.1.2a | -0.26% | -0.33% | -0.37% | 149% | 101% | 0.19% | 0.11% | 0.17% | 104% | 99% |
|  | CE8.1.2b | -0.22% | -0.31% | -0.30% | 152% | 104% | 0.21% | 0.05% | 0.15% | 106% | 102% |
|  | CE8.1.2c | -0.14% | -0.19% | -0.21% | 150% | 105% | 0.23% | 0.07% | 0.17% | 106% | 102% |
|  | CE8.1.2d | -0.30% | -0.37% | -0.38% | 152% | 105% | 0.18% | 0.08% | 0.12% | 105% | 102% |
|  | CE8.1.3 | -0.19% | -0.31% | -0.25% | 144% | 103% | 0.21% | 0.05% | 0.16% | 104% | 100% |
|  | CE8.2.1 | -0.19% | -0.26% | -0.26% | 146% | 105% | 0.20% | 0.07% | 0.20% | 108% | 104% |
|  | CE8.2.2 | -0.19% | -0.24% | -0.26% | 147% | 105% | 0.22% | 0.07% | 0.18% | 106% | 103% |
|  | CE8.2.3 | -0.19% | -0.26% | -0.25% | 142% | 104% | 0.20% | 0.05% | 0.15% | 100% | 99% |
|  | CE8.2.4 | -0.21% | -0.30% | -0.26% | 141% | 105% | 0.20% | 0.10% | 0.22% | 101% | 99% |
|  | CE8.2.5a | -0.36% | -0.63% | -0.81% | 142% | 100% | 0.15% | -0.32% | -0.24% | 106% | 101% |
|  | CE8.2.5b | -0.19% | -0.26% | -0.26% | 146% | 105% | 0.20% | 0.05% | 0.13% | 105% | 103% |
|  | CE8.2.6 | -0.19% | -0.27% | -0.26% | 143% | 104% | 0.20% | 0.08% | 0.18% | 101% | 100% |
|  | CE8.3.1a | -0.16% | -0.30% | -0.32% | 170% | 116% | 0.24% | 0.07% | 0.17% | 141% | 122% |
|  | CE8.3.1b | -0.27% | -0.29% | -0.24% | 141% | 109% | 0.18% | 0.14% | 0.24% | 104% | 105% |
|  | CE8.3.2 | -0.27% | -0.28% | -0.24% | 141% | 109% | 0.18% | 0.15% | 0.26% | 105% | 106% |
| Class F | VTM+CPR+PLT | -15.51% | -14.05% | -14.13% | 173% | 97% | -11.90% | -11.77% | -11.99% | 115% | 100% |
|  | CE8.1.1a | -15.61% | -14.25% | -14.33% | 176% | 97% | -12.01% | -11.77% | -12.05% | 116% | 100% |
|  | CE8.1.1b | -15.62% | -14.18% | -14.25% | 176% | 98% | -12.02% | -11.78% | -11.95% | 116% | 100% |
|  | CE8.1.2a | -16.95% | -15.79% | -15.92% | 182% | 95% | -13.09% | -13.00% | -13.18% | 115% | 97% |
|  | CE8.1.2b | -16.44% | -15.15% | -15.25% | 185% | 99% | -12.66% | -12.51% | -12.73% | 118% | 101% |
|  | CE8.1.2c | -15.29% | -13.87% | -13.97% | 179% | 97% | -11.79% | -11.59% | -11.76% | 117% | 100% |
|  | CE8.1.2d | -17.71% | -16.58% | -16.68% | 191% | 100% | -13.66% | -13.50% | -13.75% | 118% | 101% |
|  | CE8.1.3 | -15.57% | -14.12% | -14.27% | 171% | 96% | -12.00% | -11.84% | -12.07% | 112% | 97% |
|  | CE8.2.1 | -15.51% | -14.05% | -14.13% | 174% | 97% | -11.90% | -11.77% | -11.99% | 119% | 103% |
|  | CE8.2.2 | -15.53% | -14.03% | -14.11% | 174% | 97% | -11.94% | -11.72% | -11.94% | 119% | 103% |
|  | CE8.2.3 | -15.50% | -14.03% | -14.09% | 173% | 100% | -11.90% | -11.72% | -11.92% | 110% | 98% |
|  | CE8.2.4 | -15.62% | -13.29% | -13.31% | 169% | 99% | -12.03% | -11.00% | -11.12% | 110% | 98% |
|  | CE8.2.5a | -14.59% | -14.28% | -14.61% | 167% | 93% | -11.43% | -11.08% | -11.70% | 118% | 99% |
|  | CE8.2.5b | -15.51% | -14.05% | -14.13% | 174% | 97% | -11.97% | -11.64% | -11.89% | 118% | 104% |
|  | CE8.2.6 | -15.49% | -13.98% | -14.09% | 175% | 100% | -11.87% | -11.53% | -11.90% | 110% | 99% |
|  | CE8.3.1a | -15.81% | -14.80% | -14.98% | 175% | 113% | -12.12% | -11.98% | -12.30% | 147% | 119% |
|  | CE8.3.1b | -16.81% | -15.47% | -15.63% | 143% | 104% | -12.95% | -12.24% | -12.56% | 113% | 104% |
|  | CE8.3.2 | -16.80% | -15.44% | -15.62% | 143% | 105% | -12.93% | -12.26% | -12.57% | 114% | 105% |
| SCC 1080p | VTM+CPR+PLT | -44.77% | -41.06% | -41.27% | 158% | 77% | -25.67% | -24.49% | -24.84% | 106% | 93% |
|  | CE8.1.1a | -45.08% | -41.40% | -41.60% | 155% | 80% | -26.14% | -24.92% | -25.30% | 108% | 95% |
|  | CE8.1.1b | -45.09% | -41.40% | -41.61% | 155% | 80% | -26.12% | -24.91% | -25.30% | 108% | 95% |
|  | CE8.1.2a | -48.16% | -44.73% | -44.97% | 156% | 75% | -28.04% | -26.75% | -27.18% | 104% | 91% |
|  | CE8.1.2b | -46.92% | -43.43% | -43.67% | 164% | 79% | -27.13% | -25.86% | -26.32% | 107% | 94% |
|  | CE8.1.2c | -43.87% | -40.16% | -40.35% | 164% | 77% | -24.73% | -23.54% | -23.90% | 109% | 94% |
|  | CE8.1.2d | -49.80% | -46.50% | -46.74% | 159% | 78% | -29.13% | -27.90% | -28.39% | 106% | 93% |
|  | CE8.1.3 | -44.89% | -41.21% | -41.42% | 157% | 77% | -25.74% | -24.58% | -24.94% | 107% | 95% |
|  | CE8.2.1 | -44.77% | -41.06% | -41.27% | 157% | 75% | -25.67% | -24.47% | -24.82% | 110% | 96% |
|  | CE8.2.2 | -45.18% | -41.22% | -41.45% | 155% | 76% | -26.03% | -24.68% | -25.04% | 110% | 95% |
|  | CE8.2.3 | -44.75% | -41.03% | -41.26% | 156% | 77% | -25.64% | -24.50% | -24.80% | 102% | 90% |
|  | CE8.2.4 | -45.03% | -40.16% | -40.41% | 158% | 82% | -25.77% | -23.77% | -24.18% | 102% | 91% |
|  | CE8.2.5a | -41.75% | -40.55% | -40.56% | 157% | 70% | -24.88% | -24.46% | -24.44% | 106% | 91% |
|  | CE8.2.5b | -44.77% | -41.06% | -41.27% | 157% | 75% | -26.03% | -24.75% | -24.95% | 108% | 95% |
|  | CE8.2.6 | -44.78% | -41.04% | -41.26% | 158% | 80% | -25.65% | -24.40% | -24.77% | 102% | 91% |
|  | CE8.3.1a | -45.25% | -42.15% | -42.34% | 130% | 93% | -24.20% | -23.13% | -23.45% | 133% | 111% |
|  | CE8.3.1b | -46.17% | -42.67% | -42.88% | 106% | 89% | -25.31% | -23.46% | -23.88% | 104% | 102% |
|  | CE8.3.2 | -46.16% | -42.65% | -42.86% | 106% | 90% | -25.30% | -23.48% | -23.88% | 105% | 99% |

Results compared to CTC (without CPR), all CE results have PLT enabled

|  |  | AI Over VTM-3.0 |  |  |  |  | RA Over VTM-3.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CTC overall | VTM+PLT | 0.09% | 0.10% | 0.06% | 107% | 101% | 0.14% | 0.20% | 0.09% | 106% | 102% |
|  | CE8.2.1 | 0.09% | 0.10% | 0.06% | 107% | 101% | 0.14% | 0.20% | 0.09% | 106% | 102% |
|  | CE8.2.1* | 0.07% | 0.07% | 0.01% | 108% | 101% | 0.14% | 0.09% | 0.08% | 106% | 101% |
|  | CE8.2.2 | 0.08% | 0.09% | 0.11% | 108% | 103% | 0.14% | 0.18% | 0.10% | 106% | 102% |
|  | CE8.2.3 | 0.08% | 0.10% | 0.07% | 106% | 101% | 0.12% | 0.17% | 0.13% | 100% | 98% |
|  | CE8.2.4 | 0.06% | 0.07% | 0.05% | 106% | 102% | 0.13% | 0.16% | 0.10% | 100% | 98% |
|  | CE8.2.5a | 0.07% | 0.09% | 0.03% | 104% | 98% | 0.13% | 0.08% | 0.10% | 104% | 99% |
|  | CE8.2.5b | 0.09% | 0.10% | 0.06% | 111% | 105% | 0.11% | 0.18% | 0.12% | 105% | 101% |
|  | CE8.2.6 | 0.09% | 0.10% | 0.06% | 106% | 101% | 0.13% | 0.20% | 0.11% | 100% | 98% |
| Class F | VTM+PLT | -11.43% | -8.94% | -9.04% | 117% | 89% | -8.52% | -8.02% | -8.31% | 110% | 101% |
|  | CE8.2.1 | -11.43% | -8.94% | -9.04% | 117% | 89% | -8.52% | -8.02% | -8.31% | 110% | 101% |
|  | CE8.2.1* | -5.55% | -6.57% | -7.23% | 114% | 95% | -3.89% | -5.15% | -5.83% | 111% | 102% |
|  | CE8.2.2 | -11.65% | -9.08% | -9.19% | 118% | 91% | -8.80% | -8.28% | -8.66% | 110% | 101% |
|  | CE8.2.3 | -11.41% | -8.92% | -8.98% | 118% | 93% | -8.47% | -8.06% | -8.26% | 103% | 95% |
|  | CE8.2.4 | -11.50% | -8.30% | -8.35% | 117% | 93% | -8.56% | -7.60% | -7.78% | 102% | 95% |
|  | CE8.2.5a | -9.75% | -9.29% | -10.06% | 113% | 86% | -7.51% | -7.08% | -8.08% | 107% | 96% |
|  | CE8.2.5b | -11.43% | -8.94% | -9.04% | 121% | 94% | -8.65% | -7.95% | -8.32% | 110% | 102% |
|  | CE8.2.6 | -11.43% | -8.83% | -8.89% | 116% | 92% | -8.54% | -8.01% | -8.25% | 102% | 95% |
| SCC 1080p | VTM+PLT | -33.06% | -27.59% | -27.70% | 131% | 68% | -15.71% | -14.68% | -14.53% | 106% | 93% |
|  | CE8.2.1 | -33.06% | -27.59% | -27.70% | 131% | 68% | -15.71% | -14.68% | -14.53% | 106% | 93% |
|  | CE8.2.1* | -17.97% | -18.43% | -18.73% | 122% | 81% | -9.03% | -10.12% | -10.01% | 106% | 96% |
|  | CE8.2.2 | -34.47% | -28.70% | -28.82% | 122% | 70% | -16.56% | -15.23% | -15.12% | 105% | 94% |
|  | CE8.2.3 | -33.03% | -27.60% | -27.66% | 126% | 67% | -15.73% | -14.64% | -14.50% | 99% | 87% |
|  | CE8.2.4 | -33.15% | -26.95% | -27.08% | 127% | 69% | -15.77% | -14.33% | -14.23% | 99% | 88% |
|  | CE8.2.5a | -27.02% | -26.01% | -25.52% | 126% | 62% | -14.78% | -15.28% | -14.62% | 102% | 88% |
|  | CE8.2.5b | -33.06% | -27.59% | -27.70% | 135% | 69% | -16.64% | -15.24% | -15.02% | 108% | 94% |
|  | CE8.2.6 | -33.13% | -27.64% | -27.75% | 128% | 71% | -15.73% | -14.59% | -14.50% | 99% | 89% |

8.1.x

The following question is discussed: Should CTC enable CPR for class F? This would be realistic for the case where the encoder could know that it is screen content or natural content.

The methods of CE8.1.2 provide additional gain of >3% for class F, 8% for TGM class. These are re-using existing data from the previous CTU (assuming 2 or 4 buffers of size 64x64 each, depending on version). This is generally agreed to be practical and give good benefit. It could however be coplicated to specify as an encoder/bitstream restriction that the limits of CPR vectors are valid.

Revisit: Shan Liu and other interested experts to inspect the specification text (all 4 versions a…d), report back for a potential decision on adoption

The methods of CE8.1.1 provide additional gain (0.4% class F, 0.9% for class TGM), but modify AMVP and merge list construction. Question is raised whether VVC would require to use exactly the same principle for CPR and normal MV coding. Probably this might be OK if it does not deviate too much, and does not require much additional processing. In HEVC SCC, it was required to have exactly the same process for CPR and MV coding, which may not be the case for VVC.

Revisit: To be discussed in JVET about general design limitations we would impose on CPR vector coding

8.1.3 uses the line/columns buffers at CTU boundary (which is there for intra prediction) to enable CPR by one line/column more across CTU boundary. Gives 0.1% for class F, 0.2% for TGM. Probably, part of that gain (as far as the left boundary is concerned) is overlapping with 8.1.2.

Revisit if possible to run a combination test to see if it still gives gain on top of 8.1.2

8.2.x

Current VVC does not include palette mode, however a kind of “baseline” exists which is HEVC palette plus dual tree. This provides roughly 3%/7.5% gain over VVC+CPR for classes F/TGM. Results from CE also indicate that this gain goes to 2.5%/4.5% when combined with the improved CPR from 8.1.2. This may be even less when other aspects such as transform skip come into play. Such a relative low gain might not justify adding

Revisit: To be discussed in plenary – what should be the design targets for palette – improved compression performance may be more important than reducing complexity.

8.2.2 is a method that performs intra prediction and provides one signal in the palette that inserts the predicted sample instead of inserting a value from the palette. Gain is very low, and does not justify the additional complexity.

8.2.3 targets complexity reduction of (index map scan order is constrained by the block shape). Results show minimum loss compared to 8.2.1 (CE palette basis). At the current status of palette, no urgent need to make such changes of the basis method.

8.2.4 uses separate trees (and separate palettes for luma and chroma), but the chroma part can only use palette mode, if the corresponding luma area entirely uses palette. The palette mode signalling is removed for such CUs where it is not the case. This however introduces some dependency (including parsing) which does not allow independent decoding of luma and chroma trees any more. Furthermore, the benefit on compression is practically zero.

It is understood that the intent is to disallow cases where the luma does not use palette and the chroma uses it. However, this could also be achieved by an encoder only decision.

8.2.5a uses one tree but allows different palettes for luma and chroma. This performs better than 8.2.1*, which uses one tree and one palette. It however performs still worse than 8.2.1 which uses separate trees and separate palettes for luma and chroma. This indicates that only part of the gain of 8.2.1 versus 8.2.1* comes from the different palettes, another part comes from different partitioning.

8.2.5b is identical to 8.2.1 for intra coding, but uses same tree (but different palettes) for inter coding. For RA this gives <0.4% in TGM; <0.1% in class F (with CPR on), With CPR off, the gain is 0.9+% (TGM) amd 0.1+% (class F). At the current status of palette, no urgent need to make such changes of the basis method. It provides some (small) gain, and this aspect should be further studied. There is also a CE related proposal on this.

8.2.6 targets better compression performing by predicting the palette. This however can only be observed for the palette only compression, and is very small (0.06% for class TGM).

8.3.x

These approaches perform sample-wise DPCM as an additional concept that demonstrates some benefit for screen content types. The most viable approach (according to proponents) is 8.3.2 which can best be optimized in terms of throughput and also shows best compression. The gain over VTM+CPR is 3.7%/4.9% for classes F/TGM, and gain over VTM+CPR+PLT is 1.3%/1.4%, respectively. The encoder runtimes are significantly faster when those methods are used, as some early termination approach is employed (not searching other intra modes when the prediction works well).

The current method uses a maximum of 12 context coded bins per sample, which is much too large.

Furthermore, this would be yet another alternative prediction method (and building block) which needs to be implemented in parallel with existing ones, which needs some justification in terms of compression performance to be included.

Could the residual coding be unified with the existing approach of VVC (in that case, the method could just be seen as another intra prediction mode with transform skip)?

It is also asked how the current residual coding method would perform in the low QP range

Further study necessary.

(include summaries of contribution from JVET-M0028 subsequently)

JVET-M0050 CE8: Palette Mode in HEVC (test 8.2.1) [Y.-C. Sun, J. Lou (Alibaba), Y.-H. Chao, H. Wang, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-M0051 CE8: Palette Mode and Intra Mode Combination (test 8.2.2) [Y.-C. Sun, J. Lou (Alibaba)]

JVET-M0052 CE8: Separate Palette Coding for Luma and Chroma (test 8.2.5) [Y.-C. Sun, J. Lou (Alibaba), Y.-H. Chao, H. Wang, V. Seregin, M. Karczewicz (Qualcomm), R. Chernyak, S. Ikonin, J. Chen (Huawei)]

JVET-M0056 CE8: BDPCM with LOCO-I and independently decodable areas (test 8.3.1a) [F. Henry, A. Mohsen (Orange), P. Philippe, G. Clare (B-com)]

JVET-M0057 CE8: BDPCM with horizontal/vertical predictor and independently decodable areas (test 8.3.1b) [F. Henry, M. Abdoli, P. Philippe, G. Clare (Orange)]

JVET-M0058 CE8: BDPCM with modified binarization (test 8.3.2) [F. Henry, M. Abdoli, G. Clare, P. Philippe (Orange)]

JVET-M0332 CE8: Block vector prediction for CPR (test 8.1.1a and test 8.1.1b) [J. Nam, J. Lim, S. Kim (LGE)]

JVET-M0407 CE8: CPR reference memory reuse without increasing memory requirement (CE8.1.2a and CE8.1.2d) [X. Xu, X. Li, S. Liu (Tencent), E. Chai (Ubilinx)]

JVET-M0408 CE8: CPR reference memory reuse with reduced memory requirement (CE8.1.2b and CE8.1.2c) [X. Xu, X. Li, S. Liu (Tencent), E. Chai (Ubilinx)]

JVET-M0455 CE8: Palette index map scan order constraints (Test 8.2.3) [J. Ye, X. Xu, M. Xu, X. Li, S. Liu (Tencent)]

JVET-M0456 CE8: palette mode when dual-tree is enabled (Test 8.2.4) [J. Ye, X. Xu, X. Li, S. Liu (Tencent)]

JVET-M0457 CE8: Palette predictor list enhancement (Test 8.2.6) [J. Ye, X. Xu, M. Xu, X. Li, S. Liu (Tencent)]

JVET-M0474 CE8.1.3: Extended CPR reference with 1 buffer line [L. Pham Van, V. Seregin, W.-J. Chien, T. Hsieh, M. Karczewicz (Qualcomm)]

JVET-M0543 Crosscheck of M0474: CE8.1.3- Extended CPR reference with 1 buffer line [S.Paluri, S. Kim (LGE)] [late]

## CE9: Decoder-side motion vector derivation (8)

Contributions in this category were discussed Thursday 10 Jan. 2030–2230 (Track B chaired by JRO).

JVET-M0029 CE9: Summary report on decoder side motion vector derivation [X. Xiu, S. Esenlik]

The core experiment summary report is organized into 2 sub-tests as follows:

- CE9.1: BDOF design (3 tests)

- CE9.2: DMVR design (24 tests)

CE9.1: BDOF design

| # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 9.1.1.a | 1. use 8-tap DCTIF filters to generate the prediction samples in extended region and inside CU 2. padding for reference samples outside [w+7, h+7] for final MC (DCTIF) | Xiu, Xiaoyu | H. Liu |
| 9.1.1.b | 1. use integer positions to generate the prediction samples in extended region 2. use 8-tap DCTIF filters to generate the prediction samples inside CU | Xiu, Xiaoyu | H. Liu |
| 9.1.1.c | 1. apply different gradient calculation method for prediction samples on the CU boundaries and inside CU 2. use 8-tap DCTIF filters to generate the prediction samples inside CU | Xiu, Xiaoyu | H. Liu |

|  |  |  | VTM |  |  |  |  | Crosscheck |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test | Document | Crosschecker | Y | U | V | EncT | DecT | EncT | DecT |
| 9.1.1 a | JVET-M0487 | H. Liu | -0.01% | 0.03% | 0.02% | 103% | 102% | 105% | 104% |
| 9.1.1 b |  | H. Liu | 0.05% | 0.02% | 0.02% | 99% | 98% | 99% | 100% |
| 9.1.1 c |  | H. Liu | 0.10% | 0.03% | 0.01% | 99% | 98% | 100% | 100% |

9.1.1.a does not simplify, replaces bilinear filters in extended region by DCTIF

9.1.1.b simplifies by using no interpolation in extended region (just integer positions)

9.1.1.c changes the gradient calculation at boundaries and does not need extended region any more, it simplifies, but the design becomes less unified

Several experts supported 9.1.1.b as the best simplified design approach.

Decision: Adopt JVET-M0487 (solution 9.1.1.b)

Revisit: Specification text to be made available, to be confirmed to be consistent with software by cross-checkers, and be reviewed by spec editors.

CE9.2: DMVR design

| # | Description | Document |
| --- | --- | --- |
| 9.2.1 a DMVR base s/w | 1. padding for reference samples outside [w+7, h+7] for final MC (DCTIF) 2. use bi-linear filter to generate prediction samples [w+4, h+4] for motion refinement 3a. MRSAD is used as metric for motion refinement 3b. MRSAD calculation on every other row 4. error surface based sub-pixel refinement 5a. DMVR early termination based on MV distance 5b. DMVR early termination based on sample difference Block size criteria: CUs with height >= 8 and size > 64 and CUs with size <= 1024 Refined MV: MC, DBF, TMVP, spatial MV prediction from top and top-left CTU | JVET-M0147 (S. Sethuraman) |
| 9.2.1 b0 | CE9.2.1a + Integer DMVR, SAD, and refined MV used only for MC |  |
| 9.2.1 b | CE9.2.1b0 + Enable 32x32 forced split + enable DMVR for w*h > 1024 + Replace CU level early termination to Sub-PU level |  |
| 9.2.1 c | CE9.2.1b + Replace SAD with MR-SAD |  |
| 9.2.1 d | CE9.2.1c + Replace Integer DMVR with Bilinear MC |  |
| 9.2.1 e | CE9.2.1d + use of refined MV for deblocking, TMVP and for spatial MV prediction from top/top-left CTU neighbors |  |
| 9.2.1 f | CE9.2.1e + use of refined MV also within CTU row from VPDUs that were processed 2 VPDUs behind current VPDU |  |
| 9.2.1 f1 | CE9.2.1f + MC restricted to use samples within (64+7)*(64+7) for VPDU of CUs larger than 64x64 |  |
| 9.2.1 g | CE9.2.1e + replace sub-PU size to 16x16 from 32x32 |  |
| 9.2.2 c | CE9.2.1g + replace MR-SAD with PR-MR-SAD | JVET-M0147 (S. Sethuraman) |
| 9.2.2 d | CE9.2.2c + Change sub-CU level early termination threshold (3 times) to know DecT impact |  |
| *9.2.2 f | Results for CE9.2.2c when BDOF is disabled in both VTM3.0 and with DMVR + DMVR is enabled for ATMVP and MMVD |  |
| 9.2.2 f1 | Disabling refined MV usage for anything other than MC in CE9.2.2c |  |
| 9.2.2 f2 | Replacing PR-MR-SAD with SAD in CE9.2.2c |  |
| 9.2.3 a | No diagonal direction checking when all cross points' cost value not less than center point cost value | JVET-M0306 (X.Chen) |
| 9.2.3 b | Down sampling for MRSAD's mean value calculation |  |
| 9.2.3 c | Replacing MRSAD with SAD in CE9.2.1a |  |
| 9.2.4 a | CE9.2.1a + allow inter reconstruction to be used as intra reference | JVET-M0447 (M. Xu) |
| 9.2.4 b | CE9.2.1a + don't allow inter reconstruction to be used as intra reference |  |
| 9.2.4 c | CE9.2.1a + don't allow DMVR reconstruction to be used as intra reference |  |
| 9.2.5 a | CE9.2.1a is changed as follows: minCost equal to zero replaces minCost less than (w*h*(1<<(Max(2, 14 – Bit Depth) + (Bit Depth-8)+0)) | JVET-M0062 (T. Chujoh ) |
| 9.2.5 b | CE9.2.1a is changed as follows: minCost equal to zero replaces minCost less than (w*h*(1<<(Max(2, 14 – Bit Depth) + (Bit Depth-8)-1)) |  |
| 9.2.6 | CE9.2.1a is changed as follows: Disable DMVR for CUs with size > 4096 | JVET-M0076 (K. Unno) |
| 9.2.7 | CE9.2.1a is changed as follows: Bilinear filter is replaced by no interpolation (rounding to nearest integer sample) | JVET-M0287 (S. Esenlik) |

* Below, CE9.2.2.f result is reported compared to BDOF off anchor.

|  |  | VTM |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test | Description | Y | U | V | EncT | DecT |
| 9.2.1 a DMVR base s/w | 1. padding for reference samples outside [w+7, h+7] for final MC (DCTIF) 2. use bi-linear filter to generate prediction samples [w+4, h+4] for motion refinement 3a. MRSAD is used as metric for motion refinement 3b. MRSAD calculation on every other row 4. error surface based sub-pixel refinement 5a. DMVR early termination based on MV distance 5b. DMVR early termination based on sample difference Block size criteria: CUs with height >= 8 and size > 64 and CUs with size <= 1024 Refined MV: MC, DBF, TMVP, spatial MV prediction from top and top-left CTU DMVR is enabled for ATMVP and MMVD (disabled for all other tests exceptz 9.2.3ff.) | -0.71% | -0.81% | -0.87% | 110% | 118% |
| 9.2.1 b0 | CE9.2.1a + Integer DMVR, SAD, and refined MV used only for MC | -0.30% | -0.39% | -0.40% | 101% | 103% |
| 9.2.1 b | CE9.2.1b0 + Enable 32x32 forced split + enable DMVR for w*h > 1024 + Replace CU level early termination to Sub-PU level | -0.58% | -0.86% | -0.86% | 102% | 106% |
| 9.2.1 c | CE9.2.1b + Replace SAD with MR-SAD | -0.66% | -0.88% | -0.92% | 102% | 108% |
| 9.2.1 d | CE9.2.1c + Replace Integer DMVR with Bilinear MC | -0.74% | -1.02% | -1.07% | 102% | 111% |
| 9.2.1 e | CE9.2.1d + use of refined MV for deblocking, TMVP and for spatial MV prediction from top/top-left CTU neighbors | -1.12% | -1.25% | -1.30% | 103% | 114% |
| 9.2.1 f | CE9.2.1e + use of refined MV also within CTU row from VPDUs that were processed 2 VPDUs before current VPDU | -1.25% | -1.33% | -1.43% | 103% | 114% |
| 9.2.1 f1 | CE9.2.1f + MC restricted to use samples within (64+7)*(64+7) for VPDU of CUs larger than 64x64 | -1.25% | -1.33% | -1.42% | 103% | 114% |
| 9.2.1 g | CE9.2.1e + replace sub-PU size to 16x16 from 32x32 | -1.13% | -1.33% | -1.44% | 103% | 118% |
| 9.2.2 c | CE9.2.1g + replace MR-SAD with PR-MR-SAD | -1.07% | -1.27% | -1.36% | 103% | 118% |
| 9.2.2 d | CE9.2.2c + Change sub-CU level early termination threshold (3 times) to know DecT impact | -1.04% | -1.27% | -1.36% | 103% | 117% |
| *9.2.2 f | Results for CE9.2.2c when BDOF is disabled in both VTM3.0 and with DMVR + DMVR is enabled for ATMVP and MMVD | -1.51% | -1.58% | -1.59% | 116% | 126% |
| 9.2.2 f1 | Disabling refined MV usage for anything other than MC in CE9.2.2c | -0.72% | -1.05% | -1.11% | 102% | 115% |
| 9.2.2 f2 | Replacing PR-MR-SAD with SAD in CE9.2.2c | -1.02% | -1.29% | -1.35% | 103% | 115% |
| 9.2.3 a | CE9.2.1a, No diagonal direction checking when all cross points' cost value not less than center point cost value | -0.71% | -0.84% | -0.90% | 108% | 116% |
| 9.2.3 b | CE9.2.1a, Down sampling for MRSAD's mean value calculation | -0.68% | -0.84% | -0.89% | 108% | 115% |
| 9.2.3 c | CE9.2.1a, Replacing MRSAD with SAD | -0.60% | -0.78% | -0.84% | 107% | 115% |
| 9.2.4 a | CE9.2.1a + allow inter reconstruction to be used as intra reference | -0.71% | -0.81% | -0.87% | 108% | 115% |
| 9.2.4 b | CE9.2.1a + don't allow inter reconstruction to be used as intra reference | 3.87% | 8.82% | 9.62% | 103% | 113% |
| 9.2.4 c | CE9.2.1a + don't allow DMVR reconstruction to be used as intra reference | -0.46% | -0.34% | -0.36% | 108% | 115% |
| 9.2.5 a | CE9.2.1a is changed as follows: minCost equal to zero replaces minCost less than (w*h*(1<<(Max(2, 14 – Bit Depth) + (Bit Depth-8)+0)) | -0.67% | -0.80% | -0.86% | 109% | 118% |
| 9.2.5 b | CE9.2.1a is changed as follows: minCost equal to zero replaces minCost less than (w*h*(1<<(Max(2, 14 – Bit Depth) + (Bit Depth-8)-1)) | -0.70% | -0.81% | -0.87% | 109% | 119% |
| 9.2.6 | CE9.2.1a is changed as follows: Disable DMVR for CUs with size > 4096 | -0.89% | -0.99% | -1.08% | 111% | 119% |
| 9.2.7 | CE9.2.1a is changed as follows: Bilinear filter is replaced by no interpolation (rounding to nearest integer sample) | -0.58% | -0.69% | -0.74% | 109% | 115% |

Additional results in JVET-M0147 which show results on CE9.2.1g but without using refined MV for spatial MV prediction and deblocking: Overall -1.01%. This could be a reasonable approach avoiding some of the dependency problems that were observed in DMVR before. Revisit: Cross-check zu be provided; specification text to be made available.

It is noted that disallowing DMVR blocks for intra prediction may not be that relevant in practical pipeline implementations. Typically, within a CTU first all inter blocks would be reconstructed, and finally the intra coded blocks,

Early termination approaches do not seem to be very effective in terms of runtime reduction, and also are not beneficial for hardware.

Generally, the investigation on DMVR has led to a point where it might be manageable implementation-wise (not low complex, but still giving around 1% gain)

A problem of the methods investigated in the CE could still be that DMVR and BDOF could be applied sequentially, before finally motion comp and reconstruction can be done. JVET-M0223 considers this issue.

Next step would be to review CE related contributions (there are not too many on DMVR) and assess if they provide further improvement or complexity reduction.

Revisit: Come to a conclusion on status of DMVR

BoG (S. Esenlik) to review CE9 related contributions, and suggest aspects to be studied in CE (for BIO and DMVR).

(include summaries of contribution from JVET-M0029 subsequently)

JVET-M0062 CE9: An early termination of DMVR (Test 9.2.5) [T. Chujoh, T. Ikai (Sharp)]

JVET-M0076 CE9: Block size restriction for DMVR (test 9.2.6) [K. Unno, K. Kawamura, S. Naito (KDDI)]

JVET-M0147 CE9: Results of DMVR related Tests CE9.2.1 and CE9.2.2 [S. Sethuraman (Ittiam)]

JVET-M0887 Crosscheck of additional tests in JVET-M0147 (CE9: Results of DMVR related Tests CE9.2.1 and CE9.2.2) [T. Chujoh, T. Ikai (Sharp)] [late]

JVET-M0287 CE9: Integer DMVR (Test 9.2.7) [S. Esenlik, H. Gao, A.M. Kotra, B. Wang, J. Chen (Huawei)]

JVET-M0306 CE9: DMVR Simplifications (Test 9.2.3) [X. Chen, J. Zheng (HiSilicon)]

JVET-M0447 CE9: Constrained intra prediction with DMVR (test 9.2.4) [M. Xu, X. Li, S. Liu (Tencent)]

JVET-M0487 CE9: Simplifications on bi-directional optical flow (BDOF) (test 9.1.1) [X. Xiu, Y. He (InterDigital), C.-Y. Lai, Y.-C. Su, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

## CE10: Combined and multi-hypothesis prediction (15)

Contributions in this category were discussed Saturday 12 Jan. 1100–1345 and 1515-1815 (Track B chaired by JRO).

JVET-M0030 CE10: Summary report on combined and multi-hypothesis prediction [C.-W. Hsu, M. Winken]

Five sub CEs are created to test different methods of combined predictions, including:

- CE10.1: multi-hypothesis prediction,

- CE10.2: overlapped block motion compensation,

- CE10.3: multiple shape prediction partitions,

- CE10.4: diffusion filtering of inter- and intra-prediction signals,

- CE10.5: local illumination compensation.

There are 13, 4, 1, 2 and 2 tests for each sub CE, respectively.

CE proposals are listed as follows,

| Proposal Document # | Corresponding Tests | Author(s) | Title |
| --- | --- | --- | --- |
| JVET-M0176 | CE10.1.1 | M.-S. Chiang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek) | CE10.1.1: Multi-hypothesis prediction for improving non-skip & non-merge inter mode and merge mode |
| JVET-M0425 | CE10.1.2 | M. Winken, H. Schwarz, D. Marpe, T. Wiegand (HHI) | CE10: Multi-hypothesis inter prediction (Test 10.1.2) |
| JVET-M0290 | CE10.1.3 | W. Xu, B. Wang, H. Yang, J. Chen (Huawei) | CE10: Simplification on Combined Inter-Intra Prediction (Test 10.1.3) |
| JVET-M0177 | CE10.1.4 | M.-S. Chiang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek) | CE10.1.4: Simplification of combined inter and intra prediction |
| JVET-M0293 | CE10.1.5 | W. Xu, B. Wang, H. Yang, J. Chen (Huawei), M.-S. Chiang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek) | CE10: Simplification on Combined Inter-Intra Prediction with size restriction (Test 10.1.5) |
| JVET-M0178 | CE10.2.1 | Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, C.-C. Chen, Y.-C. Lin, Y.-W. Huang, S.-M. Lei (MediaTek), X. Xiu, Y. He (InterDigital) | CE10.2.1: Uni-prediction-based CU-boundary-only OBMC |
| JVET-M0179 | CE10.2.2 | Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, C.-C. Chen, Y.-C. Lin, Y.-W. Huang, S.-M. Lei (MediaTek), X. Xiu, Y. He (InterDigital) | CE10.2.2: Integer-MV-based CU-boundary-only OBMC |
| JVET-M0180 | CE10.2.3 | Y.-C. Lin, C.-C. Chen, C.-W. Hsu, Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek) | CE10.2.3: Subblock OBMC with uni-prediction-based OBMC at CU boundaries |
| JVET-M0181 | CE10.2.4 | Y.-C. Lin, C.-C. Chen, C.-W. Hsu, Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek) | CE10.2.4: Subblock OBMC with integer-MV-based OBMC at CU boundaries |
| JVET-M0189 | CE10.3.1 | Y. Ahn, D. Sim (Digital Insights) | CE10.3.1: AMVP mode for triangle prediction |
| JVET-M0042 | CE10.4.1 CE10.4.2 | J. Rasch, A. Henkel, J. Pfaff,H. Schwarz, D. Marpe, T. Wiegand (HHI) | CE10: Uniform Directional Diffusion Filters For Video Coding |
| JVET-M0087 | CE10.5.2 | K. Abe, T. Toma,J. Li (Panasonic) | CE10: Low pipeline latency LIC (test 10.5.2) |
| JVET-M0112 | CE10.5.3 | P. Bordes, T. Poirier, F. LeLeannec (Technicolor) | CE10: LIC confined within current CTU (test 10.5.3) |

CE10.1: Multi-hypothesis prediction

| # | Supported modes | Signaling of hypothesis | # of additional hypotheses | Block constraint in luma samples | BW reduction technique | Hypothesis inheritance | Reference frame access constraints | Color components |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CE10.1.1.a | AMVP (uni only) | merge index | 1 | >= 8x8 |  |  | up to 2 | luma + chroma |
| JVET-M0176 |  |  |  |  |  |  |  |  |
| CE10.1.1.b JVET-M0176 | skip/merge (uni or bi) | merge index (implicitly derived) | 1 or 2 |  | full-pel for additional hypotheses | no temporal | up to 4 | luma + chroma |
|  |  |  |  |  | (luma) | spatial: no CTU constraints |  |  |
| CE10.1.2.a JVET-M0425 | merge AMVP | ref index + mvp index + MVDs + weight index | 1 or 2 | > 8x8 | full-pel for additional hypotheses | no temporal | up to 4 | luma + chroma |
|  |  |  |  |  | (luma + chroma) | spatial: from left or within CTU |  |  |
| CE10.1.2.b JVET-M0425 | merge AMVP | ref index + mvp index + MVDs + weight index | 1 or 2 | > 8x8 | full-pel for additional hypotheses | not allowed | up to 4 | luma + chroma |
|  |  |  |  |  | (luma + chroma) |  |  |  |
| CE10.1.2.c JVET-M0425 | merge AMVP | ref index + mvp index + MVDs + weight index | 1 or 2 | > 8x8 | full-pel for additional hypotheses | no temporal | up to 2 | luma + chroma |
|  |  |  |  |  | (luma + chroma) | spatial: from left or within CTU |  |  |
| CE10.1.2.d JVET-M0425 | merge AMVP | ref index + mvp index + MVDs + weight index | 1 or 2 | > 8x8 | full-pel for additional hypotheses | no temporal | up to 4 | luma |
|  |  |  |  |  | (luma + chroma) | spatial: from left or within CTU |  |  |

The test results for this aspect are summarized as follows,

| # | Proposal | Config. |  |  | VTM |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Y | U | V | EncT | DecT |
| CE10.1.1.a | JVET-M0176 | RA | -0.17% | -0.13% | -0.10% | 107% | 100% |
|  |  | LB | 0.00% | 0.10% | 0.25% | 107% | 101% |
| CE10.1.1.b | JVET-M0176 | RA | -0.17% | -0.25% | -0.22% | 103% | 102% |
|  |  | LB | -0.12% | -0.19% | -0.17% | 103% | 102% |
| CE10.1.2.a | JVET-M0425 | RA | -0.29% | -0.16% | -0.12% | 107% | 97% |
|  |  | LB | -0.41% | -0.04% | -0.04% | 110% | 99% |
| CE10.1.2.b | JVET-M0425 | RA | -0.20% | -0.02% | -0.05% | 107% | 96% |
|  |  | LB | -0.30% | -0.10% | -0.17% | 111% | 102% |
| CE10.1.2.c | JVET-M0425 | RA | -0.20% | -0.10% | -0.08% | 105% | 97% |
|  |  | LB | -0.20% | -0.19% | -0.24% | 107% | 100% |
| CE10.1.2.d | JVET-M0425 | RA | -0.29% | 0.03% | 0.04% | 107% | 96% |
|  |  | LB | -0.40% | 0.06% | -0.02% | 110% | 99% |

According to the CE description, for investigating the impact of multi-hypothesis inter prediction on cache-related aspects, the cache model compiled into the reference decoder software by setting #define JVET_J0090_MEMORY_BANDWITH_MEASURE to 1 is used, in conjunction with the cache config file provided in JVET-K0451. For each bit stream, the decoder outputs a total hit ratio when using the cache model. The following table compares the average hit ratios (in percentage) of the tests with those of VTM-3.0:

data quoted from JVET-M0176:

|  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | VTM-3.0 (%) |  |  | CE10.1.1.b (%) |  |  |
| Class A1 | 99.4735 |  |  | 99.4663 |  |  |
| Class A2 | 99.5509 |  |  | 99.5461 |  |  |
| Class B | 99.5308 |  |  | 99.5249 |  |  |
| Class C | 99.3467 |  |  | 99.3435 |  |  |
| Class E |  |  |  |  |  |  |
| Overall | 99.4755 |  |  | 99.4702 |  |  |
| Class D | 99.3402 |  |  | 99.3401 |  |  |
| Class F (optional) | 98.8734 |  |  | 98.8607 |  |  |
|  | Low delay B Main10 |  |  |  |  |  |
|  | VTM-3.0 (%) |  |  | CE10.1.1.b (%) |  |  |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | 99.6490 |  |  | 99.6414 |  |  |
| Class C | 99.5511 |  |  | 99.5415 |  |  |
| Class E | 99.5562 |  |  | 99.5517 |  |  |
| Overall | 99.5854 |  |  | 99.5782 |  |  |
| Class D | 99.5090 |  |  | 99.496 |  |  |
| Class F (optional) | 99.0382 |  |  | 99.0268 |  |  |

data quoted from JVET-M0425:

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  | VTM-3.0 | CE10.1.2.a | CE10.1.2.b | CE10.1.2.c | CE10.1.2.d |
| A1 | 99.4888 | 99.4781 | 99.4878 | 99.4858 | 99.4866 |
| A2 | 99.5513 | 99.5474 | 99.5505 | 99.5488 | 99.5505 |
| B | 99.5340 | 99.5259 | 99.5330 | 99.5326 | 99.5345 |
| C | 99.3629 | 99.3593 | 99.3615 | 99.3630 | 99.3634 |
| Overall | 99.4843 | 99.4777 | 99.4832 | 99.4826 | 99.4838 |
| D | 99.3416 | 99.3379 | 99.3412 | 99.3414 | 99.3415 |
| F | 98.9343 | 98.9258 | 98.9267 | 98.9297 | 98.9291 |
|  |  |  | Low Delay B |  |  |
|  | VTM-3.0 | CE10.1.2.a | CE10.1.2.b | CE10.1.2.c | CE10.1.2.d |
| B | 99.6547 | 99.6462 | 99.6531 | 99.6539 | 99.6567 |
| C | 99.5650 | 99.5548 | 99.5594 | 99.5656 | 99.5628 |
| E | 99.5242 | 99.5147 | 99.5187 | 99.5224 | 99.5225 |
| Overall | 99.5813 | 99.5719 | 99.5771 | 99.5806 | 99.5807 |
| D | 99.5715 | 99.5657 | 99.5683 | 99.5727 | 99.5737 |
| F | 99.1095 | 99.0934 | 99.0978 | 99.1081 | 99.1034 |

1.1.a uses two hypotheses in uni prediction and merge. Basically, the same prediction could be invoked by using bi prediction. However, only one AMVP list is generated, and it may save some signalling. Encoder time increases by 7%, no gain in LB.

Are gains of 1.1.a/b additive? They are said to have been almost additive before this CE cycle, but in the current CE the combination was not tested.

Worst case memory BW of 1.1.a is uncritical, of 1.1.b was analysed as roughly 80% of VTM.

Combination of 1.1.a/b would be somewhat similar to 1.2.a, which uses multi hypothesis for both merge and AMVP, and additionally allows kind of different weighting of the hypotheses. 1.2.a is the “full set” of functionality of this proposal, whereas b..d are somewhat simplifications, mainly for the benefit of saving local storage (b), or memory bandwidth (c,d). d still uses up to 4 hypotheses (2 each in bi with >=8x16 block size restriction), but only for luma, which is the reason for worse performance in chroma. It is reported that worst case memory BW is 0.97%

It is also mentioned that potentially gains of 1.1.a might add up to 1.2.x.

Generally, the gain of all 1.1.x and 1.2.x proposals is significantly less than it was over VTM2 (cut by half or even more). In particular, the 1.1.b and 1.2.x proposals add need for more building blocks. 1.1.a is relatively simple and ce re-use existing memory access structures and MC logic, but for getting the gain of 0.17% for RA (no gain for LB), the encoder runtime also increases by 7%.

By doing more encoder checks and increasing runtime by 7%, probably it should be possible to get similar gain without a syntax change.

1.1.3…5 target simplification of VTM3 (CIIP)

For simplification aspects, the tests and corresponding results are summarized as follows, where the current design in VTM-3.0 and the differences between each test and VTM-3.0 are listed

| # | Proposal | Supported intra modes | Reference sample smoothing | PDPC | Weights for combined | CU constraints | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VTM-3.0 |  | 4: DC, PL, VER, HOR | Yes | Yes | PL, DC: equal weights | area >=64 |  |
|  |  |  |  |  | VER, HOR: fixed, position dependent weights | W<128 && H<128 |  |
| CE10.1.3.a | JVET-M0290 | 1: PL | Yes | Yes | equal weights |  |  |
| CE10.1.3.b | JVET-M0290 | 4: DC, PL, VER, HOR | No for PL* | No for PL | PL, DC: equal weights |  |  |
|  |  |  |  |  | VER, HOR: fixed, position dependent weights |  |  |
| CE10.1.3.c | JVET-M0290 | 4: DC, PL, VER, HOR | Yes | Yes | equal weights |  |  |
| CE10.1.3.d | JVET-M0290 | 1: PL | No for PL* | No | equal weights (4:4) |  | CE10.1.3.a + CE10.1.3.b |
|  |  |  |  |  | LD frame: 6:2* |  |  |
| CE10.1.4 | JVET-M0177 |  |  |  |  | VTM constraints + |  |
|  |  |  |  |  |  | area <=1024 |  |
| CE10.1.5.a | JVET-M0293 | 1: PL |  |  | equal weights | VTM constraints + | CE10.1.3.a + CE10.1.4 |
|  |  |  |  |  |  | area <=1024 |  |
| CE10.1.5.b | JVET-M0293 | 1: PL |  | No | equal weights (4:4) | VTM constraints + | CE10.1.3.a + CE10.1.4 + no PDPC |
|  |  |  |  |  | LD frame: 6:2* | area <=1024 |  |

* Not in original CE description.

The test results for this aspect are summarized as follows,

| # | Proposal | Config. |  |  | VTM |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Y | U | V | EncT | DecT |
| CE10.1.3.a | JVET-M0290 | RA | -0.01% | 0.13% | 0.09% | 99% | 100% |
|  |  | LB | 0.10% | 0.35% | 0.36% | 99% | 100% |
| CE10.1.3.b | JVET-M0290 | RA | 0.01% | 0.00% | -0.02% | 100% | 100% |
|  |  | LB | 0.01% | -0.01% | -0.07% | 100% | 101% |
| CE10.1.3.c | JVET-M0290 | RA | 0.05% | 0.12% | 0.10% | 100% | 100% |
|  |  | LB | 0.10% | 0.18% | 0.05% | 100% | 100% |
| CE10.1.3.d | JVET-M0290 | RA | 0.02% | 0.07% | 0.03% | 99% | 100% |
|  |  | LB | 0.01% | 0.12% | 0.13% | 99% | 100% |
| CE10.1.4 | JVET-M0177 | RA | 0.01% | 0.02% | 0.05% | 99% | 102% |
|  |  | LB | 0.04% | -0.04% | 0.13% | 100% | 100% |
| CE10.1.5.a | JVET-M0293 | RA | 0.02% | 0.16% | 0.12% | 98% | 100% |
|  |  | LB | 0.11% | 0.23% | 0.29% | 98% | 100% |
| CE10.1.5.b | JVET-M0293 | RA | 0.06% | 0.07% | 0.03% | 98% | 100% |
|  |  | LB | 0.07% | 0.15% | 0.03% | 98% | 99% |

It is commented that 10.1.3.b/d was not originally planned in the CE.

In the context of reviewing 10.1.3.x proposals, the following aspects were identified in CIIP:

- does it need a specific MPM derivation? If there was only one mode (as in 10.1.3a/d) it is not needed at all, and it is mentioned that there are proposals just suggesting fixed length coding

- CIIP does not have any serious latency issue. Therefore, simplifications that remove some processing steps that are otherwise used in intra prediction is not necessary.

- Sample-wise unequal weighting is not an implementation issue, whereas equal weighting would be preferrable, unless it costs compression performance or causes qualty problems.

Beyond the solutions tested in 10.1.3 more study is necessary on these aspects.

10.1.4 requests restricting the maximum block size of CIIP to 32x32 (now it is 64x64). The motivation is saving need for additional memory. As however the maximum size of a inter-only or intra-only prediction block is large anyway, the real need for such a restriction is not obvious.

10.1.5 is combining some 10.1.3.x and 10.1.4.

CE10.2: OBMC

| # | Proposal | Applied boundaries | # of blending lines | BW reduction technique | Runtime reduction technique | Cost reduction technique |
| --- | --- | --- | --- | --- | --- | --- |
| CE10.2.1 | JVET-M0178 | CU boundaries | 2: width < 8 (left); height < 8 (top) 4: otherwise | apply OBMC to uni-prediction blocks use uni-prediction to generate OBMC region | 1. reuse L shape buffer 2. apply CU size constraints | CTU row buffer removal |
|  |  |  |  |  | 3. apply MV merge 4. skip similar MVs |  |
| CE10.2.2 | JVET-M0179 | CU boundaries | 2: width < 8 (left); height < 8 (top) 3: otherwise | use integer MV to generate OBMC region | 1. reuse L shape buffer 2. apply CU size constraints | CTU row buffer removal |
|  |  |  |  |  | 3. apply MV merge 4. skip similar MVs |  |
| CE10.2.3 | JVET-M0180 | CU boundaries + sub-block boundaries | CU boundary: 2: width < 8 (left); height < 8 (top) 4: otherwise sub-block: 1: top, left, right | CU boundary: apply OBMC to uni-prediction blocks use uni-prediction to generate OBMC region | 1. reuse L shape buffer 2. apply CU size constraints | CTU row buffer removal |
|  |  |  | (when sub-block OBMC is applied, the number of blending line at CU boudnary is 1) | sub-block: only apply to uni-prediction affine blocks | 3. apply MV merge 4. skip similar MVs |  |
| CE10.2.4 | JVET-M0181 | CU boundaries + sub-block boundaries | CU boundary: 2: width < 8 (left); height < 8 (top) 3: otherwise sub-blcok: 1: top, left, right | CU boundary: use integer MV to generate OBMC region | 1. reuse L shape buffer 2. apply CU size constraints | CTU row buffer removal |
|  |  |  | (when sub-block OBMC is applied, the number of blending line at CU boudnary is 1) | sub-block: only apply to uni-prediction affine blocks | 3. apply MV merge 4. skip similar MVs |  |

All proposals impose a CU size constraint >= 64 samples. It is requested to provide a more detailed analysis of the memory bandwidth. Likely, for processing on-the-fly, the worst case would be that the current block is 8x8, and all of the top and left neighbors are 4x4. Another option would be to store samples from the current block that were already fetched to perform interpolation in the neighbor blocks. In that case, it should be reported how large the additional local buffer would be, for all four methods.

Additional analysis is shown Sat. afternoon for 10.2.1 and 10.2.2 (to be provided in updated version of JVET-M0178/9). This analysis indicates that for those tw approaches the worst-case memory bandwidth of VVC is not increased for on-the-fly fetch (and even a little less for 10.2.2), or if the pre-generation method is used, the memory BW is even lower, but 1.96/1.28 kByte is necessary as local buffers.

In terms of processing, it is reported that worst case number of sample interpolations is not increased relative to the current bi prediction case. Method 10.2.1 uses uni prediction in a 8x8 block, and additionally needs to interpolate four 4x4 areas with other vectors for OBMC. Method 10.2.2 does not use any interpolations for OBMC. For 10.2.1, weighted superposition requires at most 4 shifts and 2 adds per sample. Furthermore, since the operations are locally varying, some additional logic is necessary. For 10.2.2., the latter numbers duplicate. Furthermore, 10.2.2 is more challenging in terms of local memory acces, as 6 different sources need to be blended.

10.2.1 seems manageable from complexity perspective (but definitely adds some complexity).

The test results are summarized as follows,

| # | Proposal | Config. |  |  | VTM |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Y | U | V | EncT | DecT |
| CE10.2.1 | JVET-M0178 | RA | -0.27% | -0.58% | -0.62% | 104% | 103% |
|  |  | LB | -0.36% | -0.50% | -0.51% | 106% | 105% |
| CE10.2.2 | JVET-M0179 | RA | -0.37% | -0.91% | -0.90% | 105% | 104% |
|  |  | LB | -0.39% | -0.55% | -0.48% | 106% | 106% |
| CE10.2.3 | JVET-M0180 | RA | -0.39% | -0.59% | -0.63% | 105% | 104% |
|  |  | LB | -0.39% | -0.69% | -0.76% | 107% | 108% |
| CE10.2.4 | JVET-M0181 | RA | -0.49% | -0.91% | -0.87% | 106% | 105% |
|  |  | LB | -0.45% | -0.62% | -0.34% | 108% | 110% |

As a general note, the gain in compression performance is lower than it was with VTM2 (approx. half). However, from the results, OBMC even gives more gain for LB than for RA, and LB is in overall performance of VVC still worse than RA, this is assessed to be valuable enough. Some support, and no opposition is raised in track B against adopting it.

Decision: Adopt JVET-M0178. Specification text is available.

High-level flag for disabling it.

CE10.3: Multiple shape prediction partitions

In CE10.3, the goal is to test AMVP to be combined with non-rectangular prediction partitions within one CU. The tests and corresponding results are summarized as follows,

| # | Proposal | Config. |  |  | VTM |  |  | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Y | U | V | EncT | DecT |  |
| CE10.3.1 | JVET-M0189 | RA | -0.06% | -0.03% | -0.06% | 123% | 100% | Two triangle prediction units (PUs) for AMVP mode |
|  |  | LB | -0.07% | -0.10% | -0.01% | 122% | 103% | Restricted to uni-prediction Restricted to RefIdx0 Applied to block width >= 8 && block height >= 8 |

From the results (low gain versus high increase in encoder run time), not worthwhile to consider

CE10.4: Diffusion filters for intra and inter prediction

In CE10.4, the goal is to test prediction to be combined using filtering, where three types of filters, two directional filters and one uniform filter are used. The filters are FIR filters, applied on the prediction signal, not iterative. The 1D filters are 9-tap, symmetric, only 1 multiplication, otherwise shifts. The 2D filter is a 5-tap diamond shape, only shift/add operations. At the boundaries, pixel replication is used.

The tests and corresponding results are summarized as follows,

| # | Proposal | Config. |  |  | VTM |  |  | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Y | U | V | EncT | DecT |  |
| CE10.4.1 | JVET-M0042 | AI | -0.19% | -0.03% | -0.03% | 123% | 101% | Uniform Diffusion filters with encoder speedup and simplified filter masks |
|  |  | RA | -0.44% | -0.51% | -0.35% | 113% | 98% |  |
|  |  | LB | -0.17% | 0.27% | 0.59% | 114% | 98% |  |
|  |  | LP | -0.86% | -0.26% | -0.11% | 119% | 97% |  |
| CE10.4.2 | JVET-M0042 | AI | -0.17% | -0.03% | -0.05% | 119% | 101% | Like CE10.4.1, but • In case of temporal layer depth larger than 3, no diffusion filter is applied |
|  |  | RA | -0.42% | -0.54% | -0.42% | 111% | 98% | • Simplified encoder search for intra blocks |
|  |  | LB | -0.17% | 0.41% | 0.43% | 113% | 98% |  |
|  |  | LP | -0.87% | -0.15% | -0.10% | 118% | 96% |  |

Generally, it was agreed that this method gives interesting gain, and is straightforward to implement at the decoder. Two concerns are raised:

- As it needs to be run after the prediction signal is generated, it produces additional delay in the intra prediction loop.

- For inter blocks, it can be used for 128x128 CU, which would break concepts of 64x64VPDU.

It is requested to provide additional results without using the method in intra prediction (and the intra part of CIIP), and restrict the largest CU size to 64x64.

It is also reported that a late contribution (JVET-M0848) provides new results for the same method of 10.4.2 with some (small) encoder speedup and slightly increased performance (-0.44% for luma).

Furthermore, it would be desirable to use only the prediction samples (no reconstructed samples from current picture). A version which did that was shown in the previous CE.

Revisit after results are available.

CE10.5: Local illumination compensation

In CE10.5, the goal is to test prediction to be combined using linear model derived from reconstructed and reference neighboring samples. The tests and corresponding results are summarized as follows,

| # | Proposal | Config. |  |  | VTM |  |  | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Y | U | V | EncT | DecT |  |
| CE10.5.2 | JVET-M0087 | RA | -0.56% | -0.31% | -0.37% | 135% | 99% | 1. Remove all encoder and decoder LIC processes other than the reconstruction stage |
|  |  | LB | -0.51% | -0.44% | -0.58% | 140% | 101% | 2. Modify bS calculation of DBF for LIC boundary. |
| CE10.5.3 | JVET-M0112 | RA | -0.44% | -0.31% | -0.34% | 134% | 101% | Based on CE10.5.2 |
|  |  | LB | -0.39% | -0.40% | -0.42% | 138% | 100% | LIC is confined to use reference samples only from the current CTU |

The method of 10.5.2 is mainly for the benefit of encoders, and does not solve the latency issue that LIC imposes on decoder pipeline. The problem is that a current block needs to wait for reconstruction of the neighbors, and then it requires a certain number of cycles until the parameters of the linear model are computed. Of particular concern is the fact that many decoder implementations target processing inter and intra coded blocks independently in order to make best benefit of parallel processing. Typically, inter coded blocks of a CTU are decoded first. For this, it is undesirable that LIC of an inter coded block uses the reconstruction of intra coded neighbors. If such an approach (disabling LIC from intra coded neighbors, including CPR) would be combined with the approach of 10.5.3 (only using reconstructed inter coded samples from current CTU), the situation would be better, however would still mean that the inter reconstruction within a CTU would need to be sequential (similar as the situation in intra is). Regardless of that, LIC has some additional processing complexity (which doubles in case of bi prediction) which needs to be justified by performance. The processing should also be aligned with the 64x64 VPDU concept.

Further study on these aspects.

BoG (C.-W. Hsu, M. Winken) to review CE10 related proposals, and suggest candidates for further study in CE.

JVET-M0042 CE10: Uniform Directional Diffusion Filters For Video Coding [J. Rasch, A. Henkel, J. Pfaff, H. Schwarz, D. Marpe, T. Wiegand (HHI)]

JVET-M0087 CE10: Low pipeline latency LIC (test 10.5.2) [K. Abe, T. Toma, J. Li (Panasonic)]

JVET-M0112 CE10: LIC confined within current CTU (test 10.5.3) [P. Bordes, T. Poirier, F. Le Léannec (Technicolor)]

JVET-M0176 CE10.1.1: Multi-hypothesis prediction for improving non-skip & non-merge inter mode and merge mode [M.-S. Chiang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-M0177 CE10.1.4: Simplification of combined inter and intra prediction [M.-S. Chiang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-M0611 Crosscheck of JVET-M0177 (CE10.1.4: Simplification of combined inter and intra prediction) [B. Wang (Huawei)] [late] [miss]

JVET-M0178 CE10.2.1: Uni-prediction-based CU-boundary-only OBMC [Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, C.-C. Chen, Y.-C. Lin, Y.-W. Huang, S.-M. Lei (MediaTek), X. Xiu, Y. He (InterDigital)]

JVET-M0179 CE10.2.2: Integer-MV-based CU-boundary-only OBMC [Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, C.-C. Chen, Y.-C. Lin, Y.-W. Huang, S.-M. Lei (MediaTek), X. Xiu, Y. He (InterDigital)]

JVET-M0180 CE10.2.3: Subblock OBMC with uni-prediction-based OBMC at CU boundaries [Y.-C. Lin, C.-C. Chen, C.-W. Hsu, Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-M0181 CE10.2.4: Subblock OBMC with integer-MV-based OBMC at CU boundaries [Y.-C. Lin, C.-C. Chen, C.-W. Hsu, Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-M0189 CE10.3.1: AMVP mode for triangle prediction [Y. Ahn, D. Sim (Digital Insights)]

JVET-M0564 Cross-check of JVET-M0189: CE10.3.1 AMVP mode for triangle prediction [J. Kim, T. Na (SK Telecom), J. Shin, K. Ko (PIXTREE)] [late]

JVET-M0290 CE10: Simplification on Combined Inter-Intra Prediction (Test 10.1.3) [W. Xu, B. Wang, H. Yang, J. Chen (Huawei)]

JVET-M0293 CE10: Simplification on Combined Inter-Intra Prediction with size restriction (Test 10.1.5) [W. Xu, B. Wang, H. Yang, J. Chen (Huawei), M.-S. Chiang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-M0425 CE10: Multi-hypothesis inter prediction (Test 10.1.2) [M. Winken, H. Schwarz, D. Marpe, T. Wiegand (HHI)]

## CE11: Deblocking (9)

Contributions in this category were discussed Friday 11 Jan. 0900–XXXX (Track B chaired by JRO).

JVET-M0031 CE11: Summary Report on Deblocking [A. Norkin, A. M. Kotra]

This contribution provides a summary report of Core Experiment 11 on deblocking filtering. Two categories of proposals are covered by this CE, split into two sub-tests. These sub-tests are 1) long-tap deblocking filters, and 2) deblocking at 4x4 block boundaries.

The proposals were encoded according to two test conditions, which correspond to two anchors.

Anchor-1 is VTM-3.0 according to the CTC [1].

Anchor-2 is VTM-3.0 with ALF switched off (other conditions are the same as in Anchor 2).

| Test# | Description | Document# |
| --- | --- | --- |
| 11.1.1 | Very strong deblocking filtering with conditional activation signalling | JVET-M0092 |
| 11.1.2 | Extended deblocking filter for luma and chroma | JVET-M0075 |
| 11.1.3 | Long deblocking filters (only luma) | JVET-M0186 |
| 11.1.4 | CE11: Test results of CE11.1.5 long-tap deblocking filter | JVET-M0208 |
| 11.1.5 | Longer tap Luma deblocking filter | JVET-M0298 |
| 11.1.6 | Combination of JVET-L0403, JVET-L0327, JVET-L0405, JVET-L0140. JVET-L0337, JVET-L0572, JVET-L0072 | JVET-M0471 |
| 11.1.7 | Combination of JVET-L0403, JVET-L0327, JVET-L0405, JVET-L0140. JVET-L0337, JVET-L0572, JVET-L0072 with 6 line buffers at CTU boundary | JVET-M0471 |
| 11.1.8 | Combination of JVET-L0403, JVET-L0327, JVET-L0405, JVET-L0140. JVET-L0337, JVET-L0572, JVET-L0072 with 4 line buffers at CTU boundary | JVET-M0471 |
| 11.2.1 | Deblocking for 4xN, Nx4 blocks and 8xN, Nx8 blocks that are not aligned with 8x8 sample grid | JVET-M0299 |
| 11.2.2 | Parallel deblocking filter | JVET-M0337 |

Test CE11.1

| Tests | Luma modified (Y/N) | Chroma modified (Y/N) |
| --- | --- | --- |
| CE11.1.1 | Y | Y |
| CE11.1.2 | Y | Y |
| CE11.1.3 | Y | N |
| CE11.1.4 | Y | N |
| CE11.1.5 | Y | N |
| CE11.1.6 | Y | Y |
| CE11.1.7 | Y | Y |
| CE11.1.8 | Y | Y |

Luma deblocking complexity

| Tests | Samples from block bound. modified | Samples from block bound. deblocking decision | Max num. oper for filtering per line (add/mult/compar/shift) | Max number of oper. for decision per line (add/mult/compar/shift) | Num. line buffers | Worst case complexity increased (Y/N) |
| --- | --- | --- | --- | --- | --- | --- |
| VTM3.0 | 3+3 | 4+4 | 56 (28/2/12/14) | 94 per 8 line segment 11.75 per line | 4 |  |
| CE11.1.1 | M=3+3...16+16 | 1+1 | (2/3/2/1)*M | 6 (5/0/0/1) per line | 4 | N |
| CE11.1.2 | 7+7 | 8+8 | 246 (138/12/28/68) | 4.5 (12/0/6/0)/4 in additional to VTM | 8 | Y (for filtering) |
| CE11.1.3 | 7+7, 7+3, 3+7, 3+3 | 16 | 266 (168/34/28/36) | 12.5 (24/0/22/4)/4 in addition to VTM | 8 | N |
| CE11.1.4 | 7+7, 3+7 | 16 | 190 (124/12/0/54) | 2.5 (6/0/2/2)/4 in addition to VTM | 4 | N |
| CE11.1.5 | 7+7, 3+7 | 16 | 187(85/4/28/42) | 9 (24/4/4/4)/4 in addition to VTM | 4 | N |
| CE11.1.6 | 7+7, 7+5, 5+7, 5+5, 7+3, 3+7, 5+3, 3+5 | 8+8, 8+6, 6+8, 6+6, 8+4, 4+8, 6+4, 4+6 | 148 (74,24,28,22) | 12 (26/0/10/12)/4 in addition to VTM | 8 | N |
| CE11.1.7 | 7+7, 7+5, 5+7, 5+5, 7+3, 3+7, 5+3, 3+5 | 8+8, 8+6, 6+8, 6+6, 8+4, 4+8, 6+4, 4+6 | 148 (74,24,28,22) | 12 (26/0/10/12)/4 in addition to VTM | 6 | N |
| CE11.1.8 | 7+7, 7+5, 5+7, 5+5, 7+3, 3+7, 5+3, 3+5 | 8+8, 8+6, 6+8, 6+6, 8+4, 4+8, 6+4, 4+6 | 148 (74,24,28,22) | 12 (26/0/10/12)/4 in addition to VTM | 4 | N |

Chroma deblocking complexity

| Tests | Samples from block boundary modified | Samples from block boundary for deblocking decision | Max number of operations for filtering per line (add/mult/compar/shift) | Max number of oper. for decision per line (add/mult/compar/shift) | Number of line buffers | Worst case complexity increased (Y/N) |
| --- | --- | --- | --- | --- | --- | --- |
| VTM3.0 | 1+1 | None | 10 (6/0/2/2) | None | 2 |  |
| CE11.1.1 | M=1+1...8+8 | 1+1 | (2/3/2/1)*M | 6 (5/0/0/1) per line | 2 | Y |
| CE11.1.2 | 3+3 | 4+4 | 56 (28/2/12/14) | 17.5 (19/0/10/6)/2 per line | 4 | Y |
| CE11.1.3 | VTM | VTM | VTM | VTM | 2 | N |
| CE11.1.4 | VTM | VTM | VTM | VTM | 2 | N |
| CE11.1.5 | VTM | VTM | VTM | VTM | 2 | N |
| CE11.1.6 | 3+3 | 4+4 | 76 (48,2,12,14) | 17 (17, 0, 12, 5)/2 per line | 4 | Y |
| CE11.1.7 | 3+3 | 4+4 | 76 (48,2,12,14) | 17 (17, 0, 12, 5)/2 per line | 3 | Y |
| CE11.1.8 | 3+3 | 4+4 | 76 (48,2,12,14) | 17 (17, 0, 12, 5)/2 per line | 2 | Y |

11.1.6-8 are the same (combined from 4 previous proposals) with different mumber of line buffers (8/6/4) at CTU boundary.

11.1.5, 11.1.6-8 use sample-based adaptation of clipping range

It is suggested to translate the max number of operations per line into max number of operations per sample (also considering that the long filters are only applied for large blocks), and decode the value “M” of 11.1.1 into a number. Some of the statements in the table above (worst case increased Y/N) may not be true. More detailed analysis is needed but it appears that if proposals increase worst case complexity in terms of number of operations it would not be by a large factor.

Beyond the number of operations, the number of line buffers is another complexity issue. VTM uses 4 lines for luma and 2 lines for chroma, which is duplicated by some proposals.

Parallel processing capability is given by all proposals at some sufficiently small granularity (though larger than current VVC) – see table below.

| Tests | Smallest unit size needed to perform proposed deblocking operations separately from any other units |
| --- | --- |
| VTM3.0 | 8x8 |
| CE11.1.1 | luma 16x16, chroma 8x8 (to be verified) |
| CE11.1.2 | 16x16 |
| CE11.1.3 | 16x16 |
| CE11.1.4 | 16x16 |
| CE11.1.5 | 16x16 |
| CE11.1.6 | 32x32 |
| CE11.1.7 | 32x32 |
| CE11.1.8 | 32x32 |

It is however pointed out that some of the proposals 11.1.1-5 did not consider that VTM3 now uses subblock boundary deblocking, whereas the decision of using long filters is based only on CU boundary properties. Therefore, it could happen that a long filter is applied first, and afterwards a subblock boundary within the CU is deblocked again. This would inhibit parallelism. This could be classified as a bug, which however in terms of subjective testing might not be too harmful.

None of the proposals comes with a specification text. For the decision on using long filters, CU and subblock boundaries should be handled equal.

SNR based Results with ALF on

| Test | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT (%) | DecT (%) | Y | U | V | EncT (%) | DecT (%) |
| CE11.1.1 | 0.05% | 0.01% | 0.03% | 100% | 100% | 0.35% | 0.12% | 0.12% | 100% | 101% |
| CE11.1.2 | -0.01% | -0.56% | -0.52% | 100% | 102% | -0.06% | -0.41% | -0.32% | 100% | 102% |
| CE11.1.3 | 0.00% | 0.00% | 0.00% | 100% | 101% | -0.02% | -0.02% | 0.00% | 100% | 103% |
| CE11.1.4 | 0.01% | 0.00% | -0.01% | 102% | 103% | 0.00% | 0.01% | 0.00% | 102% | 102% |
| CE11.1.5 | 0.00% | 0.00% | 0.00% | 99% | 100% | 0.02% | -0.01% | 0.00% | 99% | 99% |
| CE11.1.6 | -0.02% | -0.53% | -0.49% | 100% | 99% | 0.00% | -0.80% | -0.82% | 100% | 102% |
| CE11.1.7 | -0.02% | -0.54% | -0.50% | 101% | 99% | -0.01% | -0.82% | -0.85% | 101% | 102% |
| CE11.1.8 | -0.02% | -0.43% | -0.39% | 101% | 103% | 0.00% | -0.71% | -0.68% | 100% | 101% |

| Test | LD-B |  |  |  |  | LD-P |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT (%) | DecT (%) | Y | U | V | EncT (%) | DecT (%) |
| CE11.1.1 | 0.40% | 0.20% | 0.15% | 101% | 102% | 0.40% | 0.22% | 0.05% | 101% | 102% |
| CE11.1.2 | 0.03% | 0.53% | 0.23% | 100% | 102% | -0.01% | 0.15% | -0.32% | 100% | 102% |
| CE11.1.3 | 0.01% | -0.02% | 0.00% | 100% | 103% | -0.02% | 0.13% | 0.03% | 100% | 102% |
| CE11.1.4 | 0.13% | -0.03% | -0.09% | 99% | 103% | 0.06% | 0.21% | -0.10% | 101% | 104% |
| CE11.1.5 | 0.13% | 0.15% | 0.07% | 98% | 99% | 0.02% | 0.17% | -0.09% | 98% | 96% |
| CE11.1.6 | 0.13% | -1.32% | -1.58% | 100% | 102% | -0.03% | -1.54% | -1.88% | 100% | 102% |
| CE11.1.7 | 0.12% | -1.38% | -1.48% | 102% | 103% | -0.02% | -1.49% | -1.86% | 101% | 102% |
| CE11.1.8 | 0.08% | -1.15% | -1.42% | 99% | 101% | -0.02% | -1.21% | -1.62% | 101% | 100% |

SNR based results with ALF off

| Test | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT (%) | DecT (%) | Y | U | V | EncT (%) | DecT (%) |
| CE11.1.1 | 0.04% | -0.03% | -0.01% | 100% | 101% | 0.36% | 0.09% | 0.07% | 101% | 102% |
| CE11.1.2 | -0.01% | -0.72% | -0.63% | 100% | 102% | -0.09% | -0.45% | -0.36% | 100% | 102% |
| CE11.1.3 | 0.00% | 0.00% | 0.00% | 100% | 102% | -0.01% | 0.00% | 0.00% | 100% | 102% |
| CE11.1.4 | 0.01% | 0.00% | 0.00% | 100% | 102% | -0.03% | -0.08% | -0.03% | 102% | 105% |
| CE11.1.5 | -0.01% | 0.00% | 0.00% | 99% | 100% | 0.00% | -0.03% | 0.03% | 98% | 99% |
| CE11.1.6 | -0.03% | -0.71% | -0.61% | 92%* | 101% | -0.02% | -0.88% | -0.89% | 84%* | 103% |
| CE11.1.7 | -0.03% | -0.71% | -0.62% | 108% | 99% | -0.04% | -0.86% | -0.86% | 100% | 102% |
| CE11.1.8 | 0.05% | -1.13% | -1.49% | 104% | 103% | -0.02% | -1.64% | -1.65% | 102% | 103% |

| Test | LD-B |  |  |  |  | LD-P |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT (%) | DecT (%) | Y | U | V | EncT (%) | DecT (%) |
| CE11.1.1 | 0.42% | 0.21% | 0.09% | 101% | 102% | 0.42% | 0.13% | 0.13% | 101% | 101% |
| CE11.1.2 | -0.01% | 0.17% | -0.41% | 100% | 102% | -0.09% | -0.43% | -0.61% | 100% | 102% |
| CE11.1.3 | -0.04% | 0.01% | -0.22% | 100% | 102% | 0.00% | -0.04% | 0.02% | 100% | 103% |
| CE11.1.4 | 0.07% | 0.19% | 0.04% | 100% | 105% | 0.01% | 0.09% | 0.02% | 99% | 103% |
| CE11.1.5 | 0.09% | 0.13% | -0.17% | 98% | 98% | -0.06% | 0.10% | -0.14% | 98% | 99% |
| CE11.1.6 | 0.04% | -1.12% | -1.74% | 89%* | 103% | -0.02% | -1.77% | -1.94% | 81%* | 102% |
| CE11.1.7 | 0.04% | -1.32% | -1.60% | 105% | 105% | -0.06% | -1.85% | -2.01% | 95% | 104% |
| CE11.1.8 | -0.03% | -0.58% | -0.50% | 104% | 105% | -0.03% | -0.77% | -0.76% | 103% | 103% |

*Unreliable encoding times.

CE11.2

| Test | Proponent(s) | Cross-checker(s) |
| --- | --- | --- |
| CE11.2.1 | Kenneth Andersson kenneth.r.andersson@ericsson.com Anand Meher Kotra Anand.meher.kotra@huawei.com Chia-Ming Tsai chia-ming.tsai@mediatek.com JVET-M0299 | Hyeongmun Jang hm.jang@lge.com |
| CE11.2.2 | Hyeongmun Jang hm.jang@lge.com JVET-M0337 | Kenneth Andersson kenneth.r.andersson@ericsson.com |

Complexity for luma

| Tests | Samples from block bound. modified | Samples from block bound. for deblocking decision | Max num. oper for filtering per line (add/mult/compar/shift) | Max number of oper. for decision per line (add/mult/compar/shift) | Num. line buffers | Worst case complexity increased (Y/N) |
| --- | --- | --- | --- | --- | --- | --- |
| CE11.2.1 | VTM (additional boundary 1+1 but then 1+1 instead of 3+3 for same boundary as VTM) | VTM (additional boundary 3+3 but then 3+3 instead of 4+4 for same boundary as VTM) | VTM (additional boundary 14 (6/2/5/1) but then 14 instead of 56 for same boundary as VTM) | VTM (additional boundary 20/4 (11/0/5/4) /4 per line but then 20/4 instead of 94/8 for same boundary as VTM) | VTM | N |
| CE11.2.2 | VTM(no filtering on block boundary of 4xN for EDGE_VER, Nx4 for EDGE_HOR) | VTM | VTM | VTM | VTM | N |

Complexity for chroma

| Tests | Samples from block bound. modified | Samples from block bound. for deblocking decision | Max num. oper for filtering per line (add/mult/compar/shift) | Max number of oper. for decision for 8-sample boundary (add/mult/compar/shift) | Num. line buffers | Worst case complexity increased (Y/N) |
| --- | --- | --- | --- | --- | --- | --- |
| CE11.2.1 | VTM | VTM | VTM | VTM | VTM | N |
| CE11.2.2 | VTM(no filtering on block boundary of 2xN for EDGE_VER and Nx2 for EDGE_HOR) | VTM | VTM | VTM | VTM | N |

It is suggested to translate the max number of operations per line into max number of operations per sample.

VTM3 operates deblocking on an 8x8 grid. If a CU boundary is not on an 8x8 grid, it is not deblocked. If a CU is on an 8x8 grid, furthermore subblocks within that CU are deblocked (provided they are on the 8x8 grid as well). 4xN blocks are deblocked whenever they are coinciding with the 8x8 grid

11.2.1 deblocks on a 4x4 grid, where a boundary is deblocked with VTM filter when the next block or subblock is more than four samples apart, and with a weak and short filter if it is only samples apart.

11.2.2 uses an 8x8 grid and VTM deblocking filter but disables deblocking whenever the boundary would be only four samples apart.

SNR based results (ALF on)

| Test | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT (%) | DecT (%) | Y | U | V | EncT (%) | DecT (%) |
| CE11.2.1 | 0.02% | 0.00% | 0.00% | 99% | 99% | -0.12% | 0.07% | 0.03% | 98% | 98% |
| CE11.2.2 | -0.09% | 0.22% | 0.15% | 100% | 99% | 0.01% | 0.15% | 0.05% | 100% | 99% |

| Test | LD-B |  |  |  |  | LD-P |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT (%) | DecT (%) | Y | U | V | EncT (%) | DecT (%) |
| CE11.2.1 | -0.14% | -0.17% | -0.01% | 98% | 98% | -0.18% | -0.06% | -0.20% | 98% | 98% |
| CE11.2.2 | 0.00% | 0.00% | 0.06% | 100% | 99% | 0.00% | -0.02% | -0.33% | 100% | 100% |

SNR based results (ALF off)

| Test | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT (%) | DecT (%) | Y | U | V | EncT (%) | DecT (%) |
| CE11.2.1 | -0.05% | -0.01% | 0.00% | 99% | 99% | -0.23% | -0.02% | -0.04% | 98% | 98% |
| CE11.2.2 | -0.03% | -0.22% | -0.41% | 100% | 100% | 0.08% | 0.18% | 0.15% | 100% | 100% |

| Test | LD-B |  |  |  |  | LD-P |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT (%) | DecT (%) | Y | U | V | EncT (%) | DecT (%) |
| CE11.2.1 | -0.21% | 0.04% | -0.23% | 98% | 98% | -0.36% | 0.14% | -0.16% | 97% | 98% |
| CE11.2.2 | 0.01% | 0.01% | -0.33% | 100% | 100% | 0.09% | -0.11% | -0.15% | 100% | 100% |

Subjective testing to be done. It is suggested to use an ”AB” comparison where each proposal is compared against VTM3 at same QP.

From the test plan, it would be 96 test cases (2 QPs 34,39, 5 sequences, 8 proposals for 11.1, 2 QPs 30,34, 4 sequences, 2 proposals for 11.2). It would be highly desirable to test both ALF on and ALF off cases. If only one of these cases is possible out of logistic reasons, the ALF off case should be used (as planned in the original CE description). The ALF on bitstreams for subjective testing still need crosscheck.

JVET-M0075 CE11: Extended Deblocking Filter (test 11.1.2) [K. Unno, K. Kawamura, S. Naito (KDDI)]

JVET-M0092 CE11: Very strong deblocking with conditional activation signaling (Test 11.1.1) [C. Helmrich, B. Bross (HHI)]

JVET-M0186 CE11.1.3: Long deblocking filters [C.-M. Tsai, T.-D. Chuang, C.-W. Hsu, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-M0208 CE11: long-tap deblocking filter (Test 11.1.4) [W. Choi, K. Choi (Samsung)]

JVET-M0298 CE11: Longer tap deblocking filter (test 11.1.5) [A. M. Kotra, B. Wang, S. Esenlik, H. Gao, J. Chen (Huawei)]

JVET-M0299 CE11: Deblocking for 4 x N, N x 4 blocks and 8 x N, N x 8 blocks that are not aligned with 8 x 8 sample grid (test 11.2.1) [K. Andersson, Z. Zhang, R. Sjöberg (Ericsson), A. M. Kotra, J. Chen, S. Esenlik, B. Wang, H. Gao (Huawei), C.-M. Tsai, C.-W. Hsu, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-M0337 CE11: Test CE11.2.1 Parallel deblocking filter [H. Jang, J. Nam, S. Kim, J. Lim (LGE)]

JVET-M0471 CE11.1.6, CE11.1.7 and CE11.1.8: Joint proposals for long deblocking from Sony, Qualcomm, Sharp, Ericsson [M. Ikeda, T. Suzuki (Sony), D. Rusanovskyy, M. Karczewicz (Qualcomm), W. Zhu, K. Misra, P. Cowan, A. Segall (Sharp Labs of America), K. Andersson, J. Enhorn, Z. Zhang, R. Sjöberg (Ericsson)]

## CE12: Mapping functions (2)

Contributions in this category were discussed Saturday 12 Jan. 1600–1645, 1700-1800 (chaired by GJS).

JVET-M0032 CE12: Summary report on mapping functions [E. François, P. Yin]

This contribution provides a summary report of Core Experiment 12 on mapping functions. CE12 evaluates approaches for mapping of HDR and SDR content, with main focus on SDR. The considered technologies are in-loop reshaping and related variants. Test results against VTM3.0 anchors are provided for each performed test. Crosschecking reports are integrated in this contribution.

CE12-1: In-loop Reshaping for SDR on coding efficiency (JVET-L0247)

| Test | Proponent | Description | Xchecker |
| --- | --- | --- | --- |
| CE12-1 | Dolby (JVET-L0247) | Test in-loop reshaping for SDR focused on coding efficiency improvement, such as making reshaping function rate adaptive, disable towards higher rates. | E. François (Technicolor) |

In JVET-L0247, the reshaper has two main components: 1) in-loop reshaping applied to the luma component; 2) additional luma-dependent chroma residue scaling applied to chroma components.

The two elements had been tested separately (see JVET-K0308) and it had been concluded that the two should be tested together.

CE12-1 was testing a method proposed at the previous meeting with some encoder improvement. However, a somewhat different method had been developed since then, and there was no longer any interest in considering the CE12-1 scheme.

CE12-2: In-loop Reshaping for SDR on In-loop Reshaping for SDR on implementation investigation

| Test | Proponent | Description | Xchecker |
| --- | --- | --- | --- |
| CE12-2 | Dolby (JVET-L0247) | Test in-loop reshaping for SDR focused on implementation investigation on pipelining of the block-wise prediction loop: such as piece-wise linear interpolation replacing LUT-based mapping, disable intra-block in inter slices. | J. Xu (ByteDance), E. François (Technicolor) |

In-loop luma reshaping: CE12-2 tests a lower complexity pipeline that also eliminates decoding latency for block-wise intra prediction in inter slice reconstruction. Intra prediction is performed in reshaped domain for both inter and intra slices.

CE12-2 also tests 16-piece PWL models for luma and chroma residue scaling instead of the 32-piece PWL models of CE12-1.

Inter slice reconstruction with in-loop luma reshaper in CE12-2 (light-green shaded blocks indicate signal in reshaped domain: luma residue; intra luma predicted; and intra luma reconstructed)

Luma-dependent chroma residue scaling is a multiplicative process implemented with fixed-point integer operation. Chroma residue scaling compensates for luma signal interaction with the chroma signal. Chroma residue scaling is applied at the TU level.

For intra the reconstructed luma is averaged

For inter, the prediction luma is averaged.

The average is used to identify an index in a PWL model. The index identifies a scaling factor cScaleInv.

The chroma residual is multiplied by that number.

BD-rate of CE12-1 and CE12-2 for SDR

| All Intra | psnrY | psnrU | psnrV | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| 12-1 | -0.89% | 1.52% | 1.15% | 103% | 100% |
| 12-2 | -0.89% | 1.49% | 1.18% | 102% | 101% |

| Random Access | psnrY | psnrU | psnrV | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| 12-1 | -1.28% | 0.53% | 0.52% | 105% | 101% |
| 12-2 | -1.30% | 1.45% | 1.44% | 105% | 104% |

| Low Delay B | psnrY | psnrU | psnrV | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| 12-1 | -0.72% | 0.46% | 0.11% | 100% | 101% |
| 12-2 | -0.76% | 1.78% | 1.42% | 98% | 105% |

The parameters are (currently) sent in the tile group header (similar to ALF). These reportedly take 40-100 bits.

Suggestions for easing implementation were:

- Disabling the chroma residual scaling for separate tree operation

- Disabling the chroma residual scaling for 2x2 chroma

- Using the prediction signal rather than the reconstruction signal for intra as well as inter

The proponent indicated that these suggestions had previously been tested and should not affect the performance, and said test results would be provided in a revision of their contribution M0427.

It was noted that in the current CTC, separate trees are used for intra slices.

It was noted that some loss was observed for the chroma, although the luma gain was enough to more than compensate for that.

A participant commented that there could be rate allocation effects that could be achieved by local delta QP or setting of QP based on temporal layer. It was remarked that some tests along those lines were reported in L0246. In L0246 some gain was reported in class B but not class A and not as much as for using this technique. Rate-distortion optimized local QP selection (e.g., testing +/1 values of QP) was one suggested way to get improvement using local QP adjustment.

As used, the PWL mapping uses 16 equal size segments for the forward mapping, so determining the index is just a shift. For the inverse mapping, though, it is not using equal spacing. There are two contributions related to this, M0640 which tries to simplify the index determination and M0109 to make the scale factor for the luma processing be determined on a 4x4 basis instead of on a per-sample basis.

The spec would be written that CPR is not affected; the prediction signal in that case does not go through the special processing.

Revisit for review of test results.

JVET-M0427 CE12: Mapping functions (test CE12-1 and CE12-2) [T. Lu, F. Pu, P. Yin, W. Husak, S. McCarthy, T. Chen (Dolby)]

## CE13: Coding tools for 360° omnidirectional video (11)

Contributions in this category were discussed XXday X Jan. XXXX–XXXX (chaired by XXX).

JVET-M0033 CE13: Summary report on coding tools for 360° omnidirectional video [P. Hanhart, J.-L. Lin, C. Pujara]

JVET-M0143 CE13: Face row based geometry padding using projection with bilinear interpolation based on test 1.1.a (Test 2.1.b) [P. Hanhart, Y. He (InterDigital)]

JVET-M0144 CE13: Adaptive frame packing based on test 1.1.a (Test 4.1) [P. Hanhart, Y. He (InterDigital)]

JVET-M0235 CE13: HEC with Pre-rotation based on test 1.1a and 1.1b (Test 4.2) [C. Pujara, A. Konda, A. Singh, R. Gadde, W. Choi, K. Choi, K.P. Choi (Samsung)] [late]

JVET-M0320 CE13: HEC with deblocking using spherical neighbors, SAO and ALF disabled across face discontinuities (Test 1.4) [X. Huangfu, Y. Sun, L. Yu (Zhejiang Univ.)] [late]

JVET-M0321 CE13: Post-filtering of seam artifacts based on test 1.1.a (Test 3.1) [X. Huangfu, Y. Sun, L. Yu (Zhejiang Univ.)] [late]

JVET-M0355 CE13: Results on CE13.2.2 and CE13.5.2 [J. Sauer, M. Bläser (RWTH Aachen University)]

JVET-M0362 CE13: In-loop filters disabled across face discontinuities (Test 1.1.a and Test1.1.b) [S.-Y. Lin, L. Liu, J.-L. Lin, Y.-C. Chang, C.-C. Ju (MediaTek), P. Hanhart, Y. He (InterDigital)]

JVET-M0363 CE13: HEC with in-loop filters using spherical neighbors (Test 1.3) [S.-Y. Lin, L. Liu, J.-L. Lin, Y.-C. Chang, C.-C. Ju (MediaTek)]

JVET-M0364 CE13: Test 1.1.a with face row based geometry padding of reference pictures (Test 2.1.a, Test 2.1.c and Test 2.1.d) [C.-H. Shih, J.-L. Lin, Y.-C. Chang, C.-C. Ju (MediaTek)]

JVET-M0367 CE13: Face row based geometry padding of reference pictures and in-loop filters using spherical neighbors (Test 5.1) [C.-H. Shih, S.-Y. Lin, L. Liu, J.-L. Lin, Y.-C. Chang, C.-C. Ju (MediaTek)]

# Non-CE Technology proposals

## CE1 related – Partitioning (5)

Contributions in this category were discussed XXday X Jan. XXXX–XXXX (chaired by XXX).

JVET-M0195 CE1-related: Non-Residual Block on VPDU Boundary [A. Tamse, M. W. Park, M. Park, K. Choi (Samsung)]

JVET-M0630 Crosscheck of JVET-M0195 (CE1-related: Non-Residual Block on VPDU Boundary) [C. Rosewarne (Canon)] [late] [miss]

JVET-M0237 CE1-related: Transform tiling with residual reordering for pipelined processing of CTUs [C. Rosewarne, A. Dorrell (Canon)]

JVET-M0285 CE1-related: Prediction Mode Restriction and Implicit Transform Splitting to Enable VPDU [Y. Zhao, S. Esenlik, H. Yang, J. Chen (Huawei)]

JVET-M0628 Crosscheck of JVET-M0285 (CE1-related: Prediction Mode Restriction and Implicit Transform Splitting to Enable VPDU) [C. Rosewarne (Canon)] [late] [miss]

JVET-M0421 Non-CE1: Split-first signalling for partitioning [A. Wieckowski, T. Nguyen, H. Schwarz, D. Marpe, T. Wiegand (HHI)]

JVET-M0780 Crosscheck of JVET-M0421 (Non-CE1: Split-first signalling for partitioning) [Y. Yasugi, T. Ikai (Sharp)] [late]

JVET-M0888 CE1-related: Picture boundary handling with VPDU constraints [C.-M. Tsai, S.-T. Hsiang, C.-W. Hsu, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)] [late]

## CE2 related – Subblock motion compensation (31)

Contributions in this category were discussed XXday X Jan. XXXX–XXXX (chaired by XXX).

JVET-M0049 CE2-related: A restriction on memory bandwidth consumption of affine mode [M. Zhou (Broadcom)]

JVET-M0441 Crosscheck of JVET-M0049 (CE2-related: A restriction on memory bandwidth consumption of affine mode) [K. Zhang (Bytedance)] [late]

JVET-M0110 CE2-related: Alignment of affine control-point motion vector and subblock motion vector [H. Huang, W.-J. Chien, V. Seregin, H. Wang, M. Karczewicz (Qualcomm)]

JVET-M0747 Crosscheck of JVET-M0110 Test 2 (CE2-related: Alignment of affine control-point motion vector and subblock motion vector) [J. Luo (InterDigital)] [late] [miss]

JVET-M0737 Crosscheck of JVET-M0110 (CE2-related: Alignment of affine control-point motion vector and subblock motion vector) [A. Tamse (Samsung)] [late] [miss]

JVET-M0598 Crosscheck of JVET-M0110 (CE2-related: Alignment of affine control-point motion vector and subblock motion) [T. Zhou, T. Ikai (Sharp)] [late] [miss]

JVET-M0116 CE2-related: ATMVP simplification [R. Yu, D. Liu, K. Andersson, R. Sjöberg (Ericsson)]

JVET-M0460 Crosscheck of JVET-M0116 (CE2-related: ATMVP simplification) [L. Zhang (Bytedance)] [late] [miss]

JVET-M0145 Non-CE2: Motion vector clipping in affine sub-block motion vector derivation [P. Hanhart, Y. He (InterDigital)]

JVET-M0735 Crosscheck of JVET-M0145 (Non-CE2: Motion vector clipping in affine sub-block motion vector derivation) [H. Chen (Huawei)] [late] [miss]

JVET-M0166 CE2-related: Simplification of constructed affine merge candidate derivation [Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-M0575 Crosscheck of JVET-M0166 (CE2-related: Simplification of constructed affine merging candidate derivation) [Y. He (InterDigital)] [late] [miss]

JVET-M0167 CE2-related: Decoupling of SbTMVP and affine merge candidate derivation in subblock merge mode [Y.-L. Hsiao, T.-D. Chuang, C.-W. Hsu, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-M0850 Cross-check of JVET-M0167 (CE2-related: Decoupling of SbTMVP and affine merge candidate derivation in subblock merge mode) [S. Sethuraman (Ittiam)] [late]

JVET-M0168 CE2-related: Simplifications for inherited affine candidates [Y.-L. Hsiao, T.-D. Chuang, C.-W. Hsu, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-M0720 Crosscheck of JVET-M0168 (CE2-related: Simplifications for inherited affine candidates) [H. Chen, T. Solovyev (Huawei)] [late] [miss]

JVET-M0192 CE2-related: MV Derivation for Affine Chroma [A. Tamse, M. W. Park, K. Choi (Samsung)]

JVET-M0603 Crosscheck of JVET-M0192 (CE2-related: MV Derivation for Affine Chroma) [G. Li (Tencent)] [late] [miss]

JVET-M0204 CE2-related: Simplification of ATMVP [M. Park, M. W. Park, S. Jeong, A. Tamse, K. Choi (Samsung)]

JVET-M0594 Crosscheck of JVET-M0204 (CE2-related: Simplification of ATMVP) [T. Zhou, T. Ikai (Sharp)] [late]

JVET-M0217 CE2-related: Constructed affine merge candidate simplification [L. Li, J Nam, N Park, H Jang, J Lim, S Kim (LGE)]

JVET-M0719 Crosscheck of JVET-M0217 (CE2-related: Constructed affine merge candidate simplification) [H. Chen (Huawei)] [late] [miss]

JVET-M0228 CE2-related: Affine mode simplifications [Y.-W. Chen, X. Wang (Kwai Inc.)]

JVET-M0601 Crosscheck of JVET-M0228 (CE2-related: Affine mode simplifications) [T.-S. Chang, Y.-C. Sun, J. Lou (Alibaba)] [late]

JVET-M0240 CE2-related: Simplification of subblock-based temporal merging candidates [H. Lee, S.-C. Lim, J. Lee, J. Kang, H. Y. Kim (ETRI)]

JVET-M0620 Crosscheck of JVET-M0240 (CE2-related: Simplification of subblock-based temporal merging candidates) [Y.-W. Chen (Kwai Inc.)] [late] [miss]

JVET-M0247 CE2 related: Joint test of AMVR for Affine Inter mode (Test 2.1.1 and Test 2.1.2) [H. Liu, K. Zhang, L. Zhang, J. Xu (Bytedance), D. Luo, Y. He, X. Xiu (InterDigital)]

JVET-M0863 Crosscheck of JVET-M0247 (CE2-related: Joint Test of AMVR for Affine Inter Mode (Test 2.1.1 and Test 2.1.2)) [K. Choi] [late]

JVET-M0266 CE2-related: History-based affine merge candidates [K. Zhang, L. Zhang, H. Liu, J. Xu, Y. Wang, P. Zhao, D. Hong (Bytedance)]

JVET-M0662 Crosscheck of JVET-M0266 (CE2-related: History-based affine merge candidates) [R.-L. Liao, C. S. Lim (Panasonic)] [late] [miss]

JVET-M0268 Non-CE2: Interweaved Prediction for Affine Motion Compensation [K. Zhang, L. Zhang, H. Liu, J. Xu, Y. Wang, P. Zhao, D. Hong (Bytedance)]

JVET-M0744 Crosscheck of JVET-M0268 (Non-CE2: Interweaved Prediction for Affine Motion Compensation) [J. Luo (InterDigital)] [late] [miss]

JVET-M0270 CE2-related: An alternative storing method for affine inheritance [K. Zhang, L. Zhang, H. Liu, J. Xu, Y. Wang, P. Zhao, D. Hong (Bytedance)]

JVET-M0572 Crosscheck of JVET-M0270 (CE2-related: An alternative storing method for affine inheritance) [G. Li (Tencent)] [late] [miss]

JVET-M0273 CE2-related: Early awareness of accessing temporal blocks in sub-block merge list construction [L. Zhang, K. Zhang, H. Liu, J. Xu, Y. Wang, P. Zhao, D. Hong (Bytedance)]

JVET-M0531 Crosscheck of JVET-M0273 (CE2-related: Early awareness of accessing temporal blocks in sub-block merge list construction) [R. Yu (Ericsson)] [late] [miss]

JVET-M0310 CE2-related: Using shorter-tap filter for 4x4 sized partition [J. Li, R.-L. Liao, C. S. Lim (Panasonic)]

JVET-M0827 Crosscheck of JVET-M0310 (CE2-related: Using shorter-tap filter for 4x4 sized partition) [Y.-C. Lin (MediaTek)] [late] [miss]

JVET-M0311 CE2-related: Memory bandwidth reduction for affine mode with less dependency [J. Li, R.-L. Liao, C. S. Lim (Panasonic)]

JVET-M0622 Crosscheck of JVET-M0311 (CE2-related: Memory bandwidth reduction for affine mode with less dependency) [Y.-W. Chen (Kwai Inc.)] [late] [miss]

JVET-M0338 Non-CE2: Simplified derivation process for SbTMVP [H. Jang, J. Nam, S. Kim, J. Lim (LGE)]

JVET-M0535 Crosscheck of JVET-M0338 (Non-CE2: Simplified neighboring spatial coding unit derivation for SbTMVP) [R. Yu (Ericsson)] [late] [miss]

JVET-M0343 Non-CE2: Simplified derivation process for SbTMVP [H. Jang, J. Nam, S. Kim, J. Lim (LGE)]

JVET-M0690 Cross check of JVET-M0343 [K. Misra (Sharp Labs of America)] [late]

JVET-M0382 CE2-related: Modification of Triangle and MMVD merge indexes coding [G. Laroche, C. Gisquet, P. Onno, J. Taquet (Canon)]

JVET-M0560 Crosscheck of JVET-M0382 (CE2-related: Modification of Triangle and MMVD merge indexes coding) [H. Lee, S.-C. Lim, J. Lee, J. Kang (ETRI)] [late] [miss]

JVET-M0400 CE2-related: Worst-case memory bandwidth reduction for VVC [W.-J. Chien, L. Pham Van, H. Huang, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-M0665 Crosscheck of JVET-M0400 [S. Jeong, K. Choi (Samsung)] [late] [miss]

JVET-M0406 CE2/4-related: Unified merge list size for block and sub-block merge modes [X. Xu, X. Li, S. Liu (Tencent)]

JVET-M0576 Crosscheck of JVET-M0406 (CE2/4-related: Unified merge list size for block and sub-block merge modes) [C.-Y. Lai (MediaTek)] [late] [miss]

JVET-M0432 CE2-related: Combination of CE2.2.3.d and affine inheritance from motion data line buffer [G. Li, X. Xu, X. Li, S. Liu (Tencent), J. Zhao, S. Kim (LGE)]

JVET-M0740 Crosscheck of JVET-M0432 (CE2-related: Combination of CE2.2.3.d and affine inheritance from motion data line buffer) [A. Tamse (Samsung)] [late] [miss]

JVET-M0434 CE2-related: Constraint on constructed affine merge candidates [G. Li, X. Xu, X. Li, S. Liu (Tencent)]

JVET-M0546 Crosscheck of JVET-M0434 (CE2-related: Constraint on constructed affine merge candidates) [L. Zhang (Bytedance)] [late] [miss]

JVET-M0462 CE2-related: 4x4 chroma affine motion compensation and motion vector rounding unification [L. Pham Van, W.-J. Chien, H. Huang, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-M0859 Crosscheck of JVET-M0462: CE2-related: 4x4 chroma affine motion compensation and motion vector rounding unification [X. Zheng, Y. Wang (DJI)] [late] [miss]

JVET-M0467 CE2-related: Symmetric MVD for Affine Bi-prediction Coding [J. Luo, Y. He (InterDigital)]

JVET-M0795 Crosscheck of JVET-M0467 (CE2-related: Symmetric MVD for Affine Bi-prediction Coding) [C.-C. Chen, W.-J. Chien (Qualcomm)] [late] [miss]

JVET-M0490 CE2-related: Simplified context model for triangular prediction mode [M. Gao, X. Li, S. Liu (Tencent)]

JVET-M0753 Crosscheck of JVET-M0490 (CE2-related: Simplified context model for triangular prediction mode) [R.-L. Liao, C. S. Lim (Panasonic)] [late] [miss]

JVET-M0515 Non-CE2.5: ATMVP Collocated Block Derivation from History-based Candidate [C.-C. Chen, W.-J. Chien, Y. Zhang, C.-H. Hung, Y. Han, H. Huang, M. Karczewicz (Qualcomm)]

JVET-M0746 Crosscheck of JVET-M0515 (Non-CE2.5: ATMVP Collocated Block Derivation from History-based Candidate) [J. Luo (InterDigital)] [late] [miss]

JVET-M0526 CE2-related: Further simplification of ATMVP collocated block derivation [S.H. Wang (Peking University), X. Zheng (DJI), S.S. Wang, S.W. Ma (Peking University)] [late]

Initial upload rejected as placeholder

JVET-M0602 Cross-check of JVET-M0526: CE2-related: Further simplification of ATMVP collocated block derivation [S. Bandyopadhyay (InterDigital)] [late] [miss]

JVET-M0702 CE2-related: Adaptive sub-block MV clip for affine blocks [X. Li, M. Gao, S. Liu (Tencent)] [late]

JVET-M0756 CE2.2.7 related: Affine temporal constructed candidates without pruning [F. Galpin, A. Robert, F. Le Léannec, T. Poirier (Technicolor)] [late] [miss]

JVET-M0812 Crosscheck of JVET-M0756: CE2.2.7 related: Affine temporal constructed candidates without pruning [L. Pham Van, G. Van der Auwera, H. Huang, M. Karczewicz (Qualcomm)] [late]

JVET-M0839 CE2-related: On number of fast merge candidates for Affine Merge mode [A. Robert, F. Le Léannec, F. Galpin (Technicolor)] [late]

JVET-M0889 Crosscheck of JVET-M0839 (CE2-related: On number of fast merge candidates for Affine Merge mode) [Y.-W. Chen (Kwai Inc.)] [late] [miss]

## CE3 related – Intra prediction and mode coding (48)

Contributions in this category were discussed XXday X Jan. XXXX–XXXX (chaired by XXX).

See also JVET-M0203 which contains a new proposal called 3.5.1 that was not part of the CE.

JVET-M0044 Non-CE3: Alternative LM Chroma Implementation [S. Keating, K. Sharman (Sony)]

JVET-M0045 Non-CE3: PDPC Restriction [S. Keating, K. Sharman (Sony)]

JVET-M0561 Crosscheck of JVET-M0045 (Non-CE3: PDPC Restriction) [J. Lee, H. Lee, S.-C. Lim, J. Kang (ETRI)] [late]

JVET-M0047 Non-CE3: Intra Angular Prediction and Modified PDPC Based on Two Reference Lines [D. Jiang, J. Lin, F. Zeng, C. Fang (Dahua)]

JVET-M0048 Non-CE3: Modified Chroma Derived Mode [F. Zeng, J. Lin, D. Jiang, C. Fang (Dahua)]

JVET-M0064 Non-CE3: CCLM table reduction and bit range control [Y. Yasugi, F. Bossen, E. Sasaki (Sharp)]

JVET-M0548 Crosscheck of JVET-M0064 (Non-CE3: CCLM table reduction and bit range control) [C.-M. Tsai (MediaTek)] [late] [miss]

JVET-M0093 Non-CE3: Improved robustness for calculation of cross-component linear model parameters [C. Helmrich (HHI)]

JVET-M0095 Non-CE3: Intra simplifications [G. Van der Auwera, A. K. Ramasubramonian, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-M0676 Crosscheck of JVET-M0095 (Non-CE3: Intra simplifications) [P. Merkle (HHI)] [late]

JVET-M0099 Non-CE3: Partial sorting for non-MPM modes [A. K. Ramasubramonian, G. Van der Auwera, T. Hsieh, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-M0658 Crosscheck of JVET-M0099 (Non-CE3: Partial sorting for non-MPM modes) [K. Choi (Samsung)] [late] [miss]

JVET-M0100 CE3-related: DM-dependent chroma intra prediction modes [G. Rath, F. Urban, F. Racapé (Technicolor)]

JVET-M0439 Crosscheck of JVET-M0100 (CE3-related: DM-dependent chroma intra prediction modes) [H. Liu (Bytedance)] [late] [miss]

JVET-M0108 CE3-related: Reducing the number of reference samples and table size in LM Chroma process [E. François, T. Poirier, F. Le Léannec (Technicolor)]

JVET-M0571 Crosscheck of JVET-M0108 (CE3-related: Reducing the number of reference samples and table size in LM Chroma process) [L. Zhang (Bytedance)] [late] [miss]

JVET-M0643 Crosscheck of JVET-M0108 (CE3-related: Reducing the number of reference samples and table size in LM Chroma process) [P. Hanhart (InterDigital)] [late] [miss]

JVET-M0122 Non-CE3: On block size restrictions for PDPC [A. Filippov, V. Rufitskiy, J. Chen (Huawei)]

JVET-M0880 Cross-check of contribution JVET-M0122, test 3.a (Non-CE3: On block size restrictions for PDPC) [F. Racapé (Technicolor)] [late] [miss]

JVET-M0806 Cross-check of contribution JVET-M0122 (Non-CE3: On block size restrictions for PDPC) [M. Schäfer, J. Pfaff (HHI)] [late]

JVET-M0138 Non-CE3: Intra reference sample deblocking [Z. Zhang, K. Andersson, R. Sjöberg (Ericsson)]

JVET-M0550 Crosscheck of JVET-M0138 (Non-CE3: Intra reference sample deblocking) [C.-M. Tsai (MediaTek)] [late] [miss]

JVET-M0139 Non-CE3: History-based intra most probable modes derivation [Z. Zhang, P. Wennersten, R. Yu, J. Ström, R. Sjöberg (Ericsson)]

JVET-M0639 Crosscheck of JVET-M0139 (Non-CE3: History-based intra most probable modes derivation) [B. Wang (Huawei)] [late] [miss]

JVET-M0443 Crosscheck of JVET-M0139 (Non-CE3: History-based intra most probable modes derivation) [J. Li, L. Zhang (Bytedance)] [late] [miss]

JVET-M0146 Non-CE3: MDLM template downsampling [P. Hanhart, Y. He (InterDigital)]

JVET-M0733 Crosscheck of JVET-M0146 (Non-CE3: MDLM template downsampling) [C. Chevance (Technicolor)] [late]

JVET-M0149 Non-CE3: On simplification of PDPC basic equation [A. Filippov, V. Rufitskiy, J. Chen (Huawei)]

JVET-M0677 Crosscheck of JVET-M0149 (Non-CE3: simplification of PDPC basic equation) [F. Racapé (Technicolor)] [late] [miss]

JVET-M0158 Non-CE3: LUT-free interpolation filters for intra prediction [A. Filippov, V. Rufitskiy, J. Chen (Huawei)]

JVET-M0805 Cross-check of contribution JVET-M0158 (Non-CE3: LUT-free interpolation filters for intra prediction) [M. Schäfer, J. Pfaff (HHI)] [late]

JVET-M0680 Crosscheck of JVET-M0158 (Non-CE3: LUT-free interpolation filters for intra prediction) [F. Racapé (Technicolor)] [late] [miss]

JVET-M0191 CE3-related: Construction of non-MPM mode list in intra prediction [S. Cha, G. Lee, G. Kim, J. Han (Sejong University)]

JVET-M0242 Crosscheck of JVET-M0191 (CE3-related: Construction of non-MPM mode list in intra prediction) [J. Lee, H. Lee, S.-C. Lim, J. Kang (ETRI)] [late]

JVET-M0210 Non-CE3: Intra prediction information coding [J. Yao, J. Zhu, W. Cai, K. Kazui (Fujitsu)]

JVET-M0758 Crosscheck of JVET-M0210 (Non-CE3: Intra prediction information coding) [C.-C. Kuo, C.-H. Yau, C.-C. Lin (ITRI)] [late] [miss]

JVET-M0211 CE3-related: Fixed Reference Samples Design for CCLM [J.-Y. Huo, X.-W. Li, J.-L. Wang, Y.-Z. Ma, F.-Z. Yang (Xidian Univ.), S. Wan (NPU), Y.-F. Yu, Y. Liu (OPPO)]

JVET-M0716 Crosscheck of JVET-M0211 (CE3-related:Fixed Reference Samples Design for CCLM) [X. Ma (Huawei)] [late] [miss]

JVET-M0212 CE3-related: Improved reference samples range for MDLM [S. Wan (NPU), Q.-H.Ran, X.-W. Li, Y.-Z. Ma, J.-Y. Huo, F.-Z. Yang (Xidian Univ.), Y.-F. Yu, Y. Liu (OPPO)]

JVET-M0717 Crosscheck of JVET-M0212 (CE3-related:Improved reference samples range for MDLM) [X. Ma (Huawei)] [late] [miss]

JVET-M0213 CE3-related: Chroma intra candidates modification based on directional DM [Y.-Z. Ma, J.-L. Wang, X.-W. Li, J.-Y. Huo, F.-Z. Yang (Xidian Univ.), S. Wan (NPU), Y.-F. Yu, Y. Liu (OPPO)]

JVET-M0718 Crosscheck of JVET-M0213 (CE3-related: Chroma intra candidates modification based on directional DM) [X. Ma (Huawei)] [late] [miss]

JVET-M0214 CE3-related: Uniform Chroma intra candidates modification based on DM [Y.-Z. Ma, X.-W. Li, J.-L. Wang, J.-Y. Huo, F.-Z. Yang (Xidian Univ.), S. Wan (NPU), Y.-F. Yu, Y. Liu (OPPO)]

JVET-M0219 CE3-related: Reduced number of reference samples for CCLM parameter calculation [J. Choi, J. Heo, S. Yoo, L. Li, J. Choi, J. Lim, S. Kim (LGE)]

JVET-M0586 Crosscheck of JVET-M0219 (CE3-related: Reduced number of reference samples for CCLM parameter calculation) [K. Zhang (Bytedance)] [late] [miss]

JVET-M0229 CE3-related: Simplification of LM Mode [Y.-W. Chen, X. Wang (Kwai Inc.)]

JVET-M0632 Crosscheck of JVET-M0229 (CE3-related: Simplification of LM Mode) [K. Abe (Panasonic)] [late] [miss]

JVET-M0238 Non-CE3: Modification of PDPC [J. Lee, H. Lee, S.-C. Lim, J. Kang, H. Y. Kim (ETRI)]

JVET-M0678 Crosscheck of JVET-M0238 (Non-CE3: Modification of PDPC) [S. Keating (Sony)] [late]

JVET-M0239 Non-CE3: Modification of MPM derivation [J. Lee, H. Lee, S.-C. Lim, J. Kang, H. Y. Kim (ETRI)]

JVET-M0609 Crosscheck of JVET-M0239 (Non-CE3: Modification of MPM derivation) [B. Wang (Huawei)] [late] [miss]

JVET-M0258 CE3-related: Chroma intra candidates modification based on non-directional DM [J.-Y. Huo, J.-L. Wang, X.-W. Li, Y.-Z. Ma, F.-Z. Yang (Xidian Univ.), S. Wan (NPU), Y.-F. Yu, Y. Liu (OPPO)]

JVET-M0274 CE3-related: Modified linear model derivation for CCLM modes [M. Wang, K. Zhang, L. Zhang, H. Liu, J. Xu, S. Wang (Bytedance), J. Li, S. Wang, W. Gao (Peking University)]

JVET-M0641 Crosscheck of JVET-M0274 (CE3-related: Modified linear model derivation for CCLM modes) [E. François (Technicolor)] [late] [miss]

JVET-M0295 CE3-related: Harmonization of MPM list construction [B. Wang, S. Esenlik, A.M. Kotra, H. Gao, J. Chen (Huawei)]

JVET-M0865 Crosscheck of JVET-M0295 (CE3-related: Harmonization of MPM list construction) [L Li (LGE)] [late] [miss]

JVET-M0324 CE3-related: Modified Chroma Intra Mode Coding [J. Park, B. Jeon (SKKU)]

JVET-M0818 Crosscheck of JVET-M0324 (CE3-related: Modified Chroma Intra Mode Coding) [L. Zhang (Bytedance)] [late] [miss]

JVET-M0356 CE3-related: simplified calculation for CCLM parameters derivation [A. Filippov, X. Ma, V. Rufitskiy, H. Yang, J. Chen (Huawei)]

JVET-M0679 Crosscheck of JVET-M0356 (CE3-related: On CCLM simplification) [F. Racapé (Technicolor)] [late] [miss]

JVET-M0723 Crosscheck of JVET-M0356 (CE3-related: simplified calculation for CCLM parameters derivation) [G. Laroche, P. Onno (Canon)] [late] [miss]

JVET-M0358 CE3-related: disabling PDPC based on availability of reference samples [V. Drugeon (Panasonic)]

JVET-M0629 Crosscheck of JVET-M0358 (CE3-related: disabling PDPC based on availability of reference samples) [C. Rosewarne (Canon)] [late] [miss]

JVET-M0365 Non-CE3: modified PDPC for horizontal and vertical modes [A. Filippov, V. Rufitskiy, J. Chen (Huawei)]

JVET-M0804 Cross-check of contribution JVET-M0365 (Non-CE3: modified PDPC for horizontal and vertical modes) [M. Schäfer, J. Pfaff (HHI)] [late]

JVET-M0383 Non-CE3: Table size reduction and bit width limitation for CCLM implementation [P. Onno, C. Gisquet, G. Laroche, J. Taquet (Canon)]

JVET-M0741 Crosscheck of JVET-M0383 (Non-CE3: Table size reduction and bit width limitation for CCLM implementation) [A. Filippov, V. Rufitskiy (Huawei)] [late] [miss]

JVET-M0384 Non-CE3: LM in the middle [C. Gisquet, G. Laroche, P. Onno, J. Taquet (Canon)]

JVET-M0689 Crosscheck of JVET-M0384 (Non-CE3: LM in the middle) [J. Lainema (Nokia)] [late] [miss]

JVET-M0391 CE3-related: Improvements on the Decoder-side Intra Mode Derivation [M. Abdoli, E. Mora, T. Guionnet, M. Raulet (ATEME)]

JVET-M0651 Crosscheck of JVET-M0391 (CE3-related: Improvements on the Decoder-side Intra Mode Derivation) [F. Henry, G. Clare (Orange)] [late] [miss]

JVET-M0392 Non-CE3: Extended Mode-Dependent Intra Smoothing [A. Filippov, V. Rufitskiy, J. Chen (Huawei)]

JVET-M0803 Cross-check of contribution JVET-M0392 (Non-CE3: Extended Mode-Dependent Intra Smoothing) [M. Schäfer, J. Pfaff (HHI)] [late]

JVET-M0426 CE3-related: Improvement on the Intra Sub-Partitions Coding Mode [S. De-Luxán-Hernández, V. George, J. Ma, T. Nguyen, H. Schwarz, D. Marpe, T. Wiegand (HHI)]

JVET-M0458 Non-CE3: Combined-Hypothesis Intra-Prediction [G. Kulupana, A. Seixas Dias, S. Blasi (BBC)]

JVET-M0613 Crosscheck of JVET-M0458 (Non-CE3: Combined-Hypothesis Intra-Prediction) [S. De-Luxán-Hernández (HHI)] [late] [miss]

JVET-M0478 Non-CE3: PDPC extension [G. Van der Auwera, A. K. Ramasubramonian, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-M0682 Crosscheck of JVET-M0478 (Non-CE3: PDPC extension) [F. Racapé (Technicolor)] [late] [miss]

JVET-M0493 CE3-related: Simplified look-up table for CCLM mode [L. Zhao, X. Zhao, X. Li, S. Liu (Tencent)]

JVET-M0624 Crosscheck of JVET-M0493 (CE3-related: Simplified look-up table for CCLM mode) [Y.-W. Chen (Kwai Inc.)] [late] [miss]

JVET-M0494 CE3-related: Modifications on MPM list generation [L. Zhao, X. Zhao, X. Li, S. Liu (Tencent)]

JVET-M0745 Crosscheck of JVET-M0494 (CE3-related: Modifications on MPM list generation) [J. Luo (InterDigital)] [late] [miss]

JVET-M0524 CE3/6-related: Unification of RQT-like transform partitioning for intra and inter blocks [H. Egilmez, V. Seregin, A. Said, M. Karczewicz (Qualcomm)]

JVET-M0528 Non-CE3: A unified luma intra mode list construction process [F. Bossen, K. Misra (Sharp Labs of America)]

JVET-M0626 Crosscheck of JVET-M0528 (Non-CE3: A unified luma intra mode list construction process) [J. Yao (Fujitsu)] [late] [miss]

JVET-M0817 Crosscheck of JVET-M0528 (Non-CE3: A unified luma intra mode list construction process) [L. Zhao, X. Zhao (Tencent)] [late] [miss]

JVET-M0653 Non-CE3: Harmonization of integer-slope directional modes without interpolation filtering process [A. Filippov, V. Rufitskiy, J. Chen (Huawei)] [late] [miss]

JVET-M0802 Cross-check of contribution JVET-M0653 (Non-CE3: Harmonization of integer-slope directional modes without interpolation filtering process) [M. Schäfer, J. Pfaff (HHI)] [late]

JVET-M0783 CE3-related: Modification of MPM list order [J. Heo, J. Choi, J. Choi, S. Yoo, L. Li, J. Lim, S. Kim (LGE)] [late]

JVET-M0799 Bit-width reduction of multiplier in CCLM derivation and prediction [K. Kawamura, S. Naito (KDDI)] [late]

JVET-M0844 Crosscheck of JVET-M0799 (Bit-width reduction of multiplier in CCLM derivation and prediction) [Y. Kato, K. Abe, T. Toma (Panasonic)] [late]

JVET-M0814 Non-CE3: block size restriction on PDPC [L. Li, J. Heo, J. Choi, S. Yoo, J. Choi, J. Lim, S. Kim (LGE)] [late] [miss]

JVET-M0881 Crosscheck of JVET-M0814 (Non-CE3: block size restriction on PDPC) [A. Filippov, V. Rufitskiy (Huawei)] [late] [miss]

JVET-M0815 CE3-related: Harmonization on MPM list [L Li, J Heo, J Choi, S Yoo, J Choi, J Lim, S.Kim (LGE)] [late] [miss]

JVET-M0832 Non-CE3: On block size restrictions for PDPC with disabled linear filtering for PDPC in the case of skew non-diagonal modes [A. Filippov, V. Rufitskiy, J. Chen (Huawei), J. Lee, J. Kang (ETRI)] [late]

JVET-M0866 Crosscheck of JVET-M0832 (Non-CE3: On block size restrictions for PDPC with disabled linear filtering for PDPC in the case of skew non-diagonal modes) [L Li (LGE)] [late] [miss]

## CE4 related – Inter prediction and motion vector coding (51)

Contributions in this category were discussed XXday X Jan. XXXX–XXXX (chaired by XXX).

JVET-M0067 Non-CE4: Weighted prediction with BDOF and bi-prediction with CU weights harmonization [T. Hashimoto, T. Chujoh, T. Ikai, E. Sasaki (Sharp)]

JVET-M0394 Crosscheck of JVET-M0067 (Non-CE4: Weighted prediction with BDOF and bi-prediction with CU weights harmonization) [P. Bordes (Technicolor)] [late]

JVET-M0068 Non-CE4: MMVD scaling fix [E. Sasaki, T. Ikai (Sharp)]

JVET-M0509 Crosscheck of JVET-M0068 (Non-CE4: MMVD scaling fix) [X. Chen (HiSilicon)] [late] [miss]

JVET-M0069 Non-CE4: Syntax change of MMVD [E. Sasaki, T. Chujoh, T. Ikai (Sharp)]

JVET-M0554 Crosscheck of JVET-M0069 (Non-CE4: Syntax change of MMVD) [G. Li (Tencent)] [late] [miss]

JVET-M0585 Crosscheck of JVET-M0069 (Non-CE4: Syntax change of MMVD) [K. Zhang (Bytedance)] [late] [miss]

JVET-M0081 Non-CE4: Simplification of AMVP list generation in AMVR [Y. Kato, K. Abe, T. Toma (Panasonic)]

JVET-M0807 Cross-check result of JVET-M0081 (Non-CE4: Simplification of AMVP list generation in AMVR) [K. Kawamura, S. Naito (KDDI)] [late]

JVET-M0111 AHG13: On bi-prediction with weighted averaging and weighted prediction [Y. Ye, J. Chen, M. Yang (Alibaba), P. Bordes, E. François (Technicolor)]

JVET-M0117 CE4-related: On MVP candidate list generation for AMVP [R. Yu, D. Liu, K. Andersson, P. Wennersten, J. Ström, R. Sjöberg (Ericsson)]

JVET-M0763 Cross-check of JVET-M0117 [H. Jang, J. Nam, S.-H. Kim, J. Lim (LGE)] [late] [miss]

JVET-M0127 CE4-related: Modification on Merge List [Y. Han, W.-J. Chien, D. Rusanovskyy, Y.-H. Chao, C.-C. Chen, H. Wang, H. Huang, C.-H. Hung, Y. Zhang, M. Karczewicz (Qualcomm)]

JVET-M0532 Crosscheck of JVET-M0127 (CE4-related: Modification on Merge List) [H. Dou, Z. Deng, L. Xu (Intel)] [late] [miss]

JVET-M0171 CE4-related: MMVD cleanups [C.-Y. Lai, T.-D. Chuang, Y.-L. Hsiao, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-M0757 Cross-check of JVET-M0171 (CE4-related: MMVD cleanups) [B.-J. Fuh, C.-H. Yau, C.-C. Lin (ITRI)] [late] [miss]

JVET-M0193 CE4-related: Pairwise Average Candidate Reduction [A. Tamse, M. W. Park, K. Choi (Samsung)]

JVET-M0551 Crosscheck of JVET-M0193 (CE4-related: Pairwise average candidate reduction) [S.-T. Hsiang (MediaTek)] [late] [miss]

JVET-M0206 CE4-related: MMVD improvements [S. Jeong, M. W. Park, K. Choi (Samsung)]

JVET-M0595 Crosscheck of JVET-M0206 (CE4-related: MMVD improvements) [T. Hashimoto, T. Ikai (Sharp)] [late] [miss]

JVET-M0220 Non-CE4: Subjective quality analysis of non-sub-block ATMVP [Y.-H Chao, W.-J Chien, M. Karczewicz (Qualcomm)]

JVET-M0230 CE4-related: Temporal MV buffer reduction [Y.-W. Chen, X. Wang (Kwai Inc.)]

JVET-M0591 Crosscheck of JVET-M0230 (Non-CE4: Temporal MV buffer reduction) [T. Zhou, T. Ikai (Sharp)] [late]

JVET-M0231 CE4-related: Regular merge flag coding [X. Wang, Y.-W. Chen (Kwai Inc.)]

JVET-M0559 Crosscheck of JVET-M0231 (Non-CE4: Regular merge flag coding) [H. Lee, S.-C. Lim, J. Lee, J. Kang (ETRI)] [late] [miss]

JVET-M0845 Crosscheck of JVET-M0231 (Non-CE4: Regular merge flag coding) [T. Hashimoto, T. Ikai (Sharp)] [late]

JVET-M0264 Non-CE4: Harmonization between HMVP and GBi [J. Li, S. Wang, W. Gao (Peking University), L. Zhang, K. Zhang, H. Liu, J. Xu (Bytedance), X. Xiu, D. Luo, Y. He (InterDigital)]

JVET-M0574 Crosscheck of JVET-M0264 (Non-CE4: Harmonization between HMVP and GBi) [J. Zhao (LGE)] [late]

JVET-M0267 Non-CE4: Harmonization of MMVD and AMVR [K. Zhang, L. Zhang, H. Liu, J. Xu, Y. Wang, P. Zhao, D. Hong (Bytedance)]

JVET-M0599 Crosscheck of JVET-M0267 (Non-CE4: Harmonization of MMVD and AMVR) [T. Hashimoto, T. Ikai (Sharp)] [late] [miss]

JVET-M0272 CE4-related: Restrictions on History-based Motion Vector Prediction [L. Zhang, K. Zhang, H. Liu, J. Xu, Y. Wang, P. Zhao, D. Hong (Bytedance)]

JVET-M0642 Crosscheck of JVET-M0272 on CE4-related: Restrictions on History-based Motion Vector Prediction [X. Li (Tencent)] [late] [miss]

JVET-M0286 Non-CE4: Simplifications for triangular prediction mode [T. Solovyev, S. Esenlik, S. Ikonin, J. Chen (Huawei)]

JVET-M0793 Crosscheck of JVET-M0286 (Non-CE4: Simplifications for Triangular Prediction Mode) [C.-C. Chen, W.-J. Chien (Qualcomm)] [late] [miss]

JVET-M0300 CE4-related: HMVP and parallel processing with tiles and tile groups [A. M. Kotra, J. Chen, B. Wang, S. Esenlik, H. Gao (Huawei)]

JVET-M0307 CE4-related: Candidates optimization on MMVD [N. Park, H. Jang, J. Nam, J. Lim, S. Kim (LGE)]

JVET-M0621 Crosscheck of JVET-M0307 (CE4-related: Candidates optimization on MMVD) [Y.-W. Chen (Kwai Inc.)] [late] [miss]

JVET-M0779 Crosscheck of JVET-M0307 (CE4-related: Candidates optimization on MMVD) [T. Hashimoto, T. Ikai (Sharp)] [late]

JVET-M0308 Non-CE4: MMVD simplification [X. Chen, J. Zheng (HiSilicon)]

JVET-M0592 Crosscheck of JVET-M0308 (Non-CE4: MMVD simplification) [T. Hashimoto, T. Ikai (Sharp)] [late]

JVET-M0314 CE4-related: MMVD improving with signaling distance table [J. Li, R.-L. Liao, C. S. Lim (Panasonic)]

JVET-M0597 Crosscheck of JVET-M0314 (CE4-related: MMVD improving with signaling distance table) [T. Hashimoto, T. Ikai (Sharp)] [late] [miss]

JVET-M0315 Non-CE4: MMVD scaling simplification [J. Li, R.-L. Liao, C. S. Lim (Panasonic)]

JVET-M0794 Crosscheck of JVET-M0315 (Non-CE4: MMVD Scaling Simplification) [C.-C. Chen, W.-J. Chien (Qualcomm)] [late] [miss]

JVET-M0330 CE4-related: Simplification of MMVD scheme [L. Xu, F. Chen, L. Wang (Hikvision)]

JVET-M0590 Crosscheck of JVET-M0330 (CE4-related: Simplification of candidate list derivation for MMVD mode) [T. Hashimoto, T. Ikai (Sharp)] [late] [miss]

JVET-M0345 CE4-related: Remove redundancy between TMVP and ATMVP [S. H. Wang (Peking University), X. Zheng (DJI), S.S. Wang, S.W. Ma (Peking University)]

JVET-M0754 Cross-check of JVET-M0345: CE4-related: Remove redundancy between TMVP and ATMVP [S. Bandyopadhyay (InterDigital)] [late] [miss]

JVET-M0346 CE4-related: Non-square compression grid for temporal motion data storage [S. H. Wang (Peking University), X. Zheng (DJI), S.S. Wang, S.W. Ma (Peking University)]

JVET-M0348 CE4-related: Improvement on 4x4 bi-prediction [X. W. Meng (Peking University), X. Zheng (DJI), S.S. Wang, S.W. Ma (Peking University)]

JVET-M0809 Crosscheck of JVET-M0348: CE4-related: Further reducing VVC memory bandwidth worst case by combining 4x4/4x8/8x4 bi-prediction with AMVR [L. Pham Van, W.-J. Chien, M. Karczewicz (Qualcomm)] [late]

JVET-M0350 CE4-related: Quadtree-based Merge Estimation Region for VVC [T. L. Fu (Peking University), X. Zheng (DJI), S.S Wang, S.W. Ma (Peking University)]

JVET-M0583 Crosscheck of JVET-M0350 (CE4-related: Quadtree-based Merge Estimation Region for VVC) [G. Li (Tencent)] [late] [miss]

JVET-M0359 Non-CE4: Modification of merge data syntax [G. Ko, D. Kim, J. Jung, J. Son, J. Kwak (WILUS)]

JVET-M0789 Crosscheck of JVET-M0359 (Non-CE4: Modification of merge data syntax) [B. Lee (Chosun Univ.)] [late]

JVET-M0369 CE4-related: Syntax changes of merge data [Y. Ahn, D. Sim (Digital Insights)] [late]

JVET-M0750 Cross-check of JVET-M0369 (CE4-related: Syntax changes of merge data) [S.-C. Lim, H. Lee, J. Lee, J. Kang (ETRI)] [late] [miss]

JVET-M0405 CE4-related: Simplified merge candidate list for small blocks [X. Xu, X. Li, S. Liu (Tencent)]

JVET-M0659 Crosscheck of JVET-M0405 (CE4-related: Simplified merge candidate list for small blocks) [K. Choi (Samsung)] [late] [miss]

JVET-M0406 CE2/4-related: Unified merge list size for block and sub-block merge modes [X. Xu, X. Li, S. Liu (Tencent)]

JVET-M0422 CE4-related: Simplified MVD coding [X. Li, X. Xu, X. Zhao, S. Liu (Tencent)]

JVET-M0440 Crosscheck of JVET-M0422 (CE4-related: Simplified MVD coding) [L. Zhang (Bytedance)] [late] [miss]

JVET-M0433 CE4-related: Constraint on GBi index inheritance in Merge Mode [G. Li, X. Xu, X. Li, S. Liu (Tencent)]

JVET-M0700 Cross-check of JVET-M0433 (CE4-related: Constraint on GBi index inheritance in Merge Mode [J. Zhao (LGE)]

JVET-M0435 CE4-related: MMVD offset table signaling [G. Li, X. Xu, X. Li, S. Liu (Tencent)]

JVET-M0623 Crosscheck of JVET-M0435 (CE4-related: MMVD offset table signaling) [Y.-W. Chen (Kwai Inc.)] [late] [miss]

JVET-M0436 AHG2: Regarding HMVP Table Size [J. Zhao, S. Kim (LGE)]

JVET-M0562 Cross-check of JVET-M0436: AHG2: Regarding HMVP Table Size [S. Bandyopadhyay (InterDigital)] [late] [miss]

JVET-M0437 Non-CE4: Size constraint on MMVD [J. Zhao, S. Kim (LGE)]

JVET-M0555 Crosscheck of JVET-M0437 (Non-CE4: Size constraint on MMVD) [G. Li (Tencent)] [late]

JVET-M0444 CE4-related: Simplified symmetric MVD based on CE4.4.3 [J. Luo, Y. He (InterDigital)]

JVET-M0721 Crosscheck of JVET-M0444 (CE4-related: Simplified symmetric MVD based on CE4.4.3) [H. Chen, T. Solovyev (Huawei)] [late] [miss]

JVET-M0448 CE4-related: Triangle merge index signaling [M. Xu, X. Li, S. Liu (Tencent)]

JVET-M0836 Crosscheck of JVET-M0448 (CE4-related: Triangle merge index signaling) [X. Wang (Kwai Inc.)] [late] [miss]

JVET-M0473 Simplified HMVP [W. Zhu, A. Segall (Sharp)]

JVET-M0778 Crosscheck of JVET-M0473: Simplified HMVP [C.-H. Hung, W.-J. Chien (Qualcomm)] [late]

JVET-M0479 Non-CE4: On clipping of scaled motion vectors [K. Misra, F. Bossen (Sharp)]

JVET-M0777 Crosscheck of JVET-M0479: Non-CE4: On clipping of scaled motion vectors [C.-H. Hung, W.-J. Chien (Qualcomm)] [late]

JVET-M0484 Non-CE4: Line buffer size reduction method for generalized bi prediction [T. Solovyev, H. Gao, S. Esenlik, S. Ikonin, J. Chen (Huawei)]

JVET-M0819 Crosscheck of JVET-M0484 (Non-CE4: Line buffer size reduction method for generalized bi prediction) [A. Robert (Technicolor)] [late]

JVET-M0519 Non-CE: Context modeling for coding the prediction mode flag [S.-T. Hsiang, S.-M. Lei (MediaTek)]

JVET-M0739 Crosscheck of JVET-M0519 (Non-CE: Context modeling for coding the prediction mode flag) [A. Tamse (Samsung)] [late] [miss]

JVET-M0502 CE4-related: Improved context for prediction mode flag [X. Zhao, X. Li, S. Liu (Tencent)]

JVET-M0657 Crosscheck of JVET-M0502 (CE4-related: Improved context for prediction mode flag) [K. Choi (Samsung)] [late] [miss]

JVET-M0513 CE7-related: Context modeling of pred_mode_flag [Y. Zhao, S. Hong, H. Yang, J. Chen (Huawei)]

JVET-M0882 Cross check of CE7-related: Context modeling of pred_mode_flag (JVET-M0513) [M. W. Park, H. Yang (Samsung)] [late]

JVET-M0507 CE4-related: Hybrid Merge Estimation Region [H. Wang, V. Seregin, W.-J. Chien, T. Hsieh, Y. Han, M. Karczewicz (Qualcomm)]

JVET-M0712 Crosscheck of JVET-M0507 (CE4-related: Hybrid Merge Estimation Region) [F. Chen, L. Wang (Hikvision)] [late] [miss]

JVET-M0837 Crosscheck of JVET-M0507 (CE4-related: Hybrid Merge Estimation Region) [X. Wang (Kwai Inc.)] [late] [miss]

JVET-M0512 Non-CE4: On Temporal Motion Buffer Compression [F. Bossen, K. Misra, A. Segall (Sharp Labs of America)]

JVET-M0764 Cross-check of JVET-M0512 [H. Jang, J. Nam, S.-H. Kim, J. Lim (LGE)] [late] [miss]

JVET-M0518 CE4-related: Supplemental results on STMVP design of CE4.2.3.a and combination with methods of JVET-M0127 (CE4.1.2.a) and JVET-M0127 [D. Rusanovskyy, Y.-H. Chao, Y. Han, W.-J. Chien, M. Karczewicz (Qualcomm)]

JVET-M0820 Crosscheck of JVET- M0518 (CE4-related: Supplemental results on STMVP design of CE4.2.3.a and combination with methods of JVET-M0127 (CE4.1.2.a) and JVET-M0127) [T. Y. Zhou, T. Ikai (Sharp)] [late]

JVET-M0625 Crosscheck of JVET-M0518 (CE4-related: Supplemental results on STMVP design of CE4.2.3.a and combination with methods of JVET-M0127 (CE4.1.2.a) and JVET-M0127) [Y.-W. Chen (Kwai Inc.)] [late] [miss]

JVET-M0627 Non-CE4: Supplementary results of combined solution of JVET-M0255, JVET-M0267 and JVET-M0069 [H. Liu, K. Zhang, L. Zhang (Bytedance), E. Sasaki, T. Chujoh, T. Ikai (Sharp)] [late] [miss]

JVET-M0726 Cross-check of JVET-M0255 (AHG11: MMVD without Fractional Distances for SCC) [A. Karabutov (Huawei)] [late]

JVET-M0661 AhG-13: On Merge List Size [X. Li, X. Xu, S. Liu (Tencent)] [late]

JVET-M0713 CE4-related: simplification of CE4.2.2 [F. Le Léannec, A. Robert, T. Poirier (Technicolor)] [late]

JVET-M0786 Crosscheck of JVET-M0713 (CE4-related: simplification of CE4.2.2) [Y.-H. Chao (Qualcomm)] [late] [miss]

JVET-M0823 CE4-related: Encoder optimization of CE4.4.5 [J. Li, R.-L. Liao, C. S. Lim (Panasonic)] [late]

JVET-M0833 Crosscheck of JVET-M0823 (CE4-related: Encoder optimization of CE4.4.5) [C.-C. Chen, W.-J. Chien (Qualcomm)] [late] [miss]

JVET-M0854 CE4-related: Combination of CE4.4.4a and CE4.4.5b [T. Hashimoto, E. Sasaki, T. Ikai (Sharp), J. Li, R.-L. Liao, C. S. Lim (Panasonic)] [late]

JVET-M0861 Crosscheck of JVET-M0854: (CE4-related: Combination of CE4.4.4a and CE4.4.5b) [V. Seregin (Qualcomm)] [late] [miss]

## CE5 related – Arithmetic coding engine (5)

Contributions in this category were discussed XXday X Jan. XXXX–XXXX (chaired by XXX).

JVET-M0089 Non-CE5: CABAC skip mode for super low delay [K. Abe, T. Toma (Panasonic)]

JVET-M0344 Crosscheck of JVET-M0089 (Non-CE5: CABAC skip mode for super low delay) [R. Hashimoto (Renesas)]

JVET-M0196 CE5-related: Counter-based multi-CABAC for partial context models [Y. Piao, K. Choi (Samsung)]

Detailed presentation of this was not requested by the presenter, due to actions taken earlier at the meeting.

JVET-M0545 Crosscheck of JVET-M0196 (CE5-related: Counter-based multi-CABAC for partial context models) [J. Dong (Qualcomm)] [late]

JVET-M0389 CE5-related: Minor optimizations for increasing the throughput of CE5.1.5 and CE5.1.6 [H. Kirchhoffer, C. Bartnik, P. Haase, T. Hinz, S. Matlage, B. Stabernack, J. Stegemann, D. Marpe, H. Schwarz, T. Wiegand (HHI)]

Detailed presentation of this was not requested by the presenter, due to actions taken earlier at the meeting.

JVET-M0395 CE5-related: Alternative implementation of CABAC range sub-interval derivation for CE5.1.5, CE5.1.6 and CE5.1.7 [P. Haase, H. Kirchhoffer, S. Matlage, H. Schwarz, D. Marpe, T. Wiegand (HHI)]

Detailed presentation of this was not requested by the presenter, due to actions taken earlier at the meeting.

JVET-M0772 CE5-related: Clean up of the context model initialization process for CE5.1.5 and CE5.1.6 [J. Stegemann, H. Kirchhoffer, D. Marpe, H. Schwarz, T. Wiegand (HHI)] [late]

Detailed presentation of this was not requested by the presenter, due to actions taken earlier at the meeting.

JVET-M0788 Crosscheck of JVET-M0772 (CE5-related: Clean up of the context model initialization process for CE5.1.5 and CE5.1.6) [Y.-C. Lin (MediaTek)] [late] [miss]

## CE6 related – Transforms and transform signalling (25)

Contributions in this category were discussed XXday X Jan. XXXX–XXXX (chaired by XXX).

Contribution JVET-M0538 had been discussed in the CE report but was not part of the CE. See the CE report for some discussion of this.

JVET-M0046 CE6-related: A study of primary transforms [M. Zhou, Y. Hu (Broadcom)]

JVET-M0465 Cross-check of JVET-M0046: CE6-related: A study of primary transforms [S. Bandyopadhyay (InterDigital)]

JVET-M0072 Non-CE6: On transform skip for larger block [T. Tsukuba, M. Ikeda, T. Suzuki (Sony)]

JVET-M0748 Crosscheck of JVET-M0072, in aspect of 8x8 transform skip extension (Non-CE6: On transform skip for lager block) [S. Yoo, J.Lim (LGE)] [late]

JVET-M0637 Crosscheck of JVET-M0072 (Non-CE6: On transform skip for lager block) [T. Toma, K. Abe (Panasonic)] [late] [miss]

JVET-M0085 CE6-related: Fast algorithm for DST-4/DCT-4 as alternative transforms for MTS [T. Toma, K. Abe (Panasonic), M. Ikeda, T. Tsukuba (Sony)]

JVET-M0800 Cross-check report of JVET-M0085 on Fast algorithm for DST-4/DCT-4 as alternative transforms for MTS (CE6-related) [K. Naser (Technicolor)] [late]

JVET-M0201 CE6-related: Syntax clean-up related to MTS [K. Choi, M. Park, M. W. Park, W. Choi (Samsung)]

JVET-M0671 Crosscheck of JVET-M0201 (CE6-related: Syntax clean-up related to MTS) [X. Cao (Hikvision)] [late] [miss]

JVET-M0202 CE6-related: Simplification related to MTS with reduced modes [K. Choi, M. Park, M. W. Park, W. Choi (Samsung)]

JVET-M0672 Crosscheck of JVET-M0202 (CE6-related: Simplification related to MTS with reduced modes) [X. Cao (Hikvision)] [late] [miss]

JVET-M0269 Non-CE6: Extension of transform skip block size to 8x8 [S. Yoo, J. Choi, J. Heo, J. Choi, L. Li, J. Lim, S. Kim (LGE)]

JVET-M0709 Crosscheck of JVET-M0269 (Non-CE6: Extension of transform skip block size to 8x8) [T. Tsukuba (Sony)] [late] [miss]

JVET-M0275 Non-CE6: On transform skip conditions [S. Yoo, J. Choi, J. Heo, J. Choi, L. Li, J. Lim, S. Kim (LGE)]

JVET-M0846 Cross-check of JVET-M0275 (Non-CE6: On transform skip conditions) [P. Keydel (HHI)] [late] [miss]

JVET-M0280 CE6-related: Context selection for entropy coding the MTS flag [S.-T. Hsiang, S.-M. Lei (MediaTek)]

JVET-M0738 Crosscheck of JVET-M0280 (CE6-related: Context selection for entropy coding the MTS flag) [A. Tamse (Samsung)] [late] [miss]

JVET-M0297 CE6-related: 32 point MTS based on skipping high frequency coefficients [M. Koo, M. Salehifar, J. Lim, S. Kim (LGE)]

JVET-M0801 Cross-check report of JVET-M0297 on 32 point MTS based on skipping high frequency coefficients (CE6-related) [K. Naser (Technicolor)] [late]

JVET-M0304 CE6-related: 2-mode MTS with shape adaptive transform selection [J. Lainema (Nokia)]

JVET-M0340 CE6-related: Simplification on MTS for intra residual coding [X. Cao, F. Chen, L.Wang (Hikvision)]

JVET-M0654 Crosscheck of JVET-M0340 (CE6-related: Simplification on MTS for intra residual coding) [K. Choi (Samsung)] [late] [miss]

JVET-M0347 CE6-related: On MTS CU flag coding [X. Cao, F. Chen, L. Wang (Hikvision)]

JVET-M0655 Crosscheck of JVET-M0347 (CE6-related: Simplification on MTS CU flag coding) [K. Choi (Samsung)] [late] [miss]

JVET-M0354 CE6-related: MTS with Haar transform for Screen Contents Coding [K. Naser, F. Galpin, T. Poirier (Technicolor)]

JVET-M0869 Cross-check report of JVET-M0354 (CE6-related: MTS with Haar transform for Screen Contents Coding) [M. Koo (LGE)] [late]

JVET-M0361 Non-CE6: Mismatch between text specification and reference software on the signalling root CBF [J. Jung, D. Kim, G. Ko, J. Son, J. Kwak (WILUS)]

JVET-M0790 Cross-check of JVET-M0361 (Non-CE6: Mismatch between text specification and reference software on the signalling root CBF) [B. Lee (Chosun Univ.)] [late]

JVET-M0366 CE6 related: Transform Simplification [C. Hollman, D. Saffar, P. Wennersten, J. Ström (Ericsson)]

JVET-M0876 Crosscheck for JVET-M0366 (CE-6 related: Transform Simplification) [J. Rasch (HHI)] [late] [miss]

JVET-M0379 CE6-related: Further Simplification on top of CE6-2.2 [K. Naser, E. François, F. Le Léannec (Technicolor)]

JVET-M0638 Crosscheck of JVET-M0379 (CE6-related: Further Simplification on top of tests CE6-2.2) [T. Toma, K. Abe (Panasonic)] [late] [miss]

JVET-M0396 CE6-related: MTS kernel derivation for efficient memory usage [S. Shrestha, A. Kumar, B. Lee (Chosun Univ), Y. Lee, J. Park (Humax)] [late] [miss]

Initial upload rejected as a placeholder.

JVET-M0773 Crosscheck of JVET-M0396 (“CE6-related: MTS kernel derivation for efficient memory usage”) [J. Jung, D. Kim, G. Ko, J. Son, J. Kwak (WILUS)] [late] [miss]

JVET-M0397 CE6-related: DST-3 based transform kernels derivation [S. Shrestha, A. Kumar, B. Lee (Chosun Univ.), Y. Lee, J. Park (Humax)]

JVET-M0852 Crosscheck of JVET-M0397 (CE6-related: DST-3 based transform kernels derivation) [J. Kim, K. Ko, J. Jung] [late] [miss]

JVET-M0398 CE6-related Further simplification of CE6-1.5 [P. Philippe (bcom Orange)]

JVET-M0480 CE6-related: Implicit transform selection for Multi directional LM [S. Iwamura, S. Nemoto, A. Ichigaya (NHK)]

JVET-M0573 Crosscheck of JVET-M0480 (CE6-related: Implicit transform selection for Multi directional LM) [K. Kazui (Fujitsu)] [late] [miss]

JVET-M0482 CE6-related: Implicit transform selection for Multi-hypothesis inter-intra mode [S. Iwamura, S. Nemoto, A. Ichigaya (NHK)]

JVET-M0569 Crosscheck of JVET-M0482 (CE6-related: Implicit transform selection for Multi-hypothesis inter-intra mode) [T. Chujoh (Sharp)] [late]

JVET-M0501 CE6 related: Unification of Transform Skip mode and MTS [X. Zhao, X. Li, S. Liu (Tencent)]

JVET-M0656 Crosscheck of JVET-M0501 (CE6 related: Unification of Transform Skip mode and MTS) [K. Choi (Samsung)] [late] [miss]

JVET-M0524 CE3/6-related: Unification of RQT-like transform partitioning for intra and inter blocks [H. Egilmez, V. Seregin, A. Said, M. Karczewicz (Qualcomm)]

JVET-M0847 Cross-check of JVET-M0524 (CE3/6-related: Unification of RQT-like transform partitioning for intra and inter blocks) [P. Philippe (bcom Orange)] [late] [miss]

JVET-M0538 CE6: Efficient Implementations of MTS with Transform Adjustments (tests 1.4a-d) [A. Said, H.E. Egilmez, Y.-H. Chao, V. Seregin, M. Karczewicz (Qualcomm)] [late]

This had been included in the CE report but was a late contribution that was not in the CE plan.

JVET-M0539 CE6-related: Efficient computation of MTS transform combinations [A. Said, H.E. Egilmez, Y.-H. Chao, V. Seregin, M. Karczewicz (Qualcomm)] [late]

JVET-M0540 CE6-related: Software tool for computing transform throughput [A. Said, H.E. Egilmez, Y.H. Chao, V. Seregin, M. Karczewicz (Qualcomm)] [late]

## CE7 related – Quantization and coefficient coding (17)

Contributions in this category were discussed XXday X Jan. XXXX–XXXX (chaired by XXX).

JVET-M0107 CE7-related: reduced local neighbourhood usage for transform coefficients coding [F. Le Léannec, Y. Chen (Technicolor)]

JVET-M0781 Cross-Check of JVET-M0107 (CE7-related: reduced local neighborhood usage for transform coefficients coding) [M. Gao (Tencent)] [late] [miss]

JVET-M0198 CE7-related: Unified Rice parameter derivation for coefficient level coding [Y. Piao, K. Choi (Samsung)]

JVET-M0668 Crosscheck of JVET-M0198 (CE7-related: Unified Rice parameter derivation for coefficient level coding) [M. Coban (Qualcomm)] [late] [miss]

JVET-M0250 Non-CE7: Simplified CSBF coding for large block-size transforms [J. Choi, J. Heo, S. Yoo, J. Choi, L. Li, J. Lim, S. Kim (LGE)]

JVET-M0760 Crosscheck of JVET-M0250 (Non-CE7: Simplified CSBF coding for large block-size transforms) [Z.-Y. Lin (MediaTek)] [late] [miss]

JVET-M0251 Non-CE7: Last position coding for large block-size transforms [J. Choi, J. Heo, S. Yoo, J. Choi, L. Li, J. Lim, S. Kim (LGE)]

JVET-M0646 Crosscheck of JVET-M0251 (Non-CE7: Last position coding for large block-size transforms) [H. Schwarz (Fraunhofer HHI)] [late] [miss]

JVET-M0257 CE7-related: Coefficient scanning and last position coding for TUs of greater than 32 width or height [M. Coban, M. Karczewicz (Qualcomm)]

JVET-M0761 Crosscheck of JVET-M0257 (CE7-related: Coefficient scanning and last position coding for TUs of greater than 32 width or height) [Z.-Y. Lin (MediaTek)] [late]

JVET-M0278 Non-CE7: Residual rearrangement for transform skipped blocks [S. Yoo, J. Choi, J. Heo, J. Choi, L. Li, J. Lim, S. Kim (LGE)]

JVET-M0683 Crosscheck of JVET-M0278 (Non-CE7: Residual rearrangement for transform skipped blocks) [Y.-C. Lin (MediaTek)] [late] [miss]

JVET-M0279 Non-CE7: Sign coding for transform skip [S. Yoo, J. Choi, J. Heo, J. Choi, L. Li, J. Lim, S. Kim (LGE)]

JVET-M0649 Crosscheck of JVET-M0279 (Non-CE7: Sign coding for transform skip) [F. Henry, G. Clare (Orange)] [late] [miss]

JVET-M0305 CE7-related: Joint coding of chrominance residuals [J. Lainema (Nokia)]

JVET-M0688 Cross-check of JVET-M0305, "CE7-related: Joint coding of chrominance residuals" [C. Gisquet (Canon)] [late] [miss]

JVET-M0469 CE7-related: Unified Rice parameter derivation for coefficient coding [M. Karczewicz, M. Coban (Qualcomm)]

JVET-M0647 Crosscheck of JVET-M0469 (CE7-related: Unified Rice parameter derivation for coefficient coding) [H. Schwarz (Fraunhofer HHI)] [late] [miss]

JVET-M0470 CE7-related: Golomb-Rice/exponential Golomb coding for abs_remainder and dec_abs_level syntax elements [M. Coban, M. Karczewicz (Qualcomm)]

JVET-M0831 Cross-check of JVET-M0470 (CE7-related: Golomb-Rice/exponential Golomb coding for abs_remainder and dec_abs_level syntax elements) [K. Sharman, S. Keating (Sony)] [late]

JVET-M0489 CE7-related: Reduced context models for transform coefficients coding [M. Gao, X. Li, X. Zhao, S. Liu (Tencent)]

JVET-M0697 Cross-check of JVET-M0489 "CE7-related: Reduced context models for transform coefficients coding" [Y. Chen, F. Le Léannec (Technicolor)] [late] [miss]

JVET-M0491 CE7-related: Reduced the maximum number of context-coded bins for transform coefficient coding [M. Gao, X. Li, X. Zhao, S. Liu (Tencent)]

JVET-M0684 Crosscheck of JVET-M0491 (CE7-related: Reduced the maximum number of context-coded bins for transform coefficient coding) [Y.-C. Lin (MediaTek)] [late] [miss]

JVET-M0558 CE7-related: Template based Rice parameter derivation [M. Karczewicz (Qualcomm)] [late]

JVET-M0660 Crosscheck of JVET-M0558 [Y. Piao, K. Choi (Samsung)] [late]

## CE8 related – Screen content coding tools (29)

Contributions in this category were discussed Friday 11 Jan. 1620–2010 and Saturday 1815-2130 (Track B chaired by JRO).

See also M0418 on context modeling for CPR.

JVET-M0151 CE8-related: Virtual search area for current picture referencing (CPR) [L. Pham Van, T. Hsieh, W.-J. Chien, V. Seregin, H. Wang, M. Karczewicz (Qualcomm)]

This contribution proposes a method to extend the CPR search area by padding the area using the line buffer and the column to the left of the current CTU. Without any additional memory cost, the luma BD-rate changes for [CTC, Class F, class SCC 1080p] with the use of the proposed technique are reported as test 1:

CPR on and PLT off:

AI: (0.02%, -0.39%, -1.23%) over VTM3+CPR on, resp.

RA: (0.02%, -0.29%, -0.60%) over VTM3+CPR on, resp.

LB: (-0.03%, -0.20%, -0.35%) over VTM3+CPR on, resp.

CPR on and PLT on:

AI: (0.03%, -0.09%, -0.31%) over VTM3+CPR+PLT on, resp.

RA:( 0.03%, -0.12%, -0.35%) over VTM3+CPR+PLT on, resp.

LB:( 0.01%, -0.27%, -0.17%) over VTM3+CPR+PLT on, resp.

Test 2: In addition, this document also reports the results of the proposed padding scheme when 4 lines above and 4 columns to the left of the current CTU are included into the CPR search area.

Test 2 is not realistic, as only one line buffer is available

Test 1 is kind of extension of CE 8.1.3, which uses only 1 line without padding

It is pointed out by one expert that a similar method of padding could also be used generally when an MV in CPR would point outside the valid range

Padding generally is not of much complexity concern

Further study.

JVET-M0174 CE8-related: Removal of subblock-based chroma MC in CPR [C.-Y. Lai, T.-D. Chuang, Y.-L. Hsiao, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

This contribution proposes a modification on current picture referencing (CPR). In the current CPR, when dual tree coding is enabled, a chroma coding unit (CU) selecting CPR inter mode has to check all collocated luma subblock motion vectors (MVs), scale the collocated luma subblock MVs for chroma, and perform subblock-based chroma motion compensation (MC) if all the collocated luma subblock MVs are available and valid. To simplify the chroma CPR inter CUs in dual tree coding, it is proposed to use only the center collocated luma subblock MV to replace subblock-based chroma MC with CU-based chroma MC. Two default MVs can be checked when the center collocated luma subblock MV is unavailable or invalid. Simulation results are as follows, where the anchor enabled CPR.

For common test condition (CTC) sequences without class F
VTM3.0-AI: {Y/U/V-BD-rate = 0.00% / -0.04% / -0.06%, EncT=100%, DecT=99%}
VTM3.0-RA: {Y/U/V-BD-rate = 0.01%/ -0.07% / -0.04%, EncT=100%, DecT=99%}

For class F sequences
VTM3.0-AI: {Y/U/V-BD-rate = -0.04% / -0.25% / -0.14%, EncT=101%, DecT=100%}
VTM3.0-RA: {Y/U/V-BD-rate = -0.04% / -0.21% / -0.07%, EncT=100%, DecT=101%}

For TGM sequences
VTM3.0-AI: {Y/U/V-BD-rate = -0.27% / -0.47% / -0.55%, EncT=101%, DecT=101%}
VTM3.0-RA: {Y/U/V-BD-rate = -0.06% / -0.25% / -0.25%, EncT=100%, DecT=99%}

It is pointed out that one of the design goals of keeping luma and chroma aligned in CPR with dual tree was to avoid luma/chroma mismatches (which might be critical in screen content with sharp edges).Not clear if this would be critical in terms of complexity; compression gain is not an argument; more evidence would be necessary that the current design is overly complicated and would need additional checks at the encoder to guarantee the compliance of a vector. The decoder complexity is not a big issue as long as the decoder can trust an encoder to deliver a valid vector.

JVET-M0769 Crosscheck of JVET-M0174 (CE8-related: Removal of subblock-based chroma MC in CPR) [T.-S. Chang, Y.-C. Sun, J. Lou (Alibaba)] [late] [miss]

JVET-M0175 CE8-related: Clarification on interaction between CPR and other inter coding tools [C.-Y. Lai, T.-D. Chuang, Y.-L. Hsiao, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

In VTM3.0 encoder, current picture referencing (CPR) for each coding unit (CU) is never tested with combined inter intra prediction (CIIP), merge mode with motion vector difference (MMVD), and subblock modes including subblock-based temporal motion vector prediction (SbTMVP) merge mode, affine merge mode, and affine inter mode. When the reference picture list contains only the current picture and no other reference picture, mh_intra_flag (indicating CIIP on/off of the CU; it is suggested to rename it as ciip_flag), mmvd_flag (indicating MMVD on/off of the CU), merge_subblock_flag (equal to 1 indicating SbTMVP merge or affine merge is on; equal to 0 indicating SbTMVP merge and affine merge are off), and inter_affine_flag (indicating affine inter mode on/off) are always zero and still signalled. In addition, in both VTM3.0 decoder and VVC draft text JVET-L1001-v7.docx, decoding process is not defined. This contribution proposes two methods to deal with this issue. Method 1 is to add a normative encoder-only constraint in the VVC draft text, i.e., conformant encoders shall not enable any of the four flags when CPR is enabled at CU level. Method 2 is to skip signalling and parsing the above four flags when the reference picture list only contains the current picture at encoder and decoder, respectively. Method 1 does not cause any BD-rate change for VTM3.0. Method 2 reportedly causes minor BD-rate impact. It is claimed that Method 2 is a preferred solution than Method 1 in order to reduce the possibility of related illegal bitstreams generated by careless encoders.

Method1 was already included in V8 of VVC draft 3, therefore the problem is solved. The addititional benefit of method 2 is minimum (if at all present, as it might require additional low-level checks of decoder).

JVET-M0770 Crosscheck of JVET-M0175 (CE8-related: Clarification on interaction between CPR and other inter coding tools) [T.-S. Chang, Y.-C. Sun, J. Lou (Alibaba)] [late] [miss]

JVET-M0253 Non-CE8: Hash-based Motion Search [J. Xu, J. Li, K. Zhang, L. Zhang (Bytedance), R. Xiong (Peking University)]

This contribution presents a hash-based motion search scheme for the VVC test model. For 4x4 to 64x64 square blocks and 4x8, 8x4 blocks, a hash table is generated for each reference picture. A hash-based block matching is then performed before existing motion estimation for those block sizes. When a block finds its match, the following motion estimation can be skipped. For TGM sequences, experimental results report 9.6% and 18.87% coding gain for RA and LD_B, with encoding time reduced to 89% and 88%. When CPR is on, 7.81% and 14.91% coding gain is reported, with encoding time reduced to 91% and 89%.

Basically, same apporach as known from HEVC-SCC

The new algorithm automatically detects if hash-based search is applicable and switches to conventional search. This does not increase encoder run time (according to tables III and IV of v2 of the doc).

Decision(SW): Adopt JVET-M0253

JVET-M0704 Crosscheck for JVET-M0253: Non-CE8: Hash-based Motion Search [X. Xu (Tencent)] [late] [miss]

JVET-M0254 Non-CE8: Subblock Operation Removal for Chroma CPR [J. Xu, K. Zhang, L. Zhang, H. Liu, Y. Wang, P. Zhao, D. Hong (Bytedance)]

In the current design of CPR, when separate tree is applied, chroma blocks inherit collocated luma blocks’ block vectors, which requires sub-block by sub-block operation. This contribution proposes to remove such a subblock-wise operation and reports a 0.26% coding gain for class TGM420.

Similar to JVET-M0174 (see notes there).

JVET-M0785 Crosscheck of JVET-M0254 (Non-CE8: Subblock Operation Removal for Chroma CPR) [Y.-H. Chao (Qualcomm)] [late]

JVET-M0255 AHG11: MMVD without Fractional Distances for SCC [H. Liu, L. Zhang, K. Zhang, J. Xu, Y. Wang, P. Zhao, D. Hong (Bytedance)]

In the merge with motion vector difference (MMVD) mode, fractional distances including 1/4-pel and 1/2-pel are included in the distance table. However, fractional distances maybe inefficient for screen contents. To address this issue, this contribution proposes to disable fraction distances adaptively in MMVD mode. When fractional distances are disabled, distances in the default table are all multiplied by 4. Simulation results reportedly show 0.76% and 1.66% luma BD-rate saving on SCC sequences in Random Access configuration and Low Delay B configuration respectively. When CPR is off, 1.58% and 2.74% luma BD-rate saving on SCC sequences are achieved in Random Access configuration and Low Delay B configuration respectively.

More related to MMVD (not SCC). A similar approach was investigated in CE 4.4.5b (see remarks there), where switching to an integer table was proposed for UHD resolution.

Initially discussed Fri 11 afternoon in track B -> move to CE4 BoG.

JVET-M0326 CE8-related: Remove the redundancy of CPR-related syntax coding [S. Ye, F. Chen, L. Wang (Hikvision)]

This document proposes to remove the redundancy of CPR-related syntax coding. CPR-P frame is called for one frame where the current picture is the only reference picture. In fact, it is not allowed to use MMVD (Merge with MVD), Affine, merge triangle mode or Combined inter merge / intra prediction (CIIP) mode in the CPR-P frame. Therefore, this contribution proposes to remove those unused tools related syntax coding for CPR blocks in the CPR-P frame. By doing so, a small gain for luma BD rate of 0.01% was observed on AI configuration.

Similar to JVET-M0175. See notes there.

JVET-M0615 Crosscheck of JVET-M0326 (CE8-related: Remove the redundancy of CPR-related syntax coding) [Y.-C. Sun (Alibaba)] [late] [miss]

JVET-M0327 CE8-related: A new CPR syntax scheme [S. Ye, F. Chen, L. Wang (Hikvision)]

This document describes a new CPR syntax scheme.

In the VTM3.0, the current (partially) decoded picture is considered as a reference picture. This current picture is put in the last position of reference picture list 0. Therefore, for a frame using the current picture as the only reference picture, it is a P frame. To distinguish it with traditional P frame, we denoted it as a CPR-P frame. The bitstream syntax follows the same syntax structure for inter coding while the decoding process is unified with inter coding. The only outstanding difference is that the block vector (which is the motion vector pointing to the current picture) always uses integer-pel resolution.

The reported BD rate changes from the VTM3.0 with CPR enabled are as follows:

Scheme 1(Using initial context values of I frame for CPR-P frame):

- In AI, -0.05%/-0.05%/-0.02% for CTC/Class F/SCC 1080p classes, separately.

Scheme 2(Remain the frame type as I frame if the original frame type is I):

- In AI, -0.05%/-0.08%/-0.04% for CTC/Class F/SCC 1080p classes, separately.

Scheme 3(Take CPR as an intra mode in IBP frame):

- In AI, -0.09%/-0.17%/-0.39% for CTC/Class F/SCC 1080p classes, separately.

Presentation deck to be uploaded.

From the discussion, it is generally agreed that it would be much more desirable to handle CPR as a separate mode, and remove the signalling via reference picture list, and not use a P picture for that purpose. For VVC, there is no need to design it the same way as in HEVC (in HEVC, P picture was re-used, as CPR was defined in a later stage).

This would also resolve the problem of prohibiting combination with other MC tools

It is however desirable to re-use building blocks from MC.

JVET-M0327 does not come with a syntax proposal.

JVET-M0483 goes a similar direction. See further notes on that contribution.

JVET-M0616 Crosscheck of JVET-M0327 (CE8-related: A new CPR syntax scheme) [Y.-C. Sun (Alibaba)] [late] [miss]

JVET-M0333 Non-CE8: Coding on block vector difference [J. Nam, J. Lim, S. Kim (LGE)]

This contribution proposes a block vector difference coding for CPR mode. Considering that the absolute value of block vector difference (i.e., abs_mvd_minus2) for CPR mode is larger than that of conventional motion vector difference, higher order (K is equal to 3) of Exp-Golomb code is applied. From experiment results, following luma BD-rate changes are observed over VTM-3.0 with CPR:

- AI: -0.02% (CTC), -0.19% (Class F), -0.41% (SCC)

- RA: 0.00% (CTC), -0.27% (Class F), -0.10% (SCC)

- LD: 0.00% (CTC), -0.15% (Class F), -0.76% (SCC)

This would require a different binarization in the context of MV coding. Revisit after discussion in JVET plenary (about importance of consistency between CPR and MV coding) whether we would encourage such low level changes.

JVET-M0705 Crosscheck for JVET-M0333: Non-CE8: Coding on block vector difference [X. Xu (Tencent)] [late] [miss]

JVET-M0334 Non-CE8: Removal of redundant syntax between CPR and other inter coding tools [J. Nam, J. Lim, S. Kim (LGE)]

When CPR tool is enabled in SPS, I-Slice is treated as P-Slice so that all syntax elements of the picture are signaled as P-Slice instead of I-Slice. Considering that current CPR mode does not work with some inter coding tools, such as MMVD, affine, multi-hypothesis, and tri-angular prediction, this contribution proposes to remove redundant syntax from those inter coding tools. Specifically, when the current decoded picture is the only reference picture for the current slice, syntax elements for inter coding tools, which does not work with CPR mode, is not signaled. From experiment results, following luma BD-rate changes are observed over VTM-3.0 with CPR:

- AI: -0.01% (CTC), -0.01% (Class F), -0.07% (Class SCC)

- RA: 0.00% (CTC), -0.03% (Class F), -0.06% (Class SCC)

Same as JVET-M0175 method 2. See further notes there.

JVET-M0767 Crosscheck of JVET-M0334 (Non-CE8: Removal of redundant syntax between CPR and other inter coding tools) [X. Xu (Tencent)] [late] [miss]

JVET-M0335 Non-CE8: modification on SbTMVP process regarding with CPR [H. Jang, J. Nam, S. Kim, J. Lim (LGE)]

In VTM-3.0, SbTMVP and TMVP are not allowed when fetched motion vector in collocated picture uses collocated picture itself as reference picture (i.e., fetched colPb is CPR coded). In current VVC specification, however there may be a coding unit, which is coded by PRED_BI with L0 referring current picture and L1 previous decoded picture as reference. In this case, SbTMVP and TMVP, which is bi-predicted mode, is supposed to be operated based on motion vector of L1. Therefore, in this proposal, SbTMVP and TMVP are to be disallowed when the picture corresponding to L0 reference index of ColPb is collocated picture. The proposed method shows 0.00% and 0.00% BD-rate change in RA and LDB respectively.

It is the opinion of other experts that somehow the usage is restricted in VVC draft 3.

Anyway, if CPR would be defined as a separate mode, the issue would be resolved anyway.

JVET-M0706 Crosscheck for JVET-M0335 Non-CE8: modification on SbTMVP process regarding with CPR [X. Xu (Tencent)] [late] [miss]

JVET-M0341 Non-CE8: MMVD harmonization with CPR [H. Jang, J. Nam, S. Kim, J. Lim (LGE)]

This contribution presents a simplified MMVD considering CPR mode. When the current decoded picture is the only reference picture for the current slice, mmvd_merge_flag is not signalled and also 4 indices for mmvd_distance_idx are allowed. From experimental results, following BD-rate changes are observed over VTM-3.0 with CPR:

- AI: -0.03% (CTC), -0.13% (Class F), -0.3% (Class SCC)

- RA: -0.01% (CTC), -0.07% (Class F), -0.13% (Class SCC)

There is no conceptual reason to disable the combination of CPR and MMVD. There are other proposals (JVET-M0541). Furthermore, there is also relation with proposals which suggest similar action (alternative integer precision MVD table), e.g. JVET-M0255.

Further study.

JVET-M0768 Crosscheck of JVET-M0341 (Non-CE8: MMVD harmonization with CPR) [X. Xu (Tencent)] [late] [miss]

JVET-M0393 Non-CE8: chroma block vector initialization for CPR in dual tree [T. Poirier, F. Le Léannec, F. Galpin (Technicolor)]

This contribution describes a method to initialize chroma block vectors when dual tree is enabled and collocated luma block uses CPR partially. The method is implemented on top of VTM-3.0 with chroma subpel interpolation activated. The method is reported to result in PSNR-Y, U, V BD-rate variations of -0.01%, 0.00%, -0.01% with 100% and 99% encoding and decoding times in AI configuration, 0%, -0.01%, -0.03% with 100% and 100% encoding and decoding times in RA configuration.

Further results in the document show somewhat higher gains for class TGM.

There are two aspects: subpel chroma (4-tap) and determination of a chroma CPR vector in case of dual tree when no luma vector is available.

With dual tree enabled (which is CTC and also been known to be most beneficial in combination with CPR), it gives 0.3% for class TGM, 0.1% for class F. This is rather low, does not justify the additional complexity (e.g. determining and checking the derived vector, interpolation).

JVET-M0751 Crosscheck for JVET-M0393: Non-CE8: chroma block vector initialization for CPR in dual tree [X. Xu (Tencent)] [late] [miss]

JVET-M0402 Non-CE8: Proposed Cleanup for Current Picture Referencing [B. Heng, M. Zhou, W. Wan (Broadcom)]

The following document discusses some potential issues with the current picture referencing (CPR) tool. It provides a few different proposals for changes that would help reduce the likelihood that the tool is used in unintended or invalid ways. Specifically:

- Separate intra-picture CPR offsets from inter-picture motion vectors.

  - Signal CPR mode with explicit cpr_flag rather than reusing ref_idx.

  - Treat CPR as intra for existing motion vector prediction.

  - Explore separate CPR vector prediction that actually generates legal CPR vectors.

- Modified handling of illegal CPR vectors.

  - Add required clipping for any CPR vectors that point outside the current CTU.

  - Initialize not-yet-decoded pixels to some pre-defined value.

- Syntax modification for illegal dual-tree chroma CPR.

  - Prevent illegal usage of chroma CPR when underlying luma CU do not use CPR mode by modifying the syntax to make it impossible.

The contribution has different aspects.

- It supports an approach to define CPR as a separate mode. This also makes it simpler to define which other tools can be combined with it

- It suggests a decoder-side mechanism to preserve the valid range of MVs

- It suggests a decoder-side mechanism to ignore a CPR flag in chroma tree if any of the colocated luma blocks does not use CPR.

There is no concrete syntax/semantics proposal associated with the proposal.

Needs further consideration (probably in future meetings)

The current approach of imposing bitstream constraints is probably viable (and sufficient for the time being), but it might be safer to provide some “sanity check” at the decoder. Whether such a check needs to be normative or not needs still to be determined. Furthermore, we cannot describe the exact decoder behaviour upon an illegal bitstream, because it is impossible to check conformance.

JVET-M0409 Non-CE8: Mismatch between text specification and reference software on ATMVP candidate derivation when CPR is enabled [X. Xu, X. Li, S. Liu (Tencent), W.-J. Chien, M. Karczewicz (Qualcomm)]

This contribution provides a specification aligned CPR software implementation regarding ATMVP CU level MV offset derivation. In a previous VVC specification, when performing alternative temporal motion vector prediction (ATMVP), a motion offset at CU level is obtained from the motion vector of the first available spatial neighboring merge candidate that has the collocated picture as its reference picture. In BMS2.1, if a neighboring merge candidate is coded in CPR mode, it is considered as not available. This implementation is inherited into VTM3.0 as part of CPR mode. In the current VVC specification, only the first available spatial candidate in the merge list will be checked. If the candidate doesn’t satisfy the condition of ATMVP neighboring block requirement, zero motion vector will be used to derive the collocated block in the collocated picture. Therefore, there is a mismatch between the specification and software implementation. Simulation results report that when the provided specification aligned implementation is compared to VTM-3.0 anchor (CPR=1):

- BD rate changes are 0.00%/0.00%/ 0.00% in AI/RA/LB configurations, respectively, for CTC average.

- BD rate changes are 0.00%/-0.01%/ -0.10% in AI/RA/LB configurations, respectively, for Class F.

- BD rate changes are 0.00%/-0.03%/ -0.10% in AI/RA/LB configurations, respectively, for SCC TGM class.

There are no apparent runtime changes observed.

Decision (SW/BF): Adopt JVET-M0409 (align software with text)

JVET-M0732 Crosscheck of JVET-M0409 (Non-CE8: Mismatch between text specification and reference software on ATMVP candidate derivation when CPR is enabled) [T.-S. Chang, Y.-C. Sun, J. Lou (Alibaba)] [late]

JVET-M0410 Non-CE8: CPR flag signaling at slice level [X. Xu, X. Li, S. Liu (Tencent)]

This contribution proposes to control the usage of CPR at slice level, in addition to the current CPR SPS flag. That means, when CPR SPS flag is true, for each slice, an encoder can choose to turn on or off the usage of CPR. On top of this change, an encoder algorithm is provided in a way that when the hash hit rate of a picture is below a certain threshold, the slice level CPR is turned off. The intent of the suggested algorithm is that CPR mode is turned off for slices where CPR may not contribute much to the BD rate reduction while CPR mode is turned on for screen content materials. Simulation results report that:

- The BD rate/runtime changes for CTC average are -0.01%/-0.01%/0.13% and 103%/101%/103% for AI/RA/LB, separately, when compared to VTM-3.0+CPR=0 anchor;

- The BD rate/runtime changes for Class F average are 0.13%/0.07%/0.03% and 96%/100%/99% for AI/RA/LB, separately, when compared to VTM-3.0+CPR=1 anchor

- The BD rate/runtime changes for Class SCC 1080p average are 0.00%/0.00%/0.01% and 98%/101%/98% for AI/RA/LB, separately, when compared to VTM-3.0+CPR=1 anchor

No urgent need to do this currently – for CTC, the usage of the sequence level flag is good enough.

JVET-M0670 Crosscheck of JVET-M0410 (Non-CE8: CPR flag signaling at slice level) [J. Nam (LGE)] [late] [miss]

JVET-M0411 Non-CE8: Inter mode related flag signaling when current picture is the only reference picture [X. Xu, X. Li, S. Liu (Tencent)]

This contribution proposes not to signal the following inter coding related flags in condition that the current picture is the only reference picture for a slice. Specifically, sub-block merge flag, affine flag, mmvd flag and intra-inter flag are not signaled. Simulation results report that BD rate changes are negligible (within +-0.1%) when compared to VTM-3.0 anchor (CPR=1).

No need for further discussion – see notes under JVET-M0175.

JVET-M0635 Crosscheck of JVET-M0411 (Non-CE8: Inter mode related flag signaling when current picture is the only reference picture) [C.-Y. Lai (MediaTek)] [late] [miss]

JVET-M0417 CE8-related: Combination test of CE8.2.2 and CE8.2.5 [Y.-C. Sun, J. Lou (Alibaba)]

This document reports the results of the combination of CE8.2.2 (palette mode and intra mode combination) and CE8.2.5 (separated palette design for the joint CUs containing both luma and chroma CBs). The tests are performed on top of CE8.2.1 (JVET-M0050). Compared with CE8.2.1, when the method is tested with CPR turned off in the setting, the results of the combination show that:

- For 4:2:0 TGM sequences, the proposed method provides 2.5%, 2.8%, and 3.0% BD-rate gains for luma with 92%, 99% and 98% for encoding time, and 101%, 97% and 92% in decoding time under AI, RA, and LD configurations, respectively.

- For Class F sequences, the proposed method provides 0.3%, 0.5%, and 1.5% BD-rate gains for luma, with 100%, 99% and 99% for encoding time, and 101%, 98% and 97% in decoding time under AI, RA, and LD configurations, respectively.

When the method is tested with CPR turned on in the setting,

- For 4:2:0 TGM sequences, the proposed method provides 0.9%, 1.3%, and 2.4% BD-rate gains for luma with 95%, 99% and 100% for encoding time, and 100%, 98% and 100% in decoding time under AI, RA, and LD configurations, respectively.

- For Class F sequences, the proposed method provides 0.0%, 0.2%, and 0.9% BD-rate gains for luma, with 99%, 100% and 103% for encoding time, and 100%, 101% and 98% in decoding time under AI, RA, and LD configurations, respectively.

It’s worthwhile to note that, compared with the results of CE8.2.2 and CE8.2.5 individually, the results of the proposed combination show further synergy between CE8.2.2 and CE8.2.5.

The contribution shows that palette has some subjective benefit.

Goal is to further improve palette over the basis configuration. Further study.

JVET-M0556 Crosscheck of JVET-M0417 (CE8-related: Combination test of CE8.2.2 and CE8.2.5) [C.-Y. Lai (MediaTek)] [late] [miss]

JVET-M0791 Crosscheck of JVET-M0417 (CE8-related: Combination test of CE8.2.2 and CE8.2.5) [Y.-W. Chen (Kwai Inc.)] [late] [miss]

JVET-M0418 CE8-related: Context modeling on pred_mode_flag when current picture is the only reference picture (CPR) [Y.-C. Sun, J. Lou (Alibaba)]

This document modifies context modeling for pred_mode_flag signalling in VTM-3.0. Three methods are proposed to add new context models based on CU luma/chroma type and CU size. Compared with VTM-3.0 with CPR turned on, under AI configuration, the results show that:

- Method 1 provides 0.2% and 0.0% BD-rate gains for luma for 4:2:0 TGM and Class F sequences, respectively.

- Method 2 provides 0.4% and 0.1% BD-rate gains for luma for 4:2:0 TGM and Class F sequences, respectively.

- Method 3 provides 0.5% and 0.2% BD-rate gains for luma for 4:2:0 TGM and Class F sequences, respectively.

It is proposed to have different context models for 4x4 and other luma CUs, and for CUs in luma/chroma trees (method 3 makes both cases different and shows highest coding gain).

This would only apply for CPR in intra picture, inter picture case is not changed.

For non-screen content sequences and COR turned on, all methods show loss for non screen content classes, compared to the existing context modelling.

No action.

JVET-M0711 Crosscheck of JVET-M0418 (CE8-related: Context modeling on pred_mode_flag when current picture is the only reference picture (CPR)) [S. Ye (Hikvision)] [late] [miss]

JVET-M0419 CE8-related: Context modeling on palette mode flag [Y.-C. Sun, J. Lou (Alibaba)]

This document modifies context modeling for palette flag signaling of CE8.2.1. Compared with CE8.2.1, the results of the proposed method show that, under AI configurations, the proposed method provides 0.1% and 0.1% BD-rate gains for luma for 4:2:0 TGM when the method is tested with CPR turned off and on, respectively.

Relative low gain in terms of TGM

JVET-M0557 Crosscheck of JVET-M0419 (CE8-related: Context modeling on palette mode flag) [C.-Y. Lai (MediaTek)] [late] [miss]

JVET-M0449 CE8-related: BDPCM entropy coding with reduced number of context coded bins [M. Xu, X. Li, X. Xu, M. Gao, S. Liu (Tencent)]

Block DPCM (BDPCM) is tested in CE8.3 and shown noticeable gains. For each residual sample, up to 12 bins are coded with context. A restriction is proposed to improve the entropy coding throughput, where up to 2 context coded bins are allowed for each sample on average. It shows almost no loss on CTC for AI and RA. Compared to the original method, it shows 0.09% and 0.13% BD-rate loss on average for class F in AI and RA configurations, respectively, and 0.12% and 0.32% BD-rate loss on average for class SCC in AI and RA configurations, respectively.

Presentation deck to be uploaded.

Further study (see also notes under CE8.3)

BDPCM plus palette was used in the results of this contribution. Generally, different combinations of enabling/disabling the various screen content specific tools should be studied to identify which gain they still provide when not used standalone.

JVET-M0648 Crosscheck of JVET-M0449 (CE8-related: BDPCM entropy coding with reduced number of context coded bins) [F. Henry, G. Clare (Orange)] [late] [miss]

JVET-M0464 Non-CE8: Unified Transform Type Signalling and Residual Coding for Transform Skip [B. Bross, T. Nguyen, P. Keydel, H. Schwarz, D. Marpe, T. Wiegand (HHI)]

This contribution proposes two modifications related to transform type signalling and residual coding for transform skip. The first modification includes aligning the cases in which transform skip (TS) and multiple transform selection (MTS) apply. This limits TS to luma transform blocks as well as extends it to transform block sizes up to 32x32. The second modification constitutes of a modified transform coefficient level coding for the TS residual. Relative to the regular residual coding case, the residual coding for TS includes no signalling of the last x/y position, coded_sub_block_flag coded for every subblock, sig_coeff_flag context modelling with reduced template, a single context model for abs_level_gt1_flag and par_level_flag, context modeling for the sign flag, additional greater than 5, 7, 9 flags, modified Rice parameter derivation for the remainder binarization and a limit for the number of context coded bins per sample. The proposed joint signalling (first modification) provides on average (BD-rate Y, enc. time, dec.time):

- Natural:   0.01%, 103%, 101% (AI),  -0.02%,  98%, 100% (RA),  -0.07%, 102%, 102% (LB)

- Class F: -1.96%, 103%,    97% (AI), -2.14%,   98%, 100% (RA), -2.79%, 103%,   99% (LB)

- TGM:    -8.04%, 106%,    88% (AI), -8.74%, 106%,   96% (RA), -9.21%, 112%,   97% (LB)

Additionally changing the residual coding for transform skip (first modification and second modification) results on average in (BD-rate Y, enc. time, dec.time):

- Natural: -0.16%, 103%, 100% (AI),    -0.07%,  98%, 101% (RA),   -0.07%, 101%, 101% (LB)

- Class F: -7.16%, 104%,   94% (AI),   -5.76%,   98%, 100% (RA),   -5.85%, 102%,   99% (LB)

- TGM:  -21.03%, 108%,   82% (AI), -15.57%, 105%,   94% (RA), -14.01%, 111%,   95% (LB)

v2 provides updated full frame results, details about the combined TS/MTS encoder search with results for different encoder operation points as well as draft text for unified TS/MTS syntax.

v3 corrects wrongly pasted results to match with the cross-check provided in JVET-M0708.

The aspect on TS/MTS in the proposal signals TS before MTS (and therefore also enables it for blocks up to 32x32), but also changes the MTS binarization

Results of TS modification are on top of VTC, would be desirable to see benefit for SC classes when CPR is on

There is also an aspect of encoder speedup, by another approach of early skip for testing MTS cases.

The modified coding method employs max 8 context coded bins per sample – much higher than current VVC limit.

The way of restricting the number of context coded bins (once the budget is consumed, TS is not used any more) is not elegant.

Question is raised whether it has subjective quality impact?

It is dicussed whether the aspect of signalling TS before MTS (and by that way enabling TS for block sizes up to 32x32, but also restricting it to luma) would rather be a straightforward syntax cleanup, which could be adopted at this meeting (but without modifying the MTS index binarization, which is discussed in CE6).

Revisit: Wait for the results with CPR, and decide whether this provides benefit for SC.

JVET-M0650 Crosscheck of JVET-M0464 (Non-CE8: Unified Transform Type Signaling and Residual Coding for Transform Skip, test TSRC-CCB8) [F. Henry, G. Clare (Orange)] [late] [miss]

JVET-M0708 Crosscheck of JVET-M0464 (Non-CE8: Unified Transform Type Signaling and Residual Coding for Transform Skip, test TS32Y/uniMTS) [T. Tsukuba (Sony)] [late] [miss]

JVET-M0834 Crosscheck of JVET-M0464 (Non-CE8: Unified Transform Type Signalling and Residual Coding for Transform Skip, test TSRC-CCB3 and CCB2) [S. Yoo, J. Lim (LGE)] [late] [miss]

JVET-M0483 CE8-related: CPR mode signaling and interaction with inter coding tools [W.-J. Chien, V. Seregin, M. Karczewicz (Qualcomm)]

This contribution proposes two schemes to align the interaction between CPR mode with all other inter coding tools. The method is implemented on top of VTM-3.0 with CPR enabled. The first scheme removes   CPR dependency in motion derivation process. It results in negligible BD-rate difference for all test configurations on regular or screen content materials. The second scheme uses CPR mode as a third mode other than intra or inter modes. Motion predictor for CPR mode and inter mode would be derived mutual-exclusively from each other. The simulations show 1.1% in RA configuration and 1.5% in LDB configuration in screen content coding materials.

(see also discussion under JVET-M0327)

In track B, the general consensus is that it is a right direction to signal CPR as a separate mode rather than using a special reference picture index. As this is a fundamental conceptual decision, this should be discussed and decided in the JVET plenary. Revisit.

JVET-M0483 comes with spec text (“method 3”), which would probably need more careful inspection and modifications (also alignment with v9 of VVC draft 3).

JVET-M0860 Cross check of JVET-M0483 [K. Misra (Sharp Labs of America)] [late]

JVET-M0541 Non-CE8: Combination of MMVD and CPR mode [Y. Li, Z. Chen (Wuhan Univ.), X. Xu, S. Liu (Tencent)] [late]

This document describes unification methods of CPR merge mode with MMVD expansions. In the proposed methods, merge candidates with default merge type (MRG_TYPE_DEFAULT_N) and CPR merge type (MRG_TYPE_CPR) may both exist in the MMVD candidate list. Thus CPR will not be excluded when MMVD is used. It is reported that the BD rate can be improved by enabling CPR with MMVD expansions.

Several methods are proposed, such as clipping the fractional pel offset into integer values. The average BD rate gain is around 0.3% for TGM, and 0.1% for class F in AI. Similar gain was reported in JVET-M0341.

Further study.

JVET-M0542 Non-CE8: Combination of Multi Hypothesis Intra and CPR mode [Y. Li, Z. Chen (Wuhan Univ.), X. Xu, S. Liu (Tencent)] [late]

This document proposes to unify the CPR mode operation with multi hypothesis intra mode. In the proposed method, a multi-hypothesis can be a combination of intra mode and CPR merge mode. Thus CPR mode is unified with inter mode coding when multi hypothesis intra mode tool is used. It is reported that the BD rate changes by enabling CPR with multi-hypothesis intra mode are negligible.

The contribution shows that it would not be a problem to release the current bitstream constraint of disallowing combination of CPR and CIIP. On the other hand, results show that it obviously does not give benefit in terms of compression performance.

May be obsolete if CPR would be a separate mode.

JVET-M0696 Crosscheck of JVET-M0542 (Non-CE8: Combination of Multi Hypothesis Intra and CPR mode) [T. Poirier (Technicolor)] [late] [miss]

JVET-M0544 Non-CE8: CPR with chroma 4x4 sub-block size when dual-tree is on [X. Xu, X. Li, S. Liu (Tencent)] [late]

This contribution proposes to unify the handling of chroma sub-block size for CPR mode to that of inter sub-block mode (4x4), when dual-tree is enabled for CPR. For each group of four 2x2 chroma sub-blocks in a chroma CU under dual-tree condition, the collocated luma block vector of the top-left 2x2 chroma sub-block is used to derive the chroma block vector for all four sub-blocks. Simulation results report that:

- The BD rate changes for CTC average are 0.00%/0.00%/0.00% when compared to VTM-3.0+CPR=1 anchor

- The BD rate changes for Class F average are 0.06%/0.11%/0.07% when compared to VTM-3.0+CPR=1 anchor

- The BD rate changes for Class SCC 1080p average are 0.57%/0.25%/-0.08% when compared to VTM-3.0+CPR=1 anchor

Does not have advantage for CPR, as due to the integer precision of displacement vectors the 2x2 subblocks are no problem. It causes loss, and there may even be the possibility of luma/chroma mismatch e.g. at edges.

JVET-M0669 Crosscheck of JVET-M0544 (Non-CE8: CPR with chroma 4x4 sub-block size when dual-tree is on) [J. Nam (LGE)]

JVET-M0765 CE8-related: Unified Screen Content and Multiview Video Coding - Experimental results [J. Samelak, M. Domański] [late]

The HEVC standard provides separate screen-content and multiview profiles. Application of the multiview coding technology provides some gains over simulcast video coding of multiple views. Unfortunately, the multiview video coding technology was adopted in the limited number of applications only. On the other hand, the frame compatible approach to compress stereoscopic video was quite common recently. Moreover, the technology of Screen Content Coding seems to be successful in real-word applications. In this paper, we provide a modification HEVC Screen Content Coding that exhibits roughly the same coding efficiency as HEVC SCC and MV-HEVC for graphics and frame-compatible multiview content, respectively.

Current development of CPR (as needed for screen content) would not fulfil the requirements for FC multiview, as it has only integer precision and does not access the entire picture. For future stereo/multiview, frame compatible may no longer the main stream, as it penalizes spatial resolution, and it may be better to keep the pictures separate.

JVET-M0822 Non-CE8: Encoder optimization for palette mode [H. Wang, Y.-H. Chao, V. Seregin, M. Karczewicz (Qualcomm)] [late]

This contribution proposes an encoder optimization for palette mode. The results show more than 20% encoding time reduction for AI for screen content sequences. When tested with CPR enabled, the partial results show that the encoding time is reduced from 157% to 115% with only 0.14% (Y) BD rate loss for TGM420 sequences, and the encoding time is reduced from 174% to 151% with 0.11% (Y) BD rate loss for class F.

Early termination criteria are used to avoid full checking of all intra modes if palette mode shows good performance (i.e. testing palette before intra modes but after CPR). Further, termination criteria for not testing palette are applied for areas that are too small (which also avoids checks in partitioning).

In the setup of the CE, this might be considered as becoming part of the CE software (for the palette basis configuration 8.2.1).

JVET-M0835 Crosscheck of JVET-M0822 (Non-CE8: Encoder optimization for palette mode) [Y.-C. Sun (Alibaba)] [late]

JVET-M0878 CE8-Related: A combination of Test CE8.1.3 and Test CE8.1.2d [L. Pham Van, T. Hsieh, V. Seregin, W.-J. Chien, M. Karczewicz (Qualcomm)] [late]

## CE9 related – Decoder-side motion vector derivation (12)

Contributions in this category were discussed XXday X Jan. XXXX–XXXX (chaired by XXX).

JVET-M0063 Non-CE9: An improvement of BDOF [T. Chujoh, T. Ikai (Sharp)]

JVET-M0652 Crosscheck of JVET-M0063 (Non-CE9: An improvement of BDOF) [K. Choi (Samsung)] [late] [miss]

JVET-M0073 Non-CE9: On early termination for BIO [K. Kondo, M. Ikeda, T. Suzuki (Sony)]

JVET-M0813 Cross-check of JVET-M0073: Non-CE9: On early termination for BDOF [S. Bandyopadhyay (InterDigital)] [late] [miss]

JVET-M0077 CE9-related: Relaxation of block size restriction for DMVR [K. Unno, K. Kawamura, S. Naito (KDDI)]

JVET-M0567 Crosscheck of JVET-M0077 (CE9-related: Relaxation of block size restriction for DMVR) [T. Chujoh (Sharp)] [late]

JVET-M0078 CE9-related: Combination of JVET-M0077 and CE9.2.5 [K. Unno, K. Kawamura, S. Naito (KDDI), T. Chujoh, T. Ikai (Sharp)]

JVET-M0636 Cross-check of JVET-M0078 (CE9-related: Combination of JVET-M0077 and CE9.2.5) [Y. Kato, K. Abe, T. Toma (Panasonic)] [late] [miss]

JVET-M0148 Non-CE9: Simplifications to DMVR search pattern and interpolation for refinement [S. Sethuraman (Ittiam)]

JVET-M0506 Crosscheck of JVET-M0148 (Non-CE9: Simplifications to DMVR search pattern and interpolation for refinement) [X. Chen (HiSilicon)] [late] [miss]

JVET-M0223 Non-CE9: Co-existence analysis for DMVR with BDOF [S. Sethuraman (Ittiam)]

JVET-M0797 Crosscheck of JVET-M0223 (Non-CE9: Co-existence Analysis for DMVR with BDOF) [C.-C. Chen, W.-J. Chien (Qualcomm)] [late] [miss]

JVET-M0241 CE9-related: A simple gradient calculation at the CU boundaries for BDOF [H. Lee, J. Kang, S.-C. Lim, J. Lee, H. Y. Kim (ETRI)]

JVET-M0568 Crosscheck of JVET-M0241 (CE9-related: A simple gradient calculation at the CU boundaries for BDOF) [T. Chujoh (Sharp)] [late]

JVET-M0249 Non-CE9: Modifications on Bi-Directional Optical Flow [H. Liu, L. Zhang, K. Zhang, J. Xu, Y. Wang, P. Zhao, D. Hong (Bytedance)]

JVET-M0731 Crosscheck of JVET-M0249 (Non-CE9: Modifications on Bi-Directional Optical Flow) [T.-H. Li, Y.-C. Yang (Foxconn)] [late] [miss]

JVET-M0284 CE9-related: BDOF Modifications to Enable 64x64 VPDU [H. Chen, X. Ma, S. Esenlik, H. Yang, J. Chen (Huawei)]

JVET-M0486 Cross-check of JVET-M0284: CE9-related: BDOF Modifications to Enable 64x64 VPDU [S Bandyopadhyay (InterDigital)] [late] [miss]

JVET-M0316 CE9-related: simplification of BDOF [J. Li, R.-L. Liao, C. S. Lim (Panasonic)]

JVET-M0701 Cross-check of JVET- M0316 (CE9-related: simplification of BDOF) [J. Zhao (LGE)] [late]

JVET-M0516 Non-CE9.2.1.e: Non-local-mean-based MRSAD and Row-subsampled Search Pattern for DMVR [C.-C. Chen, W.-J. Chien, M. Karczewicz (Qualcomm)]

JVET-M0577 Crosscheck of JVET-M0516 (Non-CE9.2.1.e: Non-local-mean-based MRSAD and Row-subsampled Search Pattern for DMVR) [S. Esenlik (Huawei)] [late] [miss]

JVET-M0517 Non-CE9: Methods for BDOF complexity reduction [S. Sethuraman (Ittiam)]

JVET-M0796 Crosscheck of JVET-M0517 (Non-CE9: Methods for BDOF Complexity Reduction) [C.-C. Chen, W.-J. Chien (Qualcomm)] [late] [miss]

JVET-M0890 CE9-related: BDOF buffer reduction and enabling VPDU based application [H. Chen, X. Ma, S. Esenlik, H. Yang, J. Chen (Huawei), K. Kondo, M. Ikeda, T. Suzuki] [late] [miss]

TBP?

## CE10 related – Combined and multi-hypothesis prediction (44)

Contributions in this category were discussed XXday X Jan. XXXX–XXXX (chaired by XXX).

JVET-M0082 CE10-related: Simplification of Multi hypothesis intra prediction [Y. Kato, K. Abe, T. Toma (Panasonic)]

JVET-M0808 Cross-check result of JVET-M0082 (CE10-related: Simplification of Multi hypothesis intra prediction) [K. Kawamura, S. Naito (KDDI)] [late]

JVET-M0088 CE10-related: LIC restriction for pipeline structure [K. Abe, T. Toma (Panasonic)]

JVET-M0371 Crosscheck of JVET-M0088 (CE10-related: LIC restriction for pipeline structure) [P. Bordes (Technicolor)] [late]

JVET-M0096 CE10.1-related: Inter-intra prediction [L. Pham Van, G. Van der Auwera, A. K. Ramasubramonian, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-M0681 Crosscheck of JVET-M0096 (CE10.1-related: Inter-intra prediction) [F. Galpin, T. Poirier (Technicolor)] [late] [miss]

JVET-M0115 CE10-related: pipeline reduction for LIC and GBI [P. Bordes, F. Galpin, T. Poirier (Technicolor)]

JVET-M0570 Crosscheck of JVET-M0115 (CE10-related: pipeline reduction for LIC and GBI) [T. Chujoh (Sharp)] [late]

JVET-M0118 CE10-related: A fix for merge triangle flag signaling [R. Yu (Ericsson)]

JVET-M0631 Crosscheck of JVET-M0118 (CE10-related: A fix for merge triangle flag signalling) [M.-S. Chiang (MediaTek)] [late] [miss]

JVET-M0182 CE10-related: Simplification of local illumination compensation [C.-M. Tsai, C.-C. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-M0596 Crosscheck of JVET-M0182 (CE10-related: Simplification of local illumination compensation) [Y. Yasugi, T. Ikai (Sharp)] [late] [miss]

JVET-M0183 CE10-related: Simplification of MPM generation for CIIP [M.-S. Chiang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-M0693 Crosscheck of JVET-M0183 (CE10-related: Simplification of MPM generation for CIIP) [Z. Zhang, K. Andersson, R. Sjöberg (Ericsson)] [late] [miss]

JVET-M0695 Crosscheck of JVET-M0183 (CE10-related: Simplification of MPM generation for CIIP) [K. Andersson (Ericsson)] [late] [miss]

JVET-M0184 CE10-related: Simplification of triangle merging candidate list derivation [T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-M0729 Crosscheck of JVET-M0184 (CE10-related: Simplification of triangle merging candidate list derivation) [M. Winken (HHI)] [late] [miss]

JVET-M0185 CE10-related: Syntax redundancy removal in triangle prediction [M.-S. Chiang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-M0582 Cross-check of JVET-M0185 (CE10-related: Syntax redundancy removal in triangle prediction) [K. Andersson, R. Yu (Ericsson)] [late] [miss]

JVET-M0190 CE10-related: Redundant syntax reduction for triangle prediction [Y. Ahn, D. Sim (Digital Insights)]

JVET-M0749 Cross-check of JVET-M0190 (CE10-related: Redundant syntax reduction for triangle prediction) [S.-C. Lim, H. Lee, J. Lee, J. Kang (ETRI)] [late] [miss]

JVET-M0194 CE10-related: Triangle Prediction Mode Harmonization [A. Tamse, M. W. Park, S. Jeong, M. Park, K. Choi (Samsung)]

JVET-M0207 CE10-related: Joint optimizations of Triangular prediction unit mode and Multi-Hypothesis prediction mode [S. Jeong, M. W. Park, A. Tamse, M. Park, K. Choi (Samsung)]

JVET-M0775 Crosscheck of JVET-M0207: CE10-related: Joint optimizations of Triangular prediction unit mode and Multi-Hypothesis prediction mode [L. Pham Van, W.-J. Chien, M. Karczewicz (Qualcomm)] [late]

JVET-M0216 CE10-related: syntax clean-up on triangle prediction [L. Li, J. Nam, N. Park, H. Jang, J. Lim, S. Kim (LGE)]

JVET-M0849 Crosscheck of JVET-M0216 (CE10-related: syntax clean-up on triangle prediction) [C. Rosewarne (Canon)] [late] [miss]

JVET-M0224 CE10-related: Local illumination compensation simplifications [S. Bandyopadhyay, X. Xiu, Y. He (InterDigital)]

JVET-M0714 Crosscheck of JVET-M0224 (CE10-related: Local illumination compensation simplifications) [P. Bordes (Technicolor)] [late] [miss]

JVET-M0232 CE10-related: Simplification of CIIP Intra mode coding [Y.-W. Chen, X. Wang (Kwai Inc.)]

JVET-M0552 Crosscheck of JVET-M0232 (Non-CE10: Simplification of CIIP Intra mode coding) [M.-S. Chiang (MediaTek)] [late] [miss]

JVET-M0233 CE10-related: Triangle prediction merge list construction [X. Wang, Y.-W. Chen (Kwai Inc.)]

JVET-M0674 Crosscheck of JVET-M0233 (Non-CE10: Triangle prediction merge list construction) [S. Esenlik (Huawei)] [late] [miss]

JVET-M0234 CE10-related: Triangle prediction merge index coding [X. Wang, Y.-W. Chen (Kwai Inc.)]

JVET-M0675 Crosscheck of JVET-M0234 (Non-CE10: Triangle prediction merge index coding) [S. Esenlik (Huawei)] [late] [miss]

JVET-M0271 CE10-related: Merge list construction process for triangular prediction mode [L. Zhang, K. Zhang, H. Liu, J. Xu, Y. Wang, P. Zhao, D. Hong (Bytedance)]

JVET-M0588 Crosscheck of JVET-M0271 (CE10-related: Merge list construction process for triangular prediction mode) [H. Dou, Z. Deng, L. Xu (Intel)] [late] [miss]

JVET-M0276 CE10-related: MPM list alignment between CIIP and intra mode [J. Li, S. Wang, W. Gao (Peking University), L. Zhang, K. Zhang, H. Liu, J. Xu (Bytedance)]

JVET-M0755 Crosscheck of JVET-M0276 (CE10-related: MPM list alignment between CIIP and intra mode) [Y.-W. Chen (Kwai Inc.)] [late] [miss]

JVET-M0283 CE10-related: Reduction of motion predictor pruning in Triangle Merge mode [A. Robert, F. Le Léannec, T. Poirier, F. Galpin (Technicolor)]

JVET-M0868 Crosscheck of JVET-M0283 (CE10-related: Reduction of motion predictor pruning in Triangle Merge mode) [A. Filippov, V. Rufitskiy (Huawei)] [late]

JVET-M0296 CE10-related: Simplification on combined inter-intra mode prediction [B. Wang, S. Esenlik, A.M. Kotra, H. Gao, J. Chen (Huawei)]

JVET-M0734 Crosscheck of JVET-M0296 (CE10-related: Simplification on combined inter-intra mode prediction) [L. Zhao (Tencent)] [late] [miss]

JVET-M0317 CE10-related: Simplification of triangular prediction unit mode [R.-L. Liao, J. Li, C. S. Lim (Panasonic)]

On deblock aspect: See notes at the end of CE11 related section.

JVET-M0710 Crosscheck of JVET-M0317 (CE10-related: Simplification of triangular prediction unit mode) [F. Chen (Hikvision)] [late] [miss]

JVET-M0328 CE10-related: Simplified triangle prediction unit mode [F. Chen, L. Wang (Hikvision)]

JVET-M0666 Crosscheck of JVET-M0328 (CE10-related: Simplified triangle prediction unit mode) [R.-L. Liao, C. S. Lim (Panasonic)] [late] [miss]

JVET-M0329 CE10-related: Modified enabling condition for triangle prediction unit mode [F. Chen, L. Wang (Hikvision)]

JVET-M0692 Crosscheck of JVET-M0329 (CE10-related: Modified enabling condition for triangle prediction unit mode) [S.H. Wang (Peking University), X. Zheng, S.S. Wang, S.W. Ma] [late] [miss]

JVET-M0331 CE10-related: A simplification method for Multi-intra-inter mode scheme [L. Xu, F. Chen, L. Wang (Hikvision)]

JVET-M0533 Crosscheck of JVET-M0331 (CE10-related: A simplification of inter prediction information derivation for Multi-intra-inter mode scheme) [X. Chen (HiSilicon)] [late] [miss]

JVET-M0349 CE10-related: Simplification of triangle prediction merging candidate list derivation [X. W. Meng (Peking University), X. Zheng (DJI), S.S. Wang, S.W. Ma (Peking University)]

JVET-M0742 Crosscheck of JVET-M0390 (CE-10: related multi-hypothesis with uni-directional inter prediction restriction) [H. Wang (Qualcomm)] [late] [miss]

JVET-M0352 CE10-related: Simplification of triangular partitions [D. Park, Y. Yoon, J.-G. Kim (KAU), J. Lee, J. Kang (ETRI)]

JVET-M0722 Crosscheck of JVET-M0352 (CE10-related: Simplification of triangular partitions) [G. Laroche (Canon)] [late] [miss]

JVET-M0357 CE10-related: Reduction of the worst-case memory bandwidth and operation number of OBMC [Y. Kidani, K. Kawamura, K. Unno, S. Naito (KDDI)]

JVET-M0828 Crosscheck of JVET-M0357 (CE10-related: Reduction of the worst-case memory bandwidth and operation number of OBMC) [Y.-C. Lin (MediaTek)] [late] [miss]

JVET-M0390 CE10 related: Multi-hypothesis with uni-directional inter prediction restriction [T. Poirier, E. François, K. Naser (Technicolor)]

JVET-M0742 Crosscheck of JVET-M0390 (CE-10: related multi-hypothesis with uni-directional inter prediction restriction) [H. Wang (Qualcomm)] [late] [miss]

JVET-M0399 CE10-related: Modifications of Triangular PU Mode [H. Wang, W.-J. Chien, V. Seregin, Y.-H. Chao, H. Huang, M. Karczewicz (Qualcomm)]

JVET-M0694 Crosscheck of JVET-M0399 (CE10-related: Modifications of Triangular PU Mode) [T. Poirier (Technicolor)] [late] [miss]

JVET-M0424 CE10-related: On enhancement of 4-tap interpolation filters [M. Sychev, J. Chen (Huawei)] [late] [miss]

Initial upload rejected as a placeholder.

JVET-M0752 Crosscheck of JVET-M0424 [A. Henkel (HHI)] [late] [miss]

JVET-M0438 CE10-related: Size constraint on Triangular Prediction [J. Zhao, S. Kim (LGE)]

JVET-M0663 Crosscheck of JVET-M0438 (CE10-related: Size constraint on Triangular Prediction) [C.-W. Kuo, R.-L. Liao, C. S. Lim (Panasonic)] [late] [miss]

JVET-M0450 CE10-related: LIC inheritance restrictions and interaction with GBI [M. Xu, X. Li, X. Xu, S. Liu (Tencent)]

JVET-M0454 CE10-related: Multi-Hypothesis Intra with Weighted Combination [A. Seixas Dias, G. Kulupana, S. Blasi (BBC)]

JVET-M0610 Crosscheck of JVET-M0454 (CE10-related: Multi-Hypothesis Intra with Weighted Combination) [B. Wang (Huawei)] [late] [miss]

JVET-M0492 CE10-related: Simplified multi-hypothesis intra-inter mode [L. Zhao, X. Zhao, X. Li, S. Liu (Tencent)]

JVET-M0589 Crosscheck of JVET-M0492 (CE10-related: Simplified multi-hypothesis intra-inter mode) [H. Dou, Z. Deng, L. Xu (Intel)] [late] [miss]

JVET-M0500 CE10-related: Unidirectional illumination compensation [V. Seregin, W.-J. Chien, T. Hsieh, N. Hu, M. Karczewicz (Qualcomm)]

JVET-M0633 Crosscheck of JVET-M0500 (CE10-realated: Unidirectional illumination compensation) [K. Abe (Panasonic)] [late] [miss]

JVET-M0525 CE10-related: Simplification of intra prediction in CIIP [P.-H. Lin, Y.-J. Chang (Foxconn)]

JVET-M0578 Crosscheck of JVET-M0525 (CE10-related: Simplification of intra prediction in CIIP) [M.-S. Chiang (MediaTek)] [late] [miss]

JVET-M0581 CE10-related: Bi-directional motion vector storage for triangular prediction [M. Bläser, J. Sauer (RWTH Aachen University)] [late]

JVET-M0829 Crosscheck of JVET-M0581 (CE10-related: Bi-directional motion vector storage for triangular prediction) [H. Gao (Huawei)] [late] [miss]

JVET-M0736 CE10-related: Triangular prediction with MMVD [M. Bläser, J. Sauer (RWTH Aachen University)] [late]

JVET-M0830 Crosscheck of JVET-M0736 (CE10-related: Triangular prediction with MMVD) [H. Gao (Huawei)] [late] [miss]

JVET-M0792 CE10-related: Combined test of multi-hypothesis inter prediction and OBMC [Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, C.-C. Chen, Y.-C. Lin, Y.-W. Huang, S.-M. Lei (MediaTek), X. Xiu, Y. He (InterDigital), M. Winken, H. Schwarz, D. Marpe, T. Wiegand (HHI)] [late] [miss]

JVET-M0884 Crosscheck of JVET-M0792 (CE10-related: Combined test of multi-hypothesis inter prediction and OBMC) [Z. Deng (Intel)] [late]

JVET-M0838 CE 10 related: M0390-M0096 combination [F. Galpin, T. Poirier, E. François (Technicolor)] [late]

JVET-M0848 CE10 related Document: Speedups for Uniform Directional Diffusion Filters For Video Coding (JVET-M0042) [J. Rasch, A. Henkel, J. Pfaff, H. Schwarz, D. Marpe, T. Wiegand (HHI)] [late] [miss]

JVET-M0851 CE10-related: Using inter merge list derivation for triangle mode [H. Wang, W.-J. Chien, V. Seregin, Y.-H. Chao, H. Huang, M. Karczewicz (Qualcomm), X. Wang, Y.-W. Chen (Kwai), T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek), A. Tamse, M. W. Park, S. Jeong, M. Park, K. Choi (Samsung)] [late]

JVET-M0883 CE10-related: Using regular merge index signaling for triangle mode [H. Wang, W.-J. Chien, V. Seregin, Y.-H. Chao, H. Huang, M. Karczewicz (Qualcomm), X. Wang, Y.-W. Chen (Kwai), T. Solovyev, S. Esenlik, S. Ikonin, J. Chen (Huawei), M. Xu, X. Li, S. Liu (Tencent)] [late]

JVET-M0886 Crosscheck of JVET-M0883 (CE10-related: Using regular merge index signaling for triangle mode) [H. Liu (Bytedance)] [late]

## CE11 related – Deblocking (4)

Contributions in this category were discussed Monday 14 Jan. 1715–1815 (Track B chaired by JRO).

JVET-M0103 Deblocking for multi-hypothesis intra inter prediction [K. Andersson, J. Enhorn, R. Yu, Z. Zhang, R. Sjöberg (Ericsson)]

Multi-hypothesis intra inter prediction can produce boundaries on both CU and sub-block level due to uniform scaling of intra and inter prediction samples. Currently no deblocking is applied for such boundaries in VVC when transform, motion or intra deblocking criterions are false. This contribution proposes to set the boundary strength to 1 for boundaries of multi-hypothesis intra inter prediction on an 8x8 grid such that the boundaries will be deblocked as other inter modes.

The proposed fix is claimed to produce better subjective quality especially noticeable at low bitrates. Reported Y/U/V BD-rates are -0.02%/-0.02%/0.03% for random access, 0.00%/-0.01%/-0.09% for low delay B and -0.05%/-0.07%/-0.34% for low delay P over VTM-3.0.

Currently, no deblocking is performed when there is a CIIP block and next to it an inter block with same motion vector. This should be corrected.

JVET-M0294 is targeting the same issue and was also discussed in this context.

JVET-M0549 Crosscheck of JVET-M0103 (Deblocking for multi-hypothesis intra inter prediction) [C.-M. Tsai (MediaTek)] [late] [miss]

JVET-M0294 CE10-related: Modification for blocks applied with Combined Inter-Intra prediction [B. Wang, A.M. Kotra, S. Esenlik, H. Gao, J. Chen (Huawei)]

This contribution proposes two modifications for blocks applied with combined inter-intra prediction (for simplification, these blocks are named as combined prediction blocks). The first modification occurs to MPM (Most Probable Mode) list construction, where combined prediction blocks are considered as inter blocks always. The second modification is applied for deblocking filter, where combined prediction blocks are considered as intra blocks in boundary strength derivation. Two solutions are proposed: the first solution applies modification only for MPM list construction, the second solution applies both modification of MPM list construction and boundary strength derivation. The BD-rate results over the VTM3.0 anchor are:

- -0.01% (RA), 0.01% (LDB), -0.04% (LDP) for luma component with the first solution

- 0.00% (RA), 0.00% (LDB), -0.07% (LDP) for luma component with the second solution

AI results are the same to the anchor as modifications have no effect on I frames, wherein no combined prediction blocks exist.

Similar to the previous but it is suggested to set boundary strength to 2, which would typically be used when one side of the boundary is intra. Furthermore, subblocks are not deblocked here.

Revisit: Proponents of M0103 and M0294 should come together and suggest a reasonable solution to fix this bug.

JVET-M0587 Crosscheck of JVET-M0294 (CE10-related: Modification for blocks applied with Combined Inter-Intra prediction) [K. Zhang (Bytedance)] [late] [miss]

JVET-M0187 CE11-related: Long deblocking filters with reduced line buffer requirement and enhanced parallel processing accessibility [C.-M. Tsai, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

This contribution proposes two aspects of modifications on top of the CE11.1.3 long deblocking filters in JVET-M0186. In both VTM3.0 and CE11.1.3, deblocking can be performed at 8x8 grids and is skipped at non-8x8 grids. The maximum numbers of to-be-read luma samples on one side of an edge during deblocking are four and eight for VTM3.0 and CE11.1.3, respectively. The maximum numbers of to-be-modified luma samples on one side of an edge during deblocking are three and seven for VTM3.0 and CE11.1.3, respectively. In the first aspect, to keep the number of luma sample lines needed to be stored in the line buffer as low as that in VTM3.0, during vertical filtering across horizontal coding tree unit (CTU) boundaries, the maximum number of to-be-read luma samples above the horizontal CTU boundary is reduced from eight to four, and in addition, the maximum number of to-be-modified luma samples above the horizontal CTU boundary is set to three. In the second aspect, to better support parallel deblocking at multiple parallel subblock boundaries on 8x8 grids, during horizontal filtering across vertical subblock boundaries and vertical filtering across horizontal subblock boundaries, if a subblock coding mode is chosen on one side of the subblock boundary, the number of to-be-read luma samples on that side of the subblock boundary is reduced from eight to four, and in addition, the maximum number of to-be-modified luma samples on that side of the subblock boundary is set to three. It is reported that the proposed deblocking results in negligible changes in BD-rate and encoding time, and the decoding time increase is 3% for VTM3.0 common test condition (CTC) and for VTM3.0 CTC with adaptive loop filter (ALF) off.

Generally similar approach as in CE11.1.7/8 and CE11.1.5, to reduce number of line buffers at CTU boundary. Further, the issue of duplicate deblocking of subblock boundaries (as mentioned under CE11) is solved for 11.1.3, in a similar fashion as used in CE11.1.6/7/8.

This would probably be something worthwhile to do as bug fix on the CE proposal (respectively to save line buffers).

JVET-M0604 Crosscheck of JVET-M0187 (CE11-related: Long deblocking filters with reduced line buffer requirement and enhanced parallel processing accessibility) [A. M. Kotra (Huawei)] [late] [miss]

JVET-M0336 Non-CE11: Considering boundary strength on CPR coded block boundary [H. Jang, J. Nam, S. Kim, J. Lim (LGE)]

This proposal presents boundary strength design on CPR coded block boundary. Specifically, following two methods for boundary strength design are suggested.

- Boundary strength is set to 0 when at least one of block is coded as CPR across the block boundary.

- Boundary strength is set to 0 when both block are coded as CPR and set to 1 when only one block is coded as CPR across the block boundary

The proposed method has been shown to provide visual quality improvement on computer generated video sequence where CPR mode occurs frequently.

It is obvious that for screen content, deblocking may not be optimum for certain cases. On the other hand, an encoder might just decide turning it off, and QP37 as shown here is no a typical operation point of screen content, as usually the target is high visual quality, and even low QP typically does not have realy high rates.

Not obvious that there is a problem and if yes, if the suggested solution is solving it.

Put the aspect of studying impact of various loop filters on screen content as a mandate for the screen content AHG.

JVET-M0605 Crosscheck of JVET-M0336 (Non-CE11: Considering boundary strength on CPR coded block boundary) [A. M. Kotra (Huawei)] [late] [miss]

JVET-M0339 CE11-related: subblock boundary filter at 8x8 Grid [H. Jang, J. Nam, S. Kim, J. Lim (LGE)]

This contribution proposes a modification on subblock boundary filtering on 8x8 grid. In order to filter all subblock boundaries on 8x8 grid, subblock boundaries where the current PU is not aligned with 8x8 grid are also filtered in the proposed method, 4x4 block boundaries for affine and 8x8 block boundaries for SbTMVP. The proposed method shows -0.01% BD-rate change for RA configuration and also gives subjective quality improvement on subblock boundaries.

This is another variant over CE11.2, which gives up the restriction that only when the CU boundary is on an 8x8 grid, the corresponding boundaries of underlying subblocks are not deblocked in current VTM. This still allows parallel processing.

One expert asks if we should not potentially give up the requirement of parallel processing (i.e. deblock kind of AVC style on 4x4 grid), instead of imposing sophisticated rules when to deblock or not, depending on block structure.

JVET-M0606 Crosscheck of JVET-M0339 (CE11-related: subblock boundary filter at 8x8 Grid) [A. M. Kotra (Huawei)] [late] [miss]

JVET-M0317 has also an aspect deblocking for triangular partitions. It is however not evident that there is a problem that causes artifacts, and therefore imposing another condition in deblocking based on triangle mode may not be necessary.

Revisit on this section: Determine next steps in deblocking after looking at viewing results.

## CE12 related – Mapping functions (2)

Contributions in this category were discussed Saturday 12 Jan. 1800–1830 (chaired by GJS).

JVET-M0109 CE12-related: block-based in-loop reshaping [E. François, C. Chevance, F. Hiron (Technicolor)]

This document describes a block-based implementation of the in-loop luma reshaping approach proposed in contributions JVET-L0246 and JVET-L0247. In-loop luma reshaping requires at the decoder two basic sample mapping operations: forward reshaping of the prediction signal, inverse reshaping of the reconstructed signal. Both operations are by default operated per sample, requiring a per-sample access to a look-up-table. In the proposed approach, the process is performed per block. For each 4x4 block, linear model parameters are computed from three luma values and their mapped version. The linear model is then applied to the block samples. The proposal results in PSNR-Y, U, V BD-rate variations compared to the VTM3.0 of -0.74%, 1.49%, 1.22% in AI configuration, -1.17%, 1.50%, 1.50% in RA configuration, -0.54%, 2.62%, 3.21% in LB configuration.

The described method does not perform quite as well as the per-sample look-up.

Some banding was reported for content with flat areas (esp. screen content).

Further work was encouraged to try to find some complexity reduction of the remapping process, although this scheme does not seem adequate as-is.

It was commented that for hardware it might actually be easier to do the per-sample processing, as this avoids the extra step of computing an average.

JVET-M0580 Crosscheck of JVET-M0109 (CE12-related: block-based in-loop luma reshaping) [T. Lu, P. Yin (Dolby)] [late]

JVET-M0640 CE12-related: in-loop reshaping with approximate inverse mapping function [E. Francois (Technicolor)] [late] [miss]

This contribution relates to the in-loop luma reshaping approach proposed in contributions JVET-L0246 and JVET-L0247. In-loop luma reshaping requires at the decoder two basic sample mapping operations: forward reshaping of the prediction signal, inverse reshaping of the reconstructed signal. In its current implementation, the forward mapping can be achieved on-the-fly per sample using simple operations, whereas inverse mapping requires more complex operations. In this contribution it is proposed to approximate the inverse mapping function using a piece-wise linear (PWL) model of limited size, each piece being defined per luma interval of fixed length, power of 2, so that simple shifting can be used to identify the interval containing the sample value to map. It is reported that for most of the natural sequences, the gain of in-loop reshaping an be preserved using the proposed approximation with a PWL model size of 32 elements. For class E, F and some HDR content, larger PWL model size is required.

As with the technique described in M0109, some artefacts and loss were observed, although further work was encouraged to try to find some complexity reduction of the remapping process, although this scheme does not seem adequate as-is.

JVET-M0703 Crosscheck of JVET-M0640 (CE12-related: in-loop luma reshaping with approximate inverse mapping function) [T. Lu, P. Yin] [late]

## CE13 related – Coding tools for 360° omnidirectional video (7)

Contributions in this category were discussed XXday X Jan. XXXX–XXXX (chaired by XXX).

JVET-M0322 CE13-related: In-loop filters disabled across face discontinuities on PHEC with 2-pixel padding [Y. Sun, X. Huangfu, L. Yu (Zhejiang Univ.)] [late]

JVET-M0810 Crosscheck of JVET-M0322 (CE13-related: In-loop filters disabled across face discontinuities on PHEC with 2-pixel padding) [P. Hanhart (InterDigital)] [late]

JVET-M0323 CE13-related: Adaptive QP to improve subjective quality for PHEC [Y. Sun, X. Huangfu, L. Yu (Zhejiang Univ.)] [late]

JVET-M0811 Crosscheck of JVET-M0323 (CE13-related: Adaptive QP to improve subjective quality for PHEC) [P. Hanhart (InterDigital)] [late]

JVET-M0534 CE13-related: HEC with Pre-rotation + Adaptive Frame Packing (Test 4.2.a+4.1) [C. Pujara, A. Konda, A. Singh, R. Gadde, W. Choi, K. Choi, K.P. Choi (Samsung)] [late]

JVET-M0645 Crosscheck of JVET-M0534 (CE13-related: HEC with Pre-rotation + Adaptive Frame Packing (Test 4.2.a+4.1)) [P. Hanhart (InterDigital)] [late] [miss]

JVET-M0225 AHG8: On wrap around motion compensation [B. Choi, W. Feng, S. Liu (Tencent)] [late]

JVET-M0368 AHG8: 360Lib support for chroma sample location in PHEC blending process [C.-H. Shih, Y.-H. Lee, J.-L. Lin, Y.-C. Chang, C.-C. Ju (MediaTek)]

JVET-M0644 Crosscheck of JVET-M0368 (AHG8: 360Lib support for chroma sample location in PHEC blending process) [P. Hanhart (InterDigital)] [late] [miss]

JVET-M0452 AHG8: Hemisphere cubemap projection format [J. Boyce, M. Dmytrychenko (Intel)] [late]

JVET-M0547 360° coding tools using uncoded areas [J. Sauer, M. Bläser (RWTH Aachen University)] [late]

## Loop filtering tools (13)

Contributions in this category were discussed Sunday 13 Jan. 1530–1800 and Monday 14 Jan. 1600-1715 (Track B chaired by JRO).

JVET-M0162 Adaptive loop filter with a maximum number of luma filters per slice constraint [C.-Y. Chen, Z.-Y. Lin, C.-Y. Lai, Y.-W. Huang, S.-M. Lei (MediaTek)]

This contribution proposes a “maximum number of luma filters per slice” constraint to adaptive loop filter (ALF) in order to reduce the storage of filter coefficients in the on-chip memory. In the current ALF design, up to 25 luma filters can be signalled and used in one slice, which requires one on-chip memory with size equal to 25 filters x 13 coefficients per filter x 8 bits per coefficient = 2600 bits. The proposed method is to constrain the number of luma filters per slice. When the maximum number of luma filters is reduced from 25 to 16, the BD-rates for AI, RA, and LB are 0.00%, 0.00%, and 0.00%, respectively and 36% on-chip memory of luma filter coefficients is saved. When the maximum number of luma filters is 12, the BD-rates for AI, RA, and LB are 0.01%, 0.01%, and -0.01%, respectively, and 52% on-chip memory of luma filter coefficients is saved. When the maximum number of luma filters is 8, the BD-rates for AI, RA, and LB are 0.04%, 0.04%, and 0.01%, respectively, and 68% on-chip memory of luma filter coefficients is saved.

Generally, not a big deal (low amount of memory saving, while possibly giving up flexibility). It is however argued by proponents that this becomes more relevant together with JVET-M0163.

JVET-M0618 Crosscheck of JVET-M0162 (Adaptive loop filter with a maximum number of luma filters per slice constraint) [Y.-W. Chen (Kwai Inc.)] [late] [miss]

JVET-M0163 Adaptive loop filter with history filters [C.-Y. Chen, Z.-Y. Lin, C.-Y. Lai, Y.-W. Huang, S.-M. Lei (MediaTek)]

This contribution proposes history filters in adaptive loop filter (ALF). The concept of history filters is to allow using history filters for the current slice to increase coding efficiency, where the history filters are decoded filters from previously decoded slices. When the maximum number of history filter set is five, where one filter set contains all signalled filters of one slice, BD-rate savings are 0.16% and 0.34% for RA and LB, respectively, and the required memory of history filter set storage is 1660 bytes (5 sets * 25 luma filters per set * 13 coefficients per luma filter * 1 byte per coefficient + 5 sets * 1 chroma filter per set * 7 coefficients per chroma filter * 1 byte per coefficient). To reduce the memory requirement, it is further proposed to apply history filters with the “maximum number of luma filters per slice” constraint in JVET-M0162. When the maximum number of luma filters per slice is reduced from 25 to 16, BD-rate savings are 0.16% and 0.33% for RA and LB, respectively, and the required memory size is 1075 bytes (65% of no constraint). When the maximum number of luma filters per slice is 12, BD-rate savings are 0.15% and 0.33% for RA and LB, respectively, and the required memory size is 815 bytes (49% of no constraint).

The proposal targets compression improvement by re-using filters from previous pictures. The argument of saving on-chip memory (as under JVET-M0162) is not fully understood, as it would be external memory that is needed to store the coefficients along with the refernce pictures.

The reduction in bit rate is relatively low, and the approach would introduce some additional dependency between parameters of pictures. The current design of coding ALF parameters independently for every picture is quite clean.

It was also asked how it is determined which filter to use. This is based on matching with the covariance matrix.

No action.

JVET-M0619 Crosscheck of JVET-M0163 (Adaptive loop filter with history filters) [Y.-W. Chen (Kwai Inc.)] [late] [miss]

JVET-M0164 Adaptive loop filter with virtual boundary processing [C.-Y. Chen, T.-D. Chuang, Z.-Y. Lin, C.-Y. Lai, Y.-W. Huang, S.-M. Lei (MediaTek)]

In the adaptive loop filter (ALF) of VTM3.0, 7x7 diamond filters with 4x4 block-based classification are used for luma, and a 5x5 diamond filter without classification is used for chroma, which induces seven luma sample line buffers and four chroma sample line buffers in ALF implementation at the decoder. In order to reduce the line buffer requirement, ALF with virtual boundary (VB) processing is proposed: when one sample located at one side of a VB is filtered, accessing samples located at the other side of the VB is forbidden. The originally required samples at the other side of the VB are replaced with padded samples. To accommodate deblocking filter (DF) and sample adaptive offset (SAO) in VTM3.0, the VBs are set as four luma lines and two chroma lines above CTU row boundaries. It is reported that ALF with VB processing results in 0.07%, 0.10%, and 0.11% BD-rates for AI, RA, and LB, respectively, while the VB processing can totally remove the line buffer requirement of ALF.

It is asked whether this might introduce artifacts that make the CTU structure visible. The proponents report that they include a weighted average after the filtering, where the original sample is superimposed in a weighted fashin with the filtered sample. It is reported that such an approach in the context of HEVC ALF was effective to avoid visibility of artifacts.

Generally, the reduction of line buffers is very desirable, and would also justify the reported compression loss. It would however definitely be necessary to assess if it might introduce viusal artifacts in the context of VVC and its ALF design.

Similar concept in JVET-M0301.

Investigate in CE, and this should include subjective testing

JVET-M0730 Crosscheck of JVET-M0164 (Adaptive loop filter with virtual boundary processing) [T.-H. Li, P.-H. Lin (Foxconn)] [late] [miss]

JVET-M0277 Fixes of enabling pcm_loop_filter_disabled_flag with pcm mode signalling under dual tree partition [L. Zhang, K. Zhang, H. Liu, J. Xu, Y. Wang, P. Zhao, D. Hong (Bytedance)]

In current VVC, PCM mode flags for luma and chroma components are signalled separately when dual tree partition is enabled. However, when the pcm_loop_filter_disabled_flag is enabled, determination of enabling/disabling SAO filtering process for chroma blocks is wrongly based on the signalled PCM mode flag for luma blocks. In addition, ALF is performed even when pcm_loop_filter_disabled_flag is true. In this contribution, these two issues are fixed wherein whether to filter a block is dependent on the signalled PCM mode flag for the associated color component. Simulation results reportedly show that there is no performance changes under common test conditions.

Decision (BF/text): Include disabling ALF as a third loop filter when the pcm_loop_filter_disabled_flag is set, as suggested in JVET-M0277

Decision (BF/SW): Disable SAO and ALF in chroma part when dual tree is used and the flag is set, and disable ALF in luma part when dual tree is used and the flag is set, as suggested in JVET-M0277

Some experts commented that it might be better to get rid of PCM mode, as it is hardly used in practical encoders.

JVET-M0614 Crosscheck of JVET-M0277 (Non-CE: Fixes of enabling pcm_loop_filter_disabled_flag with PCM mode signalling under dual tree partition) [Y.-C. Sun (Alibaba)] [late] [miss]

JVET-M0301 Non-CE: Loop filter line buffer reduction [A. M. Kotra, S. Esenlik, B. Wang, H. Gao, J. Chen (Huawei)]

The current contribution proposes a mechanism of reducing the line buffer requirement of ALF (adaptive loop filter). The contribution uses the concept of virtual boundaries (VBs) which are upward shifted horizontal CTU boundaries by “N” samples. Modified ALF block classification and modified ALF filtering are applied for the samples which are near the virtual boundary to reduce the number of line buffers required. Modified ALF block classification only uses the samples which are above the VB to classify the given 4 x 4 block which is above VB. Similarly for the classification of the 4 x 4 block below VB, samples belonging to the lines below the VB are used. Modified ALF filtering uses a combination of conditional disabling and truncated versions of the original ALF filter.

Objective results are as follows: Over VTM 3.0 Anchor, for the case when “N” is 4 (AI, RA, LDB, LDP): Luma BD-Rate of 0.07%, 0.09%, 0.08%, 0.07% with no increase in EncT and DecT.

Over VTM 3.0 Anchor, for the case when “N” is 6 (AI, RA, LDB, LDP): Luma BD-Rate of 0.10%, 0.12%, 0.16%, 0.16% with no increase in EncT and DecT.

In terms of virtual boundary, the method is following the same concept as M0164. It is however somehow different in how the classification for padded samples is determined.

Investigate in CE, and this should include subjective testing.

JVET-M0553 Crosscheck of JVET-M0301 (Non-CE: Loop filter line buffer reduction) [C.-M. Tsai (MediaTek)] [late] [miss]

JVET-M0353 In-loop filtering: Simplification of ALF coefficients merge [M. Ikeda, T. Suzuki (Sony)]

In this contribution, the simplification of ALF (Adaptive Loop Filter) coefficients merge is proposed. In VTM-3.0, ALF coefficients can be merged after deriving 25 sets of luma filter coefficients in order to reduce signaling bits overhead in encoder side. In the merge process, pairs of classification index to be merged are identified by an exhaustive search. In this contribution, three variants are presented, and they can simplify the process and provide the similar coding efficiency: variant1: AI 0.07%, RA 0.10% LDB 0.18%, variant2: AI 0.08%%, RA 0.11% LDB 0.15%, variant3: AI 0.07%, RA 0.09% LDB 0.17%, respectively.

The approach would simplify the syntax, but add a number of tables to describe which options of merging can be used (less flexible than the current method) Furthermore, all three methods give a loss. The benefit in runtime reduction is not visible. No action.

JVET-M0728 Crosscheck of JVET-M0353 (Non-CE: Simplification of ALF coefficients merge Variant 3) [N. Hu (Qualcomm)] [late] [miss]

JVET-M0724 Crosscheck of JVET-M0353 (Non-CE: Simplification of ALF coefficients merge) [G. Laroche, J. Taquet (Canon)] [late] [miss]

JVET-M0385 Non-linear Adaptive Loop Filter [J. Taquet, C. Gisquet, G. Laroche, P. Onno (Canon)]

This contribution introduces an adaptive clipping operation on the input samples values of the Adaptive Loop Filter in VTM3.0. The goal of this adaptive clipping is to introduce some non-linearities to cap the difference between the input sample value to be filtered and the other neighbor input sample values of the filter.

Compared to VTM3.0, the average Luma/Cr/Cb BDR gains are reported to be -0.44%/-0.49%/-0.69% for the AI configuration, -0.70%/-1.00%/-1.04% for the RA configuration, -0.72%/-0.94%/-1.22% for the Low Delay B configuration, and -1.02%/-1.44%/-1.58% for the Low Delay P configuration.

The method requires 24 subtractions and 24 clipping operations per sample. Threshold values for clipping are determined by solving a linear equation system and using the covariance matrix, and are signalled (12 per luma filter, 6 per chroma filter). Encoder runtime is approx. unchanged, decoder runtime increases by roughly 30% on average.

Study in CE.

JVET-M0766 Crosscheck of JVET-M0385 (Non-linear Adaptive Loop Filter) [M. Ikeda (Sony)] [late] [miss]

JVET-M0428 Encoder optimization with deblocking filter [N. Hu, V. Seregin, W.-J. Chien, M. Karczewicz (Qualcomm)]

Deblocking filter is included in VTM-3.0 to apply to reconstructed pixels in order to reduce the blocking artifacts between blocks. However, the encoder of VTM-3.0 doesn’t apply deblocking filter in rate distortion optimization (RDO). In this contribution, to enhance the coding performance, deblocking filter is applied during RDO, such that distortion is calculated between filtered reconstructed pixels and original ones. Test results reportedly show 0.58%, 0.71% and 0.66% luma gain with similar encoding and decoding time, in AI, RA and LDB configuration respectively over VTM-3.0 anchor.

Decision(SW): Adopt JVET-M0428, not for CTC

Some CE might to use this for additional comparison exercising normative vs. non-normative optimizations.

However, when VVC performance is compared e.g. against HEVC, such tricks should not be used, or HM should use such option as well.

JVET-M0612 Crosscheck of JVET-M0428 (Encoder optimization with deblocking filter) [Y.-C. Sun (Alibaba)] [late]

JVET-M0429 Coding tree block based adaptive loop filter [N. Hu, V. Seregin, H. Egilmez, M. Karczewicz (Qualcomm)]

JVET-M0243 Cross-check of JVET-M0429 (Coding tree block based adaptive loop filter) [S.-C. Lim, J. Kang, H. Lee, J. Lee (ETRI)] [late]

In this proposal, the coding tree block (CTB) based ALF scheme proposed in JVET-K0382 and JVET-L0391 was tested in VTM-3.0. Test results reportedly show 0.16%, 0.51% and 0.67% luma gain with similar encoding and decoding time, in AI, RA and LDB configuration respectively over VTM-3.0 anchor.

Each CTU can select one out of 22 filter sets (16 fixed, 5 temporal, 1 picture optimized). The 16 fixed filters were optimized from HM encoded, range of qualities as in common test conditions.

As it was said before (see comments under JVET-M0163), the usage of temporal filter sets might be undesirable, as it introduces further dependency between pictures.

(It is noted that depending on possible alternatives how the filter parameters would be signalled in HLS, the above comment might need to be revised)

It is also asked how the performance would be in non-CTC QP ranges, as the fixed sets were ptimized specifically for the CTC QPs.

Compared to previous proposals on this issue, the gain is similar, but the encoding complexity is not increased.

Study in CE, also considering the questions above, performance without temporal filters, lower QP range.

JVET-M0461 Alternate ALF filter shapes for luma [D. Socek, A. Puri (Intel)]

In the contribution, it is proposed to modify luma filter shape for adaptive loop filter (ALF). In VTM3.0, the ALF shape for luma is fixed to 7x7 diamond (25 taps and 13 unique coefficients). Two modified luma ALF shapes are considered: (1) combined 7x7 diamond and 9x9 cross filter with fully symmetric outermost cross coefficient (29 taps and 14 unique coefficients, Figure 1a), and (2) combined 7x7 diamond and 9x9 cross filter with fully symmetric two outermost cross coefficients (29 taps and 13 unique coefficients).  Alternate shapes are combined with TC offsets of -2 for LB and RA, and -6 for AI. It is reported that, compared to VTM3.0, the first alternate ALF luma shape (1) yields -0.57%, -0.17%, and -0.36% luma BD-rates for AI, RA, and LB, respectively, while the second ALF luma shape (2) yields -0.57%, -0.15%, and -0.34% luma BD-rates for AI, RA, and LB, respectively.  There are minor runtime changes in the performed tests.

Presentation deck to be uploaded.

Beyond the filter shape change, Tc for deblocking is also change.

It is commented that the new filters would require two more line buffers which is undesirable. If combined with the methods of virtual CTU boundary processing, even more padding would be necessary than with the existing 7x7 filter, such that likely gain would be considerably reduced.

JVET-M0468 Non-CE: Hadamard transform domain filter [S. Ikonin, V. Stepin, J. Chen (Huawei)]

This contribution reports simulation results of post-reconstruction filter based on Hadamard transform similar to one tested in JVET-K-CE14.3b. Depending in filter appliance condition the simulations results over VTM-3.0 are:

- Test 1: For minimum block size 4x8 and 8x4: -0.50%/-0.72%/-0.58% of the luma BD-rate change with 106%/103%/102% of encoding time and 105%/101%/101% of decoding time for AI/RA/LDB configurations correspondingly.

- Test 2: For minimum block size 8x8: -0.37%/-0.64%/-0.50% of the luma BD-rate change with 105%/103%/101% of encoding time and 105%/101%/101% of decoding time for AI/RA/LDB configurations correspondingly.

- Test 3: For minimum block size 8x8 and filtering of inter blocks only: -0.39%/-0.40% of the luma BD-rate change with 102%/101% of encoding time and 101%/101% of decoding time for RA/LDB configurations correspondingly.

Test 1 and Test 2 are technology-wise identical to what had previously been investigated in CE14 before the 12th meeting, but after more analysis on the impact on intra prediction pipelining it had not been adopted.

Test 3 is new, only using the filter for inter blocks, and always using it when coefficients are coded (CBF=1), has gain of approx. 0.4% for both RA and LB. It is an additional processing stage, with complexity as per subsequent table (from BoG of last meeting):

It is pointed out that the filtering should also be disabled for CIIP and IBC, as this would have the same latency problem as intra prediction. Whereas it was said in the last meeting for another proposal (bilateral filter) that had similar latency problem for inter and only 0.4% for RA, it is interesting to note that the post-filtering gain seems still to be preserved in VTM3. It is also verbally reported that for the bilateral filter a new reduced complexity exists (at last meeting, BF was more complex than Hadamard domain filter). The proponents of BF announce that they intend to submit a late contribution on these changes.

It is discussed what the relation with other approaches, particularly diffusion filter would be. Each of those would add another stage in the pipeline of prediction, residual generation by inverse transform, etc., where one expert argues, that this pipeline should not be extended by too many steps. Complexity-wise, the diffusion filter is simpler in terms of number of operations per sample (however, has one multiplication), and a decision on that is still to be made.

During the discussion, it is questioned whether it would be better to enable switching at block level.

Further study in CE for Test 3 with constraint on CIIP and IBC, also version with block-level flag. It should also be tested how the performance is when it is outside of the loop, i.e. not used for predicting intra blocks (in which it could be applied somewhere before deblocking).

JVET-M0787 Cross-check of JVET-M0468 (Non-CE: Hadamard transform domain filter) [J. Ström (Ericsson)] [late] [miss]

JVET-M0842 Crosscheck of JVET-M0468 (Non-CE: Hadamard transform domain filter) [M. Salehifar (LGE)] [late]

JVET-M0885 Non-CE: Reduced complexity bilateral filter [J. Ström (Ericsson)] [late]

TBP – late contribution on bilateral filter when available.

## General prediction aspects (1)

Contributions in this category were discussed XXday X Jan. XXXX–XXXX (chaired by XXX).

JVET-M0634 Affine motion mode in intra coding [S. Cao, H. Han, J. Wang, F. Liang, Y. Yu, Y. Liu] [late]

## Quantization (8)

Contributions in this category were discussed XXday X Jan. XXXX–XXXX (chaired by XXX).

JVET-M0083 AHG10: Quantization matrices for MTS [T. Toma, K. Abe (Panasonic)]

JVET-M0342 Crosscheck of JVET-M0083 (AHG10: Quantization matrices for MTS) [M. Ikeda (Sony)]

JVET-M0105 Delta QP for Chroma CU [R. Chernyak, S. Ikonin, J. Chen (Huawei)]

JVET-M0188 On quantization parameter signalling considering CU area [O. Chubach, T.-D. Chuang, C.-W. Hsu, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-M0707 Crosscheck for JVET-M0188: CE7-related: On quantization parameter signalling considering CU area [Y. Wang, X. Xu (Tencent)] [late] [miss]

JVET-M0113 CE7-related: Quantization Group size uniformity [P. de Lagrange, E. François, P. Bordes (Technicolor)]

JVET-M0114 CE7-related: implicit QP-offset based on block size [P. de Lagrange, E. François, F. Le Léannec (Technicolor)]

JVET-M0119 CE7-related: Modified dequantization scaling [K. Sharman, S. Keating (Sony)]

JVET-M0318 CE7-related: QP prediction and neighbour availability [P. de Lagrange, P. Bordes (Technicolor)]

JVET-M0685 Non-CE7: On derivation of quantization parameter predictor [K. Misra, A. Segall (Sharp Labs of America)] [late]

## Entropy coding (1)

Contributions in this category were discussed XXday X Jan. XXXX–XXXX (chaired by XXX).

JVET-M0222 Context Reduction for CABAC in VVC [Y.-H. Chao, A. Said, V. Seregin, J. Dong, M. Karczewicz (Qualcomm)]

JVET-M0617 Crosscheck of JVET-M0222 (Context Reduction for CABAC in VVC) [Y.-C. Sun (Alibaba)] [late] [miss]

## Tools based on NN technology (7)

Contributions in this category were discussed Monday 14 Jan. 1815–1945 (chaired by JRO).

JVET-M0159 AHG9: Convolutional neural network loop filter [Y.-L. Hsiao, C.-Y. Chen, T.-D. Chuang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

This document presents two modifications of convolution neural network loop filter (CNNLF) introduced in JVET-K0222. The first modification is to reduce two 4-layer networks separate for luma and chroma to only one 3-layer network shared by luma and chroma. The second modification is to conditionally signal the CNNLF parameters in the I-slice header. Compared with VTM3.0, the proposed CNNLF reportedly achieves -1.23% (Y), -10.11% (Cb), and -9.96% (Cr) BD-rates with 42% decoding time increase in random access (RA) condition without using any GPUs. After shifting coding gain from chroma to luma by increasing chroma quantization parameter (QP) offsets, the BD-rates are -2.47% (Y), +2.90% (Cb), and +3.01% (Cr). It is shown that Class C (small resolution, 832x480) has no coding gain because of the relatively “expensive” side information bits of CNNLF parameters while Class A (large resolution, 3840x2160) has higher coding gains (-3.7% luma BD-rate and +3% chroma BD-rate). Further research on CNNLF to reduce complexity and enhance training for improving coding efficiency is suggested.

It is assumed that the CNN parameters are offline trained per RA period.

Results with chroma QP offset are non-CTC, difficult to conclude something from that.

It is reported that the gain becomes lower for low resolution sequences such as class C, due to the higher relative amount of network parameters (which is about 10 kbit/s).

Software would be available.

JVET-M0771 Crosscheck of JVET-M0159 (AHG9: Convolutional neural network loop filter) [H. Dou, Z. Deng, J. Boyce (Intel)] [late] [miss]

JVET-M0215 AHG9-related: CNN-based lambda-domain rate control for intra frames [Y. Li, D. Liu, Z. Chen (USTC)]

This contribution proposes a CNN-based λ -domain rate control approach for intra frame coding. Compared with the exiting SATD-based intra rate control approach in VTM, the contribution reuses the R-lambda model in VTM inter frame rate control and train two convolutional neural networks to predict the model parameters, alpha and beta respectively. Compared with the rate control method in VTM 3.0, the proposed method can achieve an average bd-rate reduction of 2% under All Intra configuration. When considering the mismatch between target bitrate and actually coded bit rate, the CNN-based method can achieve a smaller rate control error, especially for the first I frame.

The uploaded presentation deck is different from the one shown in the presentation.

Approximately 5% run time increase compared to current rate control (CPU).

Software would be available. To be further studied in the AHG on NN technology.

Very interesting to see that NN based rate control can outperform conventional approaches, but extension of its applicability to other frames (e.g. predicting rates for the different levels of B picture hierarchy) would be desirable.

JVET-M0351 Convolutional Neural Network Filter (CNNF) for Intra Frame [C. Lin, J. Yao, L. Wang (Hikvision)] [late]

Initial upload rejected as a placeholder.

This contribution provides a convolutional neural network filter (CNNF) for intra frames. In the current VTM, multiple filters, i.e., deblocking filter (DF) and sample adaptive offset (SAO) are used to remove artifacts or improve performance. CNNF is motivated by the latest advances in deep learning and is proposed as a single type of filter to replace multiple filters in intra frame. Simulation results report -4.94%, -7.07%, -8.17% BD-rate savings for luma, and both chroma components for VTM-3.0 with AI configuration.

Same method was proposed in JVET-I0022 (by that time run on top of JEM). Similar gain.

Software was already released by that time.

JVET-M0743 Crosscheck of JVET-M0351 (AHG9: Convolutional Neural Network Filter (CNNF) for Intra Frame) [Y. Dai, D. Liu, Y. Li, F. Wu] [late] [miss]

JVET-M0508 AHG9: Test Results of Dense Residual Convolutional Neural Network based In-Loop Filter [Y. Wang, Z. Chen, Y. Li (Wuhan Univ.), L. Zhao, S. Liu, X. Li (Tencent)]

This contribution reports the test results for dense residual convolutional neural network based in-loop filter (DRNLF) JVET-L0242 according to the methodology in JVET-L1006. The proposed DRNLF is implemented on VTM 3.0. Simulation results report -2.17%, -1.47%, -1.48% BD-rate savings for luma , and both chroma components compared with VTM 3.0 under AI configuration in CPU only platform, and -2.15%, -3.04%, -1.96% for RA configuration, and -2.06%, -3.73%, -2.86% for LDB configuration.

Operated between deblocking and SAO.

Different networks were trained specifically for the different QP values of CTC.

It is noted that it would be interesting to investigate how large the loss would be when used for another QP value.

Gain over VTM3 is slightly lower than it was for VTM2.

JVET-M0510 AHG9: CNN-based in-loop filter proposed by USTC [Y. Dai, D. Liu, Y. Li, F. Wu (USTC)] [late] [miss]

This contribution presents the simulation results of an efficient network for loop filter. To reduce storage space and reduce complexity, we build two light weight deep convolutional neural networks by reduce network parameters. Simulation results report -0.96%, -0.32%, -0.45% BD-rate savings for Y, Cb, and Cr components compared with VTM3.0 under AI configuration, -0.61%, -0.25%, -0.26% for RA configuration, and -0.76%, -0.56%, -0.69% for LDB configuration.

Operated between deblocking and SAO.

For AI; Decoding time 21x/13x for the two versions used. For LB: 3-4x. Encoding time increases approx. 25%.

Can be enabled/disabled at CTU level.

Question: Does this produce visual artifacts?

JVET-M0673 Crosscheck of JVET-M0510: AHG9: CNN-based in-loop filter proposed by USTC [F. Chen (Hikvision)] [late] [miss]

JVET-M0566 Adaptive convolutional neural network loop filter [H. Yin, R. Yang, X. Fang, S. Ma, Y. Yu (Intel)] [late]

This document proposes an adaptive convolution neural network loop filter (ACNNLF) design. In this design, 3 CNN based loop filters are adaptively trained for luma and chroma respectively from the current video sequence. Each filter is a small 2 layer CNN with total 692 parameters. The encoder selects one of the three ACNNLFs for luma and chroma respectively for each CTB block during encoding. The weights of the trained set of ACNNLFs are signaled in the slice header of I frames and the index of selected ACNNLF is signaled for each CTB.

Compared with VTM-3.0-RA, the proposed ACNNLF achieves -2.37%, -1.34%, and -2.77% BD-rates for Y, U, and V, respectively, for Class A1 video sequences; -0.45%, -10.92%, and -6.19% BD-rates for Y, U, and V, respectively, for Class A2 video sequences; -0.49%, -11.29%, and -10.73% BD-rates for Y, U, and V, respectively, for Class B video sequences; and 0.12%, -3.31%, and -1.62% BD-rates for Y, U, and V, respectively, for Class C video sequences.

Operated after ALF (last loop filter)

Two different CNN for luma and chroma. At CTU level, it can be decided which out of three filters to use (or no filter). An additional weighting is signalled at slice level, which weights the residual that is generated by the network and superimposed.

Gain comes mainly from few sequences, campfire has biggest gain.

JVET-M0691 AHG9: Complexity analysis about neural network video coding tools [Y. Li, Z. Chen (Wuhan Univ.), S. Liu (Tencent)] [late] [miss]

To be reviewed in BoG.

JVET-M0872 AHG9: A Result of Convolutional Neural Network Filter [K. Kawamura, S. Naito (KDDI)] [late]

This contribution contains a performance evaluation of the convolutional neural network filter proposed in JVET-L0383. Compared with the contribution in the previous meeting, the result in this meeting comes from not only intra picture but also from inter picture. Coding gain of Y BD-rates are -1.65% / -1.82% / 00.49% for all intra, random access, and low-delay B condition, respectively.

Previous contribution had only results on AI. Gains in AI over VTM3 are similar as they were over VTM2.

BoG on planning a CE on NN based loop filters (Yiming Li). The main target of that CE should be to investigate the impact of adaptation/switching (e.g. sequence basis, CTU basis, QP related); impact of position in the loop filtering chain; investigate performance and complexity of different architectures, etc.

## High-level syntax (53)

Contributions in this category were discussed XXday X Jan. XXXX–XXXX (chaired by XXX).

### General high-level syntax and parameter sets (22)

JVET-M0776 AHG17&AHG15: A summary of JVET-M contributions on general HLS [Y.-K. Wang (Huawei)] [late]

#### General high-level syntax and NAL unit header (11)

JVET-M0101 AHG17: On VVC HLS [R. Skupin, K. Sühring, Y. Sanchez (HHI), M. M. Hannuksela, K. Kammachi-Sreedhar (Nokia), Y.-K. Wang, Hendry (Huawei), S. Wenger, B. Choi (Tencent), S. Deshpande (Sharp)]

JVET-M0120 Proposed NAL Unit Header Design Principles [S. Wenger, B. Choi, S. Liu (Tencent)]

JVET-M0131 AHG17: On NAL unit types for IRAP pictures and leading pictures [Y.-K. Wang, Hendry (Huawei)]

JVET-M0152 AHG17: On random access point for VVC [B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-M0153 AHG17: On leading picture for VVC [B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-M0156 AHG17: On component type indication for VVC [B. Choi, S. Wenger, S. Liu (Tencent)] [late]

JVET-M0157 AHG17: On picture order count for VVC [B. Choi, S. Wenger, S. Liu (Tencent)] [late]

JVET-M0161 AHG17: Signalling random access properties in the NAL unit header [L. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-M0520 AHG17: On NAL unit header design for VVC [S. Wenger, B. Choi, S. Liu (Tencent)]

JVET-M0529 AHG14: Normative Recovery Point Indication [M. Pettersson, R.Sjöberg, M. Damghanian (Ericsson)]

JVET-M0537 AHG17: On tile group signaling in NAL unit header and as non-VCL NAL unit [E. Thomas, A. Gabriel (TNO)] [late]

#### Reference picture management (3)

JVET-M0128 AHG17: On reference picture management for VVC [Y.-K. Wang, Hendry (Huawei), S. Deshpande (Sharp), M. M. Hannusela (Nokia), G. Ryu, W. Choi (Samsung), X. Wang, Y.-W. Chen (Kawi), L. Zhang (Bytedance), P. Wu, M. Li (ZTE), S.-H. Kim (LG), J. Boyce (Intel), A. M. Tourapis, D. Singer (Apple), F. Edouard, P. Andrivon (Technicolor), Y.-W. Huang, C.-W. Hsu, C.-Y. Chen, T.-D. Chuang, L. Chen (MediaTek), K. Kawamura (KDDI), Y.-C. Sun, J. Lou (Alibaba)]

After BoG discussion, this was discussed Sunday 13 January 1700 (GJS).

This contribution proposes a reference picture management approach for VVC based on direct signalling and derivation of reference picture lists 0 and 1, without use of reference picture set (RPS) as in HEVC or the sliding window plus memory management control operation (MMCO) process as in AVC.

It is asserted that the proposed approach is significantly simpler compared to the approaches in HEVC and AVC.

The proposed direct-RPL-based reference picture management approach is summarized as follows:

Two reference picture lists, list 0 and list 1, are directly signalled and derived. They are not based on RPS as in HEVC or the sliding window plus MMCO process as in AVC.

Reference picture marking is directly based on reference picture lists 0 and 1, utilizing both active and inactive entries in the reference picture lists, while only active entries may be used as reference indices in inter prediction of CTUs.

Information for derivation of the two reference picture lists is signalled by syntax elements and syntax structures in the SPS, the PPS, and the slice header. Predefined RPL structures are signalled in the SPS, for use by referencing in the slice header.

The two reference picture lists are generated for all types of slices, i.e., B, P, and I slices.

The two reference picture lists are constructed without using a reference picture list initialization process or a reference picture list modification process.

Long-term reference pictures (LTRPs) are identified by POC LSBs. When needed, additional POC LSBs are signalled for LTRPs, determined at a picture by picture basis.

In the discussion, a problem was identified with the POC MSB handling when considering random access that is not at the start of the first CVS in the “original” bitstream. Instead of signalling MSBs directly, it was suggested to use the scheme of HEVC with delta_poc_msb_present_flag[ i ] and delta_poc_msb_cycle_lt[ i ] (and remove the proposed additional MSB length syntax in the PPS).

In the discussion, it was noted that there should be a POC wrap-around prevention constraint for short-term pictures (see HEVC C.4 item 8). It was agreed that this is needed.

Decision: Adopt with modifications as described above.

JVET-M0154 AHG17: On decoded picture buffer management for VVC [B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-M0378 AHG17: RPS for VVC [R. Sjöberg, M. Damghanian, M. Pettersson (Ericsson)] [late]

#### Picture header and header parameter set (HPS) (4)

JVET-M0132 AHG17: On header parameter set (HPS) [Y.-K. Wang, Hendry, J. Chen (Huawei)]

JVET-M0260 AHG17: Carriage of tile group header parameters in higher level structures [M. M. Hannuksela (Nokia)]

JVET-M0377 AHG17: Picture header NAL unit type [R.Sjöberg, M. Damghanian, M. Pettersson (Ericsson)]

JVET-M0415 AHG17: Comments on High-Level Syntax of VVC [S. Deshpande (Sharp)]

#### Miscellaneous general HLS topics (3)

JVET-M0133 AHG17: On parsing dependency between parameter sets [Y.-K. Wang (Huawei), J. Boyce (Intel)]

JVET-M0386 AHG17: On slice_type (tile_group_type) [K. Sühring, Y. Sanchez, R. Skupin (HHI)]

JVET-M0579 On Frame Rate Support and Extraction in VVC [A. Segall, S. Deshpande (Sharp Labs of America), M. Hannuksela (Nokia)]

### Interoperability and capability points definition and signalling (1)

JVET-M0451 Update to interoperability point syntax [J. Boyce (Intel)]

### Tiling and tile partitioning (28)

JVET-M0774 AHG12: A summary of JVET-M contributions on picture partitioning [Y.-K. Wang (Huawei), M. M. Hannuksela (Nokia)] [late]

#### Tiling allowing tile size unit less than CTU size (5)

JVET-M0066 AHG12: Flexible Tile Partitioning [Y. Yasugi, T. Ikai (Sharp)]

JVET-M0423 Cross-check of JVET-M0066: AHG12: Flexible Tile Partitioning [A. Wieckowski (HHI)]

JVET-M0376 AHG12: On signaling of flexible tiles [M. Damghanian, R.Sjöberg, M. Pettersson (Ericsson)]

JVET-M0459 AHG12: On tiles with partial CTUs [R. Skupin, K. Sühring, Y. Sanchez, T. Schierl (HHI)]

JVET-M0527 AHG12: Comments on Tiles and Flexible Tile Partitioning [W. Wan, M. Zhou, T. Hellman, B. Heng, P. Chen (Broadcom)]

JVET-M0875 Request for flexible unit size tile with implementation friendly restriction [T. Ikai, Y. Yasugi (Sharp), G. Bang (ETRI), Y.-W. Chen, X. Wang (Kwai Inc.), M. Coban (Qualcomm), C.-C. Lin (ITRI), J.-L. Lin (MediaTek), P.-H. Lin (Foxconn), A. Ichigaya (NHK), K. Kawamura (KDDI), K. Kazui (Fujitsu), R. Sjöberg (Ericsson), R. Skupin, K. Sühring, Y. Sanchez, T. Schierl (HHI), L. Zhang (Bytedance)] [late]

This contribution proposes flexible unit size tile with certain restrictions, allowing tiling with tile size unit that is not a multiple of the CTU size. In the tile BoG, the implementation difficulty for less than CTU size and loop filter control for 360 video has been discussed. In this contribution, it is asserted that CTU alignment is too restricted to enable important tile use cases. It is also argued that the proposed specific restrictions alleviate the concerns implementation cost increase.

In JVET-M0066, the main part of this proposal had reportedly been implemented, tested and cross-checked. The software and working draft had been uploaded at 2018-12-28 05:28:56 and this was announced in the JVET main reflector at 2018-12-29.

The contribution reported the following points as the main difficulty for hardware implementation which could affect cost:

- Throughput capability, i.e. Partial size CTUs could need the same time processing time as full size CTUs in pipeline processing.

- Memory bandwidth / compression, i.e. temporal motion vector or other information needs to be transferred between internal memory and external memory via burst transfer.

- Address generator, i.e. address generator needs more flexible calculation which depends on tile position.

The contribution proposed that the minimum unit granularity (TileUnitSizeY) be required to be 32 or larger.

A participant commented that if the 32x32 restriction is imposed, the desired functionality can be obtained with no special support in the standard, at a small loss of coding efficiency – by using 32x32 as the CTU size.

It was also commented that the 32x32 granularity is still restrictive and may not align with natural content boundaries.

The proponent showed some test results that indicated that the penalty of using 32x32 CTUs was large. Other participants questioned whether that penalty could really be that large. The data had not been cross-checked.

A participant commented that pipelining implementation requires fetching and processing data in large chunks with a regular structure that this would interfere with.

There were strong concerns about this expressed by some participants. It was said that the 32x32 CTU case is not as difficult because these come in strings.

Further study was encouraged. It was planned to ask AHG13 to measure the effect of CTU size on coding efficiency.

#### Flexible tiling (4)

Option 3 of M0261 is effectively another way to achieve the flexible tiling functionality, through dividing pictures into rectangular sub-pictures and each sub-picture may refer to its own PPS and may hence have its own tile partitioning.

JVET-M0123 AHG12: On hierarchical tile design [Y. He, A. Hamza (InterDigital)]

JVET-M0129 AHG12: On flexible tiling [Y.-K. Wang, Hendry, M. Sychev (Huawei)]

JVET-M0374 AHG12: Flexible tiles to support MCTS use cases [R.Sjöberg, M. Damghanian, M. Pettersson (Ericsson)]

JVET-M0530 AHG12: On signalling of tiles [M. Coban, M. Karczewicz (Qualcomm)] [late]

#### Rectangular tile grouping (5)

JVET-M0121 AHG12: On Rectangular Tile Group [Y. He, A. Hamza (InterDigital)]

JVET-M0130 AHG12: On tile grouping [Y.-K. Wang, Hendry, J. Chen, M. Sychev (Huawei)]

JVET-M0160 AHG17: Flexible tile grouping for VVC [L. Chen, T.-D. Chuang, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-M0209 AHG12: On tile group configuration [W. Choi, K. Choi, K. Choi (Samsung)]

JVET-M0416 AHG12: On Tile Information Signalling [S. Deshpande (Sharp)]

#### Tile and tile group identification and addressing (5)

JVET-M0134 AHG12: On explicit signalling of tile IDs [Hendry, Y.-K. Wang, J. Chen, M. Sychev (Huawei)]

JVET-M0155 AHG12: On tile group identification for VVC [B. Choi, S. Wenger, S. Liu (Tencent)] [late]

JVET-M0373 AHG12: Merge friendly tile group address signalling [R.Sjöberg, M. Damghanian, M. Pettersson (Ericsson)]

JVET-M0430 AHG12: On Tiles and Tile Groups for VVC [R. Skupin, K. Sühring, Y. Sanchez, T. Schierl (HHI)]

JVET-M0853 AHG12: On Tile Grouping [S Deshpande (Sharp), Hendry, Y.-K. Wang (Huawei), M. M. Hannuksela (Nokia), Y. He (Interdigital), L. Chen (MediaTek), W. I. Choi (Samsung), B. D. Choi (Tencent), R. Sjöberg (Ericsson), R. Skupin (HHI)] [late]

#### MCTS and sub-picture sequence (5)

JVET-M0416 also includes one aspect on MCTS signalling in the PPS.

JVET-M0261 AHG12: On grouping of tiles [M. M. Hannuksela, A. Aminlou (Nokia)]

JVET-M0388 AHG12/AHG17: On merging of MCTSs for viewport-dependent streaming [M. M. Hannuksela (Nokia)]

JVET-M0445 AHG12: On motion constrained tiles for VVC [R. Skupin, V. George, K. Sühring, Y. Sanchez, T. Schierl (HHI)]

JVET-M0536 AHG12: On picture-level tiles and sequence-level tiles for VVC [E. Thomas, A. Gabriel (TNO)] [late]

JVET-M0870 AHG12: Proposed JVET common test conditions and evaluation procedures for MCTS and sub-pictures with boundary padding [M. Coban (Qualcomm), R. Skupin (HHI)] [late]

Discussed Monday 1940 (GJS).

This document proposes common test conditions (CTC), conversion practices, and software reference configurations to be used in evaluation of MCTS and sub-picture coding schemes.

This is for coding efficiency testing. The suggested method is to encode and decode MCTSs separately and compute and substract the duplicate header overhead data quantity to measure results. Software decoder runtimes might not be properly measured that way, since that is not likely to match how the feature would be implemented. The tested use case is a cubemap projection 360° video source.

In Track A, it was suggested to make this a CE, since it is a plan for one specific test of a particular technology.

#### Miscellaneous tiling topics (3)

JVET-M0136 AHG12: Treating tile and tile group boundaries as picture boundaries [J. Chen, Y.-K. Wang, Hendry, M. Sychev (Huawei)]

JVET-M0137 AHG12: On tile configuration signalling [M. Sychev, Hendry, Y.-K. Wang (Huawei)]

JVET-M0375 AHG12: On uniform tile spacing [M. Damghanian, R.Sjöberg, M. Pettersson (Ericsson)]

### Wavefront parallel processing (2)

JVET-M0070 AHG12: Wavefront processing in a tile group [T. Ikai, S. Deshpande, T. Chujoh, E. Sasaki, T. Aono (Sharp)]

JVET-M0071 WPP: Improved parallel processing capability with WPP [Y. Fujimoto, M. Ikeda, T. Suzuki (Sony)]

JVET-M0593 Crosscheck of JVET-M0071 (AHG12: Improved parallel processing capability with WPP) [Y. Yasugi, T. Ikai (Sharp)] [late] [miss]

# Complexity analysis and reduction (6)

Contributions in this category were discussed XXday X Jan. XXXX–XXXX (chaired by XXX).

JVET-M0245 AHG16-related: Chroma block coding and size restriction [C. Rosewarne, A. Dorrell (Canon)] [late]

Initial upload rejected as placeholder

JVET-M0821 Crosscheck of JVET-M0245 (AHG16-related: Chroma block coding and size restriction) [T. Y. Zhou, T. Ikai (Sharp)] [late]

JVET-M0065 Non-CE3: Intra chroma partitioning and prediction restriction [T. Zhou, T. Ikai (Sharp)]

JVET-M0442 Crosscheck of JVET-M0065 (Non-CE3: Intra chroma partitioning and prediction restriction) [K. Zhang (Bytedance)] [late]

JVET-M0169 CE3-related: Shared reference samples for multiple chroma intra CBs [Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-M0715 Crosscheck of JVET-M0169 (CE3-related: Shared reference samples for multiple chroma intra CBs) [X. Ma (Huawei)] [late] [miss]

JVET-M0248 AHG16: Motion compensation with padded samples for small coding units [H. Liu, J. Chon, H.-C. Chuang, L. Zhang, K. Zhang, J. Xu (Bytedance)]

JVET-M0607 Crosscheck of JVET-M0248 (AHG16: Motion compensation with padded samples for small coding units) [B. Wang (Huawei)] [late] [miss]

JVET-M0265 AHG16: Clean-up on MV Rounding [K. Zhang, L. Zhang, H. Liu, J. Xu, Y. Wang, P. Zhao, D. Hong (Bytedance)]

JVET-M0563 Cross-check of JVET-M0265 (AHG16: Clean-up on MV Rounding) [X. Chen (HiSilicon)] [late] [miss]

JVET-M0864 [AHG5] Enhancement of cache model by adopting block-based format [R. Hashimoto, S. Mochizuki (Renesas)] [late]

JVET-M0879 Crosscheck of JVET-M0864 ([AHG5] Enhancement of cache model by adopting block-based format) [T. Zhou, Y. Yasugi, T. Ikai (Sharp)] [late] [miss]

# Encoder optimization (3)

Contributions in this category were discussed XXday X Jan. XXXX–XXXX (chaired by XXX).

JVET-M0091 AHG10: Clean-up and finalization of perceptually optimized QP adaptation method in VTM [C. Helmrich (HHI)]

JVET-M0511 Bug fix for rate control under all-intra [Y. Li, D. Liu, Z. Chen (USTC)] [late]

JVET-M0600 AHG10: Quality dependency factor based rate control for VVC [Z. Liu, Z. Chen, Y. Li (Wuhan Univ.), Y. Wu, S. Liu (Tencent)] [late]

JVET-M0840 Crosscheck of JVET-M0600 (AHG10: Quality dependency factor based rate control for VVC) [X. Wang (Kwai Inc.)] [late] [miss]

# Metrics and evaluation criteria (0)

Contributions in this category were discussed XXday X Jan. XXXX–XXXX (chaired by XXX).

# Withdrawn (13)

JVET-M0041 Withdrawn

JVET-M0074 Withdrawn

JVET-M0205 Withdrawn

JVET-M0325 Withdrawn

JVET-M0370 Withdrawn

JVET-M0414 Withdrawn

JVET-M0505 Withdrawn

JVET-M0565 Withdrawn

JVET-M0608 Withdrawn

JVET-M0784 Withdrawn

JVET-M0824 Withdrawn

JVET-M0825 Withdrawn

JVET-M0841 Withdrawn

# Plenary meetings, joint Meetings, BoG Reports, and Summary of Actions Taken

## Plenary meeting Sunday 13 January 0900-1045, 1115-1400

Reports of the tracks were presented as follows:

Track A:

- The initial review had been completed for CEs assigned to Track A

- CE3-1.1.1 Intra sub-partitions coding mode (conceptually similar to prior “short-distance intra prediction”) with a different trade-off between gain and encoding run-time (at least 16 samples per partition; 2 or 4 partitions); this includes a reversal of coding order

|  | All Intra Main 10 - Over VTM-3.0 |  |  |  |  | Random Access Main 10 - Over VTM-3.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 1.1.1 | -0.59% | -0.44% | -0.47% | 112% | 103% | -0.29% | -0.31% | -0.15% | 102% | 103% |

In the plenary review, it was said that the reverse coding order part of this (which had not been discussed in the initial track review) was difficult to support in implementations, and asked whether the gain would be preserved if the reverse coding order aspect is removed. The proponent said the coding order reversal was a key part of the scheme. It was requested for a test to be performed of what the impact would be for not having that part of it. Revisit for that.

- CE3-2.4.c CCLM customization for chroma type (~6% for chroma for type 2 chroma content)

In the plenary review, it was reported that in a revision of M0142, test results had been provided for what would happen if type 2 processing were applied to type 0 content. The results indicated 0.17% degradation for luma, and ~2.1% for chroma. For class A1, the Campfire sequence had 0.83% degradation for luma, 11.36% for U, and 4.56% for V. It was noted that the subjective effect seemed likely to be greater. sps_cclm_collocated_chroma_flag = 0 would indicate type 0 processing.

- CE5-5.1.13* +new init from 5.1.2 arithmetic coding engine

| Engine | AI | RA | LB | LP | AI | RA | LB |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5.1.13* +new init from 5.1.2 | -0.98% | -0.96% | -0.75% | -0.73% | 110/105 | 104/103 | 106/103 |

- CE6-1.1a/1.6a replacing 4-pt DST-7/DCT-8 by DST-4/DCT4

|  | All Intra |  |  |  |  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # Doc. # | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE6-1.1a/1.6a | -0.17% | -0.14% | -0.11% | 101% | 99% | -0.06% | 0.05% | 0.04% | 101% | 99% | 0.02% | 0.30% | 0.13% | 100% | 98% |

This has very small benefit, but no real impact on complexity. From a spec perspective, it is just a matter of the values of numbers in tables. It increases the number of types of transforms being used in the design and it was commented that having something different just for 4x4 seems conceptually inconsistent. It was noted that DST4 is part of DCT2. It was also noted that the impact on LB is negative. Some participants commented that the gain is too small to justify a change of the spec. The proponent commented that some gain was shown for LDB with inter-MTS on (0.07% with low-delay B). No gain was shown for low QP. It was agreed in the plenary not to adopt this change.

- CE6-2.3a DST-7/DCT-8 with dual implementation support (no coding efficiency impact)

In the AI configuration, this provides a reported 9% speed-up for the decoder, 4% for the encoder (as tested).

- CE6-3.1b Block shape adaptive transform selection, but with an extra high-level flag to use DCT2 always (benefit relative to an anchor that is not using MTS: AI 1.61%, RA 0.71%, LB 0.14%)

It was asked how much benefit this has relative to an encoder that, for rectangular blocks, chooses a fixed combination using the MTS syntax. The particular transform combination that this proposal uses for rectangular blocks has a DCT2 in one direct and a non-DCT2 transform in the other direction, which is not a combination supported in the MTS syntax, so that combination could not be selected in the suggested alternative low-complexity approach. It was commented that this may call into question the way MTS has been designed. Further study of these issues was encouraged.

- CE6-4.1a Sub-block transform with residual in only one sub-block (1-d split symmetric or 1/4; if symmetric, flag to indicate which half, otherwise use the smaller sub-block; transform type of residual TU inferred)

|  |  | All Intra |  |  |  |  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Doc. # | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE6-4.1a | JVET-M0140 |  |  |  |  |  | -0.47% | -0.16% | 0.00% | 108% | 101% | -0.83% | -0.98% | -0.06% | 113% | 102% |

- CE7.4: In transform coefficient coding, the greater than 2 flag is moved to the first coding pass after the parity bit and the number of scans is reduced from 3 to 2 (very small coding efficiency improvement)

- In-loop “reshaping” seemed likely to be adopted.

Track B:

[Clean up the relationship between this section and the related notes elsewhere, avoiding duplication and adding cross-references]

As a general remark, it was established in Track B that “further study” means that technology should be studied in next CE on the subject area, whereas if such a remark is missing it implicitly means it shall not be studied in CE. If further study in an AHG is expected, that would be explicitly expressed.

Furthermore, the issue was raised that many (or most of) CE proposals came without specification text. It was agree that in future CEs, the text should be available by the time of the document deadline. Furthermore, CE contribution documents should be complete and not make it necessary to open old documents to understand the technology.

CE2: Subblock motion compensation

- CE 2.1: Affine motion compensation

Decision: Adopt JVET-M0246 (Test 2.1.2), extending AMVR to affine (but switching 1/16,1/4,full-pel). Use the AMVR high level flag for disabling both “normal” AMVR and “affine” AMVR. This provides 0.23% bit rate reduction.

In JVET plenary, it was agreed to use a separate flag, and disable this feature in CTC for LB, as the encoder runtime increase is not acceptable there.

- CE 2.2: Affine merge mode

Decision: Adopt JVET-M0381 Test 2.2.2a (reducing number of context coded bins in affine merge).

Track B initially suggested adoption of JVET-M0431, Affine merge with offset and block level signalling, and POC distamce based offset mirroring for bi pred. (test 2.2.4c on top of 2.2.4a) (using distance offsets 1/2, 1, 2, 4, and 8-pel as per table 2.1 of M0431). Add a high-level enabling flag.

In a follow-up discussion in the JVET plenary Sunday, it was agreed that the gain of 0.2% is too small to justify the additional syntax and increase in encoder runtime. No Adoption.

No action from following sub-CEs

- CE 2.3: Sub-block based merge mode

- CE 2.4: Complexity reduction

- CE 2.5: ATMVP and related topics

CE4: Inter prediction and motion vector coding

CE4.1: Merge mode simplification

Decision: Adopt JVET-M0281 (subtest 4.1.5a), Motion vector pruning: rounding before any MV pruning (unification for all DMVR cases)

CE4.2: Merge mode enhancement: No action

CE4.3: Parallel processing for merge mode: No action

CE4.4: Motion vector coding

Track B initially suggested adoption of JVET-M0403 modified MV coding (test 4.4.1a, 2 layer groups, where the first group is just the 0,0 MVD). This does not have compression benefit, but reduces the number of context coded bins from 4 to one, using joint coding of x/y. However, in the context of later discussion related to CE8, it was detected that this might have coding efficiency problems with CPR whichhad not been checked. Furthermore, it would be more difficult to check in the layer/index representation if the CPR range constraints are valid. It was also discussed in the JVET plenary that the benefit would be rather small.

Further, some revisits (depending on availability of additional data) for modifications of MMVD

CE4.5: Motion compensation constraints for complexity reduction: No action

CE8: Screen content coding tools

8.1 CPR related

It is suggested in CTC to enable CPR for class F. This would be realistic for the case where the encoder could know that it is screen content or natural content.

The methods of CE8.1.2 provide additional gain of >3% for class F, 8% for TGM class. These are re-using existing data from the previous CTU (assuming 2 or 4 buffers of size 64x64 each, depending on version). This is generally agreed to be practical and give good benefit. It could however be complicated to specify as an encoder/bitstream restriction that the limits of CPR vectors are valid. This still needs to be decided if it is mature for adoption.

Some methods provide additional gain (e.g. 0.4% class F, 0.9% for class TGM), but modify AMVP and merge list construction. Question is raised whether VVC would require to use exactly the same principle for CPR and normal MV coding. Probably this might be OK if it does not deviate too much, and does not require much additional processing. In HEVC SCC, it was required to have exactly the same process for CPR and MV coding, which may not be the case for VVC.

As a more fundamental aspect, it was discussed in the JVET plenary what general limitations we would impose on CPR design such as vector coding, and the status of CPR at large. In this context, it was agreed that it is desirable to define CPR as a separate mode that is also named differently, e.g. IBC. In terms of keeping the specification simple, it is nevertheless desirable to avoid arbitrary differences from building blocks of motion compensation, e.g. MV coding. IBC building blocks may deviate from those of motion comp if beneficial in terms of substantial improved compression or simplification. It is also notified that in the area of SCC material, “substantial” may mean several percents.

8.2: Palette related

Current VVC does not include palette mode, however a kind of “baseline” exists which is HEVC palette plus dual tree. This provides roughly 3%/7.5% gain over VVC+CPR for classes F/TGM. Results from CE also indicate that this gain goes to 2.5%/4.5% when combined with the improved CPR from 8.1.2. This may be even less when other aspects such as transform skip come into play. Such a relative low gain might not justify adding palette as an additional building block, and need for substantial amount of text in the spec.

As a more fundamental aspect, it was discussed what should be the design targets for palette. Improved compression performance may be more important than lowering complexity, to justify addition of such a mode.

In the JVET plenary, it was also discssed if it would be useful to include testing on 4:4:4 for screen content, renew the data set for screen content (revisit – action on this), and and generally provide mechanisms in VVC text and VTM to support 4:4:4 data.

8.3: Block-based DPCM

These approaches perform sample-wise DPCM as an additional concept that demonstrates some benefit for screen content types. The most viable approach (according to proponents) is 8.3.2 which can best be optimized in terms of throughput and also shows best compression. The gain over VTM+CPR is 3.7%/4.9% for classes F/TGM, and gain over VTM+CPR+PLT is 1.3%/1.4%, respectively. The encoder runtimes are significantly faster when those methods are used, as some early termination approach is employed (not searching other intra modes when the prediction works well).

The current method uses a maximum of 12 context coded bins per sample, which is much too large. No candidate for adoption, but further study in CE, with simplifications proposed in non-CE.

From CE8 related:

As mentioned above, the general consensus is that it is a right direction to signal CPR as a separate mode rather than using a special reference picture index.

JVET-M0483 comes with spec text (“method 3”), which however needs more careful inspection and modifications (also alignment with v9 of VVC draft 3).

JVET-M0464 Unified Transform Type Signalling and Residual Coding for Transform Skip

Signals TS before MTS (and therefore also enables it for blocks up to 32x32), but also changes the MTS binarization

- CTC:   0.01%, 103%, 101% (AI),  -0.02%,  98%, 100% (RA),  -0.07%, 102%, 102% (LB)

- Class F: -1.96%, 103%,    97% (AI), -2.14%,   98%, 100% (RA), -2.79%, 103%,   99% (LB)

- TGM:    -8.04%, 106%,    88% (AI), -8.74%, 106%,   96% (RA), -9.21%, 112%,   97% (LB)

Additionally changing the residual coding for transform skip (first modification and second modification) results on average in (BD-rate Y, enc. time, dec.time):

- CTC: -0.16%, 103%, 100% (AI),    -0.07%,  98%, 101% (RA),   -0.07%, 101%, 101% (LB)

- Class F: -7.16%, 104%,   94% (AI),   -5.76%,   98%, 100% (RA),   -5.85%, 102%,   99% (LB)

- TGM:  -21.03%, 108%,   82% (AI), -15.57%, 105%,   94% (RA), -14.01%, 111%,   95% (LB)

It is dicussed whether the aspect of signalling TS before MTS (and by that way enabling TS for block sizes up to 32x32, but also restricting it to luma) would rather be a straightforward syntax cleanup, which could be adopted at this meeting (but without modifying the MTS index binarization, which is discussed in CE6). However, results of TS modification are on top of VTC, would be desirable to see benefit for SC classes when CPR is on. This requires further consideration.

There are also other contributions JVET-M0072, M0269, M0279 and M0501 which also target TS.

CE9: Decoder-side motion vector derivation

CE9.1: BDOF design

Decision: Adopt JVET-M0487 (solution 9.1.1.b) which uses integer positions to generate the prediction samples in extended region, and uses 8-tap DCTIF filters to generate the prediction samples inside CU

No loss, but simplifies BDOF

CE9.2: DMVR design

Not decided yet, but good candidate for adoption: JVET-M0147 shows results without using refined MV for spatial MV prediction and deblocking: Overall -1.01%. This seems a reasonable approach avoiding all major dependency problems that were observed in DMVR before. This is however a variant for which cross-check still needs to be provided; specification text to be made available.

Generally, the investigation on DMVR has led to a point where it might be manageable implementation-wise (not low complex, but still giving around 1% gain)

A problem to be investigated: As currently tested, DMVR and BDOF could be applied sequentially, before finally motion comp and reconstruction can be done. JVET-M0223 considers this issue, still needs to be reviewed.

CE10: Combined and multi-hypothesis prediction

CE10.1:

10.1/10.2: Multi-hypothesis prediction: No action – gains are too low (and became lower than before, cut by half or even more) to justify additional complexity

In the context of reviewing 10.1.3.x proposals, which target simplifications of CIIP, the following aspects were identified:

- does it need a specific MPM derivation? If there was only one mode (as in 10.1.3a/d) it is not needed at all, and it is mentioned that there are proposals just suggesting fixed length coding

- CIIP does not have any serious latency issue. Therefore, simplifications that remove some processing steps that are otherwise used in intra prediction is not necessary.

- Sample-wise unequal weighting is not an implementation issue, whereas equal weighting would be preferrable, unless it costs compression performance or causes qualty problems.

Beyond the solutions tested in 10.1.3 more study is necessary on these aspects.

CE10.2: OBMC

Among the four proposals, version 10.2.1 seems the only one which is manageable from complexity perspective (but definitely adds some complexity). The test results are summarized as follows:

| # | Proposal | Config. |  |  | VTM |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Y | U | V | EncT | DecT |
| CE10.2.1 | JVET-M0178 | RA | -0.27% | -0.58% | -0.62% | 104% | 103% |
|  |  | LB | -0.36% | -0.50% | -0.51% | 106% | 105% |

Track B initially suggested adoption of JVET-M0178.

During the JVET plenary, it was questioned whether such small gain was justifying the additional complexity. It is commented by several experts that OBMC may have positive impact on subjective quality. However, not such proof was available (and probably difficult to get). The decision was reverted.

CE on the previous investgated technologies 10.2.1…10.2.4 should be closed.

CE10.3: Multiple shape prediction partitions: No action

CE10.4: Diffusion filters for intra and inter prediction

new version uses FIR filters, applied on the prediction signal, switchable, not iterative. Two different 1D filters are 9-tap, symmetric, only 1 multiplication, otherwise shifts. One 2D filter is a 5-tap diamond shape, only shift/add operations. At the boundaries, one sample from neighbored reconstruction is used. The approach provides gains of 0.17%/0.42%/0.17%/0.87% for AI/RA/LB/LP. The method has been brought down to acceptable complexity impact for decoder, at some penalty in performance.

Two concerns are raised:

- As it needs to be run after the prediction signal is generated, it produces additional delay in the intra prediction loop.

- For inter blocks, it can be used for 128x128 CU, which would break concepts of 64x64VPDU.

It was requested to provide additional results without using the method in intra prediction (and the intra part of CIIP), and restrict the largest CU size to 64x64.

It is also reported that a late contribution (JVET-M0848) provides new results for the same method of 10.4.2 with some (small) encoder speedup and slightly increased performance.

Furthermore, it would be desirable to use only the prediction samples (no reconstructed samples from current picture). A version which did that was shown in the previous CE.

No decision yet on this.

CE10.5: Local illumination compensation

No action on current technologies, but continuation of CE to fulfill dependency/pipeline latency requirements.

CE11: Deblocking

Sub-tests

1) long-tap deblocking filters (11.1)

2) deblocking at 4x4 block boundaries (11.2).

The proposals were encoded according to two test conditions, which correspond to two anchors.

Anchor-1 is VTM-3.0 according to the CTC [1].

Anchor-2 is VTM-3.0 with ALF switched off (other conditions are the same as in Anchor 2).

CE11.1

Technology-wise understood and complexity-wise analysed, with some additional information on complexity requested.

It was pointed out that some of the proposals 11.1.1-5 did not consider that VTM3 now uses subblock boundary deblocking, whereas the decision of using long filters is based only on CU boundary properties. Therefore, it could happen that a long filter is applied first, and afterwards a subblock boundary within the CU is deblocked again. This would inhibit parallelism. This could be classified as a bug, which however in terms of subjective testing might not be too harmful.

CE11.2

VTM3 operates deblocking on an 8x8 grid. If a CU boundary is not on an 8x8 grid, it is not deblocked. If a CU is on an 8x8 grid, furthermore subblocks within that CU are deblocked (provided they are on the 8x8 grid as well). 4xN blocks are deblocked whenever they are coinciding with the 8x8 grid

11.2.1 deblocks on a 4x4 grid, where a boundary is deblocked with VTM filter when the next block or subblock is more than four samples apart, and with a weak and short filter if it is only samples apart.

11.2.2 uses an 8x8 grid and VTM deblocking filter but disables deblocking whenever the boundary would be only four samples apart.

Viewing in CE11 to assess necessity and benefit still to be done (being prepared).

General remarks on late delivery of text – text needs to be provided.

## Closing plenary sessions

## Joint meetings

## BoGs (9)

JVET-M0782 BoG report on tiles and WPP [Y.-K. Wang, M. M. Hannuksela]

This report was discussed in Track A Monday 14 January 1430-1630, 1830-2000 (chaired by GJS)

This contribution provides the report of the BoG on tiles and WPP.

The BoG recommended the following adoptions:

- Adopt JVET-M00853-v2 and add a NOTE like the following:

NOTE: When extracting an MCTS to form a conforming bitstream, active PPSs in the extracted sub-bitstream should have signalled_tile_group_id_flag equal to 1.

JVET-M00853-v2 adds the support of rectangular tile groups in addition to the existing raster scan ordered tile groups, and enables extraction of MCTSs without changing VLC NAL units.

Decision: Adopted, with constraints as follows:

- the tiles in a tile group need to be in raster-scan order.

- the tile groups need to be in increasing address order (see below: dark orange, then light blue, then pale yellow, then dark blue, then light orange, …)

- The tile group shapes shall be such that each tile, when decoded, shall have its entire left boundary and entire top boundary consisting of a picture boundary or previously decoded tile(s). In other words, the light yellow and light green tiles in the figure below are not allowed.

The following tile group picture was drawn for illustration and discussion purposes.

The BoG recommended to adopt the software implementation in JVET-M0445 into the VTM. Decision (SW): Adopted.

The the software implementation in JVET-M0445 implements encoder motion constraints for MCTSs.

- Add loop_filter_across_tile_group_enabled_flag to the PPS, with syntax as proposed in M0160, and semantics as follows:

loop_filter_across_tile_groups_enabled_flag equal to 1 specifies that in-loop filtering operations may be performed across tile group boundaries in pictures referring to the PPS. loop_filter_across_tile_group_enabled_flag equal to 0 specifies that in-loop filtering operations are not performed across tile group boundaries in pictures referring to the PPS. The in-loop filtering operations include the deblocking filter, sample adaptive offset filter, and adaptive loop filter operations. When not present, the value of loop_filter_across_tile_groups_enabled_flag is inferred to be equal to 0.

- Add an AHG mandate on WPP into AHG 12. This was agreed in Track A.

The BoG and Track A agreed that it would be desirable to have WPP support in VVC. The syntax, semantics, and decoding process for WPP are for further study.

The BoG recommended the following to be discussed:

- Whether to allow tiling with a tile width or tile height that is not a multiple of the CTU size

The late document M0875 was discussed – see notes on that contribution.

JVET-M0527 raised some concerns related to hardware implementation. And it was commented that in addition to the issues mentioned in JVET-M0527, this design also causes issues with line buffer ing, as well as in VDPU rate.

It was commented that a benefit of the design is enable turning off loop filtering across CMP faces. It was suggested that enabling loop filtering turning off within a tile would be a lighter solution than having this design. It was commented that there is ongoing CE on this.

Discussion resumed 1920 Monday 14 January (GJS).

It was thus suggested that these two topics should be discussed and decided together.

- The latest HEVC specification specifies, as part of the semantics of an SEI message, the MCTS sub-bitstream extraction process and requires the extracted sub-bitstream to be conforming bitstream. For VVC, should we do more than this way of defining conformance for MCTSs? For example, should normative decoding process be specified for decoding of MCTSs instead of just specifying SEI messages for indication of the encoder motion constraints? There were several related proposals. For example, whether MCTS sequences are signalled in parameter sets or whether an MCTS sequence would have its own associated parameter sets. Whether to provide a level definition at the MCTS sequence level. Whether to define a normative decoder interface and conforming decoding behaviour for MCTS-specific decoding. Further study is highly encouraged.

- On whether to (allow) treat boundaries of MCTS / sub-picture sequences as picture boundaries (i.e., to do padding).

Allowing this is expected to help in coding efficiency. However, it would impose some burden for hardware implementations. Tests showing coding efficiency gain numbers and an analysis on hardware implementation complexity are needed for making a decision on this. Note that JVET-M0445 provides a software implementation that could be used as the basis and anchor for such coding efficiency comparison.

Interested parties were requested to work offline to prepare the test conditions. This effort was coordinated by M. Coban, the proposed test conditions are included in JVET-M0870.

It was thus suggested that JVET-M0870 is reviewed.

- Whether an interface for inputting/outputting reference pictures to/from the decoder could be acceptable in decoder implementation architectures.

This relates to allowing empty tiles and applying geometry padding to fill in the empty tiles in an external process as proposed in JVET-M0547. The contribution was also discussed in the 360o video BoG (see that BoG report).

It was commented that there could be other ways to indicate uncoded areas in the picture than using empty tiles.

The contribution would need an interface of outputting reference pictures (with uncoded areas) from the decoder to an external process and inputting reference pictures (with uncoded areas filled in by geometry padding) to the decoder from an external process. It was commented that this has some similarity to the external base layer concept of SHVC.

The bitstream would have some tiles that are not coded. The PPS could indicate where the uncoded areas are.

It was suggested that the tested benefit for cubeface geometry padding was about 2%. The subjective benefit may be greater. The anticipated benefit is rather limited, perhaps too small to be of interest by itself unless there is some other motivation for establishing a similar functionality. There have been other proposals for considering external management of reference picture memory and external control of the decoding process below the picture level. If there is a study of that broader potential need for a different system interface to the video decoding process, this could be part of it.

JVET-M0816 BoG report on high level syntax [J. Boyce]

This BoG report was reviewed in Track A Sunday 13 January 1530 (GJS).

The BoG suggest discussion in the track or plenary regarding:

- Ask the following questions to ALF experts, for consideration when evaluating proposed solutions to improve coding efficiency of tile group headers:

- How much bitrate do ALF parameters typically require? Perhaps typically about 200 bits (4 luma and 1 chroma), could be up to perhaps 2k bits

- Do we expect that multiple pictures would use the same ALF parameters? 0.5% for RA in M0429. It was said to be undesirable to put them in the PPS since they change frequently.

- What is the expected coding gain when ALF is used?

- Does the ALF design support having different ALF parameters in different tile groups in the same picture? This hasn’t been tested but seems necessary to support having tiles that were generated by different encoders, and potentially desirable for other reasons as well.

- If it is allowed to have different ALF parameters in different tile groups in the same picture, do we expect that they would frequently differ? See previous item. It hasn’t been tried.

The BoG met on 11 Jan 2019. The BoG also met on 12 Jan 2019, with the notes reflected in the -v2 version of the report document.

Decision: The following BoG recommendations:

- Replace the existing IRAP_NUT with 3 new NAL unit types: IDR_W_RADL, IDR_N_LP, CRA_NUT (from JVET-M0101).

- Add external means flag HandleCraAsCvsStartFlag, with similar text as in HEVC. Text provided in a v3 of JVET-M0101.

- Add a NUT value for STSA (from JVET-M0101).

- Add a NUT value for AUD (from JVET-M0101).

- Add sps_max_sub_layers_minus1 syntax element to SPS, and decoding process in 8.1.1, 8.1.2 and 8.1.3 of JVET-M0101.

- Add text of sections 7.4.2.4 to 7.4.2.4.5 on NAL unit order and AU boundary detection from JVET-M0101, which is primarily editorial, but has some technical aspects.

- Add profile_tier_level( ) syntax structure which includes sub layer level idc (similar to HEVC but without sub-layer-specific profiles).

- Add general_non_packed_constraint_flag with semantics as in JVET-M0101 (rename the flag to display_suitability_flag? – that’s editorial).

- Add the temporal scalability sub-bitstream extraction process in JVET-M0101.

- Change the sps_ref_wraparound_offset to sps_ref_wraparound_offset_minus1 and changing the units to be MinCbSizeY as in option 1, subject to review by the 360° BoG (minor cleanup).

- Add 7 new constraint flags corresponding to VVC WD 3 tools as described in M0451.

- Add reference picture signalling from M0128 (basic text version) – modified after Track A discussion as described in the notes for that document.

The BoG suggested discussion in the track or plenary regarding:

- Consider between adding 2 NUT values for RASL and RADL leading pictures or staying with no dedicated NUT values for leading pictures. Decision: Add RASL and RADL NUTs.

Further review of the BoG results was conducted Monday 14 January 1330 (GJS)

The BoG suggested discussion in the track or plenary regarding:

- Consider whether profile_space should be included. This, and profile compatibility flags, were agreed to be for further study.

- Consider removing concept of I, P, and B slice (tile group) types, but to have flags to control the types of prediction (intra prediction, current picture referencing, inter prediction with one list or two lists), and whether this signaling should be for pictures or for tile groups. It was commented that it may not be necessary to indicate at the tile group level whether CPR is used in the tile or not. (Note that a tile has no header.) No need for action on this was identified.

- Consider whether an AHG should be formed to study solutions to problem of temporal judder when temporal scalability sub-bitstream extraction is used to lower frame rate. Consider whether this should have a normative impact or be supported in an SEI message (which could perhaps be done in JCT-VC for HEVC or AVC). Such a study could include consideration of pre- and post-processing. See JVET-M0579. It was suggested to establish an AHG on layered-coding scalability and resolution adaptation, and to include this topic in its mandates.

- Consider whether there is interest to normatively specify Gradual Decoder Refresh (GDR) operation, such that a conforming CVS can start with a GDR non-IRAP picture (as proposed in JVET-M0529). Further study in an AHG was encouraged. Some issues for study include how HRD interacts with the use case and whether it would be a burden to decoders to be required to support this capability.

JVET-M0843 BoG report on CE4 related contributions [K. Zhang]

(Track B)

JVET-M0857 BoG report on intra prediction and mode coding (CE3-related) [G. Van der Auwera]

(Track A)

JVET-M0858 BoG report on CE9 related contributions [S. Esenlik]

(Track B)

JVET-M0862 BoG report on CE2 related contributions [Y. He]

(Track B)

JVET-M0873 BoG report on CE10 related contributions [C.-W. Hsu, M. Winken]

(Track B)

JVET-M0874 BoG report on CE13 and CE13 related 360° video coding [J. Boyce]

(Track A)

JVET-M0877 BoG report on CE6 related contributions [X. Zhao (Tencent] [late]

(Track A)

## List of actions taken affecting Draft 3 of VVC, VTM 3, and 360Lib

The following is a summary, in the form of a brief list, of the actions taken at the meeting that affect the text of the VVC draft text, VTM or 360Lib description. Both technical and editorial issues are included. This list is provided only as a summary – details of specific actions are noted elsewhere in this report and the list provided here may not be complete and correct. The listing of a document number only indicates that the document is related, not that it was adopted in whole or in part.

# Project planning

## Core experiment planning

…

## Drafting of specification text, encoder algorithm descriptions, and software

The following agreement has been established: the editorial team has the discretion to not integrate recorded adoptions for which the available text is grossly inadequate (and cannot be fixed with a reasonable degree of effort), if such a situation hypothetically arises. In such an event, the text would record the intent expressed by the committee without including a full integration of the available inadequate text.

## Plans for improved efficiency and contribution consideration

The group considered it important to have the full design of proposals documented to enable proper study.

Adoptions need to be based on properly drafted working draft text (on normative elements) and HM encoder algorithm descriptions – relative to the existing drafts. Proposal contributions should also provide a software implementation (or at least such software should be made available for study and testing by other participants at the meeting, and software must be made available to cross-checkers in EEs).

Suggestions for future meetings included the following generally-supported principles:

- No review of normative contributions without draft specification text

- VTM algorithm description text is strongly encouraged for non-normative contributions

- Early upload deadline to enable substantial study prior to the meeting

- Using a clock timer to ensure efficient proposal presentations (5 min) and discussions

The document upload deadline for the next meeting was planned to be Wednesday 2 January 2019 (a little later than usual due to the proximity of the New Year holiday).

As general guidance, it was suggested to avoid usage of company names in document titles, software modules etc., and not to describe a technology by using a company name.

## General issues for experiments

It was emphasized during the opening plenary on January 9 that those rules which had been set up or refined during the 12th meeting should be observed. In particular, for some CEs, results were available late, and some changes in the experimental setup (particularly in CE4) were not discussed on the JVET reflector.

Group coordinated experiments have been planned as follows:

- “Core experiments” (CEs) are the coordinated experiments on coding tools which are deemed to be interesting but require more investigation and could potentially become part of the draft standard by the next meeting.

- A CE is a test of a specific fully described technology in a specific agreed way. It is not a forum for thinking of new ideas (like an AHG).

- A description of each experiment is to be approved at the meeting at which the experiment plan is established. This should include the issues that were raised by other experts when the tool was presented, e.g., interference with other tools, contribution of different elements that are part of a package, etc. The experiment description document should provide the names of individual people, not just company names.

- Software for tools investigated in a CE will be provided in one or more separate branches of the software repository. Each CE will have a “fork” of the software, and within the CE there may be multiple branches established by the CE coordinator. The software coordinator will help coordinate the creation of these forks and branches and their naming. All JVET members will have read access to the CE software branches (using shared read-only credentials; the method for members to obtain the credentials is TBA on the reflector).

- During the experiment, revisions of the experiment plans can be made, but not substantial changes to the proposed technology.

- The CE description must match the CE testing that is done. The CE description needs to be revised if there has been some change of plans.

- The CE summary report must describe any changes that were made in the process of finalizing the CE.

- By the next meeting it is expected that at least one independent cross-checker will report a detailed analysis of each proposed feature that has been tested and confirm that the implementation is correct. Commentary on the potential benefits and disadvantages of the proposed technology in cross-checking reports is highly encouraged. Having multiple cross-checking reports is also highly encouraged (especially if the cross-checking involves more than confirmation of correct test results). The reports of cross-checking activities may (and generally should) be integrated into the CE report rather than submitted as separate documents.

It is possible to define sub-experiments within particular CEs, for example designated as CEX.a, CEX.b, etc., where X is the basic CE number.

As a general rule, it was agreed that each CE should be run under the same testing conditions using one software codebase, which should be based on the group test model software codebase. An experiment is not to be established as a CE unless there is access given to the participants in (any part of) the CE to the software used to perform the experiments.

The general agreed common conditions for single-layer coding efficiency experiments are described in the output document JVET-J1010 (update).

Experiment descriptions should be written in a way such that it is understood as a JVET output document (written from an objective “third party perspective”, not a proponent perspective – e.g. not referring to methods as “improved”, “optimized”, etc.). The experiment descriptions should generally not express opinions or suggest conclusions – rather, they should just describe what technology will be tested, how it will be tested, who will participate, etc. Responsibilities for contributions to CE work should identify individuals in addition to company names.

CE descriptions contain a basic description of the technology under test, but should not contain excessively verbose descriptions of a technology (at least not unless the technology is not adequately documented elsewhere). Instead, the CE descriptions should refer to the relevant proposal contributions for any necessary further detail. However, the complete detail of what technology will be tested must be available – either in the CE description itself or in documents that are referenced in the CE description that are also available in the JVET document archive.

Any technology must have at least one cross-check partner to establish a CE – a single proponent is not enough. It is highly desirable have more than just one proponent and one cross-checker.

Some agreements relating to CE activities were established as follows:

- Only qualified JVET members can participate in a CE.

- Participation in a CE is possible without a commitment of submitting an input document to the next meeting. Participation is requested by contacting the CE coordinator.

- All software, results, and documents produced in the CE should be announced and made available to JVET in a timely manner.

- All substantial communications about a CE, other than logistics arrangements, exchange of data, minor refinement of the test plans, and preparation of documents shall be conducted on the main JVET reflector. In the case that large amounts of data are to be distributed is recommended to send an announcement to the JVET reflector without attaching the materials, and send the materials to those who have requested it directly, or provide a link to it, or upload the data as an input contribution to the next meeting.

General timeline for CEs

T1= 3 weeks after the JVET meeting: To revise the CE description and refine questions to be answered. Questions should be discussed and agreed on JVET reflector. Any changes of planned tests after this time need to be announced and discussed on the JVET reflector.

T2 = Test model SW release + 2 weeks: Integration of all tools into a separate CE branch of the VTM is completed and announced to JVET reflector.

- Initial study by cross-checkers can begin.

- Proponents may continue to modify the software in this branch until T3

- 3rd parties encouraged to study and make contributions to the next meeting with proposed changes

T3: 3 weeks before the next JVET meeting: Any changes to the CE test branches of the software must be frozen, so the cross-checkers can know exactly what they are cross-checking. A software version tag should be created at this time and announced on the JVET reflector. The name of the cross-checkers and list of specific tests for each tool under study in the CE plan description by this time. Full test results must be provided at this time (at least for proposals targeting to be promoted to the draft standard at the next meeting).

CE reports may contain additional information about tests of straightforwared combinations of the identified technologies. Such supplemental testing needs to be clearly identified in the report if it was not part of the CE plan.

New branches may be created which combine two or more tools included in the CE document or the VTM (as applicable). [Search/remove obsolete references to BMS.]

It is not necessary to formally name cross-checkers in the initial version of the CE description document. To adopt a proposed feature at the next meeting, we would like see comprehensive cross-checking done, with analysis that the description matches the software, and recommendation of value of the tool given tradeoffs.

The establishment of a CE does not indicate that a proposed technology is mature for adoption or that the testing conducted in the CE is fully adequate for assessing the merits of the technology, and a favourable outcome of CE does not indicate a need for adoption of the technology.

Draft specification text shall be provided with CE input documents.

It was later discussed (Track B Wednesday 9 January afternoon) that it was unclear by which time such spec text should be available. It was mentioned that numerous (if not majority) of current CE contribution documents in the first uploaded version come without spec text. However, availability of spec text is important to have a detailed understanding of the technology and also to judge what its impact on the complexity of the spec will be. Revisit: To be raised in plenary.

CE plans were initially reviewed Thursday 11 Oct 1630 (GJS) and 1800 (J. Boyce); the final review during the meeting was conducted Friday 12 Oct 1100 (GJS).

Lists of participants in CE documents should be pruned to include only the active participants. Read access to software will be available to all members.

## Software development and anchor generation (update)

The planned timeline for software releases was established as follows:

- VTM3.0 will be released by 2018-11-09. VTM3.1 with non-CTC adoptions will be released later.

- Further versions of VTM may be released for additional bug fixing, as appropriate.

- Timeline of 360lib8.0: 1 week after the release of VTM3.0 (2018-11-16). Further versions may be released as appropriate for bug fixing.

# Establishment of ad hoc groups

The ad hoc groups established to progress work on particular subject areas until the next meeting are described in the table below. The discussion list for all of these ad hoc groups was agreed to be the main JVET reflector (jvet@lists.rwth-aachen.de).

| Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| Project Management (AHG1) (jvet@lists.rwth-aachen.de) - Coordinate overall JVET interim efforts. - Supervise CE and AHG studies. - Report on project status to JVET reflector. - Provide a report to next meeting on project coordination status. | J.-R. Ohm, G. J. Sullivan (co-chairs) | N |
| Draft text and test model algorithm description editing (AHG2) (jvet@lists.rwth-aachen.de) - Produce and finalize JVET-L1001 VVC text specification Working Draft 3. - Produce and finalize JVET-L1002 VVC Test Model 3 (VTM 3) Algorithm and Encoder Description. - Gather and address comments for refinement of these documents. - Coordinate with test model software development AhG to address issues relating to mismatches between software and text. | B. Bross, J. Chen (co-chairs), J. Boyce, S. Kim, S. Liu, Y. Ye (vice-chairs) | N |
| Test model software development (AHG3) (jvet@lists.rwth-aachen.de) - Coordinate development of test model (VTM) software and associated configuration files. - Produce documentation of software usage for distribution with the software. - Discuss and make recommendations on the software development process. - Propose improvements to the guideline document for developments of the test model software. - Coordinate with AHG on Draft text and test model algorithm description editing (AHG2) to identify any mismatches between software and text, and make further updates and cleanups to the software as appropriate. - Coordinate with AHG6 for integration with 360lib software. | F. Bossen, X. Li, K. Sühring (co-chairs) | N |
| Test material and visual assessment (AHG4) (jvet@lists.rwth-aachen.de) - Maintain the video sequence test material database for development of the VVC standard. - Identify and recommend appropriate test materials for use in the development of the VVC standard. - Identify missing types of video material, solicit contributions, collect, and make available a variety of video sequence test material. - Evaluate new test sequences, particularly including the material recently submitted by the Blender Foundation / Blender Animation Studio and Twitch. - Propose a new structure for the test sequence repository. - Facilitate availability of viewing equipment and facilities arrangements for the next meeting and pre-meeting testing as feasible. | T. Suzuki (chair), V. Baroncini, R. Chernyak, P. Hanhart, A. Norkin, J. Ye (vice-chairs) | N |
| Memory bandwidth consumption of coding tools (AHG5) (jvet@lists.rwth-aachen.de) - Develop improved software tools for measuring both average and worst case of memory bandwidth, and provide information for usage of these tools. - Study cache configurations for measuring decoder memory bandwidth consumption. - Identify coding tools in CEs and VTM with significant memory bandwidth impact. - Study the impact of memory bandwidth on specific application cases. | R. Hashimoto (chair), T. Ikai, X. Li, D. Luo, H. Yang, M. Zhou (vice-chairs) | N |
| 360° video conversion software development (AHG6) (jvet@lists.rwth-aachen.de) - Prepare and deliver the 360Lib-8.0 software version and common test condition configuration files according to JVET-L1012. - Generate CTC (PHEC) anchors and PERP results for VTM according to JVET-L1012, and finalize the reporting template for the common test conditions. - Produce documentation of software usage for distribution with the software. | Y. He, K. Choi (co-chairs) | N |
| Coding of HDR/WCG material (AHG7) (jvet@lists.rwth-aachen.de) - Study and evaluate available HDR/WCG test content. - Study objective metrics for quality assessment of HDR/WCG material, including investigation of the correlation between subjective and objective results of the CfP responses. - Compare the performance of the VTM and HM for HDR/WCG content. - Prepare for expert viewing of HDR content at the 13th JVET meeting if feasible. - If feasible, arrange a demonstration event for viewing of JVET-L0205 and JVET-L0245 coded material and possibly other material. - Coordinate implementation of HDR anchor aspects in the test model software with AHG3. - Study additional aspects of coding HDR/WCG content. | A. Segall (chair), E. François, W. Husak, D. Rusanovskyy (vice-chairs) | Y. Date TBA (likely in Burbank US, or DE, FR, or UK) |
| 360° video coding tools and test conditions (AHG8) (jvet@lists.rwth-aachen.de) - Study the effect on compression and subjective quality of different projections formats, resolutions, and packing layouts. - Discuss refinements of common test conditions, test sequences, and evaluation criteria. - Solicit additional test sequences, and evaluate suitability of test sequences on head-mounted displays and normal 2D displays. - Study coding tools dedicated to 360° video, their impact on compression, and implications to the core codec design. - Study the effect of viewport resolution, field of view, and viewport speed/direction on visual comfort. - Study complexity of GPU rendering of projection formats - Study syntax for signalling of projection formats | J. Boyce (chair), K. Choi, P. Hanhart, J.-L. Lin (vice-chairs) | N |
| Neural networks in video coding (AHG9) (jvet@lists.rwth-aachen.de) - Investigate the benefit of using neural networks in video compression such as CNN loop filter, intra prediction, re-sampling in adaptive resolution coding, and encoder side partition mode decisions. - Investigate the complexity impact of using neural networks in video compression. - Investigate the complexity measurement of neural network coding tools. - Investigate the impact of training materials on the performance of neural network coding tools. - Investigate the impact of the training process on performance and complexity. | S. Liu (chair), B. Choi, K. Kawamura, Y. Li, L. Wang, P. Wu, H. Yang (vice-chairs) | N |
| Encoding algorithm optimization (AHG10) (jvet@lists.rwth-aachen.de) - Study the impact of using techniques such as GOP structures and perceptually optimized adaptive quantization for encoder optimization. - Study the impact of MTS transforms on quantization matrices and the need for default matrices. - Study the impact of adaptive quantization on individual tools in the test model. - Study the quantization adaptation tool in the test model. - Investigate the feasibility of adding a CTC test category in which adaptive quantization is turned on. - Study quality metrics for measuring subjective quality using e.g. the CfP response MOS scores. - Investigate other methods of improving objective and/or subjective quality, including adaptive coding structures, adaptive quantization without signalling, and multi-pass encoding. - Study methods of rate control and their impact on performance, subjective and objective quality. | A. Duenas, A. Tourapis (co-chairs), C. Helmrich, S. Ikonin, A. Norkin, R. Sjöberg, T. Toma (vice-chairs) | N |
| Screen content coding (AHG11) (jvet@lists.rwth-aachen.de) - Investigate coding tools targeted at screen content in terms of compression benefit and implementation complexity. - Identify test materials and discuss testing conditions for screen content coding. | S. Liu (chair), J. Boyce, A. Filippov, Y.-C. Sun, J. Xu, M. Zhou (vice-chairs) | N |
| High-level parallelism and coded picture regions (AHG12) (jvet@lists.rwth-aachen.de) - Study tile group designs, including rectangular tile groups. - Study flexible (including non-CTU-aligned) tile partitioning including identifying implications on coding tools and implementation. - Study support of independently coded picture regions, including easy rewriting of such regions into a conforming sub-bitstream. - Prepare software and configurations for the test model to facilitate parallel processing tests. - Study the coding efficiency impact of parallel processing and coded picture regions. | T. Ikai (chair), M. M. Hannuksela, R. Sjöberg, R. Skupin, W. Wan, Y.-K. Wang S. Wenger (vice-chairs) | N |
| Tool reporting procedure (AHG13) (jvet@lists.rwth-aachen.de) - Prepare output document JVET-L1005, which describes the methodology of tool-off testing and a list of tools to be tested by identified testers. - Provide configurations files, bitstreams, and results of the tool-on/tool-off testing. - Use the tool usage counts and memory bandwidth usage to study the decoder complexity of features in on/off testing. - Prepare a report with results of the tests. | W.-J. Chien, J. Boyce (co-chairs), R. Chernyak, K. Choi, R. Hashimoto, Y.-W. Huang, S. Liu, D. Luo (vice-chairs) | N |
| Progressive intra refresh (AHG14) (jvet@lists.rwth-aachen.de) - Define relevant test conditions to evaluate low-latency encoding with progressive intra refresh for random access without intra frames. - Study non-normative ways to produce progressive intra refresh with minimum losses in coding efficiency. - Propose software modifications for integrating encoder-only intra refresh in the VTM model. - Characterize progressive intra refresh performance objectively and subjectively. - Study normative solutions to improve intra refresh performance against encoder-only intra refresh. | J.-M. Thiesse (chair), A. Duenas, K. Kazui, A. Tourapis (vice-chairs) | N |
| Bitstream decoding properties signalling (AHG15) (jvet@lists.rwth-aachen.de) - Study syntax alternatives for interoperability point signalling - Study selection of constraint flags to be included in the VTM and their impact on syntax, semantics, and decoding process | J. Boyce (chair), J. Chen, S. Deshpande, M. Karczewicz, A. Tourapis, Y.-K. Wang, S. Wenger (vice-chairs) | Tel. TBA (one or two, at least two weeks notice for each) |
| Implementation studies (AHG16) (jvet@lists.rwth-aachen.de) - Study draft and proposed coding tools to identify implementation issues relating to decoder pipelines, decoder throughput, and other aspects of implementation difficulty. - Solicit hardware analysis of complex tools. - Particularly consider intra reconstruction throughput for small blocks. - Provide feedback on potential solutions to address identified issues. | M. Zhou (chair), J. An, E. Chai, K. Choi, S. Ethuraman, T. Hsieh, X. Xiu (vice-chairs) | N |
| High-level syntax (AHG17) (jvet@lists.rwth-aachen.de) - Study NAL unit header, sequence parameter set, picture parameter set, and tile group header syntax designs - Study the proposed picture header designs and alternatives - Study reference picture buffering and list construction - Study random access signalling - Assist in software development and text drafting for the high-level syntax in the VVC design. | R. Sjöberg (chair), S. Deshpande, M. M. Hannuksela, R. Skupin, Y.-K. Wang, S. Wenger (vice-chairs) | N |

# Output documents

The following documents were agreed to be produced or endorsed as outputs of the meeting. Names recorded below indicate the editors responsible for the document production. Where applicable, dates of planned finalization and corresponding parent-body document numbers are also noted.

It was reminded that in cases where the JVET document is also made available as MPEG output document, a separate version under the MPEG document header should be generated. This version should be sent to GJS and JRO for upload.

Add CE for MCTS M0870.

JVET-L1000 Meeting Report of the 12th JVET Meeting [G. J. Sullivan, J.-R. Ohm] (2018-12-31, near next meeting)

Initial versions of the meeting notes (d0 … d8) were made available on a daily basis during the meeting.

JVET-L1001 Versatile Video Coding (Draft 3) [B. Bross, J. Chen, S. Liu] [WG 11 N 18027] (2018-11-23)

(Initial version planned to be made available by 2018-11-02.)

See the list of elements under section 12.6.2, as agreed by the Wed. 18 October plenary.

JVET-L1002 Algorithm description for Versatile Video Coding and Test Model 3 (VTM 3) [J. Chen, Y. Ye, S. Kim] [WG 11 N 18028] (2018-12-14)

(Initial version planned to be made available by 2018-11-09.)

See the list of elements under section 12.6.2, as agreed by the Wed. 18 October plenary.

Remains valid – not updated: JVET-K1003 Guidelines for VVC reference software development [K. Sühring] (2018-07-31)

New version?

JVET-L1004 Algorithm descriptions of projection format conversion and video quality metrics in 360Lib (Version 8) [Y. Ye, J. Boyce] (2018-11-23)

For this output, it was agreed to add discussion of chroma location.

JVET-L1005 Methodology and reporting template for coding tool testing [W.-J. Chien and J. Boyce] (2018-10-26)

For this output, it was agreed to remove discussion of BMS, update for tools adopted at the 12th meeting, and update the schedule.

JVET-L1006 Methodology and reporting template for neural network coding tool testing [Y. Li, S. Liu] (2018-10-26)

This output was produce to capture aspects specific to enable study of neural network techniques.

JVET-L1010 JVET common test conditions and software reference configurations for SDR video [F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring] (2018-10-26)

For this output, it was agreed to remove discussion of BMS and to make Class F testing mandatory (but not included in the reported averages).

JVET-L1011 JVET common test conditions and evaluation procedures for HDR/WCG video [A. Segall, E. François, S. Iwamura, D. Rusanovskyy] (2018-10-26)

A correction was needed.

JVET-L1012 JVET common test conditions and evaluation procedures for 360° video [P. Hanhart, J. Boyce, K. Choi, J.-L. Lin] (2018-10-26)

Aspects include to enable wrap-around MVs for ERP, andupdating to use PHEC as an anchor.

JVET-L1021 Description of Core Experiment 1 (CE 1): Partitioning [J. Ma, F. Le Léannec, M. W. Park]

This CE was planned to study L0313 and L0128, which are ways to deal with the intent to enable 64x64 pipeline decoding operation, trying to improve the coding efficiency relative to the current approach.

In the closing review on Friday it was commented that part of L0313 is less consistent with the 64x64 pipeline goal than L0128, and could be more difficult to implement on some hardware architectures. It was agreed to only include the rectangular variant of L0313, not the L-shaped variant, in the testing.

JVET-L1022 Description of Core Experiment 2 (CE2): Subblock motion compensation [Y. He, C.-Y. Chen, C.-C. Chen]

This CE was planned to study affine motion, planar MV prediction, subblocks for ATMVP, and related constraints.

It was noted that the draft CE plan contained a large number of subtests, and it was requested for the number to be reduced during the CE plan finalization. It was noted that part of the issue was that the plan included several combinations.

A need for coordination between CE2 and CE4 was identified, as there are some overlapping aspects.

JVET-L1023 Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding [G. Van der Auwera, J. Heo, A. Filippov]

This CE was planned to study about 11 contributed methods relating to intra prediction and intra mode coding.

JVET-L1024 Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding [H. Yang, S. Liu, K. Zhang]

This CE was planned to study list derivation, merge, motion vector coding, and inter-prediction-related constraints.

A need for coordination between CE2 and CE4 was identified, as there are some overlapping aspects.

It was asked whether non-adjacent merge candidates should be included in the CE, due to complexity concerns. A proponent said that some versions of this does not require extra memory (either with 8x8 grid or within the current 64x64 region), and suggested to study these versions. The CE finalization will consider this and may remove some subtests.

It was suggested to include some testing of using longer lists of merge candidates. Just increasing the list size of VTM 3 merge mode (currently 6) was suggested to be used as another anchor.

JVET-L1025 Description of Core Experiment 5 (CE5): Arithmetic Coding Engine [H. Kirchhoffer, A. Said]

This CE was planned to study alternative arithmetic coding engines and customized window sizes.

It was commented that including the bypass bins in the throughput testing seemed unnecessary and complicates the model. The bypass processing is the same for all cases. It was agreed to remove that aspect.

JVET-L1026 Description of Core Experiment 6 (CE6): Transforms and transform signalling [A. Said, X. Zhao]

This CE was planned to study transform core designs, fast transform factorizations, transform signalling, subblock transforms, and secondary transforms.

It was noted that the draft CE plan contained a large number of subtests, and it was requested for the number to be reduced during the CE plan finalization. It was noted that part of the issue was that the plan included several combinations.

JVET-L1027 Description of Core Experiment 7 (CE 7): Quantization and coefficient coding [H. Schwarz, M. Coban, C. Auyeung]

This CE was planned to study reducing the number of context coded bins in coefficient coding, and reducing the number of passes.

JVET-L1028 Description of Core Experiment 8 (CE8): Screen Content Coding Tools [X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu]

This CE was planned to study CPR modifications, palette mode modifications, and block-based DPCM.

JVET-L1029 Description of Core Experiment 9 (CE9): Decoder-Side Motion Vector Derivation [X. Xiu, S. Esenlik]

This CE was planned to study bidirectional optical flow, bilateral matching and template based matching techniques for DMVR.

JVET-L1030 Description of Core Experiment 10 (CE10): Combined and multi-hypothesis prediction [C.-W. Hsu, M. Winken, X. Xiu]

This CE was planned to study OBMC, multiple shape partitions, prediction with more than two hypotheses, local illumination compensation, diffusion filter, and other blending of multiple predictors (if any).

JVET-L1031 Description of Core Experiment 11 (CE11): Deblocking [A. Norkin, A. M. Kotra]

This CE was planned to study longer tap-length filters and deblocking on a 4x4 grid.

The number of actual tests was planned to be reduced, relative to the initial presented version.

JVET-L1032 Description of Core Experiment 12 (CE12): Mapping functions [E. François, P. Yin]

This CE was planned to include in-loop mapping functions for SDR.

JVET-L1033 Description of Core Experiment 13 (CE13): Coding tools for 360° omnidirectional video [P. Hanhart, J.-L. Lin, C. Pujara]

This CE was planned to study changes to in-loop filters, inter prediction, post-filtering, projection rotations and packings.

# Future meeting plans, expressions of thanks, and closing of the meeting

Future meeting plans were established according to the following guidelines:

- Meeting under ITU-T SG 16 auspices when it meets (starting meetings on the Tuesday or Wednesday of the first week and closing it on the Tuesday or Wednesday of the second week of the SG 16 meeting – a total of 6–7.5 meeting days), and

- Otherwise meeting under ISO/IEC JTC 1/SC 29/WG 11 auspices when it meets (starting meetings on the Wednesday or Thursday prior to such meetings and closing it on the last day of the WG 11 meeting – a total of 8.5 meeting days).

In cases where high workload is expected for a meeting, an earlier starting date may be defined.

Some specific future meeting plans (to be confirmed) were established as follows:

- Tue. 19 – Wed. 27 March 2019, 14th meeting under ITU-T auspices in Geneva, CH.

- Wed. 3 – Fri. 12 July 2019, 15th meeting under WG 11 auspices in Gothenburg, SE.

- Tue. 1 – Wed. 9 October 2019, 16th meeting under ITU-T auspices in Geneva, CH.

- Wed. 8 – Fri. 17 January 2020, 17th meeting under WG 11 auspices in Brussels, BE.

The agreed document deadline for the 14th JVET meeting was planned to be XXday X March 2019. Plans for scheduling of agenda items within that meeting remained TBA.

WG 11, the local hosting organization XXXX, and the supporting organizations XXXX were thanked for the excellent hosting and organization of the 13th meeting of the JVET.

XXX was thanked for providing viewing equipment used during the 13th JVET meeting.

XXXX were thanked for providing additional test material for usage in standardization efforts.

The 13th JVET meeting was closed at approximately XXXX hours on Friday 18 January 2019.

# Annex A to JVET report:
List of documents

# Annex B to JVET report:
List of meeting participants

The participants of the thirteenth meeting of the JVET, according to a sign-in sheet circulated during the meeting sessions (approximately XXX people in total), were as follows: