| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0030-v1 |
| --- | --- |

| Title: | CE10: Summary report on combined and multi-hypothesis prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Chih-Wei Hsu Martin Winken | Email: | cw.hsu@mediatek.com martin.winken@hhi.fraunhofer.de |
| Source: | CE coordinators |  |  |

_____________________________

- Abstract

A summary of Core Experiment 10 (CE10) on combined and multi-hypothesis prediction is reported. Five sub CEs are created to test different methods of combined predictions, including:

- CE10.1: multi-hypothesis prediction,

- CE10.2: overlapped block motion compensation,

- CE10.3: multiple shape prediction partitions,

- CE10.4: diffusion filtering of inter- and intra-prediction signals,

- CE10.5: local illumination compensation.

There are 13, 4, 1, 2 and 2 tests for each sub CE, respectively. All tests are evaluated based on the common test conditions defined in JVET-L1010. All tests and crosscheck results are integrated in this report.

Summary of CE Activities

Schedule report

There are three CE deadlines targeted at different goal of check point, the activities happened during these sessions are summarized as follows,

- T1 (Nov 2, 2018) is the deadline for final CE description upload:

- No changes were requested after T1.

- The modifications after T1 only included the update of the cross-checker information.

- T2 (Dec 6, 2018) is the deadline for source code check-in:

- All tests were available on the branches before T2 except CE10.5.2 and CE10.5.3

- T3 (Dec 19, 2018) is the deadline for proponents to report test results and only bug fixes are allowed for code change after T2:

- The following code check-ins after T2 are reported,

- CE10.1.2.a, CE10.1.2.b, CE10.1.2.c, CE10.1.2.d

- CE10.1.3.b, CE10.1.3.d

- CE10.2.1, CE10.2.2, CE10.2.3, CE10.2.4

- CE10.5.2, CE10.5.3

Document list

CE proposals are listed as follows,

| Proposal Document # | Corresponding Tests | Author(s) | Title |
| --- | --- | --- | --- |
| JVET-M0176 | CE10.1.1 | M.-S. Chiang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek) | CE10.1.1: Multi-hypothesis prediction for improving non-skip & non-merge inter mode and merge mode |
| JVET-M0425 | CE10.1.2 | M. Winken, H. Schwarz, D. Marpe, T. Wiegand (HHI) | CE10: Multi-hypothesis inter prediction (Test 10.1.2) |
| JVET-M0290 | CE10.1.3 | W. Xu, B. Wang, H. Yang, J. Chen (Huawei) | CE10: Simplification on Combined Inter-Intra Prediction (Test 10.1.3) |
| JVET-M0177 | CE10.1.4 | M.-S. Chiang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek) | CE10.1.4: Simplification of combined inter and intra prediction |
| JVET-M0293 | CE10.1.5 | W. Xu, B. Wang, H. Yang, J. Chen (Huawei), M.-S. Chiang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek) | CE10: Simplification on Combined Inter-Intra Prediction with size restriction (Test 10.1.5) |
| JVET-M0178 | CE10.2.1 | Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, C.-C. Chen, Y.-C. Lin, Y.-W. Huang, S.-M. Lei (MediaTek), X. Xiu, Y. He (InterDigital) | CE10.2.1: Uni-prediction-based CU-boundary-only OBMC |
| JVET-M0179 | CE10.2.2 | Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, C.-C. Chen, Y.-C. Lin, Y.-W. Huang, S.-M. Lei (MediaTek), X. Xiu, Y. He (InterDigital) | CE10.2.2: Integer-MV-based CU-boundary-only OBMC |
| JVET-M0180 | CE10.2.3 | Y.-C. Lin, C.-C. Chen, C.-W. Hsu, Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek) | CE10.2.3: Subblock OBMC with uni-prediction-based OBMC at CU boundaries |
| JVET-M0181 | CE10.2.4 | Y.-C. Lin, C.-C. Chen, C.-W. Hsu, Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek) | CE10.2.4: Subblock OBMC with integer-MV-based OBMC at CU boundaries |
| JVET-M0189 | CE10.3.1 | Y. Ahn, D. Sim (Digital Insights) | CE10.3.1: AMVP mode for triangle prediction |
| JVET-M0042 | CE10.4.1 CE10.4.2 | J. Rasch, A. Henkel, J. Pfaff,H. Schwarz, D. Marpe, T. Wiegand (HHI) | CE10: Uniform Directional Diffusion Filters For Video Coding |
| JVET-M0087 | CE10.5.2 | K. Abe, T. Toma,J. Li (Panasonic) | CE10: Low pipeline latency LIC (test 10.5.2) |
| JVET-M0112 | CE10.5.3 | P. Bordes, T. Poirier, F. LeLeannec (Technicolor) | CE10: LIC confined within current CTU (test 10.5.3) |

CE-related proposals are listed as follows,

| Proposal Document # | Author(s) | Title |
| --- | --- | --- |
| JVET-M0082 | Y. Kato, K. Abe, T. Toma (Panasonic) | CE10-related: Simplification of Multi hypothesis intra prediction |
| JVET-M0088 | K. Abe, T. Toma (Panasonic) | CE10-related: LIC restriction for pipeline structure |
| JVET-M0096 | L. Pham Van, G. Van der Auwera, A. K. Ramasubramonian,V. Seregin, M. Karczewicz (Qualcomm) | CE10.1-related: Inter-intra prediction |
| JVET-M0115 | P. Bordes, F. Galpin, T. Poirier (Technicolor) | CE10-related: pipeline reduction for LIC and GBI |
| JVET-M0182 | C.-M. Tsai, C.-C. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek) | CE10-related: Simplification of local illumination compensation |
| JVET-M0183 | M.-S. Chiang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek) | CE10-related: Simplification of MPM generation for CIIP |
| JVET-M0184 | T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek) | CE10-related: Simplification of triangle merging candidate list derivation |
| JVET-M0185 | M.-S. Chiang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek) | CE10-related: Syntax redundancy removal in triangle prediction |
| JVET-M0190 | Y. Ahn, D. Sim (Digital Insights) | CE10-related: Redundant syntax reduction for triangle prediction |
| JVET-M0194 | A. Tamse, M. W. Park, S. Jeong, M. Park, K. Choi (Samsung) | CE10-related: Triangle Prediction Mode Harmonization |
| JVET-M0207 | S. Jeong, M. W. Park, A. Tamse,M. Park, K. Choi (Samsung) | CE10-related: Joint optimizations of Triangular prediction unit mode and Multi-Hypothesis prediction mode |
| JVET-M0216 | L Li, J. Nam, N. Park, H. Jang, J. Lim, S. Kim (LGE) | CE10-related: syntax clean-up on triangle prediction |
| JVET-M0224 | S Bandyopadhyay,X Xiu, Y He (InterDigital) | CE10-related: Local Illumination compensation simplifications |
| JVET-M0232 | Y.-W. Chen, X. Wang (Kwai Inc.) | CE10-related: Simplification of CIIP Intra mode coding |
| JVET-M0233 | X. Wang, Y.-W. Chen (Kwai Inc.) | CE10-related: Triangle prediction merge list construction |
| JVET-M0234 | X. Wang, Y.-W. Chen (Kwai Inc.) | CE10-related: Triangle prediction merge index coding |
| JVET-M0271 | L. Zhang, K. Zhang, H. Liu, J. Xu, Y. Wang, P. Zhao, D. Hong(Bytedance) | CE10-related: Merge list construction process for triangular prediction mode |
| JVET-M0276 | J. Li, S. Wang, W. Gao(Peking University), L. Zhang, K. Zhang,H. Liu, J. Xu(Bytedance) | CE10-related: MPM list alignment between CIIP and intra mode |
| JVET-M0283 | A. Robert, F. LeLeannec, T. Poirier, F. Galpin (Technicolor) | CE10-related: Reduction of motion predictor pruning in Triangle Merge mode |
| JVET-M0294 | B. Wang, A.M. Kotra, S. Esenlik, H. Gao, J. Chen (Huawei) | CE10-related: Modification for blocks applied with Combined Inter-Intra prediction |
| JVET-M0296 | B. Wang, S. Esenlik, A.M. Kotra, H. Gao, J. Chen (Huawei) | CE10-related: Simplification on combined inter-intra mode prediction |
| JVET-M0317 | R.-L. Liao, J. Li, C. S. Lim (Panasonic) | CE10-related: Simplification of triangular prediction unit mode |
| JVET-M0328 | F. Chen, L. Wang (Hikvision) | CE10-related: Simplified triangle prediction unit mode |
| JVET-M0329 | F. Chen, L. Wang (Hikvision) | CE10-related: Modified enabling condition for triangle prediction unit mode |
| JVET-M0331 | L. Xu, F. Chen, L. Wang(Hikvision) | CE10-related:A simplification of inter prediction information derivation for multi-intra-inter mode |
| JVET-M0349 | X.W. Meng (Peking University), X. Zheng (DJI), S.S. Wang, S.W. Ma | CE10-related: Simplification of triangle prediction merging candidate list derivation |
| JVET-M0352 | D. Park, Y. Yoon,J.-G. Kim (KAU),J. Lee, J. Kang (ETRI) | CE10-related: Simplification of triangular partitions |
| JVET-M0357 | Y. Kidani, K. Kawamura, K. Unno, S. Naito (KDDI) | CE10-related: Reduction of the worst-case memory bandwidth and operation number of OBMC |
| JVET-M0399 | H. Wang, W.-J. Chien, V. Seregin,Y.-H. Chao, H. Huang, M. Karczewicz (Qualcomm) | CE10-related: Modifications of Triangular PU Mode |
| JVET-M0424 | M. Sychev, J. Chen | CE10-related: On enhancement of 4-tap interpolation filters |
| JVET-M0438 | J. Zhao, S. Kim (LGE) | CE10-related: Size constraint on Triangular Prediction |
| JVET-M0450 | M. Xu, X. Li, X. Xu, S. Liu (Tencent) | CE10-related: LIC inheritance restrictions and interaction with GBI |
| JVET-M0454 | A. Seixas Dias, G. Kulupana, S. Blasi (BBC) | CE10-related: Multi-Hypothesis Intra with Weighted Combination |
| JVET-M0492 | L. Zhao, X. Zhao,X. Li, S. Liu (Tencent) | CE10-related: Simplified multi-hypothesis intra-inter mode |
| JVET-M0500 | V. Seregin, W.-J. Chien, T. Hsieh, N. Hu, M. Karczewicz (Qualcomm) | CE10-related: Unidirectional illumination compensation |
| JVET-M0525 | P.-H. Lin, Y.-J. Chang (Foxconn) | CE10-related: Simplification of intra prediction in CIIP |
| JVET-M0581 | M. Bläser, J. Sauer (RWTH Aachen University) | CE10-related: Bi-directional motion vector storage for triangular prediction |

Experimental Conditions

Software

One software code base is used which is the VTM software. The VTM subversion repository can be found at https://vcgit.hhi.fraunhofer.de/JVET-L-CE10/VVCSoftware_VTM. The 3.0 release version of the software is used.

Anchors

The VTM is used as anchor, where VTM cfg is applied. Proposals are evaluated using BD rates, encoder run time, decoder time with Common Test Conditions (CTC).

For studying cache-related aspects, the cache model that can be compiled into the reference decoder software by setting #define JVET_J0090_MEMORY_BANDWITH_MEASURE to 1 is used, in conjunction with the cache cfg file provided in JVET-K0451

Summary of results

CE10.1: Multi-hypothesis prediction

In CE10.1, the goal is to test prediction to be combined coming from multiple hypotheses, where one hypothesis refers to prediction from inter mode or from intra mode. The tests can be categorized into two aspects: one is for coding performance improvement and the other is for simplification.

For coding performance improvement aspects, the tests and corresponding results are summarized as follows,

| # | Supported modes | Signaling of hypothesis | # of additional hypotheses | Block constraint in luma samples | BW reduction technique | Hypothesis inheritance | Reference frame access constraints | Color components |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CE10.1.1.a | AMVP (uni-) | merge index | 1 | >= 8x8 |  |  | up to 2 | luma + chroma |
| JVET-M0176 |  |  |  |  |  |  |  |  |
| CE10.1.1.b JVET-M0176 | skip/merge | merge index (implicitly derived) | 1 or 2 |  | full-pel for additional hypotheses | no temporal | up to 4 | luma + chroma |
|  |  |  |  |  | (luma) | spatial: no CTU constraints |  |  |
| CE10.1.2.a JVET-M0425 | merge AMVP | ref index + mvp index + MVDs + weight index | 1 or 2 | > 8x8 | full-pel for additional hypotheses | no temporal | up to 4 | luma + chroma |
|  |  |  |  |  | (luma + chroma) | spatial: from left or within CTU |  |  |
| CE10.1.2.b JVET-M0425 | merge AMVP | ref index + mvp index + MVDs + weight index | 1 or 2 | > 8x8 | full-pel for additional hypotheses | not allowed | up to 4 | luma + chroma |
|  |  |  |  |  | (luma + chroma) |  |  |  |
| CE10.1.2.c JVET-M0425 | merge AMVP | ref index + mvp index + MVDs + weight index | 1 or 2 | > 8x8 | full-pel for additional hypotheses | no temporal | up to 2 | luma + chroma |
|  |  |  |  |  | (luma + chroma) | spatial: from left or within CTU |  |  |
| CE10.1.2.d JVET-M0425 | merge AMVP | ref index + mvp index + MVDs + weight index | 1 or 2 | > 8x8 | full-pel for additional hypotheses | no temporal | up to 4 | luma |
|  |  |  |  |  | (luma + chroma) | spatial: from left or within CTU |  |  |

The test results for this aspect are summarized as follows,

| # | Proposal | Config. |  |  | VTM |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Y | U | V | EncT | DecT |
| CE10.1.1.a | JVET-M0176 | RA | -0.17% | -0.13% | -0.10% | 107% | 100% |
|  |  | LB | 0.00% | 0.10% | 0.25% | 107% | 101% |
| CE10.1.1.b | JVET-M0176 | RA | -0.17% | -0.25% | -0.22% | 103% | 102% |
|  |  | LB | -0.12% | -0.19% | -0.17% | 103% | 102% |
| CE10.1.2.a | JVET-M0425 | RA | -0.29% | -0.16% | -0.12% | 107% | 97% |
|  |  | LB | -0.41% | -0.04% | -0.04% | 110% | 99% |
| CE10.1.2.b | JVET-M0425 | RA | -0.20% | -0.02% | -0.05% | 107% | 96% |
|  |  | LB | -0.30% | -0.10% | -0.17% | 111% | 102% |
| CE10.1.2.c | JVET-M0425 | RA | -0.20% | -0.10% | -0.08% | 105% | 97% |
|  |  | LB | -0.20% | -0.19% | -0.24% | 107% | 100% |
| CE10.1.2.d | JVET-M0425 | RA | -0.29% | 0.03% | 0.04% | 107% | 96% |
|  |  | LB | -0.40% | 0.06% | -0.02% | 110% | 99% |

For simplification aspects, the tests and corresponding results are summarized as follows, where the current design in VTM-3.0 and the differences between each test and VTM-3.0 are listed

| # | Proposal | Supported intra modes | Reference sample smoothing | PDPC | Weights for combined | CU constraints | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VTM-3.0 |  | 4: DC, PL, VER, HOR | Yes | Yes | PL, DC: equal weights | area >=64 |  |
|  |  |  |  |  | VER, HOR: fixed, position dependent weights | W<128 && H<128 |  |
| CE10.1.3.a | JVET-M0290 | 1: PL |  |  | equal weights |  |  |
| CE10.1.3.b | JVET-M0290 |  | No for PL* | No for PL |  |  |  |
| CE10.1.3.c | JVET-M0290 |  |  |  | equal weights |  |  |
| CE10.1.3.d | JVET-M0290 | 1: PL | No for PL* | No | equal weights (4:4) |  | CE10.1.3.a + CE10.1.3.b |
|  |  |  |  |  | LD frame: 6:2* |  |  |
| CE10.1.4 | JVET-M0177 |  |  |  |  | VTM constraints + |  |
|  |  |  |  |  |  | area <=1024 |  |
| CE10.1.5.a | JVET-M0293 | 1: PL |  |  | equal weights | VTM constraints + | CE10.1.3.a + CE10.1.4 |
|  |  |  |  |  |  | area <=1024 |  |
| CE10.1.5.b | JVET-M0293 | 1: PL |  | No | equal weights (4:4) | VTM constraints + | CE10.1.3.a + CE10.1.4 + no PDPC |
|  |  |  |  |  | LD frame: 6:2* | area <=1024 |  |

The test results for this aspect are summarized as follows,

| # | Proposal | Config. |  |  | VTM |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Y | U | V | EncT | DecT |
| CE10.1.3.a | JVET-M0290 | RA | -0.01% | 0.13% | 0.09% | 99% | 100% |
|  |  | LB | 0.10% | 0.35% | 0.36% | 99% | 100% |
| CE10.1.3.b | JVET-M0290 | RA | 0.01% | 0.00% | -0.02% | 100% | 100% |
|  |  | LB | 0.01% | -0.01% | -0.07% | 100% | 101% |
| CE10.1.3.c | JVET-M0290 | RA | 0.05% | 0.12% | 0.10% | 100% | 100% |
|  |  | LB | 0.10% | 0.18% | 0.05% | 100% | 100% |
| CE10.1.3.d | JVET-M0290 | RA | 0.02% | 0.07% | 0.03% | 99% | 100% |
|  |  | LB | 0.01% | 0.12% | 0.13% | 99% | 100% |
| CE10.1.4 | JVET-M0177 | RA | 0.01% | 0.02% | 0.05% | 99% | 102% |
|  |  | LB | 0.04% | -0.04% | 0.13% | 100% | 100% |
| CE10.1.5.a | JVET-M0293 | RA | 0.02% | 0.16% | 0.12% | 98% | 100% |
|  |  | LB | 0.11% | 0.23% | 0.29% | 98% | 100% |
| CE10.1.5.b | JVET-M0293 | RA | 0.06% | 0.07% | 0.03% | 98% | 100% |
|  |  | LB | 0.07% | 0.15% | 0.03% | 98% | 99% |

The simulation data have been crosschecked and crosscheck results are summarized as follows,

In this summary table, questions have been answered by the cross-checker, including

Level: 1 stands for simulation only, 2 for code review and simulation, 3 for independent implementation and simulation

Results: P stands for partial results by spot-checking, F for full results

EncT/DecT: M stands for match in general, G for gap observed

| # | Tester | Cross-checker | Level? (1/2/3) | Any significant inconsistencies between the description and the provided software? | Results? (P/F) | Match? (Y/N) | EncT? (M/G) | DecT? (M/G) | Additional comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CE10.1.1.a | Man-Shu Chiang (MediaTek) | Martin Winken (HHI) | 2 | N | F | Y | M | M |  |
| CE10.1.1.b | Man-Shu Chiang (MediaTek) | Martin Winken (HHI) | 2 | N | F | Y | M | M |  |
| CE10.1.2.a | Martin Winken (HHI) | Man-Shu Chiang (MediaTek) |  |  |  |  |  |  |  |
| CE10.1.2.b | Martin Winken (HHI) | Man-Shu Chiang (MediaTek) |  |  |  |  |  |  |  |
| CE10.1.2.c | Martin Winken (HHI) | Man-Shu Chiang (MediaTek) |  |  |  |  |  |  |  |
| CE10.1.2.d | Martin Winken (HHI) | Man-Shu Chiang (MediaTek) |  |  |  |  |  |  |  |
| CE10.1.3.a | Weiwei Xu (Huawei) | Man-Shu Chiang (MediaTek) |  |  |  |  |  |  |  |
| CE10.1.3.b | Weiwei Xu (Huawei) | Ling Li (LGE) |  |  |  |  |  |  |  |
| CE10.1.3.c | Weiwei Xu (Huawei) | Ling Li (LGE) |  |  |  |  |  |  |  |
| CE10.1.3.d | Weiwei Xu (Huawei) | Andre Seixas Dias (BBC) | 2 | No | F | Y | M | M |  |
| CE10.1.4 | Man-Shu Chiang (MediaTek) | Weiwei Xu (Huawei) |  |  |  |  |  |  |  |
| CE10.1.5.a | Weiwei Xu (Huawei) Man-Shu Chiang (MediaTek) | Martin Winken (HHI) | 2 | N | F | Y | M | M |  |
| CE10.1.5.b | Weiwei Xu (Huawei) Man-Shu Chiang (MediaTek) | Martin Winken (HHI) | 2 | N | F | Y | M | M |  |

CE10.2: Overlapped block motion compensation

In CE10.2, the goal is to test prediction to be combined from using motions of neighboring coding units (CUs). The tests and corresponding results are summarized as follows,

| # | Proposal | Applied boundaries | # of blending lines | BW reduction technique | Runtime reduction technique | Cost reduction technique |
| --- | --- | --- | --- | --- | --- | --- |
| CE10.2.1 | JVET-M0178 | CU boundaries | 2: width < 8 (left); height < 8 (top) 4: otherwise | apply OBMC to uni-prediction blocks use uni-prediction to generate OBMC region | 1. reuse L shape buffer 2. apply CU size constraints | CTU row buffer removal |
|  |  |  |  |  | 3. apply MV merge 4. skip similar MVs |  |
| CE10.2.2 | JVET-M0179 | CU boundaries | 2: width < 8 (left); height < 8 (top) 3: otherwise | use integer MV to generate OBMC region | 1. reuse L shape buffer 2. apply CU size constraints | CTU row buffer removal |
|  |  |  |  |  | 3. apply MV merge 4. skip similar MVs |  |
| CE10.2.3 | JVET-M0180 | CU boundaries + sub-block boundaries | CU boundary: 2: width < 8 (left); height < 8 (top) 4: otherwise sub-blcok: 1: top, left, right | CU boundary: apply OBMC to uni-prediction blocks use uni-prediction to generate OBMC region | 1. reuse L shape buffer 2. apply CU size constraints | CTU row buffer removal |
|  |  |  | (when sub-block OBMC is applied, the number of blending line at CU boudnary is 1) | sub-block: only apply to uni-prediction affine blocks | 3. apply MV merge 4. skip similar MVs |  |
| CE10.2.4 | JVET-M0181 | CU boundaries + sub-block boundaries | CU boundary: 2: width < 8 (left); height < 8 (top) 3: otherwise sub-blcok: 1: top, left, right | CU boundary: use integer MV to generate OBMC region | 1. reuse L shape buffer 2. apply CU size constraints | CTU row buffer removal |
|  |  |  | (when sub-block OBMC is applied, the number of blending line at CU boudnary is 1) | sub-block: only apply to uni-prediction affine blocks | 3. apply MV merge 4. skip similar MVs |  |

The test results are summarized as follows,

| # | Proposal | Config. |  |  | VTM |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Y | U | V | EncT | DecT |
| CE10.2.1 | JVET-M0178 | RA | -0.27% | -0.58% | -0.62% | 104% | 103% |
|  |  | LB | -0.36% | -0.50% | -0.51% | 106% | 105% |
| CE10.2.2 | JVET-M0179 | RA | -0.37% | -0.91% | -0.90% | 105% | 104% |
|  |  | LB | -0.39% | -0.55% | -0.48% | 106% | 106% |
| CE10.2.3 | JVET-M0180 | RA | -0.39% | -0.59% | -0.63% | 105% | 104% |
|  |  | LB | -0.39% | -0.69% | -0.76% | 107% | 108% |
| CE10.2.4 | JVET-M0181 | RA | -0.49% | -0.91% | -0.87% | 106% | 105% |
|  |  | LB | -0.45% | -0.62% | -0.34% | 108% | 110% |

The simulation data have been crosschecked and crosscheck results are summarized as follows,

| # | Tester | Cross-checker | Level? (1/2/3) | Any significant inconsistencies between the description and the provided software? | Results? (P/F) | Match? (Y/N) | EncT? (M/G) | DecT? (M/G) | Additional comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CE10.2.1 | ChunChia Chen (MediaTek) Xiaoyu Xiu (InterDigital) | Sung-Chang Lim (ETRI) |  |  |  |  |  |  |  |
| CE10.2.2 | Zhi-Yi Lin (MediaTek) Xiaoyu Xiu (InterDigital) | Yoshitaka KIDANI(KDDI) |  |  |  |  |  |  |  |
| CE10.2.3 | ChunChia Chen (MediaTek) Xiaoyu Xiu (InterDigital) | Sung-Chang Lim (ETRI) |  |  |  |  |  |  |  |
| CE10.2.4 | Zhi-Yi Lin (MediaTek) Xiaoyu Xiu (InterDigital) | Yoshitaka KIDANI(KDDI) |  |  |  |  |  |  |  |

CE10.3: multiple shape prediction partitions

In CE10.3, the goal is to test prediction to be combined from non-rectangular prediction partitions within one CU. The tests and corresponding results are summarized as follows,

| # | Proposal | Config. |  |  | VTM |  |  | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Y | U | V | EncT | DecT |  |
| CE10.3.1 | JVET-M0189 | RA | -0.06% | -0.03% | -0.06% | 123% | 100% | Two triangle prediction units (PUs) for AMVP mode |
|  |  | LB | -0.07% | -0.10% | -0.01% | 122% | 103% | Restricted to uni-prediction Restricted to RefIdx0 Applied to block width > 8 && block height > 8 |

The simulation data have been crosschecked and crosscheck results are summarized as follows,

CE10.4: Diffusion filtering of inter- and intra-prediction signals

In CE10.4, the goal is to test prediction to be combined using filtering, where three types of filters, two directional filters and one weighted interpolation of the directional filters are used. The tests and corresponding results are summarized as follows,

| # | Proposal | Config. |  |  | VTM |  |  | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Y | U | V | EncT | DecT |  |
| CE10.4.1 | JVET-M0042 | AI | -0.19% | -0.03% | -0.03% | 123% | 101% | Uniform Diffusion filters with encoder speedup and simplified filter masks |
|  |  | RA | -0.44% | -0.51% | -0.35% | 113% | 98% |  |
|  |  | LB | -0.17% | 0.27% | 0.59% | 114% | 98% |  |
|  |  | LP | -0.86% | -0.26% | -0.11% | 119% | 97% |  |
| CE10.4.2 | JVET-M0042 | AI | -0.17% | -0.03% | -0.05% | 119% | 101% | Like CE10.4.1, but • In case of temporal layer depth larger than 3, no diffusion filter is applied |
|  |  | RA | -0.42% | -0.54% | -0.42% | 111% | 98% | • Simplified encoder search for intra blocks |
|  |  | LB | -0.17% | 0.41% | 0.43% | 113% | 98% |  |
|  |  | LP | -0.87% | -0.15% | -0.10% | 118% | 96% |  |

The simulation data have been crosschecked and crosscheck results are summarized as follows,

| # | Tester | Cross-checker | Level? (1/2/3) | Any significant inconsistencies between the description and the provided software? | Results? (P/F) | Match? (Y/N) | EncT? (M/G) | DecT? (M/G) | Additional comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CE10.4.1 | Jennifer Rasch (HHI) | Jacob Ström (Ericsson) | 2 | No | F | Y | M | M |  |
| CE10.4.2 | Jennifer Rasch (HHI) | Jacob Ström (Ericsson) | 2 | No | F | Y | M | M |  |

CE10.5: local illumination compensation.

In CE10.5, the goal is to test prediction to be combined using linear model derived from reconstructed and reference neighboring samples. The tests and corresponding results are summarized as follows,

| # | Proposal | Config. |  |  | VTM |  |  | Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Y | U | V | EncT | DecT |  |
| CE10.5.2 | JVET-M0087 | RA | -0.56% | -0.31% | -0.37% | 135% | 99% | 1. Remove all encoder and decoder LIC processes other than the reconstruction stage |
|  |  | LB | -0.51% | -0.44% | -0.58% | 140% | 101% | 2. Modify bS calculation of DBF for LIC boundary. |
| CE10.5.3 | JVET-M0112 | RA | -0.44% | -0.31% | -0.34% | 134% | 101% | Based on CE10.5.2 |
|  |  | LB | -0.39% | -0.40% | -0.42% | 138% | 100% | LIC is confined to use reference samples only from the current CTU |

The simulation data have been crosschecked and crosscheck results are summarized as follows,

| # | Tester | Cross-checker | Level? (1/2/3) | Any significant inconsistencies between the description and the provided software? | Results? (P/F) | Match? (Y/N) | EncT? (M/G) | DecT? (M/G) | Additional comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CE10.5.2 | K. Abe (Panasonic) | T. Anish (Samsung) | 2 | No | F | Y | M | M |  |
| CE10.5.3 | P. Bordes (Technicolor) | K. Abe (Panasonic) | 2 | No | F | Y | M | M |  |

Reference

- C.-W. Hsu, M. Winken, “Description of Core Experiment 10: Combined and multi-hypothesis prediction,” JVET-L1030, Macau, CN, October 2018.