| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0052 |
| --- | --- |

| Title: | CE8: Separate palette coding for luma and chroma (Test8.2.5) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Chen Sun, Jian Lou Yung-Hsuan Chao, Hongtao Wang, Vadim Seregin, Marta Karczewicz Roman Chernyak, Sergey Ikonin, Jianle Chen | Tel: Email: | +1-858-349-2818 yuchen.s@alibaba-inc.com +1-858-845-2163 yunghsua@qti.qualcomm.com +7-925-008-12-86 chernyak.roman@huawei.com |
| Source: | Alibaba Cloud Computing Ltd. Qualcomm Huawei Technologies Co., Ltd. |  |  |

_____________________________

- Abstract

This document reports the results of the separated palette design for the joint CUs containing both luma and chroma CBs. The tests are performed on top of CE8.2.1 (JVET-M0050).

CE8.2.5a is tested with the DualITree turned off (non-CTC) in the setting. Compared with CE8.2.1, the results of CE8.2.5a show that:

- When the method is tested with CPR=0, 4.8%, 4.0%, and 2.9% BD-rate gains for luma are shown for class F sequences and 11.5%, 6.5%, and 3.4% BD-rate gains for luma are shown for 4:2:0 TGM sequences under AI, RA, and LD configurations, respectively.

- When the method is tested with CPR=1, 2.0%, 1.3%, and 1.7% BD-rate gains for luma are shown for class F sequences and 4.4%, 2.9%, and 2.4% BD-rate gains for luma are shown for 4:2:0 TGM sequences under AI, RA, and LD configurations, respectively.

CE8.2.5b is tested with the DualITree turned on (CTC) in the setting. Compared with CE8.2.1, the results of CE8.2.5a show that:

- When the method is tested with CPR=0, 0.2%, and 1.3% BD-rate gains for luma are shown for class F sequences and 1.2%, and 2.0% BD-rate gains for luma are shown for 4:2:0 TGM sequences under RA, and LD configurations, respectively.

- When the method is tested with CPR=1, 0.1%, and 0.9% BD-rate gains for luma are shown for class F sequences and 0.5%, and 1.6% BD-rate gains for luma are shown for 4:2:0 TGM sequences under RA, and LD configurations, respectively.

CE Description

In VTM-3.0, when the separate tree structure is applied, a CU may be composed of only luma or only chroma CBs. In such cases, in CE8.2.1, palette is applied on Luma (Y component) and Chroma (Cb and Cr components) separately. However, in other cases, (1) CUs in inter-slices or (2) DualITree is turned off is SPS, palette will still be applied on Y, Cb, Cr components jointly, same as in HEVC palette.

In CE8.2.5, separate palette is used regardless whether the separate tree structure is applied. More specifically, the palette settings in CE8.2.1 and CE8.2.5 are summarized in the following table:

| DualITree is turned on in SPS (CTC) |  |  |
| --- | --- | --- |
|  | I slices (Separate tree) | P/B slices (Joint tree) |
| CE8.2.1 | Separate Palette | Joint Palette |
| CE8.2.5(b) | Separate Palette | Separate Palette |
| DualITree is turned off in SPS (non-CTC) |  |  |
|  | I slices (Joint tree) | P/B slices (Joint tree) |
| CE8.2.1 | Joint Palette | Joint Palette |
| CE8.2.5(a) | Separate Palette | Separate Palette |

Simulation results

CE8.2.5(b): DualITree is turned on (CTC tests)

The proposed method is evaluated according to the CE8 description [4] for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations with the VTM-3.0 software.

The proposed method is tested with/without CPR. Table 1 shows the results with CPR turned off in the setting. The results of CE8.2.5(b) are compared with VTM3.0 and CE8.2.1.

Table 1: Results of the CE8.2.5(b) test (CPR is turned off, DualITree is turned on)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over CE8.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.04% | 0.08% | 112% | 103% | 0.00% | 0.00% | 0.00% | 104% | 104% |
| Class A2 | 0.09% | 0.13% | 0.08% | 112% | 106% | 0.00% | 0.00% | 0.00% | 105% | 106% |
| Class B | 0.09% | 0.09% | 0.07% | 110% | 104% | 0.00% | 0.00% | 0.00% | 104% | 104% |
| Class C | 0.10% | 0.09% | -0.08% | 111% | 109% | 0.00% | 0.00% | 0.00% | 102% | 103% |
| Class E | 0.13% | 0.16% | 0.20% | 111% | 104% | 0.00% | 0.00% | 0.00% | 104% | 103% |
| Overall | 0.09% | 0.10% | 0.06% | 111% | 105% | 0.00% | 0.00% | 0.00% | 104% | 104% |
| Class D | 0.10% | 0.07% | 0.02% | 110% | 108% | 0.00% | 0.00% | 0.00% | 103% | 105% |
| Class F | -11.43% | -8.94% | -9.04% | 121% | 94% | 0.00% | 0.00% | 0.00% | 104% | 105% |
| Class SCC 1080p | -33.06% | -27.59% | -27.70% | 135% | 69% | 0.00% | 0.00% | 0.00% | 103% | 105% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  | Over CE8.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | 0.10% | 0.12% | 102% | 100% | -0.04% | -0.06% | 0.00% | 98% | 100% |
| Class A2 | 0.08% | 0.20% | 0.20% | 104% | 100% | -0.03% | -0.03% | -0.03% | 98% | 100% |
| Class B | 0.11% | 0.24% | 0.03% | 105% | 99% | -0.03% | -0.04% | 0.13% | 99% | 98% |
| Class C | 0.14% | 0.15% | 0.18% | 108% | 107% | -0.03% | 0.05% | -0.03% | 99% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.11% | 0.18% | 0.12% | 105% | 101% | -0.03% | -0.02% | 0.03% | 99% | 99% |
| Class D | 0.21% | -0.11% | 0.25% | 108% | 104% | 0.01% | 0.05% | 0.00% | 99% | 100% |
| Class F | -8.65% | -7.95% | -8.32% | 110% | 102% | -0.16% | 0.06% | -0.01% | 100% | 101% |
| Class SCC 1080p | -16.64% | -15.24% | -15.02% | 108% | 94% | -1.18% | -0.79% | -0.71% | 102% | 101% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  | Over CE8.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.08% | -0.02% | -0.08% | 110% | 103% | -0.01% | -0.01% | -0.13% | 103% | 104% |
| Class C | 0.15% | 0.39% | 0.26% | 109% | 99% | 0.00% | -0.04% | -0.22% | 103% | 105% |
| Class E | 0.21% | 1.30% | 0.26% | 109% | 102% | -0.06% | 0.05% | -0.19% | 102% | 106% |
| Overall | 0.13% | 0.45% | 0.12% | 109% | 101% | -0.02% | -0.01% | -0.18% | 103% | 105% |
| Class D | 0.23% | 0.66% | 0.26% | 111% | 106% | -0.04% | 0.12% | -0.14% | 102% | 103% |
| Class F | -5.07% | -4.98% | -5.52% | 110% | 99% | -1.32% | -0.61% | -0.78% | 101% | 99% |
| Class SCC 1080p | -8.81% | -8.43% | -7.74% | 106% | 94% | -1.97% | -1.91% | -1.52% | 101% | 99% |

Table 2 shows the results with CPR turned on in the setting. The results of CE8.2.5(b) are compared with VTM3.0 and CE8.2.1.

Table 2: Results of the CE8.2.5(b) test (CPR is turned on, DualITree is turned on)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over CE8.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.18% | 0.08% | 110% | 105% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.09% | 0.07% | 0.11% | 109% | 106% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.09% | 0.08% | 0.06% | 107% | 107% | 0.00% | 0.00% | 0.00% | 101% | 105% |
| Class C | 0.09% | 0.12% | 0.07% | 106% | 101% | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class E | 0.14% | 0.29% | 0.03% | 104% | 100% | 0.00% | 0.00% | 0.00% | 98% | 100% |
| Overall | 0.09% | 0.14% | 0.07% | 107% | 104% | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class D | 0.09% | 0.20% | -0.04% | 108% | 103% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | -4.46% | -2.46% | -2.51% | 110% | 96% | 0.00% | 0.00% | 0.00% | 99% | 101% |
| Class SCC 1080p | -12.66% | -7.93% | -7.85% | 114% | 87% | 0.00% | 0.00% | 0.00% | 101% | 101% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  | Over CE8.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | 0.20% | 0.01% | 105% | 103% | 0.01% | -0.04% | -0.12% | 97% | 99% |
| Class A2 | 0.11% | 0.10% | 0.06% | 105% | 104% | 0.00% | 0.02% | 0.03% | 97% | 98% |
| Class B | 0.10% | -0.05% | 0.08% | 104% | 100% | 0.01% | -0.06% | -0.04% | 97% | 98% |
| Class C | 0.14% | 0.05% | 0.16% | 107% | 102% | -0.02% | 0.00% | -0.14% | 98% | 99% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.11% | 0.06% | 0.09% | 105% | 102% | 0.00% | -0.02% | -0.07% | 97% | 99% |
| Class D | 0.25% | 0.29% | -0.05% | 109% | 104% | -0.02% | 0.05% | 0.15% | 99% | 100% |
| Class F | -2.71% | -2.22% | -2.49% | 109% | 102% | -0.10% | 0.13% | 0.10% | 99% | 101% |
| Class SCC 1080p | -5.16% | -4.02% | -3.65% | 102% | 95% | -0.51% | -0.44% | -0.26% | 98% | 99% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  | Over CE8.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.11% | 0.16% | 0.38% | 111% | 107% | 0.00% | 0.08% | 0.14% | 100% | 101% |
| Class C | 0.12% | 0.04% | 0.20% | 107% | 106% | -0.04% | -0.11% | 0.16% | 98% | 104% |
| Class E | 0.20% | -0.11% | -0.24% | 106% | 102% | 0.01% | -0.10% | -0.07% | 100% | 102% |
| Overall | 0.14% | 0.05% | 0.17% | 108% | 105% | -0.01% | -0.03% | 0.09% | 99% | 102% |
| Class D | 0.28% | 0.79% | -0.12% | 106% | 97% | 0.03% | -0.02% | -0.08% | 98% | 97% |
| Class F | -2.02% | -2.01% | -2.34% | 105% | 101% | -0.88% | -0.38% | -0.50% | 102% | 103% |
| Class SCC 1080p | -3.30% | -2.74% | -2.31% | 103% | 106% | -1.55% | -1.46% | -1.21% | 101% | 110% |

CE8.2.5(a): DualITree is turned off (non-CTC tests)

The proposed method is evaluated according to the CE8 description [2] for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations with the VTM-3.0 software. But, DualITree is turned off in SPS.

The proposed method is tested with/without CPR. Table 3 shows the results with CPR turned off in the setting. The results of CE8.2.5(a) are compared with VTM3.0 and CE8.2.1.

Table 3: Results of the CE8.2.5(a) test (CPR is turned off, DualITree is turned off)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over CE8.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | 0.04% | -0.03% | 103% | 94% | 0.00% | -0.02% | -0.01% | 95% | 94% |
| Class A2 | 0.05% | 0.10% | 0.06% | 105% | 98% | 0.00% | 0.02% | -0.03% | 97% | 98% |
| Class B | 0.07% | 0.08% | -0.05% | 105% | 99% | 0.00% | 0.06% | 0.01% | 98% | 99% |
| Class C | 0.09% | 0.14% | 0.13% | 104% | 99% | -0.01% | 0.02% | 0.07% | 98% | 97% |
| Class E | 0.08% | 0.09% | 0.05% | 105% | 98% | 0.00% | 0.01% | 0.02% | 97% | 97% |
| Overall | 0.07% | 0.09% | 0.03% | 104% | 98% | 0.00% | 0.02% | 0.02% | 97% | 97% |
| Class D | 0.11% | 0.00% | 0.01% | 105% | 100% | 0.01% | 0.02% | -0.03% | 98% | 98% |
| Class F | -9.75% | -9.29% | -10.06% | 113% | 86% | -4.76% | -3.13% | -3.45% | 100% | 90% |
| Class SCC 1080p | -27.02% | -26.01% | -25.52% | 126% | 62% | -11.52% | -10.15% | -9.53% | 104% | 89% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  | Over CE8.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.11% | -0.03% | 0.08% | 103% | 101% | -0.02% | -0.06% | 0.00% | 99% | 99% |
| Class A2 | 0.11% | 0.05% | 0.06% | 104% | 101% | -0.02% | 0.04% | 0.00% | 99% | 99% |
| Class B | 0.11% | 0.06% | 0.18% | 104% | 98% | -0.02% | -0.02% | -0.02% | 99% | 100% |
| Class C | 0.16% | 0.20% | 0.05% | 106% | 98% | 0.00% | 0.01% | 0.11% | 96% | 95% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.13% | 0.08% | 0.10% | 104% | 99% | -0.02% | -0.01% | 0.02% | 98% | 98% |
| Class D | 0.27% | 0.22% | 0.19% | 107% | 101% | 0.00% | -0.32% | 0.05% | 97% | 96% |
| Class F | -7.51% | -7.08% | -8.08% | 107% | 96% | -3.97% | -2.19% | -2.61% | 96% | 94% |
| Class SCC 1080p | -14.78% | -15.28% | -14.62% | 102% | 88% | -6.51% | -5.84% | -5.20% | 97% | 91% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  | Over CE8.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.07% | 0.00% | 0.39% | 102% | 98% | 0.00% | 0.01% | 0.43% | 95% | 96% |
| Class C | 0.07% | 0.00% | -0.11% | 103% | 93% | -0.05% | -0.23% | -0.26% | 96% | 91% |
| Class E | 0.20% | -0.36% | 0.00% | 103% | 94% | -0.07% | 0.07% | 0.10% | 95% | 93% |
| Overall | 0.10% | -0.09% | 0.13% | 103% | 95% | -0.04% | -0.06% | 0.12% | 95% | 93% |
| Class D | 0.18% | 0.80% | 0.42% | 107% | 98% | -0.01% | 0.70% | 0.22% | 98% | 97% |
| Class F | -4.67% | -4.83% | -5.30% | 103% | 98% | -2.87% | -0.99% | -1.57% | 95% | 96% |
| Class SCC 1080p | -8.49% | -8.97% | -8.39% | 100% | 93% | -3.44% | -3.05% | -2.61% | 97% | 98% |

Table 4 shows the results with CPR turned on in the setting. The results of CE8.2.5(a) are compared with VTM3.0 and CE8.2.1.

Table 4: Results of the CE8.2.5(a) test (CPR is turned on, DualITree is turned off)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over CE8.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.02% | 0.04% | 107% | 101% | 0.00% | 0.02% | 0.01% | 101% | 103% |
| Class A2 | 0.06% | 0.08% | 0.06% | 106% | 101% | 0.00% | 0.03% | 0.00% | 101% | 102% |
| Class B | 0.06% | 0.04% | 0.10% | 108% | 103% | -0.01% | -0.02% | 0.05% | 100% | 101% |
| Class C | 0.06% | 0.13% | 0.17% | 106% | 99% | -0.01% | 0.03% | -0.04% | 99% | 97% |
| Class E | 0.10% | 0.12% | -0.02% | 108% | 98% | -0.01% | 0.07% | 0.01% | 100% | 98% |
| Overall | 0.07% | 0.08% | 0.08% | 107% | 101% | -0.01% | 0.02% | 0.01% | 100% | 100% |
| Class D | 0.08% | 0.09% | 0.09% | 108% | 104% | 0.00% | -0.05% | 0.00% | 102% | 102% |
| Class F | -3.08% | -2.08% | -2.79% | 111% | 97% | -1.96% | -0.81% | -1.28% | 105% | 96% |
| Class SCC 1080p | -8.60% | -6.97% | -6.73% | 110% | 86% | -4.42% | -3.33% | -3.21% | 105% | 96% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  | Over CE8.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.10% | 0.00% | 0.07% | 105% | 102% | -0.03% | 0.00% | -0.01% | 99% | 99% |
| Class A2 | 0.10% | 0.05% | 0.12% | 105% | 102% | 0.02% | 0.13% | 0.02% | 96% | 96% |
| Class B | 0.10% | 0.10% | 0.13% | 106% | 100% | 0.00% | -0.04% | -0.04% | 99% | 99% |
| Class C | 0.11% | -0.06% | -0.01% | 110% | 105% | -0.02% | -0.12% | -0.01% | 100% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.10% | 0.03% | 0.08% | 107% | 102% | -0.01% | -0.02% | -0.01% | 99% | 99% |
| Class D | 0.22% | 0.01% | 0.46% | 110% | 106% | -0.02% | -0.04% | 0.17% | 100% | 100% |
| Class F | -1.92% | -0.87% | -1.68% | 106% | 99% | -1.28% | 0.05% | -0.70% | 100% | 97% |
| Class SCC 1080p | -3.98% | -3.52% | -3.08% | 102% | 95% | -2.85% | -2.36% | -2.07% | 99% | 97% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  | Over CE8.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.11% | 0.15% | 0.45% | 110% | 104% | -0.01% | 0.09% | 0.34% | 100% | 100% |
| Class C | 0.12% | -0.05% | 0.06% | 109% | 104% | 0.02% | 0.11% | -0.04% | 101% | 106% |
| Class E | 0.14% | 0.34% | -0.47% | 108% | 102% | -0.10% | 0.08% | -0.92% | 102% | 105% |
| Overall | 0.12% | 0.13% | 0.09% | 109% | 103% | -0.02% | 0.09% | -0.10% | 101% | 103% |
| Class D | 0.19% | -0.27% | -0.22% | 111% | 104% | 0.05% | -0.71% | 0.26% | 102% | 106% |
| Class F | -1.72% | -0.84% | -1.56% | 109% | 102% | -1.70% | -0.56% | -0.90% | 99% | 103% |
| Class SCC 1080p | -3.26% | -3.29% | -2.74% | 104% | 99% | -2.44% | -2.09% | -1.74% | 97% | 99% |

Reference

- Y.-H. Chao, H. Wang, V. Seregin, M. Karczewicz, Y.-C. Sun, J. An, J. Lou, “CE15-2: Palette mode of HEVC SCC”, JVET-L0336, 2018

- Y.-H. Chao, H. Wang, V. Seregin, M. Karczewicz, Y.-C. Sun, J. An, J. Lou, “CE15-related: Separated palette test on top of CE15.2”, JVET-L0672, 2018

- R. Chernyak, S. Ikonin, J. Chen, Y.-C. Sun, J. An, J. Lou, “CE15-related: Separate Palette Coding for Luma and Chroma components”, JVET-L0427, 2018

- X. Xu, Y.-C. Chao, Y.-C. Sun, J. Xu, "Description of Core Experiment 8: Screen Content Coding Tools”, JVET-L1028, 2018.

Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Huawei Technologies Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).