| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0063-v3 |
| --- | --- |

| Title: | Non-CE9: An improvement of BDOF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh Tomohiro Ikai | Tel: Email: | +81-43-299-8526 chujoh.takeshi@sharp.co.jp ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

BDOF (bi-directional optical flow) is a method to improve bi-directional prediction efficiency by using gradient information. Current specification of working draft has been designed to perform all operations within 32-bit on CTC. In this contribution, a specification corresponding to the bit-depth other than 10-bit on CTC is proposed. Since 10-bit profile shall support 8-bit coding, this specification is needed. The changes are to replace some fixed shift values to bit-depth variables. The operations within 32-bit can be kept. As a result of experiments in which the 8-bit sequence was coded with 8-bit, 0.32% gain on average has been improved.

Introduction

BDOF (bi-directional optical flow) [1] improves bi-directional prediction efficiency by using gradient information. In this contribution, a specification of BDOF corresponding to the bit-depth other than 10-bit on CTC [2] is proposed. The changes from current working draft [3] have been specified and all operations within 32-bit have been proved. The experiment of 8-bit coding has been shown to improve coding efficiency by this fix.

Proposal changes of specification

The working draft should be changed by the followings.

8.3.6.4	Bidirectional optical flow prediction process

- The variables gradientHL0[ x ][ y ], gradientVL0[ x ][ y ], gradientHL1[ x ][ y ] and gradientVL1[ x ][ y ] are derived as follows:

gradientHL0[ x ][ y ]  =  (predSamplesL0[ hx + 1 ][vy] − predSampleL0[ hx − 1 ][ vy ] ) >> shift1	(8-697)

gradientVL0[ x ][ y ]  =   (predSampleL0[ hx ][ vy + 1 ] − predSampleL0[ hx ][vy − 1 ] ) >> shift1	(8-698)

gradientHL1[ x ][ y ]  =  (predSamplesL1[ hx + 1 ][vy] − predSampleL1[ hx − 1 ][ vy ] ) >> shift1	(8-699)

gradientVL1[ x ][ y ]  =   (predSampleL1[ hx ][ vy + 1 ] − predSampleL1[ hx ][vy − 1 ] ) >> shift1	(8-700)

The variable shift1 is set equal to Max( 2, 14 − bitDepth )

- The variables temp[ x ][ y ], tempH[ x ][ y ] and tempV[ x ][ y ] are derived as follows:

diff[ x ][ y ] = (predSamplesL0[ hx ][ vy ] >> shift4 ) − ( predSamplesL1[ hx ][ vy ] >> shift4 )	(8-701)

tempH[ x ][ y ] = (gradientHL0[ x ][ y ] + gradientHL1[ x ][ y ] ) >> shift5	(8-702)

tempV[ x ][ y ] = (gradientVL0[ x ][ y ] + gradientVL1[ x ][ y ] ) >> shift5	(8-703)

The variable shift4 is set equal to Min( 8, bitDepth − 4 ) and the variable shift5 is set equal to Min( 5, bitDepth − 7 )

When bitDepth is 10-bit, shift1 is set equal to 4, shift4 is set equal to 6 and shift5 is set equal to 3. They are identical with the current specification. Therefore, there is no change on CTC.

When bitDepth is the range between 8-bit and 12-bit, maximum bit-range of predSamplesL0 and predSamplesL1 is 16-bit. Since the range of difference between predSamplesL0 and predSamplesL1 is 17-bit, if shift1 is equal to (14 – bitDepth), the range of grandientHL0, grandientVL0, grandientHL1 and grandientVL1 become (bitDepth + 3) bits. If shift4 is equal to (bitDepth – 4), the range of diff is (21 – bitDepth) because of 16 minus (bitDepth – 4) plus 1. If shift5 is equal to (bitDepth – 7), the range of tempH and tempV is 11-bit because of (bitDepth + 3) plus 1 minus (bitDepth – 7). Since sum of 6x6 block is 36 and log2(36) is grater than 5, the range of sGx2, sGy2 and sGxGy is 28-bit because of 11 plus 11 plus 6. Finally, the range of sGxdI and sGydI is (38 – bitDepth) bits because of 11 plus (21 – bitDepth) plus 6.

Therefore, it is possible to perform all operations within 32-bit, when bitDepth is from 8-bit to 12-bit.

Table 1  Bit range of step 2 and 3 in subsection 8.3.6.4

| Equation | Range |
| --- | --- |
| predSamplesL0 and predSamplesL1 | 16 |
| gradientHL0[ x ][ y ] = (predSamplesL0[ hx + 1 ][vy] − predSampleL0[ hx − 1 ][ vy ] ) >> shift1 gradientVL0[ x ][ y ] = (predSampleL0[ hx ][ vy + 1 ] − predSampleL0[ hx ][vy − 1 ] ) >> shift1 gradientHL1[ x ][ y ] = (predSamplesL1[ hx + 1 ][vy] − predSampleL1[ hx − 1 ][ vy ] ) >> shift1 gradientVL1[ x ][ y ] = (predSampleL1[ hx ][ vy + 1 ] − predSampleL1[ hx ][vy − 1 ] ) >> shift1 shift1 = Max(2, 14 – bitDepth) | bitDepth+3 |
| diff[ x ][ y ] = (predSamplesL0[ hx ][ vy ] >> shift4 ) − ( predSamplesL1[ hx ][ vy ] >> shift4 ) shift4 = Min(8, bitDepth – 4) | 21 – bitDepth |
| tempH[ x ][ y ] = (gradientHL0[ x ][ y ] + gradientHL1[ x ][ y ] ) >> shift5 tempV[ x ][ y ] = (gradientVL0[ x ][ y ] + gradientVL1[ x ][ y ] ) >> shift5 shift5 = Min(5, bitDepth – 7) | 11 |
| sGx2 = ij ( tempH[ xSb + i ][ ySb + j ] * tempH[ xSb + i ][ ySb + j ] ) with i, j = −1..4 sGy2 = ij(tempV[ xSb + i ][ ySb + j ] * tempV[ xSb + i ][ ySb + j ] ) with i, j = −1..4 sGxGy = ij(tempH[ xSb + i ][ ySb + j ] * tempV[ xSb + i ][ ySb + j ] ) with i, j −1..4 | 28 |
| sGxdI = ij( − tempH[ xSb + i ][ ySb + j ] * diff[ xSb + i ][ ySb + j ] ) with i, j = −1..4 sGydI = ij( − tempV[ xSb + i ][ ySb + j ] * diff[ xSb + i ][ ySb + j ] ) with i, j = −1..4 | 38 – bitDepth |

Experimental results

The proposed method has been implemented on VTM-3.0 software. The simulation was based on VTM configuration JVET-L1010 [2] with 8-bit coding for 8-bit SDR sequences which are class C, D, F and three class B sequences, Cactus, BasketballDrive, and BQTerrace.

The PC cluster was used for the simulation, but since the same CPUs are allocated to the same parameters of the same sequences, the execution time is almost accurate. In order to measure more accurate decoding times, the decoder was re-run all the bitstreams on single CPU.

As summary of experimental results, there were on average about 0.32% gain totally and there were some gains for all sequences. The encoding and decoding times were all most the same as the anchor.

Table 2  Summary of experimental results compared to VTM-3.0 with 8-bit coding

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0_8-bit |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.39% | -0.39% | -0.31% | 100% | 99% |
| Class C | -0.26% | -0.26% | -0.32% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.32% | -0.32% | -0.31% | 100% | 99% |
| Class D | -0.27% | -0.40% | -0.35% | 100% | 100% |
| Class F (optional) | -0.18% | -0.16% | -0.16% | 100% | 100% |

Conclusion

In this contribution, a fix of bit-depth for BDOF has been proposed. The experiment of 8-bit coding has been shown to improve 0.32% on average. The encoding and decoding times were all most the same as the anchor. Finally, we would like to recommend that the working draft and reference software should support this specification.

Reference

[1]	X. Xiu, Y. He and Y. Ye, CE9-related: Complexity reduction and bit-width control for bi-directional optical flow (BIO), Joint Video Experts Team Document, JVET-L0256, Macao, Oct. 2018.

[2]	F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, JVET common test conditions and software reference configurations for SDR video, Joint Video Experts Team Document, JVET-L1010, Macao, Oct. 2018.

[3]	B. Bross, J. Chen and S. Liu, Versatile Video Coding (Draft 3), Joint Video Experts Team Document, JVET-L1001-v7, Macao, Oct. 2018.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).