| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0073 |
| --- | --- |

| Title: | CE9-related: On early termination for BIO |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kenji Kondo, Masaru Ikeda and Teruhiko Suzuki 2-10-1 Osaki, Shinagawa-ku, Tokyo Japan | Tel: Email: | +81-50-3750-2744 kenji.kondo@sony.com, teruhiko.s@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

In this contribution, it is mentioned that the sum of absolute difference (SAD) to early exit in bi-optical flow (BIO) increases hardware complexity, and the solutions are introduced. The SAD calculation is processed in CU level but it may increase the memory size up to CTU level. To avoid memory size up, the SAD calculation is divided into the virtual pipeline data unit (VPDU). Simulation results reportedly show 0.00%, 0.01% and -0.01% BD-rate for Y, U and V component in Random Access (RA) configuration.

Problem Statement

In the VTM-3.0, the BIO technique calculates SAD between a reference L0 and L1 in CU (=PU) level. If the SAD of CU is smaller than threshold, the BIO process early exits. Figure 1 shows the block diagram for early stage in BIO process.

Figure 1 Early stage in BIO process

Where, the block SAD computes a SAD between predictions L0 and L1 in a CU level. And it’s required to store because it may be used to process usual bi-prediction if BIO early exits. Currently, a largest CTU size is 128x128 in luma pixel. It means that the memory size is up to 128x128.

On the other hand, in the VPDU concept [1], the buffer size can be kept max transform size (64x64). But, current early termination of BIO is not match the VPDU concept.

If the predictions are not stored after the SAD computation, it will allow reducing the memory size. But, it will make a large delay to process an interpolation again when BIO early exits. Also it will increase the hardware complexity to increase a throughput for interpolation process.

Solutions

To avoid storing predictions beyond VPDU size (64x64), solutions are proposed in following sections.

Solution-1

In this solution-1, an early exit region is divided into small size which is smaller than or equal to VPDU size. It’s shown in Figure 2.

Accordingly, the SAD computation is conducted for each region. Then, SAD for each region is checked, and if the value is less than the threshold, it early exits for the divided SAD region.

As shown in the section 3, it was experimentally tested in case of that the SAD size is 64x64, 32x32 and 16x16.

Figure 2: Solution-1

Solution-2

When CU size is larger than VPDU size, it doesn’t use the early termination. Thus, BIO is always process even if SAD is smaller than the threshold.

Solution-3

When CU size is larger than VPDU size, BIO is always disabled.

Experimental results

As introduced in section 2, those solutions were implemented in VTM-3.0. On the common test condition [2], the simulations were tried for only RA case because the BIO works in only RA case.

Simulation environment

For the simulations, same platform machines as below in PC cluster were used for decoder but several machines were used for encoder. The results for encoding time are not reliable.

- Encode

  - OS: CentOS 6.9

  - Compiler: g++ (GCC) 6.3.1

  - CPU: Intel Core i7-7700K 4.2GHz, Core i7-6700K 4.0 GHz and Core i5-6600K 3.5 GHz

    - SIMD: AVX2 instruction is enabled.

  - Memory: 32 GB

- Decode

  - OS: CentOS 6.9

  - Compiler: g++ (GCC) 6.3.1

  - CPU: Intel Core i5-2500K 3.3 GHz

    - SIMD: AVX instruction is enabled.

  - Memory: 24 GB

For a runtime measurement, the user time in the output log was used. For a random access case and an all intra case, a parallel encoding is used. To measure encoding time in parallel encoding, the total of all user time for each frame were calculated.

The summary of test results

Table 1: Test results for Solution-1 64x64

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM 3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.05% | 0.01% | 61% | 100% |
| Class A2 | -0.01% | 0.00% | -0.01% | 65% | 101% |
| Class B | 0.00% | -0.01% | -0.04% | 55% | 101% |
| Class C | 0.00% | 0.00% | 0.01% | 47% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.01% | -0.01% | 56% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 52% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 49% | 106% |

Table 2: Test results for Solution-1 32x32

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM 3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.03% | 0.02% | 59% | 102% |
| Class A2 | -0.01% | -0.01% | 0.05% | 62% | 102% |
| Class B | 0.00% | 0.01% | -0.03% | 54% | 102% |
| Class C | 0.00% | 0.02% | -0.02% | 47% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.01% | 0.00% | 55% | 101% |
| Class D | 0.00% | -0.06% | 0.00% | 51% | 100% |
| Class F | -0.02% | -0.04% | -0.02% | 48% | 107% |

Table 3: Test results for Solution-1 16x16

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM 3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.03% | 0.00% | 62% | 101% |
| Class A2 | -0.02% | -0.02% | 0.02% | 65% | 102% |
| Class B | -0.01% | 0.01% | -0.05% | 54% | 102% |
| Class C | 0.00% | -0.01% | -0.03% | 47% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.00% | -0.02% | 55% | 101% |
| Class D | 0.01% | -0.06% | -0.02% | 52% | 101% |
| Class F | -0.01% | -0.03% | -0.02% | 49% | 107% |

Table 4: Test results for Solution-2, BIO is always processed without an early termination when CU > VPDU size

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM 3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.03% | 0.01% | 66% | 101% |
| Class A2 | -0.03% | -0.01% | 0.05% | 63% | 101% |
| Class B | 0.00% | -0.02% | -0.04% | 71% | 101% |
| Class C | 0.00% | 0.00% | -0.01% | 59% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | -0.01% | 65% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 59% | 100% |
| Class F | -0.01% | -0.01% | -0.01% | 59% | 106% |

Table 5: Test results for Solution-3, BIO is always disabled when CU > VPDU size

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM 3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.23% | 0.19% | 0.14% | 50% | 95% |
| Class A2 | 0.30% | 0.23% | 0.20% | 50% | 98% |
| Class B | 0.15% | 0.15% | 0.06% | 52% | 96% |
| Class C | 0.02% | 0.02% | 0.00% | 46% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.16% | 0.14% | 0.09% | 50% | 97% |
| Class D | 0.09% | 0.07% | 0.06% | 50% | 99% |
| Class F | -0.04% | -0.02% | -0.01% | 48% | 96% |

As shown in simulation results, the solution-1 and solution-2, the coding efficiency impact is negligible. The decoding runtime is almost same as an anchor. For the solution-3, the coding efficiency impact is not negligible although decoding runtime decreases.

Recommendation

Based on the experimental results, it is recommended to adopt the soution-1 or solution-2.

References

[1] Chia-Ming Tsai, Chih-Wei Hsu, Tzu-Der Chuang, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei, “CE1.2.1: Constraint for binary and ternary partitions”, JVET-L0081, the 12th JVET meeting, Oct. 2018, Macao, CN.

[2] J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-L1010, the 12th JVET meeting, Oct. 2018, Macao, CN.

[2] B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 3)”, JVET-L1001-v7, the 12th JVET meeting, the 12th JVET meeting, Oct. 2018, Macao, CN.

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).