| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0073-r2 |
| --- | --- |

| Title: | Non-CE9: On early termination for BDOF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kenji Kondo, Masaru Ikeda, Teruhiko Suzuki 2-10-1 Osaki, Shinagawa-ku, Tokyo Japan | Tel: Email: | +81-50-3750-2744 kenji.kondo@sony.com, Masaru.Ikeda@sony.com, teruhiko.s@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

In this contribution, we introduce solutions to address the increase in hardware complexity due to the computation of the sum of absolute difference (SAD) for early exit in bi-directional optical flow (BDOF). Although the SAD calculation is performed at CU level, it could potentially increase the memory size up to CTU level. We propose to divide the SAD calculation according to the virtual pipeline data unit (VPDU) to avoid this potential increase in memory size. If we divide the SAD calculation from 128x128 to four 64x64 region (solution-1) to reduce memory size, our simulation results show almost no drop in BD performance i.e. 0.00%, 0.01% and -0.01% BD-rate for Y, U and V component in Random Access (RA) configuration.

Problem Statement

In the VTM-3.0, the BDOF technique calculates SAD between a reference L0 and L1 in CU (=PU) level. If the SAD of CU is smaller than threshold, the BDOF process early exits. Figure 1 shows the block diagram for early stage in BDOF process.

Figure 1 Early stage in BDOF process

Where, the block SAD computes a SAD between predictions L0 and L1 in a CU level. L0 and L1 predictions need to be stored since they may be used to process usual bi-prediction if BDOF early exits.  The stored memory size is estimated that two 128x128 buffers for L0 and L1 are needed as shown in Figure 1, because currently a largest CTU size is 128x128 in luma pixel.

On the other hand, applying the VPDU concept [1], the buffer size can be kept up to max transform size of (64x64). Current VTM-3 early termination of BDOF is not taking advantage of VPDU concept, however.

It is possible not to store the predictions after the SAD computation; thus, allowing reduction in the memory size. It could however result in a large processing delay and hardware complexity because of the need to perform and increase throughput for interpolation process in case of BDOF early exit.

Solutions

To avoid storing predictions beyond VPDU size (64x64), solutions are proposed in following sections.

Solution-1

In this solution-1, an early exit region is divided into small size which is smaller than or equal to VPDU size. It’s shown in Figure 2.

Accordingly, the SAD computation is conducted for each region. Then, SAD for each region is checked, and if the value is less than the threshold, it early exits for the divided SAD region.

As shown in the section 3, it was experimentally tested in case of that the SAD size is 64x64, 32x32 and 16x16.

Figure 2: Solution-1

Solution-2

When CU size is larger than VPDU size, it doesn’t use the early termination. Thus, BDOF is always process even if SAD is smaller than the threshold.

Solution-3

When CU size is larger than VPDU size, BDOF is always disabled.

Experimental results

As introduced in section 2, those solutions were implemented in VTM-3.0. On the common test condition [2], the simulations were tried for only RA case because the BDOF works in only RA case.

Simulation environment

For the simulations, same platform machines as below in PC cluster were used.

- Encode

  - OS: CentOS 6.9

  - Compiler: g++ (GCC) 6.3.1

  - CPU: Intel Core i7-7700K 4.2GHz

    - SIMD: AVX2 instruction is enabled.

  - Memory: 32 GB

- Decode

  - OS: CentOS 6.9

  - Compiler: g++ (GCC) 6.3.1

  - CPU: Intel Core i5-2500K 3.3 GHz

    - SIMD: AVX instruction is enabled.

  - Memory: 24 GB

For a runtime measurement, the user time in the output log was used. For a random-access case and an all intra case, a parallel encoding is used. To measure encoding time in parallel encoding, the total of all user time for each frame were calculated.

The summary of test results

Table 1: Test results for Solution-1 64x64

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM 3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.05% | 0.01% | 101% | 100% |
| Class A2 | -0.01% | 0.00% | -0.01% | 102% | 101% |
| Class B | 0.00% | -0.01% | -0.04% | 101% | 101% |
| Class C | 0.00% | 0.00% | 0.01% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.01% | -0.01% | 101% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 102% | 106% |

Table 2: Test results for Solution-1 32x32

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM 3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.03% | 0.02% | 101% | 102% |
| Class A2 | -0.01% | -0.01% | 0.05% | 102% | 102% |
| Class B | 0.00% | 0.01% | -0.03% | 101% | 102% |
| Class C | 0.00% | 0.02% | -0.02% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.01% | 0.00% | 101% | 101% |
| Class D | 0.00% | -0.06% | 0.00% | 100% | 100% |
| Class F | -0.02% | -0.04% | -0.02% | 102% | 107% |

Table 3: Test results for Solution-1 16x16

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM 3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.03% | 0.00% | 101% | 101% |
| Class A2 | -0.02% | -0.02% | 0.02% | 102% | 102% |
| Class B | -0.01% | 0.01% | -0.05% | 101% | 102% |
| Class C | 0.00% | -0.01% | -0.03% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.00% | -0.02% | 101% | 101% |
| Class D | 0.01% | -0.06% | -0.02% | 100% | 101% |
| Class F | -0.01% | -0.03% | -0.02% | 102% | 107% |

Table 4: Test results for Solution-2, BDOF is always processed without an early termination when CU > VPDU size

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM 3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.03% | 0.01% | 101% | 101% |
| Class A2 | -0.03% | -0.01% | 0.05% | 101% | 101% |
| Class B | 0.00% | -0.02% | -0.04% | 100% | 101% |
| Class C | 0.00% | 0.00% | -0.01% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | -0.01% | 100% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class F | -0.01% | -0.01% | -0.01% | 101% | 106% |

Table 5: Test results for Solution-3, BDOF is always disabled when CU > VPDU size

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM 3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.23% | 0.19% | 0.14% | 99% | 95% |
| Class A2 | 0.30% | 0.23% | 0.20% | 100% | 98% |
| Class B | 0.15% | 0.15% | 0.06% | 99% | 96% |
| Class C | 0.02% | 0.02% | 0.00% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.16% | 0.14% | 0.09% | 99% | 97% |
| Class D | 0.09% | 0.07% | 0.06% | 99% | 99% |
| Class F | -0.04% | -0.02% | -0.01% | 100% | 96% |

As shown in simulation results, the solution-1 and solution-2, the coding efficiency impact is negligible. The decoding runtime is almost same as an anchor. For the solution-3, the coding efficiency impact is not negligible although decoding runtime decreases.

Regarding to complexity, the all proposed solutions can allow to reduce memory size that is less than or equal to VPDU size from CTU size as shown in Figure 1.

Recommendation

Based on the experimental results, it is recommended to adopt the soution-1 or solution-2.

References

[1] Chia-Ming Tsai, Chih-Wei Hsu, Tzu-Der Chuang, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei, “CE1.2.1: Constraint for binary and ternary partitions”, JVET-L0081, the 12th JVET meeting, Oct. 2018, Macao, CN.

[2] J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-L1010, the 12th JVET meeting, Oct. 2018, Macao, CN.

[2] B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 3)”, JVET-L1001-v7, the 12th JVET meeting, the 12th JVET meeting, Oct. 2018, Macao, CN.

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).