| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0078 |
| --- | --- |

| Title: | CE9-related: Combination of JVET-M0077 and CE9.2.5 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kyohei Unno Kei Kawamura Sei Naito 2-1-15 Ohara, Fujimino-shi, Saitama Japan Takeshi Chujoh Tomohiro Ikai | Tel: Email: | +81-70-4560-2752 ky-unno@kddi-research.jp ki-kawamura@kddi.com +81-43-299-8526 chujoh.takeshi@sharp.co.jp ikai.tomohiro@sharp.co.jp |
| Source: | KDDI Corp. (KDDI Research, Inc.) Sharp Corporation |  |  |

_____________________________

- Abstract

This contribution presents combination test results of JVET-M0077 and CE9.2.5. CE9.2.5 is an early termination method for decoder-side motion vector refinement (DMVR). JVET-M0077 is an relaxation method of block size restriction for DMVR. BD-rate gain for luma is -0.77% and decoding time is 120% compared with VTM-3.0. The decoding time is accelerated 2% by the early termination with 0.04% luma BD-rate increasing.

Introduction

Core Experiment 9 (CE9) related to decoder-side motion vector derivation is established in 12th meeting at Macao on October, 2018 [1]. This contribution presents combination test results of JVET-M0077 and CE9.2.5 for decoder-side motion vector refinement (DMVR). The proposal is related to CE9.2. The remainder of this contribution is divided into two parts;

- Proposed method, and

- Experimental results under the CTC.

Proposed Method

Figure 1 shows processing flow of the proposed method. In the proposed method, DMVR is processed as follows.

- If a block is larger than 4096 pixels, DMVR is not applied to the block [2][3].

- If the initial matching cost is less than (w*h*(1<<(Max(2, 14 – BitDepth) + (BitDepth-8)+TH)), then the cost is regarded as zero and the DMVR processing is terminated [4]. Here, w is the width of block, h is the height of block, BitDepth is the bit depth of samples and TH is the parameter of early termination.

- If the block is larger than 1024 pixels, only central 1024 pixels region is used for DMVR [3].

Figure 1 Flow chart of proposed method.

Experimental Results

The proposed method is implemented on top of the CE9.2 base software. In order to verify the performance of the proposed method, we have run simulations according to the common test conditions of JVET-L1010 [4].

The PC cluster was used for the simulation, but since the same CPUs are allocated to the same parameters of the same sequences, the execution time is almost accurate. In order to measure more accurate decoding times, the decoder was re-run all the bitstreams on single CPU.

Results under the common test conditions

Table 1 shows the experimental results of the proposed method. It can be seen that BD-rate gain for luma is -0.77% for RA in comparison with VTM-3.0. Table 2 shows overall results comparison of JVET-M0077 and the proposed method (JVET-M0077+ CE9.2.5). It can be seen that decoding time of proposed method is accelerated 2% by the early termination with 0.04% luma BD-rate increasing.

Table 1 Experimental results for RA condition compared with VTM-3.0.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.73% | -0.75% | -0.92% | 109% | 115% |
| Class A2 | -1.12% | -1.22% | -1.21% | 111% | 122% |
| Class B | -0.59% | -0.76% | -0.80% | 111% | 120% |
| Class C | -0.77% | -0.89% | -0.89% | 111% | 123% |
| Class E |  |  |  |  |  |
| Overall | -0.77% | -0.89% | -0.93% | 111% | 120% |
| Class D | -0.72% | -0.93% | -0.90% | 112% | 125% |
| Class F (optional) | -0.30% | -0.47% | -0.43% | 108% | 125% |

Table 2 Overall results of the proposed method and M0077 for RA condition compared with VTM-3.0.

| Method | Y | U | V | EncT (*) | DecT (*) |
| --- | --- | --- | --- | --- | --- |
| JVET-M0077 | -0.81% | -0.90% | -1.00% | 111% | 122% |
| Proposed method (JVET-M0077+ CE9.2.5) | -0.77% | -0.89% | -0.93% | 111% | 120% |

* The all results in Table 2 are tested by Sharp. Therefore, EncT and DecT of JVET-M0077 are the numbers shown in cross-check results by Sharp [6].

Conclusion

This contribution presented combination test results of JVET-M0077 and CE9.2.5. CE9.2.5 was related to an early termination method for decoder-side motion vector refinement (DMVR). JVET-M0077 was an relaxation method of block size restriction for DMVR. BD-rate gain for luma was -0.77% and decoding time was 120% compared with VTM-3.0. The decoding time was accelerated 2% by early termination with 0.04% luma BD-rate increasing.

- Acknowledgment

We would like to thank Panasonic for doing crosschecking of our proposal.

References

- X. Xiu and S. Esenlik, “Description of Core Experiment 9 (CE9): Decoder Side Motion Vector Derivation”, JVET-L1029, Macao, CN, Oct. 2018.

- K. Unno, K. Kawamura, and S. Naito, “CE9: Block size restriction for DMVR (test 9.2.6)”, JVET-M0076, Marrakech, MA, Jan. 2019.

- K. Unno, K. Kawamura, and S. Naito, “CE9-related: Relaxation of block size restriction for DMVR”, JVET-M0077, Marrakech, MA, Jan. 2019.

- T. Chujoh and T. Ikai, “CE9: An early termination of DMVR (Test 9.2.5)”, JVET-M0062, Marrakech, MA, Jan. 2019.

- F. Bossen, J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations”, JVET-L1010, Macao, CN, Oct. 2018.

- T. Chujoh and T. Ikai, “Cross-check result of JVET-M0077”, JVET-Mxxxx, Marrakech, MA, Jan. 2019.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).