| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET- M0104 |
| --- | --- |

| Title: | CE2: Planar Motion Vector Prediction (test 2.3.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Na Zhang Xu Chen Jianhua Zheng Yongbing Lin | Email: | zhangna25@hisilicon.com zhengjianhua@hisilicon.com |
| Source: | HiSilicon Technologies Co., Ltd |  |  |

_____________________________

- Abstract

This contribution reports the results of Core Experiment (CE) 2.3.1 Test (a) & Test (c) for the planar motion vector prediction (PMVP) method. In 2.3.1Test (a), six different sub-block sizes are used. In 2.3.1Test (a)-(1), 8x4 sub-block size is used for bi-prediction, 4x4 sub-block size is used for uni-prediction. It provides 0.11% BD bit-rate saving. In 2.3.1Test (a)-(2), 4x8 sub-block size is used for bi-prediction, 4x4 sub-block size is used for uni-prediction. It provides 0.10% BD bit-rate saving. In 2.3.1Test (a)-(3), adaptive 8x4/4x8 sub-block size is used for bi-prediction, 4x4 sub-block size is used for uni-prediction. It provides 0.11% BD bit-rate saving. In 2.3.1Test (a)-(4), 8x8 sub-block size is used for bi-prediction, 4x4 sub-block size is used for uni-prediction. It provides 0.10% BD bit-rate saving. In 2.3.1Test (a)-(5), adaptive 8x4/4x8 sub-block size is used for bi-prediction and uni-prediction. It provides 0.10% BD bit-rate saving. In 2.3.1Test (a)-(6), 8x8 sub-block size is used for bi-prediction and uni-prediction. It provides 0.09% BD bit-rate saving. In 2.3.1Test (c), PMVP is inserted as a candidate in sub-block (affine) merge list. It provides 0.08% BD bit-rate loss.

Introduction

To generate a smooth fine granularity motion field, the planar motion vector prediction method is proposed in [1].  Planar motion vector prediction is achieved by averaging a horizontal and vertical linear interpolation on 4x4 block basis as follows.

PMVP is available when the width or the height of a block is larger than or equal to 16 and the block is not on the left or above boundary of a picture.

Proposed Method

In 2.3.1Test (a), six different sub-block sizes are used:

In 2.3.1Test (a)-(1), 8x4 sub-block size is used for bi-prediction, 4x4 sub-block size is used for uni-prediction.

In 2.3.1Test (a)-(2), 4x8 sub-block size is used for bi-prediction, 4x4 sub-block size is used for uni-prediction.

In 2.3.1Test (a)-(3), adaptive 8x4/4x8 sub-block size is used for bi-prediction, 4x4 sub-block size is used for uni-prediction.

In 2.3.1Test (a)-(4), 8x8 sub-block size is used for bi-prediction, 4x4 sub-block size is used for uni-prediction.

In 2.3.1Test (a)-(5), adaptive 8x4/4x8 sub-block size is used for bi-prediction and uni-prediction.

In 2.3.1Test (a)-(6), 8x8 sub-block size is used for bi-prediction and uni-prediction.

In 2.3.1Test (c), PMVP is inserted as a candidate in sub-block (affine) merge list. PMVP is inserted after

inherited affine merge candidates. The sub-block (affine) merge list size is 2. 8x8 sub-block size is used.

The sub-block merge candidate list is constructed as following:

1)	Insert ATMVP candidate

2)	Insert inherited affine candidates

3)	Insert planar candidate

4)	Insert constructed affine candidates

5)	Padding with zero motion vectors

Experimental Results

The proposed method has been implemented on VTM-3.0 [2], and all the tests were conducted under the common test conditions [3]. The experimental results of the 2.3.1Test (a) & Test (c) compared to VTM-3.0 are shown in Table1. The experimental results of the 2.3.1Test (a)-(1), 2.3.1Test (a)-(2), 2.3.1Test (a)-(3), 2.3.1Test (a)-(4), 2.3.1Test (a)-(5), 2.3.1Test (a)-(6), 2.3.1Test (c), compared to VTM-3.0 are shown in Table2, Table3, Table4, Table5, Table6, Table7, Table8, separately.

Table 1. Experimental results of the 2.3.1Test (a) & Test (c) compared to VTM-3.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
| Test# | Y | U | V | EncT | DecT |
| Test (a)-(1) | -0.11% | -0.10% | -0.17% | 103% | 103% |
| Test (a)-(2) | -0.10% | -0.18% | -0.18% | 103% | 103% |
| Test (a)-(3) | -0.11% | -0.13% | -0.22% | 103% | 103% |
| Test (a)-(4) | -0.11% | -0.07% | -0.15% | 103% | 101% |
| Test (a)-(5) | -0.10% | -0.15% | -0.18% | 103% | 102% |
| Test (a)-(6) | -0.09% | -0.08% | -0.12% | 103% | 101% |
| Test (c) | 0.08% | 0.12% | 0.06% | 99% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
| Test# | Y | U | V | EncT | DecT |
| Test (a)-(1) | -0.08% | 0.01% | 0.09% | 104% | 103% |
| Test (a)-(2) | -0.05% | 0.03% | 0.01% | 104% | 103% |
| Test (a)-(3) | -0.04% | -0.11% | -0.07% | 104% | 104% |
| Test (a)-(4) | -0.05% | -0.07% | -0.19% | 104% | 102% |
| Test (a)-(5) | -0.06% | 0.01% | 0.02% | 104% | 103% |
| Test (a)-(6) | -0.02% | 0.00% | 0.04% | 104% | 101% |

Table 2. Experimental results of the 2.3.1Test (a)-(1) compared to VTM-3.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vtm-3.0] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.23% | -0.23% | -0.28% | 103% | 106% |
| Class A2 | -0.12% | -0.15% | -0.09% | 103% | 104% |
| Class B | -0.10% | -0.03% | -0.21% | 103% | 102% |
| Class C | -0.04% | -0.07% | -0.09% | 103% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.11% | -0.10% | -0.17% | 103% | 103% |
| Class D | -0.02% | -0.16% | -0.19% | 103% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over [vtm-3.0] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.04% | -0.09% | 0.25% | 104% | 104% |
| Class C | -0.03% | 0.22% | 0.10% | 104% | 102% |
| Class E | -0.21% | -0.13% | -0.21% | 104% | 103% |
| Overall | -0.08% | 0.01% | 0.09% | 104% | 103% |
| Class D | -0.04% | -0.45% | -0.37% | 104% | 101% |

Table 3. Experimental results of the 2.3.1Test (a)-(2) compared to VTM-3.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vtm-3.0] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.20% | -0.37% | -0.36% | 103% | 106% |
| Class A2 | -0.12% | -0.21% | -0.15% | 104% | 104% |
| Class B | -0.08% | -0.06% | -0.21% | 103% | 102% |
| Class C | -0.03% | -0.14% | -0.04% | 103% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.10% | -0.18% | -0.18% | 103% | 103% |
| Class D | -0.06% | -0.11% | -0.11% | 103% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over [vtm-3.0] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -0.16% | -0.07% | 104% | 104% |
| Class C | 0.03% | 0.13% | 0.11% | 104% | 101% |
| Class E | -0.20% | 0.22% | 0.00% | 104% | 104% |
| Overall | -0.05% | 0.03% | 0.01% | 104% | 103% |
| Class D | -0.08% | -0.31% | -0.14% | 104% | 103% |

Table 4. Experimental results of the 2.3.1Test (a)-(3) compared to VTM-3.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vtm-3.0] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.23% | -0.30% | -0.34% | 103% | 106% |
| Class A2 | -0.13% | -0.17% | -0.21% | 103% | 103% |
| Class B | -0.09% | -0.07% | -0.22% | 103% | 103% |
| Class C | -0.02% | -0.07% | -0.13% | 103% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.11% | -0.13% | -0.22% | 103% | 103% |
| Class D | -0.01% | -0.10% | -0.17% | 103% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over [vtm-3.0] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -0.24% | -0.09% | 104% | 106% |
| Class C | 0.02% | 0.19% | 0.04% | 104% | 101% |
| Class E | -0.18% | -0.29% | -0.20% | 104% | 106% |
| Overall | -0.04% | -0.11% | -0.07% | 104% | 104% |
| Class D | -0.06% | 0.12% | 0.30% | 104% | 102% |

Table 5. Experimental results of the 2.3.1Test (a)-(4) compared to VTM-3.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vtm-3.0] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.23% | -0.19% | -0.25% | 103% | 103% |
| Class A2 | -0.11% | -0.12% | -0.13% | 103% | 102% |
| Class B | -0.09% | -0.01% | -0.13% | 103% | 101% |
| Class C | -0.03% | -0.03% | -0.13% | 103% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.11% | -0.07% | -0.15% | 103% | 101% |
| Class D | -0.04% | -0.16% | -0.14% | 103% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over [vtm-3.0] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.36% | -0.10% | 104% | 103% |
| Class C | 0.02% | 0.24% | -0.05% | 104% | 101% |
| Class E | -0.23% | 0.00% | -0.52% | 104% | 103% |
| Overall | -0.05% | -0.07% | -0.19% | 104% | 102% |
| Class D | 0.02% | 0.12% | 0.18% | 104% | 102% |

Table 6. Experimental results of the 2.3.1Test (a)-(5) compared to VTM-3.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vtm-3.0] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.23% | -0.34% | -0.30% | 103% | 106% |
| Class A2 | -0.13% | -0.19% | -0.23% | 103% | 103% |
| Class B | -0.07% | -0.06% | -0.24% | 103% | 102% |
| Class C | -0.02% | -0.09% | 0.00% | 103% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.10% | -0.15% | -0.18% | 103% | 102% |
| Class D | -0.06% | -0.31% | -0.09% | 103% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over [vtm-3.0] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | -0.17% | 0.11% | 104% | 104% |
| Class C | -0.01% | 0.15% | 0.04% | 104% | 101% |
| Class E | -0.20% | 0.15% | -0.15% | 104% | 103% |
| Overall | -0.06% | 0.01% | 0.02% | 104% | 103% |
| Class D | -0.03% | 0.23% | 0.21% | 104% | 103% |

Table 7. Experimental results of the 2.3.1Test (a)-(6) compared to VTM-3.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vtm-3.0] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.22% | -0.28% | -0.22% | 103% | 102% |
| Class A2 | -0.10% | -0.12% | -0.09% | 103% | 101% |
| Class B | -0.07% | 0.01% | -0.16% | 103% | 101% |
| Class C | -0.01% | 0.01% | -0.01% | 103% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.09% | -0.08% | -0.12% | 103% | 101% |
| Class D | -0.04% | 0.03% | -0.17% | 103% | 100% |
| Class F (optional) | 0.03% | -0.01% | 0.02% | 102% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over [vtm-3.0] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -0.26% | -0.05% | 104% | 102% |
| Class C | 0.05% | 0.20% | 0.28% | 104% | 99% |
| Class E | -0.10% | 0.17% | -0.11% | 104% | 101% |
| Overall | -0.02% | 0.00% | 0.04% | 104% | 101% |
| Class D | 0.00% | 0.46% | 0.12% | 104% | 100% |
| Class F (optional) | 0.11% | 0.09% | 0.21% | 103% | 100% |

Table 8. Experimental results of the 2.3.1Test (c) compared to VTM-3.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vtm-3.0] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | -0.06% | -0.11% | 99% | 102% |
| Class A2 | 0.17% | 0.18% | 0.19% | 99% | 100% |
| Class B | 0.08% | 0.17% | 0.09% | 99% | 100% |
| Class C | 0.05% | 0.13% | 0.04% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.08% | 0.12% | 0.06% | 99% | 100% |
| Class D | 0.05% | -0.03% | 0.04% | 99% | 100% |
| Class F (optional) | 0.05% | 0.03% | 0.00% | 100% | 100% |

Conclusion

This contribution proposes a planar motion vector prediction method. It is recommended to adopt Test (c) to the next version of VTM due to better tradeoff between performance and complexity.

Reference

- N. Zhang, Y. Lin, J. Zheng, “CE4.2.14:Planar Motion Vector Prediction,” JVET-K0135, Ljubljana, SI, 10–18 Jul. 2018.

- VTM-3.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-3.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, Macao, CN, 3–12 Oct. 2018.

Patent rights declaration(s)

HiSilicon Technologies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).