| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0109 |
| --- | --- |

| Title: | CE12-related: block-based in-loop luma reshaping |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Edouard François, Christophe Chevance, Franck Hiron | Email: | edouard.francois@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This document describes a block-based implementation of the in-loop luma reshaping approach proposed in contributions JVET-L0246 and JVET-L0247. In-loop luma reshaping requires at the decoder two basic sample mapping operations: forward reshaping of the prediction signal, inverse reshaping of the reconstructed signal. Both operations are by default operated per sample, requiring a per-sample access to a look-up-table. In the proposed approach, the process is performed per block. For each 4x4 block, linear model parameters are computed from three luma values and their mapped version. The linear model is then applied to the block samples. The proposal results in PSNR-Y, U, V BD-rate variations compared to the VTM3.0 of -0.74%, 1.49%, 1.22% in AI configuration, -1.17%, 1.50%, 1.50% in RA configuration, -0.54%, 2.62%, 3.21% in LB configuration.

# Introduction

The in-loop luma reshaper is implemented as a pair of look-up tables (LUTs), one for the forward reshaping, and one for the inverse reshaping. Only one of the two LUTs need to be signaled as the other one can be computed from the signaled LUT. The signalling of reshaping parameters mainly consists of a table of scaling factors sc[i], of size 2K, K being a parameter signalled in the stream. sc[i] corresponds to the slope of the ith segment of a forward LUT FwdLUT, of size 1024 elements, that maps input luma code values Yi to altered values Yr: Yr=FwdLUT[Yi]. Another LUT, InvLUT of size 1024 elements, is the inverse LUT that maps altered code values Yr to Yi : Yi=InvLUT[Yr]. (Yi represents the reconstruction values of Yi.)

To perform the mapping (either forward reshaping, or inverse reshaping), a per-sample sample access to a 1D-LUT is required, which may be an issue for practical implementations. The solution aims at addressing this problem.

# Proposed simplification

The proposed solution consists in performing the mapping (either forward reshaping, or inverse reshaping) on a block basis.

For each NxN block, the linear parameters are computed as the parameters of the derivative of the LUT around the average value of the block. These parameters are then applied to all samples of the block to perform the mapping.

Let pLUT be the used look-up-table, and BD the luma bit-depth.

The overall process is achieved as follows.

- Computation of the average luma over the block avgY

- Computation of the linear parameters a and b by the following steps:

  - minY = Clip3(0, (2BD – 1) - 2 × D, avgY - D)

  - a = pLUT[minY + 2 × D] - pLUT[minY]

  - avgY = Clip3(0, (2BD – 1), avgY)

  - b = pLUT[avgY] × 2 × D - avgY × a + D

- Mapping each sample Yin[p] at location p in the block using a and b as follows:

  - Ymap[p] = Clip3(minMap, maxMap, (a * Yin[p] + b) >> shiftD)

shiftD is a pre-defined parameter, and D is defined as (1<<shiftD). Also minMap = pLUT[0] and maxMap = pLUT[2BD – 1].

Instead of requiring NxN accessing to the LUT, 3 accesses are needed to compute pLUT[minY + 2 × D] - pLUT[minY] and pLUT[avgY].

# Results

The best results were obtained using block size of 4x4 (N=4) and a precision of 5 (shiftD = 5, D = 32).

For content with abrupt local variations (such as some screen content or class E sequences), the block-based approach may lead to spatial varying linear parameters leading to bad temporal predictions. A slight change of the encoding process has been made to disable the reshaping for content where abrupt local variations (estimated using the local variance) are observed.

The results are depicted in the following tables, compared to VTM3.0 (no reshaping) and to CE12-2 (reshaping per sample). Most of the gain of the reshaping is preserved. The average PSNR-Y BDR gain compared to VTM3.0 is of 0.74% in AI, 1.17% in RA, 0.54% in LB. The average PSNR-Y BDR loss compared to CE12-2 is of 0.26% in AI, 0.13% in RA, 0.22% in LB.

Compared to VTM3.0 (without reshaping)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.56% | 0.32% | -0.93% | 102% | 101% |
| Class A2 | -1.53% | 1.27% | 1.39% | 101% | 100% |
| Class B | -0.60% | 1.20% | 0.78% | 102% | 101% |
| Class C | -0.28% | 2.43% | 2.53% | 101% | 100% |
| Class E | 0.03% | 2.13% | 2.18% | 101% | 101% |
| Overall | -0.74% | 1.49% | 1.22% | 101% | 101% |
| Class D | -0.11% | 1.63% | 1.49% | 101% | 101% |
| Class F (optional) | -0.12% | 1.43% | 1.51% | 102% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.83% | 2.74% | 2.74% | 101% | 102% |
| Class A2 | -1.20% | 2.71% | 2.83% | 101% | 102% |
| Class B | -1.44% | 0.31% | -0.20% | 100% | 100% |
| Class C | -1.08% | 1.15% | 1.71% | 101% | 100% |
| Overall | -1.17% | 1.50% | 1.50% | 101% | 101% |
| Class D | -0.94% | 2.08% | 2.22% | 101% | 102% |
| Class F (optional) | -0.76% | -0.59% | -0.41% | 102% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -1.01% | 2.34% | 2.17% | 102% | 100% |
| Class C | -0.39% | 2.13% | 2.17% | 101% | 101% |
| Class E | 0.02% | 0.02% | 0.02% | 102% | 102% |
| Overall | -0.54% | 1.69% | 1.63% | 102% | 101% |
| Class D | -0.79% | 2.62% | 3.21% | 100% | 100% |
| Class F (optional) | 0.26% | 1.11% | 1.09% | 100% | 101% |

Compared to CE12-2 configuration (reshaping per sample)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.14% | 0.14% | 0.37% | 99% | 98% |
| Class A2 | 0.21% | 0.02% | 0.03% | 100% | 98% |
| Class B | 0.14% | -0.05% | -0.10% | 99% | 98% |
| Class C | 0.10% | 0.02% | 0.01% | 99% | 99% |
| Class E | 0.23% | -0.07% | 0.01% | 99% | 98% |
| Overall | 0.16% | 0.01% | 0.04% | 99% | 98% |
| Class D | 0.31% | -0.03% | 0.02% | 99% | 98% |
| Class F (optional) | 0.71% | -0.31% | -0.23% | 99% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.11% | -0.07% | 0.00% | 100% | 98% |
| Class A2 | 0.08% | 0.15% | 0.03% | 99% | 98% |
| Class B | 0.18% | 0.05% | -0.06% | 99% | 99% |
| Class C | 0.10% | 0.05% | 0.30% | 100% | 98% |
| Overall | 0.13% | 0.05% | 0.07% | 99% | 98% |
| Class D | 0.15% | 0.03% | 0.02% | 99% | 99% |
| Class F (optional) | 0.68% | -0.44% | -0.35% | 99% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | 0.26% | 0.08% | 0.04% | 99% | 99% |
| Class C | 0.18% | -0.04% | -0.04% | 99% | 98% |
| Class E | 0.22% | -0.46% | 0.86% | 99% | 98% |
| Overall | 0.22% | -0.09% | 0.22% | 99% | 99% |
| Class D | 0.26% | 0.28% | 0.24% | 99% | 99% |
| Class F (optional) | 1.03% | -0.37% | -0.35% | 99% | 99% |

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).