| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0127 |
| --- | --- |

| Title: | CE4-related: Modification on Merge List |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu Han, Wei-Jung Chien, Dmytro Rusanovskyy, Yung-Hsuan Chao, Chun-Chi Chen, Hongtao Wang, Han Huang, Chao-Hsiung Hung, Yan Zhang, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA, U.S.A. 92121 | Tel: Email: | +1-858-845-1793 yuhan@qti.qualcomm.com wchien@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution presents a change on merge list which reduces the number of pairwise candidates from 6 to 1 and remove the dependency between pairwise candidates and HMVP candidates. For the “VTM3.0 as reference” tests in RA case with the size of merge list equal to 6, the presented simplification method achieved -0.02% coding gain.

# Introduction

For the skip mode and the merge mode, a motion vector competition scheme is introduced to select a motion candidate from a given candidate set that includes spatial and temporal motion candidates. In the case of a skip mode, the residual signal is omitted. Only merge indices are transmitted for the block coded in merge or skip mode, and the current block inherits the inter prediction indicator, reference indices, and motion vectors from a neighboring block referred by the coded merge index.

In this contribution, the number of pairwise candidates are reduced from 6 to 1.

# Technical description

## Merge list in VTM3.0

In VTM3.0, the size of merge list is 6, the order of merge candidate list is as below:

- Spatial candidates for blocks A1, B1, B0 and A0.

- If number of candidates less than 4, add B2.

- TMVP candidate.

- HMVP candidates (check 5).

- Pairwise candidates (check 6).

- Zero candidates.

Figure 1: Merge candidates positions

Pairwise candidates are generated based on the merge candidates in the merge list which introduces dependency between pairwise candidates and HMVP candidates. The number of checking pairs of pairwise candidates are defined as {(0, 1), (0, 2), (1, 2), (0, 3), (1, 3), (2, 3)}, which is not necessary as the simulation results.

## Modification on merge list

In this contribution, the number of checking pair of pairwise candidates is reduced from 6 to 1. Only the combination of {(0, 1)} will be checked. Insert the pairwise candidate in front of HMVP. Remove the restriction of HMVP candidates can not be the last candidates in the merge list.

The order of proposed merge candidate list is as below (modifications are highlight with underline):

- Spatial candidates for blocks A1, B1, B0 and A0.

- If number of candidates less than 4, add B2.

- TMVP candidate.

- Pairwise candidates (check 1).

- HMVP candidates (check 6).

- Zero candidates.

# Simulation results

As descripted in CE4 [1], the simulation is based on VTM3.0 with the size of merge list equal to 6.

# Conclusion

This contribution presents a change on merge list which reduces the number of pairwise candidates from 6 to 1 and removes the dependency between pairwise candidates and HMVP candidates. For the “VTM3.0 as reference” tests in RA case with the size of merge list equal to 6, the presented simplification method achieved -0.02% coding gain.

# References

- H. Yang, S. Liu, K. Zhang, JVET-L1024, “Description of CE4: Inter prediction and motion vector coding”, Macao, October 2018.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).