| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0131-v1 |
| --- | --- |

| Title: | AHG17: On NAL unit types for IRAP pictures and leading pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Hendry 10180 Telesis Court San Diego, CA, 92121, USA | Tel: Email: | +1-908-803-3888 yekui.wang@huawei.com +1-858-375-8065 fnu.hendry@huawei.com |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution proposes the following two aspects regarding NAL unit types for IRAP pictures and leading pictures:

- For IRAP pictures, same as in JVET-M0101, three NAL unit types are used, two for IDR pictures (IDR_N_LP and IDR_W_RADL), and one for CRA pictures (CRA_NUT), but with the semantics of CRA pictures clarified as having at least one associated RASL picture. In HEVC, CRA pictures may or may not have associated RASL pictures.

- For leading pictures, no NAL unit type is assigned, rather, a single NAL unit type is used for trailing pictures and both types of leading pictures (RASL and RADL). This is option 3 as discussed in Subsection 5.3.3.4 of JVET-M0101.

It asserted that, provided that there were complaints of VCL NAL unit types specified in HEVC, and it is desirable to have the same systems operations in identifying different types of IRAP pictures, the proposal provides a better design trade-off.

# Introduction

For simplicity, Subsection 5.2 of JVET-M0101 proposes the following VCL NAL unit types for VVC:

| nal_unit_type | Name of nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| 0 | TRAIL | Coded tile group of a non-STSA trailing picture tile_group_layer_rbsp( ) | VCL |
| 1 | STSA | Coded tile group of an STSA picture tile_group_layer_rbsp( ) | VCL |
| 2..7 | RSV_VCL_2.. RSV_VCL_7 | Reserved non-IRAP VCL NAL unit types | VCL |
| 8 9 | IDR_W_RADL IDR_N_LP | Coded tile group of an IDR picture tile_group_layer_rbsp( ) | VCL |
| 10 | CRA_NUT | Coded tile group of a CRA picture tile_group_layer_rbsp( ) | VCL |
| 11 12 13 | RSV_IRAP_VCL11 RSV_IRAP_VCL12 RSV_IRAP_VCL13 | Reserved IRAP VCL NAL unit types | VCL |
| 14..15 | RSV_VCL14 RSV_VCL15 | Reserved non-IRAP VCL NAL unit types | VCL |

Subsection 5.3.3 of JVET-M0101 includes an extensive discussion on leading picture NAL unit types, with the following three options:

- Two NAL unit types, RADL and RASL, as in HEVC

- One leading picture NAL unit type

- A single NAL unit type for trailing pictures and both types of leading pictures

We suggest that the CRA picture concept here should be restricted to have at least one associated RASL picture, and a single NAL unit type should be used trailing pictures and both types of leading pictures, based on the following arguments:

- When each CRA picture is restricted to have at least one associated RASL picture, CRA pictures can be simply mapped to the appropriate SAP type (SAP 3, see Subsection 5.3.3.2 of JVET-M0101) by checking just the NAL unit type, same as for the two IDR types, without the need of further checking the leading pictures.

Of course, this would mean that a bitstream splicer appending a bitstream starting with a CRA picture associated with RASL pictures to the end of another bitstream would have to either leave the associated RASL pictures in the combined bitstream, or change the NAL unit type of the NAL units of the CRA picture from CRA_NUT to with IDR_W_RADL or IDR_N_LP while removing the associated RASL pictures.

- If this restriction is not applied, to allow such bitstream splicing with the removal of the associated RASL pictures without changing the NAL unit type in a conforming way, bitstream conformance for both situations must be specified; For that, HRD that includes two alternative sets of HRD parameters, as in HEVC, would be needed. As we know, that part of the HEVC specification is pretty complicated, and we think it is not necessary to specify such HRD operations with two alternative sets of HRD parameters defined for the reason described in the next bullet item.

- System-level removal of RASL pictures is not easy to perform and usually not done, because RASL pictures and their associated CRA picture are usually encapsulated in the same Media Segment. On the other hand, in HTTP based adaptive streaming, enabling requesting data picture by picture and such requests themselves are expensive in terms of signalling overhead and huge number HTTP requests, respectively.

- For composing an ISOBMFF file from a given bitstream, the operation of identifying IRAP picture types for mapping to ISOBMFF sync samples and SAP types (see the discussions in Subsection 5.3.3 of JVET-M0101) is fundamental and important. On the other hand, generation and inclusion of the optional SampleDependencyTypeBox (see Subsection 5.3.3.3 of JVET-M0101), which includes indications of leading picture types, is way less important, less useful, and rarely done anyway. Without using NAL unit types specifically for leading pictures, a file composer can either skip generation of the SampleDependencyTypeBox, or bear the burden of implementing parsing of tile group headers for identify the leading picture types.

In conclusion, when CRA pictures are allowed to have no associated RASL pictures, mapping CRA pictures to SAP types would require checking not only NAL unit types of the CRA picture, but also checking the types of the associated leading pictures. This can be cumbersome, even when one or two NAL unit types are used specifically for the two types of leading pictures, as sometimes all the leading pictures associated with a CRA picture need to be checked. Plus, more complicated HRD specification is needed to make sense of allowing CRA pictures to have no associated RASL pictures.

Therefore, restricting CRA pictures to have at least one associated RASL picture is a reasonable trade-off for VVC.

Regarding signalling of leading picture types, leading pictures are usually not discarded in HTTP adaptive streaming anyway, while composing the optional ISOBMFF SampleDependencyTypeBox is not important, and once IRAP picture types can be easily identified through NAL unit types only, there is no strong need of using specific NAL unit types for leading pictures.

# Proposal

Based on the above discussions, it is proposed that in VVC CRA pictures are restricted to have at least one associated RASL picture, and a single NAL unit type for trailing pictures and both types of leading pictures.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).