| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0143-v1 |
| --- | --- |

| Title: | CE13: Face row based geometry padding using projection with bilinear interpolation based on test 1.1.a (Test 2.1.b) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philippe Hanhart, Yuwen He 9276 Scranton Rd, #300 San Diego, CA 92121, USA | Tel: Email: | Philippe.Hanhart@interdigital.com Yuwen.He@interdigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This contribution reports simulation results of CE13 test 2.1.b. For inter prediction, geometry padding of reference pictures is used to pad each face row instead of using repetitive padding at the picture boundaries. Bilinear interpolation is applied for the luminance and chrominance components. Results based on end-to-end WS-PSNR show that test 2.1.b provides {Y, U, V} BD-rate savings of {-1.97%, -2.49%, -2.38%}, {-1.96%, -2.21%, -2.25%}, and {-1.40%, -1.92%, -2.02%} over PHEC, test 1.1.b, and test 1.1.a, respectively.

# Introduction

Face discontinuities in the frame packed picture may yield to seam artifacts in reconstructed viewports. One of the goals of CE13 is to study methods to remove subjective artifacts at face boundaries by using normative decoding tools. The tested coding tools include modifications to in-loop filtering and inter prediction, as well as projection format adaptation. This contribution reports the coding performance of face row based geometry padding using projection with bilinear interpolation on top of test 1.1.a (HEC with In-loop filters disabled across face discontinuities), as studied in CE13 test 2.1.b [1].

# Technical description

The inter prediction replaces the replication padding with the geometry padding when the reference pixels are outside the frame boundary (layout boundary) or across the discontinuous edge(s). The geometry padding in Figure 1 demonstrates a pixel position qb outside the Bottom face is represented by its spherical corresponding pixel position qa, which is located at the Front face. This spherical corresponding pixel is obtained by the projection mapping functions. When the derived coordinate qa is fractional, bilinear interpolation is applied for the luminance and chrominance components.

Figure 1. Geometry padding process.

# Simulation results

The software is implemented on top of test 1.1.a (HEC with in-loop filters disabled across face discontinuities) using the CE13 common codebase, which uses VTM-3.0 and 360Lib-8.0. The encoding was done under 360° video CTC [2]. The following tables only report the end-to-end WS-PSNR results. Results for the other metrics can be found in the accompanying Excel spreadsheet.

Results for test 2.1.b over PHEC, test 1.1.b (PHEC with in-loop filters disabled across face discontinuities), and test 1.1.a (HEC with in-loop filters disabled across face discontinuities) are reported in Table 1, Table 2, and Table 3, respectively.

Table 1. BD-rate for test 2.1.b over PHEC.

|  |  | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| --- | --- | --- | --- | --- |
| Class S1 | SkateboardInLot | -3.23% | -3.88% | -3.70% |
|  | ChairLift | -3.42% | -3.75% | -3.40% |
|  | KiteFlite | -0.69% | -0.42% | -0.40% |
|  | Harbor | -0.59% | -0.21% | -0.38% |
|  | Trolley | -1.06% | -0.77% | -0.77% |
|  | GasLamp | -0.81% | -0.60% | -0.65% |
| Class S2 | Balboa | -2.78% | -4.43% | -3.59% |
|  | Broadway | -2.11% | -3.34% | -3.16% |
|  | Landing2 | -2.19% | -4.13% | -3.99% |
|  | BranCastle2 | -2.84% | -3.40% | -3.81% |
| Class S1 |  | -1.63% | -1.60% | -1.55% |
| Class S2 |  | -2.48% | -3.83% | -3.63% |
| Overall |  | -1.97% | -2.49% | -2.38% |
| Enc Time[%] |  | 100% |  |  |
| Dec Time[%] |  | 109% |  |  |

Table 2. BD-rate for test 2.1.b over test 1.1.b.

|  |  | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| --- | --- | --- | --- | --- |
| Class S1 | SkateboardInLot | -3.36% | -3.40% | -4.04% |
|  | ChairLift | -3.35% | -3.64% | -3.30% |
|  | KiteFlite | -0.66% | -0.44% | -0.28% |
|  | Harbor | -0.49% | 0.12% | -0.29% |
|  | Trolley | -1.02% | -0.65% | -0.53% |
|  | GasLamp | -0.74% | -0.44% | -0.46% |
| Class S2 | Balboa | -2.82% | -4.03% | -3.65% |
|  | Broadway | -2.17% | -3.27% | -3.04% |
|  | Landing2 | -2.15% | -2.80% | -3.02% |
|  | BranCastle2 | -2.86% | -3.56% | -3.91% |
| Class S1 |  | -1.60% | -1.41% | -1.48% |
| Class S2 |  | -2.50% | -3.41% | -3.40% |
| Overall |  | -1.96% | -2.21% | -2.25% |
| Enc Time[%] |  | 100% |  |  |
| Dec Time[%] |  | 105% |  |  |

Table 3. BD-rate for test 2.1.b over test 1.1.a.

|  |  | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| --- | --- | --- | --- | --- |
| Class S1 | SkateboardInLot | -3.02% | -3.57% | -4.23% |
|  | ChairLift | -2.96% | -3.40% | -3.03% |
|  | KiteFlite | -0.05% | -0.09% | -0.05% |
|  | Harbor | -0.02% | -0.04% | -0.09% |
|  | Trolley | -0.22% | -0.33% | -0.47% |
|  | GasLamp | -0.08% | -0.08% | -0.04% |
| Class S2 | Balboa | -2.41% | -3.88% | -3.73% |
|  | Broadway | -1.80% | -2.79% | -2.96% |
|  | Landing2 | -1.50% | -2.19% | -2.36% |
|  | BranCastle2 | -1.99% | -2.84% | -3.26% |
| Class S1 |  | -1.06% | -1.25% | -1.32% |
| Class S2 |  | -1.93% | -2.92% | -3.08% |
| Overall |  | -1.40% | -1.92% | -2.02% |
| Enc Time[%] |  | 100% |  |  |
| Dec Time[%] |  | 105% |  |  |

BD-rate savings over PHEC, test 1.1.b, and test 1.1.a are around {-1.97%, -2.49%, -2.38%}, {-1.96%, -2.21%, -2.25%}, and {-1.40%, -1.92%, -2.02%}, respectively. Bit rate savings are particularly noticeable for moving camera content, i.e., SkateboardInLot, ChairLift, Balboa, BranCastle2, Broadway, and Landing2. For these sequences, the gain is between -1.50% and -3.02% compared to test 1.1.a. For static camera content, i.e., KiteFlite, Harbor, Trolley, and GasLamp, the gain is below -0.22% compared to test 1.1.a. The encoding complexity of face row based geometry padding using projection with bilinear interpolation is similar to that of PHEC and HEC. However, the decoding complexity is increased by 5% compared to PHEC and HEC due to geometry padding.

Results for bilinear interpolation (test 2.1.b) over Lanczos 3/2 interpolation (test 2.1.a) are reported in Table 4.

Table 4. BD-rate for bilinear interpolation (test 2.1.b) over Lanczos 3/2 interpolation (test 2.1.a).

|  |  | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| --- | --- | --- | --- | --- |
| Class S1 | SkateboardInLot | 0.19% | -0.05% | 0.42% |
|  | ChairLift | 0.05% | -0.08% | 0.03% |
|  | KiteFlite | -0.01% | -0.01% | -0.01% |
|  | Harbor | 0.01% | 0.00% | -0.11% |
|  | Trolley | -0.01% | -0.02% | -0.01% |
|  | GasLamp | 0.00% | 0.03% | 0.05% |
| Class S2 | Balboa | 0.01% | -0.38% | 0.16% |
|  | Broadway | 0.03% | 0.22% | -0.37% |
|  | Landing2 | -0.01% | 0.03% | -0.19% |
|  | BranCastle2 | 0.01% | 0.09% | 0.00% |
| Class S1 |  | 0.04% | -0.02% | 0.06% |
| Class S2 |  | 0.01% | -0.01% | -0.10% |
| Overall |  | 0.03% | -0.02% | 0.00% |
| Enc Time[%] |  | - |  |  |
| Dec Time[%] |  | 90% |  |  |

Simulation results show that bilinear interpolation provides similar performance as Lanczos 3/2 interpolation while reducing decoding complexity by about 10%.

# Conclusion

This contribution reports simulation result of CE13 test 2.1.b, which evaluates face row based geometry padding using projection with bilinear interpolation on top of test 1.1.a. The results show BD-rate savings of {-1.97%, -2.49%, -2.38%}, {-1.96%, -2.21%, -2.25%}, and {-1.40%, -1.92%, -2.02%} over PHEC, test 1.1.b, and test 1.1.a, respectively.

# References

- P. Hanhart, J.-L. Lin, C. Pujara, “Description of Core Experiment 13 (CE13): Coding tools for 360° omnidirectional video”, JVET-L1033, Oct. 2018, Macau, CN.

- P. Hanhart, J. Boyce, K. Choi, J.-L. Lin “JVET common test conditions and evaluation procedures for 360° video”, JVET-L1012, Oct. 2018, Macau, CN.

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).