| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0169-v1 |
| --- | --- |

| Title: | CE3-related: Shared reference samples for multiple chroma intra CBs |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhi-Yi Lin, Tzu-Der Chuang, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {zhi-yi.lin, peter.chuang, chingyeh.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution presents a method to enable parallel generation of multiple chroma intra coding blocks (CBs) within a parallel region of size larger than or equal to 16 chroma samples. In the proposed method, parallel regions are first determined, and all of the chroma blocks within the same parallel region use the reference samples that belong to neighbouring blocks of the parallel region and are at the top and left boundaries of the parallel region for chroma intra prediction. It is reported that the proposed method results in 0.04%/0.23%/0.31%, 0.07%/0.43%/0.54%, and 0.07%/0.38%/0.27% Y/Cb/Cr BD-rates with 100%,  96%, and 101% of encoding time and 102%, 100%, and 102% of decoding time for VTM3.0-AI, VTM3.0-RA, and VTM3.0-LB, respectively. It is asserted that the worst case processing throughput for chroma intra prediction can be increased to 4X of that in VTM3.0.

# Introduction

In typical hardware video encoders and decoders, processing throughput drops when a picture has more small blocks. Such processing throughput drop mainly comes from small intra blocks because small inter blocks can be processed in parallel while intra blocks has data dependency between neighbouring blocks (e.g. the predictor generation of an intra block requires top and left boundary reconstructed samples from neighbouring blocks) and have to be processed sequentially.

In HEVC, the worst case processing throughput occurs when 4x4 chroma intra blocks are processed. In VTM3.0, the size of the smallest chroma intra block is 2x2, and the reconstruction process of a chroma intra block becomes very complex after adopting tools including cross-component linear model (CCLM), 4-tap interpolation filters, position-dependent intra prediction combination (PDPC), and combined inter intra prediction (CIIP). The worst case processing throughput in VVC is only 1/4 of that in HEVC. In this contribution, a method that improves the worst case processing throughput is proposed.

# Proposed method

The proposed concept is to use shared reference samples for chroma intra blocks within a parallel region. All of the chroma blocks in the same parallel region have no data dependency on neighbouring blocks within the parallel region and hence can be processed in parallel. To achieve this, parallel regions are first determined. When a chroma block of size larger than or equal to TH chroma samples has any children block of size less than TH chroma samples or is not further split, the chroma block of size larger than or equal to TH chroma samples is treated as a parallel region. Two examples of the parallel regions with TH=16 are shown in Figure 1. In Figure 1 (a), the green block of size equal to 32 chroma samples is set as a parallel region because the size of the first or third partition of the ternary tree (TT) split is less than 16 chroma samples. In Figure 1 (b), the two green blocks of size equal to 16 chroma samples are set as two parallel regions. It can be observed that the condition for parallel region determination may introduce 2TH-chroma-smaple parallel regions and TH-chroma-sample parallel regions because of the TT partition.

(a)

(b)

Figure 1.  Examples of parallel regions

After determining the parallel regions, all of the chroma intra blocks within each parallel region use reference samples that belong to neighbouring blocks of the parallel region and are at the top and left boundaries of the parallel region for generation of chroma intra prediction samples. Therefore, the data dependency between the chroma blocks within the parallel region is removed, and all of the chroma intra prediction blocks within the parallel region can be calculated in parallel. By applying the proposed method, the worst case processing throughput occurs when 4x4, 2x8, or 8x2 chroma blocks, instead of a 2x2 chroma blocks, are processed. The worst case processing throughput is the same as that in HEVC and is 4X of that in VTM3.0.

# Experiment results

The proposed method has been implemented on top of VTM3.0 [1]. The common test conditions [2] are used for evaluation. The results are summarized in Table 1.

Table1.  Results of shared reference samples for chroma intra blocks with TH=16.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.17% | 0.09% | 96% | 101% |
| Class A2 | 0.06% | 0.21% | 0.18% | 97% | 99% |
| Class B | 0.01% | 0.13% | 0.23% | 98% | 101% |
| Class C | 0.09% | 0.63% | 0.84% | 105% | 106% |
| Class E | 0.01% | -0.03% | 0.06% | 106% | 107% |
| Overall | 0.04% | 0.23% | 0.31% | 100% | 102% |
| Class D | 0.12% | 0.96% | 1.24% | 107% | 111% |
| Class F | 0.11% | 0.68% | 0.93% | 100% | 101% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.14% | 0.19% | 95% | 112% |
| Class A2 | 0.05% | 0.26% | 0.30% | 94% | 101% |
| Class B | 0.04% | 0.42% | 0.30% | 98% | 97% |
| Class C | 0.14% | 0.81% | 1.28% | 96% | 95% |
| Class E |  |  |  |  |  |
| Overall | 0.07% | 0.43% | 0.54% | 96% | 100% |
| Class D | 0.21% | 1.08% | 1.38% | 98% | 101% |
| Class F | 0.17% | 0.76% | 0.87% | 97% | 98% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | 0.02% | 0.16% | 101% | 102% |
| Class C | 0.14% | 0.63% | 0.81% | 102% | 101% |
| Class E | 0.05% | 0.65% | -0.27% | 101% | 103% |
| Overall | 0.07% | 0.38% | 0.27% | 101% | 102% |
| Class D | 0.14% | 0.56% | 0.41% | 101% | 101% |
| Class F | 0.06% | 0.77% | 0.89% | 101% | 101% |

# Conclusions

In this proposal, a method that enables parallel chroma intra prediction within a parallel region of size larger than or equal to 16 chroma samples is proposed. It is reported that the proposed method results in 0.04%/0.23%/0.31%, 0.07%/0.43%/0.54%, and 0.07%/0.38%/0.27% Y/Cb/Cr BD-rates with 100%, 96%, and 101% of encoding time and 102%, 100%, and 102% of decoding time for VTM3.0-AI, VTM3.0-RA, and VTM3.0-LB, respectively.

# References

[1] VTM3.0: https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-L1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).