| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0177-v1 |
| --- | --- |

| Title: | CE10.1.4: Simplification of combined inter and intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Documents to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Man-Shu Chiang, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {man-shu.chiang, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to simplify combined inter and intra prediction (CIIP). The concept is to disable CIIP for coding units (CUs) with luma coding block (CB) sizes larger than 1024 luma samples for reducing intra prediction buffer size when CIIP is applied. It is reported that, compared to VTM3.0, the proposed simplification method results in 0.01% and 0.04% luma BD-rates with 1% and 0% encoding time decreases for RA and LB, respectively.

# Introduction

CIIP [1] is to combine one intra prediction and one merge indexed prediction. In a merge CU, one flag is signalled for merge mode to select an intra mode from an intra candidate list when the flag is true. For luma component, the intra candidate list is derived from 4 intra prediction modes including DC, planar, horizontal, and vertical modes, and the size of the intra candidate list can be 3 or 4 depending on the block shape. When the CU width is larger than the double of CU height, horizontal mode is exclusive of the intra mode list and when the CU height is larger than the double of CU width, vertical mode is removed from the intra mode list. One intra prediction mode selected by the intra mode index and one merge indexed prediction selected by the merge index are combined using weighted averaging. For chroma components, direct mode (DM) is always applied without extra signalling. The weights for combining predictions are described as follow. When DC or planar mode is selected or the CB width or height is smaller than 4, equal weights are applied. For those CBs with CB width and height larger than or equal to 4, when horizontal/vertical mode is selected, one CB is first vertically/horizontally split into four equal-area regions. Each weight set, denoted as (w_intrai, w_interi), where i is from 1 to 4 and (w_intra1, w_inter1) = (6, 2), (w_intra2, w_inter2) = (5, 3), (w_intra3, w_inter3) = (3, 5), and (w_intra4, w_inter4) = (2, 6), will be applied to a corresponding region. (w_intra1, w_inter1) is for the region closest to the reference samples and (w_intra4, w_inter4) is for the region farthest away from the reference samples. Then, the combined prediction can be calculated by summing up the two weighted predictions and right-shifting 3 bits. Moreover, the intra prediction mode for the intra hypothesis of predictors can be saved for reference by the following neighbouring CUs.

# Proposed methods

CIIP is disabled for CUs with luma coding block (CB) sizes larger than 1024 luma samples. When CIIP is disabled, in a merge CU, the CIIP flag does not need to be signalled and is inferred to be 0.

# Experimental results

The proposed simplification method has been implemented on VTM3.0 [2]. Table 1 shows the results of CE10.1.4. The simulations were performed following the JVET common test conditions (CTC) [3].

Table 1.  Results of CE10.1.4: Simplification of combined inter and intra prediction

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | -0.09% | 0.05% | 99% | 103% |
| Class A2 | 0.00% | 0.02% | 0.07% | 99% | 104% |
| Class B | -0.02% | 0.05% | 0.00% | 99% | 102% |
| Class C | 0.00% | 0.05% | 0.09% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.02% | 0.05% | 99% | 102% |
| Class D | 0.01% | -0.02% | 0.03% | 100% | 100% |
| Class F (optional) | 0.00% | 0.00% | 0.03% | 99% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.26% | 0.27% | 100% | 100% |
| Class C | 0.03% | 0.21% | 0.16% | 100% | 100% |
| Class E | 0.09% | 0.00% | -0.13% | 99% | 100% |
| Overall | 0.04% | -0.04% | 0.13% | 100% | 100% |
| Class D | 0.02% | 0.37% | -0.17% | 100% | 101% |
| Class F (optional) | 0.05% | 0.27% | -0.18% | 100% | 100% |

# Conclusions

This contribution proposes to simplify CIIP. Compared against VTM3.0, the proposed simplification mechanism (CE10.1.4) results in very minor BD-rate change with 1% and 0% encoding time decreases for RA and LB, respectively and can reduce the requirement of intra prediction buffer size to 1024 luma samples when CIIP is applied.

# References

- M.-S. Chiang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei, “CE10.1.1: Multi-hypothesis prediction for improving AMVP mode, skip or merge mode, and intra mode,” Document of Joint Video Experts Team, JVET-L0100.

- VTM3.0: https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-3.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-L1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).