| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0184-v1 |
| --- | --- |

| Title: | CE10-related: Simplification of triangle merging candidate list derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {peter.chuang, chingyeh.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In VTM3.0 triangle prediction, a triangle merging candidate list is introduced. In the triangle merging candidate list derivation, at most 94 times of motion vector (MV) pruning, 7 times of MV averaging, and 7 times of MV scaling are required. To reduce the complexity, it is proposed to discard the current triangle merging candidate list and reuse the existing merging candidate list of non-triangle prediction to generate a uni-prediction MV for each of the two triangular partitions. In the proposed triangle prediction, two merging candidates from the existing merging candidate list are first selected without syntax change (i.e., still using one merge_triangle_idx to select one candidate for each of the two triangular partitions). Next, to ensure uni-prediction for each triangular partition, if the first merging candidate is bi-prediction, the list 0 (L0) MV is kept for the first triangular partition, and the list 1 (L1) MV is discarded; similarly, if the second merging candidate is bi-prediction, the L1 MV is kept for the second triangular partition, and the L0 MV is discarded. Finally, an offset MV of (1/4 pel, 0) is added to the merging candidate of the second partition if the two merging candidates of the two triangular partitions are the same. In this way, the complexity of the proposed triangle merging candidate list derivation is kept as low as that of the existing merging candidate list derivation plus only two inter prediction indicator (inter_pred_idc, to indicate L0, L1, or bi-prediction) comparisons, one MV comparison, and one MV addition. Simulation results reportedly show that the average luma BD-rates are 0.02% and 0.04% in RA and LB cases, respectively.

# Introduction

In VTM3.0, triangle prediction is adopted. In the triangle prediction, a skip or merge coding unit (CU) is divided into two triangular partitions. Each triangular partition is inter-predicted using its own uni-prediction MV and reference index which are derived from a new triangle merging candidate list of five uni-prediction candidates. In the triangle merging candidate list derivation, the MVs of seven neighbouring blocks including five spatial neighbouring blocks (block-1 to block-5 as shown in Figure 2) and two temporal collocated blocks (block-6 and block-7) are used. A full pruning process is applied to spatial candidates to remove redundant candidates. At most 20 times of MV pruning is required at this candidate pruning stage. The MVs of the pruned candidates are collected and put into the triangle merging candidate list as follows. The uni-prediction MVs are put into the candidate list first and followed by the L0 MVs of the bi-prediction candidates, the L1 MVs of the bi-prediction candidates, and averaged MVs of the L0 and L1 MVs of bi-prediction candidates. To average L0 and L1 MVs, the L1 MV is scaled to the reference picture of the L0 MV. When adding one uni-prediction MV, a full pruning process comparing the to-be-added MV with all the MVs that are already in the list is performed. At most 74 times of MV pruning, 7 times of MV averaging, and 7 times of MV scaling are required at this candidate insertion stage. If the number of candidates is less than five, zero MVs are added to the list. For the entire triangle merging candidate list derivation, at most 94 times of MV pruning, 7 times of MV averaging, and 7 times of MV scaling are required. The triangle merging candidate list derivation is very complex and undesirable especially for hardware decoders.

In this contribution, it is proposed to simplify the triangle merging candidate list derivation. The proposed method is described in Section 2, and the results are reported in Section 3.

Figure 1: Position of the neighbouring blocks

# Proposed method

In this contribution, it is proposed to reuse the existing merging candidate list for generating the two uni-prediction MVs for the two triangular partitions. When the triangle prediction is applied, a merge_triangle_idx indicating the partition direction and the two selected candidates for the two triangular partitions is signalled. The index ranges from 0 to 39. A look-up table is used for deriving the partition direction and the two candidates. The syntax element and the look-up table are kept the same as those in VTM3.0, but the triangle merging candidate list replaced with the existing merging candidate list. After the two initial candidates from the existing merging candidate list are selected, for the first initial candidate, if it is uni-prediction, the uni-prediction MV is used for the first triangular partition; if it is bi-prediction, the L0 MV is used for the first triangular partition. Likewise, for the second initial candidate, if it is uni-prediction, the MV is used for the second triangular partition; if it is bi-prediction, the L1 MV is used for the second triangular partition. If the first triangular partition and the second triangular partition have exactly the same merging candidate, an offset MV of (1/4 pel, 0) is added to the merging candidate of the second triangular partition.

In this way, the complexity of the proposed triangle merging candidate list derivation is the same as that of the existing merging candidate list derivation plus only two inter prediction indicator (inter_pred_idc, to indicate L0, L1, or bi-prediction) comparisons, one MV comparison, and one MV addition.

# Simulation results

The proposed method is implemented on VTM3.0. The common test conditions [1] are used for evaluation. The results of the proposed triangle merging candidate list derivation are shown in Table 1. Simulation results reportedly show that the average luma BD-rates are 0.02% and 0.04% in RA and LB cases, respectively.

Table 1.  Results of the proposed triangle merging candidate list derivation

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | 0.03% | 0.04% | 99% | 101% |
| Class A2 | -0.05% | -0.09% | 0.06% | 96% | 99% |
| Class B | -0.01% | 0.05% | -0.02% | 101% | 99% |
| Class C | 0.07% | 0.10% | 0.12% | 100% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.03% | 0.05% | 99% | 99% |
| Class D | 0.03% | -0.13% | -0.09% | 99% | 102% |
| Class F | 0.06% | 0.04% | 0.05% | 102% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.26% | 0.06% | 101% | 99% |
| Class C | 0.08% | 0.30% | 0.31% | 99% | 96% |
| Class E | 0.04% | 0.34% | -0.03% | 102% | 100% |
| Overall | 0.04% | 0.08% | 0.12% | 100% | 98% |
| Class D | 0.03% | 0.32% | 0.34% | 106% | 104% |
| Class F | 0.03% | -0.15% | 0.22% | 98% | 98% |

# Conclusions

In this contribution, it is proposed to reuse the existing merging candidate list for generating the two uni-prediction MVs for the two triangular partitions. The triangle merging candidate list derivation is significantly simplified. Simulation results reportedly show that the average luma BD-rates are 0.02% and 0.04% in RA/LB cases, respectively.

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-L1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).