| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0226 |
| --- | --- |

| Title: | CE2: Reducing worst-case memory bandwidth of affine mode (test 2.4.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen Xianglin Wang | Email: | yiwenchen@kwai.com xianglinwang@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

Reported in this document is the results of CE2.4.1 on reducing the worst-case memory bandwidth of affine motion mode. In the proposed scheme, affine sub-block size is 4x4 for a uni-directional affine CU and 8x4/4x8 for a bi-directional affine CU. It is asserted that the proposed restrictions reduce the worst-case memory access bandwidth by about 32% for the current VVC. Three subtests are conducted for CE2.4.1. In CE2.4.1.a, affine sub-block size is 8x4 or 4x8 for a bi-directional affine coded CU based on its width and height conditions. In CE2.4.1.b and CE2.4.1.c, affine sub-block size is always 8x4 and 4x8 respectively for a bi-directional affine coded CU. The simulation results reportedly show that CE2.4.1.a introduces luma losses of 0.11% and 0.10% respectively under RA and LDB cases. CE2.4.1.b introduces luma losses of 0.10% and 0.11% respectively under RA and LDB cases. CE2.4.1.c introduces luma losses of 0.12% and 0.13% respectively under RA and LDB cases.

# Introduction

Motion compensation (MC) is usually the largest consumer of memory bandwidth in decoder implementation. Therefore, for a video coding standard, providing a reasonable upper limit on MC bandwidth requirement is extremely important in facilitating its cost-effective implementations and ensure its success in industry.

Memory access bandwidth requirement for MC is usually determined by the operating block size and the type of prediction (e.g. uni-directional or bi-directional). In VVC, the worst case of MC memory bandwidth occurs with the bi-directional MC of a 4x4 block, which is utilized by the CUs coded as bi-directional affine mode (where MC sub-block size is always 4x4).

# Proposed Implementation

According to proposal JVET-L0104[1], 4x4 blocks are restricted from using bi-directional motion compensation for affine motion mode, which results in 32% reduction of the worst-case bandwidth requirement in the current VVC. Three different schemes as described below are tested in CE2.

CE2.4.1.a) 4x4 sub-block is used for a uni-directional affine coded CU; 8x4/4x8 sub-block is used for a bi-directional affine coded CU according to its width and height conditions. 8x4 is used if its width is not shorter than its height. Otherwise 4x8 is used.

CE2.4.1.b) 4x4 sub-block is used for a uni-directional affine coded CU; 8x4 sub-block is used for a bi-directional affine coded CU.

CE2.4.1.c) 4x4 sub-block is used for a uni-directional affine coded CU; 4x8 sub-block is used for a bi-directional affine coded CU.

With the proposed restrictions, the bi-directional 8x4/4x8 size MC is the new worst-case for VVC, which is about 47% higher than the worst-case of HEVC.

# Results

The results of each subtest of CE2.4.1 are shown below against VTM3.0 [2] under CTC [3].

Table 1. Performance of CE2.4.1.a under Random Access

Table 2. Performance of CE2.4.1.a under Low Delay B

Table 3. Performance of CE2.4.1.b under Random Access

Table 4. Performance of CE2.4.1.b under Low Delay B

Table 5. Performance of CE2.4.1.c under Random Access

Table 6. Performance of CE2.4.1.c under Low Delay B

# Conclusion

In this proposal, results are provided for CE test 2.4.1, according to which the bi-directional MC block size is restricted to be at least 4x8/8x4 or larger to reduce the worst-case memory access bandwidth for motion compensation in VVC. The proposal results in 32% reduction of the worst-case bandwidth requirement for the current VVC, with about 0.1% BD-rate increase. It is recommended to consider adoption of the proposed restriction into VVC.

# Reference

- “AHG5: Reducing VVC worst-case memory bandwidth by restricting bi-directional 4x4 inter CUs/Sub-blocks”, Y.-W. Chen, X. Wang, doc. JVET-L0104, October 2018.

- “JVET common test conditions and software reference configurations for SDR video”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, doc. JVET-L1010, October 2018.

- VTM-3.0 in https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).