| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0250 |
| --- | --- |

| Title: | Non-CE7: Simplified CSBF coding for large block-size transforms |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jung-Ah Choi, Jin Heo, Sunmi Yoo, Jangwon Choi, Ling Li, Jaehyun Lim, Seung Hwan Kim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea | Tel: Email: | +82-2-6912-6561 {jungah.choi, jin78.heo, sunmi.yoo, jangwon84.choi, l.li, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution proposes a simplified design of coded_sub_block_flag (CSBF) coding. The proposed method can remove unnecessary CSBF coding processes for the high-frequency zeroing region. Experimental results for AI configuration show 0.00%, 0.00%, and 0.02% BD-rates on Y, Cb, and Cr components, respectively, with 98% decoding time compared to VTM3.0. For RA configuration, BD rate changes are observed to be -0.01% (Y), 0.00% (Cb), and 0.04% (Cr) with 99% decoding.

Introduction

In VTM3.0 [1], large TUs are partitioned into 4×4 coefficient groups, which are called sub-block. The coded_sub_block_flag (CSBF) represents whether the corresponding coefficient group has any non-zero residual coefficients or not. If CSBF is coded as 0, the 16 transform coefficients of the corresponding coefficient group are inferred to be equal to 0 and consequently, sig_coeff_flags within the coefficient group are not signalled. Otherwise, the coefficient group contains at least one non-zero transform coefficient. For some cases, when the coefficient group contains the DC and the last significant coefficient position, CSBF is inferred as 1 and not coded in the bitstream. Figure 1 shows the CSBF coding process of the VTM3.0 software.

It is known that large block partitions typically present less residuals and the energy is more concentrated in low-frequency coefficients in the transform domain. In VTM3.0 [1], high frequency transform coefficients are zeroed out for the transform blocks with size (width or height, or both width and height) equal to 64. Thus, for a W × H transform block, where W indicates the block width and H the block height, only the top-left (W == 64 ? 32 : W) × (H == 64 ? 32 : H) transform coefficients are retained.

In this proposal, unnecessary CSBF coding processes for the high-frequency zeroing region are removed which result in simpler coding process. Following section describes CSBF coding of the proposed method.

Proposed method

In the proposed method, whether the current coefficient group belongs to the high-frequency zeroing region is checked. Using this information, the unnecessary CSBF coding for the high-frequency zeroing region can be skipped and CSBF is inferred as 0.

Figure 1. CSBF coding process of VTM3.0

Figure 2. Proposed CSBF coding process

Experimental results

The proposed method has been implemented on the VTM3.0 software. The simulations were performed following the common test conditions defined in JVET-L1010 [2]. In every case, the anchor is the VTM3.0 software.

Table 1. Experimental results for all-intra (AI) test condition; anchor is VTM3.0

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.02% | 0.04% | 99% | 99% |
| Class A2 | 0.00% | -0.02% | 0.01% | 99% | 100% |
| Class B | 0.00% | 0.01% | 0.02% | 100% | 97% |
| Class C | 0.00% | 0.01% | -0.01% | 100% | 97% |
| Class E | 0.00% | 0.04% | 0.02% | 101% | 98% |
| Overall | 0.00% | 0.00% | 0.02% | 100% | 98% |
| Class D | 0.00% | 0.03% | -0.02% | 101% | 100% |
| Class F | 0.00% | 0.00% | 0.01% | 101% | 99% |

Table 2. Experimental results for random-access (RA) test condition; anchor is VTM3.0

|  |  | Random access Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.13% | 0.08% | 100% | 99% |
| Class A2 | -0.01% | 0.08% | 0.10% | 100% | 100% |
| Class B | -0.03% | 0.05% | 0.00% | 101% | 99% |
| Class C | 0.00% | -0.01% | -0.01% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.00% | 0.04% | 101% | 99% |
| Class D | 0.01% | 0.00% | -0.01% | 101% | 99% |
| Class F | 0.01% | -0.01% | 0.00% | 101% | 98% |

Reference

- B. Bross, et al., “Versatile Video Coding (Draft 3),” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-L1001, 12th meeting, Macao, CN, 3-12 Oct. 2018.

- F. Bossen, et al., “JVET common test conditions and software reference configurations for SDR video” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-L1010, 12th Meeting, Macao, CN, 3-12 Oct. 2018.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Specification

(section 7.3.4.12 in VVC specification)

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |  |
| --- | --- | --- |
| if( transform_skip_enabled_flag && ( cIdx ! = 0 ¦ ¦ tu_mts_flag[ x0 ][ y0 ] = = 0 ) && ( log2TbWidth <= 2 ) && ( log2TbHeight <= 2 ) ) |  |  |
| transform_skip_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |  |
| last_sig_coeff_x_prefix | ae(v) |
| last_sig_coeff_y_prefix | ae(v) |
| if( last_sig_coeff_x_prefix > 3 ) |  |
| last_sig_coeff_x_suffix | ae(v) |
| if( last_sig_coeff_y_prefix > 3 ) |  |
| last_sig_coeff_y_suffix | ae(v) |
| log2SbSize = ( Min( log2TbWidth, log2TbHeight ) < 2 ? 1 : 2 ) |  |
| numSbCoeff = 1 << ( log2SbSize << 1 ) |  |
| lastScanPos = numSbCoeff |  |
| log2TbWidth = Min( log2TbWidth, 5 ) |  |
| log2TbHeight = Min( log2TbHeight, 5 ) |  |
| lastSubBlock = ( 1 << ( log2TbWidth + log2TbHeight − 2 * log2SbSize ) ) − 1 |  |
| do { |  |
| if( lastScanPos = = 0 ) { |  |
| lastScanPos = numSbCoeff |  |
| lastSubBlock− − |  |
| } |  |
| lastScanPos− − |  |
| xS = DiagScanOrder[ log2TbWidth − log2SbSize ][ log2TbHeight − log2SbSize ] [ lastSubBlock ][ 0 ] |  |
| yS = DiagScanOrder[ log2TbWidth − log2SbSize ][ log2TbHeight − log2SbSize ] [ lastSubBlock ][ 1 ] |  |
| xC = ( xS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ lastScanPos ][ 0 ] |  |
| yC = ( yS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ lastScanPos ][ 1 ] |  |
| } while( ( xC != LastSignificantCoeffX ) ¦ ¦ ( yC != LastSignificantCoeffY ) ) |  |
| numSigCoeff = 0 |  |
| QState = 0 |  |
| for( i = lastSubBlock; i >= 0; i− − ) { |  |
| startQStateSb = QState |  |
| xS = DiagScanOrder[ log2TbWidth − log2SbSize ][ log2TbHeight − log2SbSize ] [ lastSubBlock ][ 0 ] |  |
| yS = DiagScanOrder[ log2TbWidth − log2SbSize ][ log2TbHeight − log2SbSize ] [ lastSubBlock ][ 1 ] |  |
| inferSbDcSigCoeffFlag = 0 |  |
| if( ( i < lastSubBlock ) && ( i > 0 ) ) { |  |
| coded_sub_block_flag[ xS ][ yS ] | ae(v) |
| inferSbDcSigCoeffFlag = 1 |  |
| } |  |
| firstSigScanPosSb = numSbCoeff |  |
| lastSigScanPosSb = −1 |  |
| remBinsPass1 = ( log2SbSize < 2 ? 6 : 28 ) |  |
| remBinsPass2 = ( log2SbSize < 2 ? 2 : 4 ) |  |
| firstPosMode0 = ( i = = lastSubBlock ? lastScanPos − 1 : numSbCoeff − 1 ) |  |
| firstPosMode1 = −1 |  |
| firstPosMode2 = −1 |  |
| for( n = ( i = = firstPosMode0; n >= 0 && remBinsPass1 >= 3; n− − ) { |  |
| xC = ( xS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 0 ] |  |
| yC = ( yS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 1 ] |  |
| if( coded_sub_block_flag[ xS ][ yS ] && ( n > 0 ¦ ¦ !inferSbDcSigCoeffFlag ) ) { |  |
| sig_coeff_flag[ xC ][ yC ] | ae(v) |
| remBinsPass1− − |  |
| if( sig_coeff_flag[ xC ][ yC ] ) |  |
| inferSbDcSigCoeffFlag = 0 |  |
| } |  |
| if( sig_coeff_flag[ xC ][ yC ] ) { |  |  |
| numSigCoeff++ |  |  |
| abs_level_gt1_flag[ n ] | ae(v) |
| remBinsPass1− − |  |
| if( abs_level_gt1_flag[ n ] ) { |  |
| par_level_flag[ n ] | ae(v) |
| remBinsPass1− − |  |
| if( remBinsPass2 > 0 ) { |  |
| remBinsPass2− − |  |
| if( remBinsPass2 = = 0 ) |  |
| firstPosMode1 = n − 1 |  |
| } |  |
| } |  |
| if( lastSigScanPosSb = = −1 ) |  |
| lastSigScanPosSb = n |  |
| firstSigScanPosSb = n |  |
| } |  |
| AbsLevelPass1[ xC ][ yC ] = sig_coeff_flag[ xC ][ yC ] + par_level_flag[ n ] + abs_level_gt1_flag[ n ] |  |
| if( dep_quant_enabled_flag ) |  |
| QState = QStateTransTable[ QState ][ AbsLevelPass1[ xC ][ yC ] & 1 ] |  |
| if( remBinsPass1 < 3 ) |  |
| firstPosMode2 = n − 1 |  |
| } |  |
| if( firstPosMode1 < firstPosMode2 ) |  |
| firstPosMode1 = firstPosMode2 |  |
| for( n = numSbCoeff − 1; n >= firstPosMode2; n− − ) |  |
| if( abs_level_gt1_flag[ n ] ) |  |
| abs_level_gt3_flag[ n ] | ae(v) |
| for( n = numSbCoeff − 1; n >= firstPosMode1; n− − ) { |  |
| xC = ( xS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 0 ] |  |
| yC = ( yS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 1 ] |  |
| if( abs_level_gt3_flag[ n ] ) |  |
| abs_remainder[ n ] | ae(v) |
| AbsLevel[ xC ][ yC ] = AbsLevelPass1[ xC ][ yC ] + 2 * ( abs_level_gt3_flag[ n ] + abs_remainder[ n ] ) |  |
| } |  |
| for( n = firstPosMode1; n > firstPosMode2; n− − ) { |  |
| xC = ( xS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 0 ] |  |
| yC = ( yS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 1 ] |  |
| if( abs_level_gt1_flag[ n ] ) |  |
| abs_remainder[ n ] | ae(v) |
| AbsLevel[ xC ][ yC ] = AbsLevelPass1[ xC ][ yC ] + 2 * abs_remainder[ n ] |  |
| } |  |
| for( n = firstPosMode2; n >= 0; n− − ) { |  |
| xC = ( xS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 0 ] |  |
| yC = ( yS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 1 ] |  |
| dec_abs_level[ n ] | ae(v) |
| if(AbsLevel[ xC ][ yC ] > 0 ) |  |
| firstSigScanPosSb = n |  |
| if( dep_quant_enabled_flag ) |  |
| QState = QStateTransTable[ QState ][ AbsLevel[ xC ][ yC ] & 1 ] |  |
| } |  |
| if( dep_quant_enabled_flag ¦ ¦ !sign_data_hiding_enabled_flag ) |  |
| signHidden = 0 |  |
| else |  |
| signHidden = ( lastSigScanPosSb − firstSigScanPosSb > 3 ? 1 : 0 ) |  |
| for( n = numSbCoeff − 1; n >= 0; n− − ) { |  |
| xC = ( xS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 0 ] |  |
| yC = ( yS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 1 ] |  |
| if( sig_coeff_flag[ xC ][ yC ] && ( !signHidden ¦ ¦ ( n != firstSigScanPosSb ) ) ) |  |
| coeff_sign_flag[ n ] | ae(v) |
| } |  |
| if( dep_quant_enabled_flag ) { |  |
| QState = startQStateSb |  |
| for( n = numSbCoeff − 1; n >= 0; n− − ) { |  |
| xC = ( xS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 0 ] |  |
| yC = ( yS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 1 ] |  |
| if( sig_coeff_flag[ xC ][ yC ] ) |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( 2 * AbsLevel[ xC ][ yC ] − ( QState > 1 ? 1 : 0 ) ) * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| QState = QStateTransTable[ QState ][ par_level_flag[ n ] ] |  |
| } else { |  |
| sumAbsLevel = 0 |  |
| for( n = numSbCoeff − 1; n >= 0; n− − ) { |  |
| xC = ( xS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 0 ] |  |
| yC = ( yS << log2SbSize ) + DiagScanOrder[ log2SbSize ][ log2SbSize ][ n ][ 1 ] |  |
| if( sig_coeff_flag[ xC ][ yC ] ) { |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = AbsLevel[ xC ][ yC ] * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| if( signHidden ) { |  |
| sumAbsLevel += AbsLevel[ xC ][ yC ] |  |
| if( ( n = = firstSigScanPosSb ) && ( sumAbsLevel % 2 ) = = 1 ) ) |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = −TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| if( tu_mts_flag[ x0 ][ y0 ] && ( cIdx = = 0 ) ) |  |
| mts_idx[ x0 ][ y0 ][ cIdx ] | ae(v) |
| } |  |  |