| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0268 |
| --- | --- |

| Title: | Non-CE2: Interweaved Prediction for Affine Motion Compensation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang, Li Zhang, Hongbin Liu, Jizheng Xu, Yue Wang, Pengwei Zhao, Dingkun Hong 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-858-380-8522 zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

With the affine motion compensation (AMC) in the VTM, a coding-block is divided into 4×4 sub-blocks, and each sub-block is assigned with individual motion vectors derived by one affine model. In this contribution, an interweaved prediction approach is proposed for AMC. With the interweaved prediction, a coding block is divided into sub-blocks with two different dividing patterns. Then two auxiliary predictions are generated by AMC with the two dividing patterns. The final prediction is calculated as a weighted-sum of the two auxiliary predictions. All sub-blocks are 4×4 with both the two dividing patterns. The bandwidth is not increased by interweaved prediction since the area used to do MC for all sub-blocks is fetched together as a whole in AMC. Two tests are conducted in this contribution. In Test 1, interweaved prediction is only applied on uni-prediction. In Test 2, interweaved prediction is applied on both uni-prediction and bi-prediction. Simulation results reportedly show 0.27% and 0.38% BD-rate savings for Test 1 and Test 2, respectively, under Random Access (RA) configurations. The BD-rate saving on sequence SlideShow is 2.13% in Test 2.

# Introduction

Fig. 1. 4*4 sub-blocks in a 16*16 AMC-coded block.

With affine motion compensation (AMC) in the VTM, a coding-block is divided into sub-blocks as small as 4×4, each of which is assigned with an individual motion vector (MV) derived by the affine model, as shown in Fig. 1. With the 4-parameter or 6-parameter affine model, a MV is derived from MVs of the two or three control points.

AMC faces a dilemma on the size of sub-blocks. With smaller sub-blocks, AMC can achieve a better coding performance but suffers a higher complexity burden.

# Interweaved Prediction

To address the dilemma of AMC, interweaved prediction as shown in Fig. 2 is proposed to achieve a finer granularity of MVs without increasing the complexity too much.

First, a coding block is divided into sub-blocks with two different dividing patterns. The first dividing pattern is the same as that in VTM as pattern 0 shown in Fig. 2, while the second dividing pattern also divides the coding-block into 4×4 sub-blocks but with a 2×2 offset as pattern 1 shown in Fig. 2.

Second, two auxiliary predictions are generated by AMC with the two dividing patterns. The MV for each sub-block in a dividing pattern is derived from CPMVs by the affine model.

The final prediction is calculated as a weighted-sum of the two auxiliary predictions, formulated as:

| . | (1) |
| --- | --- |

As shown in Fig. 3, an auxiliary prediction sample located at the center of a sub-block is associated with a weighting value 3, while an auxiliary prediction sample located at the boundary of a sub-block is associated with a weighting value 1.

To avoid tiny block motion compensation, the interweaved prediction is only applied on regions where the size of sub-blocks is 4×4 for both the two dividing patterns as shown in Fig. 4.

In VTM-3.0, the size of sub-blocks is also 4×4 on the chroma components, so interweaved prediction is applied on chroma components as well as the luma component.

The bandwidth is not increased by interweaved prediction since the area used to do MC for all sub-blocks is fetched together as a whole in AMC.

For flexibility, a flag is signaled in slice header to indicate whether interweaved prediction is used or not. In the proposed solution, the flag is always signaled to be 1.

Fig. 2. Interweaved prediction procedure.

Fig. 3. Weighting values in a sub-block.

Fig. 4 Interweaved prediction is not applied on the shaded regions.

# Experimental Results

The proposed method has been implemented on top of VTM-3.0. Two tests are performed. In test 1, inter-prediction is only applied on uni-prediction. In test 2, interweaved prediction is applied on both uni-prediction and bi-prediction. Results of RA/LB configurations for test 1 and test 2 are summarized in Table 1 and Table 2, respectively. The simulations were performed following the JVET CTC [1]. More detail results can be found in the attached spreadsheets.

Table 1: Results for Test 1

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.14% | -0.02% | 101% | 100% |  |  |  |  |  |
| Class A2 | -0.55% | -0.17% | -0.11% | 102% | 101% |  |  |  |  |  |
| Class B | -0.33% | -0.17% | -0.20% | 101% | 101% |  |  |  |  |  |
| Class C | -0.15% | -0.16% | -0.04% | 101% | 100% |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.27% | -0.16% | -0.11% | 101% | 101% |  |  |  |  |  |
| Class D | -0.21% | -0.09% | 0.02% | 101% | 106% |  |  |  |  |  |
| Class F | -0.39% | -0.40% | -0.39% | 101% | 102% |  |  |  |  |  |

Table 2: Results for Test 2

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | -0.18% | -0.12% | 103% | 102% |  |  |  |  |  |
| Class A2 | -0.74% | -0.40% | -0.28% | 104% | 104% |  |  |  |  |  |
| Class B | -0.47% | -0.39% | -0.35% | 103% | 102% |  |  |  |  |  |
| Class C | -0.22% | -0.19% | -0.13% | 102% | 102% |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.38% | -0.30% | -0.23% | 103% | 102% |  |  |  |  |  |
| Class D | -0.42% | -0.28% | -0.32% | 103% | 102% |  |  |  |  |  |
| Class F | -0.60% | -0.64% | -0.62% | 102% | 102% |  |  |  |  |  |

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-L1010, Oct. 2018.

- M. Zhou, “CE2-related: A restriction on memory bandwidth consumption of affine mode,” JVET-M0049, Jan. 2019.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).