| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0311 |
| --- | --- |

| Title: | CE2-related: Memory bandwidth reduction for affine mode with less dependency |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jingya Li Ru-ling Liao Chong Soon Lim | Tel: Email: | +65-6550-5423 Jingya.li@sg.panasonic.com Chongsoon.lim@sg.panasoni.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution is a further simplification on JVET-M0309. In this contribution, the first control point is used instead of first sub-block’s motion vector as center of motion vector constrained region of affine mode.

the BD-rate difference is 0.03% in RA, 0.01% in LD_B respectively.

# Algorithm description

Compared with contribution JVET-M0309, this contribution uses first control point instead of first sub-block’s motion vector as center of motion vector constrained region of affine mode. The purpose is to remove additional dependency of motion vector of first sub-block and other sub-blocks. Details of proposed method are as below.

For affine mode, sub-block motion vectors of an affine CU are constrained to be within a pre-defined motion vector field. values of motion vector in sub-block x, y (vix and viy) exhibit the following constraints:

MinX = (CP0x>> MV_precision << MV_precision) -H;

MaxX = (CP 0x>> MV_precision << MV_precision) +H+(1<< MV_precision) – 1;

MinY = (CP 0y>> MV_precision << MV_precision) -V;

MaxY = (CP 0y>> MV_precision << MV_precision) +V+(1<< MV_precision) – 1;

values of vix and viy exhibit the following constraints:

vix ∊ [MinX, MaxX]

viy ∊ [MinY, MaxY]

If the motion vector of any sub-block exceeds the pre-defined motion vector field, the motion vector is clipped. An illustration of the idea of constrained sub-block motion vector is given in Figure 1.

Compared with contribution JVET-M0309, Complexity of calculating MV of first sub-block is saved for calculating MV range. Therefore calculating MV range takes a total of 8 additions and 10 shifts operations for whole affine CU.

Values of H and V are chosen so that worst case memory bandwidth of affine CU will not exceed that of normal inter MC of a 8×8 bi-prediction block. Note that values of H and V are adaptive to CU size and uni-prediction or bi-prediction.

# Experimental results

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.02% | -0.02% | 100% | 97% |
| Class A2 | 0.05% | 0.15% | 0.16% | 100% | 96% |
| Class B | 0.02% | 0.11% | -0.01% | 100% | 96% |
| Class C | 0.03% | 0.07% | 0.10% | 100% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 0.09% | 0.05% | 100% | 97% |
| Class D | 0.03% | 0.12% | 0.03% | 100% | 98% |
| Class F (optional) | 0.03% | -0.03% | 0.02% | 100% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.05% | -0.11% | -0.18% | 99% | 99% |
| Class C | 0.01% | 0.07% | 0.26% | 100% | 100% |
| Class E | -0.05% | 0.24% | -0.12% | 100% | 100% |
| Overall | 0.01% | 0.04% | -0.01% | 100% | 100% |
| Class D | 0.02% | 0.09% | 0.58% | 100% | 100% |
| Class F (optional) | 0.03% | 0.37% | 0.32% | 100% | 99% |

# Conclusion

In this contribution, a solution is proposed to address the worst case memory bandwidth of VTM. It is reported that the proposed solution introduces 0.03% B-D rate loss with negligible impact on encoding and decoding time. We recommend to adopt the proposed solution into the next version of VVC.

# References

[1] J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-L1010, the 12th JVET meeting, Oct. 2018, Macao, CN.

[2] B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 3)”, JVET-L1001-v7, the 12th JVET meeting, the 12th JVET meeting, Oct. 2018, Macao, CN.

[3] Y. he, C.-Y. Chen and C-C Chen, “Description of Core Experiment 6 (CE2): Sub-block based motion prediction”, JVET-L1022, the 12th JVET meeting, Oct. 2018, Macao, CN.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Proposed text

#### Derivation process for motion vector arrays from affine control point motion vectors

…

- For xSbIdx = 0..( cbWidth >> 2 ) − 1 and ySbIdx = 0..( cbHeight >> 2 ) − 1, the following applies:

  - The luma motion vector mvLX[ xSbIdx ][ ySbIdx ] is derived as follows :

xPosCb = 2 + ( xSbIdx << 2 )		(8-338)

yPosCb = 2 + ( ySbIdx << 2 )		(8-339)

mvLX[ xSbIdx ][ ySbIdx ][ 0 ] = ( mvScaleHor + dHorX * xPosCb + dHorY * yPosCb )	(8-340)

mvLX[ xSbIdx ][ ySbIdx ][ 1 ] = ( mvScaleVer + dVerX * xPosCb + dVerY * yPosCb )	(8-341)

  - The rounding process for motion vectors as specified in clause 8.3.2.14 is invoked the with mvX set equal to  mvLX[ xSbIdx ][ ySbIdx ], rightShift set equal to 7, and leftShift set equal to 0 as inputs and the rounded  mvLX[ xSbIdx ][ ySbIdx ] as output.

  - The clipping process for motion vectors as specified in clause 8.3.3.8 is invoked with the luma coding block width cbWidth, the luma prediction block height cbHeight, the prediction list utilization flags predFlagL0 and predFlagL1, the mvCenterX set equal to mvLX[ 0 ][ 0 ] and the mvX set equal to mvLX[ xSbIdx ][ ySbIdx ] as inputs and the clipped mvLX[ xSbIdx ][ ySbIdx ] as output, X being 0 and 1.

  - The derivation process for chroma motion vectors in clause 8.3.2.13 is invoked with mvLX[ xSbIdx ][ ySbIdx ] as input, and the chroma motion vector mvCLX[ xSbIdx ][ ySbIdx ] as output.

#### Derivation process for motion vector arrays from affine control point motion vectors

Inputs to this process are

- two variables cbWidth and cbHeight specifying the width and the height of the current luma coding block

- the prediction list utilization flags predFlagL0 and predFlagL1

- the motion vector mvCenterX, X being 0 and 1, is first control motion vector of affine CU

- the motion vector mvX, X being 0 and 1.

Output of this process is

- the clipped motion vector mvX, X being 0 and 1.

For the clipping of mvX, the following applies:

- the variable mvRange is derived as follows:

  - if predFlagL0 is equal to 1 and predFlagL1 is equal to 1, the variable mvRange is derived using cbWidth + cbHeight as specified in the following Table:

| cbWidth + cbHeight | 256 | 192 | 160 | 144 | 136 | 128 | 96 | 80 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| mvRange | (52 << 4) | (34 << 4) | (21 << 4) | (11 << 4) | (4 << 4) | (24 << 4) | (15 << 4) | (8 << 4) |
| cbWidth + cbHeight | 72 | 64 | 48 | 40 | 32 | 24 | 16 |  |
| mvRange | (3 << 4) | (10 << 4) | (6 << 4) | (2 << 4) | (3 << 4) | (1 << 4) | 0 |  |

  - Otherwise, the variable mvRange is derived is derived using cbWidth + cbHeight as specified in the following Table:

| cbWidth + cbHeight | 256 | 192 | 160 | 144 | 136 | 128 | 96 | 80 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| mvRange | (102 << 4) | (69 << 4) | (44 << 4) | (26 << 4) | (14 << 4) | (49 << 4) | (33 << 4) | (20 << 4) |
| cbWidth + cbHeight | 72 | 64 | 48 | 40 | 32 | 24 | 16 |  |
| mvRange | (11 << 4) | (22 << 4) | (14 << 4) | (8 << 4) | (9 << 4) | (5 << 4) | (3 << 4) |  |

- the clipped motion vector mvX with X being 0 and 1 is derived as follows:

mvX[0] = Clip3( mvCenterX[0] - mvRange, mvCenterX[0] + mvRange, mvX[0] )

mvX[1] = Clip3( mvCenterX[1] - mvRange, mvCenterX[1] + mvRange, mvX[1] )