| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0314 |
| --- | --- |

| Title: | CE4-related: MMVD improving with signaling distance table |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jingya Li Ru-ling Liao Chong Soon Lim | Tel: Email: | +65-6550-5423 Jingya.li@sg.panasonic.com Chongsoon.lim@sg.panasoni.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution proposes to signal adaptive distance table instead of deriving at decoder side to reduce complexity based on CE4.4.4e and CE4.4.5c. Experimental results show -0.2% (RA) and **% (LDB) luma BD-rate change and **% (RA) and **% (LDB) luma BD-rate change respectively. Ignorable encoding and decoding time change is observed compared with CE4.4.4e and CE4.4.5c.

# Algorithm description

This contribution is based on CE4.4.4e and CE4.4.5c. In this proposal, it is proposed to signal adaptive distance table on slice level instead of deriving at decoder side.

(test1) CE4.4.5c + signal reordered distance table on slice level.

(test2) CE4.4.4e + signal reordered distance table on slice level.

MMVD distance represented by distance index is signaled using 3 bit binary value. Mapping between distance index and distance is given in table 1 and table2.

Additionally, a flag is signaled to indicate whether reordered distance table is signaled. If the flag is true, the reordered distance table is signed using 3x8 bits. Otherwise, the default distance table wherein distance index of ascending order 0-7 is used.

Table 1 - Binary representation of MMVD distance when resolution <4K

| Distance IDX | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Distance | 1/4-pel | 1/2-pel | 1-pel | 2-pel | 4-pel | 8-pel | 16-pel | 32-pel |
| Binary Value | 000 | 001 | 010 | 011 | 100 | 101 | 110 | 111 |

Table 2 - Binary representation of MMVD distance when resolution >=4K

| Distance IDX | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Distance | 1-pel | 2-pel | 4-pel | 8-pel | 16-pel | 32-pel | 64-pel | 128-pel |
| Binary Value | 000 | 001 | 010 | 011 | 100 | 101 | 110 | 111 |

# Experimental Results

Results of test 1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.44% | -0.62% | -0.70% |  |  |
| Class A2 | -0.28% | -0.81% | -0.69% |  |  |
| Class B | -0.13% | -0.19% | -0.36% |  |  |
| Class C | -0.05% | -0.13% | -0.07% |  |  |
| Class E |  |  |  |  |  |
| Overall | -0.20% | -0.38% | -0.42% |  |  |
| Class D | 0.04% | -0.22% | -0.16% |  |  |
| Class F | -0.09% | -0.14% | -0.17% |  |  |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 | -0.09% | -0.48% | -0.04% |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class C | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class E | #VALUE! | #VALUE! | #VALUE! |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class D | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class F |  |  |  |  |  |

.Results of test 2

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

# Conclusions

In this contribution, proposes to signal adaptive distance table instead of deriving at decoder side to reduce complexity. Experimental results show -0.2% (RA) and **% (LDB) luma BD-rate change and **% (RA) and **% (LDB) luma BD-rate change respectively. Ignorable encoding and decoding time change is observed compared with CE4.4.4e and CE4.4.5c.

We recommend to adopt the proposed solution of test2 into the next version of VVC.

# References

- M. W. Park, Y. Piao, M. Park, K. Choi, “CE4 Ultimate motion vector expression,” JVET-L0054, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 12th Meeting: Macao, CN, 3–12 Oct. 2018

- F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 12th Meeting: Macao, CN, 3–12 Oct. 2018.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Proposed Working Draft Changes for test1

### Updating process for distance_idx base table

Inputs to this process are:

- A variable W specifying the width of the current picture in luma samples,

- A variable H specifying the height of the current picture in luma samples,

- The array MMVDDistance1, MMVDDistance2.

Outputs of this process are:

- The array baseMMVDDistance.

The array MMVDDirection is derived as follows:

- If W is greater than 1920 and H is greater than 1080

Set baseMMVDDistance to MMVDDistance1 = {4, 8, 16, 32, 64, 128, 256, 512};

- Else,

Set baseMMVDDistance to MMVDDistance2 = {1, 2, 4, 8, 16, 32, 64, 128};

General decoding process

Updating process for distance_idx base table is invoked before slice decoding

### Slice decoding process

Decoding reordered distance table

# Proposed Working Draft Changes for test2

following change is added on top on working draft for test1

#### 7.4.5.5 Coding unit semantics

mmvd_direction_idx[ x0 ][ y0 ] specifies index used to derive MmvdSign[ x0 ][ y0 ] as specified in Table 7-8. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

Table 7-10 – Specification of MmvdSign[ x0 ][ y0 ] based on mmvd_direction_idx[ x0 ][ y0 ]

| mmvd_direction_idx[ x0 ][ y0 ] | MmvdSign[ x0 ][ y0 ][0] | MmvdSign[ x0 ][ y0 ][1] |
| --- | --- | --- |
| 0 | +2 | 0 |
| 1 | -2 | 0 |
| 2 | 0 | +2 |
| 3 | 0 | -2 |
| 4 | +1 | +1 |
| 5 | -1 | -1 |
| 6 | -1 | +1 |
| 7 | +1 | -1 |

Both components of of the merge plus MVD offset MmvdOffset[ x0 ][ y0 ] are derived as follows:

MmvdOffset[ x0 ][ y0 ][ 0 ] = ( MmvdDistance[ x0 ][ y0 ] << 12 ) * MmvdSign[ x0 ][ y0 ][0]	(7-64)

MmvdOffset[ x0 ][ y0 ][ 1 ] = ( MmvdDistance[ x0 ][ y0 ] << 12 ) * MmvdSign[ x0 ][ y0 ][1]	(7-65)