| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0329_v1 |
| --- | --- |

| Title: | CE10-related: Modified Enabling Condition for Triangle Prediction Unit Mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Fangdong Chen Li Wang | Tel: Email: | +86 15156063604 chenfangdong@hikvision.com |
| Source: | Hikvision |  |  |

_____________________________

- Abstract

Currently, the triangle prediction unit mode is just enabled under condition that the frame is B frame and the area of current block is larger or equal than 64. With this condition, this mode is disabled for P frames, where the significant coding gains can be achieved. Moreover, the triangle prediction unit mode can be also utilized for some narrow block with width or height equals to 4. In this contribution, the condition is modified as that the frame is inter frame, and both the width and the height of current block should be larger than 4. It is reported that the first modification about frame type can provide 1.56% gains for LP compared to VTM-3.0 with around 8% encoding time and no decoding time increase. And the second modification about block size can bring 0.05% and 0.11% loss for RA and LB respectively compared to VTM-3.0.

Introduction

In [1], the triangular prediction unit mode is proposed, and it is adopted in VTM 3.0. In the triangular prediction unit mode, a CU can be split using two triangular prediction units, in either diagonal or inverse diagonal direction. Each triangular prediction unit in the CU has its own uni-prediction motion vector and reference frame index which are derived from a uni-prediction candidate list. Triangular partitioning is only applied to motion compensated prediction, which means that the transform and quantization process is applied to the whole CU formed by combining the two triangles together. The triangular prediction unit mode is only applied to a CU which block size is larger than or equal to 8×8, and its frame type must be B frame.

Proposed Modification

Modification about frame type

Here, the triangular prediction unit mode is also enabled for P frames. Base on the test results, it can be found that the encoding complexity is high. Then a fast mode decision is proposed. In the fast mode decision method, bestIsSkip is always set as false, and the number of candidate numTriangleCandidate is reduced from 40 to 14. Here, two tests are conducted:

Test A: Enable the triangular prediction unit mode for P frames with mode decision in VTM3.0.

Test B: Enable the triangular prediction unit mode for P frames with the proposed fast mode decision method.

Test C. Apply the proposed fast mode decision method to B frames.

Modification about block size

Here, the triangular prediction unit mode is only applied to a CU which both block width and height are larger than or equal to 8. This test is Test D.

Experimental Results

To evaluate the coding performance of the proposed method, the VTM-3.0 is served as the experimental platform. The CTC specified in [2], are adopted, and all the sequences are tested across the configurations. BD-rate is used to evaluate the final coding performance.

Results of Modification about frame type

The results of Test A are given in Table I. It is reported that the proposed method achieves 1.65% gains with 23% encoding time increase and 2% decoding time increase for LP configuration. And the results of Test B are given in Table II. It is reported that the proposed method achieves 1.56% gains with 8% encoding time increase and no decoding time increase for LP configuration. Moreover, from Table III, it can be seen that the proposed fast mode decision method help to save around 9% encoding time with 0.05% and 0.17% coding loss for RA and LB configurations, respectively.

Table I Results of Test A

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1.66% | -2.31% | -2.32% | 121% | 108% |
| Class C | -1.44% | -1.92% | -2.16% | 120% | 98% |
| Class E | -1.93% | -2.02% | -2.54% | 131% | 98% |
| Overall | -1.65% | -2.11% | -2.32% | 123% | 102% |
| Class D | -1.44% | -1.83% | -1.85% | 123% | 102% |
| Class F | -0.79% | -1.79% | -1.95% | 127% | 99% |

Table II Results of Test B

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1.53% | -1.76% | -1.70% | 107% | 95% |
| Class C | -1.29% | -1.64% | -1.87% | 107% | 95% |
| Class E | -1.97% | -1.77% | -2.82% | 109% | 95% |
| Overall | -1.56% | -1.72% | -2.03% | 108% | 95% |
| Class D | -1.33% | -1.29% | -0.89% | 109% | 101% |
| Class F | -0.77% | -1.12% | -1.16% | 109% | 98% |

Table III Results of Test C

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.03% | 0.14% | 93% | 98% |
| Class A2 | 0.02% | 0.12% | 0.22% | 91% | 97% |
| Class B | 0.05% | 0.16% | 0.02% | 91% | 98% |
| Class C | 0.10% | 0.44% | 0.40% | 91% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.05% | 0.20% | 0.19% | 92% | 98% |
| Class D | 0.08% | 0.24% | 0.31% | 91% | 99% |
| Class F | 0.04% | 0.11% | 0.14% | 90% | 101% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.13% | 0.02% | 0.25% | 91% | 94% |
| Class C | 0.18% | 0.63% | 0.42% | 92% | 103% |
| Class E | 0.22% | 0.19% | -0.01% | 88% | 102% |
| Overall | 0.17% | 0.26% | 0.24% | 91% | 99% |
| Class D | 0.11% | 0.46% | 0.28% | 91% | 101% |
| Class F | 0.18% | 0.35% | 0.03% | 89% | 102% |

Results of Modification about block size

The results of Test D are given in Table IV. It is reported that the modification about block size can only bring 0.05% and 0.11% loss for RA and LB respectively compared to VTM-3.0.

Table IV Results of Test D

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.08% | 0.07% | 100% | 102% |
| Class A2 | 0.02% | 0.08% | 0.21% | 99% | 103% |
| Class B | 0.03% | 0.12% | 0.09% | 99% | 101% |
| Class C | 0.13% | 0.21% | 0.26% | 98% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.05% | 0.09% | 0.15% | 99% | 101% |
| Class D | 0.08% | 0.28% | 0.21% | 98% | 99% |
| Class F | 0.06% | 0.06% | 0.12% | 100% | 101% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.07% | -0.17% | 0.14% | 99% | 99% |
| Class C | 0.20% | 0.28% | 0.39% | 99% | 101% |
| Class E | 0.05% | 0.29% | -0.47% | 100% | 101% |
| Overall | 0.11% | 0.10% | 0.07% | 99% | 100% |
| Class D | 0.15% | 0.29% | 0.07% | 98% | 99% |
| Class F | 0.15% | 0.25% | 0.35% | 100% | 101% |

Conclusion

In this contribution, the condition is modified as that the frame is inter frame, and both the width and the height of current block should be larger than 4. The first modification brings significant coding gains, and the other one makes the triangle prediction mode hardware friendly. Based on the above results, it is suggested to adopt them into the VTM.

Reference

- R.-L. Liao, C. S. Lim, “CE10.3.1.b: Triangular prediction unit mode,” JVET-L0124, Macao, CN, 3–12 Oct. 2018.

- F. Bossen,J. Boyce, X. Li, V. Seregin, K. Suehring, “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, Macao, CN, 3–12 Oct. 2018.

Patent rights declaration(s)

Hikvision Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).