| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0331_v2 |
| --- | --- |

| Title: | CE10-related: A simplification of inter prediction information derivation for Multi-intra-inter mode scheme |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liying Xu Fangdong Chen Li Wang | Tel: Email: | 18100191186 xuliying5@hikvision.com chenfangdong@hikvision.com wangli7@hikvision.com |
| Source: | HIKVISION |  |  |

_____________________________

- Abstract

This contribution presents a simplification for multi-intra-inter mode. In the Lth JVET meeting, the combination of inter and intra prediction is adopted. However, if inter prediction is bi-prediction, three motion compensations (MCs), including two inter MCs and one additional intra compensation, are required. In this contribution, only uni-prediction merge candidates will be used for this mode. As a result, at most one MC and one intra compensation are needed to conduct for multi-intra-inter mode in the decoder. It is reported that, compared to VTM3.0, the proposed method only brings 0.06% loss for RA configuration and 0.04% loss for LB configuration.

Introduction of Multi-intra-inter scheme

Currently, multi-intra-inter mode [1] utilizes the merge candidates to get the inter prediction and utilize at most four intra modes to get the intra prediction. For the decoder, if multi-intra-inter is used for a block, parsing the merge index and intra mode is needed to get the intra and inter prediction. When the inter prediction is bi-prediction, there are two inter MCs and one additional intra compensation for this mode.

The Proposed simplification method

To reduce the number of MC, it is proposed to conduct uni-prediction to generate the inter prediction. Thus, if the candidate is unidirectional, then it is directly used to generate the inter prediction. Otherwise, that is the merge candidates gets a bi-prediction motion information, then only motion of List 0 is used to get the inter prediction. But the motion information still keeps unchanged in the motion information spanning process so that it will never impacts the neighbor blocks to get the original MV information. The scheme is depicted in Fig 1.

Fig.1 Usage of merge candidates for proposed method

Experimental Results

The simulation is carried out under the JVET common test condition [2]. It is reported in Table 1 that, compared to VTM3.0, the proposed method only brings 0.06% loss for RA configuration and 0.04% loss for LB configuration.

Table 1. Results of the proposed method

Conclusion

In this contribution, a simplification of multi-intra-inter scheme is proposed. The proposed scheme will reduce the decoder complexity within an acceptable loss. It is recommended to be adopted in VTM or further studied in the CE.

References

[1] “CE10.1.1: Multi-hypothesis prediction for improving AMVP mode, skip or merge mode, and intra mode”, JVET_L0100, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Macao, CN, 3–12 Oct. 2018

[2] “JVET common test conditions and software reference configurations for SDR video,” JVET-K1010, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Ljubljana, SI, July 2018.

Patent rights declaration(s)

Hikvision Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).