| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0334 |
| --- | --- |

| Title: | Non-CE8: Removal of redundant syntax between CPR and other inter coding tools |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junghak Nam Jaehyun Lim Seung Hwan Kim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea | Tel: Email: | junghak.nam@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

When CPR tool is enabled in SPS, I-Slice is treated as P-Slice so that all syntax elements of the picture are signaled as P-Slice instead of I-Slice. Considering that current CPR mode does not work with some inter coding tools, such as MMVD, affine, multi-hypothesis, and tri-angular prediction, this contribution proposes to remove redundant syntax from those inter coding tools. Specifically, when the current decoded picture is the only reference picture for the current slice, syntax elements for inter coding tools, which does not work with CPR mode, is not signaled. From experiment results, following luma BD-rate changes are observed over VTM-3.0 with CPR:

- AI: -0.01% (CTC), -0.01% (Class F), -0.07% (Class SCC)

- RA: 0.00% (CTC), -0.03% (Class F), -0.06% (Class SCC)

Problem statement

When CPR tool in the SPS is enabled, I-Slice is treated as P-Slice so that all syntax elements of the picture is signaled by using syntax elements of P-Slice instead of I-Slice. However, current CPR mode does not seem to work with some inter coding tools, which are listed as follows:

- MMVD (merge motion vector difference)

- Affine motion compensated prediction

- Multi-hypothesis prediction

- Tri-angular prediction

Even if the current decoded picture is the only reference picture for the current slice, relevant syntax elements for those inter coding tools are still signaled.

Proposed method

This contribution proposes removal of redundant syntax between CPR and other inter coding tools. When the current decoded picture is the only reference picture for the current slice, syntax elements for disallowed inter coding tools are not signaled.

Proposed specification

| merge_data( x0, y0, cbWidth, cbHeight ) { | Descriptor |
| --- | --- |
| if (!CurrPicIsOnlyRef) { |  |
| mmvd_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_flag[ x0 ][ y0 ] = = 1 ) { |  |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if(MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8 ) |  |
| merge_subblock_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_subblock_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumSubblockMergeCand > 1 ) |  |
| merge_subblock_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if(sps_mh_intra_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 && cbHeight < 128 ) { |  |
| mh_intra_flag[ x0 ][ y0 ] | ae(v) |
| if( mh_intra_flag[ x0 ][ y0 ] ) { |  |
| if ( cbWidth <= 2 * cbHeight ¦ ¦ cbHeight <= 2 * cbWidth ) |  |
| mh_intra_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( mh_intra_luma_mpm_flag[ x0 ][ y0 ] ) |  |
| mh_intra_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| if(sps_triangle_enabled_flag && slice_type = = B && cbWidth * cbHeight >= 16 ) |  |
| merge_triangle_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_triangle_flag[ x0 ][ y0 ] ) |  |
| merge_triangle_idx[ x0 ][ y0 ] | ae(v) |
| else if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } else { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| if( slice_type != I ) { |  |
| if( treeType ! = DUAL_TREE_CHROMA ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 ) |  |
| pred_mode_flag | ae(v) |
| } |  |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( pcm_enabled_flag && cbWidth >= MinIpcmCbSizeY && cbWidth <= MaxIpcmCbSizeY && cbHeight >= MinIpcmCbSizeY && cbHeight <= MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( cbWidth, cbHeight, treeType) |  |
| } else { |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| if( ( y0 % CtbSizeY ) > 0 ) |  |
| intra_luma_ref_idx[ x0 ][ y0 ] | ae(v) |
| if (intra_luma_ref_idx[ x0 ][ y0 ] = = 0) |  |
| intra_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_mpm_flag[ x0 ][ y0 ] ) |  |
| intra_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| else |  |
| intra_luma_mpm_remainder[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else if( treeType ! = DUAL_TREE_CHROMA ) { /* MODE_INTER */ |  |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 ) { |  |
| merge_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_flag[ x0 ][ y0 ] ) { |  |
| merge_data( x0, y0, cbWidth, cbHeight ) |  |
| } else { |  |
| if( slice_type = = B ) |  |
| inter_pred_idc[ x0 ][ y0 ] | ae(v) |
| if(!CurrPicIsOnlyRef && sps_affine_enabled_flag && cbWidth >= 16 && cbHeight >= 16 ) { |  |
| inter_affine_flag[ x0 ][ y0 ] | ae(v) |
| if( sps_affine_type_flag && inter_affine_flag[ x0 ][ y0 ] ) |  |
| cu_affine_type_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| …. |  |

Experimental results

The proposed methods have been implemented on top of the CE8 software based on VTM-3.0 and tested under common test conditions in [1].

Table 1. Performance of proposed method with CPR off in AI

|  |  |  | AI Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0-CPR off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | -0.07% | 0.04% | 129% | 103% |
| Class A2 | -0.48% | -0.53% | -0.54% | 132% | 104% |
| Class B | -0.31% | -0.39% | -0.34% | 138% | 105% |
| Class C | -0.11% | -0.21% | -0.23% | 142% | 108% |
| Class E | -0.66% | -0.80% | -0.70% | 142% | 106% |
| Overall | -0.29% | -0.39% | -0.34% | 137% | 105% |
| Class D | -0.14% | -0.26% | 0.04% | 139% | 105% |
| Class F | -12.10% | -12.03% | -12.03% | 157% | 105% |
| Class SCC | -36.96% | -36.14% | -36.39% | 110% | 90% |

Table 2. Performance of proposed method with CPR off in RA

|  |  |  | RA Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0-CPR off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.13% | -0.06% | 0.29% | 100% | 101% |
| Class A2 | -0.01% | 0.13% | 0.15% | 98% | 102% |
| Class B | 0.01% | -0.06% | -0.34% | 100% | 101% |
| Class C | 0.20% | -0.08% | -0.07% | 103% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.08% | -0.03% | -0.04% | 100% | 102% |
| Class D | 0.22% | -0.15% | 0.31% | 104% | 103% |
| Class F | -9.91% | -9.91% | -9.96% | 108% | 102% |
| Class SCC | -22.09% | -21.67% | -22.17% | 105% | 101% |

Table 3. Performance of proposed method with CPR off in LDB

|  |  |  | LDB Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0-CPR off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.06% | -0.04% | 0.28% | 107% | 102% |
| Class C | 0.11% | 0.38% | 0.43% | 111% | 104% |
| Class E | 0.12% | 1.73% | 0.21% | 106% | 101% |
| Overall | 0.09% | 0.54% | 0.31% | 108% | 102% |
| Class D | 0.13% | 0.13% | 1.22% | 113% | 104% |
| Class F | -5.25% | -5.49% | -5.15% | 109% | 103% |
| Class SCC | -13.24% | -13.46% | -13.70% | 110% | 101% |

Table 4. Performance of proposed method with CPR on in AI

|  |  |  | AI Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0-CPR on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.03% | -0.02% | 102% | 102% |
| Class A2 | -0.01% | -0.02% | 0.01% | 102% | 104% |
| Class B | -0.01% | -0.01% | 0.01% | 102% | 104% |
| Class C | -0.02% | 0.03% | 0.02% | 102% | 108% |
| Class E | -0.03% | 0.01% | -0.11% | 102% | 105% |
| Overall | -0.01% | 0.01% | -0.01% | 102% | 105% |
| Class D | -0.04% | -0.03% | -0.17% | 102% | 104% |
| Class F | -0.01% | 0.01% | 0.06% | 102% | 107% |
| Class SCC | -0.07% | -0.05% | -0.06% | 101% | 107% |

Table 5. Performance of proposed method with CPR on in RA

|  |  |  | RA Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0-CPR on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.02% | 0.06% | 101% | 101% |
| Class A2 | -0.01% | 0.06% | -0.09% | 101% | 102% |
| Class B | 0.00% | -0.05% | -0.13% | 101% | 101% |
| Class C | 0.01% | -0.02% | -0.15% | 101% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.01% | -0.09% | 101% | 102% |
| Class D | 0.07% | 0.13% | 0.16% | 101% | 104% |
| Class F | -0.03% | 0.00% | -0.01% | 101% | 103% |
| Class SCC | -0.06% | -0.10% | -0.07% | 101% | 104% |

Table 6. Performance of proposed method with CPR on in LDB

|  |  |  | LDB Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0-CPR on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.28% | 0.42% | 101% | 101% |
| Class C | -0.01% | -0.01% | 0.10% | 101% | 104% |
| Class E | -0.16% | 0.66% | 0.39% | 101% | 101% |
| Overall | 0.00% | 0.28% | 0.30% | 101% | 102% |
| Class D | -0.01% | 0.29% | 0.52% | 102% | 103% |
| Class F | -0.13% | -0.13% | 0.33% | 101% | 103% |
| Class SCC | -0.15% | -0.06% | -0.08% | 101% | 104% |

Reference

- F. Bossen, et al., “JVET common test conditions and software reference configurations for SDR video,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET- L1010, 12th meeting, Macau, CN, 08–12 Oct. 2018.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).