| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0358-v2 |
| --- | --- |

| Title: | CE3-related: disabling PDPC based on availability of reference samples |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Virginie Drugeon Monzastr. 4c 63225 Langen Germany | Tel: Email: | +49-6103-766240 virginie.drugeon@eu.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

# Abstract

This contribution proposes to disable PDPC for certain intra blocks based on the availability of reference samples around the block. The results show tiny gains in both complexity and coding efficiency. Although the benefit is very small, the author believes that this is a logical modification of the PDPC process that is more beneficial for low resolution sequences and test conditions that differ from the CTC.

The present revision contains additional results with slightly different conditions as requested by the group.

# Introduction

The contribution proposes to disable PDPC automatically based on the availability of reference samples around the block. This is a follow-up contribution to JVET-L0204 [1] presented during the October 2018 meeting in Macao, during which it was commented that disabling PDPC at block level seems straightforward, simple and logical. However, the group asked for more results, which are provided in the present contribution.

# Disable PDPC based on availability of reference samples

PDPC is applied on top of the standard angular, DC or planar intra prediction. It consists in filtering the preliminary prediction signal obtained with angular, DC or Planar intra prediction with some of the reference samples depending on the position of the pixel in the prediction signal. The weights applied to the reference samples are position dependent, such that the weights are higher for pixels located close to the reference samples (i.e. at the upper or left border of the block) and gradually fall to zero for pixels located far from the reference samples (for example the pixels in the bottom right corner of the block). PDPC therefore shares some similarities with edge filtering.

It is proposed to disable automatically PDPC for one block when some reference samples are not available for that block. The reference samples that are considered depend on the intra prediction mode. Typically, the reference samples that are likely to be the most relevant for improving the quality of the prediction signal are checked for availability to determine if PDPC is disabled or not for the block.

Specifically, when the planar or DC intra prediction mode is used, the availability of the reference samples located above and left of the block is checked. If any of them is available, then PDPC is applied, as usual. Otherwise PDPC is disabled for the block. In the present revision, results are also added for the case where PDPC is applied on top of DC or Planar prediction only when the reference samples left and above the block are all available.

When vertical intra prediction modes are used, i.e prediction modes with an index higher than 34, then the availability of the reference samples located left of the block (as illustrated in Figure 1) is checked. Indeed, since the prediction signal itself is calculated using mostly the reference samples located above the block, the reference samples left of the block are more likely to have an impact to improve the quality of the prediction when using PDPC. Therefore, if these reference samples are not available, then PDPC is disabled for the block.

Figure 1: Checking the availability of reference samples left of the block for vertical prediction directions

When horizontal intra prediction modes are used, i.e prediction modes with an index lower than 34, then the availability of the reference samples located above the block (as illustrated in Figure 2) is checked. Indeed, since the prediction signal itself is calculated using mostly the reference samples located left of the block, the reference samples above the block are more likely to have an impact to improve the quality of the prediction when using PDPC. Therefore, if these reference samples are not available, then PDPC is disabled for the block.

Figure 2: Checking the availability of reference samples above the block for horizontal prediction directions

# Results in VTM-3.0

The results for all intra configuration for VTM-3.0 are presented below. The anchor is VTM-3.0 without modifications. It is compared to VTM-3.0 including the proposed modification to PDPC. Full results can be found in the file JVET-M0358_CTC.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.04% | -0.03% | 97% | 97% |
| Class A2 | 0.00% | -0.01% | -0.01% | 98% | 99% |
| Class B | -0.01% | -0.03% | -0.04% | 101% | 99% |
| Class C | -0.03% | -0.07% | -0.05% | 99% | 100% |
| Class E | -0.03% | 0.00% | 0.04% | 99% | 97% |
| Overall | -0.01% | -0.02% | -0.02% | 99% | 98% |
| Class D | -0.02% | 0.01% | 0.05% | 99% | 99% |
| Class F (optional) | 0.00% | 0.00% | 0.02% | 97% | 93% |

Table 1: Results for all intra configuration using CTC

There is a small runtime reduction for both encoder and decoder together with a tiny gain in coding efficiency for luma. The effect is tiny, because, with the CTC, there is a difference only for the blocks located at the upper and left borders of the picture.

The results for random access configuration show hardly any change.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.09% | 0.02% | 100% | 100% |
| Class A2 | 0.00% | -0.01% | 0.16% | 100% | 100% |
| Class B | 0.00% | 0.09% | 0.10% | 100% | 102% |
| Class C | 0.01% | -0.08% | 0.20% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.02% | 0.12% | 100% | 100% |
| Class D | -0.04% | -0.52% | -0.04% | 99% | 100% |
| Class F (optional) | 0.02% | -0.04% | 0.01% | 100% | 100% |

Table 2: Results for random access configuration using CTC

# Results in VTM-3.0 using tiles

In order to test the impact for other conditions than the CTC, VTM-3.0 was slightly modified to enable the use of tiles. In particular, the macro HEVC_TILES_WPP was turned on and a couple of straightforward fixes were integrated to enable compilation. These fixes consisted in renaming a couple of variables that were wrongly named in the existing code under HEVC_TILES_WPP. Unfortunately, it was noticed that it is not possible to test the random access condition, since the encoder currently crashes after coding one picture when tiles are used. Therefore, the results in this section only cover all intra conditions.

The first test performed used 4 tiles per picture, with each tile of size (picture width / 2) X (picture height / 2). The anchor is VTM-3.0 with the above-mentioned modifications and using 4 tiles per picture. To obtain this anchor, the following encoder options are used: --TileUniformSpacing=1 --NumTileColumnsMinus1=1 --NumTileRowsMinus1=1 --LFCrossTileBoundaryFlag=0. It is compared to the same configuration including the proposed modification to PDPC. The results are shown in Table 3 below. Full results can be found in the file JVET-M0358_4tiles_per_seq.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 4 tiles |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.04% | -0.10% | 96% | 92% |
| Class A2 | -0.01% | 0.00% | -0.02% | 98% | 93% |
| Class B | -0.02% | 0.00% | -0.02% | 101% | 96% |
| Class C | -0.04% | 0.04% | -0.06% | 100% | 99% |
| Class E | -0.07% | -0.20% | -0.05% | 100% | 95% |
| Overall | -0.03% | -0.03% | -0.05% | 99% | 95% |
| Class D | -0.05% | -0.07% | 0.00% | 99% | 99% |
| Class F (optional) | -0.01% | -0.08% | 0.02% | 97% | 91% |

Table 3: Results for all intra configuration using 4 tiles per picture

The second test performed used the smallest possible size for tiles, i.e. 1 CTU, in order to check the maximum effect that this modification may have. This configuration may be relevant for super low delay systems. The anchor is VTM-3.0 with the modifications mentioned at the beginning of this section and using tiles of size 128X128. To obtain this anchor, the following encoder options are used: --TileUniformSpacing=1 --NumTileColumnsMinus1=c --NumTileRowsMinus1=r --LFCrossTileBoundaryFlag=0, where c is calculated as floor((picture width - 1) / 128) and r is calculated as floor((picture height - 1) / 128). It is compared to the same configuration including the proposed modification to PDPC. The results are shown in Table 4 below. Full results can be found in the file JVET-M0358_tiles_of_1CTU.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 tiles of 1CTU |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.13% | -0.13% | -0.12% | 96% | 93% |
| Class A2 | -0.10% | -0.12% | -0.15% | 99% | 96% |
| Class B | -0.09% | -0.09% | -0.12% | 100% | 96% |
| Class C | -0.07% | -0.11% | -0.17% | 102% | 101% |
| Class E | -0.14% | -0.17% | -0.19% | 99% | 94% |
| Overall | -0.10% | -0.12% | -0.15% | 100% | 96% |
| Class D | -0.06% | 0.02% | -0.14% | 101% | 100% |
| Class F (optional) | -0.09% | -0.08% | -0.07% | 97% | 92% |

Table 4: Results for all intra configuration using tiles of 1 CTU

This idea may also have some benefit for intra prediction in inter slices in a cyclic intra refresh scenario, or when constrained intra is used. However, these configurations have not been tested.

# Subjective quality

The subjective quality of the intra coded pictures were compared with and without the proposed modification. In order to see maximum effect, this comparison was performed for the sequences encoded with small tiles of size 128X128 (second test in section 4 above). No degradation could be observed due to the proposed modification. It was even possible to detect some tiny improvements on some sequences, although the effect is rather small and difficult to detect in most cases.

When reference samples are not available, the last available reference sample is copied instead. Therefore, along a tile vertical edge, the reference samples left of the block are not available and the last available reference sample above the block is copied instead. This can lead to vertical lines in the block after the PDPC process (see Figure 3 a below), which is solved with the proposed modification (see Figure 3 b below).

Figure 3: Subjective differences due to the proposed modifications when using small tiles

# Additional results with the AND condition

Additional results have been requested by the group, in which PDPC is applied on top of the DC or Planar prediction only when all the reference samples left and above the block are available (and not when only some of them are available, as tested above). The results in VTM-3.0 for all intra configuration using the common test conditions are shown below. Full results can be found in the file JVET-M0358_ANDcond_forDCPlanar_CTC.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.08% | 0.01% | 101% | 103% |
| Class A2 | 0.00% | -0.03% | 0.01% | 101% | 104% |
| Class B | -0.01% | -0.03% | 0.02% | 100% | 101% |
| Class C | -0.01% | -0.02% | -0.05% | 101% | 102% |
| Class E | -0.03% | -0.10% | -0.03% | 101% | 101% |
| Overall | -0.01% | -0.05% | -0.01% | 101% | 102% |
| Class D | -0.01% | -0.07% | -0.03% | 101% | 102% |
| Class F | 0.01% | 0.05% | 0.12% | 101% | 100% |

Table 5: Results for all intra configuration using CTC for the AND condition

The results in Table 5 above are very similar to the results in Table 1, so that this small modification has little impact.

The small modification has also been tested using the smallest possible size for tiles, i.e. 1 CTU, in order to check the maximum effect that this modification may have. Full results can be found in the file JVET-M0358_ANDcond_forDCPlanar_tiles_of_1CTU.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0(1CTU/1Tile) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | -0.16% | -0.16% | 100% | 98% |
| Class A2 | -0.10% | -0.13% | -0.18% | 98% | 98% |
| Class B | -0.08% | -0.14% | -0.19% | 99% | 99% |
| Class C | -0.12% | -0.09% | -0.21% | 100% | 103% |
| Class E | -0.22% | -0.26% | -0.31% | 100% | 98% |
| Overall | -0.13% | -0.15% | -0.21% | 99% | 99% |
| Class D | -0.09% | -0.10% | -0.25% | 99% | 101% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 6: Results for all intra configuration using tiles of 1 CTU for the AND condition

When comparing the results in Table 4 and Table 6, it can be seen that the AND condition provides some tiny gain compared to the OR condition, but the results are very similar. Subjective quality has not been checked for the AND condition, contrary to the OR condition.

# Conclusion

A logical modification to the current PDPC approach is proposed that consists in disabling PDPC when the relevant reference samples are not available. The results show a tiny gain in coding efficiency together with a small decrease in runtime. Additional results show that the benefit is greater for use cases different from the Common Test Conditions, for example using small tiles. As it seems to be a logical complexity reducing modification, it is suggested to adopt it in VVC.

# References

- V. Drugeon, “CE3-related: disabling PDPC based on availability of reference samples”, JVET-L0204, JVET 12th Meeting: Macao, CN, 3–12 Oct. 2018.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex A: Specification text

The proposed specification text for this modification is drafted based on the text for general intra sample prediction in JVET-L1001-v4.

##### 8.2.4.2.1	General intra sample prediction

Inputs to this process are:

- a sample location ( xTbCmp, yTbCmp ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture,

- a variable predModeIntra specifying the intra prediction mode,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height,

- a variable cIdx specifying the colour component of the current block.

Outputs of this process are the predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1.

The variables refW and refH are derived as follows:

refW = nTbW * 2		(8-71)

refH = nTbH * 2		(8-72)

The variable refIdx specifying the intra prediction reference line index is derived as follows:

refIdx = ( cIdx  = =  0 )  ?  IntraLumaRefLineIdx[ xTbCmp ][ yTbCmp ]  :  0	(8-73)

For the generation of the reference samples p[ x ][ y ] with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx, the following ordered steps apply:

- The reference sample availability marking process as specified in clause 8.2.4.2.2 is invoked with the sample location ( xTbCmp, yTbCmp ), the intra prediction reference line index refIdx, the reference sample width refW, the reference sample height refH, the colour component index cIdx as inputs, and the reference samples refUnfilt[ x ][ y ] with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = − refIdx..refW − 1, y = −1 − refIdx as output.

- When at least one sample refUnfilt[ x ][ y ] with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx is marked as "not available for intra prediction", the reference sample substitution process as specified in clause 8.2.4.2.3 is invoked with the intra prediction reference line index refIdx, the reference sample width refW, the reference sample height refH, the reference samples refUnfilt[ x ][ y ] with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx, and the colour component index cIdx as inputs, and the modified reference samples refUnfilt[ x ][ y ] with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx as output.

- The reference sample filtering process as specified in clause 8.2.4.2.4 is invoked with the intra prediction reference line index refIdx, the transform block width nTbW and height nTbH, the reference sample width refW, the reference sample height refH, the unfiltered samples refUnfilt[ x ][ y ] with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx, and the colour component index cIdx as inputs, and the reference samples p[ x ][ y ] with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx as output.

The intra sample prediction process according to predModeIntra applies as follows:

- If predModeIntra is equal to INTRA_PLANAR, the corresponding intra prediction mode process specified in clause 8.2.4.2.5 is invoked with the transform block width nTbW, and the transform block height nTbH, and the reference sample array p as inputs, and the output is the predicted sample array predSamples.

- Otherwise, if predModeIntra is equal to INTRA_DC, the corresponding intra prediction mode process specified in clause 8.2.4.2.6 is invoked with the transform block width nTbW, the transform block height nTbH, and the reference sample array p as inputs, and the output is the predicted sample array predSamples.

- Otherwise, if predModeIntra is equal to INTRA_LT_CCLM, INTRA_L_CCLM or INTRA_T_CCLM, the corresponding intra prediction mode process specified in clause 8.2.4.2.8 is invoked with the intra prediction mode predModeIntra, the sample location ( xTbC, yTbC ) set equal to ( xTbCmp, yTbCmp ), the transform block width nTbW and height nTbH, and the reference sample array p as inputs, and the output is the predicted sample array predSamples.

- Otherwise, the corresponding intra prediction mode process specified in clause 8.2.4.2.7 is invoked with the intra prediction mode predModeIntra, the intra prediction reference line index refIdx, the transform block width nTbW, the transform block height nTbH, the reference sample width refW, the reference sample height refH, the colour component index cIdx, and the reference sample array p as inputs, and the modified intra prediction mode predModeIntra and the predicted sample array predSamples as outputs.

The current luma location ( xTbY, yTbY ) is derived as follows:

( xTbY, yTbY )  =  ( cIdx  = =  0 )  ?  ( xTbCmp, yTbCmp )  :  ( xTbCmp << 1, yTbCmp << 1 )

The variables availL and availT are derived as follows:

- The availability of left neighbouring samples derivation process for a block as specified in clause 6.4.X is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xTbY, yTbY ) and the neighbouring luma location ( xTbY − 1, yTbY ) as inputs, and the output is assigned to availL.

- The availability of top neighbouring samples derivation process for a block as specified in clause 6.4.X is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xTbY, yTbY ) and the neighbouring luma location ( xTbY, yTbY − 1 ) as inputs, and the output is assigned to availT.

When refIdx is equal to 0 and one of the following conditions is true, the position-dependent prediction sample filtering process specified in clause 8.2.4.2.9 is invoked with the intra prediction mode predModeIntra, the transform block width nTbW, the transform block height nTbH, the predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1, the reference sample width refW, the reference sample height refH, the reference samples p[ x ][ y ], with x = −1, y = −1..refH − 1 and x = 0..refW − 1, y = −1, and the colour component index cIdx as inputs, and the output is the modified predicted sample array predSamples:

- predModeIntra is equal to INTRA_PLANAR and at least one of availL or availT is equal to TRUE

- predModeIntra is equal to INTRA_DC and at least one of availL or availT is equal to TRUE

- predModeIntra is equal to INTRA_ANGULAR18 and availT is equal to TRUE

- predModeIntra is equal to INTRA_ANGULAR50 and availL is equal to TRUE

- predModeIntra is less than or equal to INTRA_ANGULAR10 and availT is equal to TRUE

- predModeIntra is greater than or equal to INTRA_ANGULAR58 and availL is equal to TRUE

# Annex B: Specification text for the AND condition

The specification text has been modified to use the AND condition instead of the OR condition for DC and Planar prediction modes as requested by the group. The only difference to the specification text in Annex A is additionally highlighted in green.

##### 8.2.4.2.1	General intra sample prediction

Inputs to this process are:

- a sample location ( xTbCmp, yTbCmp ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture,

- a variable predModeIntra specifying the intra prediction mode,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height,

- a variable cIdx specifying the colour component of the current block.

Outputs of this process are the predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1.

The variables refW and refH are derived as follows:

refW = nTbW * 2		(8-71)

refH = nTbH * 2		(8-72)

The variable refIdx specifying the intra prediction reference line index is derived as follows:

refIdx = ( cIdx  = =  0 )  ?  IntraLumaRefLineIdx[ xTbCmp ][ yTbCmp ]  :  0	(8-73)

For the generation of the reference samples p[ x ][ y ] with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx, the following ordered steps apply:

- The reference sample availability marking process as specified in clause 8.2.4.2.2 is invoked with the sample location ( xTbCmp, yTbCmp ), the intra prediction reference line index refIdx, the reference sample width refW, the reference sample height refH, the colour component index cIdx as inputs, and the reference samples refUnfilt[ x ][ y ] with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = − refIdx..refW − 1, y = −1 − refIdx as output.

- When at least one sample refUnfilt[ x ][ y ] with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx is marked as "not available for intra prediction", the reference sample substitution process as specified in clause 8.2.4.2.3 is invoked with the intra prediction reference line index refIdx, the reference sample width refW, the reference sample height refH, the reference samples refUnfilt[ x ][ y ] with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx, and the colour component index cIdx as inputs, and the modified reference samples refUnfilt[ x ][ y ] with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx as output.

- The reference sample filtering process as specified in clause 8.2.4.2.4 is invoked with the intra prediction reference line index refIdx, the transform block width nTbW and height nTbH, the reference sample width refW, the reference sample height refH, the unfiltered samples refUnfilt[ x ][ y ] with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx, and the colour component index cIdx as inputs, and the reference samples p[ x ][ y ] with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx as output.

The intra sample prediction process according to predModeIntra applies as follows:

- If predModeIntra is equal to INTRA_PLANAR, the corresponding intra prediction mode process specified in clause 8.2.4.2.5 is invoked with the transform block width nTbW, and the transform block height nTbH, and the reference sample array p as inputs, and the output is the predicted sample array predSamples.

- Otherwise, if predModeIntra is equal to INTRA_DC, the corresponding intra prediction mode process specified in clause 8.2.4.2.6 is invoked with the transform block width nTbW, the transform block height nTbH, and the reference sample array p as inputs, and the output is the predicted sample array predSamples.

- Otherwise, if predModeIntra is equal to INTRA_LT_CCLM, INTRA_L_CCLM or INTRA_T_CCLM, the corresponding intra prediction mode process specified in clause 8.2.4.2.8 is invoked with the intra prediction mode predModeIntra, the sample location ( xTbC, yTbC ) set equal to ( xTbCmp, yTbCmp ), the transform block width nTbW and height nTbH, and the reference sample array p as inputs, and the output is the predicted sample array predSamples.

- Otherwise, the corresponding intra prediction mode process specified in clause 8.2.4.2.7 is invoked with the intra prediction mode predModeIntra, the intra prediction reference line index refIdx, the transform block width nTbW, the transform block height nTbH, the reference sample width refW, the reference sample height refH, the colour component index cIdx, and the reference sample array p as inputs, and the modified intra prediction mode predModeIntra and the predicted sample array predSamples as outputs.

The current luma location ( xTbY, yTbY ) is derived as follows:

( xTbY, yTbY )  =  ( cIdx  = =  0 )  ?  ( xTbCmp, yTbCmp )  :  ( xTbCmp << 1, yTbCmp << 1 )

The variables availL and availT are derived as follows:

- The availability of left neighbouring samples derivation process for a block as specified in clause 6.4.X is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xTbY, yTbY ) and the neighbouring luma location ( xTbY − 1, yTbY ) as inputs, and the output is assigned to availL.

- The availability of top neighbouring samples derivation process for a block as specified in clause 6.4.X is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xTbY, yTbY ) and the neighbouring luma location ( xTbY, yTbY − 1 ) as inputs, and the output is assigned to availT.

When refIdx is equal to 0 and one of the following conditions is true, the position-dependent prediction sample filtering process specified in clause 8.2.4.2.9 is invoked with the intra prediction mode predModeIntra, the transform block width nTbW, the transform block height nTbH, the predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1, the reference sample width refW, the reference sample height refH, the reference samples p[ x ][ y ], with x = −1, y = −1..refH − 1 and x = 0..refW − 1, y = −1, and the colour component index cIdx as inputs, and the output is the modified predicted sample array predSamples:

- predModeIntra is equal to INTRA_PLANAR and availL and availT are both equal to TRUE

- predModeIntra is equal to INTRA_DC and availL and availT are both equal to TRUE

- predModeIntra is equal to INTRA_ANGULAR18 and availT is equal to TRUE

- predModeIntra is equal to INTRA_ANGULAR50 and availL is equal to TRUE

- predModeIntra is less than or equal to INTRA_ANGULAR10 and availT is equal to TRUE

- predModeIntra is greater than or equal to INTRA_ANGULAR58 and availL is equal to TRUE