| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0382 |
| --- | --- |

| Title: | CE2-related: modification of Triangle and MMVD merge indexes coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guillaume Laroche Christophe Gisquet Patrice Onno Jonathan Taquet Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 guillaume.laroche@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution presents the results of modifying the contexts derivation for Triangle and MMVD Merge index coding. The proposed modification consists in sharing the contexts used for Triangle and MMVD Merge indexes coding with the context of the regular Merge index in order to reduce the amount of contexts inside VTM3.0.

It is reported that the proposed modification obtains the average Luma BDR results of 0.01%, 0.05%, -0.03% for respectively RA, LD and LDP configurations compared to VTM3.0.

Additional results are reported on top CE2.2.2 tests where the Subblock Merge index and Merge index is also modifying to reduce the amount of contexts dedicated to Merge index coding.

# Introduction

VTM3.0 contains several Merge modes and several have different Merge indexes coding. Yet, each of them uses at least one context for the first bit of their Merge indexes. The following table summarizes the amount of contexts for each Merge index coding. Currently 7 contexts are used for the Merge index coding.

Table 1 Number of contexts for Merge index coding for Regular, Subblock MMVD and Triangle Merge modes.

|  | Number of contexts for Merge index conding |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Regular Merge | Subblock | MMVD | Triangle |  | Total |
| VTM3.0 | 1 | 4 | 1 | 1 |  | 7 |

# Proposed modification

The proposed modification reduces the number of contexts by sharing the context of MMVD and Triangle Merge mode as depicted in the following table.

The proposed modification can be combined with CE2.2.2 tests to reduce the amount of contexts from 7 to only 1 as depicted in Table 2.

Table 2 Number of contexts for Merge index coding for Regular, Subblock MMVD and Triangle Merge modes for several configurations.

|  | Number of contexts for Merge index coding |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Subblock | Regular Merge | MMVD | Triangle |  | Total |
| VTM3.0 | 4 | 1 | 1 | 1 |  | 7 |
| Proposal | 4 | 1 |  |  |  | 5 |
| CE2.2.2.a | 1 | 1 | 1 | 1 |  | 4 |
| CE2.2.2.b | 1 |  | 1 | 1 |  | 3 |
| Proposal +CE2.2.2.a | 1 | 1 |  |  |  | 2 |
| Proposal +CE2.2.2.b | 1 |  |  |  |  | 1 |

# Experimental results

The proposed modification have been implemented on top of CE2.2.2 source code.

The following table gives the Luma average BDR results for each encoding inter configuration (RA, LD, LDP) for the proposed modification and combinations with CE2.2.2 tests compared to the number of contexts for all Merge index types.

Table 3 BDR results for all combinations.

|  | Number of contexts for Merge index coding |  |  |  |
| --- | --- | --- | --- | --- |
|  | Number of contexts for Merge indexes coding | Y BDR RA | Y BDR LD | Y BDR LDP |
| VTM3.0 | 7 | - | - | - |
| Proposal | 5 | 0.01% | 0.05% | -0.03% |
| CE2.2.2.a | 4 | 0.00% | 0.03% | -0.04% |
| CE2.2.2.b | 3 | 0.07% | 0.05% | 0.01% |
| Proposal +CE2.2.2.a | 2 | 0.02% | 0.03% | -0.02% |
| Proposal +CE2.2.2.b | 1 | 0.09% | 0.11% | 0.00% |

# Conclusion

In this contribution, we propose an additional modification to reduce the amount of contexts dedicated to the Merge index coding compared to CE2.2.2 tests. The proposed modification has no impact on coding efficiency. When combined to CE2.2.2.b the modification reduces by 7 the amount of contexts for the Merge indexes coding.

According to the minor impact on coding efficiency, the complexity reduction and the design simplification we recommend to adopt the proposed modification for the next VTM software.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).