| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0391 |
| --- | --- |

| Title: | CE3-related: Improvements on the Decoder-side Intra Mode Derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Contribution |  |  |
| Author(s) or Contact(s): | Mohsen Abdoli Elie Mora Thomas Guionnet Mickael Raulet 12D Rue de Patis Tatelin 35700 Rennes, France | Email: | m.abdoli@ateme.com e.mora@ateme.com t.guionnet@ateme.com m.raulet@ateme.com |
| Source: | ATEME |  |  |

_____________________________

# Abstract

This contribution proposes two modifications to JVET-M0094 – the Decoder-side Intra Mode Derivation (DIMD) algorithms. The first proposal changes the RDO check of the DIMD mode for the second EMT pass, while, the second proposal changes the virtual intra mode of a DIMD block presented to its corresponding Chroma block for mode coding. In VTM3.0, the original DIMD algorithm brings -0.11% BD-R gain in AI configuration. In this document, we introduce three tests based on different combinations of the above two proposals: Test 1) applying only the first proposal brings: -0.15% BD-R gain, Test 2) applying only the second proposal: -0.12% BD-R gain and Test 3) applying both proposals: -0.15% BD-R gain.

- Introduction

The general idea of DIMD is to avoid the signaling of angular Luma intra mode and leave it to be derived from the neighboring texture at the decoder side, as described in JVET-M0094 [2].

The mode derivation in DIMD is carried out on two lines of reference pixels from the CUs at left, above and above-left. These pixels are processed to produce a histogram of gradient (HoG) based on the existing 65 angular intra modes. Once the HoG is calculated, the angular mode with the largest histogram entry is derived as the intra mode of block.

The use of the DIMD is explicitly signaled with a CU-level flag. In the current implementation of DIMD on top of VTM3.0, this flag is coded before coding other prediction mode information (e.g. MRL index, MPM flag etc.).

- Proposed Modifications

## Modification of RDO loop when the EMT index is non-zero

This proposal forces DIMD to comply the current implementation of the EMT in VTM3.0 [3]. As the proposed modification concerns only the RDO process at the encoder side, its bitstream is still decodable with the decoder of JVET-M0094 [2].

In the implementation of JVET-M0094, the DIMD mode is always performed in both passes of EMT flag. This is regardless of the EMT thresholding in the second pass, where the RD cost of candidate intra modes from the first pass are compared to the best RD cost from the first pass multiplied by a pre-determined value (larger than 1), in order to determine whether to check them in the second pass.

The proposed modification applies the EMT thresholding on the DIMD mode, as well. More precisely, the RD cost of the DIMD mode from the first pass is compared to the best RD cost from the first pass multiplied by a pre-determined value, and if the first is larger than the latter, then the DIMD mode is skipped in the second EMT pass.

## Modification of virtual intra mode for Chroma mode coding

This normative proposal changes the Luma intra mode of a DIMD block provided to its corresponding Chroma block, for intra mode coding during parsing. Currently, Chroma blocks of blocks coded by DIMD use NOMODE_IDX=255 to fill their candidate mode lists. Table 1 describes the current and proposed Chroma mode derivation in red and green lines, respectively.

getIntraChromaCandModes()

{

modeList[8]={PLANAR_IDX,VER_IDX,HOR_IDX,DC_IDX,LM_CHROMA_IDX,MDLM_L_IDX,MDLM_T_IDX, DM_CHROMA_IDX};

lumaMode = cu->dimd ? NOMODE_IDX : cu->intraDir; // current

lumaMode = cu->dimd ? PLANAR_IDX : cu->intraDir; // proposal

for ( i = 1 : 4 )

{

if ( lumaMode == modeList[i] )

{

modeList[i] = VDIA_IDX;

break;

}

}

}

Table 1: Proposed modification to the construction of the Chroma candidate mode list from luma intra mode.

- Experimental results

Based on the two proposed modifications, explained in Section 2, four tests have been conducted:

- Test 1: Applying only the test described in section 2.1.

- Test 2: Applying only the test described in section 2.2.

- Test 3: Applying both tests of sections 2.1 and 2.2.

The proposed tests described in section 2 are evaluated for intra-only (AI) configuration on top of VTM-3.0 [3]. The corresponding simulations were conducted on an Intel(R) Xeon(R) Gold 6150 CPU @ 2.70GHz with CentOS Linux 7 and a GCC 4.8.5 compiler.

## Results of the proposed tests

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,21% | -0,16% | -0,13% |  |  |
| Class A2 | -0,10% | 0,01% | -0,03% |  |  |
| Class B | -0,10% | -0,05% | -0,09% |  |  |
| Class C | -0,16% | -0,15% | -0,21% |  |  |
| Class E | -0,23% | -0,14% | -0,03% |  |  |
| Overall | -0,15% | -0,10% | -0,10% |  |  |
| Class D | -0,22% | -0,26% | -0,15% |  |  |
| Class F (optional) | -0,27% | -0,29% | -0,18% |  |  |

Table 1: Results of Test 1 in the AI mode

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,17% | -0,18% | -0,05% |  |  |
| Class A2 | -0,06% | 0,01% | 0,01% |  |  |
| Class B | -0,06% | -0,01% | -0,04% |  |  |
| Class C | -0,13% | -0,11% | -0,18% |  |  |
| Class E | -0,18% | -0,12% | -0,06% |  |  |
| Overall | -0,12% | -0,08% | -0,07% |  |  |
| Class D | -0,20% | -0,10% | -0,13% |  |  |
| Class F (optional) | -0,25% | -0,14% | -0,11% |  |  |

Table 2: Results of Test 2 in the AI mode

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.21% | -0.21% | -0.14% |  | 102% |
| Class A2 | -0.10% | -0.05% | -0.04% |  | 103% |
| Class B | -0.10% | 0.00% | -0.06% |  | 106% |
| Class C | -0.16% | -0.11% | -0.26% |  | 111% |
| Class E | -0.23% | -0.22% | -0.12% |  | 109% |
| Overall | -0,15% | -0,10% | -0,13% |  | 106% |
| Class D | -0.23% | -0.17% | -0.20% |  | 108% |
| Class F (optional) | -0.28% | -0.33% | -0.21% |  | 108% |

Table 3: Results of Test 3 in the AI mode

# Cross-checks

Cross-check of Test 3 of this contribution has been carried out by Orange Labs.

# Conclusion

This contribution proposes two modifications, with three different combinations, described in Test 1,2 and 3. All the reported results are obtained on top of JVET-M0094 [2] and show improvements in the performance of DIMD.

- References

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-K1010, Ljubljana, July 2018

- E. Mora, and A. Nasrallah, and M. Raulet, “CE3-related: Decoder-side Intra Mode Derivation”, JVET-M0094, Marrakech, January 2019.

- VTM-3.0 software, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM.git, November 2018.

# Patent rights declarations

ATEME may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).