Added texts are highlighted. Deleted texts are marked with red strike

The affine motion parameter checking process

Input to this process are:

- the control point motion vectors cpMvLX[ cpIdx ], with cpIdx = 0..numCpMv − 1 and X being 0 or 1,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

Outputs of this process is:

- a variable Use8x8SubBlock

The variable pelV is set equal to 128 * 16.

The variable DMvHorX is set equal to (cpMvLX[1][0] – cpMvLX[0][0])*128/cbWidth

The variable DMvHorY is set equal to (cpMvLX[1][1] – cpMvLX[0][1])*128/cbWidth

The variable DMvVerX is set equal to (cpMvLX[2][0] – cpMvLX[0][0])*128/cbHeight

The variable DMvVerY is set equal to (cpMvLX[2][1] – cpMvLX[0][1])*128/ cbHeight

If any of the following is False, the variable isSwithFilter[LX] is set to True:

- 4 * DMvHorX > -4 * pelV

- 4 * DMvHorX < pelV

- 4 * DMvHorY > -pelV

- 4 * DMvHorY < pelV

- 4 * DMvVerX > -pelV

- 4 * DMvVerX < pelV

- 4 * DMvVerY > -4 * pelV

- 4 * DMvVerY < pelV

- 4 * DMvHorX + 4 * DMvVerX > -4 * pelV

- 4 * DMvHorX + 4 * DMvVerX < pelV

- 4 * DMvHorY + 4 * DMvVerY > -4 * pelV

- 4 * DMvHorY + 4 * DMvVerY < pelV

Otherwise the variable isSwithFilter[LX] is set to False

In interpolation function of Luma component, if isSwithFilter[LX ] == True, the chroma interpolation filter is used. Otherwise, the luma interpolation filter is used.