| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0400-v3 |
| --- | --- |

| Title: | CE2-related: Worst-case memory bandwidth reduction for VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei-Jung Chien, Luong Pham Van, Han Huang, Vadim Seregin, and Marta Karczewicz 5775 Morehouse Dr San Diego, CA 92121 | Tel: Email: | +1-858-658-5664 wchien@qti.qualcomm.com lphamvan@qti.qualcomm.com |
| Source: | Qualcomm Technologies Inc. |  |  |

_____________________________

# Abstract

This contribution proposes two different approaches to reduce the worst-case memory bandwidth of motion compensation. The first solution (block size restriction) is to restrict bi-predictions that have memory bandwidth exceeding a predefined level. In also this approach, the sub-block size of affine prediction can be set to be higher than 4x4 to ensure the memory requirement. In the second solution (switch filter), the luma interpolation of a prediction can be performed by the chroma interpolation filter to ensure the memory bandwidth requirement. The average luma BD-rate changes of the tests that achieve the worst-case bandwidth of 47%, 14% and 8% the worst-case of HEVC are:

Using block size restriction:

RA: 0.03%, 0.12% and 0.51%, respectively.

LDB: -0.01%, 0.13%, and 0.53%, respectively.

Using switch filter:

RA: -0.01%, 0.02, and 0.26%, respectively.

LDB: 0.02%, 0.01%, and 0.45%, respectively.

# Introduction

In real decoder implementation, it is desired to reduce the memory bandwidth requirement, while keeping a good trade-off between bandwidth reduction and coding performance.

Motion compensation of small block sizes needs a high bandwidth to fetch the needed data from memory. In the current VTM, the minimum size (width or height) of a luma block is 4. This leads to the following predictions having a higher memory bandwidth compared to the worst-case of HEVC (INTER8x8 bi-prediction): affine bi-prediction (4x4 sub-block), uni-prediction of an INTER4x4, bi-prediction of an INTER8x4/INTER4x8, INTER16x4/4x16, and. In this document, the memory bandwidth of a prediction is defined as the number of luma fetched pixels over a luma pixel. It is noted that in Table 1, 9x9 bi-prediction is the worst-case memory bandwidth of affine prediction which is proposed in [1].

Table 1 Memory bandwith of different block sizes for motion compensation

| Prediction | Block size | Number of fetch pixels | Number of fetched pixels /1 luma pixel | Over HEVC worst case (%) |
| --- | --- | --- | --- | --- |
| bi | 4x4 | 242 | 15.13 | 115 |
| uni | 4x4 | 121 | 7.56 | 8 |
| bi | 4x8/8x4 | 330 | 10.31 | 47 |
| bi | 4x16/16x4 | 506 | 7.91 | 12 |
| bi | 8x8 | 450 | 7.03 | 0 |
| bi | 9x9 | 512 | 8.00 | 14 |

# Proposed Method

This contribution proposes two different techniques including block size restriction and switching filter to reduces the worst-case memory bandwidth of motion compensation to a predefined level.

  - Worst-case memory bandwidth reduction based on block size restriction

This approach reduces the worst-case bandwidth by restricting the predictions of a block that has the bandwidth excessing the predefined level.

For sub-block motion compensation, e.g., affine, if the memory bandwidth of the prediction is higher than the pre-defined level, the motion compensation is performed using a higher sub-block size. The conditions to check whether the memory bandwidth of an affine CU exceeds a predefined level are based on the motion vector differences of the control points. These conditions are proposed in [1].

  - Worst-case memory bandwidth reduction based on switching filter for luma

In this solution, the filter of the luma interpolation (8-tap filter) is switched to the filter of the chroma prediction (4-tap filter) if the bandwidth of the prediction is higher than predefined level. For reference, the memory bandwidth of different luma predictions using chroma filter is summarized in Table 2.

For motion compensation of sub-block such as affine prediction, the conditions to check whether the memory bandwidth of an affine CU is higher than a predefined level are also inherited from [1]. If the control point motion differences satisfy the condition, the luma filter is used for interpolation of the luma component. Otherwise, the chroma filter is performed for interpolation of the luma component.

Table 2 Memory bandwidth of prediction using chroma interpolation filter

| Prediction | Block size | Number of fetch pixels | Number of fetched pixels /1 luma pixel | Over HEVC worst case (%) |
| --- | --- | --- | --- | --- |
| Bi | 4x4 | 98 | 6.13 | -13 |
| Uni | 4x4 | 49 | 3.06 | -56 |
| Bi | 4x8/8x4 | 154 | 4.81 | -32 |
| Bi | 4x16/16x4 | 266 | 4.16 | -41 |

  - Tests description:

Three worst-case banwidths including 47%, 14% and 8% over the worst-case of HEVC are tested.

Worst-case reduction based on block size restriction:

Test1.1: 47% over HEVC: (worst case is 4x8/8x4 bi-prediction)

9x9 affine restriction is tested for affine bi-prediction.

If the control points do not satisfy the restriction, the sub-block size is set equal to 8x8.

Test 1.2: 14% over HEVC: (worst case is affine 9x9 bi-prediction)

Include Test 1.1 restriction and bi-prediction is excluded from INTER4x8/INTER8x4.

Test 1.1: 8% over HEVC: (worst case is 4x4 uni-prediction)

Affine bi-prediction is performed using sub-block size of 8x8.

bi-prediction is excluded from INTER4x8/INTER8x4 and INTER4x16/INTER16x4.

Worst-case reduction based on switch filter:

Test2.1: 47% over HEVC: (worst case is 4x8/8x4 bi-prediction)

9x9 affine restriction is tested for affine bi-prediction.

If the control points do not satisfy the restriction, the interpolation filter is switched to chroma interpolation filter for motion compensation.

Test2.2: 14% over HEVC: (worst case is affine 9x9 bi-prediction)

Include Test 2.1 restriction and motion compensation of bi-prediction INTER4x8/INTER8x4 is switched to chroma interpolation filter.

Test 2.3: 8% over HEVC:(worst case is 4x4 uni-prediction)

Motion compensation of bi-prediction of INTER4x8/INTER8x4, INTER4x16/INTER16x4 and affine bi-prediction is switched to chroma interpolation filter.

# Experimental Results

The proposed solutions have been implemented on VTM3.0. The tests are evaluated under the RA and LDB configurations. The average luma BD-rate changes of the tests are visulaized in Figure 1. More details of the expiremental results are reported in Table 3-8.

The luma BD-rate changes of the tests which achieve the worst-case bandwith over HEVC by 47%, 14% and 8% are reported as:

Under the RA configuration:

Test 1.1, Test 1.2, and Test 1.3: 0.03%, 0.12% and 0.51%, respectively.

Test 2.1, Test 2.2, and Test 2.3: -0.01%, 0.02, and 0.25%, respectively.

Under the LDB configuration:

Test 1.1, Test 1.2, and Test 1.3: -0.01%, 0.13%, and 0.53%, respectively.

Test 2.1, Test 2.2, and Test 2.3: 0.02%, 0.01%, and 0.45%, respectively.

Figure 1. Performance visualization of the proposed techniqes at diffrernt worst-case bandwidths.

Table 3 Test1.1: 47% over HEVC – Block size restriction

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.05% | -0.01% | 99% | 100% |
| Class A2 | 0.14% | 0.16% | 0.17% | 98% | 99% |
| Class B | 0.00% | 0.00% | -0.11% | 98% | 98% |
| Class C | 0.03% | 0.06% | 0.06% | 102% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 0.06% | 0.01% | 99% | 99% |
| Class D | 0.00% | -0.11% | 0.07% | 101% | 102% |
| Class F (optional) | 0.05% | 0.03% | 0.03% | 98% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.28% | -0.10% | 101% | 100% |
| Class C | 0.00% | 0.01% | -0.04% | 102% | 102% |
| Class E | -0.05% | 0.11% | -0.05% | 99% | 99% |
| Overall | -0.01% | -0.08% | -0.07% | 101% | 100% |
| Class D | 0.03% | -0.11% | -0.10% | 100% | 95% |
| Class F (optional) | 0.08% | 0.24% | 0.05% | 99% | 98% |

Table 4 Test1.2: 14% over HEVC – Block size restriction

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.05% | 0.06% | 100% | 101% |
| Class A2 | 0.20% | 0.33% | 0.44% | 98% | 97% |
| Class B | 0.09% | 0.24% | 0.21% | 98% | 98% |
| Class C | 0.19% | 0.23% | 0.30% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.12% | 0.22% | 0.25% | 99% | 100% |
| Class D | 0.34% | 0.36% | 0.36% | 101% | 101% |
| Class F | 0.16% | 0.18% | 0.22% | 99% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.12% | -0.04% | 0.30% | 101% | 102% |
| Class C | 0.21% | 0.50% | 0.45% | 101% | 101% |
| Class E | 0.06% | 0.27% | -0.85% | 99% | 100% |
| Overall | 0.13% | 0.22% | 0.06% | 101% | 101% |
| Class D | 0.33% | 0.62% | 0.33% | 99% | 95% |
| Class F | 0.13% | 0.04% | 0.39% | 99% | 99% |

Table 5 Test1.3: 8% over HEVC – Block size restriction

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | 0.06% | 0.15% | 98% | 99% |
| Class A2 | 0.71% | 0.73% | 0.83% | 96% | 97% |
| Class B | 0.56% | 0.72% | 0.67% | 97% | 96% |
| Class C | 0.62% | 0.67% | 0.76% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.51% | 0.58% | 0.62% | 98% | 98% |
| Class D | 1.08% | 1.05% | 1.30% | 98% | 100% |
| Class F | 0.68% | 0.70% | 0.74% | 98% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.55% | 0.36% | 0.96% | 99% | 98% |
| Class C | 0.59% | 0.83% | 0.71% | 99% | 101% |
| Class E | 0.42% | 0.29% | 0.35% | 96% | 97% |
| Overall | 0.53% | 0.50% | 0.72% | 98% | 99% |
| Class D | 1.21% | 0.69% | 0.86% | 96% | 94% |
| Class F | 0.56% | 0.43% | 0.75% | 97% | 98% |

Table 6 Test2.1: 47% over HEVC – Switch filter

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.11% | -0.04% | 100% | 102% |
| Class A2 | -0.02% | 0.00% | 0.02% | 102% | 101% |
| Class B | -0.02% | 0.09% | 0.05% | 100% | 99% |
| Class C | -0.01% | -0.02% | -0.01% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.00% | 0.01% | 100% | 101% |
| Class D | 0.02% | -0.04% | 0.06% | 100% | 100% |
| Class F | 0.01% | -0.03% | -0.06% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.04% | -0.34% | -0.05% | 101% | 102% |
| Class C | -0.01% | 0.05% | -0.05% | 102% | 100% |
| Class E | 0.02% | 0.28% | 0.12% | 102% | 104% |
| Overall | 0.02% | -0.06% | 0.00% | 102% | 102% |
| Class D | 0.02% | 0.31% | -0.02% | 101% | 100% |
| Class F | 0.04% | -0.06% | 0.09% | 101% | 103% |

Table 7 Test2.2: 14% over HEVC – Switch filter

|  |  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-3.0 |  |  |  |  |  |  |  |  |  |
|  |  | Y |  | U |  | V |  | EncT |  | DecT |  |
| Class A1 |  | 0.03% |  | 0.01% |  | 0.08% |  | 100% |  | 100% |  |
| Class A2 |  | 0.01% |  | -0.03% |  | -0.05% |  | 101% |  | 101% |  |
| Class B |  | -0.01% |  | 0.10% |  | 0.01% |  | 100% |  | 99% |  |
| Class C |  | 0.06% |  | 0.08% |  | 0.05% |  | 100% |  | 101% |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall |  | 0.02% |  | 0.05% |  | 0.02% |  | 100% |  | 100% |  |
| Class D |  | 0.14% |  | -0.06% |  | 0.05% |  | 100% |  | 100% |  |
| Class F |  | 0.02% |  | -0.04% |  | 0.01% |  | 100% |  | 99% |  |
|  |  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  |  | Over VTM-3.0 |  |  |  |  |  |  |  |  |  |
|  |  | Y |  | U |  | V |  | EncT |  | DecT |  |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |
| Class B |  | 0.03% |  | -0.11% |  | 0.03% |  | 101% |  | 102% |  |
| Class C |  | 0.07% |  | 0.40% |  | 0.24% |  | 102% |  | 104% |  |
| Class E |  | -0.12% |  | 0.25% |  | -0.50% |  | 101% |  | 101% |  |
| Overall |  | 0.01% |  | 0.15% |  | -0.03% |  | 101% |  | 102% |  |
| Class D |  | 0.17% |  | 0.71% |  | 0.51% |  | 101% |  | 99% |  |
| Class F |  | 0.07% |  | -0.09% |  | 0.08% |  | 101% |  | 103% |  |

Table 8 Test2.3: 8% over HEVC – Switch filter

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.28% | -0.15% | 107% | 115% |
| Class A2 | 0.17% | 0.01% | 0.06% | 100% | 100% |
| Class B | 0.36% | 0.10% | -0.04% | 84% | 96% |
| Class C | 0.39% | 0.16% | 0.22% | 83% | 96% |
| Class E |  |  |  |  |  |
| Overall | 0.26% | 0.01% | 0.02% | 91% | 100% |
| Class D | 0.78% | 0.38% | 0.38% | 83% | 93% |
| Class F | 0.14% | 0.14% | 0.12% | 83% | 96% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.32% | 0.09% | 0.34% | 102% | 111% |
| Class C | 0.69% | 0.85% | 0.83% | 96% | 99% |
| Class E | 0.35% | 0.66% | 0.34% | 83% | 93% |
| Overall | 0.45% | 0.49% | 0.50% | 95% | 102% |
| Class D | 1.89% | 1.69% | 1.37% | 82% | 89% |
| Class F | 0.22% | 0.23% | 0.14% | 82% | 93% |

# Conclusion

This contribution proposes two different techniques to reduces the worst-case memory bandwidth of motion compensation to a predefined level. These techniques include block size restriction and switching filter for the luma component.

# Reference

[1] L. Pham Van, W.-J. Chien, H. Huang, V. Seregin, M. Karczewicz, “CE4-related: Affine restrictions for the worst-case bandwidth reduction”, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, doc: JVET-L0396, 12th Meeting: Macao, CN, 3–12 Oct. 2018.

# Patent rights declaration(s)

Qualcomm Technologies Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).