| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0427-v2 |
| --- | --- |

| Title: | CE12: Mapping functions (test CE12-1 and CE12-2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Taoran Lu, Fangjun Pu, Peng Yin, Walt Husak, Sean McCarthy, Tao Chen 432 Lakeside Drive, Sunnyvale, CA, 94085 US | Tel: Email: | {pyin, tlu, fpu}@dolby.com |  |
| Source: | Dolby Laboratories, Inc. |  |  |  |

_____________________________

# Abstract

This contribution presents test results of CE12-1 and CE12-2 for mapping functions. CE12-1 tested the in-loop reshaper in L0246 on VTM3.0 software base with focus on coding efficiency improvement. CE12-2 tested additional complexity reduction on CE12-1 for both software and hardware implementation friendliness.

- For CE12-1, average BDRate for Y/U/V:

  - RA    { -1.28%,	0.53%,	0.52% }, Enc: 105%, Dec: 101%

  - AI     { -0.89%, 1.52%, 1.15% }, Enc: 103%, Dec: 100%

  - LDB { -0.72%,	0.46%, 0.11% }, Enc: 100%, Dec: 101%

- For CE12-2, average BDRate for Y/U/V:

  - RA    { -1.30%, 1.45%, 1.44% }, Enc: 105%, Dec: 104%

  - AI     { -0.89%, 1.49%, 1.18% }, Enc: 102%, Dec: 101%

  - LDB { -0.76%, 1.78%, 1.42% }, Enc:  98%, Dec: 105%

Greater gain is observed for higher resolution HD & UHD clips (class A1/A2/B/ArenaOfValor).

- For both CE12-1 and CE12-2, average BDRate for Y: -1.4%, -1.2%. and -1.1% for RA, AI, and LDB, respectively.

CE12-2 comprises the following changes over L0246: removal of latency for reshaping in intra prediction loop; use of 16-piece PWL model for LUT signaling and constructing; simplification of arithmetic for chroma residual scaling; unification of SDR and HDR syntax and decoding process.

CE12-2 reshaper additional test results for implementation-friendly chroma residual scaling configurations are very similar to those for the fully-enabled CE12-2 reshaper configuration for luma (RA and AI) and chroma (AI) for both dual-tree and single-tree tests, and ~1% degraded for chroma (RA) for the dual-tree test.

- For CE12-2.1 (dual-tree anchor and test, chroma residual scaling disabled (intra slices), chroma residual scaling disabled for 2x2 chroma (inter slices), and use of forward-mapped pedicted luma values), average BDRate for Y/U/V:

  - RA    { -1.35%, 	2.49%, 	2.40% }, Enc: 103%, Dec: 103%

  - AI     { -0.87%, 1.30%, 1.17% }, Enc: 102%, Dec: 101%

- For CE12-2.2 (single-tree anchor and test, chroma residual scaling disabled for 2x2 chroma, and use of forward-mapped predicted luma values), average BDRate for Y/U/V:

  - RA    { -1.21%, 1.11%, 1.10% }

  - AI     { -0.90%, 1.45%, 1.15% }

It is recommended that the CE12-2 reshaper be adopted as a core coding tool in VVC for the best performance and minimum complexity.

# Introduction

In-loop reshaping has been proposed in [1] and was tested in multiple VVC core experiments [2][3][4][5] in the past JVET meetings. Test results based on VTM1 and VTM2 show consistent gain for both SDR and HDR video compared to the CTC anchor. This contribution presents the test results of CE12-1 and CE12-2 for in-loop reshaping on VTM3, according to the CE description [6].

CE12-1 addresses comments received during JVET-Lth meeting by providing encoder optimization and simplification of L0246 reshaper.

CE12-2 provides additional complexity reduction for both software and hardware implementation friendliness. The in-loop reshaper can work under minimal latency and small piece-wise linear LUT size, while retaining coding efficiency similar to CE12-1.

# Algorithm Description

## In-loop luma reshaping architecture in CE12-1

The in-loop luma reshaper architecture presented in L0246/L0247 is tested in CE12-1.

The in-loop luma reshaper is implemented as a pair of look-up tables (LUTs), but only one of the two LUTs need to be signaled as the other one can be computed from the signaled LUT. Each LUT is a one-dimensional, 10-bit, 1024-entry mapping table (1D-LUT).  One LUT is a forward LUT, FwdLUT, that maps input luma code values Yi to altered values Yr: Yr=FwdLUT[Yi]. The other LUT is an inverse LUT, InvLUT, that maps altered code values Yr to Yi : Yi=InvLUT[Yr]. (Yi represents the reconstruction values of Yi.)

For intra slices, only the InvLUT is applied.  For inter slices, both FwdLUT and InvLUT are applied. LUTs are applied before loop filtering for both intra and inter slices. Processing operations and data flow are identical for SDR and HDR.

The processing operations for intra slices are illustrated in Figure 1. InvLUT maps intra reconstructed values in the reshaped domain to intra reconstructed values in the original domain. (Yi=InvLUT[Yr])

The processing operations for inter slices are illustrated in Figure 2.  FwdLUT maps motion-compensation values in the original domain to the reshaped domain. (FwdLUT[Ypred]). InvLUT then maps inter reconstructed values in the reshaped domain to inter reconstructed values in the original domain. (Yi=InvLUT[Yres+FwdLUT[Ypred]]).

The reshaper model syntax signals a piece-wise linear (PWL) model with 32 equal pieces. (Note that in CE-12 software, the PWL model is used to precompute the 1024-entry FwdLUT and InvLUT mapping tables; but the PWL model also allows implementations to calculate identical mapping values on-the-fly without pre-computing the LUTs).

Conceptually, PWL is implemented in the following way:

Let x1, x2 be two input pivot points, and y1, y2 be their corresponding output pivot points for one piece. The output value y for any input value x between x1 and x2 can be interpolated by the following equation:

y = ((y2-y1)/(x2-x1)) * (x-x1) + y1

In fixed point implementation, the equation can be rewritten as

y = ((m * x + 2FP_PREC-1) >> FP_PREC) + c

where m is scalar, c is an offset, and FP_PREC is a constant value to specify the precision.

The syntax (Annex A) specifies the number of codewords in each piece explicitly, therefore, the decoder can compute the pivot points directly.

Figure 1. Intra slice reconstruction with in-loop luma reshaper in CE12-1 (light-green shaded blocks indicate signal in reshaped domain: luma residue; intra luma predicted; and intra luma reconstructed)

Figure 2. Inter slice reconstruction with in-loop luma reshaper in CE12-1(light-green shaded blocks indicate signal in reshaped domain: luma residue and intra luma reconstructed)

## In-loop luma reshaping architecture in CE12-2

It was pointed out in the JVET-Lth meeting that the CE12-1 reshaping architecture can introduce extra latency in hardware decoding pipeline for block-wise intra prediction in inter slice reconstruction. That is, for intra TUs in inter slices, intra prediction cannot start until the InvLUT of the previous TU is completed. This would increase latency in decoding pipeline and would be in the critical path of decode loop. In CE12-2, an alternative arrangement of the inverse reshape module for inter slice reconstruction is tested to eliminate such latency. The alternative arrangement is shown in Figure 3. The inverse reshape module is moved out of the critical intra prediction loop in inter slice decoding. Intra prediction is always performed in reshaped domain regardless of slice type. With such arrangement, intra prediction can start immediately after previous TU reconstruction is done. Such arrangement can also provide a unified process for intra mode instead of being slice dependent. Figure 4 shows the block diagram of the CE12-2 decoding process based on mode.

Figure 3. Inter slice reconstruction with in-loop luma reshaper in CE12-2 (light-green shaded blocks indicate signal in reshaped domain: luma residue; intra luma predicted; and intra luma reconstructed)

The major difference of operational domains between CE12-2 and CE12-1 is the intra prediction in inter slices. In addition, in CE12-2, the FwdLUT of inter predicted signal can be subsumed as part of motion compensation. Thus, the reconstruction pipeline is only affected by the InvLUT of the reconstructed sample, which only needs to happen any time before loop filtering, and hence is not in the critical path. Table 1 lists the operational domains in CE12-1 and CE12-2 and explains how combined merge and intra prediction (a.k.a. multi-hypothesis prediction) is performed.

Table 1 explanation of operational domains in CE12-1 and CE12-2

|  | CE12-1 | CE12-2 |
| --- | --- | --- |
| Intra Prediction (IP) in intra slice | Reshaped domain | Reshaped domain |
| Intra Prediction (IP) in inter slice | Original domain | Reshaped domain |
| Inter Prediction (MC) | Original domain | Original domain |
| Loop Filtering | Original domain | Original domain |
| Combined merge and intra prediction | IP and MC both in original domain. FwdLUT applied on combined predicted signal | IP in reshaped domain, MC in original domain. Combined predicted signal obtained from IP and FwdLUT applied MC |

Figure 4. intra mode and inter mode reconstruction with in-loop luma reshaper in CE12-2 (light-green shaded blocks indicate signal in reshaped domain: luma residue; intra luma predicted; and intra luma reconstructed)

It was also commented in the JVET-Lth meeting that the reshaper model with 32 pieces is relatively large for friendly software and hardware implementation. In CE12-2, the PWL model is simplified to half of the original size: 16 equal pieces. The simplification is made at the encoder side by combining (averaging) two adjacent pieces in 32-piece into one piece in 16-piece. For example, the 0-th and 1-th pieces in 32-piece are combined and becomes the 0-th piece in 16-piece. The 2-th and 3-th pieces in 32-piece are combined and becomes the 1-th piece in 16-piece.  The reshaper model syntax then signals a piece-wise linear model with 16 pieces. The 1024-entry mapping tables are constructed by linear interpolation using the 16-piece PWL model; or, alternatively, identical mapping values may be calculated on-the-fly without pre-computing LUTs.

## Chroma residue scaling

Chroma residue scaling compensates for luma signal interaction with the chroma signal. Chroma residue scaling is applied at the TU level.

Encoder side: CResScale=CRes*CScale=CRes/CScaleInv

Decoder side: CRes=CResScale/CScale=CResScale*CScaleInv

CRes is the original chroma residual signal and CResScale is the scaled chroma residual signal. CScale is a scaling factor calculated using FwdLUT as described in L0246 and is converted to its reciprocal CScaleInv to perform multiplication instead of division at the decoder side, thereby reducing implementation complexity. Because PWL model is used, for any input luma value in each luma PWL range, the same chroma scale value is used. Therefore, the luma dependent chroma scales are simplified to be calculated for each luma PWL range instead of each luma code value, and is stored in LUT cScaleInv[pieceIdx] (e.g. 32-piece granularity in CE12-1 or 16-piece granularity in CE12-2) instead of the 1024-entry mapping LUT cScale[Y] in L0246 [5]. The scaling operations at both encoder and decoder side are implemented with fixed point integer arithmetic with the following equation:

c’ = sign(c) * ((abs(c) * s + 2CSCALE_FP_PREC-1) >> CSCALE_FP_PREC)

where c is chroma residue, s is chroma residue scaling factor from cScaleInv[pieceIdx], pieceIdx is decided by the corresponding average luma value of the TU, and CSCALE_FP_PREC is a constant value to specify precision.

The following suggestions for easing implementation related to chroma residue scaling were made during the meeting (see section 3.3.1 for results):

- Disabling the chroma residual scaling for separate tree operation

- Disabling the chroma residual scaling for 2x2 chroma

- Using the prediction signal rather than the reconstruction signal for intra as well as inter

The value of CScaleInv is computed in the follow steps:

- If intra mode, compute average of intra predicted luma values; if inter mode, compute average of forward reshaped inter predicted luma values. That is, the average luma value avgY'TU is computed in reshaped domain.

- Find index  idx where avgY'TU belongs to in inverse mapping PWL.

- CScaleInv  = cScaleInv[idx]

Figure 4. intra mode and inter mode reconstruction with chroma residue scaling in CE12-2 (light-green shaded blocks indicate signal in reshaped domain: luma residue; intra luma predicted; and intra luma reconstructed)

## Unification of SDR and HDR syntax and decoding process

In L0247, HDR and SDR use same reconstruction process with FwdLUT and InvLUT mapping once the LUTs are constructed, but the syntax and parsing process are still slightly different: SDR uses PWL model for signaling and HDR used dQP model for signaling. In CE12-1 and CE12-2, the signaling of SDR and HDR has been unified to be the same PWL model (32-piece for CE12-1 and 16-piece for CE12-2). The piece-wise model is achieved by approximating the previous static HDR reshaping function constructed using dQP model in L0245/L0247. The HDR LUT construction process and chroma residue scale derivation process have also been made completely the same as the SDR process. Though HDR testing is out of the CE12 scope, additional HDR test results are provided in section 3.3 to validate the generalized performance of the reshaping tool.

## Encoder optimization

### Rate adaptive luma reshaping

In L0246, the SDR luma reshaping curve is content adaptive, but not rate adaptive. All QP points use the same reshaping curve for a given content. It was observed that for some content, reshaped signal increases the rate for low QP (e.g. 22) significantly and therefore was suggested to investigate rate-adaptive encoder optimization.

In CE12-1 and CE12-2, the encoder algorithm has been improved to support rate adaptive reshaping. At low QP (QP <=22), the encoder algorithm can select from the following possible scenarios: 1) use same reshaping function as high QPs; 2) use an alternative reshaping function designed for low QP, which performs moderate reshaping (close to identity) and therefore would not cause significant rate increase; 3) identity reshaping. The decision is automatically made by encoder based on statistics and the same decision applies for CE12-1 and CE12-2. This improvement resolves the RD curve crossing observed in non-rate adaptive case in L0246.

### Mode decision

The same mode decision rule as L0246 still applies for CE12-1 and CE12-2. If luma reshaper is applied to the current CU for an inter slice, weighted SSE is used for luma in mode decision. For all luma values in each piece, the same weight is used. The weight is derived from codeword assignment in that piece as: w_rsp(k) = (cw_piece(k)/(totCW/#pieces))^2, w_rsp(k) is weight for k-th piece, cw_piece(k) is the number of codewords for k-th piece, totCW the is total codewords (for 10bit, totCW=1024). For intra slice, SSE is used for luma in mode decision. Table 2 summarizes the RD cost methods used for in-loop reshaping.

Table 2 summary of the RD cost methods used for in-loop reshaping

| RD cost of luma in intra slice | Uses SSE in reshaped domain |
| --- | --- |
| RD cost of luma in inter slice | Uses weighted SSE in original domain |
| RD cost of chroma | Uses SSE in original domain |

### Slice adaptation

In-loop reshaping allows slice level adaptation. In L0246, slice adaptation only adapts to slice type (Intra or Inter). In CE12-1 and CE12-2, finer adaptation is used with respect to temporal layer. The encoder automatically decides from picture analysis statistics if reshaping should be enabled or disabled for a slice based on its type and temporal layer id. The same decision applies for both CE12-1 and CE12-2.

## Summary of differences between CE12-1, CE12-2 and L0246

Table 3 overview of the differences between CE12-1, CE12-2, and L0246

|  |  | L0246 | CE12-1 | CE12-2 |
| --- | --- | --- | --- | --- |
| Luma reshaping (normative) | FwdLUT onstruction | Integral of slopes | PWL interpolation with 32 pieces | PWL interpolation with 16 pieces |
|  | InvLUT construction | Reverse LUT by value/index search in FwdLUT | PWL interpolation with 32 pieces | PWL interpolation with 16 pieces |
|  | Syntax (see Annex A) | SDR: PWL model with 32 pieces HDR: dQP model | SDR and HDR: unified PWL model with 32 pieces | SDR and HDR: unified PWL model with 16 pieces |
|  | Decoding pipeline implication | need FwdLUT and InvLUT inside intra prediction loop in inter-slice, extra latency | need FwdLUT and InvLUT inside intra prediction loop in inter-slice, extra latency | no need for FwdLUT and InvLUT inside intra prediction loop for inter slice, unify intra mode in intra/inter slice, low latency. InvLUT not in critical path of inter prediction |
| Chroma residue scaling (normative) | Chroma residue scaling operation | Decoder side use division | Decoder side use multiplication | Decoder side use multiplication |
|  | Chroma residue scaling LUT | 1024-entry, index by TU average luma | 32-entry, index by PWL piece idx that TU avg. luma belongs to | 16-entry, index by PWL piece idx that TU avg. luma belongs to |

Table 3 provides an overview of the major differences between CE12-1, CE12-2, and L0246 for improvements and simplifications made for in-loop reshaping.

# Simulation results and analysis

The tests are performed under CTC with AI, RA and LDB test condition. AVX2 is enabled automatically by the grid engine servers for SIMD optimizations provided in the VTM3.0. The encoding and decoding time is collected from log files. Parallel overlapped encoding is used for RA tests. Decoding time is measured without YUV output.

## CE12-1 results

Table 4 to Table 6 show BD rate results of CE12-1 for AI, RA and LDB respectively. The average BD rate gain for Y/U/V is -0.89%, 1.52%, 1.15% for SDR video under AI test condition. The average BD rate gain for Y/U/V is -1.28%, 0.53%, 0.52% for SDR video under RA test condition. The average BD rate gain for Y/U/V is -0.72%, 0.46%, 0.11% for SDR video under LDB test condition.  The average encoding time is 102% and average decoding time is 101%.

Greater gain is observed for higher resolutions. For HD & UHD clips (class A1/A2/B/ArenaOfValor): Y/U/V BDRate is:

  - AI:    	-1.18%	1.04%	0.48%

  - RA:   	-1.41%	0.34%	0.15%

  - LDB: 	-1.11%	1.26%	1.25%

Table 4 CE12-1 results:  AI

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.70% | -0.05% | -1.66% | 99% | 98% |
| Class A2 | -1.69% | 1.10% | 1.12% | 102% | 99% |
| Class B | -0.73% | 1.26% | 0.86% | 101% | 99% |
| Class C | -0.38% | 2.65% | 2.73% | 106% | 102% |
| Class E | -0.21% | 2.41% | 2.36% | 109% | 102% |
| Overall | -0.89% | 1.52% | 1.15% | 103% | 100% |
| Class D | -0.45% | 1.84% | 1.70% | 110% | 101% |
| Class F | -0.81% | 1.82% | 1.85% | 107% | 102% |

Table 5 CE12-1 results:  RA

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.19% | 1.31% | 1.08% | 105% | 101% |
| Class A2 | -1.12% | 1.93% | 2.06% | 104% | 101% |
| Class B | -1.47% | -0.45% | -0.89% | 104% | 101% |
| Class C | -1.22% | 0.10% | 0.71% | 105% | 101% |
| Overall | -1.28% | 0.53% | 0.52% | 105% | 101% |
| Class D | -0.56% | 1.63% | 1.94% | 109% | 102% |
| Class F | -1.42% | -0.41% | -0.31% | 104% | 100% |

Table 6 CE12-1 results:  LDB

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -1.22% | 1.07% | 1.04% | 98% | 100% |
| Class C | -0.42% | 1.02% | 1.07% | 100% | 101% |
| Class E | -0.28% | -1.31% | -2.71% | 102% | 101% |
| Overall | -0.72% | 0.46% | 0.11% | 100% | 101% |
| Class D | -0.65% | 1.74% | 1.75% | 102% | 101% |
| Class F | -0.73% | 1.05% | 1.09% | 101% | 100% |

## CE12-2 results

Table 7 to Table 9 show BD Rate results of CE12-2 for AI, RA and LDB respectively. The average BD rate gain for Y/U/V is -0.89%, 1.49%, 1.18% for SDR video under AI test condition. The average BD rate gain for Y/U/V is -1.30%, 1.45%, 1.44% for SDR video under RA test condition. The average BD rate gain for Y/U/V is -0.76%, 1.78%, 1.42% for SDR video under LDB test condition.  The average encoding time is 102% and average decoding time is 103%. Greater gain is observed for higher resolutions. For HD & UHD clips (class A1/A2/B/ArenaOfValor): Y/U/V BDRate is:

  - AI:    	-1.19%	1.13%	0.64%

  - RA:   	-1.43%	1.22%	1.11%

  - LDB: 	-1.14%	2.37%	2.23%

Table 7 CE12-2 results:  AI

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.70% | 0.17% | -1.29% | 101% | 100% |
| Class A2 | -1.72% | 1.26% | 1.36% | 96% | 100% |
| Class B | -0.74% | 1.25% | 0.88% | 105% | 101% |
| Class C | -0.38% | 2.41% | 2.52% | 104% | 104% |
| Class E | -0.20% | 2.20% | 2.17% | 103% | 103% |
| Overall | -0.89% | 1.49% | 1.18% | 102% | 101% |
| Class D | -0.42% | 1.65% | 1.48% | 106% | 103% |
| Class F | -0.82% | 1.75% | 1.74% | 103% | 104% |

Table 8 CE12-2 results:  RA

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.94% | 2.81% | 2.74% | 106% | 103% |
| Class A2 | -1.27% | 2.56% | 2.80% | 102% | 105% |
| Class B | -1.62% | 0.26% | -0.14% | 105% | 103% |
| Class C | -1.18% | 1.10% | 1.41% | 106% | 104% |
| Overall | -1.30% | 1.45% | 1.44% | 105% | 104% |
| Class D | -1.09% | 2.05% | 2.20% | 110% | 106% |
| Class F | -1.42% | -0.14% | -0.05% | 104% | 104% |

Table 9 CE12-2 results:  LDB

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -1.27% | 2.25% | 2.12% | 96% | 103% |
| Class C | -0.56% | 2.17% | 2.20% | 93% | 104% |
| Class E | -0.19% | 0.48% | -0.81% | 111% | 108% |
| Overall | -0.76% | 1.78% | 1.42% | 98% | 105% |
| Class D | -1.05% | 2.40% | 2.93% | 105% | 104% |
| Class F | -0.75% | 1.48% | 1.44% | 105% | 106% |

Comparing CE-12-1 and low-complexity CE12-2, the performance is effectively the same for luma for all test conditions: AI, RA, and LDB.  The chroma performance of CE12-1 and CE12-2 are also the same for AI.  For RA and LDB, CE12-2 chroma performance is less than that of CE12-1 by less than 1% for RA, and 1.3% for LDB. Chroma loss in CE12-2 is mainly due to 32-piece to 16-piece simplification and some loss is due to intra prediction in reshaped domain in inter slice.

The data indicate that CE12-2 provides significant performance gains essentially the same as those provided by CE12-1, but with less complexity.

## CE12-2 additional results

### Implementation-friendly chroma residue scaling configurations

The suggestions for easing implementation of chroma residue scaling in CE12-2 reshaper (described in section 2.3) were tested with the CTC anchor (CE12-2.1) and a single-tree modification of the CTC anchor (CE12-2.2), as summarized in Table 11.  For CE12-2.1, dual-tree is enabled.  For CE12-2.2, dual-tree is diabled to make a fair comparison.  CE12-2.1 is configured with chroma residual scaing disabled for intra slices and 2x2 chroma blocks in inter slices (Table 12 and Table 13 for AI and RA, respectively).  CE12-2.2 is configured with chroma residual scaling disabled for 2x2 chroma blocks (Table 14 and Table 15 for AI and RA, respectively).  For both CE12-2.1 and CE12-2.2, the chroma scaling factor is calculated from forward-mapped predicted luma values rather than resonctructed luma values.

- For CE12-2.1, average BDRate for Y/U/V:

  - RA    { -1.35%, 2.49%, 2.40% }

  - AI     { -0.87%, 1.30%, 1.17% }

- For CE12-2.2, average BDRate for Y/U/V:

  - RA    { -1.21%, 1.11%, 1.10% }

  - AI     { -0.90%, 1.45%, 1.15% }

Table 11 configurations for testing easing of implementation for chroma residual scaling in CE12-2

| Reference |  | Test |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | dual-tree | dual-tree | chroma residue scaling | chroma 2x2 | avgY |
| CE12-2.1 | enabled | enabled | disabled: Intra slice | disabled | predicted luma |
|  |  |  | enabled: Inter slice |  |  |
| CE12-2.2 | disabled | disabled | enabled | disabled | predicted luma |

Table 12 CE12-2.1: AI

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.45% | -1.26% | -2.09% | 101% | 101% |
| Class A2 | -1.41% | -1.08% | -0.98% | 103% | 101% |
| Class B | -0.70% | 0.31% | 0.21% | 102% | 100% |
| Class C | -0.56% | 3.80% | 3.87% | 103% | 100% |
| Class E | -0.43% | 4.57% | 4.60% | 101% | 101% |
| Overall | -0.87% | 1.30% | 1.17% | 102% | 101% |
| Class D | -0.55% | 2.36% | 2.41% | 102% | 101% |
| Class F | -1.08% | 3.32% | 3.33% | 103% | 101% |

Table 13 CE12-2.1: RA

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.93% | 2.78% | 2.70% | 105% | 104% |
| Class A2 | -1.27% | 2.59% | 2.74% | 101% | 104% |
| Class B | -1.65% | 1.12% | 0.65% | 103% | 103% |
| Class C | -1.37% | 3.93% | 4.10% | 102% | 104% |
| Overall | -1.35% | 2.49% | 2.40% | 103% | 103% |
| Class D | -1.20% | 3.52% | 3.61% | 102% | 104% |
| Class F | -1.66% | 2.56% | 2.47% | 102% | 102% |

Table 14 CE12-2.2: AI

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 DualTree=0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.62% | 0.05% | -1.28% | #DIV/0! | #DIV/0! |
| Class A2 | -1.72% | 1.01% | 1.32% | #DIV/0! | #DIV/0! |
| Class B | -0.74% | 1.34% | 0.76% | #DIV/0! | #DIV/0! |
| Class C | -0.44% | 2.77% | 2.75% | #DIV/0! | #DIV/0! |
| Class E | -0.26% | 1.74% | 1.93% | #DIV/0! | #DIV/0! |
| Overall | -0.90% | 1.45% | 1.15% | #DIV/0! | #DIV/0! |
| Class D | -0.44% | 1.41% | 1.42% | #DIV/0! | #DIV/0! |
| Class F | -0.95% | 2.13% | 2.15% | #DIV/0! | #DIV/0! |

Table 15 CE12-2.2: RA

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 DualTree=0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.55% | 1.22% | 1.01% | #DIV/0! | #DIV/0! |
| Class A2 | -1.19% | 2.07% | 2.18% | #DIV/0! | #DIV/0! |
| Class B | -1.59% | 0.10% | -0.07% | #DIV/0! | #DIV/0! |
| Class C | -1.25% | 1.59% | 1.83% | #DIV/0! | #DIV/0! |
| Overall | -1.21% | 1.11% | 1.10% | #DIV/0! | #DIV/0! |
| Class D | -1.06% | 2.54% | 2.64% | #DIV/0! | #DIV/0! |
| Class F | -1.54% | 0.65% | 0.69% | #DIV/0! | #DIV/0! |

### HDR content

Additional test results for HDR-PQ content using CE12-2 in-loop reshaper are presented in Table 16. The BD rate gain for wPSNRY/PSNRL100/DE100 is -1.6%, -2.0%, 0.9% for AI and -1.6%, -1.8%, 1.9% for RA. For HDR, CE12-2 employs the 16-piece PWL model to signal the static reshaping function designed for HDR. Thus, the parsing and decoding process for HDR and SDR are the same.

The SDR results (Tables 7-9) taken together with the HDR results (Table 16) indicate that the complexity reduction in CE12-2 preserves good coding efficiency for both SDR and HDR.

Table 16 CE12-2 results:  HDR, AI and RA

|  | All Intra |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | wPsnrY | PSNRL100 | DE100 | wPsnrY | PSNRL100 | DE100 |
| Average HDR-B | -1.6% | -2.0% | 0.9% | -1.6% | -1.8% | 1.9% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 102% |  |  | 100% |  |  |

### Rate allocation analysis

In L-th JVET meeting, suggestions were made on studying the intra inter rate allocation. Rate allocation analysis has been performed with CE12-1 and CE12-2 coding results with VTM3.0 anchors as the reference. The bits used to code intra and inter slices are collected respectively and the percentage over the overall bits are calculated for each rate point. Table 17 shows a summary of intra percentage (inter percentage is 100-intra perc.) of each test class for random access tests. The rate allocation analysis indicates that the intra and inter rate allocation between the anchor and CE12 results are essentially the same.

Table 17 rate allocation analysis in RA: intra percentage of each class

|  | Anchor | CE12-1 | CE12-2 |
| --- | --- | --- | --- |
| Class A1 | 29% | 29% | 28% |
| Class A2 | 37% | 36% | 36% |
| Class B | 36% | 37% | 36% |
| Class C | 39% | 40% | 40% |
| Class D | 40% | 40% | 40% |
| Class F | 59% | 60% | 60% |
| All | 36% | 36% | 35% |

### Complexity analysis

The CE software is implemented in VTM3.0. The FwdLUT and InvLUT are constructed using piece-wise linear interpolation. All related operations, such as mapping and LUT construction, are performed with fixed point arithmetic at the decoder side. The proposed reshaper performs luma sample mapping in-place, which means that no extra storage buffer is needed for luma samples.  The in-loop reshaper does not employ any additional SIMD optimization except for those already in the VTM3.0 codebase. For chroma residue scaling, the operations are performed on residue signal, which means that if cbf=0 (no coefficient), there is no need to perform chroma scaling in the decoding process. This can potentially save many operations for low bit rate video decoding.

The signalling of a PWL model enables reshaping to be implemented in different but mathematically identical ways.  Two such implementation options are described here. In the CE12 software, the PWL model is used to precompute FwdLUT and InvLUT.  An alternative method is to calculate mapping values on-the-fly without precomputing a LUT. Table 18 shows the difference between these two implementations in terms of memory requirements. The column labeled Option 1 corresponds the case for precomputing LUTs from the PWL model. The column labeled Option 2 corresponds to the case for calculating values on-the-fly from the PWL model.

Table 18 estimated memory requirement in CE12-1 (32 piece) and CE12-2(16 piece)

|  | for cScaleInvLUT (chroma residue scaling) | for FwdLUT and InvLUT (Option1: using LUT mapping) | for Fwd and Inv mapping using PWL interpolation (Option 2) |
| --- | --- | --- | --- |
| CE12-1 | 32 * 2byte | 2 * 1024 * 10bit | 4 * 32 * 2byte |
| CE12-2 | 16 * 2byte | 2 * 1024 * 10bit | 4 * 16 * 2byte |

Worst-case hardware operations per luma sample: assume PWL operation as y = ((m * x + 2FP_PREC-1) >> FP_PREC) + c is 1 multiply, 2 adds and 1 shift.

Worst-case hardware operations per chroma sample: assume scaling operation as c’ = sign(c) * ((abs(c) * s + 2CSCALE_FP_PREC-1) >> CSCALE_FP_PREC) is 2 multiply, 1 add, 1 shift.

The hardware dependency of chroma residue scaling is the same as that for CCLM for intra mode since both use the reconstructed luma signal. Chroma residue scaling does not incur extra latency for CCLM since CCLM operates on the chroma prediction signal and chroma residue scaling operates on the chroma residue signal. For inter mode, chroma residue scaling uses predicted luma signal and therefore will have less latency than CCLM.

# Conclusion

The CE12-1 reshaper provides significant performance gain over the anchors. The CE12-2 reshaper provides significant performance gain and implementation friendliness for both SDR and HDR.

It is recommended that the CE12-2 reshaper be adopted as a core coding tool in VVC.

# Acknowledgement

We would like to thank Technicolor and ByteDance for crosschecking the CE test results.

# Reference

- X. Xiu, P. T. Lu, Hanhart, R. Vanam, Y. He, Y. Ye, T. Lu, F. Pu, P. Yin, W. Husak, T. Chen, “Description of SDR, HDR, and 360° video coding technology proposal by InterDigital Communications and Dolby Laboratories”, JVET-J0015, San Diego, USA, April 2018.

- T. Lu, F. Pu, P. Yin, W. Husak, S. McCarthy, T. Chen, “CE12: HDR In-loop Reshaping (CE12-5, 12-6, 12-7 and 12-8)”, JVET-K0308, Ljubljana, SI, July, 2018.

- F. Pu, T. Lu, P. Yin, W. Husak, S. McCarthy, T. Chen, “SDR In-loop Reshaping”, JVET-K0309, Ljubljana, SI, July, 2018.

- T. Lu, F. Pu, P. Yin, W. Husak, S. McCarthy, T. Chen, “CE12-2: HDR In-loop Reshaping”, JVET-L0245, Macao, CN, October 2018.

- T. Lu, F. Pu, P. Yin, W. Husak, S. McCarthy, T. Chen, “CE12-4: SDR In-loop Reshaping”, JVET-L0246, Macao, CN, October 2018.

- E. Francois, P. Yin, “Description of Core Experiment 12 (CE12): mapping functions”, JVET-L1032, Macao, CN, 3–12 Oct. 2018.

# Patent rights declaration(s)

Dolby Laboratories, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex A: Syntax and Semantics

See attached document delta_syntax for syntax, semantics and decoding processing. For easier reading, the syntax and semantic is also described here.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_reshaper_enabled_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

| tile_group_header( ) { | Descriptor |
| --- | --- |
| … | . |
| if( sps_reshaper_enabled_flag) { |  |
| tile_group_reshaper_model_present_flag | u(1) |
| if(tile_group_reshaper_model_present_flag ) |  |
| tile_group_reshaper_model ( ) |  |
| tile_group_reshaper_enable_flag | u(1) |
| if( tile_group_reshaper_enable_flag && (!( qtbtt_dual_tree_intra_flag && tile_group_type == I ) ) ) |  |
| tile_group_reshaper_chroma_residual_scale_flag | u(1) |
| } |  |
| byte_alignment( ) |  |
| } |  |

| tile_group_reshaper_model () { | Descriptor |
| --- | --- |
| reshaper_model_min_bin_idx | ue(v) |
| reshaper_model_delta_max_bin_idx | ue(v) |
| reshaper_model_bin_delta_abs_cw_prec_minus1 | ue(v) |
| for ( i = reshaper_model_min_bin_idx; i <= reshaper_model_max_bin_idx; i++) { |  |
| reshape_model_bin_delta_abs_CW [ i ] | u(v) |
| if( reshaper_model_bin_delta_abs_CW[ i ] ) > 0 ) |  |
| reshaper_model_bin_delta_sign_CW_flag[ i ] | u(1) |
| } |  |
| } |  |

In General sequence parameter set RBSP semantics, add the following semantics:

sps_reshaper_enabled_flag equal to 1 specifies that reshaper is used in the coded video sequence (CVS). sps_reshaper_enabled_flag equal to 0 specifies that reshaper is not used in the CVS.

-

In tile group header syntax, add the following semantics

tile_group_reshaper_model_present_flag equal to 1 specifies tile_group_reshaper_model() is present in tile group header. tile_group_reshaper_model_present_flag equal to 0 specifies tile_group_reshaper_model() is not present in tile group header. When tile_group_reshaper_model_present_flag is not present, it is inferred to be equal to 0.

tile_group_reshaper_enabled_flag equal to 1 specifies that reshaper is enabled for the current tile group. tile_group_reshaper_enabled_flag equal to 0 specifies that reshaper is not enabled for the current tile group. When tile_group_reshaper_enable_flag is not present, it is inferred to be equal to 0.

tile_group_reshaper_chroma_residual_scale_flag equal to 1 specifies that chroma residual scaling is enabled for the current tile_group. tile_group_reshaper_chroma_residual_scale_flag equal to 0 specifies that chroma residual scaling is not enabled for the current tile group. When tile_group_reshaper_chroma_residual_scale_flag is not present, it is inferred to be equal to 0.

Add tile_group_reshaper_model( ) syntax

reshape_model_min_bin_idx specifies the minimum bin(or piece) index to be used in the reshaper construction process. The value of reshape_model_min_bin_idx shall be in the range of 0 to MaxBinIdx, inclusive. The value of MaxBinIdx shall be equal to 15.

reshape_model_delta_max_bin_idx specifies the maximum allowed bin (or piece) index MaxBinIdx minus the maximum bin index to be used in the reshaper construction process. The value of reshape_model_max_bin_idx is set equal to MaxBinIdx – reshape_model_delta_max_bin_idx.

reshaper_model_bin_delta_abs_cw_prec_minus1 plus 1 specifies the number of bits used for the representation of the syntax reshape_model_bin_delta_abs_CW[ i ].

reshape_model_bin_delta_abs_CW[ i ] specifies the absolute delta codeword value for the ith bin.

reshaper_model_bin_delta_sign_CW_flag[ i ] specifies the sign of reshape_model_bin_delta_abs_CW[ i ] as follows:

–	If reshape_model_bin_delta_sign_CW_flag[ i ] is equal to 0, the corresponding variable RspDeltaCW[ i ] is a positive value.

–	Otherwise ( reshape_model_bin_delta_sign_CW_flag[ i ] is not equal to 0 ), the corresponding variable RspDeltaCW[ i ] is a negative value.

When reshape_model_bin_delta_sign_CW_flag[ i ] is not present, it is inferred to be equal to 0.

The variable RspDeltaCW[ i ] = (1  2*reshape_model_bin_delta_sign_CW [ i ]) * reshape_model_bin_delta_abs_CW [ i ];

The variable RspCW[ i ] is derived as following steps:

The variable OrgCW is set equal to (1 << BitDepthY ) / ( MaxBinIdx + 1).

–	If reshaper_model_min_bin_idx <= i <= reshaper_model_max_bin_idx

RspCW[ i ] = OrgCW + RspDeltaCW[ i ].

–	Otherwise, RspCW[ i ] = 0.

The value of RspCW [ i ] shall be in the range of 32 to 2 * OrgCW - 1 if the value of BitDepthY is equal to 10.

The variables InputPivot[ i ] with i in the range of 0 to MaxBinIdx + 1, inclusive are derived as follows

InputPivot[ i ] = i * OrgCW

The variable ReshapePivot[ i ] with i in the range of 0 to MaxBinIdx + 1, inclusive, the variable ScaleCoef[ i ] and InvScaleCoeff[ i ]with i in the range of 0 to MaxBinIdx , inclusive, are derived as follows:

shiftY = 14
ReshapePivot[ 0 ] = 0;

for( i = 0; i <= MaxBinIdx ; i++) {
	ReshapePivot[ i + 1 ] = ReshapePivot[ i ] + RspCW[ i ]

ScaleCoef[ i ] = ( RspCW[ i ] * (1 << shiftY ) + (1 << (Log2(OrgCW) - 1))) >> (Log2(OrgCW));
	if ( RspCW[ i ] == 0 )
		InvScaleCoeff[ i ] = 0; 
	else
		InvScaleCoeff[ i ] = OrgCW * ( 1 << shiftY ) / RspCW[ i ]
}

The variable ChromaScaleCoef[ i ] with i in the range of 0 to MaxBinIdx , inclusive, are derived as follows:

ChromaResidualScaleLut[64] = {16384, 16384, 16384, 16384, 16384, 16384, 16384, 8192, 8192, 8192, 8192, 5461, 5461, 5461, 5461, 4096, 4096, 4096, 4096, 3277, 3277, 3277, 3277, 2731, 2731, 2731, 2731, 2341, 2341, 2341, 2048, 2048, 2048, 1820, 1820, 1820, 1638, 1638, 1638, 1638, 1489, 1489, 1489, 1489, 1365, 1365, 1365, 1365, 1260, 1260, 1260, 1260, 1170, 1170, 1170, 1170, 1092, 1092, 1092, 1092, 1024, 1024, 1024, 1024};

shiftC = 11

–	if ( RspCW[ i ] == 0 )

ChromaScaleCoef [ i ] = (1 << shiftC)

–	 Otherwise (RspCW[ i ] != 0),

ChromaScaleCoef[ i ] = ChromaResidualScaleLut [RspCW[ i ]>>1]