| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0437 |
| --- | --- |

| Title: | Non-CE4: Size constraint on MMVD |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Zhao Seung Hwan Kim 10225 Willow Creek Rd, San Diego, CA 92131, U.S.A | Tel: Email: | 1-201-816-2000 jie.zhao@lge.com |
| Source: | LG Electronics. |  |  |

_____________________________

# Abstract

In this document, it is proposed to disable MMVD for 4x4 blocks to reduce the MMVD complexity. Experimental result show that the coding impact is 0.01%, 0.03%, 0.01% for Y, U, V, components respectively in RA case, and 0.02%,0.08%,0.08% for Y, U,V, component respectively in LD case.

# Introduction

In Oct. 2018 JVET meeting, MMVD [1] was adopted into VTM3. MMVD is used for either skip or merge modes with a proposed motion vector expression method. MMVD re-uses merge candidate as same as using in VVC. Among the merge candidates, a candidate can be selected, and is further expanded by the proposed motion vector expression method.

MMVD provides a new motion vector expression with simplified signaling. The expression method includes starting point, motion magnitude, and motion direction. Up to 2 candidates in the merge list can be used as MMVD’s starting point. There are four directions from starting points and is represented by direction index. Motion magnitude information is indicated by distance index, which indicates the pre-defined distance from the starting point information. Pre-defined distance range from 1/4 pel to 32-pel.

In VTM3, MMVD can be applied to all block sizes.

# Proposed

For a CB that uses MMVD, it needs to signal a MMVD flag, starting point, motion magnitude, and motion direction. For small block size, the gain may not justify the signalling overhead. It also increases encoder complexities by checking all the MMVD candidates. Therefore we propose to enable MMVD only if block size is larger than 4x4.

# Experimental Results

Proposed changes are implemented on top of VTM3 software. Tests are done following the CTC [2]. Experiments result showed that there is little RD performance impact by disabling MMVD for 4x4 blocks.

Table 4.2 Result of disable MMVD for 4x4 block

|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.02% | 0.03% | 100% | 100% |  |  |  |  |  |
| Class A2 | -0.01% | -0.01% | 0.08% | 100% | 99% |  |  |  |  |  |
| Class B | 0.00% | 0.04% | -0.02% | 100% | 100% | 0.02% | -0.15% | -0.01% | 99% | 98% |
| Class C | 0.04% | 0.08% | -0.01% | 101% | 102% | -0.04% | 0.17% | 0.28% | 99% | 98% |
| Class E |  |  |  |  |  | 0.10% | 0.32% | -0.03% | 100% | 97% |
| Overall | 0.01% | 0.03% | 0.01% | 100% | 100% | 0.02% | 0.08% | 0.08% | 99% | 98% |
| Class D | -0.03% | -0.06% | 0.02% | 101% | 102% | -0.01% | 0.40% | -0.11% | 100% | 98% |
| Class F | -0.05% | 0.00% | 0.02% | 101% | 102% | -0.05% | -0.01% | -0.22% | 97% | 98% |

# Conclusion

This document proposes to disable MMVD for block size 4x4 blocks to reduce MMVD complexity. Experiments show that there is very little loss. It is proposed to adopt this size constraint into VVC draft and VTM.

.

# Reference

- S. Jeong, M. Park, Y. Piao, M. Parl, K. Choi, “CE4 Ultimate motion vector expression (Test 4.5.4),” JVET-L0054, Macao, CN, Oct 2018.

- H. Yang, S. Liu, K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding,” JVT-L1024, Macao, CN, Oct 2018.

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team (JVET), JVET-L1010 Macao, CN, Oct 2018.

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).