| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0468 |
| --- | --- |

| Title: | Non-CE: Hadamard transform domain filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sergey Ikonin Victor Stepin Jianle Chen | Tel: Email: | +7-495-660-44-59 sergey.ikonin@huawei.com stepin.victor@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co., LTD |  |  |

_____________________________

# Abstract

This contribution reports simulation results of post-reconstruction filter based on Hadamard transform similar to one tested in JVET-K-CE14.3b. Depending in filter appliance condition the simulations results over VTM-3.0 are:

- For minimum block size 4x8 and 8x4: -0.50%/-0.72%/-0.58% of the luma BD-rate change with 106%/103%/102% of encoding time and 105%/101%/101% of decoding time for AI/RA/LDB configurations correspondingly.

- For minimum block size 8x8: -0.37%/-0.64%/-0.50% of the luma BD-rate change with 105%/103%/101% of encoding time and 105%/101%/101% of decoding time for AI/RA/LDB configurations correspondingly.

- For minimum block size 8x8 and filtering of inter blocks only: -0.39%/-0.40% of the luma BD-rate change with 102%/101% of encoding time and 101%/101% of decoding time for RA/LDB configurations correspondingly.

# Technology description

Hadamard transform domain filter is applied to luma reconstructed blocks right after block reconstruction. The filter was previously studied in JVET-K-CE14 (JVET-L0326). The filter parameters are explicitly derived from the coded information and filtering process is as follows.

## Filtering process

For each pixel from reconstructed block pixel processing comprises the following steps:

- Scan for 4 neighboring pixels around processing pixel including current one according to scan pattern

- 4 point Hadamard transform of read pixels

Fi,σ=Ri2Ri2+m*σ2*R(i)

Spectrum filtering based on the following formula:

,wherein (i) is index of spectrum component in Hadamard spectrum, R(i)  is spectrum component of reconstructed pixels corresponding to index, m=4 is normalization constant equal to number of spectrum components, σ is filtering parameter deriving from codec quantization parameter QP using following equation:

σ=2.64*2(0.1269*QP-11)

The first spectrum component corresponding to DC value is bypassed without filtering.

- Inverse 4 point Hadamard transform of filtered spectrum

- After filtering step the filtered pixels are placed to its original positions into accumulation buffer.

- After completing filtering of pixels the accumulated values are normalized by number of processing groups used for each pixel filtering. Due to use of padding of one sample around the block number of processing groups is equal to 4 for each pixel in the block and normalization is performed by right shifting on 2 bits.

Scan

Hadamard transform

Filtering with 16 entries 
look-up table

Inverse Hadamard transform

To accumulation buffer

A – current pixel

BCD – neighboring pixels

The filtering process is schematically presented on Figure 1.

Figure 1 Filtering process

Equivalent filter shape is 3x3 pixels as depicted on Figure 2.

Figure 2 Equivalent filter shape

It can be seen that all pixels in the block can be processed independently in case of maximum parallelism is required. It should also be noted that results of 2x2 groups filtering are reused for spatial collocated samples. Generally each new pixel in the block requires one 2x2 filtering, rest three are reused.

## LUT details

F(i,σ)=Ri   ,  AbsRi≥THRLUTRi,σ, R(i)>0-LUT-Ri,σ, R(i)≤0

It can be noted that filtering implies multiplication of spectrum component R(i) on scaling coefficient which is always less than 1. It can also be observed that at high values of R(i) scaling coefficient is close to 1. Based on these observation spectrum filtering is implemented using lookup table that allows to exclude multiplications and division from filtering operations:

, where: LUTRi,σ=R(i)3R(i)2+m*σ2.

The threshold THR is set to 128, that in case of straightforward implementation reuquires to store 128 (1 << 7) entries of 7-bit values per each QP.

The size of LUT influences on amount of on-chip memory requirements and the cost of a hardware implementation of the filter. To reduce amount of on-chip storage the LUT if calculated only for limited set of QPs starting from QP 20 with constant interval of 8. Totaly five pre-defined LUTs (for five QPs group) are stored. For filtering of current block CU’s QP is rounded to closest one from the table.

For further reduction of LUT size the N lowest bits are dropped (or ignored) during LUT generation. For all tables N is equal to 3 except of table for lower QP group where N is set to 2 and threshold is reduced to 64 (1 << 6). That makes every table for each QP group having 16 entries of 7-bit values with total size of LUT = 5x16x7 bits = 70 bytes.

More detailed description of the filter including hardware aspects analysis and results of RTL simulations can be found in (JVET-L0326). The RTL simulations shows that having multiplication free design the filter can be implemented in HW with 1 or 2 clock latency depending on frequency and manufacturing process.

The current contribution reports the test results of same filter design as was studied in JVET-K-CE14.3. The only difference is filter appliance conditions. For intra blocks the filter is applied regardless of Cbf value.

# Experimental results

According to recommendation of BoG [3] the following tests were carried out:

- Test 1: minimum block size of filter applied is 4x8 and 8x4

- Test 2: minimum block size of filter applied is 8x8

- Test 3: minimum block size of filter applied is 8x8 and filter is applied to inter blocks only

Table 1 Tests 1: minimum block size is 4x8 and 8x4

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.24% | 0.17% | 0.23% | 106% | 104% |
| Class A2 | -0.55% | 0.15% | 0.14% | 105% | 103% |
| Class B | -0.42% | 0.28% | 0.25% | 106% | 104% |
| Class C | -0.72% | 0.16% | 0.01% | 107% | 106% |
| Class E | -0.53% | 0.11% | 0.12% | 106% | 106% |
| Overall | -0.50% | 0.19% | 0.15% | 106% | 105% |
| Class D | -0.63% | 0.10% | 0.26% | 107% | 106% |
| Class F | -0.79% | -0.01% | 0.08% | 106% | 105% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.41% | -0.08% | -0.20% | 103% | 101% |
| Class A2 | -0.98% | -0.38% | -0.19% | 103% | 101% |
| Class B | -0.75% | -0.01% | -0.12% | 103% | 101% |
| Class C | -0.72% | -0.04% | 0.23% | 103% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.72% | -0.11% | -0.06% | 103% | 101% |
| Class D | -0.71% | -0.07% | 0.24% | 102% | 101% |
| Class F | -0.75% | 0.08% | 0.31% | 102% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.57% | 0.26% | 0.80% | 102% | 100% |
| Class C | -0.71% | 0.71% | 1.10% | 102% | 102% |
| Class E | -0.44% | 1.22% | 0.84% | 100% | 99% |
| Overall | -0.58% | 0.65% | 0.91% | 102% | 101% |
| Class D | -0.86% | 0.69% | 0.66% | 102% | 102% |
| Class F | -0.71% | -0.05% | -0.04% | 101% | 100% |

Table 2 Tests 2: minimum block size is 8x8

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.22% | 0.18% | 0.20% | 105% | 105% |
| Class A2 | -0.48% | 0.17% | 0.14% | 104% | 104% |
| Class B | -0.32% | 0.18% | 0.26% | 104% | 105% |
| Class C | -0.49% | 0.06% | 0.01% | 105% | 104% |
| Class E | -0.33% | 0.19% | 0.28% | 105% | 106% |
| Overall | -0.37% | 0.15% | 0.17% | 105% | 105% |
| Class D | -0.33% | 0.13% | 0.15% | 104% | 104% |
| Class F | -0.44% | 0.02% | -0.09% | 105% | 105% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.40% | -0.27% | -0.15% | 103% | 101% |
| Class A2 | -0.95% | -0.33% | -0.20% | 103% | 101% |
| Class B | -0.68% | 0.05% | -0.11% | 103% | 101% |
| Class C | -0.53% | -0.08% | -0.01% | 102% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.64% | -0.12% | -0.11% | 103% | 101% |
| Class D | -0.48% | -0.43% | -0.27% | 102% | 101% |
| Class F | -0.58% | 0.00% | 0.09% | 102% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.53% | 0.43% | 0.75% | 102% | 101% |
| Class C | -0.58% | 0.77% | 1.08% | 101% | 100% |
| Class E | -0.34% | 1.15% | 0.29% | 100% | 100% |
| Overall | -0.50% | 0.72% | 0.75% | 101% | 101% |
| Class D | -0.70% | 0.44% | 1.33% | 102% | 100% |
| Class F | -0.53% | 0.13% | 0.26% | 101% | 98% |

Table 3 Tests 3: minimum block size is 8x8 and filter is applied to inter blocks only

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.21% | -0.16% | 0.02% | 102% | 100% |
| Class A2 | -0.53% | -0.06% | 0.04% | 102% | 101% |
| Class B | -0.47% | 0.12% | 0.04% | 102% | 101% |
| Class C | -0.31% | 0.22% | 0.18% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.39% | 0.06% | 0.07% | 102% | 101% |
| Class D | -0.33% | 0.18% | 0.29% | 101% | 101% |
| Class F | -0.29% | 0.08% | 0.08% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.43% | 0.81% | 0.93% | 102% | 101% |
| Class C | -0.50% | 0.67% | 0.82% | 101% | 102% |
| Class E | -0.22% | 0.83% | 0.57% | 100% | 100% |
| Overall | -0.40% | 0.77% | 0.80% | 101% | 101% |
| Class D | -0.71% | 0.44% | 0.89% | 101% | 101% |
| Class F | -0.45% | 0.30% | 0.79% | 101% | 99% |

Table 4 Tests summary

|  | AI |  |  |  |  | RA |  |  |  |  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Test1 | -0.50% | 0.19% | 0.15% | 106% | 105% | -0.72% | -0.11% | -0.06% | 103% | 101% | -0.58% | 0.65% | 0.91% | 102% | 101% |
| Test2 | -0.37% | 0.15% | 0.17% | 105% | 105% | -0.64% | -0.12% | -0.11% | 103% | 101% | -0.50% | 0.72% | 0.75% | 101% | 101% |
| Test3 | 0.00% | 0.00% | 0.00% | 100% | 100% | -0.39% | 0.06% | 0.07% | 102% | 101% | -0.40% | 0.77% | 0.80% | 101% | 101% |

# Conclusion

The Hadamard transform based post-reconstruction filter was tested in three configurations suggested in BoG report [3]: minimum filtered block size is 8x4 and 4x8, minimum filtered block size is 8x8 and when filter is applied to inter blocks only. For these three tests simulations over VVC3.0 reportedly show -0.72%, -0.64% and -0.39% change in luma BD-rate correspondingly for RA configuration. It is recommended to include Hadamard transform based post-reconstruction filter into the next release of VVC in one of configurations tested above.

# References

- S. Ikonin, V. Stepin, D. Kuryshev, J. Chen, “CE14: Hadamard transform domain filter (Test 3)”, JVET, doc JVET-L0326, Macao, CN, September 2018

- L. Zhang, S. Ikonin, “CE14: Summary report on post-reconstruction filtering”, JVET, doc JVET-L0034, Macao, CN, September 2018.

- L. Zhang, “BoG report on CE14 and CE14 related contributions”, JVET, doc JVET-L0684, Macao, CN, September 2018.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).