| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0480-v3 |
| --- | --- |

| Title: | CE6-related: Implicit transform selection for Multi directional LM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shunsuke Iwamura Shimpei Nemoto Atsuro Ichigaya | Tel: Email: | +81-3-5494-3353 iwamura.s-gc@nhk.or.jp |
| Source: | NHK |  |  |

_____________________________

- Abstract

This contribution proposes to apply implicit transform selection for multi-directional LM (MDLM). Currently, multiple transform selection (MTS) is applied only to a luma block, where DCT-II, DST-VII, and DCT-VIII are selectively applied for horizontal and vertical direction independently by signalling tu_mts_flag and mts_idx. On the contrary, DCT-II is always applied to a chroma block for both horizontal and vertical direction. The proposed method implicitly selects transform core for chroma block when MDLM is applied, in which either top or left neighbor samples are used for linear model derivation. Experimental results show -0.42% (Cb) and -0.47% (Cr) BD-rate gain for AI, -0.28% (Cb) and -0.21% (Cr) for RA configuration without increase of encoding and decoding time. In version 3 of this contribution, the experimental results shown in Table 1 to Table 3 were corrected since the old tables describe test results of the parameter setting where min/max values of applicable chroma block size set equal to 4 and 32, whereas the setting of min/max recommended by the proponent is 4/16.

Introduction

In VTM-3.0, multiple transform selection (MTS) is introduced where DCT-II, DST-VII, and DCT-VIII are selectively applied for horizontal and vertical direction independently by signalling tu_mts_flag and mts_idx. On the contrary, DCT-II is always applied to a chroma block for both horizontal and vertical direction.

Meanwhile, multi-directional LM (MDLM) mode for chroma intra prediction was adopted in the 12th JVET meeting, in which either top or left neighbor reconstructed samples are used to derive linear model for LM intra prediction. Since only one side of the neighboring reconstructed samples are used, it can be assumed that the energy distribution of residual signal is unbalanced. Specifically, when applying MDLM using only top neighboring reconstructed samples (MDLM_T), it can be assumed that the residual signal in the top region of the current chroma block tends to be lower than that in the bottom region. Similarly, when applying MDLM using only left neighboring reconstructed samples (MDLM_L), it can be assumed that the residual signal in the left region of the current chroma block tends to be lower than that in the right region.

However, in VTM-3.0, only DCT-II can be applied to the chroma residual signal, so that there might be rooms to improve chroma coding efficiency.

Proposed method

In the proposed implicit transform selection, either DCT-II or DST-VII is applied to CUs at which either MDLM_T or MDLM_L is applied. Specifically, for MDLM_T DST-VII is applied in vertical direction and DCT-II is applied in horizontal direction. For MDLM_L, DCT-II in vertical and DST-VII in horizontal direction. Otherwise, DCT-II is applied in both vertical and horizontal directions.

Since in the binarization stage of intra_chroma_pred_mode, MDLM_T and MDLM_L are always assigned as “1111” and “1110”, respectively in the current syntax structure, the transform core to be applied to the current chrom block can be determined just after parsing intra_chroma_pred_mode.

Experimental results

The proposed algorithm was implemented on top of VTM-3.0. Simulations have been conducted under common test condition. The simulation results of all intra, random access, and low delay B configuration are shown in Table 1, 2, and 3, where (min, max) values of applicable chroma CU size is set to (4, 16). Test results shows the proposal bring no impact on both luma coding performance and encoding/decoding run time.

Table 1. Performance of the proposal against VTM-3.0 with all intra configuration

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.00% | -0.48% | -0.33% | 100% | 100% |
| Class A2 | 0.00% | -0.52% | -0.43% | 100% | 100% |
| Class B | 0.01% | -0.47% | -0.57% | 100% | 100% |
| Class C | 0.00% | -0.35% | -0.53% | 100% | 98% |
| Class E | 0.01% | -0.46% | -0.49% | 100% | 99% |
| Overall | 0.00% | -0.45% | -0.48% | 100% | 100% |
| Class D | 0.00% | -0.33% | -0.43% | 100% | 99% |
| Class F | 0.00% | -0.23% | -0.18% | 101% | 100% |

Table 2. Performance of the proposal against VTM-3.0 with random access configuration

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.03% | -0.45% | -0.07% | 101% | 101% |
| Class A2 | 0.00% | -0.29% | -0.13% | 100% | 101% |
| Class B | -0.02% | -0.34% | -0.55% | 100% | 101% |
| Class C | 0.02% | -0.34% | -0.30% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.35% | -0.31% | 100% | 100% |
| Class D | -0.02% | -0.65% | -0.31% | 100% | 99% |
| Class F | -0.05% | -0.22% | -0.33% | 100% | 100% |

Table 3. Performance of the proposal against VTM-3.0 with low delay B configuration

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.41% | -0.29% | 100% | 101% |
| Class C | 0.03% | 0.09% | 0.27% | 100% | 102% |
| Class E | -0.02% | 0.31% | -0.88% | 100% | 101% |
| Overall | 0.00% | -0.07% | -0.25% | 100% | 101% |
| Class D | 0.01% | 0.06% | -0.16% | 100% | 98% |
| Class F | 0.02% | -0.18% | -0.27% | 100% | 101% |

Additionally, possible combinations of applicable (min, max) chroma block size are also tested.

Table 4. Performance comparison between combination of (min, max) values of applicable chroma block

| Chroma block size |  | AI |  | RA |  | LB |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Min | Max | Cb | Cr | Cb | Cr | Cb | Cr |
| 4 | 32 | -0.42% | -0.47% | -0.28% | -0.21% | -0.13% | -0.28% |
| 4 | 16 | -0.45% | -0.48% | -0.35% | -0.31% | -0.07% | -0.25% |
| 4 | 8 | -0.36% | -0.43% | -0.29% | -0.18% | 0.10% | -0.21% |
| 4 | 4 | -0.17% | -0.22% | -0.14% | -0.10% | -0.05% | -0.20% |
| 8 | 32 | -0.35% | -0.34% | -0.21% | -0.13% | 0.39% | -0.13% |

From the results, the reasonable combination of (min, max) values of applicable chroma block size might be (4, 16).

Patent rights declaration(s)

NHK may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and nondiscriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).