| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0502-v2 |
| --- | --- |

| Title: | CE4-related: Improved context for prediction mode flag |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xin Zhao, Xiang Li, Shan Liu | Tel: Email: | {xinzzhao, xlxiangli, shanl} @tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution proposes an improved context design for signaling the pred_mode flag. One context is added, and the context value is derived by whether neighboring blocks are coded by intra prediction, inter prediction or inter-intra prediction mode. It is reportedly shown that, on top of VTM-3.0, average -0.09%, and -0.02% luma BD-Rate is achieved under Random Access (RA) and Low Delay B (LDB) configurations, respectively.

# Introduction

In VVC Draft 3 [1], the prediction mode flag, i.e., pred_mode_flag, is coded using only one context. In JVET-J0021 [2], the context for coding pred_mode_flag of current block is derived using the value of pred_mode_flag of neighboring blocks. As described in Figure 1, the context value is incremented by 1 if above (A) or left (L) block is coded by intra prediction mode, so the number of contexts is increased by 2 for coding pred_mode_flag.

Figure 1: Neighboring blocks used for deriving the context of pred_mode_flag

In VTM-3.0, the combined merge and inter prediction mode, namely inter-intra mode, is also enabled. Therefore, it becomes a question whether inter-intra mode should be considered as intra mode or inter mode when using a neighboring block for the context derivation. In this contribution, different solutions are tested to get a conclusion.

# Proposed method

For deriving the context used for coding pred_mode_flag, four different methods are tested, as described below.

Table 1: Description of the different methods for context derivation of pred_mode_flag

|  | Description of context (ctx) derivation |
| --- | --- |
| Method #1 (3 contexts) | ctx = A is intra ? 1 : 0 ctx += L is intra ? 1 : 0 |
| Method #2 (2 contexts) | ctx = (A is intra) ¦ ¦ (L is intra) ? 1 : 0 |
| Method #3 (3 contexts) | ctx = A is intra or inter-intra ? 1 : 0 ctx += L is intra or inter-intra ? 1 : 0 |
| Method #4 (2 contexts) | ctx = (A is intra or inter-intra) ¦ ¦ (L is intra or inter-intra) ? 1 : 0 |

# Results

Four experiments [2] are conducted to evaluate the proposed methods, and each experiment is performed under CTC condition [3]. The anchor for all the experiments is VTM-3.0. Detailed results for each experiment are reported in Table 2, Table 3, Table 4 and Table 5, respectively.

Table 2: Coding performance of Method #1 (3 contexts) [2]

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.08% | -0.03% | 101% | 100% |  |  |  |  |  |
| Class A2 | -0.09% | 0.08% | 0.08% | 101% | 100% |  |  |  |  |  |
| Class B | -0.09% | 0.09% | -0.07% | 102% | 101% | -0.01% | -0.11% | 0.09% | 100% | 103% |
| Class C | -0.08% | -0.05% | -0.01% | 102% | 105% | -0.01% | 0.29% | 0.14% | 100% | 96% |
| Class E |  |  |  |  |  | -0.03% | 0.41% | -0.13% | 102% | 102% |
| Overall | -0.09% | 0.02% | -0.02% | 102% | 102% | -0.02% | 0.15% | 0.05% | 100% | 101% |
| Class D | -0.03% | -0.01% | -0.06% | 101% | 103% | 0.00% | 0.19% | 0.16% | 103% | 104% |
| Class F | -0.05% | -0.01% | -0.01% | 102% | 104% | 0.08% | -0.16% | 0.24% | 101% | 101% |

Table 3: Coding performance of Method #2 (2 contexts)

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.02% | -0.06% | 101% | 99% |  |  |  |  |  |
| Class A2 | -0.07% | 0.04% | 0.07% | 100% | 100% |  |  |  |  |  |
| Class B | -0.10% | 0.13% | 0.01% | 102% | 101% | -0.03% | -0.24% | 0.01% | 102% | 101% |
| Class C | -0.09% | -0.01% | 0.12% | 102% | 106% | -0.02% | 0.23% | 0.11% | 102% | 100% |
| Class E |  |  |  |  |  | -0.02% | 0.29% | -0.02% | 101% | 100% |
| Overall | -0.09% | 0.04% | 0.04% | 101% | 102% | -0.02% | 0.05% | 0.04% | 102% | 101% |
| Class D | -0.05% | 0.00% | -0.09% | 101% | 103% | 0.00% | -0.32% | -0.17% | 103% | 108% |
| Class F | -0.05% | -0.06% | 0.01% | 102% | 104% | -0.07% | -0.37% | -0.52% | 101% | 99% |

Table 4: Coding performance of Method #3 (3 contexts)

Table 5: Coding performance of Method #4 (2 contexts)

# Conclusions

This proposal provides an improved context derivation for coding pred_mode_flag. With the proposed method, it is reported that average -0.09% and -0.02% luma BD-rate can be achieved for RA and LDB configuration by increasing one context, respectively. It is recommended to adopt the proposed Method #4 into VTM.

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 3),” JVET-L1001, 12th meeting, Macao, CN, 3–12 Oct. 2018.

- Y.-W. Chen, et al, “Description of SDR, HDR and 360° video coding technology proposal by Qualcomm and Technicolor – low and high complexity versions,” JVET-J0021, 10th Meeting: San Diego, US, 10–20 Apr. 2018.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, 12th meeting, Macao, CN, 3–12 Oct. 2018.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).