| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakesh, MA, 9–18 Jan. 2019 | Document: JVET-M0511-v1 |
| --- | --- |

| Title: | Bug fix for rate control under all-intra in VTM software |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Ye Li Dong Liu Zhibo Chen | Email: | liye0713@mail.ustc.edu.cn |
| Source: | University of Science and Technology of China |  |  |

_____________________________

- Abstract

In VVC Common Test conditions, a TemporalSubsampleRatio value is set to be 8 under all intra configuration. This strategy is to simplify the testing procedure. For example, the sequence FoodMarket4 has totally 300 frames, when coding in all intra configuration, only 38 frames are extracted and coded. And the actual frame rate is the original sequence frame rate set in the config file divided by 8. However, as the the sequence original frame rate takes the value of 50 fps or 60 fps. Both two numbers are indivisible by 8. So how to represent the actual frame rate is an option. In VTM, the actual frame rate is implicitly regarded as a float number when printing out the statistic information of bit-rate. However, in case of rate control is enabled, the actual frame rate is explicitly set as a int number, by a successive rounding operation of the division. This inconsistency makes the target bits calculated by the bit-rate a bit different from the actually coded bits, from which the bit-rate is calculated. In this document, we propose to explicitly set the actual frame rate to be a float number in case of TemporalSubsampleRatio is enabled.

Problem Statement

In VTM, after finishing the whole sequence, a lot of coding information are printed out, such as the bit-rate. The bit-rate is calculated as the sequence total bits divided by the sequence play-time. And the play-time is calculated through the total number of frames and the sequence frame rate. In VTM3.0, the play-time is calculated by the code below:

Double time = (Double) m_iFrameRcvd / m_iFrameRate * m_temporalSubsampleRatio;

This equation can also be written as when regarding the actual frame rate as the original frame rate divided by the ratio. In this way, the actual frame rate is taken as a float number and the final bit-rate is precisely calculated as: Bit rate = Total bits/time

When testing the rate control performance, a target bit rate is required. Commonly we first encode the sequence in CTC with rate control disabled and get the final bit rate. Then the bit rate is set as the target bit rate with rate control enabled for testing. In VTM, when rate control is enabled, the target bit is firstly calculated according to the input target bit rate parameter. Then the sequence total bits are allocated in different levels, e.g. GOP level, frame level and CTU level. However, the software explicitly takes the actual frame rate as an integer number and using a rounding operation to calculate it.

EncRCSeq::m_frameRate=(int)((double)m_iFrameRate / m_temporalSubsampleRatio + 0.5)

m_targetBits      = (int64_t)m_totalFrames * (int64_t)m_targetRate / (int64_t)m_frameRate;

When the frame rate is forced to be an integer, the calculated target bits is no longer the coded bits while the target bit rate is the same as the coded bit rate. Suppose that our rate control method works so well that the actually coded bits is the same as the target bits. However, when measuring the difference between the target bit rate and the actual bit rate, we still get a negative result. Table 1 shows the bit-rate error result of VTM-3.0-rc1 rate control method in all intra configuration. As can be seen, when the frame rate is forced to be an integer number, a stable control error of 6.25% or -4.17% is obtained(this corresponds to the two original frame rate of 60fps or 50 fps) due to the rounding effect. So it is difficult to compare the performance of different rate control methods when this error exists.

Table 1 An example of effect of the rounding effect of calculated frame rate

|  |  |  | Frame rate as an integer number |  | Frame rate as a double number |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | QP | Target bit rate (kbps) | Actual bit rate (kbps) | Control error | Actual bit rate (kbps) | Control error |
| RitualDance | 22 | 6012.26 | 5636.88 | 6.25% | 6012.64 | -0.02% |
|  | 27 | 3485.51 | 3267.97 | 6.24% | 3485.83 | 0.00% |
|  | 32 | 1962.66 | 1840.28 | 6.24% | 1962.94 | 0.00% |
|  | 37 | 1074.64 | 1007.56 | 6.23% | 1074.89 | -0.01% |
| Cactus | 22 | 12572.38 | 13096.29 | -4.17% | 12572.50 | 0.00% |
|  | 27 | 5445.90 | 5672.91 | -4.17% | 5446.06 | 0.00% |
|  | 32 | 2938.04 | 3060.63 | -4.17% | 2938.27 | -0.01% |
|  | 37 | 1584.12 | 1650.36 | -4.18% | 1584.35 | -0.01% |

Based on the above analysis, we suggest the actual frame rate be explicitly regarded as a float number and remove the rounding operation in rate control related code.

Patent rights declaration(s)

We are not aware of any current or pending patent rights relating to the information described in this contribution.