Revision 1 of the document also shows in section B VVC WD changes when the scaling of the original temporal motion vector to a fixed POC distance is disabled. Further analysis identified that some aspects may also be achieved non-normatively. These are removed in revision 1.

A. VVC WD changes when scaling the temporal motion vector to a fixed POC distance, and using mantissa-exponent representation

Modification to VVC WD are highlighted in yellow below. Note, [4] is a related proposal that proposes modifying the clipping range of scaled motion vectors to 18-bits.

Invocation of clause A, storage process for temporal luma motion information:

Clause A shall be invoked as part of the decoding process for current picture. Clause A shall be invoked after all motion information (motion vectors, reference indices, prediction utilization flags, flagging of long term reference picture) for the current picture has been decoded and its value determined. Clause A (and clauses invoked within clause A) shall be the last clause accessing the motion information in the decoding process of the current picture.

The function LongTermRefPic( aPic, aCb, refIdx, LX ), with X being 0 or 1, is defined as follows:

- If the picture with index refIdx from reference picture list LX of the slice containing coding block aCb in the picture aPic was marked as "used for long term reference" at the time when aPic was the current picture, LongTermRefPic( aPic, aCb, refIdx, LX ) is equal to 1.

- Otherwise, LongTermRefPic( aPic, aCb, refIdx, LX ) is equal to 0.

#### A. Storage process for temporal luma motion information

Inputs to this process are:

- Arrays of Prediction List Utilization Flags PredFlagL0 and PredFlagL1

- Arrays of luma motion vectors MvL0 and MvL1

- Arrays of reference indices RefIdxL0 and RefIdxL1

Outputs of this process are:

- Arrays MvL0Mantissa, MvL0Exponent, MvL1Mantissa and MvL1MExponent of luma motion vectors stored in mantissa and exponent representation

- Arrays LtrpFlagL0 and LtrpFlagL1 of flags indicating use of  Long Term Reference Pictures

The variable currPic specifies the current picture

The following applies for xL from 0 to (pic_width_in_luma_samples >> 3):

- The following applies for yL from 0 to (pic_height_in_luma_samples >> 3):

  - The variables x and y are assigned values (xL << 3) and (yL << 3) respectively

  - The variable currCb specifies the coding block at luma location (x, y)

  - The derivation process to scale and convert motion vector displacement to mantissa and exponent representation as specified in clause B is invoked with MvL0[ x ][ y ] as input, and the output is assigned to MvL0Mantissa[ x ][ y ] and MvL0Exponent[ x ][ y ]

  - Assign output of LongTermRefPic( currPic, currCb, RefIdxL0[ x ][ y ], L0 ) to LtrpFlagL0[ x ][ y ]

  - The derivation process to scale convert motion vector displacement to mantissa and exponent representation as specified in clause B is invoked with MvL1[ x ][ y ] as input, and the output is assigned to MvL1Mantissa[ x ][ y ] and MvL1Exponent[ x ][ y ]

  - Assign output of LongTermRefPic( currPic, currCb, RefIdxL1[ x ][ y ], L1 ) to ltrpFlagL1[ x ][ y ]

#### 8.3.2.12	Derivation process for collocated motion vectors

Inputs to this process are:

- a variable currCb specifying the current coding block,

- a variable colCb specifying the collocated coding block inside the collocated picture specified by ColPic,

- a luma location ( xColCb, yColCb ) specifying the top-left sample of the collocated luma coding block specified by colCb relative to the top-left luma sample of the collocated picture specified by ColPic,

- a reference index refIdxLX, with X being 0 or 1,

- a flag indicating a subblock temporal merging candidate sbFlag.

Outputs of this process are:

- the motion vector prediction mvLXCol in 1/16 fractional-sample accuracy,

- the availability flag availableFlagLXCol.

The variable currPic specifies the current picture.

The arrays predFlagL0Col[ x ][ y ], mvL0Col[ x ][ y ] and refIdxL0Col[ x ][ y ] are set equal to PredFlagL0[ x ][ y ], MvL0[ x ][ y ] and RefIdxL0[ x ][ y ], respectively, of the collocated picture specified by ColPic, and the arrays predFlagL1Col[ x ][ y ], mvL1Col[ x ][ y ] and refIdxL1Col[ x ][ y ] are set equal to PredFlagL1[ x ][ y ], MvL1[ x ][ y ] and RefIdxL1[ x ][ y ], respectively, of the collocated picture specified by ColPic.

The array predFlagL0[ x ][ y ], mvL0MantissaCol[ x ][ y ], mvL0ExponentCol[ x ][ y ], ltrpL0FlagCol[ x ][ y ], predFlagL1[ x ][ y ], mvL1MantissaCol[ x ][ y ], mvL1ExponentCol[ x ][ y ] and ltrpL1FlagCol[ x ][ y ] are set equal to PredFlagL0[ x ][ y ], MvL0Mantissa[ x ][ y ], MvL0Exponent[ x ][ y ] LtrpL0Flag[ x ][ y ], PredFlagL1[ x ][ y ], MvL1Manitssa[ x ][ y ], MvL1Exponent[ x ][ y ], and LtrpL1Flag[ x ][ y ] respectively of the collocated picture specified by ColPic.

[Ed. (BB): Define ColPic NoBackwardPredFlag.]

The variables mvLXCol and availableFlagLXCol are derived as follows:

- If colCb is coded in an intra prediction mode, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

- Otherwise, the motion vector mvCol, the reference index refIdxCol and the reference list identifier listCol are derived as follows:

  - If sbFlag is equal to 0, availableFlagLXCol is set to 1 and the following applies:

  - If predFlagL0Col[ xColCb ][ yColCb ] is equal to 0, mvCol, refIdxCol and listCol are set equal to mvL1Col[ xColCb ][ yColCb ], refIdxL1Col[ xColCb ][ yColCb ] and L1, respectively.

  - Otherwise, if predFlagL0Col[ xColCb ][ yColCb ] is equal to 1 and predFlagL1Col[ xColCb ][ yColCb ] is equal to 0, mvCol, refIdxCol and listCol are set equal to mvL0Col[ xColCb ][ yColCb ], refIdxL0Col[ xColCb ][ yColCb ] and L0, respectively.

  - Otherwise (predFlagL0Col[ xColCb ][ yColCb ] is equal to 1 and predFlagL1Col[ xColCb ][ yColCb ] is equal to 1), the following assignments are made:

      - If NoBackwardPredFlag is equal to 1, mvCol, refIdxCol and listCol are set equal to mvLXCol[ xColCb ][ yColCb ], refIdxLXCol[ xColCb ][ yColCb ] and LX, respectively.

      - Otherwise, mvCol, refIdxCol and listCol are set equal to mvLNCol[ xColCb ][ yColCb ], refIdxLNCol[ xColCb ][ yColCb ] and LN, respectively, with N being the value of collocated_from_l0_flag.

  - Otherwise (sbFlag is equal to 1), the following applies:

  - If PredFlagLXCol[ xColCb ][ yColCb ] is equal to 1, mvCol, refIdxCol, and listCol are set equal to mvLXCol[ xColCb ][ yColCb ], refIdxLXCol[ xColCb ][ yColCb ], and LX, respectively, availableFlagLXCol is set to 1.

  - Otherwise (PredFlagLXCol[ xColCb ][ yColCb ] is equal to 0), the following applies:

      - If DiffPicOrderCnt( aPic, currPic ) is less than or equal to 0 for every picture aPic in every reference picture list of the current slice and PredFlagLYCol[ xColCb ][ yColCb ] is equal to 1, mvCol, refIdxCol, and listCol are set to mvLYCol[ xColCb ][ yColCb ], refIdxLYCol[ xColCb ][ yColCb ] and LY, respectively, with Y being equal to !X where X being the value of X this process is invoked for. availableFlagLXCol is set to 1.

      - Both the components of mvLXCol are set to 0 and availableFlagLXCol is set equal to 0.

  - When availableFlagLXCol is equal to TRUE, mvLXCol and availableFlagLXCol are derived as follows:

  - If LongTermRefPic( currPic, currCb, refIdxLX, LX ) is not equal to LongTermRefPic( ColPic, colCb, refIdxCol, listCol ) ltrpLXFlagCol[ xColCb ][ yColCb ], both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

  - Otherwise, the variable availableFlagLXCol is set equal to 1, refPicListCol[ refIdxCol ] is set to be the picture with reference index refIdxCol in the reference picture list listCol of the slice containing coding block colCb in the collocated picture specified by ColPic, and the following applies:

colPocDiff = DiffPicOrderCnt( ColPic, refPicListCol[ refIdxCol ] )	(8-334)

currPocDiff = DiffPicOrderCnt( currPic, RefPicListX[ refIdxLX ] )	(8-335)

      - The derivation process for motion vector displacement from mantissa and exponent representation as specified in clause C is invoked with mvL0MantissaCol[ xColCb ][ yColCb ], mvL0ExponentCol[ xColCb ][ yColCb ] as input, and the output is assigned to mvColScaledQ.

      - If RefPicListX[ refIdxLX ] is a long-term reference picture, or colPocDiff is equal to currPocDiff, mvLXCol is derived as follows:

mvLXCol = mvCol mvColScaledQ	(8-336)

      - Otherwise, mvLXCol is derived as a scaled version of the motion vector mvCol mvColScaledQ as follows:

tx = ( 16384 + ( Abs( td 4 )  >>  1 ) ) / td 4	(8-337)

distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 )  >>  6 )	(8-338)

mvLXCol =  Clip3( −32768, 32767, Sign( distScaleFactor * mvCol mvColScaledQ ) * 
				( ( Abs( distScaleFactor * mvCol mvColScaledQ ) + 127 )  >>  8 ) )	(8-339)

where td and tb are derived as follows:

td = Clip3( −128, 127, colPocDiff )	(8-340)

tb = Clip3( −128, 127, currPocDiff )	(8-341)

#### B. Derivation process to scale and convert motion vector displacement to mantissa and exponent representation

Inputs to this process are:

- a array mvCol specifying the input motion vector,

Outputs of this process are:

- Arrays mvColScaledMantissa, mvColScaledExponent specifying the mantissa and exponent representation of the scaled temporal motion vector

The variable mvColScaled is derived as follows:

txScaled = ( 16384 + ( Abs( tdScaled )  >>  1 ) ) / tdScaled	(8-xxx)

distColScaleFactor =  ( 4 * txScaled + 32 )  >>  6 		(8-xxx)

mvColScaled =  Clip3( −32768, 32767, Sign( distColScaleFactor * mvCol ) * 
				( ( Abs( distColScaleFactor * mvCol ) + 127 )  >>  8 ) )	(8-xxx)

where tdScaled is derived as follows:

tdScaled = Clip3( −128, 127, colPocDiff )

The array mvColScaledMantissa, mvColScaledExponent is derived as follows:

for( dir = 0; dir < 2; dir++ ) {
  mantissa[ dir ] = mvColScaled[ dir ]
  exponent[ dir ] = 0
  if ( mvColScaled[ dir ] < -32 || mvColScaled[ dir ] > 31 ) {
    
    exponentOffset = 1
    while ( mvColScaled[ dir ] < -( 1 << ( exponent[ dir ] + 5 ) ) || 
                mvColScaled[ dir ] > ( ( 1 << ( exponent[ dir ] + 5 ) ) - 1 ) ) {
      exponentOffset++
    }
    offset = ( 1 << ( exponentOffset - 1 ) ) >> 1
    
    exponent[ dir ] = 1
    while ( ( mvColScaled[ dir ] + offset ) < -( 1 << ( exponent[ dir ] + 5 ) ) || 
                ( mvColScaled[ dir ] + offset ) > ( ( 1 << ( exponent[ dir ] + 5 ) ) - 1 ) ) {
      exponent[ dir ]++
      offset = ( 1 << ( exponent[ dir ] + 4 ) )
    }
    mantissa[ dir ] = ( ( mvColScaled[ dir ] + offset ) >> ( exponent[ dir ] – 1 ) ) ^ 32
  }
  mvColScaledMantissa[dir] = mantissa[ dir ] 
  mvColScaledExponent[dir] = exponent[ dir ] 
}

#### C. Derivation process for motion vector displacement from mantissa and exponent representation

Inputs to this process are:

- Arrays mvColScaledMantissa, mvColScaledExponent specifying the scaled temporal motion vector in mantissa and exponent representation,

Outputs of this process are:

- a array mvColScaledQ specifying the scaled temporal motion vector derived from its mantissa and exponent representation

The array mvColScaledQ is  derived as follows:

for( dir = 0; dir < 2; dir++ ) {
  exponent[ dir ] = mvColScaledExponent[ dir ]
  mantissa[ dir ] = mvColScaledMantissa[ dir ]
  if ( exponent[ dir ] = = 0 )
    mvColScaledQ[ dir ] = mantissa[ dir ]
  else
    mvColScaledQ[ dir ] = ( ( mantissa[ dir ] ^ 32 ) << ( exponent[ dir ] - 1 ) )
}

B. VVC WD changes when using mantissa-exponent representation

Modification to VVC WD are highlighted in yellow below.

Invocation of clause A, storage process for temporal luma motion information:

Clause A shall be invoked as part of the decoding process for current picture. Clause A shall be invoked after all motion information (motion vectors, reference indices, prediction utilization flags, flagging of long term reference picture) for the current picture has been decoded and its value determined. Clause A (and clauses invoked within clause A) shall be the last clause accessing the motion information in the decoding process of the current picture.

The function LongTermRefPic( aPic, aCb, refIdx, LX ), with X being 0 or 1, is defined as follows:

- If the picture with index refIdx from reference picture list LX of the slice containing coding block aCb in the picture aPic was marked as "used for long term reference" at the time when aPic was the current picture, LongTermRefPic( aPic, aCb, refIdx, LX ) is equal to 1.

- Otherwise, LongTermRefPic( aPic, aCb, refIdx, LX ) is equal to 0.

#### A. Storage process for temporal luma motion information

Inputs to this process are:

- Arrays of Prediction List Utilization Flags PredFlagL0 and PredFlagL1

- Arrays of luma motion vectors MvL0 and MvL1

- Arrays of reference indices RefIdxL0 and RefIdxL1

Outputs of this process are:

- Arrays MvL0Mantissa, MvL0Exponent, MvL1Mantissa and MvL1MExponent of luma motion vectors stored in mantissa and exponent representation

- Arrays LtrpFlagL0 and LtrpFlagL1 of flags indicating use of  Long Term Reference Pictures

The variable currPic specifies the current picture

The following applies for xL from 0 to (pic_width_in_luma_samples >> 3):

- The following applies for yL from 0 to (pic_height_in_luma_samples >> 3):

  - The variables x and y are assigned values (xL << 3) and (yL << 3) respectively

  - The variable currCb specifies the coding block at luma location (x, y)

  - The derivation process to convert motion vector displacement to mantissa and exponent representation as specified in clause B is invoked with MvL0[ x ][ y ] as input, and the output is assigned to MvL0Mantissa[ x ][ y ] and MvL0Exponent[ x ][ y ]

  - The derivation process to convert motion vector displacement to mantissa and exponent representation as specified in clause B is invoked with MvL1[ x ][ y ] as input, and the output is assigned to MvL1Mantissa[ x ][ y ] and MvL1Exponent[ x ][ y ]

#### 8.3.2.12	Derivation process for collocated motion vectors

Inputs to this process are:

- a variable currCb specifying the current coding block,

- a variable colCb specifying the collocated coding block inside the collocated picture specified by ColPic,

- a luma location ( xColCb, yColCb ) specifying the top-left sample of the collocated luma coding block specified by colCb relative to the top-left luma sample of the collocated picture specified by ColPic,

- a reference index refIdxLX, with X being 0 or 1,

- a flag indicating a subblock temporal merging candidate sbFlag.

Outputs of this process are:

- the motion vector prediction mvLXCol in 1/16 fractional-sample accuracy,

- the availability flag availableFlagLXCol.

The variable currPic specifies the current picture.

The arrays predFlagL0Col[ x ][ y ], mvL0Col[ x ][ y ] and refIdxL0Col[ x ][ y ] are set equal to PredFlagL0[ x ][ y ], MvL0[ x ][ y ] and RefIdxL0[ x ][ y ], respectively, of the collocated picture specified by ColPic, and the arrays predFlagL1Col[ x ][ y ], mvL1Col[ x ][ y ] and refIdxL1Col[ x ][ y ] are set equal to PredFlagL1[ x ][ y ], MvL1[ x ][ y ] and RefIdxL1[ x ][ y ], respectively, of the collocated picture specified by ColPic.

The array mvL0MantissaCol[ x ][ y ], mvL0ExponentCol[ x ][ y ], mvL1MantissaCol[ x ][ y ] and mvL1ExponentCol[ x ][ y ] are set equal to MvL0Mantissa[ x ][ y ], MvL0Exponent[ x ][ y ], MvL1Manitssa[ x ][ y ] and MvL1Exponent[ x ][ y ] respectively of the collocated picture specified by ColPic.

[Ed. (BB): Define ColPic NoBackwardPredFlag.]

The variables mvLXCol and availableFlagLXCol are derived as follows:

- If colCb is coded in an intra prediction mode, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

- Otherwise, the motion vector mvCol, the reference index refIdxCol and the reference list identifier listCol are derived as follows:

  - If sbFlag is equal to 0, availableFlagLXCol is set to 1 and the following applies:

  - If predFlagL0Col[ xColCb ][ yColCb ] is equal to 0, mvCol, refIdxCol and listCol are set equal to mvL1Col[ xColCb ][ yColCb ], refIdxL1Col[ xColCb ][ yColCb ] and L1, respectively.

  - Otherwise, if predFlagL0Col[ xColCb ][ yColCb ] is equal to 1 and predFlagL1Col[ xColCb ][ yColCb ] is equal to 0, mvCol, refIdxCol and listCol are set equal to mvL0Col[ xColCb ][ yColCb ], refIdxL0Col[ xColCb ][ yColCb ] and L0, respectively.

  - Otherwise (predFlagL0Col[ xColCb ][ yColCb ] is equal to 1 and predFlagL1Col[ xColCb ][ yColCb ] is equal to 1), the following assignments are made:

      - If NoBackwardPredFlag is equal to 1, mvCol, refIdxCol and listCol are set equal to mvLXCol[ xColCb ][ yColCb ], refIdxLXCol[ xColCb ][ yColCb ] and LX, respectively.

      - Otherwise, mvCol, refIdxCol and listCol are set equal to mvLNCol[ xColCb ][ yColCb ], refIdxLNCol[ xColCb ][ yColCb ] and LN, respectively, with N being the value of collocated_from_l0_flag.

  - Otherwise (sbFlag is equal to 1), the following applies:

  - If PredFlagLXCol[ xColCb ][ yColCb ] is equal to 1, mvCol, refIdxCol, and listCol are set equal to mvLXCol[ xColCb ][ yColCb ], refIdxLXCol[ xColCb ][ yColCb ], and LX, respectively, availableFlagLXCol is set to 1.

  - Otherwise (PredFlagLXCol[ xColCb ][ yColCb ] is equal to 0), the following applies:

      - If DiffPicOrderCnt( aPic, currPic ) is less than or equal to 0 for every picture aPic in every reference picture list of the current slice and PredFlagLYCol[ xColCb ][ yColCb ] is equal to 1, mvCol, refIdxCol, and listCol are set to mvLYCol[ xColCb ][ yColCb ], refIdxLYCol[ xColCb ][ yColCb ] and LY, respectively, with Y being equal to !X where X being the value of X this process is invoked for. availableFlagLXCol is set to 1.

      - Both the components of mvLXCol are set to 0 and availableFlagLXCol is set equal to 0.

  - When availableFlagLXCol is equal to TRUE, mvLXCol and availableFlagLXCol are derived as follows:

  - If LongTermRefPic( currPic, currCb, refIdxLX, LX ) is not equal to LongTermRefPic( ColPic, colCb, refIdxCol, listCol ), both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

  - Otherwise, the variable availableFlagLXCol is set equal to 1, refPicListCol[ refIdxCol ] is set to be the picture with reference index refIdxCol in the reference picture list listCol of the slice containing coding block colCb in the collocated picture specified by ColPic, and the following applies:

colPocDiff = DiffPicOrderCnt( ColPic, refPicListCol[ refIdxCol ] )	(8-334)

currPocDiff = DiffPicOrderCnt( currPic, RefPicListX[ refIdxLX ] )	(8-335)

      - The derivation process for motion vector displacement from mantissa and exponent representation as specified in clause C is invoked with mvL0MantissaCol[ xColCb ][ yColCb ], mvL0ExponentCol[ xColCb ][ yColCb ] as input, and the output is assigned to mvColQ.

      - If RefPicListX[ refIdxLX ] is a long-term reference picture, or colPocDiff is equal to currPocDiff, mvLXCol is derived as follows:

mvLXCol = mvCol mvColQ	(8-336)

      - Otherwise, mvLXCol is derived as a scaled version of the motion vector mvCol mvColQ as follows:

tx = ( 16384 + ( Abs( td )  >>  1 ) ) / td	(8-337)

distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 )  >>  6 )	(8-338)

mvLXCol =  Clip3( −32768, 32767, Sign( distScaleFactor * mvCol mvColQ ) * 
				( ( Abs( distScaleFactor * mvCol mvColQ ) + 127 )  >>  8 ) )	(8-339)

where td and tb are derived as follows:

td = Clip3( −128, 127, colPocDiff )	(8-340)

tb = Clip3( −128, 127, currPocDiff )	(8-341)

#### B. Derivation process to convert motion vector displacement to mantissa and exponent representation

Inputs to this process are:

- a array mvCol specifying the input motion vector,

Outputs of this process are:

- Arrays mvColMantissa, mvColExponent specifying the mantissa and exponent representation of the temporal motion vector

The array mvColMantissa, mvColExponent is derived as follows:

for( dir = 0; dir < 2; dir++ ) {
  mantissa[ dir ] = mvCol [ dir ]
  exponent[ dir ] = 0
  if ( mvCol[ dir ] < -32 || mvCol[ dir ] > 31 ) {
    
    exponentOffset = 1
    while ( mvCol[ dir ] < -( 1 << ( exponent[ dir ] + 5 ) ) || 
                mvCol[ dir ] > ( ( 1 << ( exponent[ dir ] + 5 ) ) - 1 ) ) {
      exponentOffset++
    }
    offset = ( 1 << ( exponentOffset - 1 ) ) >> 1
    
    exponent[ dir ] = 1
    while ( ( mvCol[ dir ] + offset ) < -( 1 << ( exponent[ dir ] + 5 ) ) || 
                ( mvCol[ dir ] + offset ) > ( ( 1 << ( exponent[ dir ] + 5 ) ) - 1 ) ) {
      exponent[ dir ]++
      offset = ( 1 << ( exponent[ dir ] + 4 ) )
    }
    mantissa[ dir ] = ( ( mvCol [ dir ] + offset ) >> ( exponent[ dir ] – 1 ) ) ^ 32
  }
  mvColMantissa[dir] = mantissa[ dir ] 
  mvColExponent[dir] = exponent[ dir ] 
}

#### C. Derivation process for motion vector displacement from mantissa and exponent representation

Inputs to this process are:

- Arrays mvColMantissa, mvColExponent specifying the temporal motion vector in mantissa and exponent representation,

Outputs of this process are:

- a array mvColQ specifying the temporal motion vector derived from its mantissa and exponent representation

The array mvColQ is derived as follows:

for( dir = 0; dir < 2; dir++ ) {
  exponent[ dir ] = mvColExponent[ dir ]
  mantissa[ dir ] = mvColMantissa[ dir ]
  if ( exponent[ dir ] = = 0 )
    mvColQ[ dir ] = mantissa[ dir ]
  else
    mvColQ[ dir ] = ( ( mantissa[ dir ] ^ 32 ) << ( exponent[ dir ] - 1 ) )
}