| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakesh, MO, 9-18 Jan. 2019 | Document: JVET-M0537 |
| --- | --- |

| Title: | AHG17: On signaling of tile group set with MCTS properties (NAL unit header and new parameter set) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexandre Gabriel, Emmanuel Thomas | Tel: Email: | 0031 611 843 111 {alexandre.dasilvapratasgabriel , emmanuel.thomas}@tno.nl |
| Source: | TNO |  |  |

_____________________________

# Abstract

Recent innovation in video-based applications such as tile-based streaming of omnidirectional videos has been leveraging HEVC tiles. Although proven to be possible, these applications suffer from complex implementation and low performances due to limitations of the design of HEVC tiles: rigid grid structure, absolute tile position, tile position at low-level in the bitstream, absence of high-level syntax element for tile detection, non-trivial encoding constraints, etc. More information is provided in M0536 [1]. It is proposed to define a high-level syntax that better supports these innovations.

Essentially, the proposal is:

- Adding an identifier in NAL unit header to associate a NAL unit with a set of tile groups with MCTS properties

- Creating a non-VCL NAL unit to signal the relative spatial relationship between tile group sets with MCTS properties over several frames. This new non-VCL NAL units points to a SPS and is pointed from a PPS.

It is claimed that the proposal enables:

  - Extraction, aggregation and repositioning of sequence-level tiles do not require the modification of VCL NAL unit payload. This guarantees fast access, real time processing and low complexity implementations.

  - Modification of the spatial relationships of sequence-level tiles without rewriting VCL NAL unit payload for achieving:

    - Client-side aggregation of multiple streams

    - Client-side extraction of relevant tiles to their viewport

    - Client-side repositioning without decoding VCL NAL unit payloads

# Problem Statement

There are several video applications where spatial random access is useful not only within a single frame but across several consecutive frames. However, tiles are currently specified on a frame basis which do not allow for easy manipulation of the content over several frames. This is partly because tiles have originally been designed for parallelization which is still relevant today but not fulfilling new application-driven use cases. Additional signaling have been designed in the HEVC context to patch the specification such as the Motion-Constrained Tile Set SEI message. The contribution argues that instead of patching the specification, a clean design is preferable for the new video coding standard to ensure the efficient implementation of such applications and at the same time allows new usage of tiles that are currently impossible.

More background of typical use cases aimed to be enabled are provided in M0536 [1] and repeated in annex A for convenience.

The proposal is to signal the sequence-level tile group sets, defined in the proposal M0536 [1], with the following:

- Adding an identifier in NAL unit header to associate a NAL unit with a set of tile groups with MCTS properties

- Creating a non-VCL NAL unit to signal the relative spatial relationship between tile group sets with MCTS properties over several frames as well as to define the an identifier for the corresponding tile group set

# Proposal

## Syntax and semantics

### Identifier in NAL unit header of tile groups with MCTS properties

The proposal is to define an identified for tile groups with MCTS properties. In the context of the M0536, these tile groups are called sequence-level tile group sets. With this terminology, the identifier would be nuh_sequence_level_tile_group_set_id and located in the NAL unit header. This way, extraction of sequence-level tile group set sub bitstreams merely requires the parsing of NAL unit headers. This fulfills design goals a and 3a.

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_unit_type | u(5) |
| nuh_temporal_id_plus1 | u(3) |
| nuh_sequence_level_tile_group_set_id | u(8) |
| nuh_reserved_zero_7bits | u(7) |
| } |  |

Note that nuh_sequence_level_tile_group_set_id could also have a variable size to allow more compactness or larger value ranges to address a wide variety of scenarios.

Semantics:

| Syntax element | Meaning |
| --- | --- |
| nuh_sequence_level_tile_group_set_id | When value is different of '0', the value is the id of the sequence-level to which the NAL belongs to. When value is equal to '0', the NAL unit has no specific relation with a particular sequence-level tile group set. The NAL unit type will determines whether the NAL unit is invariant to merging and extraction operations of sequence-level tile bitstreams. |

### Signaling of tile groups with MCT properties spatial relationship and dimensions

Relative spatial position

To fulfill 3a and 3b, the proposal is to define the position of the groups with MCT properties relatively to each other and not in an absolute manner using a coordinate system linked to the picture. In other words, each tile groups with MCT properties defines its neighbors north, east, south and west and the knowledge of this information for each tile group with MCT properties allows to derive absolute position information.

Relative addressing

In addition, the positions of the tiles in the picture are defined relatively from the top-left corner of the tile group with MCT properties. This way, the addresses of the tiles are not modified when extracting/aggregating/repositioning tile groups with MCT properties which thus avoids rewriting the payload of tiles.

The STGPS points to a SPS while PPSs point to STGPS as illustrated in Figure 5.

Figure 5 - Non-VCL NAL units relations

Assuming the terminology in M0536, it is proposed to be added to VVC specification:

Syntax:

| sequence_level_tile_group_parameter_set( ) { | Descriptor |
| --- | --- |
| stgps_sequence_level_tile_group_set_id | ue(v) |
| stgps_sps_id | ue(v) |
| boundary_identifier_north | u(8) |
| boundary_identifier_east | u(8) |
| boundary_identifier_south | u(8) |
| boundary_identifier_west | u(8) |
| } |  |

Semantics:

| Syntax element | Meaning |
| --- | --- |
| sequence_level_tile_group_set_id | Id of this sequence-level tile group set |
| boundary_identifier_north | Identifier at the north boundary of this sequence-level tile group set; |
| boundary_identifier_east | Identifier at the east boundary of this sequence-level tile group set; |
| boundary_identifier_south | Identifier at the south boundary of this sequence-level tile group set; |
| boundary_identifier_west | Identifier at the west boundary of this sequence-level tile group set. |

If the identifier of a boundary couple, either north/south or east/west, from different sequence-level tile group set match then the sequence-level tile group sets are adjacent, and they share a common edge at this boundary couple.

# Patent rights declaration(s)

TNO may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

| [1] | A. Gabriel and E. Thomas, AHG12: On picture-level tile and sequence-level tile for VVC, JVET-M0536. |
| --- | --- |
| [2] | "ISO/IEC 23090-2, Information technology -- Coded representation of immersive media -- Part 2: Omnidirectional media format". |

# A. Annex: Use cases for concept of tile group sets over several frames

## Use case 1: Viewport-driven transmission and decoding of VR 360 Videos

VR 360 applications do not render the whole video but only a small part of it corresponding to the user's viewport. Figure 1 shows an example of how a tiling system can enable energy consumption and bandwidth savings: if a user is looking down then only tile 6 would be needed for the application to render the user's viewport. In this case, this would allow for large savings in transmitted bitrate compared to sending the whole bitstream as well as saving decoding cycles on the client. Note that this tiling is an example and any other tiling scheme could be used to describe the VR application use case.

Figure 1 - Example partition in a 360 ERP video as recommended by OMAF [2]

This example shows that a higher-level partitioning of the frames over time is needed.

## Use case 2: Video Stream Aggregation

Another application of spatially segmented video is the mixing of sequences to produce a new video such as in teleconference or video surveillance, see Figure 2. In this use case, the application needs to combine video streams from different sources, possibly following a non-regular grid. Without this combining capability, the application would need to transcode all the bitstreams into one or run one decoder per bitstream along with synchronization control of all the decoder output.

Figure 2 – Multiple videos received by user resulting in one composite image

This example shows that a higher-level partitioning of the frames over time is needed.

## Use case 3: Region-Of-Interest extraction in 2D videos

Extractions of Regions-Of-Interest in large panoramas can be interesting for users who want to watch specific parts of one captured frame. However, the content may have a constitution that does not fit a regular tiling grid. For example, Figure 3 shows an example where the upper part of the video has the same low interest level for the viewer (the audience in the stadium) and a high interest level which is dynamic (where the ball is) in the lower part of the video. Such an application would display the region of the video the user would select or alternatively could automatically select the region based on saliency detection.

Figure 3 - example of ROIs

This example shows that a higher-level partitioning of the frames over time is needed.