| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakesh, MA, 9–18 Jan. 2019 | Document: JVET-M0543 | Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-M0xxx |
| --- | --- | --- | --- |

| Title: | Cross-check of JVET-M0474: CE8.1.3 Extended CPR reference with 1 buffer line |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | S. Paluri S. Kim | Tel: Email: | + 858 635 5300 seethal.paluri@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This document reports the cross-check results for JVET-M0474 (CE8.1.3 Extended CPR reference with 1 buffer line). All the BD-rate results of VTM are matched to what proponent provided, except minor differences on a few sequences in the SCC class for RA and LDB test conditions. The difference may be attributed to the use of different compiler versions between proponent and cross checker.

# Introduction

This report presents the cross-checker’s results for the Core Experiment CE8.1.3. In this test, the line buffer (1 line above) and one column to the left of the current CTU is added to the CPR reference. The proposed change with the restricted search area was used to compute BDR changes for CTC, Class F, class SCC 1080p. In order to improve the performance of CPR in restricted CPR reference mode without incurring additional cost, the CE8.1.3 utilizes the line buffer to extend the CRP reference. The proposed CPR reference is shown below in Figure 1.0.

Figure 1.0 Shows the Extended CPR reference of CE8.1.3.

# Simulation results

The simulation was carried out under the JVET common test condition [1]. The proposed technique has been implemented on top of VTM3.0. These tests were evaluated with PLT off and CPR on. The simulations were run on the same type of machines in Linux cluster. All the BD-rate results of VTM are matched to what proponent provided, with the exception of a few sequences in the SCC class for RA and LDB test conditions. The difference is noted to be minor. This may be attributed to the use of different compiler versions (gcc 4.8.2(cross checker) and gcc 6.2.0 (tester)). The summarized results are shown in Tables 1-6. The detailed results can be found in Excel files in the package. The simulation was run for the AI, RA and LDB configurations.

Table 1.0: Experimental results for AI test condition on VTM3 for Test 1

| AI | Over VTM 3.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08 | -0.05 | 0.06 | 131 | 105 |
| Class A2 | -0.49 | -0.55 | -0.55 | 133 | 105 |
| Class B | -0.31 | -0.40 | -0.35 | 142 | 108 |
| Class C | -0.10 | -0.17 | -0.28 | 147 | 112 |
| Class E | -0.67 | -0.76 | -0.61 | 147 | 109 |
| Overall | -0.29 | -0.37 | -0.34 | 140 | 108 |
| Class D | -0.13 | -0.26 | 0.11 | 144 | 109 |
| Class F | -12.21 | -12.14 | -12.19 | 161 | 109 |
| Class SCC 1080p | -37.14 | -36.32 | -36.57 | 145 | 102 |

Table 2.0: Experimental results for RA test condition on VTM3 for Test 1

| RA | Over VTM 3.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.12 | -0.01 | 0.24 | 101 | 101 |
| Class A2 | -0.01 | 0.06 | 0.09 | 98 | 102 |
| Class B | 0.00 | 0.05 | -0.23 | 100 | 103 |
| Class C | 0.21 | -0.08 | 0.10 | 103 | 104 |
| Class E |  |  |  |  |  |
| Overall | 0.08 | 0.01 | 0.02 | 101 | 103 |
| Class D | 0.16 | -0.37 | 0.22 | 104 | 105 |
| Class F | -9.95 | -10.05 | -9.94 | 109 | 105 |
| Class SCC 1080p | -22.17 | -21.74 | -22.25 | 112 | 102 |

Table 3.0: Experimental results for LDB test condition on VTM3 for Test 1

| LDB | Over VTM 3.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.08 | -0.15 | 0.08 | 109 | 108 |
| Class C | 0.12 | 0.16 | 0.37 | 111 | 106 |
| Class E | 0.24 | 1.11 | -0.69 | 108 | 105 |
| Overall | 0.13 | 0.27 | -0.02 | 109 | 106 |
| Class D | 0.15 | 0.21 | 0.98 | 113 | 106 |
| Class F | -5.18 | -5.41 | -5.56 | 110 | 106 |
| Class SCC 1080p | -13.02 | -13.22 | -13.55 | 112 | 102 |

Table 4.0: Experimental results for AI test condition on VTM3 for Test 2

| AI | Over VTM 3.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.11 | 0.06 | 0.13 | 140 | 105 |
| Class A2 | -0.39 | -0.43 | -0.44 | 141 | 106 |
| Class B | -0.21 | -0.34 | -0.28 | 149 | 108 |
| Class C | 0.00 | -0.19 | -0.13 | 153 | 113 |
| Class E | -0.52 | -0.67 | -0.53 | 143 | 109 |
| Overall | -0.19 | -0.31 | -0.25 | 147 | 108 |
| Class D | -0.02 | -0.05 | 0.15 | 150 | 108 |
| Class F | -15.57 | -14.12 | -14.27 | 175 | 101 |
| Class SCC 1080p | -44.89 | -41.21 | -41.21 | 162 | 81 |

Table 5.0: Experimental results for RA test condition on VTM3 for Test 2

| RA | Over VTM 3.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.22 | 0.06 | 0.31 | 105 | 101 |
| Class A2 | 0.13 | 0.14 | 0.20 | 103 | 102 |
| Class B | 0.12 | 0.06 | -0.19 | 105 | 104 |
| Class C | 0.36 | -0.06 | 0.46 | 110 | 105 |
| Class E |  |  |  |  |  |
| Overall | 0.21 | 0.05 | 0.16 | 106 | 103 |
| Class D | 0.41 | 0.00 | 0.06 | 112 | 106 |
| Class F | -11.98 | -11.79 | -12.03 | 117 | 103 |
| Class SCC 1080p | -25.74 | -24.55 | -24.91 | 109 | 99 |

Table 6.0: Experimental results for LDB test condition on VTM3 for Test 2

| LDB | Over VTM 3.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.16 | 0.01 | 0.16 | 115 | 108 |
| Class C | 0.26 | 0.32 | 0.68 | 120 | 106 |
| Class E | 0.45 | 0.48 | -0.52 | 115 | 105 |
| Overall | 0.27 | 0.23 | 0.16 | 116 | 107 |
| Class D | 0.32 | 0.35 | 0.44 | 121 | 107 |
| Class F | -6.25 | -7.17 | -7.64 | 117 | 105 |
| Class SCC 1080p | -14.76 | -14.53 | -14.56 | 115 | 99 |

# References

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team (JVET), JVET-L1010, 12th Meeting: Macao, CN, 08–12 Oct. 2018.