| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0561-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-M0045 (Non-CE3: PDPC Restriction) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jinho Lee Hahyun Lee Sung-Chang Lim Jungwon Kang 218 Gajeong-ro, Yuseong-gu, Daejeon, 34129, KOREA | Tel: Email: | +82-42-860-5656 jinosoul@etri.re.kr hanilee@etri.re.kr sclim@etri.re.kr jungwon@etri.re.kr |
| Source: | ETRI (Electronics and Telecommunications Research Institute) |  |  |

_____________________________

- Abstract

This contribution reports crosscheck results of PDPC restriction in JVET-M0045. It is reported that the average luma BD-rate of the proposed method for AI case is 0.05% compared to VTM 3.0 when PDPC is disabled on block size of 4x4.

- Algorithm description

PDPC is not applied to 4x4 block size.

Experimental results

Experiments were performed based on VTM 3.0 software under JVET common test condition.

Table 1 shows experimental results of the proposed method compared to VTM 3.0.

Table 1. Results of the proposed method over VTM 3.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.01% | 0.03% | 101% | 100% |
| Class A2 | 0.03% | 0.04% | -0.01% | 101% | 100% |
| Class B | 0.04% | -0.01% | 0.02% | 100% | 100% |
| Class C | 0.05% | 0.00% | -0.01% | 100% | 99% |
| Class E | 0.09% | 0.08% | 0.18% | 100% | 100% |
| Overall | 0.05% | 0.02% | 0.03% | 100% | 100% |
| Class D | 0.07% | -0.06% | -0.04% | 101% | 99% |
| Class F | -0.17% | -0.20% | -0.10% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.10% | 0.10% | 100% | 100% |
| Class A2 | 0.02% | -0.06% | 0.10% | 100% | 100% |
| Class B | 0.02% | 0.21% | -0.10% | 100% | 100% |
| Class C | 0.06% | -0.16% | -0.04% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 0.04% | 0.00% | 100% | 100% |
| Class D | 0.03% | -0.36% | 0.17% | 100% | 99% |
| Class F | -0.18% | -0.12% | -0.10% | 100% | 100% |