| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakesh, January 2019 | Document: JVET-M0566 |
| --- | --- |

| Title: | AHG9: Adaptive convolutional neural network loop filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hujun Yin, Rongzhen Yang, Xiaoran Fang, Shoujiang Ma, , Yue Yu, 2200 Mission College Blvd, Santa Clara, CA, 95054, USA | Tel: Email: | +1 408 410 5081 {Hujun.yin, Rongzhen.yang} @intel.com |
| Source: | Intel Corporation |  |  |

_____________________________

# Abstract

This document proposes an adaptive convolution neural network loop filter (ACNNLF) design. In this design, 3 CNN based loop filters are adaptively trained for luma and chroma respectively from the current video sequence. Each filter is a small 2 layer CNN with total 692 parameters. The encoder selects one of the three ACNNLFs for luma and chroma respectively for each CTB block during encoding. The weights of the trained set of ACNNLFs are signaled in the slice header of I frames and the index of selected ACNNLF is signaled for each CTB.

Compared with VTM-3.0-RA, the proposed ACNNLF achieves -2.37%, -1.34%, and -2.77% BD-rates for Y, U, and V, respectively, for Class A1 video sequences; -0.45%, -10.92%, and -6.19% BD-rates for Y, U, and V, respectively, for Class A2 video sequences; -0.49%, -11.29%, and -10.73% BD-rates for Y, U, and V, respectively, for Class B video sequences; and 0.12%, -3.31%, and -1.62% BD-rates for Y, U, and V, respectively, for Class C video sequences.

# Introduction

In this document, an adaptive convolution neural network loop filter (ACNNLF) design is proposed. In current VTM software, multiple in-loop filters such as deblocking filter (DBF), sample adaptive offset (SAO) and adaptive loop filter (ALF) are applied at the last stage of decoding process to improve the quality of reconstructed frames.

Convolutional neural network loop filter (CNNLF) is another type of in-loop filter that utilizes convolutional neural network (CNN) to remove compression artifacts. CNNLF has demonstrated promising performance benefits [1] [2] [3] [4] . One challenge with CNNLF is that the multi-layer CNN network used typically has a large number of parameters. It also requires a large amount of training data to achieve good performance. Therefore, it is inherently memory and computation intensive, and may lead to substantially higher codec complexity.

In [1], the number of parameters for CNNLF is reduced (4 layers, 2860 parameters) by adaptively training CNN with video data itself (on-line training). Since video content may be similar from frame to frame, relevant features in a video could be captured with a smaller number of parameters through online training. However, a 4-layer CNNLF is still quite complex compared to an ALF filter. It is desirable to further reduce the CNNLF complexity.

ALF has been extensively studied [5]. In ALF, there are typically many classes of filters. Within each class, there are a number of filters with fixed or trained weights. First a classification is applied to obtain multiple classes, which gives a partition of a set of all pixel locations. After that, a set of filters is applied for each of those classes. Therefore, the performance of ALF relies on not only how filters are designed but also how classification behaves. In VTM3.0, following ALF configuration is supported:

  - Classification 4x4

  - Luma: 5x5 and 7x7, default 7x7

Trans./
Quant.

Entropy

Coder

DBF

Input

frame

Recon.

frame

Bitstreams

Inter

Predictor

Intra

Predictor

Inv. Trans.

& Quant.

Split into

CTUs

+

-

+

+

In-loop filters

Predictor

Trans. / Quant. Coeff.

Prediction Data

Filter Ctrl. Data

SAO

ALF

ACNNLF

Figure 1. Proposed decoding scheme [2]

Chroma: 5x5

The ALF filter can be considered as a special one-layer CNN with linear activation. However, the number of filter coefficients in an ALF filter is too small to capture relevant features in the video. In order to match different video content, many ALF filters are used. Therefore, ALF makes up the deficiency of small number of filter coefficients by increasing the number of filter classes used.

As we try to further reduce the number of CNNLF parameters, we increase the number of CNNLFs to choose from to ensure the design can adapt to diverse video content without sacrificing performance. In this document, we propose an ACNNLF design:

- Small Deep-CNN loop filter with minimum number of hidden layers (2 CNN layers);

- 3 set of CNN loop filters trained online for luma and chroma respectively to better adapt to the content.

Since the number of ACNNLF filters (3) is small, it is possible to conduct exhaustive search for the optimal ACNNLF in the encoding process. The ACNNLF selection is indicated in the coded stream to the decoder. As shown in Figure 1, ACNNLF is applied after ALF in the decoding process.

# Proposed method

In this document, we propose an ACNNLF with the following properties:

- 2 layer CNN, which is the minimum possible number of layers for a deep neural network;

- 3 ACNNLFs for luma and 3 ACNNLFs for chroma adaptively trained with video data;

- Encoder selects the best ACNNLF for luma and chroma in each CTB from the trained set of ACNNLFs, to perform encoding. The weights of the trained set of ACNNLFs and index of selected ACNNLF are carried in the coded stream. The detailed signaling design is described in section 2.3;

- Decoder receives ACNNLF weights (3 sets for luma and 3 set for chroma) and the index of the selected ACNNLF and only use the selected ACNNLF to perform the filtering process for each luma and chroma block during frame reconstruction.

## Network architecture

Figure 2. ACNNLF for luma

As shown in Figure 2, the low complexity ACNNLF has 2 hidden layers, plus input and output layer. Similar to [1], packing/unpacking operations are performed at input and output. For example, if one to-be-processed luma block is Width x Height, then 2x2 subsampling is applied to generate 4 stacked subblocks of size Width/2 x Height/2, in Packing for Luma. Then two chroma blocks of the same size are stacked together to generate 6 stacked subblocks of size Width/2xHeight/2 as the filter input.

- Input & Output Image Block Size: N (default value 32). N can vary with frame size. For example, large frame size (such as 2K/4K/1080P) video may choose N=32 or 16, small frame size video may choose N=16 or 8;

- First layer CNN:  filter size L1xL1. L1 can be 1 or 3 with default value 1. ReLU (Rectified Linear Unit) is applied after the first layer CNN;

- Second layer CNN:  filter size L2xL2. L2 can be 1 or 3 or 5 with default value 3. No ReLU after second layer CNN;

- Channels after first layer CNN: M (default value 16). M can vary with frame size, 16 or 32 for large frame size (such as 2K/4K/1080P), 8 or 16 for small frame size.

Figure 3. ACNNLF for chroma

As shown in Figure 3, the chroma ACNNLF has the same input and inner structure as luma ACNNLF. The output is 2 chroma blocks.

## Training method

Online training is performed for ACNNLF. The training data comes directly from the video sequences for encoding. The training is done for each random access sequence (RAS) using N frames of video data – the current I frame and the previous N-1 frames. As shown in Figure 4, total N frames of data are collected as training data set for RAS data from the 1st frame of RAS and the previous N-1 frames (in encoding sequence) in previous RAS. In the experiments, N = 8 was used. Since the training requires frames from previous RAS, ACNNLF is disabled for RAS0.

Figure 4.Example of Training Data Set Generation

For all video frames selected for training, both the luma and chroma data of the original frames and the re-constructed frames (before ACNNLF processing) are collected as the database for online training. Each frame is partitioned into small image blocks of size NxN for training.

Multiple CNNs are trained based on the following process as shown in Figure 5:

- Start from training one ACNNLF with the architecture described in section 2.1

- Apply the first trained ACNNLF to partition the training set. The blocks yield net code rate gain are assigned to the gain set ACNNLF#1, and the blocks yield net loss are assigned to the loss set

- The loss set is used to train a new ACNNLF#2

- Apply the entire training set to the two trained ACNNLFs, re-partition the training data set to ACNNLF#1 gain set, ACNNLF#2 gain set and loss set

- Train a new ACNNLF#3 with the new loss set

- Continue the process until desired number of ACNNLFs are generated and/or training data is exhausted

- Apply the trained ACNNLFs to the entire training data set. Partition the data set into 4 subsets: subset with highest gain for ACNNLF#1, subset with highest gain for ACNNLF #2, subset with highest gain for ACNNLF#3 and subset with no gain for any ACNNLF. Further train each ACNNLF with the subset on which it has the highest gain.

Figure 5.Example of CNNLF training process for 3 CNNLFs

## Syntax design

The parameters of ACNNLF are signaled in the slice header. In RA configuration, only the I-slice has ACNNLF parameters of this RAS. The multi-level on/off control is applied at slice level and CTB level for luma and chroma components. In the slice header, syntax elements acnnlf_luma_slice_enable_flag and acnnlf_chroma_slice_enable_flag are used to indicate ACNNLF enable/disable. At CTB level, there are 2 bits indicator acnnlf_luma _ctb_idc for luma and acnnlf_chroma _ctb_idc for chroma indicating the ACNNLF supported in the current CTB.

### Sequence parameter set RBSP syntax draft

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| log2_ctu_size_minus2 | ue(v) |
| log2_min_qt_size_intra_slices_minus2 | ue(v) |
| log2_min_qt_size_inter_slices_minus2 | ue(v) |
| max_mtt_hierarchy_depth_inter_slices | ue(v) |
| max_mtt_hierarchy_depth_intra_slices | ue(v) |
| sps_acnnlf_enable_flag | u(1) |
| if ( sps_acnnlf_enable_flag ){ |  |
| log2_acnnblock_width | ue(v) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

### Slice header syntax Draft

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| slice_address | u(v) |
| slice_type | ue(v) |
| if ( sps_acnnlf_enable_flag ){ |  |
| if ( slice_type == I ) { |  |
| acnnlf_luma_params_present_flag | u(1) |
| if(acnnlf_luma_params_present_flag){ |  |
| acnnlf_luma_coeff ( ) |  |
| } |  |
| acnnlf_chroma_params_present_flag | u(1) |
| if(acnnlf_chroma_params_present_flag){ |  |
| acnnlf_chroma_coeff ( ) |  |
| } |  |
| } |  |
| acnnlf_luma _slice _enable_flag | u(1) |
| acnnlf_chroma _slice _enable_flag | u(1) |
| } |  |
| byte_alignment( ) |  |
| } |  |

### Coding tree unit syntax draft

| coding_tree_unit( ) { | Descriptor |
| --- | --- |
| xCtb = ( CtbAddrInRs % PicWidthInCtbsY ) << CtbLog2SizeY |  |
| yCtb = ( CtbAddrInRs / PicWidthInCtbsY ) << CtbLog2SizeY |  |
| if(acnnlf_luma _slice _enable_flag ){ |  |
| acnnlf_luma _ctb_idc | u(2) |
| } |  |
| if(acnnlf_chroma _slice _enable_flag ){ |  |
| acnnlf_chroma_ctb_idc | u(2) |
| } |  |
| coding_quadtree( xCtb, yCtb, CtbLog2SizeY, 0 ) |  |
| } |  |

### ACNNLF syntax draft

| acnnlf_luma_coeff ( ) { | Descriptor |
| --- | --- |
| num_luma_cnnlf | u(3) |
| num_luma_cnnlf_l1size | tu(v) |
| num_luma_cnnlf_ l1_output_channel | tu(v) |
| num_luma_cnnlf_l2size | tu(v) |
| L1_Input = 6, L1Size = num_luma_cnnlf_l1size, M = num_luma_cnnlf_ l1_output_channel, L2Size = num_luma_cnnlf_l2size, K = 4 |  |
| for( cnnIdx = 0; cnnIdx < num_luma_cnnlf; cnnIdx ++ ) |  |
| two_layers_cnnlf_coeff(L1_Input, L1Size, M, L2Size, K) |  |
| } |  |

| acnnlf_chroma_coeff ( ) { | Descriptor |
| --- | --- |
| num_chroma_cnnlf | u(3) |
| num_chroma_cnnlf_l1size | tu(v) |
| num_chroma_cnnlf_ l1_output_channel | tu(v) |
| num_chroma_cnnlf_l2size | tu(v) |
| L1_Input = 6, L1Size = num_chroma_cnnlf_l1size, M = num_chroma_cnnlf_ l1_output_channel, L2Size = num_chroma _cnnlf_l2size, K = 2 |  |
| for( cnnIdx = 0; cnnIdx < num_chroma_cnnlf; cnnIdx ++ ) |  |
| two_layers_cnnlf_coeff(L1_Input, L1Size, M, L2Size, K) |  |
| } |  |

| two_layers_cnnlf_coeff(L1_Input, L1Size, M, L2Size, K) { | Descriptor |
| --- | --- |
| for( l1Idx = 0; l1Idx < M; l1Idx++ ) { |  |
| l1_cnn_bias[l1Idx] | tu(v) |
| } |  |
| for(l1Idx = 0; l1Idx < M; l1Idx ++ ) |  |
| for( inChIdx = 0; inChIdx < L1_Input; inChIdx ++ ) |  |
| for( yIdx = 0; yIdx < L1Size; yIdx ++ ) |  |
| for( xIdx = 0; xIdx < L1Size; xIdx ++ ) |  |
| cnn_weight[l1Idx][ inChIdx][ yIdx][ xIdx] | tu(v) |
| } |  |
| for( l2Idx = 0; l2Idx < K; l2Idx++ ) |  |
| L2_cnn_bias[l2Idx] | tu(v) |
| for(l2Idx = 0; l2Idx < K; l2Idx ++ ) |  |
| for( inChIdx = 0; inChIdx < M; inChIdx ++ ) |  |
| for( yIdx = 0; yIdx < L2Size; yIdx ++ ) |  |
| for( xIdx = 0; xIdx < L2Size; xIdx ++ ) |  |
| cnn_weight[l2Idx][ inChIdx][ yIdx][ xIdx] | tu(v) |
| } |  |

acnnlf_luma_params_present_flag equal to 1 specifies that acnnlf_luma_coeff ( ) syntax structure will be present.  acnnlf_luma_params_present_flag equal to 0 specifies that the acnnlf_luma_coeff ( ) syntax structure will not be present.

acnnlf_chroma_params_present_flag equal to 1 specifies that acnnlf_chroma_coeff ( ) syntax structure  will be present. acnnlf_chroma_params_present_flag equal to 0 specifies that the acnnlf_chroma_coeff( ) sytnax structure will not be present.

# Experimental results

The following sections describe the settings and results for the experiments using VTM 3.0 following the template recommended in [6].

## Training stage

Online training based on the video sequence itself is performed for each Random Access Segment. The initial I frame and the previous N-1 frames are used for training, with N = 8 used in the experiments. QP values {22, 27, 32, 37} are used.

Table 1. ACNNLF Training Setup

| Mandatory | Information in Training Stage |  |  |
| --- | --- | --- | --- |
|  | Basic Settings: |  |  |
|  |  | learning rate: | 0.0055 |
|  |  | optimizer: | ADAM |
|  |  | batch size: | 128 |
|  |  | epoch: | 148* |
|  |  | loss function: | L1 |
|  |  | training GPU: | GTX 1080 Ti |
|  |  | training time: | 15 minutes* |
|  |  | framework: | TensorFlow |

* Measured with A1/A2 video streams, per RAS with training procedure defined in figure 2

## Inference stage

In inference stage, at encoder, all three ACNNLFs are tested to choose the best ACNNLF. At decoder, only the signaled ACNNLF for each CTB is used reconstruct the frame. Currently the ACNNLF inference module is implemented in TensorFlow in 8 bit fixed point. VTM 3.0 encoder and decoder calls TensorFlow c++ library for inference operation.

Table 2. ACNNLF Inference Stage Setup

|  | Network Details |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Total Conv. Layers | Total FC Layers | Frame-work | Param. Num | GFLOPs | Mem.P(MB) | Mem.T(MB) |
| One ACNNLF | 2 | 0 | TensorFlow module called by VTM3.0 | 692x3 (Luma) 402x3 (Chroma) | Multiplication: 264/pixel Add: 265.5/pixel | 0.0028 | 0.0448 |

## Experimental Results

The proposed ACNNLF is implemented on top of VTM-3.0. The simulations were performed following the JVET common test conditions [7].  Only RA video sequence are tested, Table 3 shows the results of the proposed scheme on Random Access for Class A1/A2, B and C.:

Table 3.  Experimental Results of the Proposed Scheme (Random Access)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.37% | -1.34% | -2.77% |  |  |
| Class A2 | -0.45% | -10.92% | -6.19% |  |  |
| Class B | -0.49% | -11.29% | -10.73% |  |  |
| Class C | 0.12% | -3.31% | -1.62% |  |  |
| Class E |  |  |  |  |  |
| Overall | -0.70% | -7.10% | -5.80% |  |  |

In the experiment, training is done separately in TensorFlow. The inference module is also implemented in TensorFlow. Encoding and decoding is done in VTM-3.0, it calls the TensorFlow module for ACNNLF filtering process. Therefore, the VTM-3.0 runtime does not include ACNNLF training time, but includes additional delay calling TensorFlow inference module.

# Conclusions

This contribution presents an ACNNLF design with 3 classes of CNN based loop filters, where each filter has only 2 CNN layers and 692 parameters. The 3 ACNNLFs are adaptively trained with video sequence data. The best ACNNLF is selected for luma and chroma respectively for each CTB at encoder and indicated to decoder in coded stream with 2 bit indicator at CTB level. Compared with VTM-3.0-RA, the proposed ACNNLF achieves -2.37%, -1.34%, and -2.77% BD-rates for Y, U, and V, respectively, for Class A1 video sequences; -0.45%, -10.92%, and -6.19% BD-rates for Y, U, and V, respectively, for Class A2 video sequences; -0.49%, -11.29%, and -10.73% BD-rates for Y, U, and V, respectively, for Class B video sequences; and 0.12%, -3.31%, and -1.62% BD-rates for Y, U, and V, respectively, for Class C video sequences. We propose further study in a Core Experiment.

# References

| [1] | C.-Y. C. T.-D. C. C.-W. H. Y.-W. H. S.-M. L. Yu-ling Hsiao, "AHG9: Convolution neural network loop filter," JVET-K0222, Media Tek Inc., 2018. |
| --- | --- |
| [2] | Z. C. Y. L. L. z. S. L. X. L. Yingbin Wang, "AHG9: Dense residual convolutional neural network based in-loop filter," JVET-L0242, Wuhan University, Tencent, 2018. |
| [3] | E. S. T. I. Tomonori Hashimoto, "AHG9: Separable convolutional neural network filter with squeeze and excitation block," JVET-K0158, Sharp Corporation, 2018. |
| [4] | Y. K. S. N. Kei Kawamura, "AHG9: Convolution neural network filter," JVET L0383, KDDI Corp., 2018. |
| [5] | K. A. C.-Y. C. Ki Zhang, "CE2: Summary report on in-loop filters," JVET-K0022, CE2 coordinators, 2018. |
| [6] | S. L. Yiming Li, "Methodology and reporting template for neural network coding tool testing," JVET-L1006, 2018. |
| [7] | J. Boyce, K. Suehring, X. Li and V. Seregin, "JVET common test conditions and software reference configuration," Joint Video Expert Team, JVET-J1010. |

# Patent rights declaration(s)

Intel may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).