| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0627 |
| --- | --- |

| Title: | Non-CE4: Supplementary results of combined solution of JVET-M0255, JVET-M0267 and JVET-M0069 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongbin Liu, Kai Zhang, Li Zhang, Jizheng Xu (ByteDance) E. Sasaki, T. Chujoh, T. Ikai (Sharp) | Tel: Email: | liuhongbin.01@bytedance.com eiichi.sasaki@sharp.co.jp ikai.tomohiro@sharp.co.jp |
| Source: | Bytedance Inc. and Sharp Corporation |  |  |

_____________________________

# Abstract

This contribution reports results of combination solution of JVET-M0255, JVET-M0267 and JVET-M0069. Three modifications on MMVD signalling are presented. First, fractional distances in merge with MVD (MMVD) are disabled for screen content videos. Second, distance index signalling in MMVD is harmonized with adaptive motion vector resolution (AMVR). Third, MMVD related syntax elements are signaled only for coding units that are coded in neither merge mode nor skip mode. Simulation results reportedly show 0.23% and 1.29% luma BD-rate saving for non-SCC sequences and SCC sequences respectively, in Random Access configuration.

# Introduction

MMVD is adopted into VVC [1] to encode the motion information more efficiently. In MMVD, one of two base merge candidates may be selected as a start point, and a MVD is further added to the start point. The MVD includes two parts, a direction index and a distance index. One of the four directions {left, right, up, down} and one of the eight distances {1/4, 1/2, 1, 2, 4, 8, 16, 32}-pel may be selected.

There are several potential issues in current MMVD design. First, fractional distances are always included in the distance table, which may be inefficient for screen content coding. Second, in entropy coding, shorter codes are assigned to shorter distances. Such design assumes short distances are preferred, which is not always true. Third, MMVD related syntax elements are signaled for coding units that are coded in merge mode or skip mode, this may be inefficient, especially in high quantization parameters cases where most CUs are coded in merge mode or skip mode.

# Proposed method

Solutions [2] [3] [4] are proposed to address the abovementioned issues. This contribution presents the results of directly combination of the three proposals.

First, fractional distances are disabled in SCC sequences [2]. A flag indicating whether fractional distances are enabled or not is signaled to the decoder for each slice. The encoder checks whether a slice is a screen content slice or not, if yes, this flag is set to be 0 and vice versa.

Second, signaling of distance is harmonized with AMVR [3]. To signal the distance in MMVD mode, the AMVR information is signaled first, followed by a MVD signaled with the specified MV resolution. When the MV resolution is 1/4-pel, MVD is chosen from {1/4-pel, ½-pel}; when the MV resolution is 1-pel, MVD is chosen from {1-pel, 2-pel}; when the MV resolution is 4-pel, MVD is chosen from {4-pel, 8-pel, 16-pel}. MVD is still coded by a truncated unary code.

Third, MMVD related syntax elements are signaled only for coding units that are coded in neither merge mode nor skip mode [4].

# Experimental results

The combined solution were implemented on top of reference software VTM-3.0 [1], and simulations were performed following the common test conditions [5]. The results are shown in Table 1.

Table 1: Coding performance of proposed method when CPR is on.

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.32% | -0.27% | -0.27% | 102% | 101% |  |  |  |  |  |
| Class A2 | -0.23% | 0.02% | 0.10% | 104% | 101% |  |  |  |  |  |
| Class B | -0.21% | 0.03% | -0.08% | 102% | 101% |  |  |  |  |  |
| Class C | -0.19% | 0.00% | -0.02% | 101% | 97% | -0.25% | -0.04% | 0.05% | 102% | 105% |
| Class E |  |  |  |  |  | -0.21% | -0.27% | -0.48% | 105% | 104% |
| Overall | -0.23% | -0.04% | -0.06% | 102% | 100% |  |  |  |  |  |
| Class D | -0.15% | 0.13% | 0.13% | 104% | 101% | -0.20% | 0.13% | -0.29% | 99% | 98% |
| Class F | -0.13% | -0.16% | -0.18% | 100% | 99% | -0.28% | -0.46% | -0.10% | 101% | 102% |
| Class SCC | -1.29% | -1.13% | -1.19% | 98% | 101% | -2.38% | -2.25% | -2.32% | 100% | 102% |

# Conclusions

This contribution reports results of combination solution of JVET-M0255, JVET-M0267 and JVET-M0069, and it can achieve 0.23% and 1.29% luma BD-rate saving for non-SCC sequences and SCC sequences respectively, in Random Access configuration. It is recommended to adopt this proposal into VVC.

# References

- VTM-3.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-3.0

- “AHG11: MMVD without Fractional Distances for SCC”, H. Liu, L. Zhang, K. Zhang etc., Doc. JVET-M0255, Jan. 2019.

- “Non-CE4: Harmonization of MMVD and AMVR”, K. Zhang, L. Zhang, H. Liu etc., Doc. JVET-M0267, Jan. 2019.

- “Non-CE4: Syntax change of MMVD”, E. Sasaki, T. Chujoh, T. Ikai, Doc. JVET-M0069, Jan. 2019.

- “JVET common test conditions and software reference configurations”, J. Boyce, K. Suehring, X. Li, V. Seregin, Doc. JVET-J1010, April 2018.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).