| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0640 |
| --- | --- |

| Title: | CE12-related: in-loop luma reshaping with approximate inverse mapping function |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Edouard François, Christophe Chevance, Franck Hiron | Email: | edouard.francois@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution relates to the in-loop luma reshaping approach proposed in contributions JVET-L0246 and JVET-L0247. In-loop luma reshaping requires at the decoder two basic sample mapping operations: forward reshaping of the prediction signal, inverse reshaping of the reconstructed signal. In its current implementation, the forward mapping can be achieved on-the-fly per sample using simple operations, whereas inverse mapping requires more complex operations. In this contribution it is proposed to approximate the inverse mapping function using a piece-wise linear (PWL) model of limited size, each piece being defined per luma interval of fixed length, power of 2, so that simple shifting can be used to identify the interval containing the sample value to map. It is reported that for most of the natural sequences, the gain of in-loop reshaping an be preserved using the proposed approximation with a PWL model size of 32 elements. For class E, F and some HDR content, larger PWL model size is required.

# Introduction

In JVET-L0246 and JVET-L0247 the in-loop luma reshaper requires to perform a forward reshaping, then an inverse reshaping.

The forward mapping function implementation is based on a scaling table, noted here scale[ k ], k=0 to N-1, N being a power of 2 (typically 16 or 32). Each scale applies to a range of luma values. The range R is fixed and power of 2 (R = 2K), and is equal to the full luma range (noted rangeY) divided by N. instance, for a 10-bit luma signal, and for N=16, R = 1024/N = 64 = 26 (K = 6).

The mapping function FwdMap is built as follows as follows:

- FwdMap[ 0 ] = 0

- Then values at each k*R index, for k=0 to N, is computed, as follows:

  - FwdMap[ (k+1)*R ] = FwdMap[ k*R ] + scale[ k ]

- Then the intermediate values at indexes x=( k*R+1 ) to ( (k+1)*R-1 ), are linearly interpolated from their surrounding values FwdMap[ (k+1)*R ] and FwdMap[ k*R ]

The process is illustrated in Figure 1. Each scale is associated to a segment of the piece-wise linear mapping function. Each segment is specified on a luma range of same length (R).

The advantage of using equi-length intervals, of length R, is that the mapping of a sample value Y can be easily performed on-the-fly by a simple access to look-up-tables of limited size (size N), using an index computed from the value Y shifting by K bits. This process is conceptually as follows:

- Computation of the index k = Y / R = Y >> K

- Ymap = scale[ k ] * Y / R + A[ k ]

where A is a pre-built offset look-up-table of size N.

Figure 1. illustration of the construction of the forward mapping function.

# Issue of the inverse mapping process

The inverse mapping function InvMap is built as the inverse of the forward mapping function. It is mathematically built as the symmetric function of FwdMap related to the line defined by y = x, as illustrated in Figure 2. The main issue with the inverse mapping is that it cannot be applied on-the-fly as easily as the forward mapping. Indeed, even if the inverse mapping function remains piece-wise linear, the identification of the index of the piece which a value Y belongs to is not straightforward. Contrarily to the forward mapping case, where a simple shifting operation is required, here this is more complicated because the indexes of the pieces of the inverse mapping function are not multiple of a pre-defined length that is power of 2. This is illustrated in Figure 2 where the indexes of the pieces of the inverse mapping functions (idx0 to idx4) are not equi-distantly distributed along the horizontal axis since by construction idxk = FwpMap[ k*R ].

Figure 2. illustration of the construction of the inverse mapping function.

# Proposed simplification

The proposal aims at simplifying the inverse mapping process, by approximating the inverse mapping function using a piece-wise-linear (PWL) function with equi-length intervals of length Rinv power of 2 (Rinv=2Kinv), and a limited number of pieces M. Hence, it is possible to perform the inverse mapping on-the-fly by a simple process, as described above for the forward mapping.

- Computation of the index k = Y / Rinv = Y >> Kinv

- Ymap = invScale[ k ] * Y + Ainv[ k ]

The approximate inverse function, noted InvMapApprox, is specified by the table of inverse scale values invScale[ k ], k = 0 to (M-1), as illustrated in Figure 3. The “real” inverse mapping function, InvMap, is indicated in dashed grey line, while the approximated version, InvMapApprox, is in full line. InvMapApprox is linear per piece of length Rinv. The slope of each piece k corresponds to invScale[ k ] (just shown for pieces 0, 2, 8 in the figure).

Figure 3. approximation of the inverse mapping function.

The approximate inverse function InvMapApprox can be initialized at the encoder and decoder as follows:

- For k=0 to M

  - InvMapApprox[ k*Rinv ] = InvMap[ k*Rinv ]

- Then the intermediate values at indexes x=( k*Rinv + 1 ) to ( (k+1)*Rinv – 1 ), are linearly interpolated from their surrounding values InvMapApprox[ (k + 1)*Rinv ] and InvMapApprox[ k*Rinv ]

The model parameters (InvMapApprox[ k*Rinv ], for k=0 to M) can be further improved at the encoder side in order to minimize the mean square error over the luma range resulting from forward mapping then inverse mapping processes:

minY=0rangeY-1Y-InvMapApprox[ FwdMap y  ]2

The slope table invScale related to the approximate inverse mapping PWL model is defined as follows:

- for k=0 to M – 1,

  - invScale[ k ] = InvMapApprox[ (k+1)*Rinv ] –  InvMapApprox [ k*Rinv ]

The inverse mapping PWL parameters can be either signaled or derived at the decoder as explained above.

A proposed syntax is provided in the table below. Syntax elements related to the inverse mapping model are highlighted in grey.

| reshaper_model() { | descriptor |
| --- | --- |
| slice_reshaper_model_present_flag | u(1) |
| if (slice_reshaper_model_present_flag) { |  |
| reshaper_model_min_bin_idx | u(v) |
| reshaper_model_max_bin_idx | u(v) |
| reshaper_model_bin_delta_abs_cw_prec_minus1 | u(v) |
| for (i= reshaper_model_min_bin_idx; i<= reshaper_model_max_bin_idx; i++) { |  |
| reshape_model_abs_CW | u(v) |
| reshape_model_sign_CW | u(1) |
| reshapeModelScale[ i ] = reshape_model_abs_CW * (1 – 2* reshape_model_sign_CW) |  |
| } |  |
| reshaper_model_inv_log2_size | u(2) |
| for (i= 0; i<= reshaperModelInvSize ; i++) { |  |
| reshape_model_abs_CW | u(v) |
| reshape_model_sign_CW | u(1) |
| reshapeModelInvScale[ i ] = reshape_model_abs_CW * (1 – 2* reshape_model_sign_CW) |  |
| } |  |
| } |  |
| } |  |

# Results

The results are depicted in the following tables, compared to VTM3.0, for different settings of the inverse PWL model size.

## Inverse scaling table size 32

The following table correspond to an inverse mapping PWL table of 32 pieces. The design simplification results in limited loss for natural SDR videos, but the impact is not good for classes E and F in LDB configuration. It seems that for this content with rather low textures, any slight change in quality has a big impact on the PSNR BD-rate. For HDR, the benefit of reshaping is impacted by using the approximate inverse function with 32 pieces. For wPSNRY metric, the most problematic content is SunRise where a loss is observed whereas for all other sequences a gain is observed.

Further evaluations were conducted to check until which PWL size it is necessary to go to perform better with such content. This is reported in the next subsections.

Table 1. BD-rate variations on SDR content

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.57% | 0.21% | -1.26% | 100% | 102% |
| Class A2 | -1.67% | 1.27% | 1.36% | 100% | 100% |
| Class B | -0.61% | 1.29% | 0.91% | 99% | 100% |
| Class C | -0.18% | 2.56% | 2.62% | 103% | 96% |
| Class E | 0.03% | 2.36% | 2.33% | 99% | 99% |
| Overall | -0.75% | 1.56% | 1.24% | 100% | 99% |
| Class D | -0.08% | 1.88% | 1.64% | 100% | 98% |
| Class F (optional) | -0.58% | 1.81% | 1.80% | 101% | 98% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.56% | 2.81% | 2.81% | 101% | 99% |
| Class A2 | -0.68% | 2.69% | 2.80% | 99% | 99% |
| Class B | -1.20% | 0.26% | -0.21% | 98% | 98% |
| Class C | -1.02% | 1.14% | 1.30% | 99% | 98% |
| Overall | -0.92% | 1.49% | 1.40% | 99% | 98% |
| Class D | -0.63% | 2.23% | 2.20% | 100% | 104% |
| Class F (optional) | -0.90% | -0.05% | 0.05% | 102% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.32% | 2.23% | 2.02% | 102% | 98% |
| Class C | 0.02% | 2.22% | 2.13% | 97% | 96% |
| Class E | 5.85% | 0.20% | -0.52% | 99% | 100% |
| Overall | 1.34% | 1.72% | 1.42% | 100% | 98% |
| Class D | -0.31% | 2.64% | 3.01% | 99% | 98% |
| Class F (optional) | 1.31% | 2.31% | 2.42% | 99% | 98% |

Table 2. BD-rate variations on HDR content

|  | All Intra: BD-rate |  |  | Random Access: BD-rate |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | DE100 | PSNRL100 | wPsnrY | DE100 | PSNRL100 | wPsnrY |
| Market | 0.2% | -1.9% | -1.5% | 1.7% | -0.3% | -0.5% |
| SunRise | 3.1% | -1.4% | -0.9% | 5.5% | 4.4% | 1.7% |
| ShowGirl2 | 0.9% | -1.8% | -1.7% | 2.0% | -0.7% | -1.0% |
| BalloonFestival | 2.2% | -2.7% | -2.2% | 3.6% | 0.9% | -1.8% |
| Hurdles | 0.6% | -0.5% | -0.1% | 3.0% | -1.8% | -1.4% |
| Starting | -0.9% | -2.2% | -1.7% | 1.9% | 0.0% | -0.7% |
| Cosmos1 | 0.7% | -2.3% | -2.7% | 2.7% | 0.5% | -2.1% |
| Average HDR-B | 1.0% | -1.8% | -1.5% | 2.9% | 0.4% | -0.8% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 101% |  |  | 98% |  |  |

## Additional results with larger Inverse scaling table size

For sizes large than 64, the approximate inverse mapping PWL parameters are not coded but directly derived at the decoder side from the inverse mapping LUT computed from the forward mapping LUT, as the approximation error becomes small enough. Only the model size is signaled.

HDR content

For HDR, we focus on the RA configuration, and report only the wPSNY BD-rate metric. Full results are in attached excel files. The following sequences take benefit from an increase of the table size: Sunrise, BalloonFestival, Starting. Sunrise is the most critical sequence since it is necessary to use a table of 512 elements to get gain.

Table 2. BD-rate wPSNRY variations on HDR content for various inverse mapping PWL sizes.

| Inv PWL size | 32 | 64 | 128 | 256 | 512 |
| --- | --- | --- | --- | --- | --- |
| Market | -0.5% | -0.6% | -0.7% | -0.9% | -1.0% |
| SunRise | 1.7% | 1.3% | 1.1% | 0.4% | -0.5% |
| ShowGirl2 | -1.0% | -0.9% | -1.0% | -1.3% | -1.3% |
| BalloonFestival | -1.8% | -1.9% | -2.0% | -2.2% | -2.5% |
| Hurdles | -1.4% | -1.2% | -0.9% | -1.3% | -1.6% |
| Starting | -0.7% | -0.8% | -1.3% | -1.5% | -1.9% |
| Cosmos1 | -2.1% | -2.1% | -2.2% | -2.2% | -2.3% |
| Average HDR-B | -0.8% | -0.9% | -1.0% | -1.3% | -1.6% |

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).