| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0776-v3 |
| --- | --- |

| Title: | A summary of JVET-M contributions on general HLS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang 10180 Telesis Court San Diego, CA, 92121, USA | Tel: Email: | +1-908-803-3888 yekui.wang@huawei.com |
| Source: | Huawei Technologies. Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution provides a summary of input contributions submitted to this JVET meeting on general high-level syntax. A list of design questions is provided to help structure the review of these documents. For efficient addressing of these documents, it is suggested that these documents are firstly reviewed by a BoG, wherein BoG-level decisions should be made as much as possible. The BoG-level decisions or recommendations are then reviewed at JVET track or plenary level.

# The list of questions

## General high-level syntax and NAL unit header

- To have variable-length NAL unit header or not? (M0120, M0161, M0520)

  - Is the guideline for NAL unit header design as proposed in M0120 agreeable?

- How many IRAP NAL unit types?

  - 3: IDR_W_RADL, IDR_N_LP, CRA_NUT? (M0101, M0152)

  - Should CRA pictures be required to have at least one RASL picture? (M0131)

  - Should CRA pictures be required to have at least one associated leading picture (RASL or RADL)? (M0152)

  - Should CRA pictures be disallowed to have any associated RADL picture? (M0153)

  - Should RASL pictures allowed to be associated only with CRA pictures, not any other types of IRAP pictures? (M0153)

  - Should IDR pictures contain syntax elements for reference picture management? (M0388 option 1)

  - To specify only one VLC NAL unit type and to have a picture header, and in picture header there is syntax element that specifies the type of the picture (IDR, CRA, STSA, Trail, RADL, RASL)? (M0388 option 2)

- To have a cvs_start_flag in the NAL unit header? (M0152)

- To specify an external-means provided flag, e.g., named HandleCraAsCvsStartFlag, for CRA pictures? (M0152)

  - When HandleCraAsCvsStartFlag is equal to 0, should the CRA picture be required to have at least one associated leading picture (RASL or RADL)? (M0152)

  - When HandleCraAsCvsStartFlag is equal to 1, should the CRA picture be disallowed to have any leading picture (RASL or RADL)? (M0152)

- How many NAL unit types for leading pictures (i.e., RASL and RADL)? (M0101)

  - 2?

  - 1? (M0153)

  - 0? (M0131)

- To signal random access type and number of leading pictures in the NAL unit header? (M0161)

- To have a NAL unit type for STSA? (M0101)

- To not have a NAL unit type for TSA? (M0101)

- To adopt the non-VCL NAL unit types as proposed in M0101?

- To have nuh_component_type in the NAL unit header for indication of the component type such as YUV, RGB, depth map, alpha map, 3D geometry map, occupancy map, etc.? (M0156)

- To have nuh_pic_id in the NAL unit header, which has the same value with the 8-bit LSB values of POC, for AU detection purpose? (M0157)

- To signal GDR in NAL unit header? (M0161, M0529)

  - If yes, to signal the recovery point and the refreshed region flag in the NAL unit header? (M0161)

  - If yes, to define a GDR NAL unit and signal the recovery point in the GDR NAL unit payload? (M0529)

- To adopt the general decoding process text as proposed in M0101?

- To add HEVC-like sections 7.4.2.3 to 7.4.2.4.5 on NAL unit order and AU boundary detection as proposed in M0101?

- To have parameter set extension mechanism as in HEVC? (M0101)

- To adopt the profile level syntax as in M0101 as a starting point?

  - To have a tier flag? (M0101)

  - To have profile_space? (M0101)

  - To have the following syntax optimization?

When maxNumSubLayersMinus1 is equal to 4, the following piece in the profile_level( ) syntax structure causes insertion of one byte of reserved bits redundantly:

| if( maxNumSubLayersMinus1 > 0 ) |  |
| --- | --- |
| for( i = maxNumSubLayersMinus1; i < 8; i++ ) |  |
| reserved_zero_2bits[ i ] | u(2) |

The redundant one reserved byte could be avoided with the following syntax:

| if( maxNumSubLayersMinus1 != 0 && maxNumSubLayersMinus1 != 4 ) |  |
| --- | --- |
| for( i = maxNumSubLayersMinus1 % 4; i < 4; i++ ) |  |
| reserved_zero_2bits[ i ] | u(2) |

- To adopt the following semantics of general_non_packed_constraint_flag? [M0101]

general_non_packed_constraint_flag equal to 1 indicates that the cropped output pictures of the decoded bitstream are suitable to be displayed. general_non_packed_constraint_flag equal to 0 indicates that the cropped output pictures of the decoded bitstream may require further processing to be suitable for displaying. [Ed. Check whether there is a better name for this flag per the updated semantics.]

NOTE 2 – Examples for content that requires additional processing before displaying are bitstreams containing one or more frame packing arrangement SEI messages, equirectangular projection SEI messages, or cubemap projection SEI messages. [Ed. This NOTE should be updated at a later phase of VVC standardization based on which SEI messages are adopted into the VVC design.]

Or should individual constraint flags for different packing purposes (e.g. frame packing, projected omnidirectional video, fisheye omnidirectional video) be used?

- To have a DPS as proposed in M0101?

  - Whether decoding parameter set should rather be a "bitstream header" NAL unit type, in which case the syntax would not include the parameter set ID and the standard would not have the activation process of the decoding parameter set?

- To adopt the HEVC-like sub-bitstream extraction process as proposed in M0101?

- Should the following as proposed in M0101 agreed as a guideline for adding information beyond the NAL unit header for use by middle-boxes?

If a need for any other additional information arises, it should be considered whether exposing such additional information to middle-boxes, file writers, or other such entities in an easily accessible manner is essential for any use cases. If so and if the NAL unit header is not considered to provide sufficient extension space for such additional information (or a part thereof), one approach could be to include such additional information into the tile group header such that:

- The information is fixed-length coded.

- The information is in such an early, fixed bit position within the tile group header that is not preceded by any entropy-coded syntax element and that is not preceded by start code emulation prevention bytes under any circumstances.

- Correct decoding of the information does not require activation or decoding of any parameter sets.

## Reference picture management

- To use a mechanism based on direct RPL signalling and derivation? (M0128, M0154)

  - If yes, to explicitly signal reference pictures to be marked as "unused for reference"? (M0154)

  - If yes, to keep the LTRP functionality but remove the naming? (M0128 discussion)

- To use a mechanism based RPS? (M0378)

  - If yes, to keep the LTRP functionality? (M0378)

## Picture header and HPS

- To add HPS or picture header NAL unit with a reference-able ID? (M0132, M0451 proposal 2)

  - If yes, to contain only ALF parameters? (M0132)

  - If yes, to add the ID in the tile group header (M0132) or to replace the PPS ID in the tile group header (M0451 proposal 2)?

- To allow PPS contain tile group header parameters as follows? (M0260 option 1)

- Tile group header parameters are divided into subsets that are selectively included either in the PPS or in the tile group header.

- Different PPSs may be activated by different tile groups of the same picture, but besides the tile group header parameters, all other parameters are required to have the same values in all PPSs activated for the same picture.

- To allow PPS contain ALF parameters and to add picture header NAL unit without a reference-able ID for containing other tile group header parameters as follows? (M0260 option 2)

- A picture header VCL NAL unit is present and precedes all tile group VCL NAL units of the picture.

- The tile group header parameters besides ALF parameters may be included in the picture header. If not present in the picture header, they are included in the tile group header(s).

- The ALF parameters may be included in the PPS, picture header, or tile group header.

- Different PPSs may be activated by different tile groups of the same picture, but besides the ALF parameters, all other parameters are required to have the same values in all PPSs activated for the same picture.

- To add picture header NAL unit without a reference-able ID for containing tile group header parameters as follows? (M0377)

  - Put syntax elements common for multiple tile groups in a single picture into common_tile_group_header( ).

  - Add an SPS flag to control whether common_tile_group_header( ) will be carried in a picture header NAL unit sent in every AU in the CVS or carried in the tile group header of each tile group NAL unit in the CVS.

## Misc. general HLS topics

- To allow parsing dependency between parameter sets? (M0133)

- To either add "_minus1" to the end of sps_ref_wraparound_offset, or to remove sps_ref_wraparound_enabled_flag and the value of sps_ref_wraparound_offset to be equal 0? If yes which option to take? (M0451 proposal 1)

- To split tile_group_type into multiple syntax elements as one of the following two ways? (M0386)

| current_picture_only_reference_flag | u(1) |
| --- | --- |
| current_picture_inter_reference_flag | u(1) |
| if( current_picture_only_reference_flag == 0 ¦¦ current_picture_inter_reference_flag == 1) |  |
| num_ref_pic_lists_active_minus1 | u(1) |

or

| current_picture_only_reference_flag | u(1) |
| --- | --- |
| if( current_picture_only_reference_flag ) |  |
| current_picture_inter_reference_flag | u(1) |
| else |  |
| num_ref_pic_lists_active_minus1 | u(1) |

- To create an AHG to further study the extraction of multiple frame rates from a VVC bit-stream with the following design goals? (M0579)

- The VVC design should allow for the encapsulation of two or more versions of a coded video sequence at different frame rates.

- The VVC design should support the efficient compression of coded video sequences that do not contain temporal judder and/or temporal strobing at all frame rate representations.

- The VVC design should support a sub-bitstream extraction process that supports extracting conforming bitstreams at different frame rates from a single bitstream where the frame rate representations remaining in the extracted bitstream do not contain temporal judder an/or temporal aliasing.

- The VVC design should allow for the encapsulation of two or more versions of a coded video sequence at different frame rates and at different level and/or profile points.

## Interoperability and capability points definition and signalling

- To adopte the update to interoperability point syntax as proposed in M0451?

# The list of contributions

### General high-level syntax and parameter sets (21)

#### General high-level syntax and NAL unit header (11)

JVET-M0101 AHG17: On VVC HLS [R. Skupin, K. Sühring, Y. Sanchez (HHI), M. M. Hannuksela, K. Kammachi-Sreedhar (Nokia), Y.-K. Wang, Hendry (Huawei), S. Wenger, B. Choi (Tencent), S. Deshpande (Sharp)]

JVET-M0120 Proposed NAL Unit Header Design Principles [S. Wenger, B. Choi, S. Liu (Tencent)]

JVET-M0131 AHG17: On NAL unit types for IRAP pictures and leading pictures [Y.-K. Wang, Hendry (Huawei)]

JVET-M0152 AHG17: On random access point for VVC [B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-M0153 AHG17: On leading picture for VVC [B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-M0156 AHG17: On component type indication for VVC [B. Choi, S. Wenger, S. Liu (Tencent)] [late] [miss]

JVET-M0157 AHG17: On picture order count for VVC [B. Choi, S. Wenger, S. Liu (Tencent)] [late] [miss]

JVET-M0161 AHG17: Signalling random access properties in the NAL unit header [L. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-M0520 AHG17: On NAL unit header design for VVC [S. Wenger, B. Choi, S. Liu (Tencent)]

JVET-M0529 AHG14: Normative Recovery Point Indication [M. Pettersson, R.Sjöberg, M. Damghanian (Ericsson)]

JVET-M0537 AHG17: On tile group signaling in NAL unit header and as non-VCL NAL unit [E. Thomas, A. Gabriel (TNO)] [late] [miss]

#### Reference picture management (3)

JVET-M0128 AHG17: On reference picture management for VVC [Y.-K. Wang, Hendry (Huawei), S. Deshpande (Sharp), M. M. Hannusela (Nokia), G. Ryu, W. Choi (Samsung), X. Wang, Y.-W. Chen (Kawi), L. Zhang (Bytedance), P. Wu, M. Li (ZTE), S.-H. Kim (LG), J. Boyce (Intel), A. M. Tourapis, D. Singer (Apple), F. Edouard, P. Andrivon (Technicolor), Y.-W. Huang, C.-W. Hsu, C.-Y. Chen, T.-D. Chuang, L. Chen (MediaTek), K. Kawamura (KDDI), Y.-C. Sun, J. Lou (Alibaba)]

JVET-M0154 AHG17: On decoded picture buffer management for VVC [B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-M0378 AHG17: RPS for VVC [R.Sjöberg, M. Damghanian, M. Pettersson (Ericsson)] [late]

#### Picture header and header parameter set (HPS) (4)

JVET-M0132 AHG17: On header parameter set (HPS) [Y.-K. Wang, Hendry, J. Chen (Huawei)]

JVET-M0260 AHG17: Carriage of tile group header parameters in higher level structures [M. M. Hannuksela (Nokia)]

JVET-M0377 AHG17: Picture header NAL unit type [R.Sjöberg, M. Damghanian, M. Pettersson (Ericsson)]

JVET-M0415 AHG17: Comments on High-Level Syntax of VVC [S. Deshpande (Sharp)]

#### Miscellaneous general HLS topics (3)

JVET-M0133 AHG17: On parsing dependency between parameter sets [Y.-K. Wang (Huawei), J. Boyce (Intel)]

JVET-M0386 AHG17: On slice_type (tile_group_type) [K. Sühring, Y. Sanchez, R. Skupin (HHI)]

JVET-M0579 On Frame Rate Support and Extraction in VVC [A. Segall, S. Deshpande (Sharp Labs of America), M. Hannuksela (Nokia)]

### Interoperability and capability points definition and signalling (1)

JVET-M0451 Update to interoperability point syntax [J. Boyce (Intel)]