| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0782-v1 |
| --- | --- |

| Title: | Report of BoG on tiles and WPP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang 10180 Telesis Court San Diego, CA, 92121, USA Miska M. Hannuksela Hatanpaan valtatie 30, Tampere, Finland | Tel: Email: | +1-908-803-3888 yekui.wang@huawei.com miska.hannuksela@nokia.com |
| Source: | BoG coordinators |  |  |

_____________________________

# Abstract

This contribution provides the report of the BoG on tiles and WPP.

# The list of questions

- To allow tiling with tile size unit less than CTU size?

It was commented that in addition to the issues mentioned in M0527, this design also cause issues with line buffer, as well as in VDPU rate.

It was commented that a benefit of the design is enable turning off loop filtering across CMP faces. It was suggested that enabling loop filtering turning off within a tile would be a lighter solution than having this design. It was commented that there is ongoing CE on this. It was suggested that these two topics should probably decided together. The BoG recommended that this is to be revisited in a larger group jointly with the loop filtering activity.

  - Yes (M0066, cross check in M0423)

    - If yes, to do the signalling optimizations in M0376?

    - If yes, to do the updates in M0459?

  - No (M0527)

- Tile grouping

  - To have rectangular tile grouping?

    - Yes? (M0121, M0130, M0160, M0209, M0416, M0261 option 2 without MCTS)

Agreed that rectangular tile grouping is a desirable feature. Proponents and interested parties were suggested to work offline to harmonize the syntax and semantics, together with the offline activity below for supporting of both rectangular tile groups and tile raster scan ordered tile groups.

      - If yes, to further allow wrap-around rectangular tile grouping? (M0209)

It was commented that this is a special case. It'd be good to see how much saving this can actually bring. For further study.

      - If yes, where to signal which tiles are included in a tile group?

For applications needing MCTS or sub-picture sequence, it would make more sense to have the information signalled in the PPS.

It was commented that for real-time ultralow latency applications with sub-picture-interval delay, because the partitioning of a picture into tile groups needs to be determined when encoding of the picture, it would make more sense to signal the information of which tiles are included in a tile group can be signalled in tile group headers.

        - Only in the PPS? (M0121, M0261, M0416)

        - In either PPS or tile group header but not both? (M0130)

        - Only in the tile group header? (M0160)

  - To keep tile grouping with tiles within a tile group in tile raster scan order?

Three reasons mentioned for keeping this. The first is to have a fall back, in case the rectangular tile grouping gets not easily usable. The second is to provide a possibility to use only this in a profile where rectangular tile grouping is not necessary. The third reason is that this seems better for ultralow delay applications.

It was commented that in ultralow delay applications, a picture is anyway not displayed before it is completed decoded. Therefore for this sub-picture based or decoding unit based behavior, a sub-picture based HRD behavior may need to be specified like in HEVC. It was counter argued from encoder point of view, allowing sending VCL NAL units of a partially encoded picture would still be desirable and that aspect does not need a sub-picture based HRD model to work.

For 2a and 2b together, there are two options:

i) Rectangular tile grouping only, allowing signalling of tile group information in either PPS or tile group header but not both

ii) (Rectangular tile grouping only with tile group information in PPS only) or (tile grouping with tiles within a tile group in tile raster scan order with tile grouping in tile group header only), the switching flag in the PPS. This option was preferred by the BoG. Interested parties were suggested to create a text, including harmonization of the syntax and semantics for rectangular tile grouping. To be coordinated by Sachin and Hendry. Revisit when the text is available.

A separate topic: for rectangular tiling group, when both tile configuration and tile group configuration are signalled in the PPS, it seems that it'd be more beneficial to signal the tile group configuration before the tile configuration.

    - Yes (M0160, M0261 options 1 and 2 without MCTS)

    - No (M0121, M0130, M0209, M0416)

  - How to signal tile group address in the tile group header?

For rectangular tile grouping, have one flag in the PPS to determine whether there is a mapping in the PPS between tile group indices and tile group IDs, and signal the tile group ID in the tile group header. If the PPS flag is equal to 0, tile group ID in the tile group header is tile group index.

The above was agreed, pending on review of a harmonized text. Proponents and interested parties were suggested to work offline to harmonize the syntax and semantics. To be coordinated by Sachin.

    - To use tile index?

This seems still good for tile raster scan ordered tile grouping, which is not expected to be used in MCTS environment thus no MCTS extraction is expected, unless rectangular tile grouping is not supported.

    - To (allow) explicit signalling of mapping between tile IDs and tile indices and signaliling of tile ID in tile group header? (M0134, M0261, M0430)

    - To use tile group ID? (M0155, M0373, M0416)

      - To signal a mapping between tile group IDs and tile group indices in the PPS? (M0416)

      - To also signal tile group ID in the NAL unit header? (M0155, M0536)

Without being in the context of the parameter sets, having this in the NAL unit header seems not useful. This was moved to the context of NAL unit header designs as part of the general HLS BoG.

- Flexible tiling for ROI and other rectangular region based picture access (e.g., 360 video)

Use cases for flexible tiling: 360 video viewport-dependent delivery optimization, multi-party video conferencing, load balancing for 360 projected video, V-PCC patching where each patch is probably good to be one tile.

The first two are based on merging multiple pieces into one bitstream for decoding by a single decoder.

If flexible tiling is not supported at all, VVC would have less capability than HEVC for 360 video viewport-dependent delivery optimization etc. due to that in VVC it is not allowed for one tile to be split into multiple VCL NAL units.

The above use cases, expect for the one about load balancing, are not for improving parallel processing.

Flexible tiling is for further study.

  - To have flexible tiling through hierarchical tiling syntax or explicit tile size (in units of CTU sizes) signalling? (M0123, M0129, M0209, M0374, M0530, JVET-L0394)

    - If yes, to have a two-level of hierarchical design?

      - Size for each tile group explicitly signalled + partitioning of tile groups into tiles rows and columns? (M0123)

      - Level 1 tile rows and columns + uniformly partitioning of level 1 tiles into level 2 tile columns and rows? (M0129)

      - Tiles are either uniform-spaced as currently, or the tile size for each tile explicitly signalled? (M0374)

      - Partitioning of pictures into tile set rows and columns + partitioning of tile sets into tile columns and rows? (M0530)

      - To allow cross-tile prediction for tiles within the same tile group, which essentially merges tiles (M0209)

  - To have flexible tiling through sub-picture sequence + allowing tile groups in one picture to possibly refer to different PPSs thus essentially no change to the tiling syntax? (M0261 option 3, M0536)

This approach would not work for use cases not requiring MCTSs or sub-picture sequences. But on the other hand, it seems that the first use case (i.e., 360 video viewport-dependent delivery optimization) is the strongest.

- On MCTS / sub-picture sequence

  - To indicate independently decodable rectangular region(s) in a sequence of pictures in MCTS-related SEI message(s), or in some parameter set (SPS, PPS, etc.)?

The BoG agreed that, if the signalling is not going to affect the decoding process, it does not make much difference between putting the information in SEI message(s) and VUI, otherwise, the information should be included in a non-VUI part of a parameter set.

    - In MCTS-related SEI message(s) similarly as in HEVC?

    - For the MCTS-application specific tiling in one type of parameter set?

      - In PPS? (M0261, M0416)

      - In sequence-level tile group parameter set? (STGPS), or maybe better to rename it as sub-picture parameter set (SPPS) (M0536)?

      - In SPS?

This aspect is for further study.

    - What information should be included?

      - Sub-picture sizes only (M0261)

      - Sequence-level tile group set boundary IDs (M0536)

  - To require that each MCTS / sub-picture to contain only one tile group? (M0261)

For ultralow delay application, it'd be good to allow for more than one tile group in one MCTS / sub-picture.

The BoG agreed to allow for more than one tile raster scan ordered tile group in one MCTS / sub-picture, but if the tile grouping mode is rectangular tile group, it is required that each MCTS / sub-picture contains only one tile group.

  - To specify conformance (level, HRD) or not for MCTS / sub-picture sequence?

What is the benefit of doing this? It helps when extracting a 360 video viewport and decode just that. Even more importantly, it helps in merging pieces into one conforming bitstream. The BoG agreed that this would be a useful thing to do.

The latest HEVC specification specifies, as part of the semantics of an SEI message, the MCTS sub-bitstream extraction process and requires the extracted sub-bitstream to be conforming bitstream. The BoG recommended to discuss in a larger group when such way of defining conformance is good for VVC.

    - Yes? (M0445)

  - To (allow) treat boundaries of MCTS / sub-picture sequences as picture boundaries (i.e., to do padding)?

Allowing this is expected to help in coding efficiency. However, it would impose some burden for hardware implementations. Tests showing coding efficiency gain numbers and an analysis on hardware implementation complexity are needed for making a decision on this. Note that M0445 provides a software implementation that could be used as the basis and anchor for such coding efficiency comparison.

Interested parties were requested to work offline to prepare the test conditions. Revisit after that is available. To be coordinated by M. Coban.

    - If to use MCTS-related SEI messages, enable treating MCTS boundaries by

      - Allowing treating tile boundaries as picture boundaries? (M0136, M0530)

      - Allowing treating tile group boundaries as picture boundaries? (M0136)

    - If to specify sub-picture sequences, enable treating MCTS boundaries by

      - Default for all sub-picture sequence boundaries? (M0261)

  - To define and include an 8-bit sequence-level tile group set ID (maybe better to rename as sub-picture ID) in NAL unit header? (M0536)

It was commented that it'd be useful for MCTS / sub-picture sequence extraction. Why not put it in the beginning of the tile group header? The tile group ID in the tile group header provides the same information for rectangular tile groups.

What is the middle-box use case for sub-picture ID based extraction / adaptation?

There was no consensus to take an action on this.

  - To adopt the software implementation in M0445 into VTM and agree to evaluate future contribution with normative motion constraints with respect to the presented implementation?

The BoG recommended to adopt the software implementation.

  - To allow VCL NAL units of different sub-pictures within one picture to have mixed IRAP and non-IRAP NAL unit types? (M0388 option 3; options 1 and 2 of M0388 seem more for AHG17)

Option 3 of M0388 is for further study.

- Misc. tiling topics

  - To signal tile configuration by either row/column numbers or tile width/height values? (M0137)

Chaired by Miska M. Hannuseka.

It was commented it'd be better to use either one only, instead of supporting both modes.

No action.

  - To signal loop_filter_across_tile_groups_enabled_flag? (M0160)

To be discussed when the proponent is available.

  - To improve the derivation of tile width/height values? (M0375)

The functionality can be achieved by explicitly signalling of the tile row height and tile column width.

How does this work with the flexible tiling proposals?

Maybe there is no need to have the uniform mode finally.

It seems interesting. BoG recommended considering this after flexible tiling is resolved.

  - To signal a one tile in tile group flag? (M0130, M0209)

Included into the first offline activity on tile grouping.

  - To signal a tile coded or uncoded flag for each tile in tile group header? (M0547)

Chaired by Miska M. Hannuseka.

This contribution is expected to be discussed in the 360 BoG too.

It was commented that there could be other ways to indicate uncoded areas in the picture than using empty tiles.

The contribution would need an interface of outputting reference pictures (with uncoded areas) from the decoder to an external process and inputting reference pictures (with uncoded areas filled in by geometry padding) to the decoder from an external process. It was commented that this has some similarity to the external base layer concept of SHVC.

The BoG recommended discussing in the plenary or a track whether an interface for inputting/outputting reference pictures to/from the decoder could be acceptable in decoder implementation architectures.

- To support WPP in addition to tiles? (M0070, M0071)

What is the need of WPP now that we have tiles? It was commented that WPP is good for support of parallel processing particularly for software implementations, and WPP hurts less coding efficiency.

It was commented that both HEVC tiles and WPP have been deployed.

It was commented that WPP implementation is available in the VTM software.

M0070: One tile group is allowed to be split into either multiple tiles or multiple WPP rows, not both.

M0071: Proposes to further study parallel processing capability with WPP by CE or AHG.

The BoG agreed that it'd be useful to have WPP support in VVC, and add an AHG mandate on WPP into AHG 12.

The syntax, semantics, and decoding process for WPP are for further study.

# The list of contributions

### Tiling and tile partitioning (26)

#### Tiling allowing tile size unit less than CTU size (5)

JVET-M0066 AHG12: Flexible Tile Partitioning [Y. Yasugi, T. Ikai (Sharp)]

JVET-M0423 Cross-check of JVET-M0066: AHG12: Flexible Tile Partitioning [A. Wieckowski (HHI)]

JVET-M0376 AHG12: On signaling of flexible tiles [M. Damghanian, R.Sjöberg, M. Pettersson (Ericsson)]

JVET-M0459 AHG12: On tiles with partial CTUs [R. Skupin, K. Sühring, Y. Sanchez, T. Schierl (HHI)]

JVET-M0527 AHG12: Comments on Tiles and Flexible Tile Partitioning [W. Wan, M. Zhou, T. Hellman, B. Heng, P. Chen (Broadcom)]

#### Flexible tiling (4)

Option 3 of M0261 is effectively another way to achieve the flexible tiling functionality, through dividing pictures into rectangular sub-pictures and each sub-picture may refer to its own PPS and may hence have its own tile partitioning.

JVET-M0123 AHG12: On hierarchical tile design [Y. He, A. Hamza (InterDigital)]

JVET-M0129 AHG12: On flexible tiling [Y.-K. Wang, Hendry, M. Sychev (Huawei)]

JVET-M0374 AHG12: Flexible tiles to support MCTS use cases [R.Sjöberg, M. Damghanian, M. Pettersson (Ericsson)]

JVET-M0530 AHG12: On signalling of tiles [M. Coban, M. Karczewicz (Qualcomm)] [late]

#### Rectangular tile grouping (5)

JVET-M0121 AHG12: On Rectangular Tile Group [Y. He, A. Hamza (InterDigital)]

JVET-M0130 AHG12: On tile grouping [Y.-K. Wang, Hendry, J. Chen, M. Sychev (Huawei)]

JVET-M0160 AHG17: Flexible tile grouping for VVC [L. Chen, T.-D. Chuang, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-M0209 AHG12: On tile group configuration [W. Choi, K. Choi, K. Choi (Samsung)]

JVET-M0416 AHG12: On Tile Information Signalling [S. Deshpande (Sharp)]

#### Tile and tile group identification and addressing (4)

JVET-M0134 AHG12: On explicit signalling of tile IDs [Hendry, Y.-K. Wang, J. Chen, M. Sychev (Huawei)]

JVET-M0155 AHG12: On tile group identification for VVC [B. Choi, S. Wenger, S. Liu (Tencent)] [late] [miss]

JVET-M0373 AHG12: Merge friendly tile group address signalling [R.Sjöberg, M. Damghanian, M. Pettersson (Ericsson)]

JVET-M0430 AHG12: On Tiles and Tile Groups for VVC [R. Skupin, K. Sühring, Y. Sanchez, T. Schierl (HHI)]

#### MCTS and sub-picture sequence (4)

JVET-M0416 also includes one aspect on MCTS signalling in the PPS.

JVET-M0261 AHG12: On grouping of tiles [M. M. Hannuksela, A. Aminlou (Nokia)]

JVET-M0388 AHG12/AHG17: On merging of MCTSs for viewport-dependent streaming [M. M. Hannuksela (Nokia)]

JVET-M0445 AHG12: On motion constrained tiles for VVC [R. Skupin, V. George, K. Sühring, Y. Sanchez, T. Schierl (HHI)]

JVET-M0536 AHG12: On picture-level tiles and sequence-level tiles for VVC [E. Thomas, A. Gabriel (TNO)] [late]

JVET-M0537 AHG17: On tile group signaling in NAL unit header and as non-VCL NAL unit [E. Thomas, A. Gabriel (TNO)] [late]

#### Miscellaneous tiling topics (4)

JVET-M0136 AHG12: Treating tile and tile group boundaries as picture boundaries [J. Chen, Y.-K. Wang, Hendry, M. Sychev (Huawei)]

JVET-M0137 AHG12: On tile configuration signalling [M. Sychev, Hendry, Y.-K. Wang (Huawei)]

JVET-M0375 AHG12: On uniform tile spacing [M. Damghanian, R.Sjöberg, M. Pettersson (Ericsson)]

JVET-M0547 360° coding tools using uncoded areas, J. Sauer, M. Bläser [late]

### Wavefront parallel processing (2)

JVET-M0070 AHG12: Wavefront processing in a tile group [T. Ikai, S. Deshpande, T. Chujoh, E. Sasaki, T. Aono (Sharp)]

JVET-M0071 WPP: Improved parallel processing capability with WPP [Y. Fujimoto, M. Ikeda, T. Suzuki (Sony)]