| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0799 |
| --- | --- |

| Title: | Bit-width reduction of multiplier in CCLM derivation and prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kei Kawamura Sei Naito 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81 80 5066 9075 ki-kawamura@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution provides a proposal of a bit-width reduction method of a multiplier in both CCLM derivation and prediction process. In the VTM3.0 implementation, a parameter in the linear prediction model has 19 bit including sign bit for the derivation process. For the prediction stage, a multiplier requires 29 bit due to the 19 bit parameter and 10 bit signal. The bit-width of multiplier in the CCLM prediction is wider than that of the motion compensation stage such as 21 bit. In this contribution, bit-width reduction of multiplier is presented which consists of four parts. Firstly, a range of chroma difference is restricted into +/− 256 for 10 bit chroma signal. Secondly, precision of a derivation table is reduced from 16 bit to 8 bit with a small value modification. Thirdly, a range of prediction parameter is restricted into +/− 8.0. Lastly, prediction parameter representation is changed from the 19 bit fixed-point number to the floating-point number with 7 bit fraction. A simulation result by the proposed method shows 0.01% / 0.03% / -0.01% for Y/U/V at the AI condition.

Introduction

In the previous meeting in Macao, CCLM derivation model is replaced from the least-mean-square model to the straight-line-equation model proposed by the JVET-L0191 [1]. SLE model reduces the number of computation operations from that of LMS model while the wide bit-width multiplier is newly introduced. Although the parameter derivation requires division operation among luma and chroma values of two pixels, two look-up-tables can avoid division operation. In the working draft, the parameter ‘a’ in the derivation stage and predSamples in the prediction stages are described as follows, respectively.

div = ( ( maxC − minC ) * ( Floor( 232 / diff ) − Floor( 216 / diff ) * 216 ) + 215 ) >> 16	(8-157)

a = ( ( maxC − minC ) * Floor( 216 / diff ) + div + add ) >> shift	(8-158)

predSamples[ x ][ y ] = Clip1C( ( ( pDsY[ x ][ y ] * a ) >> k ) + b )	(8-161)

Here, bit-width of multiplier is focused in the VTM3.0 implementation. In the derivation stage, a fractional part of the prediction parameter is 16 bit precision because of the precision of the LUT value is 16 bit, while an integer part has no restriction. Even though the parameter value is, in the most cases, expected to be less than 1.0, it means the worst case is not defined. In the prediction stage, the bit-width of a multiplier to the reference luma sample is also larger than 26 bit without sign bit. When the range of prediction parameter is restricted from −4.0 to +4.0, 29 bit multiplier is required.

Such the bit-width of the multiplier is much wider than that of the motion compensation stage, i.e. the bit-width of the multiplier is restricted into 6 bit coefficient and 10 bit signal with 1 bit sign. It is noted that bit-width of accumulator is 21 bit because of the accumulation of eight coefficients.

Proposed method

In this section, we propose the multiplier bit-width reduction method. The target complexity is that of motion compensation, i.e., the bit-width of the multiplier is restricted to the 15 bit with 1 bit sign. The proposed method consists of four parts. In the following, each parts are described, respectively.

Chroma difference restriction

The difference of chroma values of two luma min-max values are restricted to from −256 to +255. It corresponds to maxC – minC values. The motivation is reduce the bit-width of parameter ‘a’ in equation (8-158).

In the software implementation, CCLM_CHROMACROP definition is added.

LUT precision reduction

Firstly, the LUT value precision is reduced from 17 bit to 9 bit. Secondly, the maximum value in the table is modified from 256 to 255, i.e. 8 bit precision is realized. The motivation is the same as the above restriction. An introduced error is negligible by both two bit-width reduction methods.

In the software implementation, CCLM_BITDEPTH and CCLM_1BITREDUCE defines are corresponding to the above methods, respectively.

Parameter range restriction

Even though the chroma difference restriction is applied, the range of parameter ‘a’ depends on the luma difference values. For further bit-width reduction, the range of the parameter is restricted from −8.0 to +8.0. It is noted that +8.0 is not included in the range because of singed value representation.

In the software implementation, CCLM_ALPHACROP definitions is added.

Floating point representation

As the introduction mentioned, a magnitude of the parameter ‘a’ is less than 1.0 for the most cases while the maximum magnitude is 8.0 by the previous restriction. Maintaining the effective digit, floating point representation is introduced. The significand and exponent have 7 bit and 3 bit respectively.

In the software implementation, CCLM_FLOAT_ALPHA definition is added.

Another condition settings as appendix

For best of our knowledge, the above settings provides the lowest BD-rate less for the all intra condition. The chroma difference restriction significantly affects the BD-rate loss. As an appendix, another parameter settings with a simulation result is provided. The chroma difference restriction is from −128 to +127. Consequently, LUT precision can be increased from 8 bit to 9 bit while the parameter range restriction is decreased from −4.0 to +4.0. Furthermore, parameter representation is 12 bit fixed-point, i.e., complexity rule is violated at the prediction stage.

Simulation results

The proposed method is implemented on the top of VTM3.0. A test condition follows the JVET common test condition [2]. The summary of BD-rate is shown in the following table.

Table 1 Simulation results of All Intra condition by the proposed method.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.06% | 0.02% | 100% | 100% |
| Class A2 | 0.00% | 0.07% | 0.00% | 100% | 101% |
| Class B | 0.00% | 0.03% | -0.02% | 99% | 100% |
| Class C | 0.01% | 0.04% | -0.11% | 100% | 100% |
| Class E | 0.00% | -0.01% | 0.09% | 100% | 100% |
| Overall | 0.01% | 0.03% | -0.01% | 100% | 100% |
| Class D | 0.00% | 0.06% | 0.09% | 99% | 100% |
| Class F | 0.05% | 0.27% | 0.21% | 101% | 99% |

Table 2 Simulation results of All Intra condition by the proposed method as an appendix settings.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.53% | 0.83% | 1.38% | 100% | 100% |
| Class A2 | 0.16% | 0.33% | 0.10% | 100% | 100% |
| Class B | 0.06% | 0.04% | 0.24% | 100% | 100% |
| Class C | 0.12% | 0.59% | 0.37% | 100% | 100% |
| Class E | 0.01% | -0.01% | 0.29% | 100% | 100% |
| Overall | 0.16% | 0.33% | 0.45% | 100% | 100% |
| Class D | 0.10% | 0.25% | 0.19% | 99% | 99% |
| Class F | 0.45% | 1.40% | 1.53% | 101% | 100% |

Conclusion

In this contribution, we proposed the bit-width reduction method of the multiplier in both CCLM derivation and prediction process. The proposed bit-width reduction of multiplier consisted of four parts. Firstly, a range of chroma difference was restricted into +/− 256 for 10 bit chroma signal. Secondly, precision of a derivation table was reduced from 16 bit to 8 bit with a small value modification. Thirdly, a range of prediction parameter was restricted into +/− 8.0. Lastly, prediction parameter representation was changed from the 19 bit fixed-point number to the floating-point number with 7 bit fraction. The simulation result by the proposed method showed 0.01% / 0.03% / -0.01% for Y/U/V at the AI condition.

References

- X Ma, H Yang, J Chen, “CE3: Multi-directional LM (Test 4.2.1, 4.2.2, 4.2.3, 4.2.4)”, JVET-L0191, , Macao, CN, 3–12 Oct. 2018.

- F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-L1010, Macao, CN, 3–12 Oct. 2018.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).