| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0816-v2 |
| --- | --- |

| Title: | BoG report on high level syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jill Boyce | Tel: Email: | jill.boyce@intel.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

The BoG met on 11 Jan 2019. The BoG also met on 12 Jan 2019, with the notes reflected in the –v2 version of this document.

The BoG recommends the following:

- Replace the existing IRAP_NUT with 3 new NAL unit types: IDR_W_RADL, IDR_N_LP, CRA_NUT. (from JVET-M0101)

- Add external means flag HandleCraAsCvsStartFlag, with similar text as in HEVC. Text provided in a v3 of JVET-M0101.

- Add a NUT value for STSA (from JVET-M0101)

- Add a NUT value for AUD. (from JVET-M0101)

- Add sps_max_sub_layers_minus1 syntax element to SPS, and decoding process in 8.1.1, 8.1.2 and 8.1.3 of JVET-M0101.

- Add text of sections 7.4.2.4 to 7.4.2.4.5 on NAL unit order and AU boundary detection from JVET-M0101, which is primarily editorial, but has some technical aspects

- Add profile_tier_level( ) syntax structure which includes sub layer level idc

- Add general_non_packed_constraint_flag with semantics as in JVET-M0101.

- Add the temporal scalability sub-bitstream extraction process in JVET-M0101.

- Change the sps_ref_wraparound_offset to sps_ref_wraparound_offset_minus1 and changing the units to be MinCbSizeY as in option 1, subject to review by the 360 BoG.

- Add reference picture signaling from M0128 (basic text version)

- Add 7 new constraint flags corresponding to VVC WD 3 tools as described in M0451.

The BoG suggest discussion in the track or plenary regarding:

- Consider between adding 2 NUT values for RASL and RADL leading pictures or staying with no dedicated NUT values for leading pictures

- Consider if profile_space should be included

- Consider removing concept of I, P, and B slice (tile group) types, but to have flags to control the types of prediction (intra prediction, current picture referencing, inter prediction with one list or two lists), and if this signaling should be for pictures or for tile groups.

- Consider if an AHG should be formed to study solutions to problem of temporal judder when temporal scalability sub-bitstream extraction is used to lower frame rate. Consider if should be done normatively or in an SEI message (which could perhaps be done in JCT-VC for HEVC or AVC)

- Consider if there is interest to normatively specify Gradual Decoder Refresh (GDR) operation, such that a conforming CVS can start with a GDR non-IRAP picture (as proposed in JVET-M0529.)

- Ask the following questions to ALF experts, for consideration when evaluating proposed solutions to improve coding efficiency of tile group headers:

- How much bitrate do ALF parameters typically require?

- Do we expect that multiple pictures would use the same ALF parameters?

- What is the expected coding gain when ALF is used?

- Does the ALF design support having different ALF parameters in different tile groups in the same picture?

- If it is allowed to have different ALF parameters in different tile groups in the same picture, do we expect that they would frequently differ?

# Contributions

### General high-level syntax and parameter sets (21)

#### General high-level syntax and NAL unit header (11)

JVET-M0101 AHG17: On VVC HLS [R. Skupin, K. Sühring, Y. Sanchez (HHI), M. M. Hannuksela, K. Kammachi-Sreedhar (Nokia), Y.-K. Wang, Hendry (Huawei), S. Wenger, B. Choi (Tencent), S. Deshpande (Sharp)]

JVET-M0120 Proposed NAL Unit Header Design Principles [S. Wenger, B. Choi, S. Liu (Tencent)]

JVET-M0131 AHG17: On NAL unit types for IRAP pictures and leading pictures [Y.-K. Wang, Hendry (Huawei)]

JVET-M0152 AHG17: On random access point for VVC [B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-M0153 AHG17: On leading picture for VVC [B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-M0156 AHG17: On component type indication for VVC [B. Choi, S. Wenger, S. Liu (Tencent)] [late]

JVET-M0157 AHG17: On picture order count for VVC [B. Choi, S. Wenger, S. Liu (Tencent)] [late]

JVET-M0161 AHG17: Signalling random access properties in the NAL unit header [L. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

Proposes a new GDR NUT and a refreshed_region_flag and GDR pic count in the NUH of GDR NUTs. Is related to M0529, can be further studied in the same context.

Proposes an end_nuh_field_flag in the NUH, but doesn’t have conditional presence of syntax based on it.

Proposes signaling number of leading pictures. This could be problematic with temporal scalability if a leading picture were dropped.

JVET-M0520 AHG17: On NAL unit header design for VVC [S. Wenger, B. Choi, S. Liu (Tencent)]

This contribution proposes a NAL unit header design for VVC, following the guidance of H.264/SVC and MVC’s syntax.  In particular, certain NAL unit types trigger the extension of the NAL unit header from minimal 8 bits to larger (still octet aligned) structures.  It is asserted that such a design is a) extensible towards technologies such as SVC and MVC without spending bits or a significant amount of numbering space on “hooks”, b) allows for the conditional presence of syntax elements not found in H.264’s and HEVC’s NAL unit headers, based on application needs, and c) is still lightweight enough to be efficiently implemented in a MANE (and trivially implementable in a decoder).

Perhaps can go back to an 8-bit NUH for most common type and extend for less common types. For longer headers, may have to worry about start code emulation.

RTP payload formats frequently have extension bits to increase the length of the RTP header.

HEVC design used the VPS to interpret layer_id. It was suggested that tile group ID would also need a  parameter set to interpret.

JVET-M0529 AHG14: Normative Recovery Point Indication [M. Pettersson, R.Sjöberg, M. Damghanian (Ericsson)]

This contribution proposes to add a recovery point indication NAL unit to the VVC specification, using a new non-VLC NAL unit type. The position of the recovery point indication NAL unit in the bitstream specifies the start of the recovery period. The end of the recovery period, the recovery point picture, is specified by a recovery_poc_cnt UVLC code word, which is the only payload element in the proposed recovery point indication NAL unit RBSP. It is further proposed that starting the decoding at a recovery point is supported through the following process:

- decode a recovery point indication NAL unit that defines a recovery point begin (RPB) access unit and the start and end picture of the recovery point

- start decoding at the start picture

- generate unavailable pictures from the RPS of the start picture

- suppress output of the start picture and all pictures that follow in decoding order until the end picture is decoded

In comparison to the recovery point SEI message in HEVC, the proposed solution is normatively specified and does not contain the exact match flag nor the broken link flag. It is claimed that having the indication of the recovery point normatively specified in VVC has the following advantages:

- The encoder will know that a decoder shall support the recovery point indication NAL unit. Thus, no additional IRAP pictures need to be encoded to ensure random access when the decoder implementation is unknown.

- By a normative recovery point signaling in the VVC specification, there is no need for external specifications to mandate the SEI message.

- Indicating the presence of recovery points non-normatively in an SEI message may cause issues if support of recovery point coding tools would be specified normatively in VVC, for example if syntax or processes are different for any picture in the recovery period than for other pictures.

In order to make recovery points normative in VVC, the contribution proposes that a CVS may start with a recovery point signaled by the new NAL unit type. Alternatives for normatively specifying the recovery point are discussed in the contribution.

Would need to define a decoding process that starts without an IRAP picture.

Defines a new NUT which is sent in its own NAL unit. Why make a separate NUT vs. including in some other HLS location? Another alternative would be to use add a new NAL unit type for the coded picture. M0161 is related.

It was suggested that software implementation would be needed

Would there be a normative requirement on conforming pictures before the recovery point?

Encoding in a drone was suggested as an interesting use case.

Further study encouraged.

BoG suggests discussion in the track about if it would be useful to normatively specify GDR such that it can start a CVS?

JVET-M0537 AHG17: On tile group signaling in NAL unit header and as non-VCL NAL unit [E. Thomas, A. Gabriel (TNO)] [late]

#### Reference picture management (3)

JVET-M0128 AHG17: On reference picture management for VVC [Y.-K. Wang, Hendry (Huawei), S. Deshpande (Sharp), M. M. Hannusela (Nokia), G. Ryu, W. Choi (Samsung), X. Wang, Y.-W. Chen (Kawi), L. Zhang (Bytedance), P. Wu, M. Li (ZTE), S.-H. Kim (LG), J. Boyce (Intel), A. M. Tourapis, D. Singer (Apple), F. Edouard, P. Andrivon (Technicolor), Y.-W. Huang, C.-W. Hsu, C.-Y. Chen, T.-D. Chuang, L. Chen (MediaTek), K. Kawamura (KDDI), Y.-C. Sun, J. Lou (Alibaba)]

JVET-M0154 AHG17: On decoded picture buffer management for VVC [B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-M0378 AHG17: RPS for VVC [R.Sjöberg, M. Damghanian, M. Pettersson (Ericsson)] [late]

#### Picture header and header parameter set (HPS) (4)

JVET-M0132 AHG17: On header parameter set (HPS) [Y.-K. Wang, Hendry, J. Chen (Huawei)]

This contribution proposes a design of header parameter set (HPS) for carriage of ALF parameters only. When tile_group_alf_enabled_flag is equal to 1, an HPS is referenced by a tile group header, and the ALF parameters carried in the HPS is used by the tile group.

In the latest VVC WD, ALF parameters are directly signalled in the tile group header. It is asserted that this causes unnecessary redundant transmission of ALF parameters when there are multiple tile groups of a picture while at least two of the tile groups contain the same ALF parameters and when multiple pictures use the same ALF parameters. It asserted that using HPSs can avoid such unnecessary redundant transmission of ALF parameters.

It is asserted that allowing inclusion of other types of parameters in the HPS would require either of the following:

- A complicated partial HPS update mechanism, which was asserted to be a headache when designing the adaptation parameter set (APS) during HEVC development, and an agreeable technical solution was not found finally, as before that problem was solved, the headache was gone when ALF was removed from the HEVC draft text.

- Multiple HPS types, each carrying one type of parameters; this approach would require the tile group header to reference multiple HPS IDs or one HPS ID plus an HPS type.

It was commented that a picture header cannot be shared across pictures. There was a suggestion that it be renamed ALF parameter set.

The proposal has the header parameter set activated by a tile group, and the value can be different for tile groups in the same picture.

Why is SPS_id included? May not be needed.

Is there any prediction of ALF parameters from picture-to-picture? Not in the current design.

May want to restrict from updating the HPS within the same picture. May need more than 64 HPS id values.

JVET-M0260 AHG17: Carriage of tile group header parameters in higher level structures [M. M. Hannuksela (Nokia)]

The following options are proposed in this contribution:

- PPS containing tile group header parameters.

- Tile group header parameters are divided into subsets that are selectively included either in the PPS or in the tile group header.

- Different PPSs may be activated by different tile groups of the same picture, but besides the tile group header parameters, all other parameters are required to have the same values in all PPSs activated for the same picture.

- PPS containing ALF parameters and picture header containing other tile group header parameters.

- A picture header VCL NAL unit is present and precedes all tile group VCL NAL units of the picture.

- The tile group header parameters besides ALF parameters may be included in the picture header. If not present in the picture header, they are included in the tile group header(s).

- The ALF parameters may be included in the PPS, picture header, or tile group header.

- Different PPSs may be activated by different tile groups of the same picture, but besides the ALF parameters, all other parameters are required to have the same values in all PPSs activated for the same picture.

The proponent prefers option 2, since it keeps the conceptual design of including such parameters into the PPS that may be applicable to more than one picture, whereas parameters that are likely to change on picture basis are not included in the PPS. A picture header NAL unit as in option 2 was considered simpler than a header parameter set, since the activation of a picture header takes place implicitly.

A concern was raised about out-of-band transmission of PPS if referring to APS_id in the PPS, since APS parameter are expected to change frequently.

For an extracted MCTS sub-bitstream, would need to also include the picture header.

Provides ability to override in the tile group header parameters that were present in the PPS.

JVET-M0377 AHG17: Picture header NAL unit type [R.Sjöberg, M. Damghanian, M. Pettersson (Ericsson)]

This contribution proposes a picture header network abstraction layer (NAL) unit with a dedicated NAL unit type PIC_NUT and a payload consisting of syntax elements from the current tile group header.

The main idea is to put syntax elements that are common for multiple tile groups in a single picture into a separate syntax table called common_tile_group_header(). It is proposed to add an SPS flag to control whether the syntax table elements will be carried in a picture header NAL unit sent in every AU in the CVS or carried in the tile group header of each tile group NAL unit in the CVS.

The proposed design contains the following elements:

- A new NAL unit type PIC_NUT with picture_header_rbsp() payload.

- A new syntax table, common_tile_group_header() consisting of syntax elements from the current tile group header.

- A new SPS flag, sps_picture_header_enabled_flag, that specifies whether PIC_NUT NAL units are present in the CVS or not. The flag may be exposed as a tool on/off indicator.

- If the flag is equal to 1, each access unit (AU) in the CVS shall contain a PIC_NUT NAL unit carrying common_tile_group_header() syntax and there shall be no common_tile_group_header() syntax in the tile group header.

- If the flag is equal to 0, there shall be no PIC_NUT NAL unit in the CVS. Each tile group header in the CVS shall then include the common_tile_group_header() syntax.

The proponents claim that the proposed design reduces the header overhead for use cases where each picture consists of a substantial number of tile groups.

The proponents also claim that the picture header design enables detection of lost picture header NAL units (since each AU shall contain one if they are enabled).

A picture header cannot be used by multiple pictures.

Having a picture header for each picture can help with error resiliency since its absence would be detectible, while missing PPSs or HPSs are not detectible, since the PPS_id or HPS_id value is reused. A suggestion was made that PPS_id could be forced to change values for each picture to also provide detection of a missing PPS, but that might be a problem for splicing.

It was suggested to switch between the two alternative proposed constraints on a per picture basis.

It was suggested to add tile group header parameter update functionality to this design.

Proposes the PPS_id be in both the picture header and the tile group header.

Had proposed to include all tile group header parameters in the common picture header, but based on discussion, the proponent suggests that it may make sense to include only the ALF parameters.

JVET-M0415 AHG17: Comments on High-Level Syntax of VVC [S. Deshpande (Sharp)]

This document proposes comments on high-level syntax of VVC WD.  It includes following proposals:

- Modifications are proposed to the horizontal reference wrap-around signaling in VVC WD.

- It is proposed to define a NAL unit type which includes signaling information which is applicable to multiple  tile groups in a picture.

Pic header with NUT proposed, which could include ALF parameters or other parameters. Can be reused for multiple pictures that share the same PPS.

Although called pic header, it seems very like a parameter set. Tile group header contains a tile group pic id and the pic header contains PPS_id, which is not present in the tile group.

No parameter override capability was proposed, but could be extended to support.

#### Miscellaneous general HLS topics (3)

JVET-M0133 AHG17: On parsing dependency between parameter sets [Y.-K. Wang (Huawei), J. Boyce (Intel)]

JVET-M0386 AHG17: On slice_type (tile_group_type) [K. Sühring, Y. Sanchez, R. Skupin (HHI)]

JVET-M0579 On Frame Rate Support and Extraction in VVC [A. Segall, S. Deshpande (Sharp Labs of America), M. Hannuksela (Nokia)]

# Discussion

Discussion generally followed the questions listed in JVET-M0776.

- To have variable-length NAL unit header or not? (M0120, M0161, M0520)

  - Is the guideline for NAL unit header design as proposed in M0120 agreeable?

Consider  variable length NAL unit header design if we need to carry additional syntax elements in the NUH that we couldn’t efficiently fit otherwise.

2 How many IRAP NAL unit types?

  - 3: IDR_W_RADL, IDR_N_LP, CRA_NUT? (M0101, M0152)

Some of the motivation is to align w/ Systems stream access points

In previous Macau meeting, there had been an agreement in principle to add support for CRA indication in NUH.

BoG recommends to replace the existing IRAP_NUT with 3 new NAL unit types: IDR_W_RADL, IDR_N_LP, CRA_NUT.

  - Should CRA pictures be required to have at least one RASL picture? (M0131)

Asserted that this could simplify file format mapping.

If required, could require a splicer or middle box to relabel or to add an EOS NAL unit.

HRD implications. Two sets of HRD parameters. We don’t have an HRD design yet.

In HEVC there were some slice header syntax dependencies on NUT value, so if a splicer would have to relabel, we would have to be sure that it wouldn’t impact slice header syntax in VVC design.

CRAs have reference picture signaling while IDRs do not need them, which could be a problem when relabeling.

  - Should CRA pictures be required to have at least one associated leading picture (RASL or RADL)? (M0152)

  - Should CRA pictures be disallowed to have any associated RADL picture? (M0153)

May save NUT values by disallowing.

  - Should RASL pictures allowed to be associated only with CRA pictures, not any other types of IRAP pictures? (M0153)

This is already in the existing design.

  - Should IDR pictures contain syntax elements for reference picture management? (M0388 option 1)

Use case of 360 video could benefit from this or other immersive media use cases or motion constrained tiles use cases, where some regions are intra refreshed and others not. Use case is of interest, but other solutions may also address the use case. Further study encouraged.

  - To specify only one VLC NAL unit type and to have a picture header, and in picture header there is syntax element that specifies the type of the picture (IDR, CRA, STSA, Trail, RADL, RASL)? (M0388 option 2)

- To have a cvs_start_flag in the NAL unit header? (M0152)

Adding a bit to all NUHs seems unnecessary, since would only apply to some NUTs, especially CRA.

- To signal tile group ID in the NAL unit header? (M0155, M0536)

Question about how this differs from using a fixed length field at the beginning of the tile group header.

Comment that tile group header has start code emulation while NUH does not.

May want to be used with a variable length NUH, since not all use cases would require this.

Can we save bitrates in parameter sets and headers for still picture profile?

- To specify an external-means provided flag, e.g., named HandleCraAsCvsStartFlag, for CRA pictures? (M0152)

HEVC has the same external-means flag, which is used in the decoding process.

BoG recommends to add external means flag HandleCraAsCvsStartFlag, with similar text as in HEVC. Text will be provided in a v3 of JVET-M0101.

- When HandleCraAsCvsStartFlag is equal to 0, should the CRA picture be required to have at least one associated leading picture (RASL or RADL)? (M0152)

- When HandleCraAsCvsStartFlag is equal to 1, should the CRA picture be disallowed to have any leading picture (RASL or RADL)? (M0152)

This would place a burden on the external means to remove leading pictures.

- How many NAL unit types for leading pictures (i.e., RASL and RADL)? (M0101)

  - 2?

  - 1? (M0153)

  - 0? (M0131)

BoG suggests discussion in the track to consider between adding 2 NUT values (for RASL and RADL) or staying with no dedicated NUT values for leading pictures.

- To signal random access type and number of leading pictures in the NAL unit header? (M0161)

- To have a NAL unit type for STSA? (M0101)

BoG recommends to add a NUT value for STSA.

- To adopt the non-VCL NAL unit types as proposed in M0101?

Some broadcast application standards mandate the use of AUD for AVC and HEVC.

If we were to adopt a DPS, we would want to also add a NUT value for DPS.

BoG recommends to add a NUT value for AUD.

- To have nuh_component_type in the NAL unit header for indication of the component type such as YUV, RGB, depth map, alpha map, 3D geometry map, occupancy map, etc.? (M0156)

It was suggested that the HEVC layer_id design may be able to be used for this purpose. It was commented that some of these features are not within the current scope of the VVC design.

- To have nuh_pic_id in the NAL unit header, which has the same value with the 8-bit LSB values of POC, for AU detection purpose? (M0157)

It was commented that always including 8-bits in the NUH is expensive for use cases that don’t need this feature.

- To adopt the general decoding process text as proposed in M0101?

This includes adding sps_max_sub_layers_minus1 syntax element to SPS, which is used for signaling sub layer level idc.

BoG recommends to adopt adding sps_max_sub_layers_minus1 syntax element to SPS and decoding process in 8.1.1, 8.1.2 and 8.1.3 of M0101.

- To add HEVC-like sections 7.4.2.4 to 7.4.2.4.5 on NAL unit order and AU boundary detection as proposed in M0101?

BoG recommends to adopt text of sections 7.4.2.4 to 7.4.2.4.5 on NAL unit order and AU boundary detection from JVET-M0101, which is primarily editorial, but has some technical aspects.

- To have parameter set extension mechanism as in HEVC? (M0101)

Having parameter set extensibility is desirable.

It was suggested to consider if the extension flag and condition could be eliminated while still providing the extension capability. For further study.

- To adopt the profile level syntax as in M0101 as a starting point?

  - To have a tier flag? (M0101)

  - To have profile_space? (M0101)

There was agreement that the sub layer profile idc and sub_layer_constraint_info( ) is unnecessary, and that tier should be included.

BoG recommends to add profile_tier_level( ) syntax structure which includes sub layer level idc.

BoG suggests discussion in the track about if profile_space should be included.

- To modify the semantics of general_non_packed_constraint_flag as in M0101, that begins: general_non_packed_constraint_flag equal to 1 indicates that the cropped output pictures of the decoded bitstream are suitable to be displayed

BoG recommends to add general_non_packed_constraint_flag with semantics as in M0101.

- To have a DPS as proposed in M0101?

  - Whether decoding parameter set should rather be a "bitstream header" NAL unit type, in which case the syntax would not include the parameter set ID and the standard would not have the activation process of the decoding parameter set?

There was agreement that having a decoding parameter set would be useful. Further study for the syntax and relationship with SPS syntax. It was questioned if having the DPS as a parameter set is necessary, since it will be the same for the entire bitstream. It was suggested that this should be able to be signaled via external means, even if parameter set structure is not used.

- To adopt the HEVC-like sub-bitstream extraction process as proposed in M0101?

BoG recommends to adopt the temporal scalability sub-bitstream extraction process in M0101.

## Reference picture management

Chaired by K. Suehring.

General discussion about LT picture functionality: it was asserted that the functionality is still useful. The term could be avoided to shorten the standard text.

To use a mechanism based on direct RPL signalling and derivation? (M0128, M0154)

The proposal is aiming for simplifying the process. The RPL approach does not have a separate list initialization and list modification process. It was pointed out, that the difference may not be measurable in terms of run time.

It is clear from the list construction process, which picture are active for prediction in the current picture.

Software is available.

It was mentioned that if reference picture list reordering is necessary RPS may be a bit more bit efficient.

There is no results available for reordering.

The proposed limit on RPL definitions in the SPS is 64 per list, which is twice as many as in RPS.

The difference of the two concepts seems small.

It was noted that the long-term reference concept should be tested as well as non-CTC GOP structures.

The BoG recommends to adopt M0128 (basic text version)

  - If yes, to explicitly signal reference pictures to be marked as "unused for reference"? (M0154)

Withdrawn.

  - If yes, to keep the LTRP functionality but remove the naming? (M0128 discussion)

No, use basic text.

- To use a mechanism based RPS? (M0378)

It was pointed out that the shortened text may create different results than the long text. The long text is the one that is proposed.

  - If yes, to keep the LTRP functionality? (M0378)

The proponents of M0128 pointed out that the proposed process avoids intermediate steps that are necessary for RPS bases reference picture list construction.

## Picture header and HPS

- To add HPS or picture header NAL unit with a reference-able ID? (M0132, M0451 proposal 2)

  - If yes, to contain only ALF parameters? (M0132)

  - If yes, to add the ID in the tile group header (M0132) or to replace the PPS ID in the tile group header (M0451 proposal 2)?

The main motivation is for coding efficiency savings by avoiding replicating ALF parameters for multiple tile groups in the same picture, or for multiple pictures.

Does the ALF tool support having different ALF parameters in different tile groups in the same picture?

Should only ALF parameters be included or also other parameters?

Do we expect that multiple pictures would use the same ALF parameters? May be impacted by the temporal hierarchy.

Suggestion to get some experimental data to help guide our decision at a future meeting.

Suggestion to be future proof in our design.

How likely is that ALF parameters will change in regions of traditional content?

Questions to ask at the JVET track/plenary to ALF experts:

- How much bitrate do ALF parameters typically require?

- Do we expect that multiple pictures would use the same ALF parameters?

- What is the expected coding gain when ALF is used?

- Does the ALF design support having different ALF parameters in different tile groups in the same picture?

- If it is allowed to have different ALF parameters in different tile groups in the same picture, do we expect that they would frequently differ?

## Misc. general HLS topics

- To allow parsing dependency between parameter sets? (M0133)

- To either add "_minus1" to the end of sps_ref_wraparound_offset, or to remove sps_ref_wraparound_enabled_flag and the value of sps_ref_wraparound_offset to be equal 0? If yes which option to take? (M0415 proposal 1)

BoG recommends adoption to change the sps_ref_wraparound_offset to sps_ref_wraparound_offset_minus1 and changing the units to be MinCbSizeY as in option 1, subject to review by the 360 BoG.

It was suggested to review the chroma sample interpolation process in section 8.3.6.3.4 of VVC WD3 to check if it can handle non-4:2:0.

- To split tile_group_type into multiple syntax elements as one of the following two ways? (M0386)

Proposed to no longer have I, P, B slice types, but to have flags to control the types of prediction (intra prediction, current picture referencing, inter prediction with one list or two lists)

It was agreed that this proposal is interesting, and option 1 was preferred.

It was suggested that the syntax should be flexible and not impose syntactic restrictions of tool combinations, and instead let those be profile constraints.

Should this be for picture signaling or tile group signalling?

BoG suggests discussion in the track.

- To create an AHG to further study the extraction of multiple frame rates from a VVC bit-stream with the following design goals? (M0579)

Temporal judder can be a problem with temporal scalability extraction of 60 fps from a 120 fps sequence, such as used by ATSC.

Would the de-judder post-processing be part of the conformance point definition, or could it be done as an SEI message.  This could be included in a mandate in the AHG to study. If done as an SEI message, could be done in JCT-VC for HEVC (or AVC).

BoG suggests discussion in the track.

## Interoperability and capability points definition and signalling

Chaired by K. Suehring

- To adopt the update to interoperability point syntax as proposed in M0451?

Ordering of syntax elements and naming was changed to match order and naming of tools in JVET-L1001.

7 new flags for tools adopted at the Macao JVET meeting.

The semantics was reviewed, “requirement of bitstream conformace” could be left out.

It was suggested to group the syntax element in a structure. It seems likely that the syntax would be moved to a DPS.

It was suggested to add reserved bits for future extension.

It was suggested to make the presence of the enable-flags dependent on the value of the constraint-flag.

The BoG recommends to adopt the 7 new flags as described in the contribution.

## Background

### HEVC NAL unit types

Table 7-1 – NAL unit type codes and NAL unit type classes

| nal_unit_type | Name of nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| 0 1 | TRAIL_N TRAIL_R | Coded slice segment of a non-TSA, non-STSA trailing picture slice_segment_layer_rbsp( ) | VCL |
| 2 3 | TSA_N TSA_R | Coded slice segment of a TSA picture slice_segment_layer_rbsp( ) | VCL |
| 4 5 | STSA_N STSA_R | Coded slice segment of an STSA picture slice_segment_layer_rbsp( ) | VCL |
| 6 7 | RADL_N RADL_R | Coded slice segment of a RADL picture slice_segment_layer_rbsp( ) | VCL |
| 8 9 | RASL_N RASL_R | Coded slice segment of a RASL picture slice_segment_layer_rbsp( ) | VCL |
| 10 12 14 | RSV_VCL_N10 RSV_VCL_N12 RSV_VCL_N14 | Reserved non-IRAP SLNR VCL NAL unit types | VCL |
| 11 13 15 | RSV_VCL_R11 RSV_VCL_R13 RSV_VCL_R15 | Reserved non-IRAP sub-layer reference VCL NAL unit types | VCL |
| 16 17 18 | BLA_W_LP BLA_W_RADL BLA_N_LP | Coded slice segment of a BLA picture slice_segment_layer_rbsp( ) | VCL |
| 19 20 | IDR_W_RADL IDR_N_LP | Coded slice segment of an IDR picture slice_segment_layer_rbsp( ) | VCL |
| 21 | CRA_NUT | Coded slice segment of a CRA picture slice_segment_layer_rbsp( ) | VCL |
| 22 23 | RSV_IRAP_VCL22 RSV_IRAP_VCL23 | Reserved IRAP VCL NAL unit types | VCL |
| 24..31 | RSV_VCL24.. RSV_VCL31 | Reserved non-IRAP VCL NAL unit types | VCL |
| 32 | VPS_NUT | Video parameter set video_parameter_set_rbsp( ) | non-VCL |
| 33 | SPS_NUT | Sequence parameter set seq_parameter_set_rbsp( ) | non-VCL |
| 34 | PPS_NUT | Picture parameter set pic_parameter_set_rbsp( ) | non-VCL |
| 35 | AUD_NUT | Access unit delimiter access_unit_delimiter_rbsp( ) | non-VCL |
| 36 | EOS_NUT | End of sequence end_of_seq_rbsp( ) | non-VCL |
| 37 | EOB_NUT | End of bitstream end_of_bitstream_rbsp( ) | non-VCL |
| 38 | FD_NUT | Filler data filler_data_rbsp( ) | non-VCL |
| 39 40 | PREFIX_SEI_NUT SUFFIX_SEI_NUT | Supplemental enhancement information sei_rbsp( ) | non-VCL |
| 41..47 | RSV_NVCL41.. RSV_NVCL47 | Reserved | non-VCL |
| 48..63 | UNSPEC48.. UNSPEC63 | Unspecified | non-VCL |

### HEVC NAL unit header syntax

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_unit_type | u(6) |
| nuh_layer_id | u(6) |
| nuh_temporal_id_plus1 | u(3) |
| } |  |

### VVC WD 3 NAL unit header syntax

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_unit_type | u(5) |
| nuh_temporal_id_plus1 | u(3) |
| nuh_reserved_zero_7bits | u(7) |
| } |  |

### VVC WD 3 NAL unit types

Table 7-1 – NAL unit type codes and NAL unit type classes

| nal_unit_type | Name of nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| 0 | NON_IRAP_NUT | Coded tile_group of a non-IRAP picture tile_group_layer_rbsp( ) | VCL |
| 1 | IRAP_NUT | Coded tile_group of an IRAP picture tile_group_layer_rbsp( ) | VCL |
| 2-15 | RSV_VCL_NUT | Reserved VCL NAL Units | VCL |
| 16 | SPS_NUT | Sequence parameter set seq_parameter_set_rbsp( ) | non-VCL |
| 17 | PPS_NUT | Picture parameter set pic_parameter_set_rbsp( ) | non-VCL |
| 18 | EOS_NUT | End of sequence end_of_seq_rbsp( ) | non-VCL |
| 19 | EOB_NUT | End of bitstream end_of_bitstream_rbsp( ) | non-VCL |
| 20, 21 | PREFIX_SEI_NUT SUFFIX_SEI_NUT | Supplemental enhancement information sei_rbsp( ) | non-VCL |
| 22-26 | RSV_NVCL | Reserved | non-VCL |
| 27-31 | UNSPEC | Unspecified | non-VCL |