| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0816 |
| --- | --- |

| Title: | BoG report on high level syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jill Boyce | Tel: Email: | jill.boyce@intel.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

The BoG met on 11 Jan 2019. The BoG plans to meet again to discussion remaining topics.

The BoG recommends the following:

- Replace the existing IRAP_NUT with 3 new NAL unit types: IDR_W_RADL, IDR_N_LP, CRA_NUT. (from JVET-M0101)

- Add external means flag HandleCraAsCvsStartFlag, with similar text as in HEVC. Text will be provided in a v3 of JVET-M0101

- Add a NUT value for STSA (from JVET-M0101)

- Add a NUT value for AUD. (from JVET-M0101)

- Add sps_max_sub_layers_minus1 syntax element to SPS, and decoding process in 8.1.1, 8.1.2 and 8.1.3 of M0101.

- Add text of sections 7.4.2.4 to 7.4.2.4.5 on NAL unit order and AU boundary detection from JVET-M0101, which is primarily editorial, but has some technical aspects

- Add profile_tier_level( ) syntax structure which includes sub layer level idc, subject to review of text.

- Add general_non_packed_constraint_flag with semantics as in M0101.

- Add the temporal scalability sub-bitstream extraction process in M0101.

- Change the sps_ref_wraparound_offset to sps_ref_wraparound_offset_minus1 and changing the units to be MinCbSizeY as in option 1, subject to review by the 360 BoG.

- Add reference picture signaling from M0128 (basic text version)

- Add 7 new constraint flags corresponding to VVC WD 3 tools as described in M0451.

The BoG suggest discussion in the track regarding:

- Consider between adding 2 NUT values for RASL and RADL leading pictures or staying with no dedicated NUT values for leading pictures

- Consider if profile_space should be included

- Consider removing concept of I, P, and B slice (tile group) types, but to have flags to control the types of prediction (intra prediction, current picture referencing, inter prediction with one list or two lists), and if this signaling should be for pictures or for tile groups.

- Consider if an AHG should be formed to study solutions to problem of temporal judder when temporal scalability sub-bitstream extraction is used to lower frame rate. Consider if should be done normatively or in an SEI message (which could perhaps be done in JCT-VC for HEVC or AVC)

# Contributions

### General high-level syntax and parameter sets (21)

#### General high-level syntax and NAL unit header (11)

JVET-M0101 AHG17: On VVC HLS [R. Skupin, K. Sühring, Y. Sanchez (HHI), M. M. Hannuksela, K. Kammachi-Sreedhar (Nokia), Y.-K. Wang, Hendry (Huawei), S. Wenger, B. Choi (Tencent), S. Deshpande (Sharp)]

JVET-M0120 Proposed NAL Unit Header Design Principles [S. Wenger, B. Choi, S. Liu (Tencent)]

JVET-M0131 AHG17: On NAL unit types for IRAP pictures and leading pictures [Y.-K. Wang, Hendry (Huawei)]

JVET-M0152 AHG17: On random access point for VVC [B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-M0153 AHG17: On leading picture for VVC [B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-M0156 AHG17: On component type indication for VVC [B. Choi, S. Wenger, S. Liu (Tencent)] [late]

JVET-M0157 AHG17: On picture order count for VVC [B. Choi, S. Wenger, S. Liu (Tencent)] [late]

JVET-M0161 AHG17: Signalling random access properties in the NAL unit header [L. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-M0520 AHG17: On NAL unit header design for VVC [S. Wenger, B. Choi, S. Liu (Tencent)]

This contribution proposes a NAL unit header design for VVC, following the guidance of H.264/SVC and MVC’s syntax.  In particular, certain NAL unit types trigger the extension of the NAL unit header from minimal 8 bits to larger (still octet aligned) structures.  It is asserted that such a design is a) extensible towards technologies such as SVC and MVC without spending bits or a significant amount of numbering space on “hooks”, b) allows for the conditional presence of syntax elements not found in H.264’s and HEVC’s NAL unit headers, based on application needs, and c) is still lightweight enough to be efficiently implemented in a MANE (and trivially implementable in a decoder).

Perhaps can go back to an 8-bit NUH for most common type and extend for less common types. For longer headers, may have to worry about start code emulation.

RTP payload formats frequently have extension bits to increase the length of the RTP header.

HEVC design used the VPS to interpret layer_id. It was suggested that tile group ID would also need a  parameter set to interpret.

JVET-M0529 AHG14: Normative Recovery Point Indication [M. Pettersson, R.Sjöberg, M. Damghanian (Ericsson)]

JVET-M0537 AHG17: On tile group signaling in NAL unit header and as non-VCL NAL unit [E. Thomas, A. Gabriel (TNO)] [late]

#### Reference picture management (3)

JVET-M0128 AHG17: On reference picture management for VVC [Y.-K. Wang, Hendry (Huawei), S. Deshpande (Sharp), M. M. Hannusela (Nokia), G. Ryu, W. Choi (Samsung), X. Wang, Y.-W. Chen (Kawi), L. Zhang (Bytedance), P. Wu, M. Li (ZTE), S.-H. Kim (LG), J. Boyce (Intel), A. M. Tourapis, D. Singer (Apple), F. Edouard, P. Andrivon (Technicolor), Y.-W. Huang, C.-W. Hsu, C.-Y. Chen, T.-D. Chuang, L. Chen (MediaTek), K. Kawamura (KDDI), Y.-C. Sun, J. Lou (Alibaba)]

JVET-M0154 AHG17: On decoded picture buffer management for VVC [B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-M0378 AHG17: RPS for VVC [R.Sjöberg, M. Damghanian, M. Pettersson (Ericsson)] [late]

#### Picture header and header parameter set (HPS) (4)

JVET-M0132 AHG17: On header parameter set (HPS) [Y.-K. Wang, Hendry, J. Chen (Huawei)]

JVET-M0260 AHG17: Carriage of tile group header parameters in higher level structures [M. M. Hannuksela (Nokia)]

JVET-M0377 AHG17: Picture header NAL unit type [R.Sjöberg, M. Damghanian, M. Pettersson (Ericsson)]

JVET-M0415 AHG17: Comments on High-Level Syntax of VVC [S. Deshpande (Sharp)]

#### Miscellaneous general HLS topics (3)

JVET-M0133 AHG17: On parsing dependency between parameter sets [Y.-K. Wang (Huawei), J. Boyce (Intel)]

JVET-M0386 AHG17: On slice_type (tile_group_type) [K. Sühring, Y. Sanchez, R. Skupin (HHI)]

JVET-M0579 On Frame Rate Support and Extraction in VVC [A. Segall, S. Deshpande (Sharp Labs of America), M. Hannuksela (Nokia)]

# Discussion

Discussion generally followed the questions listed in JVET-M0776.

- To have variable-length NAL unit header or not? (M0120, M0161, M0520)

  - Is the guideline for NAL unit header design as proposed in M0120 agreeable?

Consider  variable length NAL unit header design if we need to carry additional syntax elements in the NUH that we couldn’t efficiently fit otherwise.

2 How many IRAP NAL unit types?

  - 3: IDR_W_RADL, IDR_N_LP, CRA_NUT? (M0101, M0152)

Some of the motivation is to align w/ Systems stream access points

In previous Macau meeting, there had been an agreement in principle to add support for CRA indication in NUH.

BoG recommends to replace the existing IRAP_NUT with 3 new NAL unit types: IDR_W_RADL, IDR_N_LP, CRA_NUT.

  - Should CRA pictures be required to have at least one RASL picture? (M0131)

Asserted that this could simplify file format mapping.

If required, could require a splicer or middle box to relabel or to add an EOS NAL unit.

HRD implications. Two sets of HRD parameters. We don’t have an HRD design yet.

In HEVC there were some slice header syntax dependencies on NUT value, so if a splicer would have to relabel, we would have to be sure that it wouldn’t impact slice header syntax in VVC design.

CRAs have reference picture signaling while IDRs do not need them, which could be a problem when relabeling.

  - Should CRA pictures be required to have at least one associated leading picture (RASL or RADL)? (M0152)

  - Should CRA pictures be disallowed to have any associated RADL picture? (M0153)

May save NUT values by disallowing.

Revisit after discussion about leading pictures.

  - Should RASL pictures allowed to be associated only with CRA pictures, not any other types of IRAP pictures? (M0153)

This is already in the existing design.

  - Should IDR pictures contain syntax elements for reference picture management? (M0388 option 1)

Use case of 360 video could benefit from this or other immersive media use cases or motion constrained tiles use cases, where some regions are intra refreshed and others not. Use case is of interest, but other solutions may also address the use case. Further study encouraged.

  - To specify only one VLC NAL unit type and to have a picture header, and in picture header there is syntax element that specifies the type of the picture (IDR, CRA, STSA, Trail, RADL, RASL)? (M0388 option 2)

- To have a cvs_start_flag in the NAL unit header? (M0152)

Adding a bit to all NUHs seems unnecessary, since would only apply to some NUTs, especially CRA.

- To signal tile group ID in the NAL unit header? (M0155, M0536)

Question about how this differs from using a fixed length field at the beginning of the tile group header.

Comment that tile group header has start code emulation while NUH does not.

May want to be used with a variable length NUH, since not all use cases would require this.

Can we save bitrates in parameter sets and headers for still picture profile?

- To specify an external-means provided flag, e.g., named HandleCraAsCvsStartFlag, for CRA pictures? (M0152)

HEVC has the same external-means flag, which is used in the decoding process.

BoG recommends to add external means flag HandleCraAsCvsStartFlag, with similar text as in HEVC. Text will be provided in a v3 of JVET-M0101.

- When HandleCraAsCvsStartFlag is equal to 0, should the CRA picture be required to have at least one associated leading picture (RASL or RADL)? (M0152)

- When HandleCraAsCvsStartFlag is equal to 1, should the CRA picture be disallowed to have any leading picture (RASL or RADL)? (M0152)

This would place a burden on the external means to remove leading pictures.

- How many NAL unit types for leading pictures (i.e., RASL and RADL)? (M0101)

  - 2?

  - 1? (M0153)

  - 0? (M0131)

BoG suggests discussion in the track to consider between adding 2 NUT values (for RASL and RADL) or staying with no dedicated NUT values for leading pictures.

- To signal random access type and number of leading pictures in the NAL unit header? (M0161)

Proponent not present. Revisit.

- To have a NAL unit type for STSA? (M0101)

BoG recommends to add a NUT value for STSA.

- To adopt the non-VCL NAL unit types as proposed in M0101?

Some broadcast application standards mandate the use of AUD for AVC and HEVC.

If we were to adopt a DPS, we would want to also add a NUT value for DPS.

BoG recommends to add a NUT value for AUD.

- To have nuh_component_type in the NAL unit header for indication of the component type such as YUV, RGB, depth map, alpha map, 3D geometry map, occupancy map, etc.? (M0156)

It was suggested that the HEVC layer_id design may be able to be used for this purpose. It was commented that some of these features are not within the current scope of the VVC design.

- To have nuh_pic_id in the NAL unit header, which has the same value with the 8-bit LSB values of POC, for AU detection purpose? (M0157)

It was commented that always including 8-bits in the NUH is expensive for use cases that don’t need this feature.

- To adopt the general decoding process text as proposed in M0101?

This includes adding sps_max_sub_layers_minus1 syntax element to SPS, which is used for signaling sub layer level idc.

BoG recommends to adopt adding sps_max_sub_layers_minus1 syntax element to SPS and decoding process in 8.1.1, 8.1.2 and 8.1.3 of M0101.

- To add HEVC-like sections 7.4.2.4 to 7.4.2.4.5 on NAL unit order and AU boundary detection as proposed in M0101?

BoG recommends to adopt text of sections 7.4.2.4 to 7.4.2.4.5 on NAL unit order and AU boundary detection from JVET-M0101, which is primarily editorial, but has some technical aspects.

- To have parameter set extension mechanism as in HEVC? (M0101)

Having parameter set extensibility is desirable.

It was suggested to consider if the extension flag and condition could be eliminated while still providing the extension capability.

Revisit.

- To adopt the profile level syntax as in M0101 as a starting point?

  - To have a tier flag? (M0101)

  - To have profile_space? (M0101)

There was agreement that the sub layer profile idc and sub_layer_constraint_info( ) is unnecessary, and that tier should be included.

BoG recommends to add profile_tier_level( ) syntax structure which includes sub layer level idc, subject to review of text.

BoG suggests discussion in the track about if profile_space should be included.

- To modify the semantics of general_non_packed_constraint_flag as in M0101, that begins: general_non_packed_constraint_flag equal to 1 indicates that the cropped output pictures of the decoded bitstream are suitable to be displayed

BoG recommends to add general_non_packed_constraint_flag with semantics as in M0101.

- To have a DPS as proposed in M0101?

  - Whether decoding parameter set should rather be a "bitstream header" NAL unit type, in which case the syntax would not include the parameter set ID and the standard would not have the activation process of the decoding parameter set?

There was agreement that having a decoding parameter set would be useful. Further study for the syntax and relationship with SPS syntax. It was questioned if having the DPS as a parameter set is necessary, since it will be the same for the entire bitstream. It was suggested that this should be able to be signaled via external means, even if parameter set structure is not used.

- To adopt the HEVC-like sub-bitstream extraction process as proposed in M0101?

BoG recommends to adopt the temporal scalability sub-bitstream extraction process in M0101.

## Reference picture management

Chaired by K. Suehring.

General discussion about LT picture functionality: it was asserted that the functionality is still useful. The term could be avoided to shorten the standard text.

To use a mechanism based on direct RPL signalling and derivation? (M0128, M0154)

The proposal is aiming for simplifying the process. The RPL approach does not have a separate list initialization and list modification process. It was pointed out, that the difference may not be measurable in terms of run time.

It is clear from the list construction process, which picture are active for prediction in the current picture.

Software is available.

It was mentioned that if reference picture list reordering is necessary RPS may be a bit more bit efficient.

There is no results available for reordering.

The proposed limit on RPL definitions in the SPS is 64 per list, which is twice as many as in RPS.

The difference of the two concepts seems small.

It was noted that the long-term reference concept should be tested as well as non-CTC GOP structures.

The BoG recommends to adopt M0128 (basic text version)

  - If yes, to explicitly signal reference pictures to be marked as "unused for reference"? (M0154)

Withdrawn.

  - If yes, to keep the LTRP functionality but remove the naming? (M0128 discussion)

No, use basic text.

- To use a mechanism based RPS? (M0378)

It was pointed out that the shortened text may create different results than the long text. The long text is the one that is proposed.

  - If yes, to keep the LTRP functionality? (M0378)

The proponents of M0128 pointed out that the proposed process avoids intermediate steps that are necessary for RPS bases reference picture list construction.

## Picture header and HPS

- To add HPS or picture header NAL unit with a reference-able ID? (M0132, M0451 proposal 2)

  - If yes, to contain only ALF parameters? (M0132)

  - If yes, to add the ID in the tile group header (M0132) or to replace the PPS ID in the tile group header (M0451 proposal 2)?

The main motivation is for coding efficiency savings by avoiding replicating ALF parameters for multiple tile groups in the same picture, or for multiple pictures.

## Misc. general HLS topics

- To allow parsing dependency between parameter sets? (M0133)

- To either add "_minus1" to the end of sps_ref_wraparound_offset, or to remove sps_ref_wraparound_enabled_flag and the value of sps_ref_wraparound_offset to be equal 0? If yes which option to take? (M0415 proposal 1)

BoG recommends adoption to change the sps_ref_wraparound_offset to sps_ref_wraparound_offset_minus1 and changing the units to be MinCbSizeY as in option 1, subject to review by the 360 BoG.

It was suggested to review the chroma sample interpolation process in section 8.3.6.3.4 of VVC WD3 to check if it can handle non-4:2:0.

- To split tile_group_type into multiple syntax elements as one of the following two ways? (M0386)

Proposed to no longer have I, P, B slice types, but to have flags to control the types of prediction (intra prediction, current picture referencing, inter prediction with one list or two lists)

It was agreed that this proposal is interesting, and option 1 was preferred.

It was suggested that the syntax should be flexible and not impose syntactic restrictions of tool combinations, and instead let those be profile constraints.

Should this be for picture signaling or tile group signalling?

BoG suggests discussion in the track.

- To create an AHG to further study the extraction of multiple frame rates from a VVC bit-stream with the following design goals? (M0579)

Temporal judder can be a problem with temporal scalability extraction of 60 fps from a 120 fps sequence, such as used by ATSC.

Would the de-judder post-processing be part of the conformance point definition, or could it be done as an SEI message.  This could be included in a mandate in the AHG to study. If done as an SEI message, could be done in JCT-VC for HEVC (or AVC).

BoG suggests discussion in the track.

## Interoperability and capability points definition and signalling

Chaired by K. Suehring

- To adopt the update to interoperability point syntax as proposed in M0451?

Ordering of syntax elements and naming was changed to match order and naming of tools in JVET-L1001.

7 new flags for tools adopted at the Macao JVET meeting.

The semantics was reviewed, “requirement of bitstream conformace” could be left out.

It was suggested to group the syntax element in a structure. It seems likely that the syntax would be moved to a DPS.

It was suggested to add reserved bits for future extension.

It was suggested to make the presence of the enable-flags dependent on the value of the constraint-flag.

The BoG recommends to adopt the 7 new flags as described in the contribution.

## Background

### HEVC NAL unit types

Table 7-1 – NAL unit type codes and NAL unit type classes

| nal_unit_type | Name of nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| 0 1 | TRAIL_N TRAIL_R | Coded slice segment of a non-TSA, non-STSA trailing picture slice_segment_layer_rbsp( ) | VCL |
| 2 3 | TSA_N TSA_R | Coded slice segment of a TSA picture slice_segment_layer_rbsp( ) | VCL |
| 4 5 | STSA_N STSA_R | Coded slice segment of an STSA picture slice_segment_layer_rbsp( ) | VCL |
| 6 7 | RADL_N RADL_R | Coded slice segment of a RADL picture slice_segment_layer_rbsp( ) | VCL |
| 8 9 | RASL_N RASL_R | Coded slice segment of a RASL picture slice_segment_layer_rbsp( ) | VCL |
| 10 12 14 | RSV_VCL_N10 RSV_VCL_N12 RSV_VCL_N14 | Reserved non-IRAP SLNR VCL NAL unit types | VCL |
| 11 13 15 | RSV_VCL_R11 RSV_VCL_R13 RSV_VCL_R15 | Reserved non-IRAP sub-layer reference VCL NAL unit types | VCL |
| 16 17 18 | BLA_W_LP BLA_W_RADL BLA_N_LP | Coded slice segment of a BLA picture slice_segment_layer_rbsp( ) | VCL |
| 19 20 | IDR_W_RADL IDR_N_LP | Coded slice segment of an IDR picture slice_segment_layer_rbsp( ) | VCL |
| 21 | CRA_NUT | Coded slice segment of a CRA picture slice_segment_layer_rbsp( ) | VCL |
| 22 23 | RSV_IRAP_VCL22 RSV_IRAP_VCL23 | Reserved IRAP VCL NAL unit types | VCL |
| 24..31 | RSV_VCL24.. RSV_VCL31 | Reserved non-IRAP VCL NAL unit types | VCL |
| 32 | VPS_NUT | Video parameter set video_parameter_set_rbsp( ) | non-VCL |
| 33 | SPS_NUT | Sequence parameter set seq_parameter_set_rbsp( ) | non-VCL |
| 34 | PPS_NUT | Picture parameter set pic_parameter_set_rbsp( ) | non-VCL |
| 35 | AUD_NUT | Access unit delimiter access_unit_delimiter_rbsp( ) | non-VCL |
| 36 | EOS_NUT | End of sequence end_of_seq_rbsp( ) | non-VCL |
| 37 | EOB_NUT | End of bitstream end_of_bitstream_rbsp( ) | non-VCL |
| 38 | FD_NUT | Filler data filler_data_rbsp( ) | non-VCL |
| 39 40 | PREFIX_SEI_NUT SUFFIX_SEI_NUT | Supplemental enhancement information sei_rbsp( ) | non-VCL |
| 41..47 | RSV_NVCL41.. RSV_NVCL47 | Reserved | non-VCL |
| 48..63 | UNSPEC48.. UNSPEC63 | Unspecified | non-VCL |

### HEVC NAL unit header syntax

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_unit_type | u(6) |
| nuh_layer_id | u(6) |
| nuh_temporal_id_plus1 | u(3) |
| } |  |

### VVC WD 3 NAL unit header syntax

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_unit_type | u(5) |
| nuh_temporal_id_plus1 | u(3) |
| nuh_reserved_zero_7bits | u(7) |
| } |  |

### VVC WD 3 NAL unit types

Table 7-1 – NAL unit type codes and NAL unit type classes

| nal_unit_type | Name of nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| 0 | NON_IRAP_NUT | Coded tile_group of a non-IRAP picture tile_group_layer_rbsp( ) | VCL |
| 1 | IRAP_NUT | Coded tile_group of an IRAP picture tile_group_layer_rbsp( ) | VCL |
| 2-15 | RSV_VCL_NUT | Reserved VCL NAL Units | VCL |
| 16 | SPS_NUT | Sequence parameter set seq_parameter_set_rbsp( ) | non-VCL |
| 17 | PPS_NUT | Picture parameter set pic_parameter_set_rbsp( ) | non-VCL |
| 18 | EOS_NUT | End of sequence end_of_seq_rbsp( ) | non-VCL |
| 19 | EOB_NUT | End of bitstream end_of_bitstream_rbsp( ) | non-VCL |
| 20, 21 | PREFIX_SEI_NUT SUFFIX_SEI_NUT | Supplemental enhancement information sei_rbsp( ) | non-VCL |
| 22-26 | RSV_NVCL | Reserved | non-VCL |
| 27-31 | UNSPEC | Unspecified | non-VCL |