| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0832-v1 |
| --- | --- |

| Title: | Non-CE3: On block size restrictions for PDPC with disabled linear filtering for PDPC in the case of skew non-diagonal modes |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexey Filippov, Vasily Rufitskiy, Jianle Chen Jinho Lee Jungwon Kang | Email: | alexey.filippov@huawei.com vasily.rufitskiy@huawei.com jianle.chen@huawei.com jinosoul@etri.re.kr jungwon@etri.re.kr |
| Source: | Huawei Technologies Co., ETRI |  |  |

_____________________________

# Abstract

This document describes 3 combinations of block size restrictions for PDPC proposed in JVET-M0122 with disabling linear filtering for PDPC in the case of skew non-diagonal modes as proposed in JVET-M0238. The current proposal is based on Tests 3, 5 and 7 of JVET-M0122.

# Introduction

PDPC (Position Dependent Prediction Combination) is an efficient tool used to improve compression performance of conventional intra prediction modes. However, PDPC can increase worst-case complexity. In this contribution, we propose several ways of how to reduce its adverse impact on worst-case complexity by disabling PDPC for some block sizes and intra prediction modes. To compensate for some coding efficiency loss caused by disabling PDPC, we modify calculation of PDPC scales to make them mode-dependent. For all the tests, linear filtering is disabled for PDPC in the case of skew non-diagonal modes.

# Proposal

In this contribution, we test the combinations of Tests 3, 5 and 7 described in JVET-M0122 with disabled linear filtering for PDPC in the case of skew non-diagonal modes as proposed in JVET-M0238 (see Table 1).

Table 1. Summary of the proposed tests

| Test # | Description |
| --- | --- |
| 1 | Test 3 of JVET-M0122 with disabled linear filtering for PDPC in the case of skew non-diagonal modes as proposed in JVET-M0238 |
| 2 | Test 5 of JVET-M0122 with disabled linear filtering for PDPC in the case of skew non-diagonal modes as proposed in JVET-M0238 |
| 3 | Test 7 of JVET-M0122 with disabled linear filtering for PDPC in the case of skew non-diagonal modes as proposed in JVET-M0238 |

# Experiment Results

The results reportedly obtained for Tests 1-3 using the common test conditions [1] are presented in Tables 2-4 for AI and RA configurations.

Table 2. Experimental results for Test 1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,05% | -0,10% | 0,00% | 101% | 98% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | 0,04% | 0,03% | 0,00% | 101% | 99% |
| Class C | 0,03% | -0,02% | -0,18% | 100% | 100% |
| Class E | 0,07% | -0,05% | 0,01% | 101% | 99% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | 0,05% | -0,10% | 0,02% | 100% | 100% |
| Class F | -0,12% | -0,17% | -0,12% | 102% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | 0,03% | 0,13% | 0,03% | 100% | 99% |
| Class C | 0,01% | -0,15% | -0,04% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | 0,02% | -0,45% | -0,32% | 100% | 99% |
| Class F | -0,10% | -0,15% | -0,05% | 100% | 99% |

Table 3. Experimental results for Test 2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,09% | -0,02% | -0,07% | 99% | 98% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | 0,03% | -0,03% | -0,02% | 99% | 98% |
| Class C | 0,00% | -0,11% | -0,10% | 100% | 100% |
| Class E | 0,02% | -0,03% | 0,01% | 100% | 99% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | 0,07% | -0,09% | -0,08% | 100% | 99% |
| Class F | -0,19% | -0,23% | -0,19% | 101% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | 0,01% | 0,14% | -0,01% | 99% | 99% |
| Class C | 0,06% | -0,19% | 0,02% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | 0,03% | -0,36% | -0,25% | 100% | 99% |
| Class F | -0,23% | -0,20% | -0,11% | 99% | 99% |

Table 4. Experimental results for Test 3

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,08% | 0,02% | 0,03% | 100% | 98% |
| Class A2 | -0,05% | -0,02% | -0,11% | 99% | 99% |
| Class B | 0,01% | 0,00% | -0,03% | 99% | 98% |
| Class C | -0,03% | -0,07% | -0,17% | 100% | 100% |
| Class E | 0,00% | -0,09% | -0,12% | 100% | 100% |
| Overall | -0,03% | -0,03% | -0,08% | 100% | 99% |
| Class D | 0,02% | -0,09% | -0,14% | 100% | 100% |
| Class F | -0,15% | -0,16% | -0,01% | 102% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | -0,01% | 0,16% | 0,07% | 100% | 99% |
| Class C | -0,01% | -0,13% | -0,07% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | 0,02% | -0,39% | -0,12% | 100% | 99% |
| Class F | -0,16% | -0,11% | -0,11% | 100% | 99% |

# Conclusion

We propose to adopt the proposed modifications for PDPC into the VTM software and the VVC spec draft.

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-L1010, 12th JVET Meeting, Macao, China, October 2018.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd and ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).